| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5110200 (5.11.2: Table of requirements for “Train Trip” procedure) Total: 13 Test case(s) |  |
| REF: | Subset-076-5-2-5110200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | Second version review for Baseline 3; Feature renamed : from FT5110300 to FT5110200 | INECO/MULTITEL |
| Version 2.9.1 05.07.2013 | All | Third version Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 11.07.2013 | All | Forth version TC10 (old TC 527.1): Deleted; new TC 10 is now the old FT5110300.TC8 (old 28.1); TC13 (old TC 573.1): 1 new requirement and 2 steps added; First delivery of UNISIG review comments included. | MULTITEL/INECO |
| Version 2.9.3 04.10.2013 | TC4, TC11, TC12, TC13 | Modifications requested by issues 13 to 16 of Review002_Subset-076-5-2-5110200v292 | MULTITEL |
| Version 2.9.4 05.12.2013 | All | Add missing tables: Sequence of test and message description This is a part of response to Review1.0- the other parts of the response will be delivered in v294 too | MULTITEL |
| Version 2.9.5 10.12.2013 | TC1 | Response to issue 2 from Review004_Subset-076-5-2-5110200_v294 | MULTITEL |
| Version 2.9.6 09.01.2014 | TC1, TC2, TC3, TC4, TC5, TC7, TC8, TC9, TC13 | Response to issues from Review001_Subset-076-5-2-5110200_v295 | MULTITEL |
| Version 2.9.7 13.01.2014 | TC3 | Response to issue 1 from Review001_Subset-076-5-2-5110200_v296 | MULTITEL |
| Version 2.9.8 22.01.2014 | TC12 | Apply of issue 1 from Review001_Subset-076-5-2-5110200_v297.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train is running in L0/LNTC. The train switches to TR mode due to the reception of a Message 16 ‘Unconditional Emergency Stop’. |  | 1 |
| Applicable Mode/Level Combinations | L0 : SB, UN LNTC : SB, SN |  |  |
| Target of Test | The train is running in L0/LNTC. The train switches to TR mode due to the reception of a Message 16 ‘Unconditional Emergency Stop’. The Emergency Brakes are applied. During the TR mode, while braking a border to a level 0 or NTC area is passed and the Emergency Brakes remain applied. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 D020 |  |
|  |  | Subset-026-5.11.2.2 S050 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train is in the TR mode, check - on the TIU interface that the train is still applying the EB; - on the DMI that the train is still applying the EB. |
| Constraints | - Train Trip is not inhibited. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4/6/13 | UN/SB/SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | Level transition order to level 2 |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC | UN/SB/SN | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | - | L0/ LNTC | UN/SB/SN | - |
| 2 | L0/ LNTC | UN/SB/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded | O | JRU | - | L0/ LNTC | UN/SB/SN | - |
| 3 | L0/ LNTC | UN/SB/SN | The on-board equipment switches to Level 2. | - | - | Use_FT5100100.13 If the on-board equipment is in L0 Use_FT5100100.21 If the on-board equipment is in LNTC | L2 | UN/SB/SN | - |
| 4 | L2 | TR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | On-board equipment switches to TR | L2 | TR | - |
| 5 | L2 | TR | Emergency brake commanded | O | TIU | No revocation until standstill and acknowledgement by driver | L2 | TR | - |
| 6 | L2 | TR | The EB application icon is DISPLAYED (ST01 icon in the C area). | O | DMI | - | L2 | TR | - |
| 7 | L2 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 | TR | - |
| 8 | L2 | TR | The on-board equipment switches to Level 0 or to Level NTC. | - | - | Use_FT5100100.13 If the on-board equipment switches to L0 Use_FT5100100.30 If the on-board equipment switches to LNTC | L0/LNTC | TR | - |
| 9 | L0/LNTC | TR | Emergency brake commanded | O | TIU | No revocation until standstill and acknowledgement by driver | L0/LNTC | TR | - |
| 10 | L0/LNTC | TR | The EB application icon is DISPLAYED (ST01 icon in the C area). | O | DMI | - | L0/LNTC | TR | - |
| 11 | L0/LNTC | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L0/LNTC | TR | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train is running in L1 with an MA, track description, and a stored track condition. The train switches to TR mode due to a balise group (BGa) sending a message with an incompatible ETCS version. During the TR mode, it receives an new MA which should be rejected and a new track condition which should be accepted. The track conditions received during the TR mode and before the switch to TR mode are applied during the TR mode and before it reaches standstill. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS / OS |  |  |
| Target of Test | When entering the TR mode, all current MA (if any) and track description data, except track conditions, shall be deleted. During the TR mode, new MA and track description data, except track conditions, shall not be accepted. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 D020 |  |
|  |  | Subset-026-5.11.2.2 A035 |  |
|  |  | Subset-026-4.8.4.2 Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |
|  |  | Subset-026-4.8.4.2 Movement Authority + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.10.1.3 line Movement Authority |  |
|  |  | Subset-026-4.10.1.3 line Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train switches to TR mode, check on the DMI that: - in the FS mode: the planning information area (D area) becomes empty, - in the FS / OS mode : the distance to target digital is removed. When the train is in the TR mode and receives a new MA and track description (including mode profile), check - that they are rejected because of TR mode : - on the TIU interface that the train is still applying the EB; - on the DMI that the train is still applying the EB; - on the DMI that the newly received MA should not be displayed in the planning area; - in the JRU that newly received MA should not be displayed in the planning area. When the train is in the TR mode and before it reaches standstill, check that : - the track conditions received during the TR mode is applied; - the track conditions received before the switch to TR mode is applied. |
| Constraints | - Train Trip is not inhibited. - If the on-board equipment is in the FS mode, OS mode, SR mode at switching to the TR mode, the train speed should be in the PIM, TSM or RSM supervision status (see the ERA_ERTMS_015560 v330 document). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Linking | STORED / NOT STORED | If the linking information is stored on-board and the BGs are marked as linked, they have to be included in the list of balises groups. |
| Movement Authority | STORED | L_ENDSECTION allowing the train to go further than the position where the train switches to TR |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Track conditions excluding big metal masses | STORED | M_TRACKCOND = 0101 b |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/ OS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | D0. Only in the OS mode. | L1 | FS/ OS | - |
| 2 | L1 | FS/ OS | The speed and distance monitoring information is displayed | O | DMI | D0. Only in the OS mode. Verify only that the distance to target digital is displayed. | L1 | FS/ OS | - |
| 3 | L1 | FS/ OS | The Track Condition symbol "Close air conditioning intake announcement" is displayed. | O | DMI | D1: The max safe front end of the train reaches a location in rear of the beginning of the “air tightness” area. The symbol TC19 shall be used to indicate an automatic execution/ the symbol TC21 when the driver is requested to act. | L1 | FS/ OS | - |
| 4 | L1 | FS/ OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=1¦Bit64=1) is recorded. | O | JRU | D1 | L1 | FS/ OS | - |
| 5 | L1 | FS/ OS | Close air tightness. | O | TIU | D1. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual and the driver action will be required. | L1 | FS/ OS | - |
| 6 | L1 | FS/ OS | The train switches to TR mode for another reason than EOA/LOA overpassed. | - | - | D2. Use_FT4060300.36 The train is in level 1/ 2 or 3 and passes over a balise group (BGa) sending a message with an incompatible ETCS version. The train is tripped and an indication is given to the driver. | L1 | TR | - |
| 7 | L1 | TR | EB is applied (until standstill and acknowledgement by driver) when entering TR mode. | O | TIU | D2. | L1 | TR | - |
| 8 | L1 | TR | The planning information area (D area) becomes empty because the MA and the track description data are deleted. | O | DMI | D3. Only if the on-board equipment was in the FS mode before switching to the TR mode. | L1 | TR | - |
| 9 | L1 | TR | The speed and distance monitoring information is removed | O | DMI | D3. Verify that the distance to target digital is removed. | L1 | TR | - |
| 10 | L1 | TR | The on-board is in TR mode. A new MA and track description covering all the MA length are read but rejected because of TR mode. A powerless section with pantograph to be lowered is received. | I | BTM | D4. A powerless section with pantograph to be lowered is received with : M_TRACKCOND = 3 D_TRACKCOND and L_TRACKCOND set in order to define this area as soon as possible and before the switch to PT. | L1 | TR | - |
| 11 | L1 | TR | The received BTM telegram is RECORDED. | O | JRU | D4. | L1 | TR | - |
| 12 | L1 | TR | EB is still applied. | O | TIU | D4. The received MA is not taken into account. | L1 | TR | - |
| 13 | L1 | TR | The EB application icon is still DISPLAYED (ST01 icon in the C area). | O | DMI | - | L1 | TR | - |
| 14 | L1 | TR | The newly received MA should not be displayed in the planning area. | O | DMI | The received MA is not taken into account. | L1 | TR | - |
| 15 | L1 | TR | The newly received track conditions should not be displayed in the planning area. | O | DMI | If “Air conditioning intake closed” is displayed at this distance/ the track condition newly received has not been rejected. | L1 | TR | - |
| 16 | L1 | TR | No EB revocation is RECORDED. | O | JRU | The received MA is not taken into account. | L1 | TR | - |
| 17 | L1 | TR | The powerless section where lowering of the pantograph (received during the the TR mode) should be indicated to the driver at this distance. | - | - | D4. Use_FT5180200.1 The on-board equipment has received track condition information indicating the location of a powerless section where lowering of the pantograph is required. The on-board equipment displays to the driver a profile indicating the location where lowering of the pantograph is required and where raising of the pantograph is permitted. | L1 | TR | - |
| 18 | L1 | TR | The Track Condition symbol "Close air conditioning intake announcement" is removed. | O | DMI | D5: The train reaches the distance of the track condition area received before the TRIP procedure started. Only if the related functionality is manually executed by the driver. Symbol is TC21. | L1 | TR | - |
| 19 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit64=0) is recorded. | O | JRU | D5. Only if the related functionality is manually executed by the driver. | L1 | TR | - |
| 20 | L1 | TR | The Track Condition symbol "Air conditioning intake closed" is displayed. | O | DMI | D5. Only if the related functionality is manually executed by the driver. Symbol is TC19. | L1 | TR | - |
| 21 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=1) is recorded. | O | JRU | D5. Only if the related functionality is manually executed by the driver. | L1 | TR | - |
| 22 | L1 | TR | The Track Condition symbol "Air conditioning intake closed" is removed. | O | DMI | D6: The min safe rear end of the train reaches the end location of the air tightness area. Symbol is TC19. | L1 | TR | - |
| 23 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=0) is recorded. | O | JRU | D6 | L1 | TR | - |
| 24 | L1 | TR | The Track Condition symbol "Open air conditioning intake" is displayed. | O | DMI | D6. The symbol TC20 shall be used to indicate an automatic execution/ the symbol TC22 when the driver is requested to act. | L1 | TR | - |
| 25 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=1¦Bit65=1) is recorded. | O | JRU | D6 | L1 | TR | - |
| 26 | L1 | TR | Close air tightness. | O | TIU | D6. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual/ the driver action will be required. | L1 | TR | - |
| 27 | L1 | TR | The Track Condition symbol "Open air conditioning intake" is removed. | O | DMI | D6 + 5 seconds. The symbol TC20 shall be used to indicate an automatic execution/ the symbol TC22 when the driver is requested to act. | L1 | TR | - |
| 28 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=0¦Bit65=0) is recorded. | O | JRU | D6 + 5 seconds. | L1 | TR | - |

| Step 10: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | - |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | 0 | - |
| N_TOTAL | 3 | 1 | Should be in accordance with the number of balises (N) in the BG. |
| M_DUP | 2 | FINITE VALUE | - |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_ENDSECTION | 15 | - | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International static speed profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | Set in order to cover the full length of the MA. |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | Set in order to cover the full length of the MA. |
| Q_GDIR | 1 | FINITE VALUE | - |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 d | - |
| D_TRACKCOND | 15 | FINITE VALUE | Set in order to put this air tightness area before the air tightness area (received before the on-board switches to the TR mode). |
| L_TRACKCOND | 15 | FINITE VALUE | Set in order to put this air tightness area before the air tightness area (received before the on-board switches to the TR mode). |
| M_TRACKCOND | 4 | 0011 b | Powerless section with pantograph to be lowered |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | 1 | Should be in accordance with the number of balises (N) in the BG. |
| N_TOTAL | 3 | 1 | Should be in accordance with the number of balises (N) in the BG. |
| M_DUP | 2 | FINITE VALUE | - |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train is running in L2 / L3 with an MA, track description, and a stored track condition. The train switches to TR mode due to a linking reaction. During the TR mode, it receives an new MA which should be rejected and a new track condition which should be accepted. The train stops, switches to the PT mode and continues in FS. The track conditions received before the switch to TR mode and during the TR mode are applied. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / OS |  |  |
| Target of Test | When entering the TR mode, all current MA (if any) and track description data, except track conditions, shall be deleted. During the TR mode, new MA and track description data, except track conditions, shall not be accepted. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 D020 |  |
|  |  | Subset-026-5.11.2.2 A030 |  |
|  |  | Subset-026-5.11.2.2 A035 |  |
|  |  | Subset-026-5.11.2.2 S050 |  |
|  |  | Subset-026-4.8.4.2 Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |
|  |  | Subset-026-4.8.4.2 Movement Authority + (optional) Mode Profile + (optional) List of Balises for SH area |  |
|  |  | Subset-026-4.10.1.3 line Movement Authority |  |
|  |  | Subset-026-4.10.1.3 line Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | - When the train switches to TR mode, check on the DMI that: - in the FS mode: the planning information area (D area) becomes empty, - in the FS / OS mode : the distance to target digital is removed. - When the train is in the TR mode and receives a new MA and track description (including mode profile), check that they are rejected because of TR mode : - on the TIU interface that the train is still applying the EB; - on the DMI that the train is still applying the EB; - on the DMI that the newly received MA should not be displayed in the planning area; - in the JRU that newly received MA should not be displayed in the planning area. - When the train exited the TR mode, exited the PT mode and continues in FS mode, check that : - the track conditions received before the switch to TR mode is applied; - the track conditions received during the TR mode is applied. |
| Constraints | - No "Emergency stop" are stored on-board. - Train Trip is not inhibited. - If the on-board equipment is in the FS mode or OS mode at switching to the TR mode, the train speed should be in the PIM, TSM or RSM supervision status (see the ERA_ERTMS_015560 v330 document). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | Established | "Ack of train data" has been received. |
| Linking | STORED | Q_LINKREACTION = 0 d (for the BG making the on-board equipment switching to TR. |
| Movement Authority | STORED | L_ENDSECTION allowing the train to go further than the position where the train switches to TR |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Track conditions excluding big metal masses | STORED | M_TRACKCOND = 0101 b |
| Unconditional Emergency Stop | NOT STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 *) | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/OS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | D0. Only in the OS mode. | L2/L3 | FS/OS | - |
| 2 | L2/L3 | FS/OS | The speed and distance monitoring information is displayed | O | DMI | D0. Only in the OS mode. Verify only that the distance to target digital is displayed. | L2/L3 | FS/OS | - |
| 3 | L2/L3 | FS/OS | The Track Condition symbol "Close air conditioning intake announcement" is displayed. | O | DMI | D1: The max safe front end of the train reaches a location in rear of the beginning of the “air tightness” area. The symbol TC19 is used to indicate an automatic execution and TC21 one, when the driver is requested to act. | L2/L3 | FS/OS | - |
| 4 | L2/L3 | FS/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit62=1¦Bit64=1) is recorded | O | JRU | D1. | L2/L3 | FS/OS | - |
| 5 | L2/L3 | FS/OS | Close air tightness. | O | TIU | D1. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual and the driver action will be required. | L2/L3 | FS/OS | - |
| 6 | L2/L3 | FS/OS | The train switches to TR mode for another reason than EOA/LOA overpassed. | - | - | D2. Use_FT4060300.25 The train is in FS / LS / OS mode and the train/engine performs transition to TR due to a linking reaction with the expected balise group. Transition Id [17] | L2/L3 | TR | - |
| 7 | L2/L3 | TR | EB is applied (until standstill and acknowledgement by driver) when entering TR mode. | O | TIU | D2. | L2/L3 | TR | - |
| 8 | L2/L3 | TR | The planning information area (D area) becomes empty because the MA and the track description data are deleted. | O | DMI | D2. Only if the on-board equipment was in the FS mode before switching to the TR mode. | L2/L3 | TR | - |
| 9 | L2/L3 | TR | The speed and distance monitoring information is removed | O | DMI | D2. Verify that the distance to target digital is removed. | L2/L3 | TR | - |
| 10 | L2/L3 | TR | A new MA and track description (including mode profile) are read but rejected because of TR mode. A second tightness area is received. | I | RTM | D3. A second area tightness area is received with :"M_TRACKCOND = 5"D_TRACKCOND and L_TRACKCOND set in order to put this air tightness area after the first air tightness area. | L2/L3 | TR | - |
| 11 | L2/L3 | TR | The received RTM message is RECORDED. | O | JRU | D3. | L2/L3 | TR | - |
| 12 | L2/L3 | TR | EB is still applied. | O | TIU | D3. The received MA is not taken into account. | L2/L3 | TR | - |
| 13 | L2/L3 | TR | The EB application icon is still DISPLAYED (ST01 icon in the C area). | O | DMI | D3. | L2/L3 | TR | - |
| 14 | L2/L3 | TR | The newly received MA should not be displayed in the planning area. | O | DMI | D3. The received MA is not taken into account. | L2/L3 | TR | - |
| 15 | L2/L3 | TR | No EB revocation is RECORDED. | O | JRU | D3. The received MA is not taken into account. | L2/L3 | TR | - |
| 16 | L2/L3 | TR | The on-board equipment switches from TR mode to PT mode. | - | - | D4. Use_FT5110200.13 Before the train reaches standstill/ no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1/ 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3/ a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. | L2/L3 | PT | - |
| 17 | L2/L3 | PT | The driver request a new MA and the RBC sends one MA that allows the train to continue in FS mode until the track condition area. | - | - | D4. Train is at standstill (V_TRAIN=0; TIME= 30). The on-board equipment is in PT mode. Use_FT5110200.5 Train is at standstill in L2/L3 and PT mode. The on-board reports mode change to RBC. After receiving RBC acknowledgement of exit Trip mode and no pending emergency stops to be revoked it is possible to accept new MA | L2/L3 | FS | - |
| 18 | L2/L3 | FS | The Track Condition symbol "Close air conditioning intake announcement" is removed. | O | DMI | The train starts running. D5: The train reaches the distance of the track condition area. Only if the related functionality is manually executed by the driver. Symbol is TC21. | L2/L3 | FS | - |
| 19 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit64=0) is recorded. | O | JRU | D5. Only if the related functionality is manually executed by the driver. | L2/L3 | FS | - |
| 20 | L2/L3 | FS | The Track Condition symbol "Air conditioning intake closed" is displayed. | O | DMI | D5. Only if the related functionality is manually executed by the driver. Symbol is TC19. | L2/L3 | FS | - |
| 21 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=1) is recorded. | O | JRU | D5. Only if the related functionality is manually executed by the driver. | L2/L3 | FS | - |
| 22 | L2/L3 | FS | The Track Condition symbol "Air conditioning intake closed" is removed. | O | DMI | D6: The min safe rear end of the train reaches the end location of the air tightness area (received before the transition to the TRIP mode started. Symbol is TC19. | L2/L3 | FS | - |
| 23 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=0) is recorded. | O | JRU | D6 | L2/L3 | FS | - |
| 24 | L2/L3 | FS | The Track Condition symbol "Open air conditioning intake" is displayed. | O | DMI | D6. The symbol TC20 shall be used to indicate an automatic execution/ the symbol TC22 when the driver is requested to act. | L2/L3 | FS | - |
| 25 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=1¦Bit65=1) is recorded. | O | JRU | D6 | L2/L3 | FS | - |
| 26 | L2/L3 | FS | Close air tightness. | O | TIU | D6. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual/ the driver action will be required. | L2/L3 | FS | - |
| 27 | L2/L3 | FS | The Track Condition symbol "Open air conditioning intake" is removed. | O | DMI | D7 | L2/L3 | FS | - |
| 28 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=0¦Bit65=0) is recorded. | O | JRU | D7 | L2/L3 | FS | - |
| 29 | L2/L3 | FS | The second air tightness area received during the TR mode has been stored and should be indicated to the driver at this distance. | - | - | D8. Use_FT5180600.1 The on-board equipment has received track condition information indicating the location of an air tightness section where is "Closing air conditioning intake" required. The on-board equipment displays to the driver a profile indicating the location where "Closing air conditioning intake" is required and where "Opening air conditioning intake" is permitted. | L2/L3 | FS | - |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 movement authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | Set in order to cover the full length of the MA. |
| Q_GDIR | 1 | FINITE VALUE | - |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International static speed profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | Set in order to cover the full length of the MA. |
| V_STATIC | 7 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | Track condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 d | - |
| D_TRACKCOND | 15 | FINITE VALUE | Set in order to put this air tightness area after the first (received before the on-board switches to the TR mode) air tightness area. |
| L_TRACKCOND | 15 | FINITE VALUE | Set in order to put this air tightness area after the first (received before the on-board switches to the TR mode) air tightness area. |
| M_TRACKCOND | 4 | 0101 b | Area tightness area |
| N_ITER | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | Established | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | Train is at standstill in (L2/L3) and PT mode. The on-board reported mode change to RBC and revoked all pending emergency stops, if any. Driver selects “start” and the on-board accepts a new MA from RBC.(new TC) |  | 4 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment offers to Driver the possibility to select “Start” After driver selection the on-board sends a MA request to the RBC and waits for the answer. The on-board request new MA to RBC SR authorization is received from RBC the on-board propose SR mode to the driver and requests for the acknowledgement |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.11.1.6 |  |
|  |  | Subset-026-5.11.2.2 S140 |  |
|  |  | Subset-026-5.11.2.2 S140 b) |  |
|  |  | Subset-026-5.11.2.2 S150 |  |
|  |  | Subset-026-5.11.2.2 S150 a) |  |
|  |  | Subset-026-5.11.2.2 S160 |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-8.7.1 Message 2: SR Authorisation |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : “Start” selection is displayed Hour glass symbol (ST05) is DISPLAYED SR mode propose to Driver, symbol MO10 is DISPLAYED. Check on the JRU that : Driver action is RECORDED An MA request, message 132 is RECORDED. The displayed symbol ST05 is RECORDED The acknowledgement for SR mode, symbol MO10 is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | Deleted when TR mode |
| Train Data | STORED | Previously entered |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The on-board offers the possibility to the driver to select “ Start” | O | DMI | (only if train data has been previously entered) | N/A | N/A | - |
| 2 | N/A | N/A | Driver selects “Start” | I | DMI | To request RBC for a New MA for L2/L3 | N/A | N/A | - |
| 3 | N/A | N/A | Driver action is RECORDED. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The on-board requests to RBC for a New MA by sending message 132 “MA request” | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | Radio message 132 is RECORDED | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The on-board equipment waits for RBC to answer. Hour glass symbol (ST05) is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The displayed symbol/ ST05 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Radio message 2: SR Authorisation is received from RBC | I | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | Radio message 2 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The mode acknowledgement symbol/ MO10 is DISPLAYED. | O | DMI | The on-board equipment proposes SR mode to the Driver and requests for the driver acknowledgment.3 | N/A | N/A | - |
| 11 | N/A | N/A | The displayed symbol/ MO10 is RECORDED. | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA Request |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| NID_ENGINE | 1 | 0 | No acknowledgement is required |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | Packet 0 or 1 |

| Step 8: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | must be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE, SPECIAL VALUES | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| Q_SCALE | 2 | FINITE VALUE |  |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1 |
| NID_BG(k) | 14 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | Established | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED Or UNKNOWN | Optional packet D_SR is STORED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 UNCHANGED | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The acknowledgement for SR mode, symbol MO14 is DISPLAYED. Post Trip, symbol MO06 (PT) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | Train is at standstill in L2/L3 and PT mode. The on-board has reported mode change to RBC. After receiving RBC acknowledgement of exit Trip mode and no pending emergency stops to be revoked it is possible to accept new MA. |  | 5 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that: - the RBC acknowledges PT mode and - RBC do not revokes emergency stop and - the driver selects “Start” and - the on-board request new MA to RBC and - MA and track description for FS mode is accepted |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.14.1.2.1 |  |
|  |  | Subset-026-4.8.3.1.1 Recognition of exit from TRIP mode |  |
|  |  | Subset-026-4.8.4.2 Recognition of exit from TRIP mode |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |
|  |  | Subset-026-5.11.2.2 A115 |  |
|  |  | Subset-026-5.11.2.2 S120 |  |
|  |  | Subset-026-5.11.2.2 D130 |  |
|  |  | Subset-026-5.11.2.2 S140 |  |
|  |  | Subset-026-5.11.2.2 S140 b) |  |
|  |  | Subset-026-5.11.2.2 S150 |  |
|  |  | Subset-026-5.11.2.2 S150 c) |  |
|  |  | Subset-026-8.7.3 Message 6: Recognition of exit from TRIP mode |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The reason (Trip Reason) is DISPLAYED while the mode is PT The driver is allowed to select Start Hour glass symbol (ST05) is DISPLAYED in Main Window Check on the JRU that : All messages exchanged between Track and Train are RECORDED. The driver action (select Start) is RECORDED The displayed symbol ST05 is RECORDED New MA, and Track data are RECORDED. |
| Constraints | No new MA or track data is accepted during trip mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | Previously entered |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | PT | SA-DATA.Indication with Euroradio Message "Recognition of exit from TRIP mode" (NID_MESSAGE=6) is received | I | RTM | New Messages are accepted after ack. from RBC and release of EB. RBC sends message 6: “Recognition of exist from TR” acknowledging the mode change. | N/A | PT | - |
| 2 | N/A | PT | RBC sends message 6: “Recognition of exist from TR” acknowledging the mode change. | O | JRU | Radio message 6 is RECORDED. | N/A | PT | - |
| 3 | N/A | PT | The Driver is allowed to select “Start” | O | DMI | (only if train data has been previously entered) There are no pending emergency stops to be revoked by RBC | N/A | PT | - |
| 4 | N/A | PT | The Trip Reason is DISPLAYED | O | DMI | The trip reason is displayed while the train is in PT mode | N/A | PT | - |
| 5 | N/A | PT | Driver selects “Start” | I | DMI | To request RBC for a New MA for L2/L3 | N/A | PT | - |
| 6 | N/A | PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | - | N/A | PT | - |
| 7 | N/A | PT | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | The on-board requests to RBC for a New MA by sending message 132 “MA request” | N/A | PT | - |
| 8 | N/A | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Radio message 132 is RECORDED | N/A | PT | - |
| 9 | N/A | PT | Hour glass symbol (ST05) is DISPLAYED. | O | DMI | The on-board equipment waits for RBC to answer. Hour glass symbol (ST05) is DISPLAYED. | N/A | PT | - |
| 10 | N/A | PT | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit42=1>) is recorded | O | JRU | The displayed symbol/ ST05 is RECORDED. | N/A | PT | - |
| 11 | N/A | PT | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Message 3: “Movement Authority” with packet 15 and optional packets as 21/ 27 (more packets can be added) | N/A | PT | - |
| 12 | N/A | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Radio message 3 is RECORDED. | N/A | PT | - |
| 13 | N/A | PT | The speed and distance monitoring information is displayed | O | DMI | - | N/A | FS | - |
| 14 | N/A | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | FS | - |
| 15 | N/A | FS | NOT The Trip Reason is DISPLAYED | O | DMI | - | N/A | FS | - |

| Step 1: Radio Message 6 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 6 | Recognition of exit from TRIP mode |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | No acknowledgement is required |
| NID_LRBG | 10+14 | FINITE VALUE | - |

| Step 7: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA Request |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| NID_ENGINE | 1 | 0 | No acknowledgement is required |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | Packet 0 or 1 |

| Step 11: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | must be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE, SPECIAL VALUES | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | new version |
| International SSP | STORED | new version |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED. | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train is at standstill, the driver acknowledges the train trip. The level is 0 or NTC and no valid Train Data is on-board, The on-board performs transition to SH mode. Transition Id [68]. (new TC) |  | 6 |
| Applicable Mode/Level Combinations | L0: TR LNTC: TR |  |  |
| Target of Test | The target of this test is to check that: When the train is at standstill, the driver acknowledges the trip transition and the level is 0 or NTC, And no valid Train Data is on-board, The on-board equipment performs the transition to SH mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.8.1.2 |  |
|  |  | Subset-026-4.6.2 row 4 column 12 |  |
|  |  | Subset-026-4.6.3 Condition Id [68] |  |
|  |  | Subset-026-5.11.2.2 D085 |  |
|  |  | Subset-026-5.11.2.2 A140 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Trip acknowledgement, symbol MO05 is displayed only when the train comes to standstill The mode initially displayed on the DMI changes to SH when the driver acknowledges the trip mode Check on the TIU that: The EB is RELEASED Check on the JRU that: The Trip acknowledgement, symbol MO05 is RECORDED at standstill. The acknowledgement of the driver is RECORDED The new mode SH, symbol MO01 is RECORDED. |
| Constraints | The train is at standstill and starting condition: no valid Train Data is on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Train Data | UNKNOWN/INVALID | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | TR | The request to driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED | O | DMI | As soon as the train has come to standstill | L0 / LNTC | TR | - |
| 2 | L0 / LNTC | TR | The displayed symbol MO05 is RECORDED. | O | JRU | - | L0 / LNTC | TR | - |
| 3 | L0 / LNTC | TR | Driver acknowledges train trip | I | DMI | - | L0 / LNTC | TR | - |
| 4 | L0 / LNTC | TR | Driver action is RECORDED | O | JRU | - | L0 / LNTC | TR | - |
| 5 | L0 / LNTC | TR | EB is RELEASED | O | TIU | - | L0 / LNTC | TR | - |
| 6 | L0 / LNTC | TR | EB release is RECORDED | O | JRU | - | L0 / LNTC | TR | - |
| 7 | L0 / LNTC | TR | The new SH mode/ symbol MO01 is DISPLAYED | O | DMI | On-board switches to SH mode When the driver acknowledges TR mode/ the equipment has no train data and the current level is 0 / NTC/ the onboard equipment transits to Shunting mode immediately/ | L0 / LNTC | SH | - |
| 8 | L0 / LNTC | SH | The displayed symbol MO01 is RECORDED. | O | JRU | - | L0 / LNTC | SH | - |
| 9 | L0 / LNTC | SH | EB is RELEASED | O | TIU | - | L0 / LNTC | SH | - |
| 10 | L0 / LNTC | SH | EB release is RECORDED | O | JRU | - | L0 / LNTC | SH | - |
| 11 | L0 / LNTC | SH | EB indication is REMOVED. | O | DMI | - | L0 / LNTC | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 CHANGED | SH |
| ERTMS/ETCS level | 0 / 1 UNCHANGED | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start“, because the train data are available on-board (new TC) |  | 7 |
| Applicable Mode/Level Combinations | L1: PT |  |  |
| Target of Test | The target of this test case is to check that in PT mode and level 1 the on-board equipment : allows the driver to select “Start” because Train Data are available on-board, SR mode is proposed to Driver |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 D110 |  |
|  |  | Subset-026-5.11.2.2 S140 |  |
|  |  | Subset-026-5.11.2.2 S140 a) |  |
|  |  | Subset-026-5.11.2.2 S160 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board equipment allows the driver to select “Start” the on-board propose SR mode to the driver, symbol MO10 is DISPLAYED. Check on the JRU that : Driver action is RECORDED. The acknowledgement of SR mode, symbol MO10 is RECORDED. |
| Constraints | Train data are stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Train Data | VALID | - |
| ERTMS/ETCS level | 2 | L1 The status of ERTMS Level is VALID |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The On-Board equipment waits for the driver to select “Start“. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The Driver selects “Start” | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Driver selection is RECORDED. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The Mode symbol "Acknowledge Staff Responsible" is displayed. | O | DMI | DMI Object: Symbol MO10 | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 2 UNCHANGED | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Acknowledge Staff Responsible" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) Desk is OPEN | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | In PT mode, the ERTMS/ETCS on-board equipment shall NOT offer the possibility to the driver to select „start“, because the train data is not available. |  | 8 |
| Applicable Mode/Level Combinations | L1: PT L2: PT L3. PT |  |  |
| Target of Test | The target of this test case is to check that in PT mode the on-board equipment : Does not allow the driver to select “Start”, Does only allow to select SH |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 S140 |  |
|  |  | Subset-026-5.11.2.2 S140 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment does not allow the driver to select “Start” The On-Board equipment allows the driver to select “SH” |
| Constraints | No valid train data are stored on-board. If the level is Level 2 / 3, the radio session must be open. No pending emergency stop stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | Mandatory if in Level 2 and 3 (if there was no radio session available the on-board should offer the possibility to select “Override EOA” as well |
| Unconditional Emergency Stop | NOT STORED | - |
| Train Data | INVALID or UNKNOWN | - |
| ERTMS/ETCS level | 2/ 3/ 4 | L1/ L2/ L3 The status of ERTMS Level is VALID |
| Driver ID | VALID | A valid Driver ID is available on-board |
| Train Running Number | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Mandatory if in Level 2 and 3 (if there was no radio session available the on-board should offer the possibility to select “Override EOA” as well |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <START> button in the <MAIN> window is disabled. | O | DMI | The On-Board equipment does not allows the driver to select “Start“ because no valid Train data available on-board. | N/A | N/A | - |
| 2 | N/A | N/A | The <SHUNTING> button in the <MAIN> window is enable. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Driver presses Shunting. | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | Driver action is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 UNCHANGED | PT |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Mandatory if in Level 2 and 3 |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) Desk is OPEN | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | Train is at standstill in (L2/L3) and PT mode. The on-board has reported mode change to RBC. After receiving RBC acknowledgement and emergency stop revocation it is possible to accept new MA |  | 9 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test is to check that: - The RBC acknowledges exit from TR mode and RBC revokes emergency stop - The driver selection of “Start” - The on-board request new MA to RBC - MA and track description for OS/LS/SH modes is accepted - After driver selection of mode the on-board requests for the acknowledgement |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Recognition of exit from TRIP mode |  |
|  |  | Subset-026-4.8.4.2 Recognition of exit from TRIP mode |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |
|  |  | Subset-026-5.11.2.2 A115 |  |
|  |  | Subset-026-5.11.2.2 S120 |  |
|  |  | Subset-026-5.11.2.2 D130 |  |
|  |  | Subset-026-5.11.2.2 S130 |  |
|  |  | Subset-026-5.11.2.2 S140 |  |
|  |  | Subset-026-5.11.2.2 S140 b) |  |
|  |  | Subset-026-5.11.2.2 S150 |  |
|  |  | Subset-026-5.11.2.2 S150 b) |  |
|  |  | Subset-026-5.11.2.2 S170 |  |
|  |  | Subset-026-8.7.3 Message 6: Recognition of exit from TRIP mode |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : “Start” selection is displayed Hour glass symbol (ST05) is DISPLAYED Check on the JRU that : All messages exchanged between Track and Train are RECORDED. Driver action is RECORDED. The displayed symbol ST05 is RECORDED. New MA, and Track data is RECORDED. |
| Constraints | Train is running in (L2/L3) TRIP mode, and after acknowledgement of mode change sends an PT report to RBC. No new MA or track data is accepted during trip mode. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | Deleted when entering TR mode |
| Movement Authority | NOT STORED | Deleted when TR mode |
| Gradient Profile | NOT STORED | Deleted when TR mode |
| International SSP | NOT STORED | Deleted when TR mode |
| Unconditional Emergency Stop | STORED | Either this or Conditional Emergency Stop must be stored on-board |
| Conditional Emergency Stop | STORED | Either this or Unconditional Emergency Stop must be stored on-board |
| Train Data | STORED | Previously entered |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| EB is RELEASED | O | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | RBC sends message 6: “Recognition of exist from TR” acknowledging the mode change. | I | RTM | New Messages are accepted after ack. from RBC and release of EB. | N/A | N/A | - |
| 2 | N/A | N/A | Radio message 6 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The RBC sends message 18 to revoke a pending Emergency Stop. | I | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | Radio message 18 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The on-board offers the possibility to the driver to select “ start” | O | DMI | (only if train data has been previously entered) | N/A | N/A | - |
| 6 | N/A | N/A | Driver selects “Start” | I | DMI | To request RBC for a New MA for L2/L3 | N/A | N/A | - |
| 7 | N/A | N/A | Driver action is RECORDED. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The on-board requests to RBC for a New MA by sending message 132 “MA request” | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | Radio message 132 is RECORDED | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The on-board equipment waits for RBC to answer. Hour glass symbol (ST05) is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The displayed symbol/ ST05 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Radio message 3: New MA with optional packets as SSP/ GP and Mode Profile are received and accepted from RBC | I | RTM | Message 3: “Movement Authority” with packet 15 and optional packets as 21/ 27/ 80 and 49.(for OS/ LS/ SH) | N/A | N/A | - |
| 13 | N/A | N/A | Radio message 3 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | New MA/ Track description and Mode Profile are RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | The on-board equipment proposes OS/LS/SH modes to the Driver. and requests for the driver acknowledgment. The mode acknowledgement symbol is DISPLAYED. | O | DMI | One of the symbols: MO02 - Acknowledgement for Shunting MO08 - Acknowledgement for On Sight MO22 - Acknowledgement for Limited Supervision | N/A | N/A | - |
| 16 | N/A | N/A | The displayed symbol is RECORDED. | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 6 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 6 | Recognition of exit from TRIP mode |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | No acknowledgement is required |
| NID_LRBG | 10+14 | FINITE VALUE | - |

| Step 3: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | Emergency stop revocation |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | 0 | No acknowledgement is required |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| NID_EM | - | - | Identification number of the emergency stop message |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA Request |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| NID_ENGINE | 1 | 0 | No acknowledgement is required |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | Packet 0 or 1 |

| Step 12: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | must be unique |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE, SPECIAL VALUES | On-board train time |
| M_ACK | 1 | 1 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | Required mode for a part of the MA 00 – OS 01 – SH 10 - LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Iteration levels(set to zero if only one section) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 UNCHANGED | PT |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED or UNKNOWN | Linking Information is stored if an optional packet 5 is included in the received message “Movement Authority” |
| Movement Authority | STORED | New MA is stored (after Release of SB and ACK from RBC) |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | OS/LS/SH |
| ERTMS/ETCS level | 3 / 4 | L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| EB is RELEASED. | O | TIU | - |
| Post Trip (PT) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The train has left an EOA/LOA and is TRIPPED. EB is being applied until standstill. No MA or track description data will be accepted during TRIP mode. |  | 10 |
| Applicable Mode/Level Combinations | L2: TR L3: TR |  |  |
| Target of Test | The target of this test is to check that: The train is braking in TRIP mode. EB is applied, until standstill. Current MA and track description data, except track conditions, is deleted (by on-board). No new MA or track data is accepted during trip mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.11.2.2 A035 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : - Current mode, symbol MO04 is DISPLAYED - EB symbol is DISPLAYED. Check on the TIU that: - EB is APPLIED. Check on the JRU that : - Current MA and track description are DELETED. - New MA and track description are not accepted. - EB is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | The on-board equipment has just changed to PT. The EB is still applied and will be released immediately in step 1, see below. |
| Current symbol mode is DISPLAYED. EB application is DISPLAYED. | O | DMI | MO04 ST01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Emergency Brake intervention" is displayed | O | DMI | Symbol ST01 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 12, packet 21, packet 27 is received | I | BTM | Select the corresponding step between 5 and 6 | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | The received message (either from BTM or RTM) is RECORDED. | O | JRU | The received MA and track description are not accepted. | N/A | N/A | - |
| 8 | N/A | N/A | Train reaches standstill | I | INT | EB applied | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International SSP |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 UNCHANGED | TR |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| Current symbol mode is DISPLAYED. EB application is DISPLAYED. | O | DMI | MO04 ST01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | The ETCS onboard equipment switches to SN Mode after the driver has acknowledged the train trip. The train is at standstill, level NTC and valid Train Data is on-board. Transition Id [63]. |  | 11 |
| Applicable Mode/Level Combinations | LNTC: TR |  |  |
| Target of Test | The target of this test is to check that: At standstill, in level NTC and valid Train Data is on-board. the onboard equipment switches to SN mode after the driver acknowledges train trip EB is released after that. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.13.1.4 |  |
|  |  | Subset-026-4.4.2.1 |  |
|  |  | Subset-026-4.6.2 row 16 column 12 |  |
|  |  | Subset-026-4.6.3 Condition Id [63] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Train Trip |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |
|  |  | Subset-026-5.11.2.2 S050 |  |
|  |  | Subset-026-5.11.2.2 S060 |  |
|  |  | Subset-026-5.11.2.2 D80 |  |
|  |  | Subset-026-5.11.2.2 D085 |  |
|  |  | Subset-026-5.11.2.2 D090 |  |
|  |  | Subset-026-5.11.2.2 A150 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : SN mode indication is DISPLAYED. EB indication is REMOVED. Check on the JRU that: Train trip acknowledgement, symbol MO05 is RECORDED. Driver acknowledgment is RECORDED SN Mode is RECORDED. EB release is RECORDED. Check on the TIU that: EB is RELEASED. |
| Constraints | The train is at standstill and starting condition: the train data must be valid. After the mode change to SN(level NTC), the supervision with regards to the train trip depends on the active NTC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Train Data | VALID | - |
| ERTMS/ETCS level | 1 | L NTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | TR | Request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED | O | DMI | As soon as the train has come to standstill | LNTC | TR | - |
| 2 | LNTC | TR | The displayed symbol MO05 for Trip acknowledgment is RECORDED | O | JRU | - | LNTC | TR | - |
| 3 | LNTC | TR | Driver acknowledges train trip. | I | DMI | - | LNTC | TR | - |
| 4 | LNTC | TR | Driver action is RECORDED. | O | JRU | - | LNTC | TR | - |
| 5 | LNTC | TR | SN Mode/ symbol MO19 is displayed. | O | DMI | The train switches to SN mode Valid Train Data is on-board | LNTC | SN | - |
| 6 | LNTC | SN | SN Mode/ symbol MO19 is RECORDED. | O | JRU | - | LNTC | SN | - |
| 7 | LNTC | SN | Emergency Brake is RELEASED. | O | TIU | - | LNTC | SN | - |
| 8 | LNTC | SN | EB release is RECORDED. | O | JRU | - | LNTC | SN | - |
| 9 | LNTC | SN | EB indication is REMOVED. | O | DMI | - | LNTC | SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 CHANGED | SN |
| Train Data | VALID | - |
| ERTMS/ETCS level | 1 UNCHANGED | L NTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | When the train is in level 0, Trip Mode, at standstill, valid Train Data on-board and the driver acknowledges the train trip, there is a transition to UN mode. Transition Id [62] |  | 12 |
| Applicable Mode/Level Combinations | L0 : TR |  |  |
| Target of Test | The target of this test is to check that: When the train is in level 0, Trip Mode, at standstill, valid Train Data on-board and the driver acknowledges the train trip, there is a transition to UN mode.. The EB is released after that. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.3 |  |
|  |  | Subset-026-4.6.2 row 11 column 12 |  |
|  |  | Subset-026-4.6.3 Condition Id [62] |  |
|  |  | Subset-026-4.4.13.1.4 |  |
|  |  | Subset-026-5.11.2.2 S050 |  |
|  |  | Subset-026-5.11.2.2 S060 |  |
|  |  | Subset-026-5.11.2.2 D80 |  |
|  |  | Subset-026-5.11.2.2 D085 |  |
|  |  | Subset-026-5.11.2.2 D090 |  |
|  |  | Subset-026-5.11.2.2 A145 |  |
|  |  | Subset-026-4.5.2.1 table - Determine train speed, train acceleration, train standstill |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The acknowledgment of TR mode, symbol MO05 is DISPLAYED to the driver at standstill. The mode initially displayed on the DMI changes to UN when the driver acknowledges the trip mode. Check on the TIU that: The EB is RELEASED Check on the JRU that: The Trip acknowledgment, symbol MO05 is RECORDED. Driver acknowledgment is RECORDED. The new mode UN is RECORDED. EB release is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is APPLIED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | TR | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED. | O | DMI | As soon as the train has come to standstill | L0 | TR | - |
| 2 | L0 | TR | The displayed symbol MO05 is RECORDED | O | JRU | - | L0 | TR | - |
| 3 | L0 | TR | Driver acknowledges train trip. | I | DMI | - | L0 | TR | - |
| 4 | L0 | TR | Driver action is RECORDED. | O | JRU | - | L0 | TR | - |
| 5 | L0 | TR | The new mode UN / symbol MO16 is DISPLAYED. | O | DMI | On-board switches to UN mode When the driver acknowledges TR mode and the current level is 0/ the onboard equipment transits to Unfitted mode immediately/ but if the previous mode was shunting/ the equipment has no train data/ so the transition to unfitted mode shouldn’t be allowed. | L0 | UN | - |
| 6 | L0 | UN | The new mode UN/ symbol MO16 is RECORDED. | O | JRU | - | L0 | UN | - |
| 7 | L0 | UN | EB is RELEASED. | O | TIU | - | L0 | UN | - |
| 8 | L0 | UN | EB release is RECORDED | O | JRU | - | L0 | UN | - |
| 9 | L0 | UN | EB indication is REMOVED. | O | DMI | - | L0 | UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 CHANGED | UN |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 UNCHANGED | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.11.2: Table of requirements for “Train Trip” procedure |  | 5110200 |
| Test Case of Feature | Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  | 13 |
| Applicable Mode/Level Combinations | L1 : TR L2 : TR L3 : TR |  |  |
| Target of Test | The target of this test case is to check that : no train TRIP acknowledgement message is displayed until standstill on-board switches to PT mode after Driver Acknowledgement and display the Trip Reason in level 2 or 3, the On-Board sends a position report to the track-side when the mode changes |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.13.1.4 |  |
|  |  | Subset-026-4.4.14.1.1 |  |
|  |  | Subset-026-4.4.14.1.2 |  |
|  |  | Subset-026-4.4.14.1.2.1 |  |
|  |  | Subset-026-4.6.2 row 13 column 12 |  |
|  |  | Subset-026-4.6.3 Condition Id [7] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Train Trip |  |
|  |  | Subset-026-4.7.2 table - Output information Trip reason |  |
|  |  | Subset-026-5.11.2.2 S050 |  |
|  |  | Subset-026-5.11.2.2 S060 |  |
|  |  | Subset-026-5.11.2.2 D80 |  |
|  |  | Subset-026-5.11.2.2 A105 |  |
|  |  | Subset-026-5.11.2.2 D110 |  |
|  |  | Subset-026-5.11.2.2 A115 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : - Trip acknowledgement (symbol MO05) is displayed only when the train comes to standstill. - The mode initially displayed on the DMI changes to PT when the driver acknowledges the trip mode. - The Trip Reason is displayed while the Train is in PT mode Check on the TIU that: - The EB is still APPLIED - The EB is RELEASED after mode change to PT Check on the JRU that : - The Trip acknowledgement, symbol MO05 is RECORDED at standstill. - The acknowledgement of the driver is recorded - The new current mode PT, symbol MO06 is recorded. - A position report is recorded |
| Constraints | During the execution of the test the train is at Standstill. In level 2 or 3, a communication session is opened. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | Only in level 2 or level 3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in level 2 or level 3 |
| EB is APPLIED | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill (V_TRAIN>0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | TR | No Request for driver acknowledgement to Train Trip is DISPLAYED | O | DMI | The train is still moving | L1 / L2 / L3 | TR | - |
| 2 | L1 / L2 / L3 | TR | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED | O | DMI | As soon as the train has come to standstill | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | The displayed symbol MO05is RECORDED | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | The driver acknowledges the TR Mode | I | DMI | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | The driver acknowledgement is RECORDED. | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | The new mode PT/ symbol MO06 is DISPLAYED | O | DMI | The on-board equipment switches to PT mode. | L1 / L2 / L3 | PT | - |
| 7 | L1 / L2 / L3 | PT | The new mode PT/ symbol MO06 is RECORDED | O | JRU | - | L1 / L2 / L3 | PT | - |
| 8 | L1 / L2 / L3 | PT | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Inform trackside about mode change with existing communication session Only in level 2/3 | L1 / L2 / L3 | PT | - |
| 9 | L1 / L2 / L3 | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in level 2/3 | L1 / L2 / L3 | PT | - |
| 10 | L1 / L2 / L3 | PT | The reason (Trip Reason) is DISPLAYED | O | DMI | The trip reason is displayed while the train is in PT mode | L1 / L2 / L3 | PT | - |
| 11 | L1 / L2 / L3 | PT | Emergency brake not commanded | O | TIU | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. | L1 / L2 / L3 | PT | - |
| 12 | L1 / L2 / L3 | PT | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | L1 / L2 / L3 | PT | - |
| 13 | L1 / L2 / L3 | PT | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | L1 / L2 / L3 | PT | - |
| 14 | L1 / L2 / L3 | PT | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | L1 / L2 / L3 | PT | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 CHANGED | PT |
| Radio communication session | ESTABLISHED | Only in level 2 or level 3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in level 2 or level 3 |
| Emergency brake is RELEASED. | O | TIU | - |
| PT mode, symbol MO06 is DISPLAYED The Trip Reason is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN = 0 |