| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3170200 (3.17.2: Determination of the operated system version) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-3170200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 01.08.2013 | All | Second version/Lab Internal modification: TC_Editorv27b tool used; Correction of Requirements blank spaces. | INECO |
| Version 2.9.2 25.09.2013 | All | Third version/ UNISIG comments from 26.08.2013 included | INECO |
| Version 2.9.3 02.12.2013 | All | Fourth version/ Updated after bilateral meeting with UNISIG on 29.11.2013 | INECO |
| Version 2.9.4 26.12.2013 | All | Fifth version/ Internal version: cross-reference checked; old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If the on-board equipment is in level 0, NTC or 1 (e.g. entrance in level 2/3 area), the RBC system version number X shall be operated when the transition to level 2/3 is executed. |  | 1 |
| Applicable Mode/Level Combinations | L0 : SH / UN / TR / NL LNTC : SH / TR / NL / SN L1: FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The RBC system version number X shall be operated when the transition to level 2/3 is executed. The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 a) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version number X is the same as the balise system version Operated system version number X after level transition is the same as the RBC system version number X Check on the RTM that: Message 32 RBC system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED |
| Constraints | The on-board equipment operates with one system version number X different to RBC system version number X but the same to non-RBC constituents and on system version ordered from BG (before level transition). NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| Radio communication session | NOT ESTABLISHED | M_VERSION=010 0000 |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Operated System Version | STORED | Version 1.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | One Balise Group message (N_TOTAL=1) containing packet 42 is received | I | BTM | 001 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-CONNECT. Request is transmitted | O | RTM | - | Mobile terminal is registered | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 4 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-CONNECT. Confirm is received | I | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 5 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 6 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 7 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | 010 0000 | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 8 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | Communication session established for on-board. | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 9 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted. | O | RTM | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 10 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | Communication session established for trackside RBC ID/Phone Number are valid | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 11 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | Train data send by on-board and ACK by RBC. | - | - | - | USE_FT4080401.1 Train data ACK by RBC | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 12 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | - | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 13 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 14 | L0/ LNTC/ L1 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV | Transition to level 2/3 is performed | - | - | - | USE_FT5100100.13 from L0 to L2 or USE_FT5100100.21 from LNTC to L2 or USE_FT5100100.37 from L1 to L2 or USE_FT5100100.26 from L0 to L3 or USE_FT5100100.20 from LNTC to L3 or USE_FT5100100.36 from L1 to L3 | L2 / L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 15 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 16 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | The driver presses "System version" | I | DMI | - | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 17 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 18 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0000) is received | I | BTM | 010 0000 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 19 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 20 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | The driver presses "System version" | I | DMI | - | - | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |
| 21 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | L2/ L3 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity: NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 18: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of SoM procedure or order received from trackside, if the on-board equipment is in level 2/3, the RBC system version number X shall be operated immediately. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: If the on-board equipment is in level 2/3 (SoM procedure), the RBC system version number X shall be operated immediately The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 b) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version number X is the same as the RBC system version number X Check on the RTM that: Message 32 RBC/RIU system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED at every operated system version change. |
| Constraints | The on-board equipment operates with one system version number X different to RBC system version number X. SoM procedure at S0. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC/RIU System Version | NOT STORED | - |
| Operated System Version | STORED | Version 1.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | - | Symbol MO13 | L2/L3 | SB | - |
| 2 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | Check inside common header that SYSTEM_VERSION variable is version 1.1 | L2/L3 | SB | - |
| 3 | L2/L3 | SB | The driver re-validates the "Driver ID" | I | DMI | - | Driver ID - TBR Coming from SB | Na | Na | - |
| 4 | L2/L3 | SB | The driver re-validates the “Train Running Number” | I | DMI | - | Train running number -TBR. Coming from SB | L2/L3 | SB | - |
| 5 | L2/L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; DRIVER_ID; NID_OPERATIONAL) is recorded | O | JRU | - | Check that two variables are recorded. | L2/L3 | SB | - |
| 6 | L2/L3 | SB | The Level symbol "Level 2/Level 3" is displayed | O | DMI | - | Symbol LE04 or LE05 ERTMS/ETCS Level and train position are valid | L2/L3 | SB | - |
| 7 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit4=1 or Bit5=1>) is recorded | O | JRU | - | - | L2/L3 | SB | - |
| 8 | L2/L3 | SB | SA-CONNECT. Request is transmitted | O | RTM | - | Mobile terminal is registered | L2/L3 | SB | - |
| 9 | L2/L3 | SB | SA-CONNECT. Confirm is received | I | RTM | - | - | L2/L3 | SB | - |
| 10 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | - | L2/L3 | SB | - |
| 11 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | L2/L3 | SB | - |
| 12 | L2/L3 | SB | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32; M_VERSION=010 0000) is received | I | RTM | 010 0000 | - | L2/L3 | SB | - |
| 13 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; 010 0001) is recorded | O | JRU | 010 0001 | Communication session established for on-board. | L2/L3 | SB | - |
| 14 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted. | O | RTM | - | - | L2/L3 | SB | - |
| 15 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | Communication session established for trackside RBC ID/Phone Number are valid | L2/L3 | SB | - |
| 16 | L2/L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L2/L3 | SB | - |
| 17 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157, Q_STATUS=1) containing packet 0 is transmitted | O | RTM | 010 0001 | - | L2/L3 | SB | - |
| 18 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=157; Q_STATUS=1; NID_PACKET=0) is recorded | O | JRU | 010 0001 | - | L2/L3 | SB | - |
| 19 | L2/L3 | SB | The driver presses "System version" | I | DMI | - | - | L2/L3 | SB | - |
| 20 | L2/L3 | SB | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 | L2/L3 | SB | - |

| Step 10: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 12: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version of the ERTMS/ETCS system |

| Step 14: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 17: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 1 | VALID |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 6 | SB |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of session established with an accepting RBC (RBC/RBC Handover), the accepting RBC system version number X shall be operated as soon as the engine has passed the RBC/RBC border location with its maximum safe front end. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR / TR / NL |  |  |
| Target of Test | The target of this test case is to check that: The accepting RBC system version number X shall be operated when the engine passes the RBC/RBC border location with its max safe front end. The on-board equipment shall operate with only one system version at a time. The system version of the RBC shall take precedence on the operated system version in relation to non-RBC constituents and system version ordered from balise group. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 c) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the Handing Over RBC system version Operated system version number X after handing over is the same as the Accepting RBC system version number X and the highest Operated system version number Y. Check on the RTM that: Message 32 RBC system version is RECEIVED. Check on the JRU that: Reception of message 32 is RECORDED General message is RECORDED at every operated system version change |
| Constraints | The on-board equipment operates with one system version number X different to accepting RBC system version number X Two communication sessions can be handled simultaneously. RBC Handover has a different system version number X than RBC accepting. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 7/ 11 /12 | FS/ OS/ SR/ TR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| RBC/RIU System Version | STORED | Handing over RBC system version 2.0 |
| Operated System Version | STORED | Version 2.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Handing over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | 010 0000 | Handing Over RBC: The radio message contains the RBC transition border. (D_RBCTR ≠ 0) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=001 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | Accepting RBC: A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | to the Accepting RBC | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | 001 0000 | Accepting RBC: The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111} | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted | O | RTM | - | Accepting RBC: The communication session with the Accepting RBC is established | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | - | Accepting RBC: The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | 001 0000 | Accepting RBC: This step does not apply for SL or NL mode. | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 17 | N/A | N/A | Train maximum safe front end location = LRBG + D_RBCTR | I | INT | - | Steps 18 and 20 are managed at the same time. | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Handing Over RBC. | O | RTM | - | Train location: The maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Accepting RBC. | O | RTM | - | Train position: The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10 M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 22 | N/A | N/A | The On board is supervised by the Accepting RBC. Reception of an MA from the Accepting RBC | - | - | - | USE_FT4080425.3 Transmission of L2/L3 movement Authority from Accepting RBC | N/A | N/A | - |
| 23 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0001) is recorded | O | JRU | - | Check inside common header that SYSTEM_VERSION variable is version 1.1 | N/A | N/A | - |
| 24 | N/A | N/A | The new MA is displayed | O | DMI | - | - | N/A | N/A | - |
| 25 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing the order to execute immediately the handover packet 131 (Q_DIR=0; Q_LOCACC=10) is received | I | BTM | 010 0000 | The balise group is placed at the RBC/RBC transition border.(D_RBCTR = 0) | N/A | N/A | - |
| 26 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0001) is recorded. | O | JRU | 001 0001 | - | N/A | N/A | - |
| 27 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 28 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 Every JRU message recorded contains the currently operated system version M_VERSION=001 0001 | N/A | N/A | - |
| 29 | N/A | N/A | Train min safe rear end location = LRBG + D_RBCTR+L_TRAIN | I | INT | - | The minimum rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 30 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted to Handing Over RBC. | O | RTM | - | - | N/A | N/A | - |
| 31 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | version 1.0 |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 13: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION (k) given only if M_VOLTAGE (k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 15: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 20 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 25: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Accepting RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 30: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG 1 |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 7/ 11/ 12 | FS/ OS/ SR/ TR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 1.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Connected with Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case of the on-board equipment switches from level 2/3 to another level, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: In case the on-board equipment switches from level 2/3 to another level, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. The on-board equipment shall operate with only one system version at a time. On receiving the order to operate system version from balise group, the on-board equipment shall immediately operate the system version number X given in the order. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.3 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 d) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the RBC system version before transition to Level 1 Operated system version is the same as the BG system version after Level transition to Level 1 Operated system version number X is the same as ordered by track side Operated system version number X is the same as the BG system version Check on the BTM that: Packet 41 Level transition order is RECEIVED. Packet 2 System version order is RECEIVED. Packet 5 Linking is RECEIVED. Check on the JRU that: Reception of packet 41 is RECORDED Reception of message 136 is RECORDED Reception of packet 2 is RECORDED Reception of packet 5 is RECORDED General message is RECORDED at every operated system version change |
| Constraints | The on-board equipment operates with one system version different to System version order. The RBC operated system version should be different from the operated system version of Non-RBC constituents. The TC does not terminate the communication session. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 /12 | FS / OS/ SR / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to Level 1 at a further location |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| RBC/RIU System Version | STORED | Version 1.0 |
| Operated System Version | STORED | Version 1.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | RBC System version is 1.0 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L2 / L3 | FS / LS / OS / SR | - |
| 2 | L2 / L3 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 Every JRU message recorded contains the currently operated system version M_VERSION=001 0001 | L2 / L3 | FS / LS / OS / SR | - |
| 3 | L2 / L3 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 41 (M_LEVELTR=2) is received | I | BTM | 010 0000 | - | L2 / L3 | FS / LS / OS / SR | - |
| 4 | L2 / L3 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L2 / L3 | FS / LS / OS / SR | - |
| 5 | L2 / L3 | FS / LS / OS / SR | The Level symbol "Level 1" is displayed | O | DMI | - | Level transition to level 1 is executed. DMI object: Symbol LE03 | L1 | FS / LS / OS / SR | - |
| 6 | L1 | FS / LS / OS / SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit3=1¦Bit4=0¦Bit5=0>) is recorded | O | JRU | - | - | L1 | FS / LS / OS / SR | - |
| 7 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L1 | FS / LS / OS / SR | - |
| 8 | L1 | FS / LS / OS / SR | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | - | Train passes the level transition border with its min safe rear end. | L1 | FS / LS / OS / SR | - |
| 9 | L1 | FS / LS / OS / SR | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L1 | FS / LS / OS / SR | - |
| 10 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 11 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | L1 | FS / LS / OS / SR | - |
| 12 | L1 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0001) is received | I | BTM | 001 0001 | - | L1 | FS / LS / OS / SR | - |
| 13 | L1 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | L1 | FS / LS / OS / SR | - |
| 14 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | L1 | FS / LS / OS / SR | - |
| 15 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 16 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 Every JRU message recorded contains the currently operated system version M_VERSION=001 0001 | L1 | FS / LS / OS / SR | - |
| 17 | L1 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1) containing packet 5 is received | I | BTM | 010 0000 | - | L1 | FS / LS / OS / SR | - |
| 18 | L1 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L1 | FS / LS / OS / SR | - |
| 19 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | L1 | FS / LS / OS / SR | - |
| 20 | L1 | FS / LS / OS / SR | The driver presses "System version" | I | DMI | - | - | L1 | FS / LS / OS / SR | - |
| 21 | L1 | FS / LS / OS / SR | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | L1 | FS / LS / OS / SR | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | - |
| M_LEVELTR | 3 | 2 | Level 1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 2 | Transition to level 1 |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0001 | Version 1.1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | FINITE VALUE | N. of balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY (k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ / 2/ 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | The TC does not terminate the communication session |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | In case the engine passes the RBC/RBC border location with its maximum safe front end and no session is established with the accepting RBC, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. |  | 5 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: In case the engine passes the RBC/RBC border location with its max safe front end and no session is established with the accepting RBC, the system version control in relation to non-RBC constituents shall be again applied and the balise group orders shall be again considered. The on-board equipment shall operate with only one system version at a time. On receiving the order to operate system version from balise group, the on-board equipment shall immediately operate the system version number X given in the order. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.3 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-3.17.2.8 |  |
|  |  | Subset-026-3.17.2.8 e) |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version number X is the same as the RBC system version before the handover execution Operated system version number X is the same as the BG system version after passing RBC/RBC border. Operated system version number X is the same as ordered by trackside Check on the BTM that: Packet 131 RBC transition order is RECEIVED. Packet 2 System version order is RECEIVED. Check on the RTM that: Message 136 Train position report is SENT. Check on the JRU that: General message is RECORDED after operated system version is changed Reception of packet 131 is RECORDED Message 136 is RECORDED Reception of packet 2 is RECORDED |
| Constraints | No session is established with accepting RBC. Only one communication session can be handled. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | Session established with handing RBC. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC/RIU System Version | STORED | Version 2.0 |
| Operated System Version | STORED | Version 2.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | with handing RBC. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 is received. | I | RTM | 010 0000 | The radio message from the Handing Over RBC contains the RBC transition border. D_RBCTR ≠ 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 3 | N/A | N/A | Train maximum safe front end location = DLRBG + D_RBCTR | I | INT | - | The maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | Message transmitted to Handing Over RBC. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 131 is received. | I | BTM | 001 0000 | The balise group is placed at the RBC/RBC transition border. D_RBCTR = 32767 | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 8 | N/A | N/A | Train min safe rear end location = LRBG + L_TRAIN + D_RBCTR | I | INT | - | The minimum rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | Message transmitted to Handing Over RBC. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; M_VERSION=010 0001 ) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 11 | N/A | N/A | The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | - | USE_FT3050500.5 (termination of communication session ordered by RBC) | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | ST03 is removed “No Connection”: absence of symbol. | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) | O | JRU | - | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-CONNECT. Confirm is not received | I | RTM | - | By any cause the communication session with Accepting RBC is not established. | N/A | N/A | - |
| 16 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=001 0000) is received | I | BTM | 010 0000 | - | N/A | N/A | - |
| 17 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 18 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 19 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 20 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 Every message recorded in JRU contains the currently operated system version M_VERSION=001 0001 | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 20 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | FINITE VALUE | No. of balise |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 32767 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Valid for the train |
| NID BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | BG 1 | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |

| Step 16: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 2.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 001 0000 | Version 1.0 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | NOT ESTABLISHED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Operated System Version | STORED | Version 1.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | No connection with Accepting RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If a mismatch has been detected between the country or region identifier read from a balise/loop and the corresponding identifier for which a set of national values is used onboard, the on-board equipment shall consider the system version number X transmitted by balise/loop as the operated one. |  | 6 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: If a mismatch has been detected between the country or region identifier read from a balise/loop and the corresponding identifier for which a set of national values is used onboard, the on-board equipment shall operate the system version number X transmitted by balise/loop. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.1.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.6 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the stored one before reading the BG Operated system version is the system version number X transmitted by balise after reading the BG Check on the BTM that: Packet 5 Linking is RECEIVED. Check on the JRU that: Reception of packet 5 is RECORDED General message is RECORDED after operated system version is changed |
| Constraints | The on-board equipment operates with one system version different to the system version number X transmitted by balise. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| National Values | STORED | NID_C (1) |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (NID_C(2)) containing packet 5 is received | I | BTM | 001 0000 | NID_C(2) different from NID_C(1) stored | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=001 0001) is recorded | O | JRU | 001 0001 | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 1.1 Every JRU message recorded contains the currently operated system version M_VERSION=001 0001 | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 010 0000 | Version 1.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C(2) different from NID_C(1) stored |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 5 | Linking |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY (k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| National Values | STORED | NID_C (2) |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 1.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | If the on-board equipment does not support the system version number X transmitted by a non-RBC trackside constituent or the one specified in the balise group order, it shall consider the operated system version as unchanged. |  | 7 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR / SH / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: On-board equipment shall consider the operated system version as unchanged if the on-board equipment does not support the system version number X transmitted in the balise group order. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.7 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version number X is the same as the stored one before reading the BG Operated system version number X remains unchanged after reading the BG Check on the BTM that: Packet 2 "System version order" is RECEIVED. Check on the JRU that: Reception of packet 2 is RECORDED A General Message ( M_VERSION changed) is NOT recorded |
| Constraints | The on-board equipment does not support the system version number X transmitted in the order by balise group order. The non-RBC constituent operates with same system version as the balise group order. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is 2.1 Every message recorded in JRU contains the currently operated system version M_VERSION=010 0001 | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 2 (M_VERSION=100 0000) is received | I | BTM | 100 0000 | System version 4.0 ordered by the balise group version is not supported by the system. | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=010 0001) is recorded | O | JRU | 010 0001 | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version remains unchanged 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | 100 0000 | Version 4.0 |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | 100 0000 | Version 4.0 Version not supported |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 6/ 7/ 8/ 11/12/ 14 | FS/ OS/ SR/ SH/ TR/ PT/ NL/ LS/ RV |
| ERTMS/ETCS level | 2 | L1 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | The system version currently operated when the on-board equipment is switched off (i.e. enters No Power mode) shall be retained and re-used when powered on. |  | 8 |
| Applicable Mode/Level Combinations | L0 : SH / UN / SB / TR / NL LNTC : SH / SB / TR / NL / SN L1 / L2 / L3: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The system version operated when the on-board equipment is switched off shall be retained and re-used when powered on. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.9 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.10.1.3 line Operated System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version is the same as the stored one before entering NP mode. Check on the JRU that: A general message indicating a change of operated System version changed is not recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 6/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | Operated system version is 2.1 Every message recorded in JRU contains the currently operated system version M_VERSION=010 0001 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The on-board switches to NP mode | - | - | USE_FT4060300.1 | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | NP | The on-board switches to SB mode from NP mode. | - | - | USE_FT4060300.10 The system version shall be re-used when on-board equipment is powered on. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | Desk is opened. | - | - | USE_FT5040300.24 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI shows the operated system version | O | DMI | Operated system version is 2.1 Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | Acceptance of System Version Order by modes and levels. |  | 9 |
| Applicable Mode/Level Combinations | L0 : SH/ UN/ PS/ SL/ SB/ TR/ NL LNTC: SH/ PS/ SL/ SB/ TR/ NL/ SN L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV L2: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives an order to change the Operating System version from one Balise group. The on-board equipment stores and accepts the information and operates immediately with the system version number X given in the order. Only data related with one system version can be contained in the message. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.8.3.1.1 System Version order |  |
|  |  | Subset-026-4.8.4.2 System Version order |  |
|  |  | Subset-026-7.4.2.1 Packet Number 2: System Version order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operating System version number X, same values as the one ordered by packet 2, is displayed. Check on the JRU: The balise message with packet 2 received by the on-board is RECORDED. New Operating system version is RECORDED. |
| Constraints | The on-board equipment operates with one system version number X different to System Version number X order. Packet 2 shall be set with a value for M_VERSION different from the currently operating one. NOTE: Rule used to define M_VERSION column: For RBC and BTM steps the M_VERSION represents the version of the components and in case of JRU steps the M_VERSION values is the CURRENTLY OPERATED VERSION. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3 / 4 | L0/ LNTC/ L1/ L2 / L3 |
| Operated System Version | STORED | Version A |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 2 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Operated system version is A Every JRU message recorded contains the currently operating system version | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 2 (M_VERSION=B) is received | I | BTM | A | M_VERSION number X contained in the order is different from the current operating one | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; M_VERSION=B) is recorded | O | JRU | B | - | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_VERSION=B) is recorded | O | JRU | B | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver presses "System version" | I | DMI | - | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI shows the operated system version | O | DMI | - | Check that Operated system version changed to M_VERSION value number X given in the order and is different from the initial one. Every JRU message recorded contains the currently operating system version | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | A (Actual Operated System Version) |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 2 | System version order |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | B (Different value from Operating System Version number X) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3 / 4 | L0/ LNTC/ L1/ L2 / L3 |
| Operated System Version | STORED | Version B |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.17.2: Determination of the operated system version |  | 3170200 |
| Test Case of Feature | The system version currently operated when the on-board equipment loses the information (failure situation), the highest supported system version shall be used. |  | 10 |
| Applicable Mode/Level Combinations | L0 : SH / UN / SB / TR / NL LNTC : SH / SB / TR / NL / SN L1 / L2 / L3: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that: The system version operated when the on-board equipment loses the information (failure situation) shall be the highest supported system version. The on-board equipment shall operate with only one system version at a time. The function "System Version Management" is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.2.1 |  |
|  |  | Subset-026-3.17.2.2 |  |
|  |  | Subset-026-3.17.2.9.1 |  |
|  |  | Subset-026-4.5.2.1 table - System Version Management |  |
|  |  | Subset-026-4.7.2 table - Output information Operated System Version |  |
|  |  | Subset-026-4.10.1.3 line Operated System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Operated system version after on-board equipment loses the information (failure situation) is the highest supported system version. Check on the JRU that: A general message indicating a change of operated System version changed is recorded |
| Constraints | The on-board equipment operates with one system version different to the highest supported system version. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 6/ 7/ 8/ 11/12/ 13/ 14 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 1.1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | DMI shows the operated system version | O | DMI | Operated system version is 1.1 Every message recorded in JRU contains the currently operated system version M_VERSION=001 0001 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ SB/ UN/ TR/ PT/ NL/ LS/ SN/ RV | The on-board switches to SF mode | - | - | USE_FT4060300.4 The system version shall be changed by the highest supported system version. | L0/ LNTC/ L1/ L2/ L3 | SF | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SF | The on-board switches to NP mode form SF mode | - | - | USE_FT4060300.1 | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | NP | The on-board switches to SB mode from NP mode. | - | - | USE_FT4060300.10 The system version shall be re-used when on-board equipment is powered on. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | Desk is opened. | - | - | USE_FT5040300.24 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver presses "System version" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 9 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI shows the operated system version | O | DMI | Operated system version is 2.1 (highest supported system version). Every JRU message recorded contains the currently operated system version M_VERSION=010 0001 | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Operated System Version | STORED | Version 2.1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |