| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 9990500 (A3.5: Handling of Actions in Specific Situations) Total: 39 Test case(s) |  |
| REF: | Subset-076-5-2-9990500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 04.11.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | All | corrected preceded-/followed by and Use steps | DLR |
| Version 2.9.3 12.12.2013 | All | review | DLR |
| Version 2.9.4 28.01.2014 | TC 38 | Three steps (9,10 and 13) in the "Sequence of Steps" table have been removed | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 10.03.2015 | 36, 37 | Created new TC | DLR |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the change of National Values, which is ordered from the trackside for a specific location, is not reverted when passing over the location again moving backwards. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, OS, SR SH, SL, NL, LS L2: FS, OS, SR SH, SL, NL, LS L3: FS, OS, SR SH, SL, NL, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment does not revert its National Values, when passing over the defined location again moving backwards. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • the driver can change the Driver ID while running (M_NVDERUN = 1) Check on the JRU that: • the new Driver ID is recorded |
| Constraints | The National Values with M_NVDERUN=1, valid from D_VALIDNV on, has been received on-board. The current Driver ID entered during the Start of Mission is ID0. Besides, the value of D_NVROLL has to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 11 / 12 / 13 | FS / OS / SR / SH / UN / SL / NL / LS / SN |
| National Values | STORED | M_NVDERUN=0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | The train passes over the location where the last stored NV shall start being valid (defined by D_VALIDNV) The National Value M_NVDERUN changes from 0 to 1 | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Main” | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects “Driver ID” | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The Driver ID window is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies the "Driver ID" | I | DMI | Driver ID = ID1 | N/A | N/A | - |
| 7 | N/A | N/A | The driver validates the "Driver ID" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=ID1) is recorded | O | JRU | The new Driver ID value is recorded | N/A | N/A | - |
| 9 | N/A | N/A | The driver closes the "Main" window | I | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | Train is at standstill | I | INT | The train will have to be stopped very close to the location defined by D_VALIDNV, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 12 | N/A | N/A | The train moves backwards and overpasses the location defined by the stored D_VALIDNV | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | Train is at standstill | I | INT | The train has to be stopped far enough to allow to complete the steps from 15 to 23 before passing the location where the last stored NV shall start being valid (defined by D_VALIDNV) | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving | I | INT | The train moves slow enough to allow to complete the steps from 15 to 23 before passing the location where the last stored NV shall start being valid (defined by D_VALIDNV) | N/A | N/A | - |
| 15 | N/A | N/A | The driver selects “Main” | I | DMI | - | N/A | N/A | - |
| 16 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 17 | N/A | N/A | The driver selects “Driver ID” | I | DMI | - | N/A | N/A | - |
| 18 | N/A | N/A | The Driver ID window is displayed | O | DMI | - | N/A | N/A | - |
| 19 | N/A | N/A | The driver modifies the "Driver ID" | I | DMI | Driver ID = ID0 | N/A | N/A | - |
| 20 | N/A | N/A | The driver validates the "Driver ID" | I | DMI | - | N/A | N/A | - |
| 21 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=ID0) is recorded | O | JRU | The new Driver ID value is recorded | N/A | N/A | - |
| 22 | N/A | N/A | The driver closes the "Main" window | I | DMI | - | N/A | N/A | - |
| 23 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |
| 24 | N/A | N/A | Train is moving | I | INT | The train passes over the location where the last stored NV shall start being valid (defined by D_VALIDNV) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 11 / 12 / 13 | FS / OS / SR / SH / UN / SL / NL / LS / SN |
| National Values | STORED | M_NVDERUN=1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to RBC/RBC handover will be taken only once, although the train passes over a defined location more than once. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes an action related to RBC/RBC only once, although it passes over a defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 f) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the Handing over RBC only once when it reaches the location for the RBC transition. Check on the JRU that : • The information sent to the Handing over RBC is recorded only once. |
| Constraints | There is an RBC transition order stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With the Handing-over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The maximum safe front end of the train passes the location of the RBC transition. The train maximum safe front end position is equal to D_LRBG + D_RBCTR. A radio message 136 (train position report) is sent to the Handing Over RBC. | N/A | N/A | - |
| 2 | N/A | N/A | The radio message 136 to the Handing Over RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The maximum safe front end of the train passes the location of the RBC transition. A radio message 136 (train position report) is sent to the Accepting RBC. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message 136 to the Accepting RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped so that its max safe front end is very close to the location defined by D_LRBG + D_RBCTR, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the location defined by D_LRBG + D_RBCTR with its max safe front end again. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | No position report is sent to the Handing over RBC. | N/A | N/A | - |
| 8 | N/A | N/A | No position report to the Handing over RBC is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 10 | N/A | N/A | Use_FT4080425.4,5,7 Transmission of L2/L3 movement Authority from Handing over RBC | - | - | Since the on-board is no longer supervised by the handing over RBC, it will not use handing over MA’s | N/A | N/A | - |
| 11 | N/A | N/A | The MA sent by the Handing over RBC is not displayed. | O | DMI | The MA has been rejected. | N/A | N/A | - |
| 12 | N/A | N/A | The train moves forward and overpasses the location defined by D_LRBG + D_RBCTR with its max safe front end again. | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | No position report is sent to the Handing over RBC. | N/A | N/A | - |
| 14 | N/A | N/A | No position report to the Handing over RBC is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With both RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to track condition information, with the exception of big metal masses, will be taken only once, although the train passes over a defined location more than once. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, NL L2: FS, LS, OS, NL L3: FS, LS, OS, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment performs an action only once, although it passes over a defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The action to be performed (low pantograph) is performed only once |
| Constraints | There is some track condition information stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 11 / 12 | FS / OS / NL / LS |
| Track conditions excluding big metal masses | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | When the max safe front end of the train reaches the location defined by D_LRBG + D_TRACKCOND, the pantograph output of the TIU is activated (lower pantograph) | O | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped so that its max safe front end is very close to the location defined by D_LRBG + D_TRACKCOND, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 3 | N/A | N/A | The train moves backwards and overpasses the location defined by D_LRBG + D_TRACKCOND with its max safe front end. | I | INT | - | N/A | N/A | - |
| 4 | N/A | N/A | No new action is performed concerning the pantograph. | O | TIU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 6 | N/A | N/A | The train moves forward and overpasses the location defined by D_LRBG + D_TRACKCOND with its max safe front end. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | No new action is performed concerning the pantograph. | O | TIU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 11 / 12 | FS / OS / NL / LS |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the permission to initiate Reversing mode is only displayed once, right before passing over the end of the Reversing area, but it is not shown again after passing again over this location (due to backwards movement: initiated by the driver or due to roll-away, but not because the driver selects RV mode). |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment permits to initiate Reversing mode only once, although it passes over a defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 h) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The permission to initiate Reversing mode is displayed only once Check on the JRU that : • The permission to initiate Reversing mode is recorded only once |
| Constraints | There is some reversing area information stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver stops the train inside the area where reversing is permitted. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that the reversing of movement is permitted. | O | DMI | The driver does not select RV mode | N/A | N/A | - |
| 3 | N/A | N/A | The information to the driver is recorded. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The train starts moving again. | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train a bit further away from the end of the reversing area. | I | INT | The train will have to be stopped very close to the end of the reversing area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the end of the reversing area again. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 8 | N/A | N/A | No information is displayed about the reversing of movement. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | It is not possible to select RV mode. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the information about track ahead free request is displayed only once, although the train passes over the start location of the area more than once. |  | 5 |
| Applicable Mode/Level Combinations | L2: LS, OS, SR L3: LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests Track Ahead free only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 i) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The Track Ahead free request is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | There is a Track Ahead Free request stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Track Ahead Free Request | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Track Ahead Free Request | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ‘Track Ahead Free Request’ is displayed to the driver, according to the D_LRBG + D_TAFDISPLAY. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | Display of ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges the ‘Track Ahead Free Request’ | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver acknowledgement is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | Radio message 149 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | ‘Track Ahead Free Request’ is removed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Removal ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by D_LRBG + D_TAFDISPLAY, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 10 | N/A | N/A | The train moves backwards and overpasses the location defined by D_LRBG + D_TAFDISPLAY again. | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 14 | N/A | N/A | The train moves forward and overpasses the location defined by D_LRBG + D_TAFDISPLAY again. | I | INT | - | N/A | N/A | - |
| 15 | N/A | N/A | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - | N/A | N/A | - |
| 16 | N/A | N/A | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - | N/A | N/A | - |

| Step 5: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the information about track ahead free request is removed once and for all, although the train passes over the location where the information has to be removed more than once. |  | 6 |
| Applicable Mode/Level Combinations | L2: LS, OS, SR L3: LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops requesting the Track Ahead free only once, although passes over the defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 i) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The Track Ahead free request is removed only once and it is not display again after moving backwards. Check on the JRU that : • The removal of the message is recorded only once |
| Constraints | There is a Track Ahead Free request stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Track Ahead Free Request | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Track Ahead Free is being requested | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ‘Track Ahead Free Request’ is being displayed to the driver. | O | DMI | The driver does not acknowledge the ‘Track Ahead Free Request’. | N/A | N/A | - |
| 2 | N/A | N/A | The train overpasses the end of the “Track Ahead Free Request” area. | I | INT | - | N/A | N/A | - |
| 3 | N/A | N/A | ‘Track Ahead Free Request’ is removed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | Removal of ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the end of the TAF area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the location of the end of the TAF area again. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 10 | N/A | N/A | The train moves forward and overpasses the location of the end of the TAF area again. | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | No ‘Track Ahead Free Request’ is displayed. | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | No ‘Track Ahead Free Request’ is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the location to start calculating the geographical position is taken into account only once, although the train passes over the location more than once |  | 7 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, NL L2: FS, LS, OS, SR, NL L3: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes into account the starting location of the geographical position only once, although it passes over the defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 j) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The geographical position starts to be displayed, on request from the driver, only once |
| Constraints | There is some Geographical position information stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Geographical Position | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train reaches the location where the geographical position starts to be calculated. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified in the stored Packet 79. | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests of The Geographical Position DISPLAY is recorded in the JRU | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The Geographical position is DISPLAYED according to the stored value of M_POSITION | O | DMI | Route kilometre (variable M_POSITION) is based on the received packet 79. | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the beginning of the area where the geographical position starts to be calculated, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the location where the geographical position started to be calculated. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | The geographical position continues to be displayed as normal. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 9 | N/A | N/A | The train moves forward and overpasses the location where the geographical position started to be calculated. | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | The geographical position continues to be displayed as normal. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Geographical Position | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the location to stop calculating the geographical position is taken into account only once, although the train passes over the location more than once |  | 8 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, NL L2: FS, LS, OS, SR, NL L3: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes into account the location to stop calculating the geographical position only once, although it passes over the defined location more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 j) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The geographical position stops to be displayed |
| Constraints | The geographical position has been received and it is being calculated on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Geographical Position | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train reaches the location where the geographical position stops to be calculated. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified in the stored Packet 79. | N/A | N/A | - |
| 3 | N/A | N/A | No Geographical position is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the end of the area where the geographical position is calculated, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 5 | N/A | N/A | The train moves backwards and overpasses the location where the geographical position stopped to be calculated. | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests the DISPLAY of the Geographical Position | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | No Geographical position is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Geographical Position | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that a level transition order is executed once, and it is not reverted when moving backwards and passing over the defined location again. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, SL, NL L2: FS, LS, OS, SR, SL, NL L3: FS, LS, OS, SR, SL, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment executes the level transition once, although it passes over the defined location in backwards direction. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The level transition is not reverted when passing over the same location again in backwards direction Check on the JRU that : • The level transition is RECORDED once |
| Constraints | There is a level transition announcement stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 11 / 12 | FS / OS / SR / UN / SL / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train passes over the location where the stored level transition order has to be executed. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The train changes from Level X to Level Y. The new Level Y is displayed | O | DMI | Levels X and Y will depend on the actual test. | N/A | N/A | - |
| 3 | N/A | N/A | The new Level Y is recorded. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by the stored value of D_LEVELTR, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 5 | N/A | N/A | The train moves backwards and overpasses the location of the already executed level transition order. | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The level is not reverted. The current level is still Level Y. | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | No level change is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The train is stopped. | I | INT | The brakes might have been applied. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 11 / 12 | FS / OS / SR / UN / SL / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that an acknowledgement for a level transition announcement is displayed and acknowledged only once, although it passes over the beginning of the acknowledgement area twice, because it moves backwards and forwards. |  | 10 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, SL, NL L2: FS, LS, OS, SR, SL, NL L3: FS, LS, OS, SR, SL, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment displays the acknowledgement of the level transition only once, although it passes over the defined location twice. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The acknowledgement of the level transition is displayed only once Check on the JRU that : • The acknowledgement of the level transition is RECORDED only once |
| Constraints | There is a level transition announcement stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 11 / 12 | FS / OS / SR / UN / SL / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train passes over the beginning of the acknowledgement area for the level transition. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | An acknowledgement for level transition is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver acknowledges the level transition announcement. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver’s action is recorded. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the beginning of the acknowledgement area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 7 | N/A | N/A | The train moves backwards and overpasses the beginning of the acknowledgement area. | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | No acknowledgement for level transition is displayed. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | No acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 11 | N/A | N/A | The train moves forward. | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | The train passes over the beginning of the acknowledgement area for the level transition again. | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | No acknowledgement for level transition is displayed. | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | No acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 11 / 12 | FS / OS / SR / UN / SL / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that a plain or a fixed text message, which is ordered from the trackside for a specific location, starts to be displayed only once, although the train passes over this location again while moving backwards and then forwards. |  | 11 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, NL L2: FS, LS, OS, SR, NL L3: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment displays a plain or a fixed text message only once, although it passes over the defined location again. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The plain or fixed text message is displayed only once. Check on the JRU that : • The displayed message is recorded only once |
| Constraints | There is fixed or plain text information stored on-board, that will be removed when the timer defined by T_TEXTDISPLAY expires. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Plain Text Information (not location based) | STORED | The text message will be removed during the execution of the TC because the timer defined by T_TEXTDISPLAY expires. |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | When the train reaches the location defined by D_TEXTDISPLAY, the plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The displayed message is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by D_TEXTDISPLAY, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 4 | N/A | N/A | The timer defined by T_TEXTDISPLAY expires, and the plain or fixed text message is removed. | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The removal of the message is recorded. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the location defined by D_TEXTDISPLAY | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | No plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | No plain or fixed text message is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 10 | N/A | N/A | The train moves forward. | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | The train passes over the location defined by D_TEXTDISPLAY again. | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | No plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | No plain or fixed text message is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that a plain or a fixed text message, which is ordered from the trackside for a specific location, is not reverted. The on-board will stop displaying a plain or fixed text message once, although the train passes over the location to stop displaying the text message again while moving backwards and then forwards. |  | 12 |
| Applicable Mode/Level Combinations | L0: UN L1: FS, LS, OS, SR, NL L2: FS, LS, OS, SR, NL L3: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment will stop displaying a plain or fixed text message once, although it passes over the location where the text message should be removed again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The plain or fixed text message is removed after passing the location defined by L_TEXTDISPLAY. Check on the JRU that : • The removal of the message is recorded only once |
| Constraints | There is fixed or plain text information stored on-board, that will be removed when the train overpasses the location defined by L_TEXTDISPLAY. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Plain Text Information (location based) | STORED | The text message will be removed during the execution of the TC because the train overpasses the location defined by L_TEXTDISPLAY |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | When the train reaches the location defined by D_TEXTDISPLAY, the plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The displayed message is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | When the train passes over the location defined by D_TEXTDISPLAY + L_TEXTDISPLAY, the text message is removed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The removal of the text message is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by D_TEXTDISPLAY + L_TEXTDISPLAY, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 6 | N/A | N/A | The train moves backwards and overpasses the location defined by D_TEXTDISPLAY + L_TEXTDISPLAY. | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | No plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | No plain or fixed text message is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 10 | N/A | N/A | The train moves forward. | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | The train passes over the location defined by D_TEXTDISPLAY + L_TEXTDISPLAY again. | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | No plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | No plain or fixed text message is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the request to acknowledge a mode profile, which is ordered from the trackside for a specific location, is displayed only once, although the train passes over this location again. |  | 13 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests an acknowledgement of mode transition, although it passes over the defined location again. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The request to acknowledge mode transition is displayed only once Check on the JRU that : • The displayed message is recorded only once |
| Constraints | There is some mode transition information stored on-board. If the current mode is OS mode, only a mode profile for changing to SH mode will be stored on-board. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | When the train reaches the location defined by D_MAMODE – L_ACKMAMODE, the on-board equipment requests an acknowledgement for changing to X mode. | O | DMI | X can be OS, SH or LS mode. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | The acknowledgement request is recorded. | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | The driver acknowledges the mode transition announcement. | I | DMI | - | L1 / L2 / L3 | FS / OS / LS | - |
| 4 | L1 / L2 / L3 | FS / OS / LS | The driver’s acknowledgement is recorded. | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 5 | L1 / L2 / L3 | FS / OS / LS | The system switches to X mode | I | INT | X can be OS, SH or LS mode. | L1 / L2 / L3 | OS / SH / LS | - |
| 6 | L1 / L2 / L3 | OS / SH / LS | Indication of X mode is DISPLAYED | O | DMI | X can be OS, SH or LS mode. | L1 / L2 / L3 | OS / SH / LS | - |
| 7 | L1 / L2 / L3 | OS / SH / LS | The transition to X mode is RECORDED | O | JRU | X can be OS, SH or LS mode. | L1 / L2 / L3 | OS / SH / LS | - |
| 8 | L1 / L2 / L3 | OS / SH / LS | Use_FT3060500.13 “The on-board equipment sends a position report after a mode change“ | I | INT | Inform trackside about mode change with existing communication session (if level 2/3) | L1 / L2 / L3 | OS / SH / LS | - |
| 9 | L1 / L2 / L3 | OS / SH / LS | The driver stops the train. | I | INT | The train will have to be stopped very close to the location defined by D_MAMODE – L_ACKMAMODE, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | L1 / L2 / L3 | OS / SH / LS | - |
| 10 | L1 / L2 / L3 | OS / SH / LS | The train moves backwards and overpasses the location defined by D_MAMODE – L_ACKMAMODE. | I | INT | - | L1 / L2 / L3 | OS / SH / LS | - |
| 11 | L1 / L2 / L3 | OS / SH / LS | No transition to the previous mode occurs. | O | DMI | - | L1 / L2 / L3 | OS / SH / LS | - |
| 12 | L1 / L2 / L3 | OS / SH / LS | No acknowledgement request of X mode is displayed. | O | DMI | X can be OS, SH or LS mode | L1 / L2 / L3 | OS / SH / LS | - |
| 13 | L1 / L2 / L3 | OS / SH / LS | No acknowledgement request of X mode is recorded. | O | JRU | X can be OS, SH or LS mode | L1 / L2 / L3 | OS / SH / LS | - |
| 14 | L1 / L2 / L3 | OS / SH / LS | The driver stops the train. | I | INT | The brakes might have been applied. | L1 / L2 / L3 | OS / SH / LS | - |
| 15 | L1 / L2 / L3 | OS / SH / LS | The train moves forward. | I | INT | - | L1 / L2 / L3 | OS / SH / LS | - |
| 16 | L1 / L2 / L3 | OS / SH / LS | The train passes over the location defined by D_MAMODE – L_ACKMAMODE again. | I | INT | - | L1 / L2 / L3 | OS / SH / LS | - |
| 17 | L1 / L2 / L3 | OS / SH / LS | No acknowledgement request of X mode is displayed. | O | DMI | X can be OS, SH or LS mode | L1 / L2 / L3 | OS / SH / LS | - |
| 18 | L1 / L2 / L3 | OS / SH / LS | No acknowledgement request of X mode is recorded. | O | JRU | X can be OS, SH or LS mode | L1 / L2 / L3 | OS / SH / LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 3 / 12 | OS / SH / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the acceptance of infill information will start only once, although the train passes over the beginning of the infill area more than once. |  | 14 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment starts accepting infill information only once, although it passes over the beginning of the infill area more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit only once, as soon as the location indicated in the order is passed. Check on the JRU that : • The information sent to the radio infill unit is recorded once at the beginning of the infill area |
| Constraints | There is some radio infill information stored on-board and there is already a communication session established with the radio infill unit. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio Infill Area information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Between the on-board and the radio infill unit |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board equipment will start accepting information from this location. The train reaches the location defined by D_INFILL (stored on-board). The on-board unit shall inform the radio infill unit using message 153. as soon as the train overpasses the beginning of the infill area (stored on-board). Message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 2 | N/A | N/A | Message 153 (Radio infill request) is recorded. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver stops the train. | I | INT | The train will have to be stopped very close to the location where Message 153 was sent (defined by D_INFILL), since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 4 | N/A | N/A | The train moves backwards and overpasses the location defined by D_INFILL. | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | No message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 6 | N/A | N/A | No message 153 is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver stops the train. | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 8 | N/A | N/A | The train moves forward, and it passes over the location defined by D_INFILL again. | I | INT | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | No message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 10 | N/A | N/A | No message 153 is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Start or end of infill |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | FINITE VALUE | Start: end of inStart: end of inStart: end of infillfillfill |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Between the on-board and the radio infill unit |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall stop accepting infill information only once, although the train passes over the end of the infill area more than once. |  | 15 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops accepting infill information only once, although it passes over the end of the infill area more than once. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit only once, when it detects that the main BG is missed. Check on the JRU that : • The information sent to the radio infill unit is recorded only once at the end of the infill area |
| Constraints | There is some radio infill information stored on-board, with the linking reaction for the main BG set to “No reaction”. There is already a communication session established with the radio infill unit. Besides, the value of D_NVROLL will have to be the highest one. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Packet 5 with Q_LINKREACTION = “No reaction” |
| Radio Infill Area information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Between the on-board and the radio infill unit |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train passes over the last possible location of the main balise group with the min safe front end, and it detects nothing. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as it detects that the main BG has been missed. Message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The train is at standstill | I | INT | The train will have to be stopped very close to the location where Message 153 was sent (after the SB is applied), since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving | I | INT | The train moves backwards and overpasses the last possible location of the main BG with its min safe front end again. | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | No message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 7 | N/A | N/A | NOT MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The train is at standstill | I | INT | The brakes might have been applied. | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving | I | INT | The train moves forwards and overpasses the last possible location of the main BG with its min safe front end again. | N/A | N/A | - |
| 10 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | No message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 11 | N/A | N/A | NOT MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Start or end of infill |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | FINITE VALUE | Start: end of inStart: end of inStart: end of infillfillfill |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Between the on-board and the radio infill unit |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the request to acknowledge new level transition, which is ordered from the trackside for a specific location, is displayed only once, although the train passes over this location twice. |  | 16 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | he target of this test is to check that: • The on-board equipment requests an acknowledgement of level transition only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The request to acknowledge the level transition is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included, with a high enough value for Q_LOCACC. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 5 (D_LINK (B) = 1000), packet 41 ( D_LEVELTR = 1500, M_LEVLETR = different from the current level, L_ACKLEVELTR = 495) is received | I | BTM | D=d0 The train passes the Balise Group A containing: · Packet 5 linking i. D_LINK (B) = 1000 m · Packet 41 level transition order: i. D_LEVELTR = 1500 ii. M_LEVLETR = different from the current level iii. L_ACKLEVELTR = 495 Q_LOACC of BG A (which is the LRBG from now on) and BG B must be high enough (20m, for example). The value of Q_LOACC of BG A is known from previous linking or from previous National/Default Value. No Packet 5 will be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) with Packet 5 containing: i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio Message 24 with Packet 5 inside is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D= d0 + 1005m An acknowledgement for level transition is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges the level transition announcement. | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver’s action is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1020m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the acknowledgement has to be shown (D=d0+1005). | N/A | N/A | - |
| 10 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | D= d0 + 1025m No acknowledgement for level transition is displayed. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1005, but no request is made. | N/A | N/A | - |
| 12 | N/A | N/A | No acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 1500 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 20 | - |
| N_ITER | 5 | 0 | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall stop accepting infill information only once, although the train passes over the end of the infill area twice. |  | 17 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops accepting infill information only once, although it passes twice over the end of the infill area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit only once, when it detects that the main BG is missed. Check on the JRU that : • The information sent to the radio infill unit is recorded only once at the end of the infill area • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000 m, Q_LINKREACTION (B) = 10, Q_LOCACC (B) = 60 m, D_LINK (C) = 50 m), packet 133 (D_INFILL = 500) is received | I | BTM | D = d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LINKREACTION (B) = 10 (No reaction) iii. Q_LOCACC (B) = 60 m iv. D_LINK (C) = 50 m • Packet 133 radio infill area information Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. The BG B is the main BG. | N/A | N/A | - |
| 2 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3090300.1 “Starting event is received from the balise.“ | - | - | The on-board establishes a radio communication session with the radio infill unit. | N/A | N/A | - |
| 4 | N/A | N/A | D = d0 + 1120 m The on-board misses the main BG (B). | I | INT | At this location the train passes over the last possible location of the balise group with the min safe front end (for calculating the confidence interval, only Q_LOCACC has been taken into account, not the odometer error, so this might occur a bit further away). | N/A | N/A | - |
| 5 | N/A | N/A | No reaction is applied. There is no brake intervention. | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as it detects the BG (B) is missed. Message 153 (Radio-infill request) is SENT | N/A | N/A | - |
| 7 | N/A | N/A | Message 153 (Radio infill request) is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1150 m Balise group C is received When receiving BGc inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1050m. So, the train will pass twice over the location where the radio infill area finishes (D=d0+1050 m). | N/A | N/A | - |
| 9 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | D = d0 + 1220 No message 153 (Radio-infill request) is SENT | O | DMI | From the point of view of the train, this location is actually D = d0 + 1120 m, but no message is sent. This is an indirect way to check that the on-board stopped accepting infill information from the radio infill unit only once (at d0 + 1120 m). | N/A | N/A | - |
| 11 | N/A | N/A | No message 153 is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 10 | - |
| Q_LOCACC | 6 | 60 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | 50 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | 500 | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Start or end of infill |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 1 | Exit |

| Step 8: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the acceptance of infill information will start once, although the train never passes over the beginning of the infill area. |  | 18 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment starts accepting infill information once, although it never passes over the beginning of the infill area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.9.3.10 a) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit once, as soon as it reads the next announced BG.. Check on the JRU that : • The information sent to the radio infill unit is recorded once at the beginning of the infill area • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Not Established | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 133 (D_INFILL = 990) is received | I | BTM | D=d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 133 radio infill area information: i. D_INFILL = 990 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | The Balise Group Message BGa is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 940 m Balise group B is received This BG must be different from the main BG, which will be further away. When receiving BG B inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the radio infill area starts (D=d0+990 m). | N/A | N/A | - |
| 4 | N/A | N/A | Balise Group Message BGb is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Connection request to the RTM Mandatory parameter: NID_C, NID_RIU, application type Optional parameter: network address (NID_RADIO), mobile network ID, quality of service. If network address is not provided to RTM or the value “FFFF FFFF FFFF FFFF” is provided to RTM, the on-board part of the RTM has to use the short number according to the EIRENE requirements. | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM If connection establishment was not successful, the connection request (step 5) shall be repeated until successful or a defined number of times (see Appendix A3.1). | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | Message 155: Initiation of a communication session is SENT | N/A | N/A | - |
| 8 | N/A | N/A | Message 155: Initiation of a communication session is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | Message 32: DeterminationRBC/RIU System Version is RECEIVED Communication session established for on-board | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | Use_FT4080406.TC1,2 ”Supervision of ETCS version compatibility for Radio Infill session” | - | - | System versions are compatible | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | Message 159: Session established is SENT | N/A | N/A | - |
| 13 | N/A | N/A | Message 159: Session established is RECORDED | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as the location (received during step 1) is passed. Message 153 (Radio-infill request) is SENT The on-board equipment will start accepting information from this location. | N/A | N/A | - |
| 15 | N/A | N/A | Message 153 (Radio infill request) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | 990 | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 12: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 14: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Start or end of infill |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 0 | Train is entering the radio infill area. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall stop accepting infill information once, although the train never passes over the end of the infill area. |  | 19 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops accepting infill information once, although it never passes over the end of the infill area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.9.3.10 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit once, when it detects that the main BG is missed. Check on the JRU that : • The information sent to the radio infill unit is recorded once at the end of the infill area • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000, Q_LINKREACTION (B) = 10, Q_LOCACC (B) = 60, D_LINK (C) = 170), packet 133 (Q_RIU = 1, D_INFILL = 500) is received | I | BTM | D = d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LINKREACTION (B) = 10 (No reaction) iii. Q_LOCACC (B) = 60 m iv. D_LINK (C) = 170 m • Packet 133 radio infill area information Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. The BG B is the main BG. | N/A | N/A | - |
| 2 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3090300.1 “Starting event is received from the balise.“ | - | - | The on-board establishes a radio communication session with the radio infill unit. | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1070 m Balise group C is received At this location, the max safe front end of the train is inside the expectation window of BG C, and the message will be accepted. So, the confidence interval is readjusted, and the estimated front end will be recalculated and changed to d0+1170m. So, the train will never pass over the location where the radio infill area finishes (D=d0+1120 m). | N/A | N/A | - |
| 5 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as it detects the BG (B) is missed. Message 153 (Radio infill request) is SENT | N/A | N/A | - |
| 7 | N/A | N/A | Message 153 (Radio infill request) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 10 | - |
| Q_LOCACC | 6 | 60 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | 170 | - |
| Q_NEWCOUNTRY(k) | 1 | 0 | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | 500 | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Start or end of infill |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 1 | Exit |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to RBC/RBC handover will be taken only once, although the train passes over a defined location twice. |  | 20 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes an action related to RBC/RBC only once, although it passes twice over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 f) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the Handing over RBC only once when it reaches the location for the RBC transition. Check on the JRU that : • The information sent to the Handing over RBC is recorded once. • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With the Handing over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 5 (D_LINK (B) = 1000), packet 131 (D_RBCTR = 1100) is received | I | RTM | A radio message 3 (Movement authority) containing an announcement of the RBC transition (packet 131) and linking information (packet 5) is received from the Handing Over RBC. • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 131: i. D_RBCTR = 1100 The linking information refers to an LRBG (BG A), which is located at D = d0 m. Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | The radio message 3 with packets 5 & 131 is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | A connect request to the Accepting RBC is sent A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | A connection indication from the Accepting RBC is received on-board | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | A radio message 155 (initiation of communication session) is sent to the Accepting RBC | N/A | N/A | - |
| 6 | N/A | N/A | The radio message 155 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | A radio message 32 (RBC/RIU System Version) is received from the Accepting RBC The Version of the ERTMS/ETCS language is valid for the train | N/A | N/A | - |
| 8 | N/A | N/A | The radio message 32 is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | A radio message 159 (session established) is sent to the Accepting RBC The communication session with the Accepting RBC is established | N/A | N/A | - |
| 10 | N/A | N/A | The radio message 159 is RECORDED | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | A radio message 129 (Validated train data) is sent to the Accepting RBC The radio message contains also position report data. | N/A | N/A | - |
| 12 | N/A | N/A | The radio message 129 is RECORDED | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | A radio message 8 from the Accepting RBC is received on-board. | N/A | N/A | - |
| 14 | N/A | N/A | The radio message 8 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | D = d0 + 1040 Train maximum safe front end location = LRBG + D_RBCTR A radio message 136 (train position report) is sent to the Handing Over RBC. The maximum safe front end of the train passes the location of the RBC transition. For calculating the confidence interval, only Q_LOCACC has been taken into account here, not the odometer error, since it will depend on the particular EVC, so this might occur a bit earlier. | N/A | N/A | - |
| 16 | N/A | N/A | The radio message 136 to the Handing Over RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC. The maximum safe front end of the train passes the location of the RBC transition. For calculating the confidence interval, only Q_LOCACC has been taken into account here, not the odometer error, since it will depend on the particular EVC, so this might occur a bit earlier. | N/A | N/A | - |
| 18 | N/A | N/A | The radio message 136 to the Accepting RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | SA-DATA.Request with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 42 is received | I | RTM | Handing over RBC sends disconnection order to the OBU | N/A | N/A | - |
| 20 | N/A | N/A | The radio message 24 to the Handing Over RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 + 1050 m Balise group B is received When receiving BG B inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the max safe front end of the train will pass twice over the location for the RBC transition (D=d0+1040 m). | N/A | N/A | - |
| 22 | N/A | N/A | The balise group message is recorded. | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | Use_FT4080425.3 “A L2/L3 MA is transmitted by radio and is received by the onboard equipment “ | - | - | Transmission of L2/L3 movement Authority from Handing over RBC. From the point of view of the train, this location is actually D = d0 + 1020. D = d0 + 1070 m The on.board is no more supervised by the handing over RBC. The On-board will not use handing over MA’s. | N/A | N/A | - |
| 24 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | DMI | Since the MA has been rejected, the MA sent by the Handing over RBC is not displayed. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 25 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Since the MA has been rejected, no new speed and distance information is recorded. | N/A | N/A | - |
| 26 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | D = d0 + 1090 m No position report is sent to the Handing over RBC. From the point of view of the train, this location is actually D = d0 + 1040. | N/A | N/A | - |
| 27 | N/A | N/A | D = d0 + 1090 m No position report to the Handing over RBC is recorded. | O | JRU | From the point of view of the train, this location is actually D = d0 + 1040. | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Acceptin. RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 15: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 17: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 19: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : _x005F_x000D_NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 21: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With both RBCs |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to RBC/RBC handover will be taken once, although the train never passes over a defined location. |  | 21 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes an action related to RBC/RBC once, although it never passes over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 f) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the Handing over RBC once, although it never passes over the location for the RBC transition Check on the JRU that : • The information sent to the Handing RBC is recorded once. • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With the Handing over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 5 (D_LINK (B) = 1000), packet 131 (D_RBCTR = 990) is received | I | RTM | A radio message 3 (Movement authority) containing an announcement of the RBC transition (packet 131) and linking information (packet 5) is received from the Handing Over RBC. • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 131: i. D_RBCTR = 990 m The linking information refers to an LRBG (BG A), which is located at D = d0 m. Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | The radio message 3 with packets 5 & 131 is RECORDED | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | The radio message 155 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train | N/A | N/A | - |
| 8 | N/A | N/A | The radio message 32 is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | The communication session with the Accepting RBC is established | N/A | N/A | - |
| 10 | N/A | N/A | The radio message 159 is RECORDED | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | The radio message contains also position report data. | N/A | N/A | - |
| 12 | N/A | N/A | The radio message 129 is RECORDED | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 14 | N/A | N/A | The radio message 8 is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 + 900 m When receiving BG B inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the max safe front end of the train will never pass over the location for the RBC transition (D=d0+990 m). Balise group B is received | N/A | N/A | - |
| 16 | N/A | N/A | The balise group message is recorded. | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Handing Over RBC. Although the max safe front end of the train never passes over the location of the RBC transition. | N/A | N/A | - |
| 18 | N/A | N/A | The radio message 136 to the Handing Over RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | A radio message 136 (train position report) is sent to the Accepting RBC. Although the max safe front end of the train never passes over the location of the RBC transition. | N/A | N/A | - |
| 20 | N/A | N/A | The radio message 136 to the Accepting RBC is RECORDED | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | Use_FT4080425.3,4,5,7 Transmission of L2/L3 movement Authority from Handing over RBC | - | - | From the point of view of the train, this location is actually D = d0 + 1000. The on.board is no more supervised by the handing over RBC. The On-board will not use handing over MA’s | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 990 | - |
| NID_C | 10 | FINITE VALUE | Acceptin. RBC identity. |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| Step 15: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 17: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | D_RBCTR - L_DOUBTUNDER | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 19: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | D_RBCTR - L_DOUBTUNDER | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | With both RBCs |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to track condition information, with the exception of big metal masses, will be taken only once, although the train passes over a defined location twice. |  | 22 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment performs an action only once, although it passes twice over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The action to be performed (low pantograph) is performed only once Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 68 (D_TRACKCOND = 1070, L_TRACKCOND = 100, M_TRACKCOND = 0011 ) is received | I | BTM | D=d0 The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 30 m • Packet 68 Track Conditions: i. D_TRACKCOND = 1070 ii. L_TRACKCOND = 100 iii. M_TRACKCOND = 0011 (lower pantograph) Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packets 5 and 68 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 68 (D_TRACKCOND = 1070, L_TRACKCOND = 100, M_TRACKCOND = 0011 ) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) containing: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 30 m • Packet 68 Track Conditions: i. D_TRACKCOND = 1070 ii. L_TRACKCOND = 100 iii. M_TRACKCOND = 0011 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | Na | - |
| 5 | N/A | N/A | D= d0 + 1010m When reaching this distance the pantograph output of the TIU shall be activated (lower pantograph) | O | TIU | The Max safe front end of the train will be at least 60 m in front of the estimated front end. | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1030m Balise group B is received When receiving the BG (B) inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the pantograph should be lowered (D=d0+1010). | N/A | N/A | - |
| 7 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | D = d0 + 1040 No new action is performed concerning the pantograph. | O | TIU | From the train point of view, this location is actually D = d0 + 1010.Here is were the pantograph should be lowered in a secure way, but it is already down (see upper steps). | N/A | N/A | - |
| 9 | N/A | N/A | D = d0 + 1200 m TIU signal for Pantograph is set to UP | O | TIU | The Min safe front end of the train will be at least 30 m in rear of the estimated front end (D = d0 + 1070 + 30 + 100 m). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 30 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | 1070 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 100 | - |
| M_TRACKCOND | 4 | 11 | - |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | Bga | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 30 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | 1070 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 100 | - |
| M_TRACKCOND | 4 | 11 | - |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if current level is Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that actions related to track condition information, with the exception of big metal masses, will be taken once, although the train never passes over a defined location. |  | 23 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment performs an action once, although it never passes over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.12.1.2.1 |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The action to be performed (low pantograph) is performed once Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 68 (D_TRACKCOND = 990, L_TRACKCOND = 100, M_TRACKCOND = 0011 ) is received | I | BTM | D=d0 The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 30 m • Packet 68 Track Conditions: i. D_TRACKCOND = 990 ii. L_TRACKCOND = 100 iii. M_TRACKCOND = 0011 (lower pantograph) Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packets 5 and 68 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 68 (D_TRACKCOND = 990, L_TRACKCOND = 100, M_TRACKCOND = 0011 ) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) containing: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 30 m • Packet 68 Track Conditions: i. D_TRACKCOND = 990 ii. L_TRACKCOND = 100 iii. M_TRACKCOND = 0011 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 900m Balise group B is received When receiving the second BG (B), with the max safe front end inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the pantograph should be lowered (D=d0+990). | N/A | N/A | - |
| 6 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The TIU is activated (lower pantograph) | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | Raise pantograph | O | TIU | The Min safe front end of the train will be at least 30 m in rear of the estimated front end (D = d0 + 990 + 30 + 100 m = d0 + 1120 m). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 30 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | 990 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 100 | - |
| M_TRACKCOND | 4 | 11 | - |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 30 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | 990 | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | 100 | - |
| M_TRACKCOND | 4 | 11 | - |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if current level is Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the permission to initiate Reversing mode is only displayed once, right before passing over the end of the Reversing area, but it is not shown again after passing again over this location (due to the train position adjustment). |  | 24 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment permits to initiate Reversing mode only once, although it passes twice over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 h) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The permission to initiate Reversing mode is displayed only once Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 138 (D_STARTREVERSE = 900, L_REVERSEAREA = 130), packet 139 () is received | I | BTM | D = d0 m The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 138 Reversing area information: i. D_STARTREVERSE = 900 ii. L_REVERSEAREA = 130 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packets 5 and 138 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 138 (D_STARTREVERSE = 900, L_REVERSEAREA = 130), packet 139 () is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) containing: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 138 Reversing area information: i. D_STARTREVERSE = 900 ii. L_REVERSEAREA = 130 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D = d0 + 1010 m The driver stops the train. | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver is informed that the reversing of movement is permitted. | O | DMI | The driver does not select RV mode | N/A | N/A | - |
| 7 | N/A | N/A | The information to the driver is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The train starts moving again. | I | INT | - | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 + 1040 m Balise group B is received When receiving the BG (B) inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass over the location where the reversing area finishes twice (D = d0 + 1030 m) | N/A | N/A | - |
| 10 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | D = d0 + 1060 m The driver stops the train. | I | INT | From the train point of view, this location is actually D = d0 + 1020 m. | N/A | N/A | - |
| 12 | N/A | N/A | NOT The Status symbol "Reversing permitted" is displayed | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | Direction controller backward | I | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | NOT The Mode symbol "Acknowledge Reversing" is displayed | O | DMI | DMI symbol: MO15 | N/A | N/A | - |
| 15 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit30=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | Direction controller forward | I | TIU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | 900 | - |
| L_REVERSEAREA | 15 | 130 | - |
| NID_PACKET | 8 | 139 | The Packet must be relevant for the train. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | 900 | - |
| L_REVERSEAREA | 15 | 130 | - |
| NID_PACKET | 8 | 139 | The Packet must be relevant for the train. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if current level is Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the permission to initiate Reversing mode is displayed once (at standstill), although the train never passes over the beginning of the reversing area. |  | 25 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment permits to initiate Reversing mode once, although it never passes the beginning of the reversing area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-5.13.1.3 |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 h) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The permission to initiate Reversing mode is displayed once Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the current level is Level2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 138 (D_STARTREVERSE = 990 L_REVERSEAREA = 1000), packet 139 () is received | I | BTM | D = d0 m The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 138 Reversing area information: i. D_STARTREVERSE = 990 ii. L_REVERSEAREA = 100 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packets 5 and 138 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30), packet 138 (D_STARTREVERSE = 990 L_REVERSEAREA = 1000), packet 139 () is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) containing: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 138 Reversing area information: i. D_STARTREVERSE = 990 ii. L_REVERSEAREA = 100 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 + 930 m Balise group B is received When receiving the BG (B), with the max safe front end inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the reversing area finishes (D=d0+990). | N/A | N/A | - |
| 5 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | D = d0 + 950 m The driver stops the train. | I | INT | From the train point of view, this location is actually D = d0 + 1020 m. | N/A | N/A | - |
| 7 | N/A | N/A | The driver is informed that the reversing of movement is permitted. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The information to the driver is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | 990 | - |
| L_REVERSEAREA | 15 | 100 | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | 990 | - |
| L_REVERSEAREA | 15 | 100 | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if current level is Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the information about track ahead free request is displayed only once, although the train passes over the start location of the area twice. |  | 26 |
| Applicable Mode/Level Combinations | L2: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests Track Ahead free only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 i) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The Track Ahead free request is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 m The train passes the Balise Group A Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 30) is received | I | RTM | The on-board receives Message 24 (referring to LRBG = BG A) containing at least: • Packet 5 linking i. D_LINK (B) = 1000 m | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The on-board receives Message 34 (Track Ahead Free Request): • D_TAFDISPLAY = 1010 | N/A | N/A | - |
| 6 | N/A | N/A | The received message 34 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | D = d0 + 1010 m The ‘Track Ahead Free Request’ is displayed to the driver | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Display of ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver acknowledges the ‘Track Ahead Free Request’ | I | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledgement is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted | O | RTM | The train sends the message 149 ‘Track Ahead Free Granted’ | N/A | N/A | - |
| 12 | N/A | N/A | The message 149 is RECORDED | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The ‘Track Ahead Free Request’ is cancelled | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Removal ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1060 m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the acknowledgement has to be shown (D=d0+1010 m). | N/A | N/A | - |
| 16 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | D= d0 + 1070 m No acknowledgement for the mode transition is displayed. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1010, but no request is made. | N/A | N/A | - |
| 18 | N/A | N/A | No acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 5: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 11: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 15: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the request to acknowledge new level transition, which is ordered from the trackside for a specific location, is not reverted. This means that the acknowledgement message is displayed once although the train never passes over the announced location. |  | 27 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests an acknowledgement of level transition once, although it never passes over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-5.10.4.1 a) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The request to acknowledge new level transition is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included with a high enough value for Q_LOCACC |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only for Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 41 (D_LEVELTR = 1500, V_RELEASEDP=3, M_LEVLETR = different from the current level, L_ACKLEVELTR = 510) is received | I | BTM | D=d0 The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 41 level transition order: i. D_LEVELTR = 1500 ii. M_LEVLETR = different from the current level iii. L_ACKLEVELTR = 510 Q_LOACC of BG A (which is the LRBG from now on) and BG B must be high enough (20m, for example). The value of Q_LOACC of BG A is known from previous linking or from previous National/Default Value. No Packet 5 will be included here if the current level is Level 2 | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000, Q_LOCACC = 20) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) with Packet 5 containing: i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio Message 24 with Packet 5 inside is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 980m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the acknowledgement has to be shown (D = d0+990m). | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | An acknowledgement for level transition is displayed | O | DMI | From the point of view of the train, this location is actually D = d0 + 1000, so the request is made here (although the location D=d0+990m has been passed for the train). | N/A | N/A | - |
| 8 | N/A | N/A | The acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver acknowledges the level transition announcement. | I | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver’s action is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 1500 | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | 510 | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 20 | - |
| N_ITER | 5 | 0 | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only for Level 2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the information about track ahead free request is removed once, although the train never passes over location where the information has to be removed. |  | 28 |
| Applicable Mode/Level Combinations | L2: OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment stops requesting the Track Ahead free, although it never passes over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.15.5.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 i) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The Track Ahead free request is removed once Check on the JRU that : • The removal of the message is recorded once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included with a high enough value for Q_LOCACC |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () is received | I | BTM | D = d0 m The train passes the Balise Group A Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives Message 24 (referring to LRBG = BG A) containing at least: • Packet 5 linking i. D_LINK (B) = 1000 m | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | The on-board receives Message 34 (Track Ahead Free Request): • D_TAFDISPLAY = 900 • L_TAFDISPLAY = 90 | N/A | N/A | - |
| 6 | N/A | N/A | The received message 34 is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | D = d0 + 900 m The ‘Track Ahead Free Request’ is displayed to the driver | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Display of ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The driver does not acknowledges the ‘Track Ahead Free Request’ | I | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 950 m Balise group B is received When receiving the BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the acknowledgement has to be removed (D=d0+990 m). | N/A | N/A | - |
| 11 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The ‘Track Ahead Free Request’ is removed. | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | Removal ‘Track Ahead Free Request’ is RECORDED | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 5: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | 1010 | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 10: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the location to start calculating the geographical position is taken into account only once, although the train passes over the location twice. |  | 29 |
| Applicable Mode/Level Combinations | L0: UN, NL L1: FS, LS, OS, SR, NL L2: FS, LS, OS, SR, NL |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes into account the starting location of the geographical position only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 j) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The geographical position starts to be displayed, on request from the driver, only once Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 79 (NID_BG = BG A, D_POSOFF = 1010) is received | I | BTM | D = d0 m The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 79 Track Conditions: i. NID_BG = BG A ii. D_POSOFF = 1010 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packet 5 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BG A) containing: • Packet 5 linking i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D = d0 + 1010 The driver requests the DISPLAY of the Geographical Position | I | DMI | After the train front end has passed over the location specified by D_POSOFF value. | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests of The Geographical Position DISPLAY is recorded in the JRU | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Geographical position is DISPLAYED according to the new M_POSITION | O | DMI | Route kilometre (variable M_POSITION) is based on the received packet 79. | N/A | N/A | - |
| 8 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1060 m Balise group B is received When receiving the BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location defined by D_POSOFF (D=d0+1010 m). | N/A | N/A | - |
| 9 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | D= d0 + 1070 m The geographical position continues to be displayed as normal. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1010. No change in the geographical position is made different from the ongoing calculation. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | Bga | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | 1010 | - |
| Q_MPOSITION | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | 0 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 8: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 | FS / OS / SR / UN / NL / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the location to stop calculating the geographical position is taken into account once, although the train never passes over the location. |  | 30 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment takes into account the location to stop calculating the geographical position once, although it never passes over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.6.6.9 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 j) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The geographical position stops to be displayed. Check on the JRU that : • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Some geographical position information has been received and it is being calculated on-board. Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 79 (NID_BG = BG A, D_POSOFF = 990, M_POSITION = 16’777’215) is received | I | BTM | D = d0 m The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 79 Track Conditions: i. NID_BG = BG A ii. D_POSOFF = 990 iii. M_POSITION = 16’777’215 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packet 5 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BG A) containing: • Packet 5 linking i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D = d0 + 900 The driver requests the DISPLAY of the Geographical Position | I | DMI | This is to check that the geographical location is still being calculated on-board. | N/A | N/A | - |
| 6 | N/A | N/A | The driver requests of The Geographical Position DISPLAY is recorded in the JRU | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Geographical position is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 940 m Balise group B is received When receiving the BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location defined by D_POSOFF (D=d0+990 m). | N/A | N/A | - |
| 9 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | No Geographical position is DISPLAYED | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 79 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | Bga | Geographical Position Reference Balise Group. |
| D_POSOFF | 15 | 990 | - |
| Q_MPOSITION | 1 | 16’777’215 | Geographical Position counting direction. |
| M_POSITION | 24 | FINITE VALUE | Track kilometre reference value. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | Geographical Position Reference Balise Group. |
| D_POSOFF(k) | 15 | FINITE VALUE | - |
| Q_MPOSITION(k) | 1 | FINITE VALUE | Geographical Position counting direction. |
| M_POSITION(k) | 24 | FINITE VALUE | Track kilometre reference value. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | Bga | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 8: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | Bgb | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that a plain or a fixed text message, which is ordered from the trackside for a specific location, is displayed only once, although the train passes over this location twice. |  | 31 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment displays a plain or a fixed text message only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The plain or fixed text message is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 72 (D_TEXTDISPLAY = 1005, L_TEXTDISPLAY = 40) is received | I | BTM | D=d0 The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 72 or 76 (plain or fixed text message(: i. D_TEXTDISPLAY = 1005 ii. L_TEXTDISPLAY = 40 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. No Packet 5 will be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) with Packet 5 containing: i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D= d0 + 1005m The plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The displayed message is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | D = d0 + 1045 The plain or fixed text message is removed. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The removal of the message is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1060m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the plain or fixed text message has to be shown (D=d0+1005). | N/A | N/A | - |
| 10 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | D= d0 + 1065m No plain or fixed text message is displayed. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1005, but no message is displayed. | N/A | N/A | - |
| 12 | N/A | N/A | No plain or fixed text message is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | 1005 | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | 40 | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that a plain or a fixed text message, which is ordered from the trackside for a specific location, is not reverted. The on-board will stop displaying a plain or fixed text message, although the train never passes over the location where the text message should be removed. |  | 32 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment will stop displaying a plain or fixed text message, although it never passes over the location where the text message should be removed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.12.3.4.6 |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The plain or fixed text message is removed after reading the BG message Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 72 (D_TEXTDISPLAY = 900, L_TEXTDISPLAY = 90) is received | I | BTM | D=d0 The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 72 or 76 (plain or fixed text message(: i. D_TEXTDISPLAY = 900 ii. L_TEXTDISPLAY = 90 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. No Packet 5 will be included here if the current level is Level 2 | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (B) = 1000) is received | I | RTM | The on-board receives one Message 24 (referring to LRBG = BGa) with Packet 5 containing: i. D_LINK (B) = 1000 m Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D= d0 + 900m The plain or fixed text message is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The displayed message is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 950m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the text message has to be removed from the DMI (D = d0+990m). | N/A | N/A | - |
| 8 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The plain or fixed text message is removed. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1000. | N/A | N/A | - |
| 10 | N/A | N/A | The removal of the message is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | 900 | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | 90 | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | Bga | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the request to acknowledge a mode profile, which is ordered from the trackside for a specific location, is displayed only once, although the train passes over this location twice. |  | 33 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests an acknowledgement of OS, LS or SH mode only once, although it passes twice over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The request to acknowledge OS, LS or SH mode is displayed only once Check on the JRU that : • The displayed message is recorded only once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 12, packet 5 (D_LINK (B) = 1000), packet 80 (D_MAMODE = 1500, M_MAMODE = 00 OR 01 OR 10, L_ACKMAMODE = 490) is received | I | BTM | D=d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 80 level transition order: i. D_MAMODE = 1500 ii. M_MAMODE = 00 (OS) OR 01 (SH) OR 10 (LS) iii. L_ACKMAMODE = 490 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packet 12 will also be sent. Packet 5 and Packet 80 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement authority" (NID_MESSAGE=3) containing packet 5 (D_LINK (B) = 1000), packet 80 (D_MAMODE = 1500, M_MAMODE = 00 OR 01 OR 10, L_ACKMAMODE = 490) is received | I | RTM | The on-board receives one Message 3 (referring to LRBG = BGa) containing at least: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 80 level transition order: i. D_MAMODE = 1500 ii. M_MAMODE = 00 (OS) OR 01 (SH) OR 10 (LS) iii. L_ACKMAMODE = 490 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | D = d0 + 1010 An acknowledgement for OS, LS or SH mode is displayed | O | DMI | This event occurs here because the beginning of the acknowledgement area for OS, LS or SH is supervised with the estimated front end. | N/A | N/A | - |
| 6 | N/A | N/A | The acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges the mode transition announcement. | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver’s acknowledgement is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | FS / OS / LS | The system switches to OS, LS or SH mode | I | INT | - | N/A | OS / LS / SH | - |
| 10 | N/A | N/A | Use_FT4070201.23 “On Sight mode is permanently displayed to the driver when the on-board equipment is in On Sight mode “ OR Use_FT4070201.7 “Limited Supervision mode is permanently displayed to the driver when the on-board equipment is in Limited Supervision mode“ OR Use_FT4070201.20 “When the onboard is in Shunting mode a permanent indication of Shunting is displayed.“ | O | DMI | Indication of OS, LS or SH mode is DISPLAYED | N/A | N/A | - |
| 11 | N/A | N/A | The transition to OS, LS or SH mode is RECORDED | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Use_FT3060500.13 “The on-board equipment sends a position report after a mode change“ | I | INT | Inform trackside about mode change with existing communication session (if level 2/3) | N/A | N/A | - |
| 13 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 1050 m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the acknowledgement has to be shown (D=d0+1010 m). | N/A | N/A | - |
| 14 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | D= d0 + 1060 m No acknowledgement for the mode transition is displayed. | O | DMI | From the point of view of the train, this location is actually D = d0 + 1010, but no request is made. | N/A | N/A | - |
| 16 | N/A | N/A | No acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 1500 | - |
| M_MAMODE | 2 | 01 OR 03 OR 12 | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | 490 | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | 01 OR 03 OR 12 | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 60 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 1500 | - |
| M_MAMODE | 2 | 0 | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | 490 | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 13: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 3 / 12 | OS / SH / LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the request to acknowledge a mode profile, which is ordered from the trackside for a specific location, is not reverted. The on-board equipment will request to acknowledge a mode profile once, although the train never passes over the defined location. |  | 34 |
| Applicable Mode/Level Combinations | L1: FS, LS L2: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment requests an acknowledgement of OS, LS or SH mode only once, although it never passes over the defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.9.3.2 a) |  |
|  |  | Subset-026-5.19.3.2 a) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The request to acknowledge OS, LS or SH mode is displayed once Check on the JRU that : • The displayed message is recorded once • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 12, packet 5 (D_LINK (B) = 1000), packet 80 (D_MAMODE = 1500, M_MAMODE = 00 OR 01 OR 10, L_ACKMAMODE = 510) is received | I | BTM | D=d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 80 level transition order: i. D_MAMODE = 1500 ii. M_MAMODE = 00 (OS) OR 01 (SH) OR 10 (LS) iii. L_ACKMAMODE = 510 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. Packet 12 will also be sent. Packet 5 and Packet 80 will not be included here if the current level is Level 2. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement authority" (NID_MESSAGE=3) containing packet 5 (D_LINK (B) = 1000), packet 80 (D_MAMODE = 1500, M_MAMODE = 00 OR 01 OR 10, L_ACKMAMODE = 510) is received | I | RTM | The on-board receives one Message 3 (referring to LRBG = BGa) containing at least: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 80 level transition order: i. D_MAMODE = 1500 ii. M_MAMODE = 00 (OS) OR 01 (SH) OR 10 (LS) iii. L_ACKMAMODE = 510 Only if the current level is Level 2. | N/A | N/A | - |
| 4 | N/A | N/A | The radio message is recorded. | O | JRU | Only if the current level is Level 2. | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message () is received | I | BTM | D= d0 + 950 m Balise group B is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will never pass over the location where the mode profile has to be acknowledged (D = d0+990m). | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | An acknowledgement request for OS, LS or SH mode is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The acknowledgement request is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 1500 | - |
| M_MAMODE | 2 | 01 OR 03 OR 12 | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | 510 | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | 01 OR 03 OR 12 | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 1500 | - |
| M_MAMODE | 2 | 0 | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | 510 | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 | L1 / L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The on-board equipment shall ensure that the acceptance of infill information will start only once, although the train passes over the beginning of the infill area twice. |  | 35 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment starts accepting infill information only once, although it passes twice over the beginning of the infill area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 e) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The on-board informs the radio infill unit only once, as soon as the location indicated in the order is passed. Check on the JRU that : • The information sent to the radio infill unit is recorded once at the beginning of the infill area • Balise Group BGa Message is RECORDED. • Balise group BGb Message is RECORDED |
| Constraints | Q_LOCACC of BG A (which is the LRBG given at the beginning of the TC) must be high enough. This value is known from previous linking or from previous National/Default Value. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | If Linking information is stored, Balise Group BGa shall be included. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message () containing packet 5 (D_LINK (B) = 1000), packet 133 (D_INFILL = 1010) is received | I | BTM | D=d0 m The train passes the Balise Group Bga: • Packet 5 linking i. D_LINK (B) = 1000 m • Packet 133 radio infill area information: i. D_INFILL = 1010 Q_LOCACC of BG A (which is the LRBG from now on) and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | N/A | N/A | - |
| 2 | N/A | N/A | The Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | D = d0 + D_INFILL Connection request to the RTM Mandatory parameter: NID_C, NID_RIU, application type Optional parameter: network address (NID_RADIO), mobile network ID, quality of service. If network address is not provided to RTM or the value “FFFF FFFF FFFF FFFF” is provided to RTM, the on-board part of the RTM has to use the short number according to the EIRENE requirements. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM If connection establishment was not successful, the connection request (step 2) shall be repeated until successful or a defined number of times (see Appendix A3.1). | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | Message 155: Initiation of a communication session is SENT | N/A | N/A | - |
| 6 | N/A | N/A | Message 155: Initiation of a communication session is RECORDED | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | Message 32: DeterminationRBC/RIU System Version is RECEIVED | N/A | N/A | - |
| 8 | N/A | N/A | Message 32: DeterminationRBC/RIU System Version is RECORDED | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | Message 159: Session established is SENT | N/A | N/A | - |
| 10 | N/A | N/A | Message 159: Session established is RECORDED | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as the location (received during step 1) is passed. Message 153 (Radio infill request) is SENT The on-board equipment will start accepting information from this location. | N/A | N/A | - |
| 12 | N/A | N/A | Message 153 (Radio infill request) is recorded. | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | One Balise Group message is received | I | BTM | D= d0 + 1050 m Balise group B is received This BG must be different from the main BG, which will be further away. When receiving BGb inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the radio infill area starts (D=d0+1010 m). | N/A | N/A | - |
| 14 | N/A | N/A | Balise Group Message is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) is transmitted | O | RTM | D = d0 + 1060 No message 153 (Radio-infill request) is SENT From the point of view of the train, this location is actually D = d0 + 1010, but no message is sent. This is an indirect way to check that the on-board is already accepting infill information from the radio infill unit. | N/A | N/A | - |
| 16 | N/A | N/A | No message 153 is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1000 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | 1010 | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identity of the country of the "target" main balise group |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 0 | Start: end of inStart: end of inStart: end of infillfillfill |

| Step 13: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The train receives a track condition Big Metal Masses information. The on-board equipment shall ensure that actions related to track condition information, with the exception of big metal masses, will be taken only once, although the train passes over a defined location twice. |  | 36 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, NL |  |  |
| Target of Test | The target of this test is to check that: • The train receives a track condition Big Metal Masses information. • The on-board equipment performs the action track condition Big Metal Masses twice over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 f) |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.19 Packet Number 67: Track Conditions Big Metal Masses |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The train overpasses a BG with a Packet Number 67: ‘Track Condition Big Metal Masses’. • The Track Condition Big Metal Masses is updated and replaces the stored Track Condition Big Metal Masses, if one was stored On-board. |
| Constraints | For SB mode: - only if a cab is active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 11 / 12 / 15 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS |
| Movement Authority | STORED | If applicable for operated mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. Design test sequence as follows (variable D is the distance travelled by the train since test beginning) | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 2 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; M_DUP=0; ) containing packet 5, 67 and packet 255 is received | I | BTM | D=d0 The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 60 m • Packet 67 Track Conditions: i. D_TRACKCOND = 1070 ii. L_TRACKCOND = 50 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value. | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 3 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 4 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | ‘Track Condition Big Metal Masses’ is updated | I | INT | Internal D= d0 + 1010m When reaching this distance the Track Condition Big Metal Masses’ is updated The Max safe antenna position of the train will be at least 60 m in front of the estimated front end. | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 5 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | NOT BTM alarm due to big metal masses is received | I | INT | Somewhere between D=1010m and D=1070 an air-gap error occurs. A phantom signal (as may be caused by passing a metal bridge for example) shall be simulated by sending an incoherent signal over the air-gap. The On-board Transmission Equipment is allowed to give an alarm to the ERTMS/ETCS Kernel (Subset-036-6.2.1.7). Inside the big metal mass area the ERTMS/ETCS Kernel ignores the alarm. | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 6 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | One Balise Group message () is received | I | BTM | D= d0 + 1060m Balise group B is received When receiving the BG (B) inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the ‘Track Condition Big Metal Masses’ is updated (D=d0+1010). | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 7 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | The Balise Group Message is RECORDED. | O | JRU | - | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 8 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | ‘Track Condition Big Metal Masses’ is updated | I | INT | Internal From the train point of view, this location is actually D = d0 + 1010. When reaching this distance the Track Condition Big Metal Masses’ is updated The Max safe antenna position of the train will be at least 60 m in front of the estimated front end. | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |
| 9 | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | NOT BTM alarm due to big metal masses is received | I | INT | Somewhere between D=1010m and D=1070 an air-gap error occurs (From the train point of view ). A phantom signal (as may be caused by passing a metal bridge for example) shall be simulated by sending an incoherent signal over the air-gap. The On-board Transmission Equipment is allowed to give an alarm to the ERTMS/ETCS Kernel (Subset-036-6.2.1.7). Inside the big metal mass area the ERTMS/ETCS Kernel ignores the alarm. | L1 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 67 | Track Condition Big Metal Masses |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_TRACKCOND | 15 | 0 | - |
| L_TRACKCOND | 15 | 1070 | The distance for which integrity check alarms of balise transmission shall be ignored. Longer as the distance to the next Position where a newer Packet number 67 is received |
| N_ITER | 5 | 0 | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | The distance for which integrity check alarms of balise transmission shall be ignored. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 11 / 12 / 15 | FS / OS / SR / SH / SL / SB / TR / NL / LS / PS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The train receives a track condition information “non stopping area”. The on-board equipment shall ensure that actions related to track condition information, with the exception of “non stopping area”, will be taken only once, although the train passes over a defined location twice. |  | 37 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that: • The train receives a track condition “non stopping area” information. • The on-board equipment performs the action track condition “non stopping area” twice over a defined location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 g) |  |
|  |  | Subset-026-7.4.2.20 Packet Number 68: Track Condition |  |
|  |  | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The train overpasses a BG with a Packet Number 68: track condition information “non stopping area”. • The Track Condition “non stopping area” is updated and replaces the stored Track Condition “non stopping area”, if one was stored On-board. |
| Constraints | For SB mode: - only if a cab is active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Movement Authority | STORED | If applicable for operated mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB / PT / LS | Train is moving (V_TRAIN=40) | I | INT | The train is NOT at standstill. Design test sequence as follows (variable D is the distance travelled by the train since test beginning) | L1 | FS / OS / SR / SB / PT / LS | - |
| 2 | L1 | FS / OS / SR / SB / PT / LS | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>; ) containing packet 68 ( D_TRACKCOND>0; L_TRACKCOND=FINITE VALUE, M_TRACKCOND=0000), and packet 255 is received | I | BTM | 0000 = Non stopping area. Initial state: stopping permitted D=d0 The train passes over the Balise Group A: • Packet 5 linking i. D_LINK (B) = 1000 m ii. Q_LOCACC = 60 m • Packet 68 Track Conditions: i. D_TRACKCOND = 1070 ii. L_TRACKCOND = 50 Q_LOCACC of BG A and BG B must be high enough (60m, for example). Q_LOCACC of BG A is known from previous linking or National/Default Value | L1 | FS / OS / SR / SB / PT / LS | - |
| 3 | L1 | FS / OS / SR / SB / PT / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 4 | L1 | FS / OS / SR / SB / PT / LS | Train is moving (V_TRAIN=40) | I | INT | D= d0 + 1010m When reaching this distance the Track Condition begins The Max safe front end of the train will be at least 60 m in front of the estimated front end. | L1 | FS / OS / SR / SB / PT / LS | - |
| 5 | L1 | FS / OS / SR / SB / PT / LS | The Track Condition symbol "Non stopping area" is displayed | O | DMI | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 6 | L1 | FS / OS / SR / SB / PT / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit 53=1>) | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 7 | L1 | FS / OS / SR / SB / PT / LS | One Balise Group message () is received | I | BTM | D= d0 + 1060m Balise group B is received When receiving the BG (B) inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1000m. So, the train will pass twice over the location where the Track Condition is updated (D=d0+1010). | L1 | FS / OS / SR / SB / PT / LS | - |
| 8 | L1 | FS / OS / SR / SB / PT / LS | The Balise Group Message is RECORDED. | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 9 | L1 | FS / OS / SR / SB / PT / LS | The Track Condition symbol "Non stopping area" is removed | O | DMI | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 10 | L1 | FS / OS / SR / SB / PT / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit 53=0>) | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 11 | L1 | FS / OS / SR / SB / PT / LS | The Track Condition symbol "Non stopping area" is displayed | O | DMI | From the train point of view, this location is actually D = d0 + 1010. When reaching this distance the Track Condition is updated The Max safe front end of the train will be at least 60 m in front of the estimated front end. | L1 | FS / OS / SR / SB / PT / LS | - |
| 12 | L1 | FS / OS / SR / SB / PT / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit 53=1>) | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 13 | L1 | FS / OS / SR / SB / PT / LS | The Track Condition symbol "Non stopping area" is removed | O | DMI | - | L1 | FS / OS / SR / SB / PT / LS | - |
| 14 | L1 | FS / OS / SR / SB / PT / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit 53=0>) | O | JRU | - | L1 | FS / OS / SR / SB / PT / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | Track Condition |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | Longer than the distance to the next Position where a newer Packet number 68 is received |
| M_TRACKCOND | 4 | 0000 | 0000 = Non stopping area. Initial state: stopping permitted |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | 1070 | - |
| L_TRACKCOND(k) | 15 | 50 | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGa" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group Two balises |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | "BGb" |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The train passes a non protected Level Crossing that doesn’t require to stop in rear of the LX start location, includes the LX speed in the MRSP, rolls away backwards passing the location where the MRSP has been updated and doesn't update again the MRSP. |  | 38 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment doesn't revert nor execute twice the substitution of the supervision of the LX start location as EOA/SvL by the inclusion of the LX speed restriction in the MRSP (see 5.16.2 and 5.16.3) in case of reverse movement of the train. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 k) |  |
|  |  | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.2 |  |
|  |  | Subset-026-5.16.1.3 |  |
|  |  | Subset-026-5.16.3.1 |  |
|  |  | Subset-026-5.16.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - LX speed |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • No updated of the Permitted Speed occurs Check on the JRU that: • No update of MRSP occurs |
| Constraints | A packet 88 about a not protected LX has been received. This message doesn’t require to stop the train in rear of the non protected LX. If in LS, V_LX lower than V_NVLIMSUPERV (default value = 100 km/h) and V_MAXTRAIN and the LSSMA is displayed. The train moves with V_TRAIN < V_PERM. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | D_NVROLL = 327.660 km |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected Q_STOPLX=0 No stop required |
| LSSMA display toggle on order | STORED | Only in LS mode |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Only in FS and LS mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT5160000.6: Procedure passing a non protected Level Crossing | - | - | The train has already passed the location where the MRSP has been updated including the V_LX such as V_PERM=V_LX (FS and OS) or V_LSSMA=V_LX (LS) | N/A | N/A | - |
| 2 | N/A | N/A | Train is at standstill (V_TRAIN=0d) | I | INT | D_LRBG=d0 | N/A | N/A | - |
| 3 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; V_TRAIN=0d) is recorded | O | JRU | The stop of the train is RECORDED | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN>0d;Backwards) | I | INT | Train is rolling away backwards | N/A | N/A | - |
| 8 | N/A | N/A | Train position reaches the starting position of FT5160000.6 | I | INT | The train has overpassed the position where the V_LX has been taken into account in the MRSP. This step covers both the cases of SBI1 (where estimated front end is considered) and SBI2 (where Max safe front end is considered) (D_LRBG+L_DOUBTOVER<D_LRBG(Step4_FT5160000.6)) | N/A | N/A | - |
| 9 | N/A | N/A | NOT The speed and distance monitoring information is updated | O | DMI | The DMI doesn’t change the indication of the permitted speed (equal to V_LX) | N/A | N/A | - |
| 10 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No update of EoA and SvL and no changes in the MRSP. | N/A | N/A | - |
| 11 | N/A | N/A | Train is at standstill (V_TRAIN=0d) | I | INT | - | N/A | N/A | - |
| 12 | N/A | N/A | Train is moving (V_TRAIN>0d;Forward) | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | Train position reaches the location D_LRBG=d0 | I | INT | The train has overpassed the position where the V_LX has been taken into account in the MRSP. This step covers both the cases of SBI1 (where estimated front end is considered) and SBI2 (where Max safe front end is considered) | N/A | N/A | - |
| 14 | N/A | N/A | NOT The speed and distance monitoring information is updated | O | DMI | The DMI doesn’t change the indication of the permitted speed (equal to V_LX) | N/A | N/A | - |
| 15 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No update of EoA and SvL and no changes in the MRSP. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990500 |
| Test Case of Feature | The train receives Level Crossing Information, includes the LX speed in the MRSP, passes a BG that resets its location error. The correct location is before the point where the LX speed has been included and the train does not update again the MRSP. |  | 39 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment doesn't revert nor execute twice the substitution of the supervision of the LX start location as EOA/SvL by the inclusion of the LX speed restriction in the MRSP (see 5.16.2 and 5.16.3) in case of an adjustment of train position on passing a new LRBG |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-A.3.5.1 |  |
|  |  | Subset-026-A.3.5.1 k) |  |
|  |  | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.2 |  |
|  |  | Subset-026-5.16.1.3 |  |
|  |  | Subset-026-5.16.3.1 |  |
|  |  | Subset-026-5.16.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - LX speed |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The train includes the LX speed in the MRSP • The Permitted Speed does not change after the train location is corrected by receiving the information from a new LRBG (In OS the Permitted Speed is requested by the driver) Check on the JRU that : • The MRSP is updated only once (i.e. no update occurs after the train location is corrected by receiving the information from a new LRBG) |
| Constraints | The train moves with V_TRAIN < V_PERM. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Balise Group BGa shall be included in linking information |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| LSSMA display toggle on order | STORED | Only in LS mode |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | Both | RTM | Only if the level is L2 or L3 |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Only in FS and LS mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 5 (D_LINK (BGb)=1930), packet 88 (Q_LXSTATUS=1; Q_STOPLX=0; D_LX=2000) is recieved | I | BTM | Estimated train position=d0 The train passes the Balise Group BGa: • Packet 5 linking D_LINK (BGb) = 1930 m Q_LOCACC = 60 m • Packet 88 Level Crossing information i. Q_LXSTATUS=1 ii.Q_STOPLX=0 iii. D_LX=2000 iv. V_LX=40 No Packet 5 will be included here if the current level is Level 2 OR Level 3 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 (D_LINK (BGb) = 1930) is received | I | RTM | OPTIONAL STEP. Only in Level 2 OR Level 3 The on-board receives one Message 24 (referring to LRBG = BGa) with Packet 5 containing: D_LINK (BGb) = 1930 m Q_LOCACC = 60 m | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | OPTIONAL STEP. Only in Level 2 OR Level 3 | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode The train approaches the LX start location, the MRSP decreases as the ERTMS/ETCS on-board equipment supervises the LX start location as both the EOA and SvL with no release speed. If in FS mode, the Permitted Speed related to the current MA is displayed | N/A | N/A | - |
| 8 | N/A | N/A | The train reaches with the estimated or the max safe front end (depending whether the SBI at LX speed is the SBI1 or the SBI2) the location of the braking to target Permitted speed supervision limit calculated for the LX speed with V_TRAIN < V_PERM | I | INT | (see 3.13.9.3.5.11&12 for the calculation of this location) | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is updated | O | DMI | The Permitted Speed stops decreasing. V_PERM remains set to the value V_LX The ERTMS/ETCS on-board equipment shall no longer supervise the LX start location as both the EOA and SvL and shall immediately include the LX speed restriction in the MRSP | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = V_LX) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in FS and OS Mode | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_LSSMA = V_LX) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in LS Mode | N/A | N/A | - |
| 12 | N/A | N/A | One Balise Group message is received | I | BTM | Estimated train position= d0 + 1990 m Balise group BGb is received When receiving the second BG inside the expectation window, the confidence interval will be readjusted, and the estimated front end will be recalculated and changed to d0+1930 m. So, the curves SBI1 (where estimated front end is considered) and SBI2 (where Max safe front end is considered) presents a value greater than V_LX | N/A | N/A | - |
| 13 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | NOT The speed and distance monitoring information is updated | O | DMI | The DMI doesn’t change the indication of the permitted speed (equal to V_LX) | N/A | N/A | - |
| 15 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | No update of EoA and SvL and no changes in the MRSP. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1930 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 60 | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level crossing information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | 2000 | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | 8 | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 0 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | BGa | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | 1930 | - |
| Q_NEWCOUNTRY | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | FINITE VALUE | - |
| Q_LOCACC | 6 | 60 | - |
| N_ITER | 5 | 0 | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| Established | I/O | RTM | Only if the level is L2 or L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |