| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5080400 (5.8.4: End of Override procedure) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-5080400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, TCs renumbered | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC2 | Modifications requested by Review001_Subset076-5-2-5080400_v291 | MULTITEL |
| Version 2.9.3 20.11.2013 | All | Modifications requested by Review002_Subset076-5-2-5080400_v291 | MULTITEL |
| Version 2.9.4 06.12.2013 | TC1, TC2 | Modifications requested by issue 45 from Review005_Subset-076-5-2-5080400_v293 | MULTITEL |
| Version 2.9.5 09.01.2014 | TC1, TC2 | Modifications requested by issue 1 from Review001_Subset-076-5-2-5080400_v294 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects Override. When a new value for D_NVOVTRP is received on-board (by means of Packet 3, with NID_C equal to the currently being used) and it becomes applicable, the ongoing “Override” supervision will use this new value. However, the starting location shall remain unchanged. The Override procedure ends when train has run more than the "distance for train trip suppression when Override function is triggered" |  | 1 |
| Applicable Mode/Level Combinations | L0: UN L1: SR, SH L2: SR, SH L3: SR, SH LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The driver selects Override. The train is under the Override procedure and is being supervised against D_NVOVTRP. The train receives a newer set of National Values that shall be valid and applicable immediately. The value of D_NVOVTRP is updated and the train is supervised against a new value that has its origin in the location where Override was selected. The function Override (trip inhibition) is active The Override procedure ends when train has run more than the "distance for train trip suppression when Override function is triggered" |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.8.1 |  |
|  |  | Subset-026-3.18.2.11 |  |
|  |  | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-4.5.2.1 table - Override (Trip inhibition) note 4 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.4.1 |  |
|  |  | Subset-026-5.8.4.1 b) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects Override The Override Status is displayed before and after receiving the message (balise or radio) with Packet 3 The Override Status disappears after the distance given by D_NVOVTRP is covered Check on the JRU that : The driver selects Override Packet 3 National Values, Including D_NVOVTRP & correct value of NID_C is RECORDED Check on the DMI/JRU/TIU that: The trip inhibition ends together with the end of the Override status |
| Constraints | Override selection is allowed by the on-board equipment. Packet 3 will be sent to the on-board with D_NVOVTRP different from the value currently being used. End of Override procedure will not finish for any reason, but for the fact that the train runs more than the distance defined by the value of the last received D_NVOVTRP |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2/3/4/13 | SR/SH/UN/SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The driver selects "Override EOA" | I | DMI | The train speed is below max. speed for override (V_NVALLOWOVTRP) | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 2 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 3 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 4 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 5 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including D_NVOVTRP & a correct value for NID_C is RECEIVED. | I | BTM | Choose the BTM or the RTM step. D_NVOVTRP will be different from the one currently being used NID_C will be the same than the one stored on-board | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 6 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including D_NVOVTRP & a correct value for NID_C is RECEIVED. | I | RTM | Choose the BTM or the RTM step. D_NVOVTRP will be different from the one currently being used NID_C will be the same than the one stored on-board | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 7 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including D_NVOVTRP & a correct value for NID_C is RECORDED | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 8 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | As soon as d > d0 +D_NVOVTRP the “override status” is REMOVED | O | DMI | d is current the position of the train d0 is the position of the train when the “Override” button was triggered | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 9 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The cancellation of override status is RECORDED | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 10 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Override is not active and the train switches to TR | - | - | Use_FT4060300.28: The train switches from UN/SN to TR. Transition Id [39] OR Use_FT4060300.32: The switches from SH to TR. Condition Id [49] OR Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 000 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| T_TRAIN | 1 | FINITE VALUE | On-board train time |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | D_NVOVTRP will be different from the one currently being used |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| Override status is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects Override. When a new value for T_NVOVTRP is received on-board (by means of Packet 3) and it becomes applicable, the ongoing “Override” supervision will use this new value. However, the starting time shall remain unchanged. The Override procedure ends when the "max. time for train trip suppression when Override function is triggered" elapses |  | 2 |
| Applicable Mode/Level Combinations | L0: UN L1: SR, SH L2: SR, SH L3: SR, SH LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The driver selects Override. The train is under the Override EOA procedure and is being supervised against T_NVOVTRP (an already stored national value or the default value). The train receives a newer set of National Values that shall be valid and applicable immediately. The new value of T_NVOVTRP received is updated and the train is supervised against a new value that has its origin at the time when Override was selected. The function Override (trip inhibition) is active The Override procedure ends when the "max. time for train trip suppression when Override function is triggered" elapses |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.2.2 |  |
|  |  | Subset-026-3.18.2.3 |  |
|  |  | Subset-026-3.18.2.4 |  |
|  |  | Subset-026-3.18.2.8.1 |  |
|  |  | Subset-026-3.18.2.11 |  |
|  |  | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-4.5.2.1 table - Override (Trip inhibition) note 4 |  |
|  |  | Subset-026-4.8.3.1.1 National Values |  |
|  |  | Subset-026-4.8.4.2 National Values |  |
|  |  | Subset-026-5.8.4.1 |  |
|  |  | Subset-026-5.8.4.1 a) |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects Override The Override Status is displayed before and after receiving the message (balise or radio) with Packet 3 The Override Status disappears after the period of time given by T_NVOVTRP is passed Check on the JRU that : The driver selects Override Packet 3 National Values, Including T_NVOVTRP & correct value of NID_C is RECORDED Check on the DMI/JRU/TIU that: The trip inhibition ends together with the end of the Override status |
| Constraints | Override selection is allowed by the on-board equipment. Packet 3 will be sent to the on-board with T_NVOVTRP different from the value currently being used (an already stored national value or the default value). End of Override procedure will not finish for any reason, but for the fact that the time defined by the value of the last received T_NVOVTRP expires |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2/3/4/13 | SR/SH/UN/ SN |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The driver selects "Override EOA" | I | DMI | The train speed is below max. speed for override (V_NVALLOWOVTRP) | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 2 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 3 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 4 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 5 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including T_NVOVTRP & a correct value for NID_C is RECEIVED. | I | BTM | Choose the BTM or the RTM step. T_NVOVTRP will be different from the one currently being used | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 6 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including T_NVOVTRP & a correct value for NID_C is RECEIVED. | I | RTM | Choose the BTM or the RTM step. T_NVOVTRP will be different from the one currently being used | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 7 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Packet 3: National Values, Including T_NVOVTRP & a correct value for NID_C is RECORDED | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 8 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | As soon as t > t0 +T_NVOVTRP the “override status” is REMOVED | O | DMI | The received set of NV are applicable. t is the current time t0 is the time when the “Override EOA” button was triggered | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 9 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | The cancellation of override status is RECORDED | O | JRU | - | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | - |
| 10 | L0/LNTC/L1/L2/L3 | SR/SH/UN/SN | Override is not active and the train switches to TR | - | - | Use_FT4060300.28: The train switches from UN/SN to TR. Transition Id [39] OR Use_FT4060300.32: The switches from SH to TR. Condition Id [49] OR Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 000 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | T_NVOVTRP will be different from the one currently being used |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | FINITE VALUE | Type of media |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | Must correspond to the computed record size(on-board) |
| T_TRAIN | 32 | FINITE VALUE | On-board train time |
| M_ACK | 1 | FINITE VALUE | Acknowledge of message required = 1, Not required =0 |
| T_TRAIN | 1 | FINITE VALUE | On-board train time |
| NID_LRBG | 10+14 | FINITE VALUE | ID number of LRBG |
| NID_PACKET | 8 | 3 | Packet 3 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | NID_C will be the same than the one stored on-board |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies. |
| V_NVSHUNT | 7 | FINITE VALUE | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | T_NVOVTRP will be different from the one currently being used |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow. |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT(n) | 7 | FINITE VALUE | 0 km/h |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1. If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1. Valid between V_NVKVINT and V_NVKVINT(1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23. |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT | 7 | FINITE VALUE | - |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1). If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP1. |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1.Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP2. |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | 0 |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k. |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = . Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k). |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1.Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1). Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k). |
| L_NVKRINT | 5 | FINITE VALUE | 0 m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1). |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1). |
| M_NVKTINT | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | Only necessary if Packet 3 : National Values is received via radio |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only necessary if Packet 3 : National Values is received via radio. |
| NOT RELEVANT | - | TIU | - |
| Override status is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system switches to SR mode and override status is displayed. The Override procedure ends when in level 2,3, an MA is received from the RBC. |  | 3 |
| Applicable Mode/Level Combinations | L2/L3: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that: The function Override (trip inhibition) is active The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The override status indication is removed when the train receives a new MA over the radio interface The former EOA/LOA that was in use before the Override activation is deleted when SR mode is left |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.3.1.3 |  |
|  |  | Subset-026-5.8.3.1.3 b) |  |
|  |  | Subset-026-5.8.4.1 f) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the override status is removed when in level 2,3, an MA is received from the RBC. Check on the DMI that the former EOA/LOA that was in use before the Override activation is deleted when SR mode is left. |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the train receives the new MA . (While the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed, the train does not pass EOA/LOA given in previous modes, the train does not pass a balise group with "stop if in SR mode" information, the train does not pass a balise group with MA information, the train does not received a SR authorisation) . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/12 | FS/OS/LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | TARGET DISTANCE > D_NVOVTRP |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 3/4 | L2/L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/OS/LS | The driver selects "Override EOA" | I | DMI | - | L2/L3 | FS/OS/LS | - |
| 2 | L2/L3 | FS/OS/LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | L2/L3 | FS/OS/LS | - |
| 3 | L2/L3 | FS/OS/LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L2/L3 | SR | - |
| 4 | L2/L3 | SR | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L2/L3 | SR | - |
| 5 | L2/L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L2/L3 | SR | - |
| 6 | L2/L3 | SR | D=d1 (d1 can be different or not from do) The train receives a new MA and the on-board switches to FS mode | - | - | Use_FT4060300.21 : The train is in level 2 or 3 in SR and a new MA, SSP and Gradient Profile is received on-board. No specific mode is required by a Mode Profile. The system switches to FS immediately. Transition condition Id [31] | L2/L3 | FS | - |
| 7 | L2/L3 | FS | The override status is REMOVED | O | DMI | - | L2/L3 | FS | - |
| 8 | L2/L3 | FS | The cancellation of override status is RECORDED | O | JRU | - | L2/L3 | FS | - |
| 9 | L2/L3 | FS | The former EOA/LOA, that was in use before the Override activation, has been replaced with the one of the new MA received | O | DMI | - | L2/L3 | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 12 | OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LS/OS mode is DISPLAYED. Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the train crosses a balise group with information "stop if in SR mode", the “override status” is removed so the train trip suppression is cancelled |  | 4 |
| Applicable Mode/Level Combinations | L1/L2/L3: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that: The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The override status indication is removed when the train crosses a balise group with information "stop if in SR mode", The former EOA/LOA shall be deleted The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-5.8.3.1.1 |  |
|  |  | Subset-026-5.8.3.1.3 |  |
|  |  | Subset-026-5.8.3.1.3 a) |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 d) |  |
|  |  | Subset-026-7.4.2.33 Packet Number 137: Stop if in SR |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI/JRU that: The EOA/LOA is retained after the activation of Override The override status is removed when the train crosses a BG with information "stop if in SR mode" Check on the JRU that: NO transition to TR is recorded after reception of information "stop if in SR mode" when Override is active. Check on the DMI/JRU/TIU that: The trip inhibition ends when the train crosses a BG with information "stop if in SR mode" |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before a balise group with "stop if in SR mode" information for the train direction is crossed (while the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed, the train does not pass EOA/LOA given in previous modes, the train doesn’t received a SR authorisation). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/12 | FS/OS/LS |
| Movement Authority | STORED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED / NOT STORED | If stored, it does not include the identity of the balise groups sent in this TC |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | FS/OS/LS | Use_FT4060300.56 Driver selects “override”, train speed is under or equal to the speed limit for triggering the “override” function and the mode is FS, LS and OS. Condition Id [37] | - | - | D=do The driver selects override request so the system switches to SR mode and the override status is displayed | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | NOT The Target distance is updated | O | DMI | - | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | One Balise Group message containing packet 137 is received | I | BTM | Train crosses a balise group with ‘Stop if in SR’ information | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | BG with P137 | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The former EOA/LOA is deleted No EOA/LOA is Displayed | O | DMI | - | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | The override status is REMOVED | O | DMI | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=0>) is recorded | O | JRU | The cancellation of override status is RECORDED | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [54] not executed because Override is active | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | No transition to TR mode occurs when the minimum safe antenna position passes over the location of the former EOA/LOA | O | DMI | - | L1/L2/L3 | SR | - |
| 11 | L1/L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 137 | Stop if in SR information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SRSTOP | 1 | 0 | Stop if in SR |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 /3 /4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is DISPLAYED. Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The train is in SR mode and override status is displayed. As soon as the train crosses a balise group not in the list of expected balises in SR mode, the “override status” is removed so the train trip suppression is cancelled |  | 5 |
| Applicable Mode/Level Combinations | L2/L3: SR |  |  |
| Target of Test | The target of this test is to check that: The override status is indicated to the driver as long as the train trip is suppressed The override status indication is removed when the train crosses a balise group not listed in the list of expected balises in SR mode. The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 g) |  |
|  |  | Subset-026-7.4.2.16 Packet Number 63: List of Balises in SR Authority |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the override status is removed when the train crosses a BG group not in the list of expected balises in SR mode. Check on the JRU that NO transition to TR is recorded after reception of a BG not in the list of expected balises for SR mode when Override is active. Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a BG not in the list of expected balises for SR mode when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the train crosses a balise group not in the list of expected balises (while the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed, the train does not pass EOA given in previous modes, the train doesn’t received a SR authorisation). The BG found is marked as linked and it is not included in List of expected Balises for SR. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 3/4 | L2/L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. "Override status" is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SR | The train is in SR mode and the override status is displayed. | O | DMI | - | L2/L3 | SR | - |
| 2 | L2/L3 | SR | Message 2 from RBC is received( SR authorisation ) INCLUDING PACKET 63 (LIST OF BALISES IN SR AUTHORITY) | I | RTM | - | L2/L3 | SR | - |
| 3 | L2/L3 | SR | Message recorded | O | JRU | - | L2/L3 | SR | - |
| 4 | L2/L3 | SR | Train crosses a balise group group not in the list of expected balises | I | BTM | - | L2/L3 | SR | - |
| 5 | L2/L3 | SR | Received balise message is RECORDED | O | JRU | - | L2/L3 | SR | - |
| 6 | L2/L3 | SR | The override status is REMOVED | O | DMI | - | L2/L3 | SR | - |
| 7 | L2/L3 | SR | The cancellation of override status is RECORDED | O | JRU | - | L2/L3 | SR | - |
| 8 | L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L2/L3 | SR | - |
| 9 | L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [36] not executed because Override is active | L2/L3 | SR | - |
| 10 | L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.27: The switches from SR to TR. Transition Id [36] | L2/L3 | TR | - |

| Step 2: Radio Message 2 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 2 | SR AUTHORISATION |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| Q_SCALE | 8 | FINITE VALUE | - |
| D_SR | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 63 | - |
| Q_DIR | 2 | 1 | NOMINAL |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | 5 | NUMBER OF BALISES TO BE PASSED |
| Q_NEWCOUNTRY | 1 | 1 | - |
| NID_C(K) | 10 | FINITE VALUE | - |
| NID_BG(K) | 14 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is DISPLAYED. Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, and override status is displayed. As soon as the train overpasses the SR distance supervised before overriding with its estimated front end, the “override status” is removed so the train trip suppression is cancelled |  | 6 |
| Applicable Mode/Level Combinations | L1/L2/L3: SR |  |  |
| Target of Test | The target of this test is to check that: The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The override status indication is removed when the train overpasses the SR distance supervised before overriding with its estimated front end. The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.4.1 h) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-5.8.3.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the override status is removed when the train overpasses the SR distance with its estimated front end. Check on the JRU that NO transition to TR is recorded after overpassing the SR distance supervised before overriding when Override is active. Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an information "stop if in SR mode" when Override is active”. |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the end of this TC (while the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed, the train does not pass EOA given in previous modes , the train doesn’t received a SR authorisation). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | V_TRAIN < V_NVALLOWOVTRP | I | INT | - | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | D=do The driver selects override and the override status is displayed | I | DMI | do is the position of the train when the driver selects override | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | The driver selection is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The override status is REMOVED | O | DMI | Train overpasses the SR distance supervised before overriding | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The cancellation of override status is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [42] not executed because Override is active | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is DISPLAYED. Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the minimum safe antenna position of the train passes the former EOA/LOA, override status is removed so the train trip suppression is cancelled |  | 7 |
| Applicable Mode/Level Combinations | L1/L2/L3: PT L2/L3: SB |  |  |
| Target of Test | The target of this test is to check that: The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The override status indication is removed when the minimum safe antenna position (calculated by subtracting distance between active EUROBALISE antenna and the front end of the train from the min safe front end position) of the train passes the former EOA /LOA The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1.1 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 c) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the override status is removed when the minimum safe antenna position of the train passes the former EOA/LOA Check on the JRU that NO transition to TR is recorded after overpassing the former EOA/LOA when Override is active Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an information "stop if in SR mode" when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the minimum safe antenna position of the train passes the former EOA/LOA. (While the override status is displayed, D_NVOVTRP is not passed, and the train does not receive a new proceed information, the train does not pass a balise with "stop if in SR mode" information, the train does not received a SR authorisation) . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6/8 | SB/PT |
| Movement Authority | STORED | TARGET DISTANCE < D_NVOVTRP |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L 3 |
| Train Running Number | VALID | Due to SB or PT mode, the train running number shall be valid. |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SB/PT | The driver selects "Override EOA" | I | DMI | - | L1/L2/L3 | SB/PT | - |
| 2 | L1/L2/L3 | SB/PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | L1/L2/L3 | SB/PT | - |
| 3 | L1/L2/L3 | SB/PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | The Mode symbol "Override EOA is active" is removed | O | DMI | As soon as the minimum safe antenna position of the train exceeds the former EOA/LOA As the mode was SB or PT, the current position of the train front when activating Override shall be considered as the former EOA/LOA. | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | The cancellation of override status is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [43] not executed because Override is active | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2/3/4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the train receives a new MA from a balise group, the on-board switches to FS and the “override status” is removed so the train trip suppression is cancelled |  | 8 |
| Applicable Mode/Level Combinations | L1: FS/LS/OS |  |  |
| Target of Test | The target of this test is to check that: The function Override (trip inhibition) is active The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The override status indication is removed when the train receives a new MA from a balise The former EOA/LOA that was in use before the Override activation is deleted when SR mode is left |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-5.8.3.1.3 |  |
|  |  | Subset-026-5.8.3.1.3 b) |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.4.1 e) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that the override status is removed when the train passes a balise group giving an MA with no signalling related speed restriction of value zero. Check on the DMI that the former EOA/LOA that was in use before the Override activation is deleted when SR mode is left. Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an MA with no signalling related speed restriction of value zero. |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the train receives the new MA . (While the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed, the train does not pass EOA/LOA given in previous modes, the train does not pass a balise with "stop if in SR mode" information, the train does not received a SR authorisation) . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/12 | FS/OS/LS |
| Movement Authority | STORED | TARGET DISTANCE D_NVOVTRP |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is DISPLAYED. Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/OS/LS | The driver selects "Override EOA" | I | DMI | - | L1 | FS/OS/LS | - |
| 2 | L1 | FS/OS/LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded | O | JRU | - | L1 | FS/OS/LS | - |
| 3 | L1 | FS/OS/LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L1 | SR | - |
| 4 | L1 | SR | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L1 | SR | - |
| 5 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L1 | SR | - |
| 6 | L1 | SR | One Balise Group message containing packet 12 (<V_MAIN different from 0>), packet 21, packet 27 is received | I | BTM | - | L1 | SR | - |
| 7 | L1 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=12; <V_MAIN different from 0>; NID_PACKET=21; NID_PACKET=27) is recorded | O | JRU | - | L1 | SR | - |
| 8 | L1 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The train switches to FS | L1 | FS | - |
| 9 | L1 | FS | The override status is REMOVED | O | DMI | - | L1 | FS | - |
| 10 | L1 | FS | The cancellation of override status is RECORDED | O | JRU | - | L1 | FS | - |
| 11 | L1 | FS | The former EOA/LOA, that was in use before the Override activation, has been replaced with the one of the new MA received | O | DMI | - | L1 | FS | - |
| 12 | L1 | FS | Override is not active and the train switches to TR | - | - | Use_FT4060300.26: The switches from FS/LS/OS/SR to TR. Transition Id [18] | L1 | TR | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 001 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Different from 0 |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 001 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR mode is DISPLAYED. Override status is NOT DISPLAYED. V_NVSUPOVTRP is NOT DISPLAYED as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system does not change the mode (SH). While passing a balise with “Danger for shunting” information and indicating “stop if in SH”, the train does not switch to TR. As soon as the train passes the balise with “Danger for shunting information”, the train trip suppression is cancelled. |  | 9 |
| Applicable Mode/Level Combinations | L0, LNTC, L1, L2, L3: SH |  |  |
| Target of Test | The target of this test is to check that: There is no switch to mode SR, when override is selected. The Selection of override avoids a train trip, when passing the balise with “danger for shunting” information and indicating “stop if in SH”, After the train passes a balise with “Danger for shunting information” the train trip suppression is cancelled The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 d) |  |
|  |  | Subset-026-5.8.3.1 b) |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-3.11.10.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train don’t switch to SR mode. The permitted speed is identical to V_NVALLOWOVTRP Check on JRU: Selection of override request is RECORDED The balise message with packet 132 is RECORDED The cancellation of override status is RECORDED NO transition to TR is recorded after reception of information “danger for Shunting” when Override is active Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an information “danger for Shunting” when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before a balise with “Danger for shunting is passed (While the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP is not passed, the train does not pass”, the train does not receive a new MA, the train does not receive a SR authorisation) V_NVSHUNT > V_NVSUPOVTRP according to most restrictive value V_NVSUPOVTRP is identical with permitted speed. If in L0 or LNTC, the “Danger for shunting information” will be received together with an with an immediate level transition order to L1/2/3. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1/L2/L3 | SH | The driver selects “override request” | I | DMI | The “override request” is available only when the train speed is below V_NVALLOWOVTRP (execute with no standstill) | L0/LNTC/L1/L2/L3 | SH | - |
| 2 | L0/LNTC/L1/L2/L3 | SH | The selection of “override request” is RECORDED | O | JRU | As soon as the override request is received | L0/LNTC/L1/L2/L3 | SH | - |
| 3 | L0/LNTC/L1/L2/L3 | SH | The driver requests the indication of permitted speed. | I | DMI | - | L0/LNTC/L1/L2/L3 | SH | - |
| 4 | L0/LNTC/L1/L2/L3 | SH | The drivers request is RECORDED | O | JRU | - | L0/LNTC/L1/L2/L3 | SH | - |
| 5 | L0/LNTC/L1/L2/L3 | SH | The permitted speed is indicated to the driver. | O | DMI | Train speed is supervised against V_NVSUPOVTRP (excepted in LNTC) and V_NVSHUNT (permitted speed = V_NVSUPOVTRP) | L0/LNTC/L1/L2/L3 | SH | - |
| 6 | L0/LNTC/L1/L2/L3 | SH | The train passes a balise group containing “Danger for SH” information (packet 132) and indicating “Stop if in SH” Q_ASPECT = 0 The train does not switch to TR because the train trip is inhibited | I | BTM | see TC constraints | L0/LNTC/L1/L2/L3 | SH | - |
| 7 | L0/LNTC/L1/L2/L3 | SH | The balise message with packet 132 is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 8 | L1/L2/L3 | SH | The permitted speed (V_NVSHUNT) is DISPLAYED on driver request | O | DMI | Train speed is not supervised against V_NVSUPOVTRP anymore | L1/L2/L3 | SH | - |
| 9 | L1/L2/L3 | SH | The override status is REMOVED | O | DMI | - | L1/L2/L3 | SH | - |
| 10 | L1/L2/L3 | SH | The cancellation of override status is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 11 | L1/L2/L3 | SH | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SH | - |
| 12 | L1/L2/L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [49] not executed because Override is active | L1/L2/L3 | SH | - |
| 13 | L1/L2/L3 | SH | Override is not active and the train switches to TR | - | - | Use_FT4060300.32: The switches from SH to TR. Condition Id [49] | L1/L2/L3 | TR | - |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 132 | Danger for Shunting information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | 0 | Stop if in SH |
| NID_PACKET | 8 | 41 | Only if level is L0 or LNTC |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Immediate transition order |
| M_LEVELTR | 3 | 2 / 3 / 4 | L1/2/3 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "override status" is NOT DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board Equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects override request, the system does not change the mode. As soon as the train crosses a balise group not in the list of expected balises in SH mode, the train trip suppression is cancelled |  | 10 |
| Applicable Mode/Level Combinations | L1/L2/L3: SH |  |  |
| Target of Test | The target of this test is to check that: There is no switch to SR mode, when override is selected. The selection of override avoids a train trip, until the train crosses a balise group not listed in the list of expected balises. The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 b) |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 g) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The train does not switch to SR mode The train does not switch to TR mode, when crossing the balise group. Check on JRU: Selection of override request is RECORDED. The balise message is RECORDED. The cancellation of override status is RECORDED. NO transition to TR is recorded after reception of a balise group the identity of which is not in the list of expected balises related to SH mode when Override is active Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group the identity of which is not in the list of expected balises related to SH mode when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the train crosses a balise group not in the list of expected balises in SH mode . (While the override status is displayed, D_NVOVTRP is not passed, T_NVOVTRP has not elapsed). List of balises for SH Area stored on-board. The BG found is not in the list of balises for SH Area |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SH | The driver select “override request” | I | DMI | The “override request” is available only when the train speed is below V_NVALLOWOVTRP (execute with no standstill) | L1/L2/L3 | SH | - |
| 2 | L1/L2/L3 | SH | The selection of “override request” is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 3 | L1/L2/L3 | SH | The override status is DISPLAYED | O | DMI | - | L1/L2/L3 | SH | - |
| 4 | L1/L2/L3 | SH | The display of override status is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 5 | L1/L2/L3 | SH | Train reads a message while crossing a balise group not in the list of expected balises. | I | BTM | - | L1/L2/L3 | SH | - |
| 6 | L1/L2/L3 | SH | Received balise message is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 7 | L1/L2/L3 | SH | The train mode is SH. The train does not switch to TR because the train trip is inhibited. | O | DMI | - | L1/L2/L3 | SH | - |
| 8 | L1/L2/L3 | SH | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SH | - |
| 9 | L1/L2/L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [52] not executed because Override is active | L1/L2/L3 | SH | - |
| 10 | L1/L2/L3 | SH | The override status is REMOVED | O | DMI | - | L1/L2/L3 | SH | - |
| 11 | L1/L2/L3 | SH | The cancellation of override status is RECORDED | O | JRU | - | L1/L2/L3 | SH | - |
| 12 | L1/L2/L3 | SH | Override is not active and the train switches to TR | - | - | Use_FT4060300.34: The switches from SH to TR. Transition Id [52] | L1/L2/L3 | TR | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | - | Optional Packet. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Override status is NOT DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects “override request”, the system does not change the mode and override status is displayed. While passing a balise with "stop if in SR mode", the train does not switch to TR. As soon as the train passes the balise with "stop if in SR mode" information , override status is removed so the train trip suppression is cancelled |  | 11 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that:. The override status is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed. While the override status is displayed, the train trip is suppressed so that the train does not switch to trip while passing the balise with "stop if in SR mode" information The override status indication is removed when the train passes a balise with "stop if in SR mode" information. At this moment, the train trip suppression is cancelled The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.4.1 d) |  |
|  |  | Subset-026-3.11.10.1 |  |
|  |  | Subset-026-7.4.2.33 Packet Number 137: Stop if in SR |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The override status is displayed when the driver selects “override request” The train does not switch to TR mode while passing the balise group with "stop if in SR mode", The “override status” is removed when the train passes a balise group with "stop if in SR mode" Check on the JRU that: Selection of “override request“ is recorded Balise telegram with packet137 is recorded Indication and cancellation of “override status” is recorded NO transition to TR is recorded after reception of information "stop if in SR mode" when Override is active. Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an information "stop if in SR mode" when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before the BG with "stop if in SR mode" is crossed (While the override status is displayed, T_NVOVTRP is not passed, and D_NVOVTRP is not passed, no MA is received,) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED / NOT STORED | If stored, it does not include the identity of the balise groups sent in this TC |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 2/3/4 | L1/L2/L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1/L2/L3 | SR | The driver selects “override request” | I | DMI | The “override request” is available only when the train speed is below V_NVALLOWOVTRP | L1/L2/L3 | SR | - |
| 2 | L1/L2/L3 | SR | The selection of “override request” is RECORDED | O | JRU | As soon as the override request is received | L1/L2/L3 | SR | - |
| 3 | L1/L2/L3 | SR | The “override status” is DISPLAYED. | O | DMI | As soon as the override request is received | L1/L2/L3 | SR | - |
| 4 | L1/L2/L3 | SR | The indication of “override status” is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 5 | L1/L2/L3 | SR | The driver requests the indication of permitted speed. | I | DMI | - | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | The drivers request is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | The permitted speed V_NVSUPOVTRP is indicated to the driver. | O | DMI | - | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | One Balise Group message containing packet 137 (Q_SRSTOP=0) is received | I | BTM | The train passes a balise group containing "stop if in SR mode" information (packet 137) and indicating "stop if in SR mode" The balise group is reached before the max time for train trip suppression is passed or the train has run more than the max distance for train trip suppression. The train does not switch to TR because the train trip is inhibited | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The balise message with packet 137 is RECORDED | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | The “override status” indication is REMOVED | O | DMI | - | L1/L2/L3 | SR | - |
| 11 | L1/L2/L3 | SR | The cancellation of override status is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 12 | L1/L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SR | - |
| 13 | L1/L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR | L1/L2/L3 | SR | - |
| 14 | L1/L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| Step 8: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 137 | Stop if in SR information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SRSTOP | 1 | 0 | Stop if in SR |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second balise |
| N_TOTAL | 3 | 001 | 2 balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | BG1 | BG 1 |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2/3/4 UNCHANGED | L1/L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "override status" is not DISPLAYED. V_NVSUPOVTRP is not displayed as permitted speed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.8.4: End of Override procedure |  | 5080400 |
| Test Case of Feature | The driver selects “override request”, the system does not change the mode (UN/SN) and override status is displayed. The train reaches the border of a level transition and switches to L1/2/3. Up to now no MA has been received but the override status is displayed so the on-board switches automatically to SR mode instead of TR mode. As soon as a balise group with "stop if in SR mode" information for the train direction is crossed, “override status” is removed so the train trip suppression is cancelled |  | 12 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: The “override status” is indicated to the driver since the driver selects “override request”, and as long as the train trip is suppressed The on-board does not enter in TR mode when it enters in the level 1/2/3 area as it has not received MA data The mode changes to SR only when the train enters in level 1/2/3 area. The “override status” indication is removed when as a balise group with "stop if in SR mode" information for the train direction is crossed, The function Override (trip inhibition) is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - Override (Trip inhibition) |  |
|  |  | Subset-026-5.8.4.1 d) |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 a) |  |
|  |  | Subset-026-5.8.2.1 b) |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 c) |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-3.11.10.1 |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-7.4.2.33 Packet Number 137: Stop if in SR |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The “override status” is displayed when the driver selects “override request”. The SR mode is displayed when the level is changed to L1/L2/L3. The V_NVSTFF is displayed as permitted speed on driver request if the train is in SR mode. The “override status” indication is cancelled when a balise group with "stop if in SR mode" information for the train direction is crossed. Check on the JRU that: The override request received by the on-board equipment is recorded. The indication of the “override status” on the DMI is recorded. The cancellation of the “override status” indication on the DMI is recorded. No MA data are recorded. The transition to level 1/2/3 is recorded. NO transition to TR is recorded after reception of information "stop if in SR mode" when Override is active.” Check on the DMI/JRU/TIU that: The trip inhibition ends when the train passes a balise group giving an information "stop if in SR mode" when Override is active |
| Constraints | The other conditions of re-activation of the transition to trip should not occur before a balise group with “stop if in SR” information for the train direction is crossed. (While the override status is displayed, D_NVOVTRP is not exceeded, T_NVOVTRP has not elapsed, the train does not receive a new proceed information.) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Movement Authority | UNKNOWN | - |
| Train Data | STORED | Validated Train Data is stored. |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Possibility for driver input "Override request". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| V_TRAIN < V_NVALLOWOVTRP | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC | UN/SN | The driver selects “override request” | I | DMI | - | L0/LNTC | UN/SN | - |
| 2 | L0/LNTC | UN/SN | The “override request” received by the on-board equipment is RECORDED | O | JRU | Recorded when received on-board | L0/LNTC | UN/SN | - |
| 3 | L0/LNTC | UN/SN | The “override status” is DISPLAYED | O | DMI | - | L0/LNTC | UN/SN | - |
| 4 | L0/LNTC | UN/SN | The indication of the “override status” is RECORDED | O | JRU | - | L0/LNTC | UN/SN | - |
| 5 | L0/LNTC | UN/SN | The train enters in the level 1/2/3 area after t<<T_NVOVTRP and the on-board enters in SR mode | - | - | Use_FT4060300.52 : The Override status is active, the level changes to level 1 and the system automatically switches to mode SR Condition Id [44] OR Use_FT4060300.53 : The Override status is active, no unconditional emergency stop message has been received, the level changes to level 2 or 3 and the system automatically switches to mode SR Condition Id [45] | L1/L2/L3 | SR | - |
| 6 | L1/L2/L3 | SR | The driver requests the indication of permitted speed. | I | DMI | - | L1/L2/L3 | SR | - |
| 7 | L1/L2/L3 | SR | The drivers request is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 8 | L1/L2/L3 | SR | The permitted speed V_NVSUPOVTRP is indicated to the driver. | O | DMI | - | L1/L2/L3 | SR | - |
| 9 | L1/L2/L3 | SR | One Balise Group message containing packet 137 (Q_SRSTOP=0) is received | I | BTM | - | L1/L2/L3 | SR | - |
| 10 | L1/L2/L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | NID_PACKET=137; Q_SRSTOP=0 | L1/L2/L3 | SR | - |
| 11 | L1/L2/L3 | SR | V_NVSUPOVTRP is NOT DISPLAYED as permitted speed V_NVSTFF is DISPLAYED as permitted speed | O | DMI | - | L1/L2/L3 | SR | - |
| 12 | L1/L2/L3 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V_NV_STFF>) is recorded | O | JRU | - | L1/L2/L3 | SR | - |
| 13 | L1/L2/L3 | SR | The Mode symbol "Override EOA is active" is removed | O | DMI | - | L1/L2/L3 | SR | - |
| 14 | L1/L2/L3 | SR | The cancellation of the “override status” indication is RECORDED | O | JRU | - | L1/L2/L3 | SR | - |
| 15 | L1/L2/L3 | SR | Emergency brake not commanded | O | TIU | - | L1/L2/L3 | SR | - |
| 16 | L1/L2/L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The train does not switch to TR Transition Condition Id [54] not executed because Override is active | L1/L2/L3 | SR | - |
| 17 | L1/L2/L3 | SR | Override is not active and the train switches to TR | - | - | Use_FT4060300.35: The switches from SR to TR. Transition Id [54] | L1/L2/L3 | TR | - |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 001 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 137 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SRSTOP | 1 | 0 | Stop if in SR mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 001 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Movement Authority | UNKNOWN | - |
| ERTMS/ETCS level | 2,3,4 | L1/L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Override status is NOT DISPLAYED. Level 1/2/3 is DISPLAYED. TR mode is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |