| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4040700 (4.4.7: STAND BY) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-4040700 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 25.09.2013 | All | review | DLR |
| Version 2.9.2 03.12.2013 | All | review | DLR |
| Version 2.9.3 13.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.7: STAND BY |  | 4040700 |
| Test Case of Feature | In Stand-By mode, the desk of the engine can be open or closed. No interaction with the driver shall be possible as long as the desk is closed, except isolation of the ERTMS/ETCS on-board equipment. |  | 1 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is: • The desk of the engine can be open or closed • to check the suppression of interaction while desk is closed and OBU is isolated • to check that isolation of the ERTMS/ETCS on-board equipment is possible when the desk is closed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.7.1.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the desk that: - No interaction is possible while desk is closed Check on the DMI that: - No interaction is possible when the desk is closed, - except isolation of the ERTMS/ETCS on-board equipment |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L NTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Cab active | I | TIU | The driver opens the desk | N/A | N/A | - |
| 3 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | M_CAB_A_STATUS = 1 | N/A | N/A | - |
| 4 | N/A | N/A | Cab not active | I | TIU | The driver closes the desk | N/A | N/A | - |
| 5 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | M_CAB_A_STATUS = 0 | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | Check that no interaction is possible, except for • desk opening • isolation of the ERTMS/ETCS on-board equipment | N/A | N/A | - |
| 7 | N/A | N/A | No interaction with the DMI is possible | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | The driver isolates the ERTMS/ETCS on-board equipment. OBU enters isolation mode (M_MODE = 10) | N/A | N/A | - |
| 9 | N/A | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=10) is recorded | O | JRU | - | N/A | IS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 | IS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L NTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Entries are shown as is | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |