| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3050600 (3.5.6: Registering to the Radio Network) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-3050600 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.10.2013 | All | review | DLR |
| Version 2.9.2 05.12.2013 | All | review | DLR |
| Version 2.9.3 19.12.2013 | All | review | DLR |
| Version 2.9.4 22.01.2014 | TC6, TC7 TC3 | “Followed by” step changed to “Use_” step “Preceeded by” changed to “Use_” step. M_MODE in Starting conditions changed accordingly. Level/Mode of new “Use_” steps inserted accordingly. | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall order the registration of its connected Mobile Terminal to a Radio Network at Power Up |  | 1 |
| Applicable Mode/Level Combinations | L0: NP LNTC: NP L1: NP L2: NP L3: NP |  |  |
| Target of Test | The target of this test is to check that: - The mobile terminal registers to a default radio network, because No Network Radio Identity is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.1 |  |
|  |  | Subset-026-3.5.6.1 a) |  |
|  |  | Subset-026-3.5.6.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | After the driver powers up the onboard unit, to check that a registration to the default network is established. |
| Constraints | No NID_MN is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | NOT STORED | NID_MN=UNKNOWN |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | No mobile terminal is duly registered to a radio network. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | NP | Use_FT4060300.10 (The ETCS on-board equipment is powered. Transition Id [4]) | - | - | The driver powers up the onboard unit | N/A | SB | - |
| 2 | N/A | SB | USE_FT5040300.24 (Start of Mission procedure is engaged) | - | - | The driver opens desk | N/A | SB | - |
| 3 | N/A | N/A | NOT The system status message "Radio network registration failed" is displayed | O | DMI | The onboard system registers to a default radio network. In fact the way to check that registration is made to the default radio network is to check the +COPS command sent. This is however not possible at the current laboratory implementation, so only the checking of the "Radio Network Registration Failed" is possible at this stage. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Is registered to a radio network but no safe connection is set-up yet |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall order the registration of its connected Mobile Terminal to a Radio Network |  | 2 |
| Applicable Mode/Level Combinations | L2: FS / OS / SR / UN / SB / PT / NL / LS / SN L3: FS / OS / SR / UN / SB / PT / NL / LS / SN |  |  |
| Target of Test | The target of this test is to check that - The Radio Network ID can be entered when the train is at standstill - The mobile terminal registers to a radio network |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.1 |  |
|  |  | Subset-026-3.5.6.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | At standstill, to check that - The driver is asked to re-enter the Radio Network ID - The onboard system registers to radio network with the entered ID |
| Constraints | A radio Network ID is memorized onboard. The train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SB / PT / NL / LS / SN |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | STORED | A valid Driver ID is stored onboard |
| Radio Network ID | STORED | NID_MIN stored onboard |
| RBC ID/Phone Number | INVALID or UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The RBC contact window is DISPLAYED | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | Standstill is the condition for the Radio Network ID to be entered. | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses the "Radio Network ID" button | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The "Radio Network ID" Window is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver enters a new Radio Network ID | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The system status message "Radio network registration failed" is displayed | O | DMI | The onboard system registers to radio network with the entered ID. In fact the way to check that registration is made to the default radio network is to check the +COPS command sent. This is however not possible at the current laboratory implementation, so only the checking of the "Radio Network Registration Failed" is possible at this stage. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SB / PT / NL / LS / SN |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Is registered to a radio network but no safe connection is set-up yet |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall memorize the last received Radio Network Identity and shall use it when powered up again. |  | 3 |
| Applicable Mode/Level Combinations | L0: SB LNTC: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: • The onboard is powered down • The onboard is powered up • The Radio Network Identity is still stored onboard • The onboard uses the Radio Network Identity to registers the mobile terminal to the radio network |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.1 |  |
|  |  | Subset-026-3.5.6.1 a) |  |
|  |  | Subset-026-3.5.6.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • A NID_MN is stored onboard • The onboard device is switch off • The onboard device is powered up • The mobile terminal registers to the radio network by using the stored NID_MN |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 9/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ SF/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | NID_MN |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS / OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ SF/ NL/ / SN/ RV | Use_FT4060300.1 (The ETCS on-board system is NOT powered. Transition Id [29]) | - | - | Is registered to a radio network | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | NP | Use_FT4060300.10 (The ETCS on-board equipment is powered .Transition Id [4]) | - | - | The driver powers up the onboard unit | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | SB | Use_FT5040300.24 (Start of Mission procedure is engaged) | - | - | The driver opens desk | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SB | The system status message "Radio network registration failed" is not displayed | O | DMI | The onboard system registers to a radio network using the stored NID_MN | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | SB | Use_FT5040300.7 “The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown.” | - | - | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | SB | Use_FT5040300.10 "The on-board equipment requests the driver to enter an ERTMS/ETCS Level within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. " | - | - | The driver selects "Level 2" or "Level 3" | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | The driver selects "Radio Network ID" | I | DMI | - | L2 / L3 | SB | - |
| 8 | L2 / L3 | SB | The 'Radio Network ID Window' is displayed | O | DMI | Check that the Network ID is the same as before power down | L2 / L3 | SB | - |
| 9 | L2 / L3 | SB | The driver closes the “Radio Network ID” window | I | DMI | - | L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Is registered to a radio network but no safe connection is set-up yet |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment receives a radio infill area information. The packet is REJECTED, because no mobile terminal is duly registered to a radio network. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, SR, OS, LS |  |  |
| Target of Test | The target of this test is to check that - The telegram including packet 133 is rejected. - No radio infill area information is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • a balise telegram including packet 133 (Radio infill area information) is rejected and recorded. Check on the RTM: • no connection request to the RTM is sent |
| Constraints | No mobile terminal is duly registered to a radio network |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | No mobile terminal is duly registered to a radio network. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 133 and packet 255 is received | I | BTM | The BG sends a telegram including packet 133 (Radio infill area information) to establish a communication session with RIU to the train. The order is REJECTED., because no mobile terminal is duly registered to a readio network | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | No radio connection request to the RIU is sent | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 16 | Version of the ERTMS/ETCS system; Class 1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | Packet Number 133: Radio infill area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | The number is to be entered "left adjusted" starting with the first digit to be dialled. Padding by the special value F shall be added after the least significant digit of the number. |
| D_INFILL | 15 | FINITE VALUE | Distance from location where to connect to a radion infill unit |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | No mobile terminal is duly registered to a radio network. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall REJECT any order to contact an RBC as long as no mobile terminal is duly registered to a radio network. |  | 5 |
| Applicable Mode/Level Combinations | L2: SB, FS, SR, OS, TR, PT, NL, RV, LS L3: SB, FS, SR, OS, TR, PT, NL, RV, LS |  |  |
| Target of Test | The target of this test is to check that: After receiving a session management (packet 42) from the RBC: • The message is rejected by the on-board equipment and nothing changes |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.7 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • a BG message with one packet 42 (Session management) is RECORDED Check on the RTM: • no connection request to the RTM is sent |
| Constraints | No mobile terminal is registered. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | NID_MN |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | No mobile terminal is duly registered to a radio network. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 and packet 255 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | The train does not try to set up a safe connection. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | No mobile terminal is duly registered to a radio network. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall order the registration of its connected Mobile Terminal to a Radio Network if ordered from the trackside. If the onboard equipment has two mobile terminals, the second mobile terminal is NOT USED by the onboard equipment and it is yet NOT REGISTERED to a radio network, and it will have to be registered. |  | 6 |
| Applicable Mode/Level Combinations | L0: NP, SL, SB, SH, UN, NL LNTC: SL, SB, TR, NL, SN L1: SL, SB, SH, FS, UN, SR, OS, TR, PT, NL, RV, LS L2: SL, SB, SH, FS, UN, SR, OS, TR, PT, NL, RV, LS L3: SL, SB, SH, FS, UN, SR, OS, TR, PT, NL, RV, LS |  |  |
| Target of Test | The target of this test is to check that - The not yet registered and a not used mobile terminal registers to a radio network - The trackside orders the establishment of a communication session with the RBC that belongs to the radio network to which the onboard equipment has registered to, and the communication session is established successfully. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.1 c) |  |
|  |  | Subset-026-3.5.6.5 |  |
|  |  | Subset-026-3.5.6.5 a) |  |
|  |  | Subset-026-3.5.6.5 b) |  |
|  |  | Subset-026-3.5.6.5 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The (second or unique) mobile terminal registers to a radio network by means of establishing a communication session. |
| Constraints | The (second or unique) mobile terminal is not used by the onboard equipment and it is not registered to the requested radio network. A cab needs to be active (SB mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | NID_MN=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if the onboard has two mobile terminals and the acceptance of the information Radio Network Registration (Packet 45) is received via RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The mobile terminal registers to the radio network | I | INT | In order to properly test this step, the following step will have to be tested using the appropriate TC. | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3150100.1 (Two communication session are opened. The on-board receives an announcement of the RBC transition from the Handing Over RBC....) if the on-board equipment has two mobile terminals and it received the packet 45 (Radio Network Registration) via RTM (through the already connected mobile terminal), so a Handover should be performed OR Use_FT3050300.22 (System version is compatible and session is established. ) if the on-board equipment received the packet 45 (Radio Network Registration) via BG, so the trackside will order the establishment of a communication session. | - | - | This will test that setting up of a safe connection with the new radio network is successful | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / SE / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | NID_MN |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | The (second or unique) mobile terminal is registered to a radio network |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.6: Registering to the Radio Network |  | 3050600 |
| Test Case of Feature | The onboard equipment shall order the registration of its connected Mobile Terminal to a Radio Network if ordered from the trackside. A second mobile terminal is USED from onboard equipment and it is yet NOT REGISTERED to the new radio network. |  | 7 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, NP, NL LNTC: SL, SB, TR, NL, SE, SN L1: FS, LS, OS, SR, SH, UN, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, UN, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, UN, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The mobile terminal registers to a radio network AFTER the radio communication session is terminated and the safe connection is released • The trackside orders the establishment of a communication session with the RBC that belongs to the radio network to which the on-board equipment has registered to, and the communication session is established successesfully |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.6.1 |  |
|  |  | Subset-026-3.5.6.1 c) |  |
|  |  | Subset-026-3.5.6.5 |  |
|  |  | Subset-026-3.5.6.5 a) |  |
|  |  | Subset-026-3.5.6.5 c) |  |
|  |  | Subset-026-3.5.6.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The second mobile terminal ONLY registers to a radio network if the communication session is terminated and the safe connection is released. • The on-board equipment registers to the radio network by means of establishing a communication session |
| Constraints | Constraints The onboard is equipped with two mobile terminals. The second mobile terminal is not registered to the new radio network, but a communication session and a save connection is already established. A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| RBC Transition Order | STORED | Packet 131 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Radio Network ID | STORED | NID_MN=FINiTE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is connected |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | Current situation: The train performs a RBC handover. Mobile 2 is connected with handing over RBC; Mobile 1 establishes a connection to accepting RBC | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3050500.5 (The on-board equipment terminates a communication session ordered by RBC) | - | - | The RBC handover is completely performed.; The RBC initiates a termination of the communication session. (The safe connection is also released) | N/A | N/A | - |
| 3 | N/A | N/A | The mobile terminal 2 registers to the radio network | I | INT | In order to properly test this step, the following step will have to be tested using the appropriate TC. | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3150100.1 (Two communication session are opened. The on-board receives an announcement of the RBC transition from the Handing Over RBC. It establishes a communication session with the Accepting RBC and it transmits its train data. It sends a position report to the Handing Over RBC and to the Accepting RBC when the maximum safe front end of the train passes the location of the RBC transition. An order to execute the handover immediately is received from the border balise group. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It receives an order to terminate the communication session with the Handing Over RBC.) | - | - | A new Handover should be performed: USE a TC in FT3150100; This will test that setting up of a safe connection with the new radio network is successful. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L 2 / L 3 |
| Radio Network ID | STORED | NID_MN=CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | The mobile terminal 2 is connected because it has previously registered to a radio network; Radio link of both mobile terminals is connected |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |