| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4040600 (4.4.6: SLEEPING) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-4040600 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.09.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | All | review | DLR |
| Version 2.9.3 12.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | The train is not stopped due to a safety critical fault in a sleeping engine. The on-board memorises the fault and handles it when the engine leaves the Sleeping mode. |  | 1 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test case is: • to check the suppression of transition to system failure during sleeping mode. • switch after opening of desk to SF mode (With transition SL - SB - SF) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.5.1.1 |  |
|  |  | Subset-026-4.6.2 row 2 column 9 |  |
|  |  | Subset-026-4.6.2 row 14 column 2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: - The safety fault is RECORDED if the train gets access to the JRU. - A mode change to SF is RECORDED only when the desk is opened. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| EB is RELEASED. | O | TIU | To check that the EB is not applied in the first part of the test it must be released at the start. |
| Mode is NOT DISPLAYED. | O | DMI | No mode is displayed in SL. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | SL | Train is moving (V_TRAIN=40) | I | INT | - | N/A | SL | - |
| 2 | N/A | SL | A safety critical fault occurs | I | INT | A safety critical fault has occurred in the sleeping engine. | Na | SL | - |
| 3 | N/A | SL | Use_FT4040600.5: “The train runs in SL mode and a safety critical fault has occurred. The on-board equipment establishes a communication session (to report the fault to the RBC).” | - | - | Only if level 2/3 with no communication session established | N/A | SL | - |
| 4 | N/A | SL | Use_FT4040600.4: “The on-board equipment sends a radio message 136 “Train Position Report” containing the packet 4 “Error reporting” to the RBC.” | - | - | Only if a communication session is established. | N/A | SL | - |
| 5 | N/A | SL | NOT The Mode symbol "System Failure" is displayed | O | DMI | - | N/A | SL | - |
| 6 | N/A | SL | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | A mode change to SF is NOT RECORDED | N/A | SL | - |
| 7 | N/A | SL | Emergency brake not commanded | O | TIU | In SF the EB is always applied so a released brake indicates that the train has not changed to SF. | N/A | SL | - |
| 8 | N/A | SL | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | EB application is NOT RECORDED | N/A | SL | - |
| 9 | N/A | SL | Cab active | I | TIU | - | N/A | SL | - |
| 10 | N/A | SL | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=1) is recorded | O | JRU | - | N/A | SL | - |
| 11 | N/A | SL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | When a desk is open the on-board shall switch to SB (transition condition [2]). | N/A | SB | - |
| 12 | N/A | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | The fault from Step 2 shall be memorized and handled as soon as SL is left. | N/A | SF | - |
| 13 | N/A | SF | The Mode symbol “System Failure” is displayed | O | DMI | ERA_ERTMS_015560_v3.3.0 8.2.3.1.2 Exception: If MO18 cannot be displayed on the total image display area, the SF mode shall be indicated by any other means (e.g. blank screen, frozen screen, dedicated lamp, sound,...). | N/A | SF | - |
| 14 | N/A | SF | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit33=1>) is recorded | O | JRU | Only if “System Failure” is displayed | N/A | SF | - |
| 15 | N/A | SF | Use_FT4040500.1 | - | - | Permanent application of emergency brake in SF mode | N/A | SF | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 | SF |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | On-board equipment does not switch to Stand By mode after loss of sleeping signal when the train is not at standstill. |  | 3 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that: • The on-board system does not switch to Stand By mode after losing the sleeping input signal when the train is not at standstill. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • The train does not change to SB mode. |
| Constraints | The train is in SL Mode and is running (V_TRAIN > 0). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | Only if in Level 2/3 |
| Train Position | STORED | V_TRAIN>0 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Remote Control is DETECTED. | I | TIU | - |
| NOT RELEVANT | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Remote control signal is lost. | I | TIU | The ERTMS/ETCS on-board equipment does not switch to SB mode. | N/A | N/A | - |
| 2 | N/A | N/A | NOT The Mode symbol "Stand By" is displayed | O | DMI | SB Mode indication is NOT DISPLAYED. | N/A | N/A | - |
| 3 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | SB Mode indication is NOT RECORDED. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | I/O | RTM | Only if in Level 2/3 |
| Remote Control is NOT DETECTED. | I | TIU | - |
| NOT The Mode symbol "Stand By" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | A safety critical fault occurs. The on-board equipment sends a radio message 136 “Train Position Report” containing the packet 4 “Error reporting” to the RBC. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, OS, SR, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that : • The on-board informs the RBC when a safety critical fault error occurs. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-3.6.5.1.4 l) |  |
|  |  | Subset-026-3.16.4.1 |  |
|  |  | Subset-026-8.4.4.4.2 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-4.4.6.1.10 |  |
|  |  | Subset-026-4.4.6.1.10 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • A radio message 136 (Train Position Report) is sent to the RBC Check on the JRU that: • The radio message 136 (Train Position Report) with the packet 4 sent by the on-board is recorded. |
| Constraints | The train is running and a safety critical fault in a sleeping engine occurs. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 and packet 4 is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 6 | Safety critical fault |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | The train runs in SL mode and a safety critical fault has occurred. The on-board equipment establishes a communication session (to report the fault to the RBC). |  | 5 |
| Applicable Mode/Level Combinations | L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that : • In level 2/3 the on-board equipment establishes a communication session if none is established when a safety critical fault occurs. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.10 |  |
|  |  | Subset-026-4.4.6.1.10 c) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: “Initiation of a communication session“ is RECORDED • Message 32: “RBC/RIU System Version” is RECORDED • Message 159: “Session established” is RECORDED |
| Constraints | A safety critical fault in a sleeping engine has occurred. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Identification number of the emergency stop message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 7: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | The on-board equipment establishes a communication session and sends a position report to inform about a mode change. |  | 6 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: • The on-board opens a communication session when entering or exiting Sleeping mode. • The on-board sends a position report after a mode change. The message includes the new mode value. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 b) |  |
|  |  | Subset-026-4.4.6.1.10 |  |
|  |  | Subset-026-4.4.6.1.10 b) |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • After a mode change, the onboard equipment sends a position report to inform trackside about it. |
| Constraints | A communication session has not been established yet. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Cab not active | I | TIU | all desks connected to the ERTMS/ETCS on-board equipment are closed | N/A | N/A | - |
| 3 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=0) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | L2/L3 | SB | Sleeping requested | I | TIU | - | L2/L3 | SL | - |
| 5 | N/A | N/A | SLEEPING INPUT (NID_MESSAGE_JRU=30) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=5) is recorded | O | JRU | Mode change is recorded | N/A | N/A | - |
| 7 | N/A | N/A | Use_FT3050300.26 “When there is a mode change, not considered as an End of Mission, the on-board shall establish a communication session.“ | - | - | - | N/A | N/A | - |
| 8 | N/A | N/A | Use_FT3050500.5: The on-board equipment terminates a communication session ordered by RBC. | - | - | - | N/A | N/A | - |
| 9 | L2/L3 | SL | Cab active | I | TIU | The driver opens cab A The mode changes to SB | L2/L3 | SB | - |
| 10 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=1) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Change mode is recorded | N/A | N/A | - |
| 12 | N/A | N/A | Use_FT3050300.26 “When there is a mode change, not considered as an End of Mission, the on-board shall establish a communication session.“ | - | - | - | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT3050500.5: The on-board equipment terminates a communication session ordered by RBC. | - | - | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | Desk is opened in sleeping mode |  | 7 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that: If the train is running in SL mode and the driver opens a desk: • the on-board switches immediately to SB mode. • Brake is APPLIED. • Supervision data are deleted / reset / unchanged / to be revalidated (according to SRS 4.10.1.3) when entering SB Mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.7 |  |
|  |  | Subset-026-4.6.2 row 2 column 9 |  |
|  |  | Subset-026-4.6.3 Condition Id [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • SB mode indication is DISPLAYED. • Brake indication is DISPLAYED. Check on the JRU that: • Mode transition to SB is RECORDED. • Brake application is RECORDED. |
| Constraints | • Desk is closed. • The on-board is in SL mode (How the train entered in SL mode does not matter) • Train is running. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | L2 / L3 only |
| National Values | VALID | Concerning D_NVROLL, if national values are not available the default value is used. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| Remote Control is DETECTED | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SL | Cab active | I | TIU | The driver opens cab A The remote control is disabled The mode changes to SB | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=1) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Change mode is recorded | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SB | Use_FT9990200.15 "The standstill protection prevents the train from moving. " | - | - | Standstill supervision is performed | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Remote Control is not detected. Brakes are not applied. | - | TIU | - |
| SB mode is displayed. Brake Indication is removed. | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.6: SLEEPING |  | 4040600 |
| Test Case of Feature | As the engine is remote controlled by the leading engine, its ERTMS/ETCS on-board equipment shall not perform any train movement supervision. |  | 8 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that: If the train is in SL mode: • the train does not apply the brake when driving forward • the train does not apply the brake when driving backward |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Brake indication is not DISPLAYED. Check on the JRU that: • Brake application is not RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Remote Control is DETECTED | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Train is moving | I | INT | Train is moving forward, significantly more than D_NVROLL. | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 10 | N/A | N/A | Train is moving | I | INT | Train is moving backward, significantly more than D_NVROLL. | N/A | N/A | - |
| 11 | N/A | N/A | NOT The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | NOT DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | NOT Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 16 | N/A | N/A | NOT EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Remote Control is DETECTED | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |