| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3060400 (3.6.4: Train Position Confidence Interval) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-3060400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 14.11.2013 | All | review | DLR |
| Version 2.9.2 06.12.2013 | All | review | DLR |
| Version 2.9.3 22.01.2014 | TC14 | “Preceded by” changed to “Use_” | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | The train passes linked Balise Group BGa. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SH, PS, SL, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: If next linked balise group has been read, the confidence interval shall be reset taking the location accuracy into account. • The on-board equipment updates L_DOUBTOVER and L_DOUBTUNDER variables to reduce their values. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 a) |  |
|  |  | Subset-026-3.6.4.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The variables L_DOUBTUNDER and L_DOUBTOVER are stored in each JRU record. Check on the JRU that : • Balise Group BGa Message is RECORDED. • Variables L_DOUBTOVER and L_DOUBTUNDER have reduced their values. Check that in the previous JRU records a position report including a value of the variables L_DOUBTUNDER and L_DOUBTOVER are stored in JRU. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / TR / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| Linking | VALID | If Linking information is stored, Balise Group BGa shall be included. (The onboard equipment could not have Linking information) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train passes the Balise Group BGa. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (NID_BG=<BGa>)containing packet 255 is received | I | BTM | Packets inside telegram are not relevant. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Balise Group Message is RECORDED. | N/A | N/A | - |
| 4 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | L_DOUBTUNDER and L_DOUBTOVER have reduced their values after having recorded the Balise Group BGa. Check that value of L_DOUBTUNDER and L_DOUBTOVER in position report is equal to Q_LOCACC if linking info is available, to Q_NVLOCACC if not, or 12 m if neither linking info nor national values are available. The variables L_DOUBTUNDER and L_DOUBTOVER are stored in each JRU record. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only in L2 / L3 Step 5 and 6 equal to FT3060500.4 (The on-board equipment sends a position report by means of Packet 0: “Position report”.) | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2 / L3 L_DOUBTUNDER and L_DOUBTOVER have reduced their values Check that value of L_DOUBTUNDER and L_DOUBTOVER in position report is equal to Q_LOCACC if linking info is available, to Q_NVLOCACC if not, or 12 m if neither linking info nor national values are available. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked |
| NID_PACKET | 8 | FINITE VALUE | Packets inside telegram are not relevant. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. 2 of N |
| N_TOTAL | 3 | N | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | FINITE VALUE | Packets inside telegram are not relevant. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGa" | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / TR / NL / LS / SN / RV / PS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / TR / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | Only in L2 / L3 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | The train receives from RBC a message which is referred to a previously received balise group different from the LRBG. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that: • The train recalculates the distance between the location related information and the LRBG. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.3 |  |
|  |  | Subset-026-3.6.4.3 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train has a valid L2/L3 MA and has a valid linking package stored on-board. • The train overpasses a BG marked as linked which is included in the known Linking package (BGx) and uses BGx as LRBGx. • The train overpasses a BG marked as linked which is included in the known Linking package (BGy) and uses BGy as LRBGy. • The train receives Radio message which has a reference to the first LRBG (BGx). • The train must calculate the distance between the position where the content of the radio message shall start (location related information = lri) and the position of the LRBG (BGy) of the train. This will be calculated as: (Distance LRBGx - lri) - (Distance LRBGx - LRBGy) under using of the linking information’s stored on-board. |
| Constraints | Communication Session between on-board and trackside is ESTABLISHED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Includes BGx and BGy |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train is moving and has a L2/L3 MA and a linking package stored onboard | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 255 is received | I | BTM | The train passes over ‘BGx’ which is the new LRBG Q_LINK = ‘1’; BG is empty | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The Balise group is RECORDED | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 (NID_LRBG="BGx") is transmitted | O | RTM | NID_LRBG = ‘BGx’ | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message containing packet 255 is received | I | BTM | The train passes over ‘BGy’ which is the new LRBG Q_LINK = ‘1’; BG is empty | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 (NID_LRBG="BGy") is transmitted | O | RTM | NID_LRBG = ‘BGy’ | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 27 (V_STATIC[0]≠V_STATIC[1]) is received | I | RTM | NID_LRBG = ‘BGx’; V_STATIC is unequal to the previous one. The train must recalculate the distance between valid LRBG (BGy) and the start location for the new MA. | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position where the new MA is valid | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The permitted speed is unequal to the previous one. | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | The change of the SSP Data is RECORDED. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGx" | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGx" | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGy" | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGy" | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGx" | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP The permitted speed is unequal to the previous one. (V_STATIC[0]≠V_STATIC[1]) |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | The train receives from an unlinked BG a TSR message which starts behind the position of the next BG which is linked. |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR L3: FS, OS, SR |  |  |
| Target of Test | The target of this test is to check that: • The train recalculates the distance between the location related information and the LRBG. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.3 |  |
|  |  | Subset-026-3.6.4.3 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train has a valid L2/L3 MA and has a valid linking package stored on-board. • The train overpasses a BG marked as linked which is included in the known Linking package (BGx) and uses BGx as LRBGx. • The train overpasses a BG marked as unlinked which is NOT included in the known Linking package (BGy) and uses BGy NOT as LRBGy. Within the BG is a TSR for the Area behind the next BG (BGz) • The train overpasses a BG marked as linked which is NOT included in the known Linking package (BGz) and uses BGz as LRBGz. • The train must calculate the distance between the position where the content of the BGy message shall start (location related information = lri) and the position of the LRBG (BGz) of the train. This will be calculated as: (Distance BGy - lri) - (Distance BGy - LRBGz) under using of the estimated travelled distance. |
| Constraints | Communication Session between on-board and trackside is ESTABLISHED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Linking | STORED | Includes BGx only. Does NOT include BGy and BGz. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train is moving and has a L2/L3 MA and a linking package stored onboard | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 255 is received | I | BTM | The train passes over linked ‘BGx’ which is the new LRBG Q_LINK = ‘1’; BG is empty | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 (NID_LRBG="BGx") is transmitted | O | RTM | NID_LRBG = ‘BGx’ | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message containing packet 65 and packet 255 is received | I | BTM | The train passes over unlinked ‘BGy’ which is NOT the new LRBG including Packet Number 65: ‘Temporary Speed Restriction’ Q_LINK = ‘0’; ‘BGy’ includes a TSR packet which starts behind the position of the next BG; V_TSR is less than the permitted speed. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_LRBG="BGx") is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The TSR information’s are displayed | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message containing packet 255 is received | I | BTM | The train passes over linked (but not included in the linking information on-board) ‘BGz’ which is the new LRBG Q_LINK = ‘1’; BG is empty | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio message (NID_MESSAGE=136) containing packet 0 (NID_LRBG="BGz") is transmitted | O | RTM | NID_LRBG = ‘BGz’ | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses the position where the TSR is valid | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | The begin of the TSR is RECORDED. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGx" | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | V_TSR is less than the permitted speed. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | "BGz" | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | Supervision of location related information transmitted by a balise group marked as unlinked and referred to this balise group. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test case is to check that the ERTMS/ETCS on-board equipment shall temporarily apply by analogy clauses 3.6.4.2, 3.6.4.2.1, 3.6.4.2.3 and 3.6.4.4 to determine an additional confidence interval. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.7.1 |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The train min safe front end position at which the Trip is initiated is calculated referring to the BGa, and to the stored LRBG |
| Constraints | The SSP and gradient already available on-board or given together with the MA cover the full length of the MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train passes over a balise group (BGa). | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | One Balise Group message (NID_BG=<BGa>) containing packet 12, packet 21, packet 27 and packet 255 is received | I | BTM | The BGa is marked as unlinked (Q_LINK=0), so it is NOT set as the new LRBG The BGa contains a MA (movement authority), which is accepted | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) This step is to check, that the train determines an additional confidence interval | I | INT | The on-board equipment detects that the minimum safe antenna position (calculated by subtracting distance between active Eurobalise antenna and the front end of the train from the min safe front end position) has passed the EOA/LOA location, so train Trip is initiated. Check that the calculation of the min safe front end position used to initiate Trip is referred to the BGa, and NOT to LRBG. Hint: since BGa was found AFTER the LRBG, the confidence interval associated to the position of BGa should be smaller than the one associated to LRBG. | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | Emergency brake commanded | O | TIU | The Emergency Brakes is APPLIED | L1 | TR | - |
| 6 | L1 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 | TR | - |
| 7 | L1 | TR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 8 | L1 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L1 | TR | - |
| 9 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | MASK=0000000000002000040000 | L1 | TR | - |
| 10 | L1 | TR | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 | TR | - |
| 11 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | MASK=0000000000002000040000 | L1 | TR | - |

| Step 2: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Excepts 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol "Trip" is displayed and the Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | Supervision of location related information transmitted by a balise group marked as unlinked and referred to this balise group |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test case is to check that the ERTMS/ETCS on-board equipment shall stop applying by analogy clauses 3.6.4.2, 3.6.4.2.1, 3.6.4.2.3 and 3.6.4.4 to determine an additional confidence interval when a further received balise group becomes the LRBG. The second reason for stopping, namely, deletion of location related information on-board, is tested, cannot be tested. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 c) |  |
|  |  | Subset-026-3.6.4.7.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The train min safe front end position at which the Trip is initiated is calculated referring to the BGb, and neither to the initial LRBG not to BGa |
| Constraints | The SSP and gradient already available on-board or given together with the MA cover the full length of the MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | NOT ESTABLISHED | NOT AVAILABLE |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train passes over a balise group (BGa). | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | One Balise Group message (NID_BG=<BGa>) containing packet 12, packet 21, packet 27 and packet 255 is received | I | BTM | The BGa is marked as unlinked, so it is NOT set as the new LRBG The BGa contains a MA (movement authority), which is accepted | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | One Balise Group message (NID_BG=<BGb>) containing packet 12, packet 21, packet 27 and packet 255 is received | I | BTM | Before passing the current EOA/LOA, the train passes over another balise group (BGb). The BGb is marked as linked, so it IS set as the new LRBG The BGb contains a new MA (movement authority), which is accepted | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The on-board equipment detects that the minimum safe antenna position (calculated by subtracting distance between active Eurobalise antenna and the front end of the train from the min safe front end position) has passed the current EOA/LOA location, so train Trip is initiated. Check that the calculation of the min safe front end position used to initiate Trip is referred to the BGb, and NEITHER to the initial LRBG, NOR to BGa. Hint: since BGb was found AFTER the initial LRBG and after the BGa, the confidence interval associated to the position of BGb should be smaller than the one associated to LRBG and smaller than the one associated to BGa. | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | Emergency brake commanded | O | TIU | The Emergency Brakes is APPLIED | L1 | TR | - |
| 8 | L1 | TR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 9 | L1 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L1 | TR | - |
| 10 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 | TR | - |
| 11 | L1 | TR | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 | TR | - |
| 12 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 | TR | - |

| Step 2: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 10000 | Version of the ERTMS/ETCS system Class 1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Excepts 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 10000 | Version of the ERTMS/ETCS system Class 1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Excepts 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | Emergency brakes are APPLIED |
| The Mode symbol "Trip" is displayed and The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | Supervision of location related information transmitted by a balise group marked as unlinked and referred to this balise group |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | If another balise group marked as unlinked is received before the additional confidence interval is deleted: a) the additional confidence interval shall be reset in relation to this new balise group marked as unlinked b) the location related information referred to the previous balise group marked as unlinked shall be relocated by subtracting the estimated travelled distance between both balise groups from the distances that are counted from this previous balise group marked as unlinked. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.7.2 |  |
|  |  | Subset-026-3.6.4.7.2 a) |  |
|  |  | Subset-026-3.6.4.7.2 b) |  |
|  |  | Subset-026-3.6.4.7.1 |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the interfaces that: • The train min safe front end position at which the Trip is initiated is calculated referring to the BGb, and neither to the stored LRBG, nor to the BGa |
| Constraints | The SSP and gradient already available on-board or given together with the MA covers the full length of the MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | NOT ESTABLISHED | NOT AVAILABLE |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) | I | INT | The train passes over a balise group (BGa). | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | One Balise Group message (NID_BG=<BGa>) containing packet 12, packet 21, packet 27 and packet 255 is received | I | BTM | The BGa is marked as unlinked, so it is NOT set as the new LRBG The BGa contains a MA (movement authority), which is accepted | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | One Balise Group message (NID_BG=<BGb>) containing packet 12, packet 21, packet 27 and packet 255 is received | I | BTM | Before passing the current EOA/LOA, the train passes over another balise group (BGb). The BGb is marked as unlinked, so it is NOT set as the new LRBG The BGb contains a new MA (movement authority), which is accepted | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | Train is moving (V_TRAIN=40) This step is to check, that the train determines an additional confidence interval | I | INT | The on-board equipment detects that the minimum safe antenna position (calculated by subtracting distance between active Eurobalise antenna and the front end of the train from the min safe front end position) has passed the EOA/LOA location, so train Trip is initiated. Check that the calculation of the min safe front end position used to initiate Trip is referred to the BGb, and NEITHER to LRBG, NOR to BGa. Hint: since BGb was found AFTER the LRBG and after the BGa, the confidence interval associated to the position of BGb should be smaller than the one associated to LRBG and smaller than the one associated to BGa. | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | Emergency brake commanded | O | TIU | The Emergency Brakes is APPLIED | L1 | TR | - |
| 8 | L1 | TR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 9 | L1 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L1 | TR | - |
| 10 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 | TR | - |
| 11 | L1 | TR | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 | TR | - |
| 12 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 | TR | - |

| Step 2: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 1000 | Version of the ERTMS/ETCS system Class 1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGa" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 10000 | Version of the ERTMS/ETCS system Class 1 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. 1 of N |
| N_TOTAL | 3 | N | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group Except 254 |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | "BGb" | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as unlinked |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol "Trip" is displayed and the Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.4: Train Position Confidence Interval |  | 3060400 |
| Test Case of Feature | The train passes Balise Group BGa. |  | 7 |
| Applicable Mode/Level Combinations | L2: SH, FS, SR, OS, TR, PT, NL, RV, LS, PS L3: SH, FS, SR, OS, TR, PT, NL, RV, LS, PS |  |  |
| Target of Test | The target of this test is to check that: If next balise group has been read, the confidence interval shall be reset taking the location accuracy into account. The location accuracy is NOT available by Linking information and the corresponding National Value is NOT applicable. • The on-board equipment updates L_DOUBTOVER and L_DOUBTUNDER variables to reduce their values. • The fixed value “default location accuracy of a balise group” is used |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.2 |  |
|  |  | Subset-026-3.6.4.2 a) |  |
|  |  | Subset-026-3.6.4.2 b) |  |
|  |  | Subset-026-3.6.4.2.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The variables L_DOUBTUNDER and L_DOUBTOVER are stored in each JRU record. Check on the JRU that : • Balise Group BGa Message is RECORDED. • Variables L_DOUBTOVER and L_DOUBTUNDER have reduced their values to the default location accuracy. Check that L_DOUBTUNDER and L_DOUNBOVER have reduced their values with respect to the values transmitted before overpassing the LRBG. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 7 / 11 / 12 / 14 / 15 | FS / OS / SR / SH / TR / NL / LS / RV / PS |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT3050300.19 "Onboard receives a “session management” packet, the system version is compatible and session is established" | - | - | Check values of L_DOUBTUNDER and L_DOUNBOVER in the position report sent | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (M_LOC=001) is received | I | RTM | Order to send positions reports at every LRBG compliant balise group. | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1; Q_LINK=1) containing packet 255 is received | I | BTM | Packets inside telegram are not relevant. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | USE_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" | - | - | Check that L_DOUBTUNDER and L_DOUNBOVER have reduced their values with respect to the values transmitted in FT3050300.19 | N/A | N/A | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position Report Parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | FINITE VALUE | - |
| D_CYCLOC | 15 | FINITE VALUE | - |
| M_LOC | 3 | 1 | Every LRBG compliant balise group. |
| N_ITER | 5 | FINITE VALUE | - |
| D_LOC(k) | 15 | FINITE VALUE | - |
| Q_LGTLOC(k) | 1 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 7 / 11 / 12 / 14 / 15 | FS / OS / SR / SH / TR / NL / LS / RV / PS |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |