| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080402 (4.8.4: Accepted Information depending on the modes - Adhesion factor) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-4080402 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 11.11.2013 | ALL | REVIEW | CEDEX |
| Version 2.9.3 12.12.2013 | TC8 | Editorial changes | CEDEX |
| Version 2.9.4 18.12.2013 | all | review | CEDEX |
| Version 2.9.5 19.12.2013 | all | review | CEDEX |
| Version 2.9.6 20.12.2013 | all | review | CEDEX |
| Version 2.9.7 14.01.2014 | all | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via RBC. The train stores and accepts a new adhesion factor send by RBC |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment accepts a new relevant adhesion factor send by RBC. the ERTMS/ETCS on-board equipment informs the driver because the value of the adhesion factor is “slippery rail” |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: an indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Message 24: General message including Packet number 71: Adhesion factor is RECORDED The status for the indication of the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED. |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the RBC (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR /LS |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71: Adhesion factor is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | The end of the adhesion area is passed.Distance defined by L_ADHESION in previous packet 71. As the previously sent new adhesion factor is no more valid | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | SLIPERY RAIL |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | 1 | non-slippery rail |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is no longer DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via RBC. The train stores and accepts a new adhesion factor which has been sent by the RBC because a level transition is stored onboard. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR L0: UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment stores accepts a new relevant adhesion factor send by RBC if a level transition announcement to level 2 or 3 has been received onboard. the ERTMS/ETCS on-board equipment informs the driver because the value of the adhesion factor is slippery rail. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: an indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Message 24: General message including Packet number 71: Adhesion factor is RECORDED The status for the indication of the the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the RBC (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 12/ 14 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | 3 / 4 | L2 / L3 Level Transition announcement to Level 2 or 3 |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / L1 / LNTC |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71: Adhesion factor is received | I | RTM | - | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | - |
| 2 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; (NID_MESSAGE=24) is recorded | O | JRU | - | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | - |
| 3 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | Use_FT5100100.13,20,21,26,36,37 | - | - | The train changes to level 2/3. | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 4 | L2/ L3 | FS / OS / SR / UN / LS / SN | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 5 | L2/ L3 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 6 | L2/ L3 | FS / OS / SR / UN / LS / SN | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | The end of the adhesion area is passed.Distance defined by L_ADHESION in previous packet 71. As the previously sent new adhesion factor is no more valid | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 7 | L2/ L3 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | L2/ L3 | FS / OS / SR / UN / LS / SN | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | SLIPERY RAIL |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR /LS |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | 1 | non-slippery rail |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is no longer DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via Balise Group. The train stores and accepts a new adhesion factor sent by a balise |  | 3 |
| Applicable Mode/Level Combinations | L1: FS / OS / SR / SB / LS |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment accepts a new relevant adhesion factor send by a balise group. the ERTMS/ETCS on-board equipment informs the driver that the adhesion factor because the value of the adhesion factor is “slippery rail |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: an indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Eurobalise telegram including Packet number 71: Adhesion factor is RECORDED The status for the indication of the the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED. |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the balise (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. A cab needs to be active and the train data must be valid (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 71: Adhesion factor is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | The end of the adhesion area is passed.Distance defined by L_ADHESION in previous packet 71. As the previously sent new adhesion factor is no more valid | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Up-link telegram (track to train) (1). |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0). |
| N_PIG | 3 | FINITE VALUE | Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group. |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Value must be the same for the whole group of balises. To make sure the balise group is not considered as missed. ( |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0). |
| NID_PACKET | 8 | 71 | Packet number 71: Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | Defines the distance to the position, where the change of adhesions factor is taking place. |
| L_ADHESION | 15 | VALUE > 0 | There must be a length where the adhesion factor is valid. |
| M_ADHESION | 1 | 0 | Slippery rail” must be the value, in order for the on-board equipment to inform the driver |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Adhesion factor | 1 | non-slipery rail |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is no longer DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via Balise Group. The train stores and accepts a new adhesion factor send by a balise group if a level transition announcement to level 1 has been received on-board. |  | 4 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment stores and accepts a new relevant adhesion factor sent by a balise group. the ERTMS/ETCS on-board equipment informs the driver because the value of the adhesion factor is “slippery rail” |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: After the change of level,and when the D_ADHESION is reached, an indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED. Check on the JRU that: Eurobalise telegram including Packet number 71: Adhesion factor is RECORDED The status for the indication of the the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED. |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the balise (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Level Transition Order | 2 | L1 Level Transition to Level 1 |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 /LNTC | UN/ SN | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 71: Adhesion factor is received | I | BTM | - | L0 /LNTC | UN/ SN | - |
| 2 | L0 /LNTC | UN/ SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 /LNTC | UN/ SN | - |
| 3 | L0 /LNTC | UN/ SN | USE_FT5100100.15,23 | - | - | The onboard equipment changes to L1. The on-board will have to change to a different mode also. (Select the step to change to FS/SR/OS inside FT5100100 ) | L1 | FS/SR/OS | - |
| 4 | L1 | FS/SR/OS | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | L1 | FS/SR/OS | - |
| 5 | L1 | FS/SR/OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | L1 | FS/SR/OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Up-link telegram (track to train) (1). |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0). |
| N_PIG | 3 | FINITE VALUE | Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group. |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Value must be the same for the whole group of balises. To make sure the balise group is not considered as missed. |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0). |
| NID_PACKET | 8 | 71 | Packet number 71: Adhesion factor |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | Defines the distance to the position, where the change of adhesions factor is taking place. |
| L_ADHESION | 15 | VALUE > 0 | There must be a length where the adhesion factor is valid. |
| M_ADHESION | 1 | 0 (Slippery rail) | “Slippery rail” must be the value, in order for the on-board equipment to inform the driver |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 | FS / OS / SR |
| Adhesion factor | 0 | slippery rail |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via infill Balise Group. The train accepts a new adhesion factor sent by an infill balise group. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS / LS |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment accepts a new relevant adhesion factor sent by an infill balise group. the ERTMS/ETCS on-board equipment informs the driver because the value of the adhesion factor is “slippery rail”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: An indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED. Check on the JRU that: Eurobalise telegram including Packet number 71: Adhesion factor is RECORDED. The status of the indication of the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED. |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the balise (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. The linking reaction to the main balies = no reaction |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Including the main balise group |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 71: Adhesion factor, packet 136: infill location reference and packet 12: Level 1 Movement Authority is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Packet Number 136: infill location reference and Packet Number 12: Level 1 Movement Authority are also RECORDED but not relevant for the adhesion factor update. | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | IT ARRIVE TO THE MAIN SIGNAL LOCATION BUT , IT IS MISSED IN ORDER TO PROVE THE INFORMATION USED IS THE INFILL ONE LINKING REACTION= NO REACTION | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving | I | INT | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | The end of the adhesion area is passed.Distance defined by L_ADHESION in previous packet 71. As the previously sent new adhesion factor is no more valid | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Adhesion factor | 1 | non-slipery rail |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | BTM | The icon for the Slippery rail is no longer DISPLAYED. |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via RBC. The train stores rejects a new adhesion factor sent by the RBC in L0 /L NTC |  | 6 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / SB / PT/ LS L0: UN LNTC:, SN |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment rejects a new relevant adhesion factor sent by the RBC in level1, 0 or NTC |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: No indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Message 24: General message including Packet number 71: Adhesion factor is RECORDED |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the RBC (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71: Adhesion factor is received | I | RTM | The information is rejected. | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | - |
| 2 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | - |
| 3 | L0 / L1 / LNTC | FS / OS / SR / UN / LS / SN | Use_FT5100100.13,20,21,26,36,37 | - | - | The train changes to level 2/3. | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 4 | L2/ L3 | FS / OS / SR / UN / LS / SN | NOT The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION, | L2/ L3 | FS / OS / SR / UN / LS / SN | - |
| 5 | L2/ L3 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is NOT recorded | O | JRU | - | L2/ L3 | FS / OS / SR / UN / LS / SN | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | SLIPPERY RAIL |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 /12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | STORED | Same value than in the starting conditions. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is not DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via Balise Group. The train REJECTS a new adhesion factor that has been sent by a balise group in any level other than L1. |  | 7 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR L3: FS / LS / OS / SR L0: UN LNTC: SN |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment stores and rejects a new relevant adhesion factor sent by a balise group. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: No indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED. Check on the JRU that: Eurobalise telegram including Packet number 71: Adhesion factor is RECORDED |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the balise (Packet 71). If both values were the same, no change can be tested. No level transition order to L1 is stored |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 /LNTC/L2 /L3 | FS / OS / SR / UN / LS / SN | One Balise Group message (N_TOTAL=0) containing packet 71 is received | I | BTM | - | L0 /LNTC/L2 /L3 | FS / OS / SR / UN / LS / SN | - |
| 2 | L0 /LNTC/L2 /L3 | FS / OS / SR / UN / LS / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 /LNTC/L2 /L3 | FS / OS / SR / UN / LS / SN | - |
| 3 | L0 /LNTC/L2 /L3 | FS / OS / SR / UN / LS / SN | USE_FT5100100.15,23,28,32 | - | - | The train changes to level 1. | L1 | FS / OS / SR / LS / | - |
| 4 | L1 | FS / OS / SR / LS / | NOT The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is defined (after the level Transition). After the level trasition Defined by D_ADHESION. | L1 | FS / OS / SR / LS / | - |
| 5 | L1 | FS / OS / SR / LS / | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is NOT recorded | O | JRU | - | L1 | FS / OS / SR / LS / | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / LS |
| Adhesion factor | STORED | Same value than in the starting conditions. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | The train REJECTS a new adhesion factor that has been sent by the RBC (if the current level is L2/L3) or by a BG (if the current level is L1), because the current mode obliges to do it |  | 8 |
| Applicable Mode/Level Combinations | L1: PS / SH / SL / TR / PT / NL / RV L2 / L3: PS / SH / SL / TR / PT / NL / RV |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment rejects a new relevant adhesion factor because of the current mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: No indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Message 24: General message including Packet number 71: Adhesion factor is RECORDED, if the testing level is L2/L3 Eurobalise telegram including Packet number 71: Adhesion factor is RECORDED, if the testing level is L1 |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to be different from the value sent by the RBC or the BG (Packet 71). If both values were the same, no change could be tested.. In PT mode and L2/L3, the information ”Recognition of Exit from TR mode” with a more recent time stamp has not been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 8 / 11 / 14 / 15 | SH /SL / TR/ PT / NL / RV / PS |
| Radio communication session | ESTABLISHED | Only for testing in L2 / L3 |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | Only for testing in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71 is received | I | RTM | Only for L2 / L3 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2 / L3 | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message containing packet 71 is received | I | BTM | Only for L1 | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 5 | N/A | N/A | NOT The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid (after the level transition). After the level trasition Defined by D_ADHESION. | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is NOT recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | SLIPPERY RAIL |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | SLIPERY RAIL |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 8 / 11 / 14/ 15 | SH /SL / TR/ PT / NL / RV / PS |
| Radio communication session | ESTABLISHED | Only for testing in L2 / L3 |
| Adhesion factor | STORED | Same value than in the starting conditions. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only for testing in L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Adhesion factor |  | 4080402 |
| Test Case of Feature | Update of Adhesion Data via RBC. The train stores and accepts a new adhesion factor send by RBC in SB mode. |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of test is to check that: the ERTMS/ETCS on-board equipment accepts a new relevant adhesion factor send by RBC. the ERTMS/ETCS on-board equipment informs the driver because the value of the adhesion factor is “slippery rail” |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.18.4.6.2.3 |  |
|  |  | Subset-026-4.7.2 table - Output information Adhesion factor "slippery rail" |  |
|  |  | Subset-026-4.8.3.1.1 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 Adhesion factor |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.22 Packet Number 71: Adhesion factor |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: an indication of “slippery rail” (an icon) for the adhesion factor is DISPLAYED Check on the JRU that: Message 24: General message including Packet number 71: Adhesion factor is RECORDED The status for the indication of the value “slippery rail” of the adhesion factor that is DISPLAYED on the DMI is RECORDED. |
| Constraints | The adhesion factor (M_ADHESION) stored on-board (starting conditions) must be “non-slippery rail”, in order to differ from the factor send by the RBC (Packet 71). If both values were the same, no change can be tested. The train must move to reach the border where the adhesion factor changes. A cab needs to be active and the train data must be valid (SB Mode) after change of mode The train must move to reach the border where the adhesion factor changes. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | STORED | M_ADHESION = 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 71: Adhesion factor is received | I | RTM | - | L2/L3 | SB | - |
| 2 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | - | L2/L3 | SB | - |
| 3 | L2/L3 | SB | USE_FT4060300.48 | - | - | - | L2/L3 | FS | - |
| 4 | L2/L3 | FS | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The train reaches the beginning of the area where the new adhesion factor is valid. Defined by D_ADHESION. | L2/L3 | FS | - |
| 5 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | L2/L3 | FS | - |
| 6 | L2/L3 | FS | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | The end of the adhesion area is passed.Distance defined by L_ADHESION in previous packet 71. As the previously sent new adhesion factor is no more valid | L2/L3 | FS | - |
| 7 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | L2/L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 71 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_ADHESION | 15 | FINITE VALUE | - |
| L_ADHESION | 15 | FINITE VALUE | - |
| M_ADHESION | 1 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Adhesion factor | STORED | NON SLIPPERY RAIL |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Status symbol "Adhesion factor - slippery rail" is displayed | - | DMI | The icon for the Slippery rail is no longer DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |