| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3050400 (3.5.4: Maintaining a communication session) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3050400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 09.08.2013 | All | review | DLR |
| Version 2.9.2 21.11.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.4: Maintaining a communication session |  | 3050400 |
| Test Case of Feature | The on-board equipment tries to set-up a new safe connection but the connection cannot be established. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS / LS / SR / OS |  |  |
| Target of Test | The target of this test case is to check that : - in case of a loss of the safe radio connection with a RIU - on-board equipment shall immediately try to set-up a new safe radio connection - attempts shall be repeated, until the train passes the location indicated in the RIU order “Terminate the communication session", while the safe radio connection is NOT set-up and the session is NOT considered as terminated |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.4.3 |  |
|  |  | Subset-026-3.5.4.3 c) |  |
|  |  | Subset-026-3.5.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board equipment tries to set-up a new safe radio connection. Check on the DMI that : The driver is informed that no connection is established, since it has not been possible to set-up a new safe radio connection. |
| Constraints | A communication session with a RIU was established and the safe radio connection has been lost. The defined time to consider the session as established shall start as soon as EURORADIO has indicated the loss of the safe radio connection. The trackside has not ordered a disconnection. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 133 is received | I | BTM | The packet 133 contains the RIU order "Terminate the communication session" | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | Loss of the safe radio connection. A disconnection indication (primitive) is received on-board. The safe radio connection command is indicating the disturbance with a comment. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Second attempt: The on-board equipment tries to set-up a new safe radio connection. According to EURORADIO specifications at the latest when the connection establishment timer, started in Step 4, reaches its time-out value of 40 s. This step will be repeated a number of times. | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 s after loss of radio connection | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | The train passes the location indicated in packet 133 before 5 minutes since Step 4 have passed. The on-board equipment then stops to try to set-up a new radio connection. | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.4: Maintaining a communication session |  | 3050400 |
| Test Case of Feature | In case a message has to be sent during the loss of the safe radio connection, this message shall be considered as sent. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SH, TR, PT, SB, NL, RV, SL L2: FS, LS, OS, SR, SH, TR, PT, SB, NL, RV, SL L3: FS, LS, OS, SR, SH, TR, PT, SB, NL, RV, SL |  |  |
| Target of Test | The target of this test case is to check that if: - a message has to be sent during the loss of the safe radio connection - this message shall be considered as sent, i.e. no attempt to resent it shall take place |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.4.5 |  |
|  |  | Subset-026-3.5.4.1 |  |
|  |  | Subset-026-3.5.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: - Safe connection is lost - After reestablishment of safe connection, the on-board DOES NOT try to resend message 136. Check on the JRU that : - Message 136: “Train position report” is RECORDED |
| Constraints | - On-board equipment has just switched into one of the applicable modes - A communication session is already established with an RBC (L2/L3) or a RIU (L1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_F3050700.3 (The ERTMS/ETCS shall execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status if the “connection status” timer expires . Condition Id [2]. ) | - | - | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 is received | I | BTM | - | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The safe radio connection has been lost, but since no disconnection has been ordered, the communication session is still considered as established. (See chapter 3.5.4.1) The radio message cannot be transmitted. | N/A | N/A | - |
| 5 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Message is considered as sent. | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | When EURORADIO indicates the loss of the safe radio connection, the ERTMS/ETCS on-board equipment shall immediately try to set-up a new safe radio connection. (See chapter 3.5.4.2) | N/A | N/A | - |
| 9 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containin packet 0 is transmitted | O | RTM | The message "Train Position Report" (NID_MESSAGE=136) is considered as sent and is not sent again. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.4: Maintaining a communication session |  | 3050400 |
| Test Case of Feature | The on-board equipment tries to set-up a new safe connection and the connection is re-established. |  | 3 |
| Applicable Mode/Level Combinations | L1: LS, FS, OS L2: LS, FS, OS L3: LS, FS, OS |  |  |
| Target of Test | The target of this test is to check that: In Level 1 or 2 or 3, in case of a loss of the safe radio connection and if the trackside has not ordered a disconnection, the on-board equipment and the RBC consider the communication session still established for a defined time. (the defined time starts as soon as the EURORADIO has indicated the loss of the safe radio connection) and the on-board equipment immediately tries to set-up a new safe connection. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.4.1 |  |
|  |  | Subset-026-3.5.4.2 |  |
|  |  | Subset-026-3.5.4.3 |  |
|  |  | Subset-026-3.5.4.3 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : • The on-board equipment tries to set-up a new safe connection. |
| Constraints | A communication session is established and the safe radio connection has been lost. The trackside has not ordered a disconnection. Maintaining radio communication session with radio infill unit is also covered by this test case. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | The connection is lost. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The request is sent repeatedly by EURORADIO. No answer is received within the first 60s. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45s after loss of the safe radio connection. | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | The safe radio connection is set-up again | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1 and Bit41=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.4: Maintaining a communication session |  | 3050400 |
| Test Case of Feature | The on-board equipment tries to set-up a new safe connection but the connection cannot be established. |  | 4 |
| Applicable Mode/Level Combinations | L1: LS, FS, OS L2: LS, FS, OS L3: LS, FS, OS |  |  |
| Target of Test | The target of this test is to check that: In Level 1 or 2 or 3, in case of loss of the safe radio connection and if the trackside has not ordered a disconnection, the on-board equipment and the RBC consider the communication session still established and the on-board equipment tries to set-up a new safe radio connection. If the safe radio connection is not re-established after the defined time (as defined in A3.1), both, on-board equipment and trackside, shall consider the session as terminated, and the ERTMS/ ETCS on-board equipment shall inform the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.4.1 |  |
|  |  | Subset-026-3.5.4.2 |  |
|  |  | Subset-026-3.5.4.2.1 |  |
|  |  | Subset-026-3.5.4.3 |  |
|  |  | Subset-026-3.5.4.3 b) |  |
|  |  | Subset-026-A.3.1 - Maximum time to maintain a communication session in case of failed re-connection attempts |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The on-board equipment tries to set-up a new safe radio connection. Check on the DMI that : • The driver is informed that no connection is established, since it has not been possible to set-up a new safe radio connection. |
| Constraints | A communication session was established and the safe radio connection has been lost. The defined time to consider the session as established, shall start as soon as EURORADIO has indicated the loss of the safe radio connection. The trackside has not ordered a disconnection. Maintaining radio communication session with radio infill unit is also covered by this test case. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS /LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | The connection is lost. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The attempts for trying to re-establish the radio connection are repeated for five minutes (as defined in A3.1). So, this step will be repeated a number of times. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45s after loss of the safe radio connection. | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-CONNECT.Confirm is received | I | RTM | No answer. The ETCS Onboard tries to set-up a new safe radio connection. Five minutes have passed since the EURORADIO indicated the loss of the safe connection. The on-board equipment and trackside consider the communication session as terminated. The on-board equipment continues to try to set-up a new connection. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS /LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |