| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5120200 (5.12.2: The driver uses the same engine) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5120200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 12.11.2013 | All | Second version/ UNISIG Review from 03.10.2013 included; TCEditor_v28 tool used | INECO |
| Version 2.9.2 07.01.2014 | All | Third version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.12.2: The driver uses the same engine |  | 5120200 |
| Test Case of Feature | In level 2/3, the driver closes desk “A” but doesn´t switch the power off. The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk “B”. The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that : The end of mission procedure is executed on desk A closure. The start of mission procedure is executed on desk B opening The new train position data are calculated when the on-board equipment detects the change of direction. The announced and not used geographical references are deleted after the change of orientation. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.6.4.3 |  |
|  |  | Subset-026-5.12.2.5 |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Geographical Position related to the first track kilometre reference is displayed. Check on the JRU that : CAB A status is recorded The end of mission procedure is recorded. (triggered by transmission of radio message 150 ) CAB B status is recorded. Termination of the communication session is recorded ( messages 24 / 156 / 39 / 155 / 32 / 159 ) Start of mission procedure is recorded. Establishment of the communication session is recorded. New SoM train position is recorded. (Check also the special values in the position report of EoM and SoM) |
| Constraints | The train has passed over the first balise group. Information about Geographical position already stored. The driver uses the same engine. At the end of the TC the Geographical Position information related to the reference BG (1) is still applicable. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 8/ 11/ 12/ 14 | FS/ OS/ SR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | A radio communication session is established with the RBC |
| Geographical Position | STORED | NID_PACKET=79 Q_NEWCOUNTRY (1) =STORED NID_C (1) =STORED NID_BG (1) =STORED D_POSOFF (1) =STORED Q_MPOSITION (1) =STORED M_POSITION (1) =STORED N_ITER=1 Q_NEWCOUNTRY(2) =STORED NID_C(2) =STORED NID_BG(2) =STORED D_POSOFF(2) =STORED Q_MPOSITION(2) =STORED M_POSITION(2) =STORED NID_BG (1)= Geographical Position Reference Balise Group 1 NID_BG (2) = Geographical Position Reference Balise Group 2 |
| Train Position | VALID | Q_DLRBG=0 Q_DIRLRBG=0 Q_DIRTRAIN=1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Cab active | I | TIU | Desk A open |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | Driver closes Desk A. The on-board equipment switches to SB mode. | - | - | USE_FT4060300.8 (The on-board equipment switches to SB mode (Desk are closed (Transition [28])) | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The end of mission procedure is triggered. | - | - | USE_FT5050300.3 (Inform track side about end of mission, message 150) | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the RBC is terminated and the safe connection is released. | - | - | USE_ FT3050500.5 (Termination of communication session ordered by RBC) | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | Cab active | I | TIU | SB mode does NOT change to NP. The driver opens desk B: | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_B_STATUS=1) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | Start of mission procedure is triggered. | - | - | USE_FT5040300.8 | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | One Mobile Terminal is registered to a Radio Network, the driver revalidate the RBC-ID/phone number if its status is “invalid” or "valid". | - | - | USE_FT5040300.39 | L2 / L3 | SB | - |
| 8 | L2 / L3 | SB | The on-board equipment establishes a communication session with the RBC via RBC ID and RBC phone number: The radio message 155 (initiation of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 32 (configuration determination) is received from RBC and recorded in the JRU. The radio message 159 (session established) is sent by the on-board equipment to RBC and recorded in the JRU. The communication session with the RBC is established. | - | - | USE_FT3050300.23 (Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number ) | L2 / L3 | SB | - |
| 9 | L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; TRAIN_POSITION=<new current train position>) is recorded | O | JRU | The new train position data are calculated according to the new cab. (“When the driver changes the cab, the reference of the system is changed to the other cab in “train length” meters.) Check that the parameter TRAIN_POSITION of this radio message (sent by on-board unit to RBC every 5 seconds triggered by current speed update) has changed. The position changed by “train length” in meters. | L2 / L3 | SB | - |
| 10 | L2 / L3 | SB | SA-DATA. Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted | O | RTM | The on-board equipment sends its new position data to the RBC according to its new orientation (no acknowledgement from RBC is required) | L2 / L3 | SB | - |
| 11 | L2 / L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 12 | L2 / L3 | SB | The driver selects ‘Train Data Entry’. | - | - | USE_FT5040300.6 | L2 / L3 | SB | - |
| 13 | L2 / L3 | SB | The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is “invalid”. Radio communication session is established. | - | - | USE_FT5040300.4 | L2 / L3 | SB | - |
| 14 | L2 / L3 | SB | The driver selects START. | I | DMI | - | L2 / L3 | SB | - |
| 15 | L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 16 | L2 / L3 | SB | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | - | L2 / L3 | SB | - |
| 17 | L2 / L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 18 | L2 / L3 | SB | SA-DATA. Indication with Euroradio Message "MA with Shifted Location Reference" (NID_MESSAGE=33) containing packet 15, (MA), packet 21 (Gradient) and packet 27 (SSP) is received. | I | RTM | - | L2 / L3 | SB | - |
| 19 | L2 / L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 20 | L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | Triggering event: When the mode changes | L2 / L3 | FS | - |
| 21 | L2 / L3 | FS | The train is moving. | I | INT | - | L2 / L3 | FS | - |
| 22 | L2 / L3 | FS | One Balise Group message (N_TOTAL=1) containing packet 255 is received. | I | BTM | The train overpasses the second Geographical Position reference Balise Group (BG2). | L2 / L3 | FS | - |
| 23 | L2 / L3 | FS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 24 | L2 / L3 | FS | The driver presses "Geographical Position" | I | DMI | Only if "Geographical Position" is not yet displayed. The train has travelled the offset distance from the second Reference Balise Group | L2 / L3 | FS | - |
| 25 | L2 / L3 | FS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1110) is recorded | O | JRU | Only if "Geographical Position" is not yet displayed | L2 / L3 | FS | - |
| 26 | L2 / L3 | FS | The "Geographical Position" is displayed. | O | DMI | The Geographical Position related to the FIRST track kilometre reference is still be displayed as the Geographical Position related to the SECOND track kilometre reference was deleted after change of orientation. | L2 / L3 | FS | - |

| Step 10: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | SoM Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 01 | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 16: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 18: Radio Message 33 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 33 | MA with Shift Location Reference |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | Reference distance |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 22: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Geographical Position | STORED | Stored Geographical Position is related with BG1: NID_PACKET=79 Q_NEWCOUNTRY (1)=STORED NID_C (1)=STORED NID_BG (1)=STORED D_POSOFF (1)=STORED Q_MPOSITION (1)=STORED M_POSITION (1)=STORED N_ITER=0 The announced and not used geographical references have been deleted after the train changes orientation. |
| Train Position | VALID | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| CAB active | I | TIU | Desk B is opened on the same engine |
| The "Geographical Position" is displayed | O | DMI | The Geographical Position related to the first track kilometre reference is displayed and NOT the Geographical Position related to the Second track kilometre reference (the Second Geographical references was deleted after change of orientation). |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.12.2: The driver uses the same engine |  | 5120200 |
| Test Case of Feature | In level L0 /NTC/ 1, the driver closes desk "A". The on-board equipment executes the ‘end of mission’ procedure. The driver opens desk "B". The on-board equipment executes the ‘start of mission’ procedure. Same engine is used. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN LNTC : SN L1: FS/ LS/ OS/ SR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that : The end of mission procedure is executed on desk A closure. The start of mission procedure is executed on desk B opening |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.12.2.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: CAB A status is recorded CAB B status is recorded. Start of Mission procedure is recorded. |
| Constraints | Train is at standstill. The driver uses the same engine. MA, Track description and Train Data may be deleted (previous mode dependent…see Chapter 4, section 4.10) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/ 8/ 11/ 12/ 13/ 14 | FS/ OS/ SR/ UN/ PT/ NL/ LS/ SN/ RV |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | Desk A open |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ PT/ NL/ SN/ RV | Driver closes Desk A: The on-board equipment switches to SB mode. | - | - | USE_FT4060300.9 (The on-board equipment switches to SB mode (Desk are closed (Transition [28])) | L0 / LNTC/ L1 | SB | - |
| 2 | L0 / LNTC/ L1 | SB | CAB B active | I | TIU | The driver opens desk B: the start of mission procedure is triggered. | L0 / LNTC/ L1 | SB | - |
| 3 | L0 / LNTC/ L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_B_STATUS=1) is recorded | O | JRU | - | L0 / LNTC/ L1 | SB | - |
| 4 | L0 / LNTC/ L1 | SB | Start of mission procedure is triggered. | - | - | USE_FT5040300.8 (Driver ID is revalidated) | L0 / LNTC/ L1 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| CAB active | I | TIU | Desk B is opened on the same engine |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |