| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4041500 (4.4.15: NON LEADING) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-4041500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 23.09.2013 | All | review | DLR |
| Version 2.9.2 13.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.15: NON LEADING |  | 4041500 |
| Test Case of Feature | The train Onboard System shall use a “non leading input signal” from the train interface for entering to NL mode |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, SB LNTC: SH, SB L1: FS, LS, OS, SR, SH, SB L2: FS, LS, OS, SR, SH, SB L3: FS, LS, OS, SR, SH, SB |  |  |
| Target of Test | The train Onboard System shall use a “non leading input signal” from the train interface for entering to NL mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.15.1.1.2 |  |
|  |  | Subset-026-4.6.3 Condition Id [46] |  |
|  |  | Subset-026-4.4.15.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • non leading input signal is RECEIVED. Check on the DMI that: • Changing to NL mode is DISPLAYED. Check on the JRU that: • Changing to NL mode is RECORDED. |
| Constraints | - Use a non leading input signal from train interface |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 12 | FS / OS / SR / SH / SB / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | Train is at standstill (V_TRAIN=0) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | The driver selects “Main” | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | The <Main> window is displayed. | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | The Non Leading button in the Main window is disabled | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | Non-leading permitted | I | TIU | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | NON LEADING INPUT (NID_MESSAGE_JRU=32) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | The Non Leading button in the Main window is enabled | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 8 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | The driver selects "NON LEADING" | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 9 | L0 / LNTC / L1 / L2 / L3 | FS / LS / OS / SR / SH / SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 10 | L0 / LNTC / L1 / L2 / L3 | NL | The Mode symbol "Non-leading" is displayed | O | DMI | (Symbol MO12 is displayed) | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 11 | L0 / LNTC / L1 / L2 / L3 | NL | Train is moving (V_TRAIN=40) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 12 | L0 / LNTC / L1 / L2 / L3 | NL | No supervision information is displayed | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 13 | L0 / LNTC / L1 / L2 / L3 | NL | Service brake not commanded | O | TIU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 14 | L0 / LNTC / L1 / L2 / L3 | NL | Emergency brake not commanded | O | TIU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| indication of NL mode | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.15: NON LEADING |  | 4041500 |
| Test Case of Feature | If the “non leading input signal" is no longer present, the switch to Stand-By mode shall be made only if the train is at standstill. |  | 2 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL L2: NL L3: NL |  |  |
| Target of Test | Enter to SB mode, when the train Onboard System doesn’t receive a non leading input signal any more from train interface and train is at standstill |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.15.1.1.3 |  |
|  |  | Subset-026-4.6.3 Condition Id [47] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • non leading input signal is NOT RECEIVED. Check on the DMI that: • SB mode is DISPLAYED. Check on the JRU that: • Switch to SB mode is RECORDED. |
| Constraints | - Train in standstill - Non leading input signal is no longer present |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | NL | Train is at standstill (V_TRAIN=0) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | NL | Non-leading not permitted | I | TIU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | NL | NON LEADING INPUT (NID_MESSAGE_JRU=32) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | NL | The Mode symbol "Non-leading" is removed | O | DMI | (Symbol MO12 is NOT displayed) | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | NL | The Mode symbol "Stand By" is displayed | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| indication of SB mode | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.15: NON LEADING |  | 4041500 |
| Test Case of Feature | If the “non leading input signal" is no longer present, the switch to Stand-By mode shall be made only if the train is at standstill. |  | 3 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL L2: NL L3: NL |  |  |
| Target of Test | Enter NOT to SB mode, when the train Onboard System doesn’t receive a non leading input signal any more from train interface and train is NOT at standstill |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.15.1.1.3 |  |
|  |  | Subset-026-4.6.3 Condition Id [47] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train is NOT at standstill Check on the TIU that: • non leading input signal is NOT RECEIVED. |
| Constraints | - Train in NOT standstill - Non leading input signal is no longer present |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | NL | Train is moving (V_TRAIN=40) | I | INT | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | NL | Non-leading not permitted | I | TIU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | NL | NON LEADING INPUT (NID_MESSAGE_JRU=32) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | NL | NOT The Mode symbol "Non-leading" is removed | O | DMI | (Symbol MO12 is still displayed) | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | NL | NOT The Mode symbol "Stand By" is displayed | O | DMI | - | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | NL | Train is at standstill (V_TRAIN=0) | I | INT | After standstill the train losing the NL signal switches to SB. | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | SB | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6; ) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| indication of NL mode | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |