| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080410 (4.8.4: Accepted Information depending on the modes - Default Gradient for TSR) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-4080410 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 29.11.2013 | All | Review | CEDEX |
| Version 2.9.3 12.12.2013 | All | Review | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN L1: SR L2: SR L3: SR |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the the applicable level and mode. The message is ACCEPTED because a level transition is stored on-board. |  | 2 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in SN, when an announcement of L1/L2/L3 is stored on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | L1/L2/L3 are announced. |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | LNTC | SN | - |
| 2 | LNTC | SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | LNTC | SN | - |
| 3 | LNTC | SN | Train is at standstill (V_TRAIN=0; TIME= 10) | I | INT | - | LNTC | SN | - |
| 4 | LNTC | SN | The driver selects "Override EoA" | I | DMI | - | LNTC | SN | - |
| 5 | LNTC | SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded | O | JRU | - | LNTC | SN | - |
| 6 | LNTC | SN | Train is moving | I | INT | - | LNTC | SN | - |
| 7 | LNTC | SN | USE_FT4060300.TC52 Or USE_FT4060300.TC53 | - | - | TC52 for Level 1 and TC53 For Level 2/3 | L1 / L2 / L3 | SR | - |
| 8 | L1 / L2 / L3 | SR | USE_FT5100400.TC28 Or USE_FT5100400.TC31 Or USE_FT5100400.TC34 | - | - | TC28 for Level 1 TC31 for Level 2 TC34 for Level 3 | L1 / L2 / L3 | SR | - |
| 9 | L1 / L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the the applicable level and mode. The message is ACCEPTED. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, OS, LS L2: FS, OS, LS L3: FS, OS, LS |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in FS, OS and LS. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 /12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L1 / L2 / L3 | FS / OS / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / LS | USE_FT4060300.TC56 | - | - | - | L1 / L2 / L3 | SR | - |
| 4 | L1 / L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 4 |
| Applicable Mode/Level Combinations | L0: SB |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in L0: SB. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in SB requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| Train Data | STORED | Valid Train Data are stored on-board |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | SB | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L0 | SB | - |
| 2 | L0 | SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 | SB | - |
| 3 | L0 | SB | USE_FT5040300.27 AND USE_FT4060300.TC58 | - | - | - | L0 | UN | - |
| 4 | L0 | UN | USE_FT5080400.12 | - | - | This is for changing to L1/L2/L3 and SR mode. | L1 / L2 / L3 | SR | - |
| 5 | L1 / L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 5 |
| Applicable Mode/Level Combinations | L0: TR |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in L0: TR. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | TR | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L0 | TR | - |
| 2 | L0 | TR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 | TR | - |
| 3 | L0 | TR | USE_FT5110200.TC12 | - | - | - | L0 | UN | - |
| 4 | L0 | UN | USE_FT5080400.12 | - | - | This is for changing to L1/L2/L3 and SR mode. | L1 / L2 / L3 | SR | - |
| 5 | L1 / L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 6 |
| Applicable Mode/Level Combinations | L1: TR L2: TR L3: TR |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in L1/L2/L3: TR. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | TR | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L1 / L2 / L3 | TR | - |
| 2 | L1 / L2 / L3 | TR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | USE_FT5110200.TC13 | - | - | - | L1 / L2 / L3 | PT | - |
| 4 | L1 / L2 / L3 | PT | USE_FT4060300.TC49 OR USE_FT4060300.TC50 | - | - | TC50 for Level 1 and TC51 For Level 2/3 | L1 / L2 / L3 | SR | - |
| 5 | L1 / L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 7 |
| Applicable Mode/Level Combinations | L2: SB, PT L3: SB, PT |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in L2/3: SB, PT. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in SB or PT requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| Train Data | STORED | Valid Train Data are stored on-board |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB / PT | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L2 / L3 | SB / PT | - |
| 2 | L2 / L3 | SB / PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | SB / PT | - |
| 3 | L2 / L3 | SB / PT | USE_FT4060300.TC50 | - | - | - | L2 / L3 | SR | - |
| 4 | L2 / L3 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is ACCEPTED. |  | 8 |
| Applicable Mode/Level Combinations | L1: SB |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is accepted by the on-board in L1: SB. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the positive cases, where the information is accepted, FT3130233.TC2, calculating default gradient for TSR, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in SB or PT requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| Train Data | STORED | Valid Train Data are stored on-board |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | SB | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is ACCEPTED. | L1 | SB | - |
| 2 | L1 | SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | SB | - |
| 3 | L1 | SB | USE_FT4060300.TC49 | - | - | - | L1 | SR | - |
| 4 | L1 | SR | USE_FT3130233.TC2 | - | - | Use of default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is REJECTED because a level transition is NOT stored on-board. |  | 9 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is rejected by the on-board in SN, when no announcement of L1/L2/L3 is stored on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | NOT STORED | L1/L2/L3 are NOT announced. |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is REJECTED. | LNTC | SN | - |
| 2 | LNTC | SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | LNTC | SN | - |
| 3 | LNTC | SN | Train is at standstill (V_TRAIN=0; TIME= 10) | I | INT | - | LNTC | SN | - |
| 4 | LNTC | SN | The driver selects "Override EoA" | I | DMI | - | LNTC | SN | - |
| 5 | LNTC | SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded | O | JRU | - | LNTC | SN | - |
| 6 | LNTC | SN | Train is moving | I | INT | - | LNTC | SN | - |
| 7 | LNTC | SN | USE_FT4060300.TC52 Or USE_FT4060300.TC53 | - | - | TC52 for Level 1 and TC53 For Level 2/3 | L1 / L2 / L3 | SR | - |
| 8 | L1 / L2 / L3 | SR | USE_FT5100400.TC28 Or USE_FT5100400.TC31 Or USE_FT5100400.TC34 | - | - | TC28 for Level 1 TC31 for Level 2 TC34 for Level 3 | L1 / L2 / L3 | SR | - |
| 9 | L1 / L2 / L3 | SR | USE_FT3130233.TC3 | - | - | Use gradient=0 when no default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 / L2 / L3 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is REJECTED because the cabin is not active. |  | 10 |
| Applicable Mode/Level Combinations | L0: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is rejected by the on-board in SB when the cabin is not active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in SB requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2/ L3 | SB | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is REJECTED. | L0 / L1 / L2/ L3 | SB | - |
| 2 | L0 / L1 / L2/ L3 | SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 / L2/ L3 | SB | - |
| 3 | L0 / L1 / L2/ L3 | SB | Cab active | I | TIU | - | L0 / L1 / L2/ L3 | SB | - |
| 4 | L0 / L1 / L2/ L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | - | L0 / L1 / L2/ L3 | SB | - |
| 5 | L0 / L1 / L2/ L3 | SB | USE_FT4060300.TC58 OR USE_FT4060300.TC49 OR USE_FT4060300.TC50 | - | - | TC59 for Level 0, TC50 for Level 1 and TC51 for Level 2/3 | L0 / L1 / L2/ L3 | UN / SR | - |
| 6 | L0 / L1 / L2/ L3 | UN / SR | USE_FT3130233.TC3 | - | - | Use gradient=0 when no default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L0 / L1 / L2/ L3 | UN / SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is REJECTED because valid train data are not stored. |  | 11 |
| Applicable Mode/Level Combinations | L0: SB L1: SB L2: SB L3: SB |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is rejected by the on-board in SB when valid train data are not stored. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to UN or SR, depending on the level. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in SB requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| Train Data | NOT STORED | Valid Train Data are NOT stored. |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2/ L3 | SB | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is REJECTED. | L0 / L1 / L2/ L3 | SB | - |
| 2 | L0 / L1 / L2/ L3 | SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / L1 / L2/ L3 | SB | - |
| 3 | L0 / L1 / L2/ L3 | SB | USE_FT4060300.TC58 OR USE_FT4060300.TC49 OR USE_FT4060300.TC50 | - | - | TC59 for Level 0, TC50 for Level 1 and TC51 for Level 2/3. Within the transition from SB to UN / SR, the Train Data are entered/validated. | L0 / L1 / L2/ L3 | UN / SR | - |
| 4 | L0 / L1 / L2/ L3 | UN / SR | USE_FT3130233.TC3 | - | - | Use gradient=0 when no default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L0 / L1 / L2/ L3 | UN / SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 | SR / UN |
| Train Data | STORED | Valid Train Data are stored. |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Gradient for TSR |  | 4080410 |
| Test Case of Feature | The onboard device receives a message with a default gradient for temporary speed restriction in the applicable level and mode. The message is REJECTED. |  | 12 |
| Applicable Mode/Level Combinations | L1: PT |  |  |
| Target of Test | To verify that the default gradient for temporary speed restriction transmitted via Balise Group is rejected by the on-board in L1: PT. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 Default Gradient for TSR |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-7.4.2.37 Packet Number 141: Default Gradient for Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED.This functionality shall be checked through FT3130233, where the use of gradients for Target speed monitoring are evaluated. For the negative cases, where the information is rejected, FT3130233.TC3, using zero gradient for braking to a target, shall be used. For the particular starting conditions in this test case, prior to checking the use of the default gradient, it is necessary to perform a mode transition to SR. Check on the JRU that: The balise telegram with packet 141 RECEIVED from the Balise Group containing the temporary speed restriction information has been RECORDED. |
| Constraints | The reception of balise information in PT requires the train moving without an inmediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| National Values | STORED | D_NVROLL allows the train in SB to read a BG before applying the movement protection |
| Train Data | STORED | Valid Train Data are stored on-board |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | PT | One Balise Group message (N_TOTAL=1) containing packet 65 and packet 141 is received | I | BTM | The message is REJECTED. | L1 | PT | - |
| 2 | L1 | PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | PT | - |
| 3 | L1 | PT | USE_FT4060300.TC49 | - | - | - | L1 | SR | - |
| 4 | L1 | SR | USE_FT3130233.TC3 | - | - | Use gradient=0 when no default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring. | L1 | SR | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 141 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_TSR | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |