| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4040500 (4.4.5: SYSTEM FAILURE) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-4040500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 26.08.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.5: SYSTEM FAILURE |  | 4040500 |
| Test Case of Feature | Permanent application of emergency brakes in SF mode |  | 1 |
| Applicable Mode/Level Combinations | L0: SF L1: SF L2: SF L3: SF LNTC: SF |  |  |
| Target of Test | The target of this test case is to check that: - the emergency brake command is permanently transmitted from TIU to train as long as in SF mode - the emergency brake command is recorded in JRU - no emergency brake revocation is recorded in JRU as long as in SF mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.5.1.2 |  |
|  |  | Subset-026-4.5.2.1 table - Command Emergency Brake |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that - the “apply emergency brake”-command is permanently transmitted from TIU to train equipment (emergency brake) as long as in SF mode Check on the JRU that - the emergency brake command is RECORDED - no emergency brake revocation is RECORDED as long as in SF mode |
| Constraints | The SF mode is indicated on DMI. Mode changes to and from SF are not tested in the present feature. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 | SF |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "System Failure" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Emergency brake commanded | O | TIU | Immediatly after switching to SF mode. The emergency brakes are permanently applied while in SF mode. | N/A | N/A | - |
| 2 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | No emergency brake revocation is recorded in JRU as long as in SF mode. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 | SF |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The system status message "Emergency stop" is displayed The Mode symbol "System Failure" is displayed | O | DMI | The command “apply emergency brake” is permanently applied while in SF mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |