| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 6060400 (6.6.4 Handling of air gap data related to system version number X.Y = 2.0) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-6060400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.1 08.03.2016 | All | Baseline 3 2nd Release version | MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.4 Handling of air gap data related to system version number X.Y = 2.0 |  | 6060400 |
| Test Case of Feature | The on-board equipment with the system version number 2.1 send a translated "On-board supported system versions" to the RBC certified with X=2.0. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ NL |  |  |
| Target of Test | The target of this test is to check that: the transmission of "On-board supported system versions" (message 159) by the on-board equipment to the RBC certified with X=2.0 contains a packet 3 (instead of a packet 2) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.4.1.1 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 3 |  |
|  |  | Subset-026-6.6.3.4.5 [6] |  |
|  |  | Subset-026-6.6.4.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The establishment of a radio communication session initiated by the on-board equipment is recorded. |
| Constraints | Radio communication session not yet established. Only for SB mode: Train at standstill and A Cab is active . |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | VALID |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | NID_OPERATIONAL = NID_OPERATIONAL1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | 010 0000 | Message 32: “RBC/RIU System Version” | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | Versions are compatible Communication session established for on-board. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 3 is transmitted. | O | RTM | 010 0000 | Type of action according to 6.6.4.1.2: 6.6.3.4.1.1 shall apply with the translation referred in table 6.6.3.4.5:[U][6] | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Communication session established for trackside. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 5 is transmitted | O | RTM | 010 0000 | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | Message 136 with packet 5 is recorded. | N/A | N/A | - |

| Step 3: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | Configuration Determination |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 010 0000 | Version of the ERTMS/ETCS language. |

| Step 7: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_PACKET | 8 | 3 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_RADIO(k) | 64 | FINITE VALUE | - |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 0 or 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 5 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11/ 12 | FS/ OS/ SR/ SB/ NL/ LS |
| Radio communication session | ESTABLISHED | CHANGED |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |