| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110200 (3.11.2: Definition of Static Speed Restriction) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3110200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 17.07.2013 | All | Second version/ UNISIG review comments from 16/07added. | INECO |
| Version 2.9.2 10.12.2013 | All | Third version/Internal modification; TC_Editorv27; Correction of Requirements blank space; Additional remarks the author: Review sheet 005. | INECO |
| Version 2.9.3 26.12.2013 | All | Fourth version/ Internal version: Cross-reference error corrected. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.2: Definition of Static Speed Restriction |  | 3110200 |
| Test Case of Feature | Supervision of Static Speed Restriction depending on train length. (Q_FRONT shall be set to 0). |  | 1 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The location of the speed increase is supervised taking into account the min safe rear end of the train. The function SSP is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.2.4 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : In FS mode, the correct permitted speed has to be shown on the DMI depending on train position / In OS mode, the permitted speed depending on train length delay is shown on driver request. In LS mode, the DMI aspect coherent with Overspeed Status is displayed. |
| Constraints | Several Static Speed Restrictions are defined with speed increased. V_STATIC(1)< V_STATIC(2) The permitted speed after leaving SSP area must be higher than SSP permitted speed. The variable Q_FRONT stored on-board must be 0. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | Q_FRONT = 0 N_ITER ≠ 0 V_STATIC(2) > V_STATIC(1) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS V_STATIC(1) is displayed (In OS Permitted Speed Limit is only shown at drivers request) (In LS mode, V_STATIC should be lower than the LS mode speed and maximum train speed. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | Inside D_STATIC(1) | N/A | N/A | - |
| 2 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode, all the time a permitted speed equal to the Static Speed will be shown. V_TRAIN < V_STATIC(1) If in LS mode the train speed will be a bit over the V_STATIC(1). The DMI aspect is coherent with Overspeed Status. V_TRAIN>V_STATIC(1) | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TRAIN ><V_STATIC(1)>; M_SDMTYPE = 0; M_SDMSUPSTAT=2>) is recorded. | O | JRU | Optional Step. Applies only if the applicable mode is LS. M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving. | I | INT | The train reaches D_STATIC(2) with its safe front end | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode, all the time a permitted speed equal to the Static Speed will be shown. (V_TRAIN<V_STATIC(1)) If in LS mode the train speed will be a bit over the V_STATIC(1). The DMI aspect is coherent with Normal Status. V_TRAIN>V_STATIC(1) | N/A | N/A | - |
| 8 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM<V_STATIC(1)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving. | I | INT | D_STATIC(1) + full length of the train is reached The min safe rear end of the train passes the end of the lower static speed restriction. | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | If in FS or OS mode permitted speed equal to V_STATIC(2) is displayed. V_TRAIN< V_STATIC(2) The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section If in LS mode the DMI aspect is coherent with Overspeed status in a ceiling speed monitoring section. V_TRAIN>V_STATIC(2) | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = <V_STATIC(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2 ) is recorded. | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Check that V_PERM is updated with V_STATIC (2) Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| International SSP | STORED | Q_FRONT = 0 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | In FS or OS mode V_STATIC(2) is displayed. Normal status. In LS mode V_TRAIN > V_STATIC (2). Overspeed status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |