| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3150100 (3.15.1: RBC/RBC Handover) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-3150100 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/Creation | DLR |
| Version 2.9.0 31.03.2013 | All | Second version/review for Baseline 3 | INECO |
| Version 2.9.1 30.06.2013 | All | Third Version/ TC3 deleted; Format Template updated; Dictionary steps used; UNISIG review comments from 14.03.2013 included. | INECO |
| Version 2.9.2 12.06.4862 | All | Fourth version/Internal modification; TC_Editorv27; Correction of Requirements blank space; FT renumbered. | INECO |
| Version 2.9.3 26.12.2013 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC; Send a position report to the Handing Over RBC when the maximum safe front end of the train passes the given location and Send a position report to the Handing Over RBC when the minimum safe rear end of the train passes the given location. Safe Radio Connection Indication is displayed continuously. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR / SL / TR / NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives an announcement of the RBC/RBC handover. The on-board equipment establishes immediately a communication session with the Accepting RBC and sends its train data. A fixed test message is received from Accepting RBC, but is not attended until RBC handover is performed. The on-board equipment sends a position report to the Handing Over RBC and to the Accepting RBC with the use of the position report parameters valid for the Handing Over RBC. The on-board equipment sends a position report to the Handing Over RBC and to the Accepting RBC when the maximum safe front end of the train passes the location of the RBC transition. The Status symbol "Safe radio connection - Connection Up" is displayed continuously. Reception of new MAs from Handing Over and Accepting RBC. Only the MA received from the Accepting RBC is used. The train receives an order to execute the handover immediately from the balise group located at the RBC/RBC border. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The communication session is terminated on request of the Handing Over RBC. The Status symbol "Safe radio connection - Connection Up" is displayed continuously. The Fixed text message is displayed. The function “Manage RBC/RBC Handover (only level 2/3)” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.2.4 |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Manage RBC/RBC handover (only level 2/3) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: The Speed and Distance are updated on the DMI according to the Accepting RBC MA after the train has sent a position report to the Accepting RBC The safe radio connection indicated to the driver switches from the indication status of the safe radio connection with the Handing Over RBC to the one with Accepting RBC. “Connection Up” indication, symbol ST03 is DISPLAYED. The fixed text message is displayed after performing the RBC/RBC Handover Check on JRU that: Reception of radio message 24 with packet 131 from the Handing Over RBC is recorded. Establishment of the communication session with the Accepting RBC (exchange of radio message 155, 32, 159) is recorded Reception of radio message 24 with packet 76 from the Accepting RBC is recorded Transmission of radio message 129 (validated train data) with packet 0 and 11 to the Accepting RBC is recorded. Reception of radio message 8 from Accepting RBC is recorded. Transmission of radio message 136 with packet 0 to the Handing Over RBC and to the Accepting RBC is recorded. DMI symbol status of Safe Connection – Up is recorded. Reception of radio message 3 with packet 15 from Handing Over and Accepting RBC are recorded. The balise message containing the order to execute immediately the handover (packet 131) is recorded Transmission of message 136 to the Handing Over RBC is recorded. Reception of radio message 24 containing packet 42 from the Handing Over RBC is recorded. Terminating the communication session with the Handing Over RBC (exchange of radio message 156, 39) is recorded. DMI symbol status of Safe Connection – Up is recorded. Fixed text message is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | With regards to Handing Over RBC |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | from the Handing Over RBC D_RBCTR ≠ 0 The radio message contains a movement authority including the RBC transition border. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received | I | RTM | Packet is stored onboard till handover is finished. Accepting RBC is the sender of the Fixed text message. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | NOT "fixed text message" is displayed | O | DMI | At this location | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting) with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR | N/A | N/A | - |
| 23 | N/A | N/A | The "fixed text message" is displayed. | O | DMI | - | N/A | N/A | - |
| 24 | N/A | N/A | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded | O | JRU | - | N/A | N/A | - |
| 25 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 26 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 27 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting) when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 28 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 29 | N/A | N/A | The indication to the driver is the same. The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | The switch of the indication status of the safe radio connection from the Handing Over RBC with the Accepting RBC is done here. DMI Object: ST03 | N/A | N/A | - |
| 30 | N/A | N/A | The on-board is supervised by the Accepting RBC. Reception of an MA from the Accepting RBC | - | - | USE_FT4080425.3 Transmission of L2/L3 movement Authority from Accepting RBC | N/A | N/A | - |
| 31 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | MA is updated. | N/A | N/A | - |
| 32 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 33 | N/A | N/A | SA-DATA. Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | The on-board is no more supervised by the handing over RBC. The On-board will not use handing over MA’s | N/A | N/A | - |
| 34 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 35 | N/A | N/A | NOT The speed and distance monitoring information is displayed. | O | DMI | The MA sent by the Handing over RBC is not displayed | N/A | N/A | - |
| 36 | N/A | N/A | Balise Group message containing packet 131 is received | I | BTM | D_RBCTR = 0 The balise group containing the order to execute immediately the handover is placed at the RBC/RBC transition border. | N/A | N/A | - |
| 37 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 38 | N/A | N/A | Train is moving. | I | INT | Train min safe rear end location = LRBG + D_RBCTR The minimum safe rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 39 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To the Handing Over RBC | N/A | N/A | - |
| 40 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 41 | N/A | N/A | Termination of communication session ordered by RBC: The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |
| 42 | N/A | N/A | Train is moving. | I | INT | The RBC transition (handover) is performed | N/A | N/A | - |
| 43 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | The indication to the driver is the same. Safe Connection indication of Accepting RBC | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event condition. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0 |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 14: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 16: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 25: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 27: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 33: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 36: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | now |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting. RBC identity. |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 39: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| Fixed Text Information (location based) | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The Fixed Text message is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment shall establish the communication session with the Accepting RBC; Send a position report to the Handing Over RBC when the maximum safe front end of the train passes the given location and Send a position report to the Handing Over RBC when the minimum safe rear end of the train passes the given location when receiving an order to switch to another RBC at a given location. The balise message from the border balise group containing the order to execute immediately the handover (packet 131) is not (yet) received but the handover is still executed when the train with its max safe front end passes the border location. The on-board equipment is able to handle two communication sessions at the same time. |  | 2 |
| Applicable Mode/Level Combinations | L2/ L3: FS / LS / OS / SR / SL / TR / NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives an announcement of the RBC/RBC handover. The on-board equipment establishes immediately a communication session with the Accepting RBC and sends its train data. The on-board equipment receives a message (including a MA) from the Accepting RBC (before having sent any position report to the Accepting RBC see subset 026 - 3.15.1.3.6) and stores without using the information. Reception of an MA from the Accepting RBC but MA is not updated by the on-board equipment. The maximum safe front end of the train passes the location of the RBC transition. The on-board equipment sends a position report to both RBCs. The on-board equipment now uses the MA information received from the Accepting RBC. The on-board equipment receives a message (including a MA) from the Handing Over RBC and not uses this information. The balise message from the border balise group containing the order to execute immediately the handover (packet 131) is not (yet) received but the handover is still executed. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The communication session is terminated on request of the Handing Over RBC. Warning: It is possible that this Test Case could not be performed if an on-board sends the position reports to the Handing Over RBC and to the Accepting one so closely that it is not possible to send it a message from the Accepting RBC between the sending of the two position reports. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-5.15.1.4 |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: The Speed and Distance are updated on the DMI according to the Accepting RBC MA after the train has sent a position report to the Accepting RBC. Check on JRU that: The radio message 24 with packet 131 received from Handing Over RBC is recorded. The radio message 155 sent to the Accepting RBC is recorded. The radio message 32 received from the Accepting RBC is recorded. The radio message 159 sent to the Accepting RBC is recorded. The radio message 129 with packet 0 and 11 sent to the Accepting RBC is recorded. The radio message 8 sent by the Accepting RBC is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC and the Accepting RBC when the maximum safe front end of the train passes the location of the RBC transition is recorded. The balise message containing the order to execute immediately the handover (packet 131) is NOT (yet) recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition is recorded. Reception of radio message 24 containing packet 42 from the Handing Over RBC is recorded. Terminating the communication session with the Handing Over RBC (exchange of radio message 156, 39) is recorded. |
| Constraints | The on-board equipment is able to manage two communication sessions. Due to some on-board design, it could be not possible to perform this test case if it is not possible to send to the on-board an Accepting RBC message between the two on-board position reports. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | from the Handing Over RBC D_RBCTR ≠ 0 The radio message contains a movement authority including the RBC transition border. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 15 | N/A | N/A | The on-board is supervised by the Accepting RBC. Reception of an MA from the Accepting RBC. | - | - | USE_FT4080425. 3 Transmission of L2/L3 movement Authority from Accepting RBC | N/A | N/A | - |
| 16 | N/A | N/A | NOT. The speed and distance monitoring information is displayed. | O | DMI | MA is not updated because the on-board has not yet sent any position report to the Accepting RBC | N/A | N/A | - |
| 17 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting) | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | - | N/A | N/A | - |
| 23 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 24 | N/A | N/A | Transmission of L2/L3 movement Authority from Handing over RBC The on-board is no more supervised by the handing over RBC. The On-board will not use handing over MA’s | - | - | USE_ FT4080425.3 | N/A | N/A | - |
| 25 | N/A | N/A | NOT. The speed and distance monitoring information is displayed. | O | DMI | MA sent by the Handing over RBC | N/A | N/A | - |
| 26 | N/A | N/A | Train is moving. | I | INT | The balise group containing the order to execute immediately the handover and placed at the RBC/RBC transition border is not read.¡ (Balise Group message containing packet 131 is not read) Train min safe rear end location = LRBG + D_RBCTR The minimum rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 27 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To the Handing Over RBC | N/A | N/A | - |
| 28 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 29 | N/A | N/A | Termination of communication session ordered by RBC. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 27: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | One communication session is opened. The on-board receives an announcement of the RBC transition from the Handing Over RBC. It sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. At the transition border, no balise message containing an order to execute the handover immediately is received but the transition is done anyway. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It receives an order to terminate the communication session with the Handing Over RBC. As soon as the communication session with the Handing Over RBC is terminated due to crossing the border, it establishes a communication session with the Accepting RBC and it transmits its train data. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR / SL / TR / NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. The on-board accepts information from Handing Over RBC furthermore The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The on-board equipment continues with the transition although no balise message is received at the RBC border The communication session is terminated on request of the Handing Over RBC. The Status symbol "Safe radio connection - Connection Up" is removed. The on-board equipment shall wait until the session with the Handing over RBC is terminated due to crossing the border and then establishes immediately a communication session with the Accepting RBC and sends its train data. The Status symbol "Safe radio connection - Connection Up" is displayed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.15.1.4 |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 b) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The “Connection Up” indication, symbol ST03, is not displayed any longer when the safe connection is released with the Handing over RBC and is displayed again when the safe connection has been established with the Accepting RBC. Check on JRU that: The radio message 24 with optional packet 131 received from Handing Over RBC is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition is recorded. NOT balise message containing the order to execute immediately the handover (packet 131) is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition is recorded. The radio message 24 with packet 42 received from the Handing Over RBC is recorded. The radio message 156 sent to the Handing Over RBC is recorded. The radio message 39 received from the Handing Over RBC is recorded. The radio message 155 sent to the Accepting RBC is recorded. The radio message 32 received from the Accepting RBC is recorded. The radio message 159 sent to the Accepting RBC is recorded. The radio message 129 with packets 0 and 11 sent to the Accepting RBC is recorded. The radio message 8 received from the Accepting RBC is recorded |
| Constraints | The on-board equipment can handle only one communication session |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | With regards to Handing Over RBC |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received. | I | RTM | from the Handing Over RBC D_RBCTR ≠ 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving. | I | INT | The balise group containing the order to execute immediately the handover and placed at the RBC/RBC transition border is not read. Train location : Train min safe rear end location = LRBG + D_RBCTR | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the min safe rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | DMI Object: ST03 | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 24 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 25 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 16: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 18: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 20: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 22: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 24: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | One communication session is opened. The on-board receives an announcement of the RBC transition from the Handing Over RBC. It sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. At the transition border, it passes a balise group containing an order to execute the handover immediately. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It receives an order to terminate the communication session with the Handing Over RBC. As soon as the communication session with the Handing Over RBC is terminated due to crossing the border, it establishes a communication session with the Accepting RBC and it transmits its train data. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SL/ TR/ NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. The on-board accepts information from Handing Over RBC furthermore The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The communication session is terminated on request of the Handing Over RBC. The on-board equipment shall wait until the session with the Handing over RBC is terminated due to crossing the border and then establishes immediately a communication session with the Accepting RBC and sends its train data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-4.8.4.2 RBC Transition Order |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: The radio message 24 with optional packet 131 received from Handing Over RBC is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition is recorded. The balise message containing the order to execute immediately the handover (packet 131) is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition is recorded. The radio message 24 with packet 42 received from the Handing Over RBC is recorded. Reception of radio message 24 containing packet 42 from the Handing Over RBC is recorded. Terminating the communication session with the Handing Over RBC (exchange of radio message 156, 39) is recorded. The radio message 129 with packets 0 and 11 sent to the Accepting RBC is recorded. The radio message 8 received from the Accepting RBC is recorded |
| Constraints | The on-board equipment can handle only one communication session |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | from the Handing Over RBC D_RBCTR ≠ 0 The radio message contains a movement authority including the RBC transition border. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group message containing packet 131 is received. | I | BTM | The balise group containing the order to execute immediately the handover and placed at the RBC/RBC transition border is read. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving. | I | INT | Train location : Train min safe rear end location = LRBG + D_RBCTR | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the min safe rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |
| 12 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | now |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting. RBC identity. |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 14: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 16: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 18: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 20: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG 1 .No more balises should be overrun than the BG with RBC Transition Order |
| D_LRBG | 15 | FINITE VALUE | The train has passed the RBC transition border with its safe full length (> D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 22: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG1 |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board system |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment executes an End of mission procedure and is connected to two RBCs. It executes the End of mission procedure with both of them. |  | 5 |
| Applicable Mode/Level Combinations | L2: SH / SB L3: SH / SB |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment executes the End of mission with both RBCs The on-board system sends Message 150 ‘End of Mission’ to both RBCs. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.1.3.4.1 |  |
|  |  | Subset-026-5.5.3.1.3 |  |
|  |  | Subset-026-5.5.3.1.3 a) |  |
|  |  | Subset-026-5.6.2.2 A100 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.10 Message 150: End of mission |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: Both messages 150 ‘End of Mission’ are RECORDED. |
| Constraints | Communication Session between on-board and RBC1 and RBC2 is ESTABLISHED. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 | SH / SB |
| Radio communication session | ESTABLISHED | With Both RBCs |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | The on-board system sends this message to the RBC1 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 is transmitted | O | RTM | The on-board system sends this message to the RBC2 | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | -. Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| Step 3: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | -. Not: 4294967295 (unknown) |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | 0 | If Q_LENGTH = “Train integrity confirmed by integrity monitoring device” or “Train integrity confirmed by driver” |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 6 | SH / SB |
| Radio communication session | ESTABLISHED | With Both RBCs |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment can handle two communication sessions. The on-board equipment manages only one RBC/RBC transition at a time, therefore a new transition order replaces a previously received order. |  | 6 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SL/ TR/ NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives an announcement of the RBC/RBC transition from the Handing Over RBC (with D_RBCTR (1)) The on-board equipment establishes immediately a communication session with the Accepting RBC1 and sends its train data. Few meters afterwards another announcement of the RBC/RBC transition from the Handing Over RBC (with D_RBCTR (2)) is received by the on-board (D_RBCTR (2) grater then D_RBCTR(1)) The new transition order replaces the previous received one. The on-board equipment terminates the communication session with the Accepting RBC1 and starts to connect to the Accepting RBC2. When the train runs the distances equal to D_LRBG+ D_RBCTR (2) the RBC/RBC transition is performed. The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The communication session is terminated on request of the Handing Over RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.15.1.5 |  |
|  |  | Subset-026-3.5.3.5.2 |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: First radio message 24 with packet 131 received from Handing Over RBC is recorded. The on-board establishing a communication session with the Accepting RBC 1 and sends its train data is RECORDED. Second radio message 24 with packet 131 received from Handing Over RBC is recorded. The on-board equipment terminating the communication session with Accepting RBC 1 is RECORDED. The on-board establishing a communication session with the Accepting RBC 2 and sends its train data is RECORDED. RBC/RBC Transition performed at D_LRBG+D_RBCTR (2) is RECORDED. Both Position report messages are RECORDED. The termination of the communication session with the Handing Over RBC is RECORDED. |
| Constraints | The on-board equipment is able to manage two communication sessions The on-board equipment manages only one RBC/RBC transition at a time. D_RBCTR (2) > D_RBCTR(1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | The radio message contains a movement authority including the RBC transition border at a distance D_RBCTR (1) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC (1) | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC 1) | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 14 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting RBC (1)) with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | D_RBCTR(2) ≠ 0 and > D_RBCTR(1) The new transition order replaces the previous one. The connection order refers to another Accepting RBC (2) | N/A | N/A | - |
| 20 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted. | O | RTM | To Accepting RBC (1) | N/A | N/A | - |
| 22 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | From Accepting RBC (1) The on-board considers the communication session terminated with Accepting RBC1. | N/A | N/A | - |
| 24 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 25 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | To Accepting RBC (1) | N/A | N/A | - |
| 26 | N/A | N/A | SA-DISCONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 27 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | To Accepting RBC (2) | N/A | N/A | - |
| 28 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC (2) | N/A | N/A | - |
| 29 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 30 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 31 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 32 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 33 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 34 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC 2) | N/A | N/A | - |
| 35 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 36 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 37 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 38 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 39 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR(2) | N/A | N/A | - |
| 40 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Handing Over) when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 41 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 42 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To both RBCs (Accepting RBC (2)) when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 43 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 44 | N/A | N/A | The on-board is supervised by the Accepting RBC. Reception of an MA from the Accepting RBC | - | - | USE_FT4080425.3 Transmission of L2/L3 movement Authority from Accepting RBC(2) | N/A | N/A | - |
| 45 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | MA is updated. | N/A | N/A | - |
| 46 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 47 | N/A | N/A | The on-board is no more supervised by the handing over RBC. The On-board will not use handing over MA’s | - | - | USE_ FT4080425.3 Transmission of L2/L3 movement Authority from Handing over RBC | N/A | N/A | - |
| 48 | N/A | N/A | NOT. The speed and distance monitoring information is displayed. | O | DMI | The MA sent by the Handing over RBC is not displayed | N/A | N/A | - |
| 49 | N/A | N/A | Balise Group message containing packet 131 is received | I | BTM | D_RBCTR = 0 The balise group containing the order to execute immediately the handover is placed at the RBC/RBC transition border. | N/A | N/A | - |
| 50 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 51 | N/A | N/A | Train is moving. | I | INT | Train min safe rear end location = LRBG + D_RBCTR(2) The minimum safe rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 52 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To the Handing Over RBC | N/A | N/A | - |
| 53 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 54 | N/A | N/A | Termination of communication session ordered by RBC: The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |
| 55 | N/A | N/A | Train is moving. | I | INT | The RBC transition (handover) is performed | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC(1) identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC (1) radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 13: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 15: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 17: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (To Accepting RBC (1)) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 19: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 and > D_RBCTR(1) |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC(2) identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC(2) radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 21: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session with Accepting RBC1 |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 23: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 29: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 31: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 33: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 35: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 37: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 40: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to Handing Over RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 42: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to Accepting RBC (2)) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 49: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | now |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting. RBC identity. |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 52: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to Handing Over) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Accepting RBC 2 |
| Position Report parameters | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment can handle one communication session. The on-board equipment manages only one RBC/RBC transition at a time, therefore a new transition order replaces a previously received order. |  | 7 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SL/ TR/ NL |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment receives an announcement of the RBC/RBC transition from the Handing Over RBC (with D_RBCTR (1)) Few meters afterwards another announcement of the RBC/RBC transition from the Handing Over RBC (with D_RBCTR (2)) is received by the on-board (D_RBCTR(2) >D_RBCTR(1)) The new transition order replaces the previous received one. When the train runs the distances equal to D_LRBG + D_RBCTR(2) the RBC/RBC transition is performed. The on-board equipment sends a position report to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. The on-board accepts information from Handing Over RBC furthermore The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. The on-board equipment continues with the transition although no balise message is received at the RBC border The communication session is terminated on request of the Handing Over RBC. The on-board equipment shall wait until the session with the Handing over RBC is terminated due to crossing the border and then establishes immediately a communication session with the Accepting RBC and sends its train data. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.15.1.5 |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that: Both radio messages 24 with optional packet 131 received from Handing Over RBC are recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition, D_RBC (2) is recorded. The radio message 136 sent by the on-board equipment to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition, D_RBC (2) is recorded. The radio message 24 with packet 42 received from the Handing Over RBC is recorded. The radio message 156 sent to the Handing Over RBC is recorded. The radio message 39 received from the Handing Over RBC is recorded. The radio message 155 sent to the Accepting RBC is recorded. The radio message 32 received from the Accepting RBC is recorded. The radio message 159 sent to the Accepting RBC is recorded. The radio message 129 with packets 0 and 11 sent to the Accepting RBC is recorded. The radio message 8 received from the Accepting RBC is recorded |
| Constraints | The on-board equipment can handle only one communication session. D_RBCTR (2> D_RBCTR(1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | The radio message contains a movement authority including the RBC transition border at a distance D_RBCTR (1) D_RBCTR(1) ≠ 0 NID_RBC (1) NID_RADIO(1) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | The radio message contains a movement authority including the RBC transition border at a distance D_RBCTR (2) but the radio connection is with another RBC(2). D_RBCTR(2) ≠ 0 and >D_RBCTR (1) NID_RBC (2) NID_RADIO(2) This new transition order replaces the previous one. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving | I | INT | Train Location: Train maximum safe front end location = LRBG + D_RBCTR(2) | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving | I | INT | Train Location : Train min safe rear end location = LRBG + D_RBCTR(2) | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC when the minimum rear end of the train passes the location of the RBC transition | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | Termination of communication session ordered by RBC: The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | - | - | USE_FT3050500.5 | N/A | N/A | - |
| 12 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | To Accepting RBC (2) as soon as the safe connection with Handing over RBC is release. | N/A | N/A | - |
| 13 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC (2) | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC 2) | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. This step does not apply for SL or NL mode. | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | This step does not apply for SL or NL mode. | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | This step does not apply for SL or NL mode. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC(1) identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC(1) radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 and >D_RBCTR(1) |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC(2) identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC(2) radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR(2)-L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 14: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 16: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 18: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 20: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BG1.No more balises should be overrun than the BG with RBC Transition Order |
| D_LRBG | 15 | FINITE VALUE | The train has passed the RBC transition border with its safe full length >D_RBCTR +L_TRAIN + L_DOUBTOVER |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 22: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | BG1 |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL/ LS |
| Radio communication session | ESTABLISHED | The communication session is established with the Accepting RBC2 |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC. When the train front end passes the announced border the on-board shall substitute the current valid RBC ID/phone number with those of the Accepting RBC. After the substitution the on-board shall retain the RBC ID/phone number of the Handing Over RBC unit until at least Min safe rear end of the train has passed the border and the communication session with this Handing Over RBC has been terminated OR The RBC transition order is deleted according to A.3.4 or 4.10 |  | 8 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / OS / SR / LS |  |  |
| Target of Test | The target of this test is to check that : When the train front end passes the announced border the on-board substitutes the current valid RBC ID/phone number with those of the Accepting RBC After the substitution, the on-board retains the RBC ID/phone number of the Handing Over RBC unit the minimum safe rear end of the train passes the border and the communication session with this Handing Over RBC has been terminated or the RBC transition order is deleted (according to A.3.4 or 4.10) NOTE: It is not specified the status of the retained RBC ID/phone number of the Handing Over RBC after any of the before mentioned conditions is fulfilled, so the status is not checked in this TC. The function “Manage RBC/RBC Handover (only level 2/3)” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 f) |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.15.1.3.7 |  |
|  |  | Subset-026-3.15.1.3.8 |  |
|  |  | Subset-026-3.15.1.3.8 a) |  |
|  |  | Subset-026-3.15.1.3.8 b) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Manage RBC/RBC handover (only level 2/3) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: when the train maximum safe front end has not yet passed the RBC/RBC transition border the MA sent by the Accepting RBC is NOT USED (RBC ID/phone number of the Accepting RBC are not yet valid); when the train maximum safe front end passes the RBC/RBC transition border the MA previously received by the Accepting RBC is USED (RBC ID/phone number of the Accepting RBC become valid); when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border the MA sent by the Handing Over RBC is NOT USED (RBC ID/phone number of the Handing Over RBC are no more valid); Check on JRU that: when the train maximum safe front end has not yet passed the RBC/RBC transition border the MA sent by the Accepting RBC is RECORDED; when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border, the radio connection with the Handing Over RBC is lost and the on-board re-establishes the safe radio connection; the MA sent by the Handing Over RBC after the reconnection is RECORDED (RBC ID/phone number of the Handing Over RBC are no more valid but the connection with the Handing Over RBC is established again); Use this Test Case at the end of a Test Sequence: the On-board begins but does not complete the RBC handover within this Test Case |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Handing Over RBC Not in OS mode or SR mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | From the Handing Over RBC D_RBCTR ≠ 0 The radio message contains RBC transition order. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Accepting RBC with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | The On-board is supervised by the Handing Over RBC. The MA from the Accepting RBC is stored on-board till the Accepting RBC becomes the supervising RBC | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 24 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 25 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 26 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR | N/A | N/A | - |
| 27 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 28 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 29 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Accepting RBC when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 30 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 31 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | The switch of the indication status of the safe radio connection from the Handing Over RBC with the Accepting RBC is done here. The indication to the driver is the same. DMI Object: ST03 | N/A | N/A | - |
| 32 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | MA is updated using the previously received MA that was stored on-board | N/A | N/A | - |
| 33 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 34 | N/A | N/A | Train is at standstill | I | INT | Train location: train maximum safe rear end location < LRBG + D_RBCTR (the maximum safe rear end has not yet passed the RBC transition border) | N/A | N/A | - |
| 35 | N/A | N/A | Unplug signal cable or turn off the module for sending RBC messages | I | INT | t=t0 The target is to cause Radio connection loss | N/A | N/A | - |
| 36 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | - | N/A | N/A | - |
| 37 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step shall be repeated until t=t0+45. | N/A | N/A | - |
| 38 | N/A | N/A | Plug signal cable or turn on the module for sending RBC messages | I | INT | t>t0+45 The target is to allow Radio connection to be established. | N/A | N/A | - |
| 39 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 40 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | The on-board is no more supervised by the Handing Over RBC. The On-board will not use handing over MA’s | N/A | N/A | - |
| 41 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 42 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | DMI | The MA sent by the Handing over RBC is not used to update the speed and distance monitoring information | N/A | N/A | - |
| 43 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 8: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 10: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 12: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 14: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 16: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 22: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 27: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 29: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 40: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC. When an order to execute the RBC transition immediately is received the on-board shall substitute the current valid RBC ID/phone number with those of the Accepting RBC. After the substitution the on-board shall retain the RBC ID/phone number of the Handing Over RBC unit until at least Min safe rear end of the train has passed the border and the communication session with this Handing Over RBC has been terminated OR The RBC transition order is deleted according to A.3.4 or 4.10 |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / OS / SR / LS |  |  |
| Target of Test | The target of this test is to check that : When the on-board receives the order to execute the RBC transition immediately it substitutes the current valid RBC ID/phone number with those of the Accepting RBC After the substitution, the on-board retains the RBC ID/phone number of the Handing Over RBC unit the minimum safe rear end of the train passes the border and the communication session with this Handing Over RBC has been terminated or the RBC transition order is deleted (according to A.3.4 or 4.10) NOTE: It is not specified the status of the retained RBC ID/phone number of the Handing Over RBC after any of the before mentioned conditions is fulfilled, so the status is not checked in this TC. The function “Manage RBC/RBC Handover (only level 2/3)” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 f) |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.15.1.3.7 |  |
|  |  | Subset-026-3.15.1.3.8 |  |
|  |  | Subset-026-3.15.1.3.8 a) |  |
|  |  | Subset-026-3.15.1.3.8 b) |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Manage RBC/RBC handover (only level 2/3) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: when the on-board has not yet received the order to execute the RBC transition the MA sent by the Accepting RBC is NOT USED (RBC ID/phone number of the Accepting RBC are not yet valid); when the on-board receives the order to execute the RBC transition the MA previously received by the Accepting RBC is USED (RBC ID/phone number of the Accepting RBC become valid); when the train is at standstill after the on-board has received the order to execute the RBC transition and the minimum safe rear end has not yet passed the RBC/RBC transition border the MA sent by the Handing Over RBC is NOT USED (RBC ID/phone number of the Handing Over RBC are no more valid); Check on JRU that: when the on-board has not yet received the order to execute the RBC transition the MA sent by the Accepting RBC is RECORDED; when the train is at standstill after the on-board has received the order to execute the RBC transition and the minimum safe rear end has not yet passed the RBC/RBC transition border, the radio connection with the Handing Over RBC is lost and the on-board re-establishes the safe radio connection; the MA sent by the Handing Over RBC after the reconnection is RECORDED (RBC ID/phone number of the Handing Over RBC are no more valid but the connection with the Handing Over RBC is established again); Use this Test Case at the end of a Test Sequence: the On-board begins but does not complete the RBC handover within this Test Case |
| Constraints | The on-board equipment is able to manage two communication sessions. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Handing Over RBC Not in OS mode or in SR mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | From the Handing Over RBC D_RBCTR ≠ 0 The radio message contains RBC transition order. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Accepting RBC with the use of the position report parameters valid for the Handing Over RBC | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | The On-board is supervised by the Handing Over RBC. The MA from the Accepting RBC is stored on-board till the Accepting RBC becomes the supervising RBC | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 24 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 25 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 26 | N/A | N/A | Train is moving. | I | INT | Train location: train maximum safe front end location < LRBG + D_RBCTR (the maximum safe front end has not yet passed the RBC transition border) | N/A | N/A | - |
| 27 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR=0) is received | I | RTM | From the Handing Over RBC D_RBCTR=0 The radio message contains RBC transition order (now). | N/A | N/A | - |
| 28 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 29 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 30 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 31 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Accepting RBC when the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 32 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 33 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | The switch of the indication status of the safe radio connection from the Handing Over RBC with the Accepting RBC is done here. The indication to the driver is the same. DMI Object: ST03 | N/A | N/A | - |
| 34 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | MA is updated using the previously received MA that was stored on-board | N/A | N/A | - |
| 35 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 36 | N/A | N/A | Train is at standstill | I | INT | Train location: train maximum safe rear end location < LRBG + D_RBCTR (the maximum safe rear end has not yet passed the RBC transition border) | N/A | N/A | - |
| 37 | N/A | N/A | Unplug signal cable or turn off the module for sending RBC messages | I | INT | t=t0 The target is to cause Radio connection loss | N/A | N/A | - |
| 38 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | - | N/A | N/A | - |
| 39 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step shall be repeated until t=t0+45. | N/A | N/A | - |
| 40 | N/A | N/A | Plug signal cable or turn on the module for sending RBC messages | I | INT | t>t0+45 The target is to allow Radio connection to be established. | N/A | N/A | - |
| 41 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 42 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | The on-board is no more supervised by the Handing Over RBC. The On-board will not use handing over MA’s | N/A | N/A | - |
| 43 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 44 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | DMI | The MA sent by the Handing over RBC is not used to update the speed and distance monitoring information | N/A | N/A | - |
| 45 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 8: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 10: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 12: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 14: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 16: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 22: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 27: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 29: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 31: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report (to both RBC) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 42: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment is able to handle only one communication session at a time. When the maximum safe front end of the train passes the location of the RBC transition, the on-board equipment sends a position report to the Handing Over RBC. When the train front end passes the announced border the on-board shall substitute the current valid RBC ID/phone number with those of the Accepting RBC. After the substitution the on-board shall retain the RBC ID/phone number of the Handing Over RBC unit until at least Min safe rear end of the train has passed the border and the communication session with this Handing Over RBC has been terminated OR The RBC transition order is deleted according to A.3.4 or 4.10 The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It receives from the Handing Over RBC the order to terminate the communication session. As soon as the communication session with the Handing Over RBC is terminated, it establishes a communication session with the Accepting RBC and it transmits its train data. |  | 10 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / OS / SR / LS |  |  |
| Target of Test | The target of this test is to check that : When the train front end passes the announced border the on-board substitutes the current valid RBC ID/phone number with those of the Accepting RBC After the substitution, the on-board retains the RBC ID/phone number of the Handing Over RBC unit the minimum safe rear end of the train passes the border and the communication session with this Handing Over RBC has been terminated or the RBC transition order is deleted (according to A.3.4 or 4.10) NOTE: It is not specified the status of the retained RBC ID/phone number of the Handing Over RBC after any of the before mentioned conditions is fulfilled, so the status is not checked in this TC. The function “Manage RBC/RBC Handover (only level 2/3)” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 f) |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.15.1.3.7 |  |
|  |  | Subset-026-3.15.1.3.8 |  |
|  |  | Subset-026-3.15.1.3.8 a) |  |
|  |  | Subset-026-3.15.1.3.8 b) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 b) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Manage RBC/RBC handover (only level 2/3) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border, the safe radio connection with the Handing Over RBC is lost and then re-established; After the reconnection, the MA sent by the Handing Over RBC is USED (RBC ID/phone number of the Handing Over RBC are valid); When the minimum safe rear end has passed the RBC/RBC transition border and the connection to the Accepting RBC is set-up, the MA sent by it is USED (RBC ID/phone number of the Accepting RBC are valid); Check on JRU that: when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border, the radio connection with the Handing Over RBC is lost and the on-board re-establishes the safe radio connection; the messages exchanged during the safe radio connection re-establishment with the Handing Over RBC are RECORDED; the MA sent by the Handing Over RBC after the reconnection is RECORDED; the messages exchanged during the safe radio connection establishment with the Accepting RBC are RECORDED; the MA sent by the Accepting RBC is RECORDED. |
| Constraints | The on-board equipment is able to manage one communication session at a time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Handing Over RBC Not in OS mode or SR mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | From the Handing Over RBC D_RBCTR ≠ 0 The radio message contains RBC transition order.. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving. | I | INT | Train location: Train maximum safe front end location = LRBG + D_RBCTR The maximum safe front end of the train passes the location of the RBC transition | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is at standstill | I | INT | Train location: train maximum safe rear end location < LRBG + D_RBCTR (the maximum safe rear end has not yet passed the RBC transition border) | N/A | N/A | - |
| 10 | N/A | N/A | Unplug signal cable or turn off the module for sending RBC messages | I | INT | t=t0 The target is to cause Radio connection loss | N/A | N/A | - |
| 11 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step shall be repeated until t=t0+45. | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | DMI Object: ST04 | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=Bit41=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | Plug signal cable or turn on the module for sending RBC messages | I | INT | t>t0+45 The target is to allow Radio connection to be established. | N/A | N/A | - |
| 16 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | MA received by the Handing Over RBC | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | Train is moving. | I | INT | The balise group containing the order to execute immediately the handover and placed at the RBC/RBC transition border is not read. Train location : Train max safe front end location = LRBG + L_TRAIN + D_RBCTR | N/A | N/A | - |
| 22 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the min safe rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 23 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 24 | N/A | N/A | Use_FT3050500.5: Terminating a communication session | - | - | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | N/A | N/A | - |
| 25 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | N/A | N/A | - |
| 26 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=Bit40=0) is recorded | O | JRU | - | N/A | N/A | - |
| 27 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 28 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 29 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | DMI Object: ST03 | N/A | N/A | - |
| 30 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 31 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 32 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 33 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 34 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 35 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 36 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 37 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. | N/A | N/A | - |
| 38 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 39 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | - | N/A | N/A | - |
| 40 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 41 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | MA received by the Accepting RBC | N/A | N/A | - |
| 42 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 43 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 44 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 17: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 22: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 31: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 33: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 35: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 37: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 39: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 41: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The on-board equipment is able to handle only one communication session at a time. When an order to execute the RBC transition immediately is received the on-board shall substitute the current valid RBC ID/phone number with those of the Accepting RBC. After the substitution the on-board shall retain the RBC ID/phone number of the Handing Over RBC unit until at least Min safe rear end of the train has passed the border and the communication session with this Handing Over RBC has been terminated OR The RBC transition order is deleted according to A.3.4 or 4.10 The on-board equipment sends a position report to the Handing Over RBC when the minimum safe rear end of the train passes the location of the RBC transition. It receives from the Handing Over RBC the order to terminate the communication session. As soon as the communication session with the Handing Over RBC is terminated, it establishes a communication session with the Accepting RBC and it transmits its train data. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / OS / SR / LS |  |  |
| Target of Test | The target of this test is to check that : When the on-board receives the order to execute the RBC transition immediately it substitutes the current valid RBC ID/phone number with those of the Accepting RBC After the substitution, the on-board retains the RBC ID/phone number of the Handing Over RBC unit the minimum safe rear end of the train passes the border and the communication session with this Handing Over RBC has been terminated or the RBC transition order is deleted (according to A.3.4 or 4.10) NOTE: It is not specified the status of the retained RBC ID/phone number of the Handing Over RBC after any of the before mentioned conditions is fulfilled, so the status is not checked in this TC. The function “Manage RBC/RBC Handover (only level 2/3)” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 f) |  |
|  |  | Subset-026-3.15.1.3.1 |  |
|  |  | Subset-026-3.15.1.3.1 a) |  |
|  |  | Subset-026-3.15.1.3.1 b) |  |
|  |  | Subset-026-3.15.1.3.1 c) |  |
|  |  | Subset-026-3.15.1.3.2 |  |
|  |  | Subset-026-3.15.1.3.3 |  |
|  |  | Subset-026-3.15.1.3.4 |  |
|  |  | Subset-026-3.15.1.3.5 |  |
|  |  | Subset-026-3.15.1.3.6 |  |
|  |  | Subset-026-3.15.1.3.7 |  |
|  |  | Subset-026-3.15.1.3.8 |  |
|  |  | Subset-026-3.15.1.3.8 a) |  |
|  |  | Subset-026-3.15.1.3.8 b) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-3.5.7.6 |  |
|  |  | Subset-026-3.5.7.6 b) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Manage RBC/RBC handover (only level 2/3) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI: when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border, the safe radio connection with the Handing Over RBC is lost and then re-established; After the reconnection, the MA sent by the Handing Over RBC is USED (RBC ID/phone number of the Handing Over RBC are valid); When the minimum safe rear end has passed the RBC/RBC transition border and the connection to the Accepting RBC is set-up, the MA sent by it is USED (RBC ID/phone number of the Accepting RBC are valid); Check on JRU that: when the train is at standstill after the maximum safe front end has passed the RBC/RBC transition border and the minimum safe rear end has not yet passed the RBC/RBC transition border, the radio connection with the Handing Over RBC is lost and the on-board re-establishes the safe radio connection; the messages exchanged during the safe radio connection re-establishment with the Handing Over RBC are RECORDED; the MA sent by the Handing Over RBC after the reconnection is RECORDED; the messages exchanged during the safe radio connection establishment with the Accepting RBC are RECORDED; the MA sent by the Accepting RBC is RECORDED. |
| Constraints | The on-board equipment is able to manage one communication session at a time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Handing Over RBC |
| Position Report parameters | STORED | From Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Handing Over RBC Not in OS mode or SR mode |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; Request to show supervision limit=00011011b) is recorded | O | JRU | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | OPTIONAL STEP: Step mandatory only in OS Mode and in SR mode | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR≠ 0) is received | I | RTM | From the Handing Over RBC D_RBCTR ≠ 0 The radio message contains RBC transition order. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving. | I | INT | Train location: train maximum safe front end location < LRBG + D_RBCTR (the maximum safe front end has not yet passed the RBC transition border). | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR=0) is received | I | RTM | From the Handing Over RBC D_RBCTR=0 The radio message contains RBC transition order (now). | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the maximum safe front end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | Train is at standstill | I | INT | Train location: train maximum safe rear end location < LRBG + D_RBCTR (the maximum safe rear end has not yet passed the RBC transition border) | N/A | N/A | - |
| 12 | N/A | N/A | Unplug signal cable or turn off the module for sending RBC messages | I | INT | t=t0 The target is to cause Radio connection loss | N/A | N/A | - |
| 13 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step shall be repeated until t=t0+45. | N/A | N/A | - |
| 15 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | DMI Object: ST04 | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=Bit41=1) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | Plug signal cable or turn on the module for sending RBC messages | I | INT | t>t0+45 The target is to allow Radio connection to be established. | N/A | N/A | - |
| 18 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | MA received by the Handing Over RBC | N/A | N/A | - |
| 20 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 22 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | Train is moving. | I | INT | The balise group containing the order to execute immediately the handover and placed at the RBC/RBC transition border is not read. Train location : Train max safe front end location = LRBG + L_TRAIN + D_RBCTR | N/A | N/A | - |
| 24 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | To Handing Over RBC When the min safe rear end of the train passes the location of the RBC transition. | N/A | N/A | - |
| 25 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 26 | N/A | N/A | Use_FT3050500.5: Terminating a communication session | - | - | The on-board equipment receives an order to terminate the communication session from the RBC, it terminates the radio communication session: The radio message 24 (general message) containing an order to terminate the communication session (packet 42) is received from RBC and recorded in the JRU. The radio message 156 (termination of communication session) is sent by the on-board equipment to RBC and recorded in the JRU. The radio message 39 (acknowledgement of termination of communication session) is received from RBC and recorded in the JRU. The communication session with the Handing Over RBC is terminated and the safe connection is released. | N/A | N/A | - |
| 27 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | N/A | N/A | - |
| 28 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=Bit40=0) is recorded | O | JRU | - | N/A | N/A | - |
| 29 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 30 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | A safe connection is established with the Accepting RBC | N/A | N/A | - |
| 31 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | DMI Object: ST03 | N/A | N/A | - |
| 32 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 33 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 34 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 35 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) | N/A | N/A | - |
| 36 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 37 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 38 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside (Accepting RBC) | N/A | N/A | - |
| 39 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. | O | RTM | The radio message contains also position report data. | N/A | N/A | - |
| 40 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 41 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. | I | RTM | - | N/A | N/A | - |
| 42 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 43 | N/A | N/A | SA-DATA.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received | I | RTM | MA received by the Accepting RBC | N/A | N/A | - |
| 44 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 45 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 46 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | To avoid a message 146 |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 7: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 131 | RBC transition order |
| Q_DIR | 2 | FINITE VALUE | Valid for current train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | 0 | - |
| NID_C | 10 | FINITE VALUE | Valid for the train. |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC identity |
| NID_RADIO | 64 | FINITE VALUE | Accepting RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | 0 | Sleeping equipment don’t consider session management information (to avoid TIU influence) |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Max safe front end is considered (D_RBCTR - L_DOUBTUNDER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 19: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 24: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Min safe rear end is considered (D_RBCTR + L_TRAIN + L_DOUBTOVER) |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 33: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 35: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Compatible Version |

| Step 37: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 39: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | < D_RBCTR |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 41: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0+ t’ with t’ > 0 |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |

| Step 43: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | With Accepting RBC |
| Position Report parameters | STORED | From Handing Over RBC and from Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed The speed and distance monitoring information is displayed | O | DMI | With regards to Accepting RBC |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.1: RBC/RBC Handover |  | 3150100 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment has reported that the train has passed with its min safe rear end the announced border and no order to terminate the session is received from the Handing Over RBC within a fixed waiting time (see Appendix A.3.1) from the time the position report was sent. The ERTMS/ETCS on-board equipment send a position report with the fixed waiting time after each repetition and the order to terminate the session is received between the second and the third repetition. |  | 12 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment sends to the handing over RBC a radio message 136 “Train Position Report” indicating the min safe rear end has passed the announced border. No answer is received from the RBC. The on-board repeats two times each 15s the sending of a radio message 136 “Train Position Report” indicating the min safe rear end has passed the announced border. The answer is received from the RBC after the second repetition. The on-board equipment terminates the safe connection with Handing over RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 a) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-3.15.1.3.9 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |
|  |  | Subset-026-5.15.4.3 |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-8.6.4 Message 136: Train position report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The three radio messages 136 (Train Position Report) sent by the on-board are recorded with the correct timing. The radio message 24 (General message containing packet 42 (Q_RBC=0)) is recorded. The radio message 156 (Termination of a communication session) sent by the on-board is recorded. |
| Constraints | A communication session with the Handing Over RBC shall exist. Both cases can be given: One communication session or two communication sessions can be handled by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The min safe rear end passes the RBC transition border. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Position report message indicating the min safe rear end has passed the RBC transition border. To Handing Over RBC. | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | No answer is received from the Handing Over RBC and a time out occurs (15s) The first repetition of position report message indicating the min safe rear end has passed the RBC transition border. To Handing Over RBC. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | No answer is received from the Handing Over RBC and a time out occurs (15s) The second repetition of position report message indicating the min safe rear end has passed the RBC transition border. To Handing Over RBC. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Use_FT3050500.5: Terminating a communication session | - | - | Use FT3050500.5: the on-board equipment terminates the communication session as ordered by RBC | N/A | N/A | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DIRLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| Q_DLRBG | 2 | FINITE VALUE | Location data indicates that train has left the handing over RBC area. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | With Handing Over RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | With regards to Handing Over RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |