| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4041800 (4.4.18: REVERSING) Total: 10 Test case(s) |  |
| REF: | Subset-076-5-2-4041800 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 25.09.2013 | All | review | DLR |
| Version 2.9.2 03.12.2013 | All | review | DLR |
| Version 2.9.3 13.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | On-board supervision of distance allowed to reverse in reversing area |  | 1 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall supervise train movements against a maximum distance to run in the direction opposite to the trains orientation, given from track-side. • The fixed reference location being the end location of the area where reversing of movement is permitted at the time of reception of this reversing area information an emergency brake shall be commanded if over-passing this distance. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.1 |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-3.6.4.6 |  |
|  |  | Subset-026-3.15.4.8 |  |
|  |  | Subset-026-3.15.4.5 |  |
|  |  | Subset-026-4.4.18.1.3 |  |
|  |  | Subset-026-4.4.18.1.3 b) |  |
|  |  | Subset-026-4.4.18.1.5 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn for reversing distance exceeded |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The Emergency Brake is APPLIED when the train overpasses the end location of the maximum distance to run in the opposite direction by the train front end. Check on the DMI that: • Distance remaining to reverse movement is DISPLAYED. • Train speed is DISPLAYED • The Emergency Brake indication is DISPLAYED. Check on the JRU that : • The Emergency Brake intervention is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Position Report parameters | STORED | V_TRAIN≠0 (Train is running) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Train speed, permitted speed, remaining distance are DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | Permitted distance to run is passed. | N/A | N/A | - |
| 2 | N/A | N/A | The system status message "RV distance exceeded" is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Emergency brake commanded | O | TIU | The Emergency Brake is APPLIED. | N/A | N/A | - |
| 5 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Emergency Brake indication is DISPLAYED. | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Emergency Brake intervention is RECORDED. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Supervision Information | STORED | D_REVERSE=FINITE VALUE |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | The Emergency Brake is APPLIED. |
| The speed and distance monitoring information is displayed and The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Distance allowed to reverse movement is DISPLAYED. The Emergency Brake indication is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | The fixed reference location shall remain unchanged in case of an update of reverse movement distance. |  | 2 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: • The fixed reference location shall remain unchanged in case of an update of reverse movement distance. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.1 |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-3.15.4.2.1 |  |
|  |  | Subset-026-3.15.4.3 |  |
|  |  | Subset-026-3.15.4.5 |  |
|  |  | Subset-026-4.4.18.1.3 |  |
|  |  | Subset-026-4.4.18.1.3 b) |  |
|  |  | Subset-026-4.4.18.1.5 |  |
|  |  | Subset-026-4.4.18.1.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • Distance remaining to reverse movement is changed. • Train speed is DISPLAYED |
| Constraints | L_REVERSEAREA is long enough to include the D_REVERSE value of the updating reversing supervision information |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Position Report parameters | STORED | V_TRAIN≠0 (Train is running) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Train speed, permitted speed, remaining distance are DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train is running in RV mode and the permitted distance is displayed. The train ran the distance d0 (fixed location reference to current position) | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 139 is received | I | BTM | Only for L1 D_REVERSE (new) > D_REVERSE (old) | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for L1 | N/A | N/A | - |
| 4 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 139 is received | I | RTM | Only for L2/ L3 D_REVERSE (new) > D_REVERSE (old) | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for L2/ L3 | N/A | N/A | - |
| 6 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=14; D_LRBG=FINITE_VALUE) is recorded | O | JRU | Check that train position respects the train length (cab A/cab B is taken into account) | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The permitted distance to run in RV mode is updated. Permitted distance = D_REVERSE (new) – d0 | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Supervision Information | STORED | D_REVERSE=CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Distance allowed to reverse movement is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | The emergency brake is released due to the extension of the reversing distance. |  | 3 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: • The emergency brake is released at once when the reversing distance has been extended. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.1 |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-3.14.1.7.1 |  |
|  |  | Subset-026-3.15.4.3 |  |
|  |  | Subset-026-3.15.4.5 |  |
|  |  | Subset-026-4.4.18.1.3 |  |
|  |  | Subset-026-4.4.18.1.3 b) |  |
|  |  | Subset-026-4.4.18.1.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The Emergency Brake is RELEASED when the extension of the reversing area is received. Check on the DMI that: • Distance remaining to reverse movement is changed. • Emergency Brake indication is removed. Check on the JRU that: • Emergency Brake release is RECORDED |
| Constraints | The train is braking because it has overpassed the distance allowed to reverse in reversing area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Position Report parameters | STORED | V_TRAIN≠0 (Train is running) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT4041800.2 (The fixed reference location shall remain unchanged in case of an update of reverse movement distance.) | - | - | - | N/A | N/A | - |
| 2 | N/A | N/A | Emergency brake not commanded | O | TIU | The emergency brake is RELEASED | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The emergency brake indication is REMOVED | N/A | N/A | - |
| 4 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=0 (Release) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Supervision Information | STORED | D_REVERSE=CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake not commanded | O | TIU | The emergency brake is RELEASED |
| The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The emergency brake indication is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | The emergency brake is released at standstill after driver acknowledgment. |  | 4 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: • The emergency brake is released when the train is at standstill after the driver acknowledgment |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.7.1 |  |
|  |  | Subset-026-3.15.4.5 |  |
|  |  | Subset-026-4.4.18.1.3 |  |
|  |  | Subset-026-4.4.18.1.3 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The Emergency Brake is RELEASED after driver acknowledgement. Check on the DMI that: • An acknowledge is displayed to the driver • Emergency Brake indication is removed. Check on the JRU that: • Emergency Brake release is RECORDED |
| Constraints | The train is braking because it has overpassed the distance allowed to reverse in reversing area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | A request for driver acknowledgement to “permitted distance to run in RV mode overpassed” is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver acknowledges | I | DMI | The driver acknowledgement and standstill are required for brake release | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0000) is recorded | O | JRU | Driver acknowledgement is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | Emergency brake not commanded | O | TIU | The emergency brake is RELEASED | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The emergency brake indication is REMOVED | N/A | N/A | - |
| 7 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) or =0 (Release) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake not commanded | O | TIU | The emergency brake is RELEASED |
| The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The emergency brake indication is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | On-board supervision of reversing speed given by track-side. |  | 5 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment supervises train movements against a ceiling train speed for reversing area given by track-side as mode related speed restriction. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.7.1 |  |
|  |  | Subset-026-3.13.7.2 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.18.1.3 |  |
|  |  | Subset-026-4.4.18.1.3 a) |  |
|  |  | Subset-026-4.5.2.1 table - Ceiling Speed Monitoring only (no braking curve) based on MRSP |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The train speed shall be supervised and DISPLAYED until the ceiling allowed reversing speed. • Train speed is DISPLAYED • Permitted Speed is DISPLAYED. Check on the JRU that : • The current train speed is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 V_REVERSE=FINITE VALUE (Maximum speed in RV mode) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Permitted train speed is DISPLAYED. Current train speed is DISPLAYED. | N/A | N/A | - |
| 2 | N/A | N/A | The ERTMS/ETCS on-board equipment supervises train speed versus a mode related speed restriction given by trackside (V_REVERSE). | - | INT | Takes just a few milliseconds → No hour glass is shown | N/A | N/A | - |
| 3 | N/A | N/A | V_TRAIN > V_MRSP + dV_warning(V MRSP) | I | INT | The train speed exceeds the Warning supervision limit | - | - | - |
| 4 | N/A | N/A | The Warning Status information is activated | O | DMI | While the Warning Status information is active, the audible information S2 shall be played. | - | - | - |
| 5 | N/A | N/A | V_TRAIN < V_MRSP + dV_warning(V MRSP) | - | INT | The train speed goes below the Warning supervision limit | N/A | N/A | - |
| 6 | N/A | N/A | The Warning Status information is dectivated | - | INT | The driver decelerates and the Warning Indication is REMOVED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Supervision Information | STORED | V_REVERSE=FINITE VALUE (Maximum speed in RV mode) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | Train speed lower than or equal to permitted train speed is DISPLAYED. Permitted train speed is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | Possibility for RBC to send a new distance to run, new maximum speed and new reference location in RV mode when L2 or L3 is active. |  | 6 |
| Applicable Mode/Level Combinations | L2: RV L3: RV |  |  |
| Target of Test | The target of this test case is to check that: - the on-board equipment receives, accepts and indicates - a new distance allowed to run in RV mode and - and a new maximum speed and - a new reference location for the new permitted distance to run |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that - the remaining distance to run is displayed (target distance) - the displayed target distance changes after receiving a new distance allowed to run in RV mode. - the new maximum speed is displayed - the new reference location for the new permitted distance to run is displayed Check on the JRU that - the received packet 139 is recorded. |
| Constraints | Value of new transmitted variables D_REVERSE and V_REVERSE are not equal to the previous stored value but greater than the previous. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE is valid |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Reversing" is displayed The Level symbol "Level <L2 / L3>" is displayed The speed and distance monitoring information is displayed | O | DMI | Current level and mode are DISPLAYED Target distance is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 138 and packet 139 is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The on-board equipment accepts the new distance to run in RV mode and the indication changes to the new target distance (the previous distance is overwritten). Check that the new distance uses as reference location the end of the reversing area given by packet 138. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | Reversing area information New reference location. The new reference location is the end of the new reversing area given by this packet 138 that the onboard will use only for the purpose of distance referencing. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | Reversing supervision information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | value not equal to the previous stored value but greater than the previous |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | STORED | - |
| Reversing Supervision Information | STORED | D_REVERSE is valid (“new” value) and refers to the new reference location V_REVERSE is valid (“new” value). |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Reversing" is displayed The Level symbol "Level <L2 / L3>" is displayed The speed and distance monitoring information is displayed | O | DMI | Current level and mode are DISPLAYED New target distance is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | Application of the emergency brake after an overpassed reversing distance, and while the reversing distance is still overpassed. |  | 7 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: After the release of a brake command initiated due to an overpassed reversing distance, and while the reversing distance is still overpassed, the ERTMS/ETCS on-board equipment shall command the emergency brake for any further movement in the direction opposite to the train orientation. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.4.18.1.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • The Emergency Brake is commanded Check on the DMI that: • The Emergency Brake indication is DISPLAYED. Check on the JRU that : • The Emergency Brake intervention is RECORDED |
| Constraints | A brake command initiated due to an overpassed reversing distance has been released for example by the execution of a TC testing requirement 3.14.1.7.1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 D_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train starts moving. Movement in the direction opposite to the train orientation. The train exceeds allowed distance to run in the direction opposite to the train orientation. | N/A | N/A | - |
| 2 | N/A | N/A | Emergency brake commanded | O | TIU | The Emergency Brake is APPLIED | N/A | N/A | - |
| 3 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The emergency brake indication is DISPLAYED. | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | The Emergency Brake intervention is RECORDED. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | - |
| Reversing Supervision Information | STORED | D_REVERSE=FINITE VALUE |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | The Emergency Brake is APPLIED. |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Emergency Brake indication is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | In case the SBI supervision limit is exceeded, the ERTMS/ETCS on-board equipment shall command the emergency brake instead of the service brake. |  | 8 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | The target of this test is to check that: In case the SBI supervision limit is exceeded (refer to chapter 3 table 5, triggering condition t4), the ERTMS/ETCS on-board equipment shall command the emergency brake instead of the service brake. For the revocation of the brake command, refer to 3.13.10.2.4. Only triggering condition t4 of requirement 3.13.10.3.3 table 5 is tested in this TC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.18.1.6 |  |
|  |  | Subset-026-3.13.10.3.3 table 5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • Emergency brake command is TRIGGERED. Check on the DMI that: • The Emergency Brake indication is DISPLAYED. Check on the JRU that : • The Emergency Brake intervention is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 V_REVERSE=FINITE VALUE |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | - | INT | SBI supervision speed limit is exceeded | N/A | N/A | - |
| 2 | N/A | N/A | Emergency brake commanded | O | TIU | The Emergency brake is TRIGGERED. Although only the SBI speed limit has been exceeded. | N/A | N/A | - |
| 3 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | M_BRAKE_COMMAND_STATE=1 (Command) | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Emergency brake indication is DISPLAYED. | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 V_REVERSE=FINITE VALUE |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | The Emergency Brake is APPLIED. |
| The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The Emergency Brake indication is DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | The alarm which indicates a malfunction for on-board balise transmission function shall be ignored from on-board unit. |  | 9 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | In case there is an alarm reporting a malfunction for the onboard balise transmission function, the ERTMS/ETCS onboard equipment shall ignore this alarm. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.18.1.11 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that: • Service brake is not commanded |
| Constraints | The train is in RV mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train overpasses a metal bridge. The On-board Transmission Equipment is allowed to give an alarm to the ERTMS/ETCS Kernel (Subset-036-6.2.1.7). | N/A | N/A | - |
| 2 | N/A | N/A | Service brake not commanded | O | TIU | The on-board ignores the alarm. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.4.18: REVERSING |  | 4041800 |
| Test Case of Feature | If train in RV mode and system version number transmitted by any balise is greater than the highest version number supported by the on-board unit, the information from the balise shall be ignored, the train shall not be tripped, the driver shall not be informed. |  | 10 |
| Applicable Mode/Level Combinations | L1: RV L2: RV L3: RV |  |  |
| Target of Test | In case the ERTMS/ETCS system version number X transmitted by any balise is greater than the highest version X supported by the on-board equipment (refer to 3.17.3.5), the information from this balise shall be ignored, the train shall not be tripped and the driver shall not be informed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.18.1.12 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • No trip symbol is displayed Check on the JRU that : • Version number transmitted by balise is RECORDED. |
| Constraints | The train is in RV mode |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 72 packet 255 is received | I | BTM | The train overpasses a balise in reverse direction and a telegram with a text message from balise is transmitted to train. The highest supported system version by the onboard unit is lower than the system version number in the balise telegram. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | The train does not switch to TR mode. The driver is not informed about system version number error. | N/A | N/A | - |
| 4 | N/A | N/A | NOT Emergency brake commanded | O | TIU | The train does not apply the emergency brake | N/A | N/A | - |
| 5 | N/A | N/A | NOT The "plain text message" is displayed | O | DMI | The information from the balise is ignored by onboard, i.e. the text message is NOT displayed | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/N) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | 011 0000 | Version of the ERTMS/ETCS system Version 3.0 |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | N | Total number of balises in the balise group N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Reversing Area Information | STORED | Packet 138 |
| Reversing Supervision Information | STORED | Packet 139 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |