| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4090100 (4.9.1: What happens to accepted and stored information when entering a given level) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4090100 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 24.09.2013 | All | Second version/ UNISIG Review from 03.09.2013 included; TCEditor_v29 tool used; FT renumbered. | INECO |
| Version 2.9.2 08.12.2013 | TC2 | Third version/ UNISIG Review sheet from 5.12.2013 included; TC2- applicable modes affected. | INECO |
| Version 2.9.3 02.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.9.1: What happens to accepted and stored information when entering a given level |  | 4090100 |
| Test Case of Feature | After a level transition to level 1, the status of the stored information changes to unknown. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that : In level 2/ 3 the train has this information stored on-board : MA Request Parameters (packet 57 where variable T_CYCRQST<255), Position Report Parameters (packet 58 where the variable T_CYCLOC: 1 to 254) and Track Ahead Free Request (Message 34) The on-board has performed a level transition to Level 1 Once in level 1, all the information stored on-board is DELETED. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.9.1.3 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: At the location indicated in Message 34 (D_TAFDISPLAY and L_TAFDISPLAY) NO message is DISPLAYED to the driver. Check on the JRU that : Level transition to L1 is RECORDED. Message 136 is RECORDED. Message “MA request” is NOT recorded after T_CYCRQST expires. Message “Position Report Parameters” is NOT recorded after T_ CYCLOC expires. |
| Constraints | The level 2/3 MA stored on-board shall cover at least the first section of the level 1 area. A Level transition order is stored and train has reached the location indicated by the Track Ahead Free Request |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| MA Request Parameters | STORED | - |
| Position Report parameters | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Track Ahead Free Request | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS | The train just entered in Level 1. | - | - | USE_FT5100100.32 for level transition L2 to L1 Or USE_FT5100100.28 for level transition L3 to L1 | L1 | FS / LS / OS | - |
| 2 | L1 | FS / LS / OS | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The position refers to the front end of the respective engine with regards to the train orientation. | L1 | FS / LS / OS | - |
| 3 | L1 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 4 | L1 | FS / LS / OS | NOT SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | After the T_CYCRQST expires the on-board does NOT send any message MA request. | L1 | FS / LS / OS | - |
| 5 | L1 | FS / LS / OS | NOT SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | After the T_ CYCLOC expires the on-board does NOT send any message Position Report Parameters. | L1 | FS / LS / OS | - |
| 6 | L1 | FS / LS / OS | NOT The Driver Request symbol "Track Ahead Free" is displayed | O | DMI | When the train reaches the location indicated by the Track Ahead Free Request the on-board does NOT display a request to the driver. | L1 | FS / LS / OS | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | BGa / SBGa |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| MA Request Parameters | NOT STORED | - |
| Position Report parameters | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Track Ahead Free Request | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.9.1: What happens to accepted and stored information when entering a given level |  | 4090100 |
| Test Case of Feature | In case of entering level 0, NTC or 1, the information “Inhibition of revocable TSRs from balises in L2/L3” shall be deleted. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: If on-board equipment enters level 0, NTC or 1, the information “Inhibition of revocable TSRs from balises in L2/L3” is deleted. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.9.1.3.1 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: At the location indicated in packet 65, Permitted speed is equal to V_TSR and this speed is DISPLAYED to the driver. In LS mode the DMI aspect coherent with Overspeed status is displayed. Check on the JRU that: Level transitions are recorded. Telegram containing packet 65 is RECORDED. |
| Constraints | The information “Inhibition of revocable TSRs from balises in L2/L3” is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Inhibition of revocable TSRs from balises in L2/3 | STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L2 / L3>" is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS/ LS/ OS/ SR | The train just entered in Level 1 / Level NTC / Level 0. | - | - | USE_FT5100100.32 for level transition L2/L1 Or USE_FT5100100.30 for level transition L2/LNTC Or USE_FT5100100.11 for level transition L2/L0 Or USE_FT5100100.28 for level transition L3/L1 Or USE_FT5100100.25 for level transition L3/LNTC Or USE_FT5100100.29 for level transition L3/L0 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR | - |
| 2 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR | The Train is inside L0/LNTC/ L1 area. | I | INT | The information “Inhibition of revocable TSRs from balises in L2/L3” is deleted. | L0/ LNTC/ L1 | FS/ LS/ OS/ SR | - |
| 3 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR | The train enters in Level 2/ Level 3 from Level 1 / Level NTC / Level 0 | - | - | USE_FT5100100.37 for level transition L1/L2 Or USE_FT5100100.21 for level transition LNTC/L2 Or USE_FT5100100.13 for level transition L0/L2 Or USE_FT5100100.36 for level transition L1/L3 Or USE_FT5100100.20 for level transition LNTC/L3 Or USE_FT5100100.26 for level transition L0/L3 | L2 / L3 | FS/ LS/ OS/ SR | - |
| 4 | L2/ L3 | FS/ LS/ OS/ SR | One Balise Group message containing packet 65 is RECEIVED | I | BTM | Packet 65: The value of V_TSR < V_STATIC | L2 / L3 | FS/ LS/ OS/ SR | - |
| 5 | L2/ L3 | FS/ OS/ SR/ LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS/ LS/ OS/ SR | - |
| 6 | L2/ L3 | FS/ OS/ SR/ LS | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Apply only if the applicable modes are OS or SR. | L2 / L3 | FS/ LS/ OS/ SR | - |
| 7 | L2/ L3 | FS/ LS/ OS/ SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded | O | JRU | Apply only if the applicable modes are OS or SR. | L2 / L3 | FS/ LS/ OS/ SR | - |
| 8 | L2/ L3 | FS/ LS/ OS/ SR | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode: V_TRAIN<V_TSR The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section . Permitted speed is updated with V_TSR. If in LS mode: V_TRAIN > V_TSR The DMI aspect is coherent with Overspeed status in a ceiling speed monitoring section. | L2 / L3 | FS/ LS/ OS/ SR | - |
| 9 | L2/ L3 | FS/ LS/ OS/ SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<same as V_TSR> km/h; M_SDMTYPE= 0; M_SDMSUPSTAT=0 / 2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Check that V_PERM is updated with V_TSR Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | L2 / L3 | FS/ LS/ OS/ SR | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | - |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | - |
| Inhibition of revocable TSRs from balises in L2/3 | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode V_PERMITTED = V_TSR. V_TRAIN < V_TSR. Normal status. If in LS mode V_TRAIN > V_TSR. Overspeed status. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |