| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3150800 (3.15.8 Cold Movement Detection) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-3150800 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New section in Baseline 3 | INECO |
| Version 2.9.1 19.07.2013 | All | Second version/UNISIG review remarks from 01/07/2013 included; TC7 created new; | INECO |
| Version 2.9.2 01.08.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.3 17.12.2013 | All | Fourth version/ Requirements related with chapter 7 and 8 added. | INECO |
| Version 2.9.4 26.12.2013 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off no Cold movement occurred. When powered up again (after 72 hours max) the on-board equipment updates the status of information stored on-board. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test is to check that: If no CM has occurred the on-board equipment qualifies the train position, ETCS level, RBC ID Phone number and Table of trackside supported levels as valid when switched on again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - No Cold movement occurred |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.15 Message 157: SoM Position Report |  |
|  |  | Subset-026-4.10.1.3 Line Table of Priority of trackside supported levels |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: Radio communication session is established. Check on the DMI that: Table of trackside supported levels is valid. Check on the JRU that: The variable NID_LRBG ≠ 16777215 (Unknown). The variable M_LEVEL ≠ 5-7. The variable NID_RBC = last RBC number. The variable NID_RADIO = last Radio number. |
| Constraints | ERTMS/ETCS on-board unit is powered down. Cold movement detection is available. Information from packets 41 and 42 shall be accepted before entering in NP. Train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | INVALID | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Table of priority of trackside supported levels | INVALID | STORED (Packet 41 accepted before entering in NP mode) |
| Radio Network ID | INVALID | STORED |
| RBC ID/Phone Number | INVALID | STORED (Packet 42 accepted before entering in NP mode) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | NP | No cold movement occurs while equipment is switched off | I | INT | - | L2/L3 | NP | - |
| 2 | L2/L3 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L2/L3 | SB | - |
| 3 | L2/L3 | SB | Cold movement not detected is received | I | INT | - | L2/L3 | SB | - |
| 4 | L2/L3 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=0) is recorded. | O | JRU | - | L2/L3 | SB | - |
| 5 | L2/L3 | SB | CAB active | I | TIU | - | L2/L3 | SB | - |
| 6 | L2/L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L2/L3 | SB | - |
| 7 | L2/L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L2/L3 | SB | - |
| 8 | L2/L3 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L2/L3 | SB | - |
| 9 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L2/L3 | SB | - |
| 10 | L2/L3 | SB | The driver enters the "Driver ID" | I | DMI | Driver ID and Train running number Deleted in NP | L2/L3 | SB | - |
| 11 | L2/L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | L2/L3 | SB | - |
| 12 | L2/L3 | SB | The driver enters the "Train Running Number" | I | DMI | - | L2/L3 | SB | - |
| 13 | L2/L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | When Train running number is changed | L2/L3 | SB | - |
| 14 | L2/L3 | SB | The Level symbol "Level 2/Level 3" is displayed | O | DMI | Symbol LE_04 or LE_05 ERTMS/ETCS Level and train position are valid | L2/L3 | SB | - |
| 15 | L2/L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit4=1 or Bit5=1>) is recorded | O | JRU | - | L2/L3 | SB | - |
| 16 | L2/L3 | SB | SA-CONNECT. Request is transmitted | O | RTM | Mobile terminal is registered | L2/L3 | SB | - |
| 17 | L2/L3 | SB | SA-CONNECT. Confirm is received | I | RTM | - | L2/L3 | SB | - |
| 18 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2/L3 | SB | - |
| 19 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | SB | - |
| 20 | L2/L3 | SB | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | L2/L3 | SB | - |
| 21 | L2/L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | L2/L3 | SB | - |
| 22 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted. | O | RTM | - | L2/L3 | SB | - |
| 23 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside RBC ID/Phone Number are valid | L2/L3 | SB | - |
| 24 | L2/L3 | SB | SA-DATA. Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0/1 is transmitted | O | RTM | Stored position is valid | L2/L3 | SB | - |
| 25 | L2/L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | SB | - |
| 26 | L2/L3 | SB | The <Main> window is displayed. | O | DMI | - | L2/L3 | SB | - |
| 27 | L2/L3 | SB | The driver selects "Level <>" | I | DMI | Table of trackside supported levels is shown The new level can be the same as the previous one. | L2/L3 | SB | - |
| 28 | L2/L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2/L3 | SB | - |

| Step 18: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 20: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 22: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session Established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 24: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | SoM Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Table of priority of trackside supported levels | VALID | STORED |
| Radio Network ID | VALID | STORED |
| RBC ID/Phone Number | VALID | STORED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE RADIO CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level < L2 / L3 >" is displayed. | O | DMI | On-board supported levels |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off Cold movement occurred. The ERTMS on-board is able to detect and record whether the engine has been moved or not during a period at least 72 hours. |  | 2 |
| Applicable Mode/Level Combinations | L0/ LNTC/ L1/ L2/ L3: SB |  |  |
| Target of Test | The target of this test is to check that: If CM has occurred the on-board equipment qualifies the Table of trackside supported levels as unknown when switched on again. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - Cold movement detected or Cold movement information not available |  |
|  |  | Subset-026-4.11.1.3 |  |
|  |  | Subset-026-4.10.1.3 Line Table of Priority of trackside supported levels |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Table of trackside supported levels is unknown. Every on-board supported level can be selected. |
| Constraints | ERTMS/ETCS on-board unit is powered down. Cold movement detected. Information from packets 41 shall be accepted before entering in NP Train is at standstill. NOTE: Use this TC after SOM procedure to test that S2 and S3 requirements are also covered when Cold Movement Occurred.(Stored info as Train Position, ERTMS/ETCS Level and RBC ID/ Phone number remains invalid - Empty cases from table 4.11.1.1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Table of priority of trackside supported levels | STORED | INVALID (Packet 41 accepted before entering in NP mode) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | NP | Cold movement occurs while equipment is switched off | I | INT | - | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | SB | Cold movement detected is received | I | INT | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=1) is recorded. | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | CAB active | I | TIU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/ LNTC/ L1/ L2/ L3 | SB | The mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 9 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 10 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver enters the "Driver ID" | I | DMI | Driver ID and Train running number Deleted in NP | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 11 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 12 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver enters the "Train Running Number" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 13 | L0/ LNTC/ L1/ L2/ L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | When Train running number is changed | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 14 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver selects "Level <>" | I | DMI | No table of trackside supported levels is shown. Any of the on-board supported level can be selected. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 15 | L0/ LNTC/ L1/ L2/ L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Table of priority of trackside supported levels | STORED | UNKNOWN |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L0 / L1 / L2 / L3 / LNTC>" is displayed. | O | DMI | DMI Object: One of the Symbols LE01-05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off no Cold movement occurred. When powered up again (before 72 hours max) the on-board equipment updates the status of information stored by on-board equipment. |  | 3 |
| Applicable Mode/Level Combinations | L1: SB |  |  |
| Target of Test | The target of this test is to check that: If no CM has occurred the on-board equipment qualifies the EOLM information as valid when switched on again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - No Cold movement occurred |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The speed and distance monitoring information is displayed (update of SR Distance received from Loop, V_TARGET/V_RELEASE set to 0) Check on the JRU that: Message from Loop is stored The speed and distance monitoring information is recorded.(V_TARGET/V_RELEASE=0) |
| Constraints | ERTMS/ETCS on-board unit is powered down. Cold movement detection is available The train has already received an EOLM marker before entering in NP. Train is at standstill. The train is placed into an Infill by loop area. SR distance received from loop is different than National Value or Default Value.(D_SR ≠ D_NVSTFF / default value) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| EOLM information | STORED | INVALID (Packet 134 accepted before entering in NP mode) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | Location: Inside Loop area |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | NP | No cold movement occurs while equipment is switched off | I | INT | - | L1 | NP | - |
| 2 | L1 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L1 | SB | - |
| 3 | L1 | SB | Cold movement not detected is received | I | INT | - | L1 | SB | - |
| 4 | L1 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=0) is recorded. | O | JRU | - | L1 | SB | - |
| 5 | L1 | SB | CAB active | I | TIU | - | L1 | SB | - |
| 6 | L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L1 | SB | - |
| 7 | L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L1 | SB | - |
| 8 | L1 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L1 | SB | - |
| 9 | L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L1 | SB | - |
| 10 | L1 | SB | The on-board equipment performs SoM procedure for Level 1. | - | - | USE_FT5040300 (several TCs) | L1 | SB | - |
| 11 | L1 | SB | The on-board equipment switch to SR mode | - | - | USE_FT4060300.49 (Cond Id [8]) | L1 | SR | - |
| 12 | L1 | SR | The train is moving. | I | INT | - | L1 | SR | - |
| 13 | L1 | SR | One Euroloop message containing packet 13 is received. | I | LTM | Packet Number 13: Staff Responsible distance information from loop. D_SR ≠ D_NVSTFF/Default value | L1 | SR | - |
| 14 | L1 | SR | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L1 | SR | - |
| 15 | L1 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1 | SR | - |
| 16 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 | SR | - |
| 17 | L1 | SR | The speed and distance monitoring information is displayed. | O | DMI | As the permitted distance to run in SR was update to the one received from loop, the ERTMS/ETCS on-board equipment shall supervise braking curves with a target of zero to the end of this distance. | L1 | SR | - |
| 18 | L1 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; V_TARGET=0; V_RELEASE=0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) | L1 | SR | - |

| Step 13: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop (Same identity as received in packet 134) |
| NID_PACKET | 8 | 13 | Staff Responsible distance information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | Shall be a value different from D_NVSTFF and Default value |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY (k) = 1 |
| D_SR (k) | 15 | FINITE VALUE | Reference balise |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | VALID |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | D_SR updated |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off Cold movement occurred. The ERTMS on-board is able to detect and record whether the engine has been moved or not during a period at least 72 hours. |  | 4 |
| Applicable Mode/Level Combinations | L1: SB |  |  |
| Target of Test | The target of this test is to check that: If CM has occurred the on-board equipment qualifies the EOLM information as unknown when switched on again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - No Cold movement occurred |  |
|  |  | Subset-026-4.11.1.3 |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The speed and distance monitoring information is displayed (Distance not update with the SR Distance received from Loop, V_PERM maintained) Check on the JRU that: Message from Loop is recorded. The speed and distance monitoring information is recorded. |
| Constraints | ERTMS/ETCS on-board unit is powered down .Cold movement detected. Train is at standstill The train is placed into an Infill loop area. SR distance received from loop is different than National Value or Default Value.(D_SR ≠ D_NVSTFF / default value) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| EOLM information | STORED | INVALID |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | Location: Inside Loop area |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | NP | Cold movement occurs while equipment is switched off. | I | INT | - | L1 | NP | - |
| 2 | L1 | NP | ERTMS/ETCS on-board unit is powered up | I | INT | The on-board equipment changes to SB mode. | L1 | SB | - |
| 3 | L1 | SB | Cold movement detected is received | I | INT | - | L1 | SB | - |
| 4 | L1 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=1) is recorded. | O | JRU | - | L1 | SB | - |
| 5 | L1 | SB | CAB active | I | TIU | - | L1 | SB | - |
| 6 | L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L1 | SB | - |
| 7 | L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L1 | SB | - |
| 8 | L1 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L1 | SB | - |
| 9 | L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L1 | SB | - |
| 10 | L1 | SB | The on-board equipment performs SoM procedure for Level 1. | - | - | USE_FT5040300 (several TCs) | L1 | SB | - |
| 11 | L1 | SB | The on-board equipment switch to SR mode | - | - | USE_FT4060300.49 (Cond Id [8]) | L1 | SR | - |
| 12 | L1 | SR | The train is moving. | I | INT | - | L1 | SR | - |
| 13 | L1 | SR | One Euroloop message containing packet 13 is received. | I | LTM | Packet Number 13: Staff Responsible distance information from loop. D_SR ≠ D_NVSTFF/Default value | L1 | SR | - |
| 14 | L1 | SR | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L1 | SR | - |
| 15 | L1 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1 | SR | - |
| 16 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 | SR | - |
| 17 | L1 | SR | The speed and distance monitoring information is displayed. | O | DMI | The permitted distance to run in SR is the National Value /Default value but not D_SR. No change on permitted speed at the end of D_SR. | L1 | SR | - |
| 18 | L1 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM) is recorded | O | JRU | - | L1 | SR | - |

| Step 13: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop (Same identity as received in packet 134) |
| NID_PACKET | 8 | 13 | Staff Responsible distance information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | Shall be a value different from D_NVSTFF and Default value |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY (k) = 1 |
| D_SR (k) | 15 | FINITE VALUE | Reference balise |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | UNKNOWN |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | Distance updated to D_NVSTFF or the Default value |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off Cold movement information was not available. The ERTMS on-board is not able to detect and record whether the engine has been moved or not during a period at least 72 hours. |  | 5 |
| Applicable Mode/Level Combinations | L0/ LNTC/ L1/ L2/ L3: SB |  |  |
| Target of Test | The target of this test is to check that: If CM information is not available, the on-board equipment qualifies the Table of trackside supported levels as unknown when switched on again. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - Cold movement detected or Cold movement information not available |  |
|  |  | Subset-026-4.11.1.3 |  |
|  |  | Subset-026-4.10.1.3 Line Table of Priority of trackside supported levels |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Table of trackside supported levels is unknown. Any on-board supported level can be selected. |
| Constraints | ERTMS/ETCS on-board unit is powered down .Cold movement information not available. Information from packet 41 shall be accepted before entering in NP Train is at standstill. NOTE: Use this TC after SOM procedure to test that S2 and S3 requirements are also covered when Cold Movement information is not available.( Stored info as Train Position, ERTMS/ETCS Level and RBC ID/ Phone number remains invalid - Empty cases from table 4.11.1.1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Table of priority of trackside supported levels | STORED | INVALID (Packet 41 accepted before entering in NP mode) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | NP | ERTMS/ETCS on-board unit is powered down. | I | INT | Cold Movement information is not available. | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=2) is recorded. | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SB | CAB active | I | TIU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/ LNTC/ L1/ L2/ L3 | SB | The mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 9 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver enters the "Driver ID" | I | DMI | Driver ID and Train running number Deleted in NP | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 10 | L0/ LNTC/ L1/ L2/ L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 11 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver enters the "Train Running Number" | I | DMI | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 12 | L0/ LNTC/ L1/ L2/ L3 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | When Train running number is changed | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 13 | L0/ LNTC/ L1/ L2/ L3 | SB | The driver selects "Level <> " | I | DMI | No table of trackside supported levels is shown. Any of the on-board supported level can be selected. | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 14 | L0/ LNTC/ L1/ L2/ L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Table of priority of trackside supported levels | STORED | UNKNOWN |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L0 / L1 / L2 / L3 / LNTC>" is displayed. | O | DMI | DMI Object: One of the Symbols LE01-05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off Cold movement information was not available. The ERTMS on-board is able to detect and record whether the engine has been moved or not during a period at least 72 hours. |  | 6 |
| Applicable Mode/Level Combinations | L1: SB |  |  |
| Target of Test | The target of this test is to check that: If CM information is not available, the on-board equipment qualifies the EOLM information as unknown when switched on again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - Cold movement detected or Cold movement information not available |  |
|  |  | Subset-026-4.11.1.3 |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The speed and distance monitoring information is displayed (Distance not update with the SR Distance received from Loop; V_PERM maintained) Check on the JRU that: Message from Loop is recorded. The speed and distance monitoring information is recorded. |
| Constraints | ERTMS/ETCS on-board unit is powered down .Cold movement information not available. The train has already received an EOLM marker before entering in NP Train is at standstill The train is placed into an Infill by loop area. SR distance received from loop is different than National Value or Default Value.(D_SR ≠ D_NVSTFF / default value) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| EOLM information | STORED | INVALID (Packet 134 accepted before entering in NP mode) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | Location: Infill Loop Area |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | NP | ERTMS/ETCS on-board unit is powered down | I | INT | Cold Movement information is not available. | L1 | NP | - |
| 2 | L1 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L1 | SB | - |
| 3 | L1 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=2) is recorded. | O | JRU | - | L1 | SB | - |
| 4 | L1 | SB | CAB active | I | TIU | - | L1 | SB | - |
| 5 | L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L1 | SB | - |
| 6 | L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L1 | SB | - |
| 7 | L1 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L1 | SB | - |
| 8 | L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L1 | SB | - |
| 9 | L1 | SB | The on-board equipment performs SoM procedure for Level 1. | - | - | USE_FT5040300 (several TCs) | L1 | SB | - |
| 10 | L1 | SB | The on-board equipment switch to SR mode | - | - | USE_FT4060300.49 (Cond Id [8]) | L1 | SR | - |
| 11 | L1 | SR | The train is moving. | I | INT | - | L1 | SR | - |
| 12 | L1 | SR | One Euroloop message containing packet 13 is received. | I | LTM | Packet Number 13: Staff Responsible distance information from loop. D_SR ≠ D_NVSTFF/Default value | L1 | SR | - |
| 13 | L1 | SR | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L1 | SR | - |
| 14 | L1 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1 | SR | - |
| 15 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 | SR | - |
| 16 | L1 | SR | The speed and distance monitoring information is displayed. | O | DMI | The permitted distance to run in SR is the National Value /Default value but not D_SR. No change on permitted speed at the end of D_SR. | L1 | SR | - |
| 17 | L1 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM) is recorded | O | JRU | - | L1 | SR | - |

| Step 12: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop (Same identity as received in packet 134) |
| NID_PACKET | 8 | 13 | Staff Responsible distance information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | Shall be a value different from D_NVSTFF and Default value |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY (k) = 1 |
| D_SR (k) | 15 | FINITE VALUE | Reference balise |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | UNKNOWN |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | Distance updated to D_NVSTFF or the Default value |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.8 Cold Movement Detection |  | 3150800 |
| Test Case of Feature | While on-board equipment was switched off no Cold movement occurred. When powered up again (after 72 hours max) the on-board equipment updates the status of information stored on-board. |  | 7 |
| Applicable Mode/Level Combinations | L0/ LNTC/ L1: SB |  |  |
| Target of Test | The target of this test is to check that: If no CM has occurred the on-board equipment qualifies the ETCS level and Table of trackside supported levels as valid when switched on again |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.8.1 |  |
|  |  | Subset-026-3.15.8.2 |  |
|  |  | Subset-026-4.5.2.1 table - Cold Movement Detection |  |
|  |  | Subset-026-4.11.1.1 table - No Cold movement occurred |  |
|  |  | Subset-026-4.10.1.3 Line Table of Priority of trackside supported levels |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Table of trackside supported levels is valid. Check on the JRU that: The variable M_LEVEL ≠ 5-7 is recorded. |
| Constraints | ERTMS/ETCS on-board unit is powered down. Cold movement detection is available. Information from packet 41 shall be accepted before entering in NP. Train is at standstill. NOTE: Use this TC after SOM procedure to test that S2 and S3 requirements are covered when Cold Movement Occurred. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |
| Table of priority of trackside supported levels | STORED | INVALID (Packet 41 accepted before entering in NP mode) |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | NP | No cold movement occurs while equipment is switched off | I | INT | - | L0/LNTC/L1 | NP | - |
| 2 | L0/LNTC/L1 | NP | ERTMS/ETCS on-board unit is powered up. | I | INT | The on-board equipment changes to SB mode. | L0/LNTC/L1 | SB | - |
| 3 | L0/LNTC/L1 | SB | Cold movement not detected is received | I | INT | - | L0/LNTC/L1 | SB | - |
| 4 | L0/LNTC/L1 | SB | INFORMATION FROM COLD MOVEMENT DETECTOR (NID_MESSAGE_JRU=15; M_COLD_MVT=0) is recorded. | O | JRU | - | L0/LNTC/L1 | SB | - |
| 5 | L0/LNTC/L1 | SB | CAB active | I | TIU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0/LNTC/L1 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 7 | L0/LNTC/L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |
| 8 | L0/LNTC/L1 | SB | The Mode symbol "Stand By" is displayed. | O | DMI | Symbol MO13 | L0/LNTC/L1 | SB | - |
| 9 | L0/LNTC/L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0/LNTC/L1 | SB | - |
| 10 | L0/LNTC/L1 | SB | The driver enters the "Driver ID" | I | DMI | Driver ID and Train running number Deleted in NP | L0/LNTC/L1 | SB | - |
| 11 | L0/LNTC/L1 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | A “GENERAL MESSAGE” is recorded every 5s by the on-board equipment. | L0/LNTC/L1 | SB | - |
| 12 | L0/LNTC/L1 | SB | The driver enters the "Train Running Number" | I | DMI | - | L0/LNTC/L1 | SB | - |
| 13 | L0/LNTC/L1 | SB | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | When Train running number is changed | L0/LNTC/L1 | SB | - |
| 14 | L0/LNTC/L1 | SB | The Level symbol "Level <L0 /LNTC /L1>" is displayed. | O | DMI | DMI Object: Symbol LE01/LE02/LE03 ERTMS/ETCS Level and train position are valid | L0/LNTC/L1 | SB | - |
| 15 | L0/LNTC/L1 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit=01/Bit02=1/ Bit03=1>) is recorded | O | JRU | - | L0/LNTC/L1 | SB | - |
| 16 | L0/LNTC/L1 | SB | The <Main> window is displayed. | O | DMI | - | L0/LNTC/L1 | SB | - |
| 17 | L0/LNTC/L1 | SB | The driver selects "Level <>". | I | DMI | Table of trackside supported levels is shown The new level can be the same as the previous one. | L0/LNTC/L1 | SB | - |
| 18 | L0/LNTC/L1 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0/LNTC/L1 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |
| Table of priority of trackside supported levels | STORED | VALID |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level < L0/ LNTC/ L1>" is displayed. | O | DMI | On-board supported levels |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |