| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3080200 (3.8.2: MA request to the RBC) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-3080200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.0a 05.10.2012 | All | First version/review for Baseline 3 | DLR |
| Version 2.9.1 21.10.2013 | All | review | DLR |
| Version 2.9.2 04.12.2013 | TC9 | added TC9 for covering the requirements 3.8.2.7.1 and 3.8.2.7.2 | DLR |
| Version 2.9.3 12.12.2013 | TC9 | review | DLR |
| Version 2.9.4 22.01.2014 | TC2 TC7 | Constraint added to replace former “Preceeded by” TC Target of Test changed: sentence "(until the driver presses the “Override EoA” button)" removed | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | The train receives a MA which includes timers for the sections and MA request parameters which includes T_TIMEOUTRQST different from 1023. A time (T_TIMEOUTRQST) before the section timer expires, the train sends a MA request to the RBC The train receives the a MA which includes timers for the sections and MA request parameters packet which includes T_MAR=1023 The section timer expires and no MA request has been sent. The train stops before the entry point of the revoked section |  | 1 |
| Applicable Mode/Level Combinations | L2: FS / OS / LS |  |  |
| Target of Test | The target of this test is to check that : The MA request parameters shall be valid until new MA request parameters are received When the train receives the MA request parameters packet which includesT_TIMEOUTRQST=1023, no MA request is sent before the timer expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of perturbation location (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : The radio message 132 “MA request” is sent at time t = t' + T_SECTIONTIMER -T_TIMEOUTRQST before the section timer expires. The radio message 132 “MA request” is not sent before the section timer of MA recevied at step 5 expires. Check on both the JRU that : The radio message 132 “MA request” is recorded a time t = t' + T_SECTIONTIMER -T_TIMEOUTRQST before the section timer expires. The radio message 132 “MA request” is not recorded before the section timer of MA recevied at step 5 expires. |
| Constraints | First MA request parameters: Ttimeoutrqst in packet 57 shall be different from 1023 and Tmar=255 Second MA request parameters: Ttimeoutrqst =1023in packet in packet 57: MA request parameters and Tmar=255 The MAs received have sctions and timers related to them |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_TIMEOUTRQST≠1023; T_MAR=255) is received | I | RTM | T_TRAIN=t'. If tested in OS or LS a mode profile is received with the MA | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | T_TRAIN= t'+T_SECTIONTIMER – T_TIMEOUTREQUEST | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_TIMEOUTRQST=1023; T_MAR=255) is received | I | RTM | If tested in OS or LS a mode profile is received with the MA | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | Between the reception of the second Message 3 (step 5) and the expiration of the timer, no MA request is sent. | N/A | N/A | - |
| 8 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | The train stops before the entry point of the revoked section | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | 1023 | No time-out of target speed at the EMA |
| N_ITER | 5 | FINITE VALUE | N_ITER > 0 for the MA to have multiple sections |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | 1 | Section timer information to follow |
| T_SECTIONTIMER(k) | 10 | ≠1023, ≠0 | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | 1 | - |
| T_SECTIONTIMER | 10 | ≠1023, ≠0 | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 1 | - |
| T_ENDTIMER | 10 | ≠1023, ≠0 | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | - |
| T_TIMEOUTRQST | 10 | ≠1023 | - |
| T_CYCRQST | 8 | 255 | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' + T_CYCRQST | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 / 2 | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | 1 | Section timer information to follow |
| T_SECTIONTIMER(k) | 10 | ≠1023, ≠0 | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | 1 | - |
| T_SECTIONTIMER | 10 | ≠1023, ≠0 | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 1 | - |
| T_ENDTIMER | 10 | ≠1023, ≠0 | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | - |
| T_TIMEOUTRQST | 10 | 1023 | - |
| T_CYCRQST | 8 | 255 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session with the RBC is still established. |
| MA Request Parameters | STORED | T_MAR=255 T_TIMEOUTRQST=1023 |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location. The on-board equipment informs the RBC by sending a radio message 132 “MA request”. |  | 2 |
| Applicable Mode/Level Combinations | L2: FS, OS, LS L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to check that : The on-board informs the RBC that the current MA and the track description have been shortened when the train reaches standstill. The function “Request MA on track description deletion” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.7.3 |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 d) |  |
|  |  | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |
|  |  | Subset-026-4.5.2.1 table - Request MA on track description deletion (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: A radio message 132 (MA request) is sent to the RBC. Check on the JRU that: The radio message 132 (MA request) sent by the on-board is recorded. |
| Constraints | A situation which leads to shorten the current MA and track description to the current position of the on-board equipment or the new stop location has occurred |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132; Q_MARQSTREASON=01000) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | The radio message 132 (MA request) sent by the on-board equipment is RECORDED. | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 1000 | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | A train with a valid MA with sections standing in the section which precedes the end section (L_ENDSECTION), requests a new MA a defined time before the stop location time out expires and continue to request it after that time out expired (until the train receives a new MA from RBC) |  | 3 |
| Applicable Mode/Level Combinations | L2 : FS, OS, LS L3: FS, OS, LS |  |  |
| Target of Test | The target of this test is to check that: a train with a MA with several sections is standing in the Section which precedes the end section. At the proper moment the on-board starts to request new MAs cyclically (each T_CYCRQST SECONDS), and continues to request it even after the time out of the End Section expired. - The on-board function “ Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3)“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.7.3 |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.2.8 d) |  |
|  |  | Subset-026-3.8.4.2.1 |  |
|  |  | Subset-026-3.8.4.2.1 a) |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of perturbation location (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Calculate the time T = [ T_TRAINlast received MA + T_SECTIONTIMER (End Section timer) - T_TIMEOUTRQST SEC ]. The simulators of track and of train motion will drive the train in order to locate it in the Section previous to the End Section at the starting time of the test. When the [T_SECTIONTIMER (End Section timer) – T_TIMEOUTRQST] expires Check on JRU and RTM that the message 132 is sent by the train to RBC when the train time reaches the value T = [ T_TRAINlast received MA + T_SECTIONTIMER (End Section timer) - T_TIMEOUTRQST] Check on JRU and RTM that the message 132 is repeated each interval T_CYCRQST until other condition (MA received) breaks the cyclic request |
| Constraints | The test is preceded by a situation with the following conditions: the reception onboard of a MA of N sections (message 3) the reception onboard from RBC of packet 57 (MA Request parameters)... onboard reports valid position to RBC (messages recorded on JRU) the train has passed Time-out stop location of the previous section with the min safe front end of the train. There is no time out related to the previous sections, so the track description has not been deleted and the MA has not been shortened The Timer for End Section expires before the train has passed the section Time-out stop location of End Section and has not already entered in the End Section. The On-board has deleted the track-side description (Q_MARQSTREASON= x1xxx ) when the MA is shortened to the entry of the End Section. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | End section: L_ENDSECTION<327.670 km Q_SECTIONTIMER=1 T_SECTIONTIMER=FINITE VALUE D_SECTIONTIMERSTOPLOC=FINITE VALUE |
| MA Request Parameters | STORED | T_MAR=255 To avoid MA requests for other conditions T_CYCRQST<255 To avoid use of fixed value from A3.1 (T_CYCRQST ≠ 60), T_TIMEOUTRQST<1023 to ensure the use of stored MA request parameter |
| Train Position | STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is displayed | O | DMI | The target distance value must be consistent with the assumption: the train is in section N-1 (the previous section to the End Section), has passed the section N-1 Time-out stop location and the min safe front end is in front of the End Section Time-out stop location. Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Train is in the section which precedes the last one. The Time-out stop location of that previous Section is passed. T = [ T_TRAINlast received MA + T_SECTIONTIMER - T_TIMEOUTRQST] | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | As intervals of T_CYCRQST seconds expire, other MA requests are cyclically issued. The test is designed in order to allow several MA requests before the T_SECTIONTIMER expires. Check that the difference of time between the first and the second MA requests is effectively equal to T_CYCRQST. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | All other MA requests are recorded in JRU | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed/removed | O | DMI | T_SECTIONTIMER of End section expires MA is shortened to the entry point of End Section. Target distance, permanently displayed, changes suddenly its value (to the entry point of End Section). The train has not passed the section End Section Time-out stop location. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Intervals of T_CYCRQST seconds expire: the train continue to request MA in L2 until it receives a MA or until the driver selects override | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Each MA request (Message 132) is RECORDED. Check that the difference of time between two consecutive MA requests is effectively equal to T_CYCRQST. | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Check that ME requests are sent periodically every T_CYCRQST seconds |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| Step 6: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Check that ME requests are sent periodically every T_CYCRQST seconds |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 1000 | Track description deleted |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Consistent with the shortened MA to the entry point of End Section |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Target distance is displayed: to the entry point of End Section. | O | DMI | Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | A train with a valid MA with sections which has passed the entry point of End Section and the min safe front end has not passed the End Section Time-out stop location, stops to request a new MA when the time out of End Section expires. EB is applied and the TR mode is entered |  | 4 |
| Applicable Mode/Level Combinations | L2 :FS, OS, LS |  |  |
| Target of Test | The target of this test case is to check that a train with a MA with sections, is standing in End Section. The front end has passed the End Section entry point and the min safe front end has not passed the End Section Time-out stop location. At the proper time (according to MA request parameters) the on-board starts to request MAs cyclically, and continue until the time out of End Section expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.4.2.1 |  |
|  |  | Subset-026-3.8.4.2.1 a) |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-4.5.2.1 table - Determine train speed, train acceleration, train standstill |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Calculate the time T = [ T_TRAINlast received MA + T_SECTIONTIMER (End Section timer)- T_TIMEOUTRQST SEC ]. The simulators of track and of train motion will drive the train in order to locate it in the End Section Time-out stop location is not passed with the min safe front end at the starting time of the test. When the time T = [ T_TRAINlast received MA + T_SECTIONTIMER (End Section timer) - T_TIMEOUTRQST SEC ] is reached Check on RTM and JRU that the message 132 is sent by the train to RBC Check on RTM and JRU that the message 132 is repeated all T_CYCRQST seconds until the time out of End Section expires and the min safe front end of the train has not passed the Time-out stop location of End Section. |
| Constraints | The test is preceded by: the reception onboard of a MA of N sections (message 3) the reception onboard from RBC of packet 57 (MA Request parameters): variables T_TIMEOUTRQST and T_CYCRQST the train has entered the End Section, and the min safe front end has not passed the End Section Time-out stop location There is no time out related to the previous sections, so the track description has not been deleted and the MA has not been shortened MA request parameter stored on-board and T_MAR = 255 to make it not relevant in this test case |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| MA Request Parameters | STORED | End section: L_ENDSECTION<327.670 km Q_SECTIONTIMER=1 T_SECTIONTIMER=FINITE VALUE D_SECTIONTIMERSTOPLOC=FINITE VALUE |
| Plain Text Information (not location based) | STORED | T_MAR=255 To avoid MA requests for other conditions T_CYCRQST≠60 To avoid the use of default value, because the packet 57 was received. T_TIMEOUTRQST<FINITE VALUE |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TARGET DISTANCE DISPLAYED | O | DMI | Target distance value must be consistent with the assumption that the train has entered End Section and the min safe front end has not passed the Time-out stop location of End Section. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The train min safe front end has entered the End Section but the Time Out stop location has not already reached. T = [ T_TRAINlast received MA + T_SECTIONTIMER - T_TIMEOUTRQST] The onboard equipment issues the first MA request | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Request of MA is recorded | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | As each intervals of T_CYCRQST seconds expires, the onboard equipment issues the next requests of MA. The test is designed in order to allow several MA requests before the T_SECTIONTIMER EXPIRES | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | MA requests are recorded on JRU | N/A | N/A | - |
| 5 | L2 | OS / FS / LS | Use_FT4060300.24 "The train is in FS / LS / OS and the train/engine overpasses the EOA/LOA of its MA. The system switches to TR Mode. Transition Id [16]" | - | - | T_SECTIONTIMER expires MA is shortened to the entry point of End Section, because the min front end of the train has not passed the Time-out stop location: On-board equipment goes in TR mode Target distance disappears on the screen TR mode is displayed | L2 | TR | - |
| 6 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | The min safe front end has not passed the Time-out stop location of End Section |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 | L2 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | UNKNOWN | Deleted |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| TR MODE DISPLAYED | O | DMI | Target distance value must be consistent with the assumption that the train has entered End Section and the min safe front end has not passed the Time-out stop location of End Section. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | Cyclical MA request due to approach of target indication point |  | 5 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that : The on-board equipment sends a request for a new MA (radio message 132) according to value of the variable T_MAR ( Time before reaching the perturbation location ) previously received (FT4080423.TC2 “transmission of MA request parameters from RBC”). The on-board equipment cyclically requests the RBC for a new MA according to the value of the variable T_CYCRQST previously received (FT4080423.TC2 “transmission of MA request parameters from RBC”) until the RBC sends a new MA. The on-board function “ Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3)“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 a) |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 b) |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of perturbation location (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on both the RTM that : The radio message 132 “MA request” is sent to the RBC immediately as soon as the train reaches the perturbation location to issue the first MA request. The radio message 132 is cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority”. The difference of time between two consecutive MA requests is equal to T_CYCRSQST. The radio message 3 “Movement Authority“ is received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”). The radio message 132 is NO LONGER sent to the RBC as soon as a new MA is received from the RBC. Check on both the JRU that : The radio message 132 “MA request” sent to the RBC immediately as soon as the train reaches the perturbation location to issue the first MA request is recorded. The radio message 132 cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority” is recorded. The difference of time between two consecutive recorded MA requests is equal to T_CYCRQST. The radio message 3 “Movement Authority“ received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”) is recorded. There are no more recordings of radio message 132 sent to the RBC as soon as a new MA is received from the RBC.Check on both the RTM that : The radio message 132 “MA request” is sent to the RBC immediately as soon as the train reaches the perturbation location to issue the first MA request. The radio message 132 is cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority”. The difference of time between two consecutive MA requests is equal to T_CYCRSQST. The radio message 3 “Movement Authority“ is received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”). The radio message 132 is NO LONGER sent to the RBC as soon as a new MA is received from the RBC. Check on both the JRU that : The radio message 132 “MA request” sent to the RBC immediately as soon as the train reaches the perturbation location to issue the first MA request is recorded. The radio message 132 cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority” is recorded. The difference of time between two consecutive recorded MA requests is equal to T_CYCRQST. The radio message 3 “Movement Authority“ received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”) is recorded. There are no more recordings of radio message 132 sent to the RBC as soon as a new MA is received from the RBC.Check on both the RTM that : The radio message 132 “MA request” is sent to the RBC immediately as soon as the train reaches the pre-indication location for the EOA/LOA to issue the first MA request. The radio message 132 is cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority”. The difference of time between two consecutive MA requests is equal to T_CYCRSQST. The radio message 3 “Movement Authority“ is received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”). The radio message 132 is NO LONGER sent to the RBC as soon as a new MA is received from the RBC. Check on both the JRU that : The radio message 132 “MA request” sent to the RBC immediately as soon as the train reaches the pre-indication location for the EOA/LOA to issue the first MA request is recorded. The radio message 132 cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority” is recorded. The difference of time between two consecutive recorded MA requests is equal to T_CYCRQST. The radio message 3 “Movement Authority“ received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”) is recorded. There are no more recordings of radio message 132 sent to the RBC as soon as a new MA is received from the RBC.Check on both the RTM that : The radio message 132 “MA request” is sent to the RBC immediately as soon as the train reaches the pre-indication location for the EOA/LOA to issue the first MA request. The radio message 132 is cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority”. The difference of time between two consecutive MA requests is equal to T_CYCRSQST. The radio message 3 “Movement Authority“ is received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”). The radio message 132 is NO LONGER sent to the RBC as soon as a new MA is received from the RBC. Check on both the JRU that : The radio message 132 “MA request” sent to the RBC immediately as soon as the train reaches the pre-indication location for the EOA/LOA to issue the first MA request is recorded. The radio message 132 cyclically sent to the RBC until reception from the RBC of the radio message 3 “Movement Authority” is recorded. The difference of time between two consecutive recorded MA requests is equal to T_CYCRQST. The radio message 3 “Movement Authority“ received from the RBC (described in one of the test cases of FT4080425 “Transmission of L2/L3 MA from RBC”) is recorded. There are no more recordings of radio message 132 sent to the RBC as soon as a new MA is received from the RBC. |
| Constraints | The RBC will NOT respond to the MA request from the on-board equipment during a time at least equal to N * T_CYCRQST (T_CYCRQST is the time between two cyclic MA requests (defined in packet 57) and N ( 1 shall be chosen by the tester). There are an initial MA and a complete track description on-board. The track description data obviously covers the current MA and also covers the new MA received from the RBC after the cyclic MA requests by the on-board equipment. We assume within the scope of this test case that the new MA sent by the RBC will extend the current MA with an EoA which ensures that the time before reaching the perturbation location (defined with T_MAR) is not reached in the next cycle given with T_CYCRQST. The MA request is sent with Q_TRACKDEL = 0 ((track description not deleted) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| Movement Authority | STORED | There is a packet 15 on-board |
| Gradient Profile | STORED | There is a packet 21 on-board |
| International SSP | STORED | There is a packet 27 on-board |
| MA Request Parameters | STORED | T_CYCRQST<255; T_CYCRQST≠15 T_MAR=FINITE VALUE T_TIMEOUTRQST=1023 There is a packet 57 on-board (either previously received from trackside or on-board default values) Time before reaching the perturbation location Time before any section timer expires: special value “1023” (Not relevant) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The train is running and would reach the pre-indication location in T_MAR seconds if it would be driving at warning speed. the first MA request is issued: T = t’ | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | T = t’ + N * T_CYCRQST. As intervals of T_CYCRQST seconds expire, other MA requests are cyclically issued. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT4080425.3 “A L2/L3 MA is transmitted by radio and is received by the onboard equipment ” OR Use_FT3080500.3 "The train has reached the Indication Limit of previous Movement Authority. It receives an extended Movement Authority from RBC." | - | - | T > t’ + N * T_CYCRQST The on-board equipment receives the radio message 3 “Movement Authority” before reaching the end of the initial MA. + The radio message 3 “Movement Authority” OR The new MA received from the RBC is accepted by the on-board equipment and extends the previous MA (observable on the DMI) | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The on-board equipment NO LONGER sends the radio message 132 “MA request“ to the RBC. Check on those two interfaces that the on-board equipment no longer requests the RBC for a new MA T = t’ + (N+1)*T_CYCRQST | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t' |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xxx1x | Time before reaching perturbation location reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | - |
| M_LEVEL | 3 | 3 / 4 | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ + T_CYCRQST * N where N ϵ {1, 2, ...} |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xxx1x | Time before reaching perturbation location reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | - |
| M_LEVEL | 3 | 3 / 4 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is still established. |
| Movement Authority | STORED | Extended. A new packet 15 has been recieved |
| Gradient Profile | STORED | Unchanged |
| International SSP | STORED | Unchanged |
| Axle load speed profile | STORED | Unchanged |
| MA Request Parameters | STORED | T_CYCRQST<255; T_CYCRQST≠15 T_MAR=FINITE VALUE T_TIMEOUTRQST=1023 Unchanged Unchanged Unchanged |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | The MA request is sent a defined time before reaching the perturbation location . |  | 6 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that : • The MA request parameters shall be valid until new MA request parameters are received • When the train receives the MA request parameters packet which includes T_CYCRQST =255, no MA request is sent again after the first MA request has been sent • When the train receives the MA request parameters packet which includes T_MAR =255, no MA request is sent before reaching the indication point • The given data are valid until new MA request parameters are given from the RBC. • The on-board function “ Request MA Cyclically respect to approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3)“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 a) |  |
|  |  | Subset-026-3.8.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of perturbation location (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : • The radio message 132 “MA request” is sent a time (T_MAR) before reaching the perturbation location. • The radio message 132 “MA request” is not sent again • The radio message 132 “MA request” is not sent to the RBC before reaching the perturbation location. Check on both the JRU that : • The radio message 132 “MA request” is recorded a time (T_MAR) before reaching the perturbation location. • The radio message 132 “MA request” is not recorded again • The radio message 132 “MA request” is not recorded before reaching the perturbation location. |
| Constraints | • First MA request parameters: T_MAR in packet 57 shall be different from 255 and T_CYCRSQST =255 • Second MA request parameters: T_MAR =255 in packet in packet 57: MA request parameters • The MAs received has no timers included • The length of the fist MA received shall be long enough in order to be able to send the MA request a time T_MAR before reaching the perturbation location • The length of the second MA received shall be long enough in order not have reached the perturbation location when receiving the MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_CYCRQST=255; T_MAR≠255) is received | I | RTM | T_TRAIN=t'. If tested in OS/MA a mode profile is received with the MA | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The train is running and would reach the pre-indication location in T_MAR seconds if it would be driving at warning speed. the first MA request is issued: T = t’ . | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | The train stops before the EOA and no MA request has been sent again. It has to be checked in the RTM and JRU that no MA request has been sent again since the first MA request was sent. | N/A | N/A | - |
| 6 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_MAR=255) is received | I | RTM | If tested in OS/LS a mode profile is received with the MA | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train reaches the perturbation location t of the MA | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | I | RTM | No MA request has been sent to the RBC. | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | ≠255 | - |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | FINITE VALUE | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' + T_CYRQST | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 12 | - |
| M_LEVEL | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | - |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is still established. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | The train is running in ERTMS level 2 or 3 and reaches the location to issue the first MA request. The on-board equipment then cyclically requests the RBC for a new MA. The train reaches the end of the current MA and stops since no new MA is received from the RBC. |  | 7 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment sends a request for a new MA (radio message 132) when the train front has passed the resulting location regarding to the value of the variable T_MAR (time before reaching the perturbation location) previously received (feature 94 “transmission of MA request parameters from RBC”). • The on-board equipment cyclically requests the RBC for a new MA according to the value of the variable T_CYCRQST. • As no new MA is received from the RBC on time, the train stops at the end of the current MA. The on-board equipment nevertheless carries on cyclically requesting the RBC for a new MA. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 a) |  |
|  |  | Subset-026-3.8.2.3.1 |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 b) |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on both the RTM that : • The radio message 132 “MA request” is sent to the RBC immediately as soon as the train reaches the location to issue the first MA request. • The radio message 132 is cyclically sent to the RBC until the RBC responds. • The difference of time between two consecutive MA requests is equal to default value of T_CYCRQST 10 sec. • The radio message 132 is still sent to the RBC after the train has stopped at the end of current MA (since no new MA was received from the RBC). Check on both the JRU that : • The radio message 132 “MA request” sent to the RBC immediately as soon as the train reaches the location to issue the first MA request is recorded. • The radio message 132 cyclically sent to the RBC until the RBC responds is recorded. • The difference of time between two consecutive recorded MA requests is equal to default value of T_CYCRQST.10 sec. • There are still recordings of radio message 132 sent to the RBC since no new MA was received from the RBC after the train has stopped at the end of current MA. |
| Constraints | • NO new MA is sent by the RBC to the on-board equipment before the train reaches the end of the current MA. • There are an initial MA and a complete track description on-board. • The MA request is sent with Q_MARQSTREASON = xxx1x (Time before reaching the perturbation location reached). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| MA Request Parameters | STORED | T_TIMEOUTRQST=1023 T_CYCRQST=10 No MA request triggering with regards to this function |
| Train Position | V_TRAIN > 0 | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The train is running and reaches the location to issue the first MA request : T = t’. At T = t’ + T_CYCRQST second MA request is issued. Check that the difference of time between two consecutive MA requests is effectively equal to T_CYCRQST (10 sec). Steps 1 and 2 are executed repeatedly every T_CYCRQST seconds N times (N is chosen by the tester but shall be at least 2). | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | The train is at standstill at the end of the current MA | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The train informs the RBC about standstill. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | The on-board equipment carries on requesting the RBC for a MA every T_CYCRQST seconds until the driver presses “Override EoA” button Steps 6 and 7 are executed repeatedly until the driver presses “Override EoA” button. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ + M * T_CYCRQST Where M is 0 in the first execution of step 1, and is incremented by 1 at each subsequent execution. |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xxx1x | Time before reaching the perturbation location reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 / 2 | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t’ + (N+K) * T_CYCRQST Where N is the number of times steps 1 and 2 are executed and K is 0 in the first execution of step 6, and is incremented by 1 at each subsequent execution. |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | xxx1x | Time before reaching the perturbation location reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 / 2 | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Unchanged. No new MA has been recieved. |
| Gradient Profile | STORED | Unchanged |
| International SSP | STORED | Unchanged |
| MA Request Parameters | STORED | T_TIMEOUTRQST=1023 Unchanged |
| Train Position | V_TRAIN = 0 | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | The train receives a MA which includes a validity time for the target speed at the EMA and MA request parameters which includes T_TIMEOUTRQST different from 1023. A time (T_TIMEOUTRQST) before the validity time for the target speed at the EMA expires, the train sends a MA request to the RBC. The train receives the a MA which includes a validity time for the target speed at the EMA and MA request parameters packet which includes T_MAR=1023. The validity time for the target speed at the EMA expires and no MA request has been sent. When the timer expires, the speed associated to the Limit of Authority is set to zero. |  | 8 |
| Applicable Mode/Level Combinations | L2: FS / OS / LS |  |  |
| Target of Test | The target of this test is to check that : The MA request parameters shall be valid until new MA request parameters are received When the train receives the MA request parameters packet which includes T_TIMEOUTRQST=1023, no MA request is sent before the validity time for the target speed at the EMA expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.1 |  |
|  |  | Subset-026-3.8.2.3 |  |
|  |  | Subset-026-3.8.2.3 b) |  |
|  |  | Subset-026-3.8.2.8 |  |
|  |  | Subset-026-3.8.2.8 c) |  |
|  |  | Subset-026-3.8.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically respect to approach of perturbation location (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) |  |
|  |  | Subset-026-8.6.3 Message 132: MA request |  |
|  |  | Subset-026-3.8.4.3.1 |  |
|  |  | Subset-026-3.8.4.3.1 a) |  |
|  |  | Subset-026-3.8.4.3.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : The radio message 132 “MA request” is sent at time t = t' + T_EMA -T_TIMEOUTRQST before the validity time for the target speed at the EMA expires. The radio message 132 “MA request” is not sent before the validity time for the target speed at the EMA of MA recevied at step 5 expires. Check on both the JRU that : The radio message 132 “MA request” is recorded a time t = t' + T_EMA -T_TIMEOUTRQST before the validity time for the target speed at the EMA expires. The radio message 132 “MA request” is not recorded before the validity time for the target speed at the EMA of MA recevied at step 5 expires. |
| Constraints | First MA request parameters: T_TIMEOUTRQST in packet 57 shall be different from 1023 and T_MAR=255 Second MA request parameters: T_TIMEOUTRQST=1023 in packet in packet 57: MA request parameters and T_MAR=255 The MAs received have sections with Permitted speed at the End of Movement Authority and validity time for the target speed at theEMA. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | A radio communication session is established. |
| ERTMS/ETCS level | 3 | L2 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_TIMEOUTRQST≠1023; T_MAR=255) is received | I | RTM | T_TRAIN=t'. If tested in OS or LS a mode profile is received with the MA | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | T_TRAIN= t'+T_EMA – T_TIMEOUTREQUEST | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 and packet 57 (T_TIMEOUTRQST=1023; T_MAR=255) is received | I | RTM | If tested in OS a mode profile is received with the MA | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | Between the reception of the second Message 3 (step 5) and the expiration of the validity time for the target speed at the EMA, no MA request is sent. | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | At t=to+T_EMA, the timer expires, so the speed associated to the Limit of Authority is set to zero. Use_FT9990400.21 for the exhaustive list of location based information stored on-board, which shall be deleted accordingly. Restricted in LS. Some Information is available on DMI under certain conditions | N/A | N/A | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | >0 | - |
| T_EMA | 10 | FINITE VALUE | Must not be 1023 |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | 0 | No section timers |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | 0 | No section timer |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 0 | No end section timer |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | - |
| T_TIMEOUTRQST | 10 | ≠1023 | - |
| T_CYCRQST | 8 | 255 | - |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t' + T_CYCRQST | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | 100 | Time before a section timer/EMA speed timer expires reached |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | 0 / 1 | Reverse or nominal |
| Q_DLRBG | 2 | 0 / 1 | Reverse or nominal |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | 1 / 2 | Train integrity is confirmed |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | 0 / 1 | Reverse or nominal |
| M_MODE | 4 | 0 / 1 / 12 | FS / OS / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | >0 | - |
| T_EMA | 10 | ≠1023 | Must not be „no time-out“ |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | 0 | No section timer |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | 0 | No end section timer |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | 255 | - |
| T_TIMEOUTRQST | 10 | 1023 | - |
| T_CYCRQST | 8 | 255 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | STORED | Unchanged |
| Not yet applicable National Values | UNKNOWN | Deleted beyond the new SvL |
| Linking | UNKNOWN | Deleted beyond the new SvL |
| Movement Authority | UNKNOWN | Deleted beyond the new SvL |
| Gradient Profile | UNKNOWN | Deleted beyond the new SvL |
| International SSP | UNKNOWN | Deleted beyond the new SvL |
| Axle load speed profile | UNKNOWN | Deleted beyond the new SvL |
| STM max speed | STORED | Unchanged |
| STM system speed/distance | STORED | Unchanged |
| Level Transition Order | STORED | Unchanged |
| Stop Shunting on desk opening | STORED | Unchanged |
| List of Balises for SH area | UNKNOWN | Deleted |
| MA Request Parameters | STORED | Unchanged |
| Position Report parameters | STORED | Unchanged |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | The considered situations cannot occur when a list of balises to be used in SR is available onboard. Indeed, the onboard is in SR mode and since no MA or track description are stored onboard, no new SvL may be defined. |
| Temporary Speed Restrictions | STORED | Unchanged |
| Inhibition of revocable TSRs from balises in L2/3 | STORED | Unchanged |
| Default Gradient for TSR | STORED | Unchanged |
| Signalling related Speed Restriction | UNKNOWN | Beyond the new SvL or in case of situation a, beyond the stop location of the accepted CES |
| Route Suitability Data | UNKNOWN | Deleted beyond the new SvL |
| Plain Text Information (location based) | UNKNOWN | Only if the location where to start to display the text is beyond the new SvL; otherwise all the text information (i.e. including end location where to stop display, if any) shall remain unchanged |
| Plain Text Information (not location based) | STORED | Unchanged |
| Fixed Text Information (location based) | UNKNOWN | Only if the location where to start to display the text is beyond the new SvL; otherwise all the text information (i.e. including end location where to stop display, if any) shall remain unchanged |
| Fixed Text Information (not location based) | STORED | Unchanged |
| Geographical Position | STORED | Unchanged |
| Mode Profile | UNKNOWN | Deleted beyond the new SvL or in case of situation a, beyond the stop location of the accepted CES. If the start location of the Mode Profile is beyond the new SvL, the acknowledgement window of the Mode Profile shall be deleted as well. |
| RBC Transition Order | UNKNOWN | Deleted beyond the new SvL |
| Radio Infill Area information | UNKNOWN | Deleted beyond the new SvL |
| EOLM information | STORED | Unchanged |
| Track conditions excluding big metal masses | RESET | Deleted beyond the new SvL |
| Track condition big metal masses | RESET | Deleted beyond the new SvL |
| Unconditional Emergency Stop | STORED | Unchanged |
| Conditional Emergency Stop | STORED | Unchanged |
| Train Position | STORED | Unchanged |
| Train Data | STORED | Unchanged |
| Adhesion factor | STORED | Unchanged |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | Unchanged |
| Driver ID | STORED | Unchanged |
| Radio Network ID | STORED | Unchanged |
| RBC ID/Phone Number | STORED | Unchanged |
| Train Running Number | STORED | Unchanged |
| Reversing Area Information | UNKNOWN | Deleted beyond the new SvL |
| Reversing Supervision Information | STORED | Unchanged |
| Track Ahead Free Request | STORED | Unchanged |
| Level Crossing information | STORED | Unchanged |
| Permitted Braking Distance Information | UNKNOWN | Deleted beyond the new SvL |
| RBC/RIU System Version | STORED | Unchanged |
| Operated System Version | STORED | Unchanged |
| Language used to display information to the driver | STORED | Unchanged |
| Virtual Balise Covers | STORED | Unchanged |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed/removed as specified in End Conditions | O | DMI | Restricted in LS. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.8.2: MA request to the RBC |  | 3080200 |
| Test Case of Feature | if a level 2/3 transition is announced and a communication session is already established, an MA request shall be sent to the RBC when the information "Track ahead free up to level 2/3 transition location" is received from balise group. The ERTMS/ETCS on-board equipment shall also inform the RBC about the identity of the level 2/3 transition location balise group, as received through the information "Track ahead free up to level 2/3 transition location". |  | 9 |
| Applicable Mode/Level Combinations | L0: UN / SB / TR / PS L1: FS / OS / SR / SB / TR / LS / PS LNTC: TR / SN / PS |  |  |
| Target of Test | The train should send an MA Request to the RBC when the information "Track ahead free up to level 2/3 transition location" is received from balise group. And should send a Level 2/3 transition information about the level 2/3 transition location balise group, |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.2.7.1 |  |
|  |  | Subset-026-3.8.2.7.2 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA on reception of "track ahead free up to the level 2/3 transition location" (only level 0,1,NTC) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | - The train overpasses a BG with informations about Track ahead free up to level 2/3 transition location - The train sends a MA Request to the RBC - The train sends a Level 2/3 transition information with the identity of the level 2/3 transition location balise group |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / SR / UN / SB / TR / LS / SN / PS | 0 / 1 / 2 / 4 / 6 / 7 / 12 / 13 / 15 |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | For L1 section |
| Level Transition Order | STORED | Level transition order for Level 2/3 is announced |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 90 (NID_BG="BGx"), packet 255 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0, packet 9 ( NID_LTRBG = "BGx") is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 90 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | BGx | Level 2/3 transition location balise group. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 9 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_LTRBG | 24 | BGx | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / SR / UN / SB / TR / LS / SN / PS | 0 / 1 / 2 / 4 / 6 / 7 / 12 / 13 / 15 |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | For L1 section |
| Level Transition Order | STORED | Level transition order for Level 2/3 is announced |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |