| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3050300 (3.5.3: Establishing a communication session) Total: 31 Test case(s) |  |
| REF: | Subset-076-5-2-3050300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.10.2013 | All | review | DLR |
| Version 2.9.2 02.12.2013 | All | review | DLR |
| Version 2.9.3 06.12.2013 | 14 | Changed Level in Sequence of test from L2 to L1 | DLR |
| Version 2.9.4 08.01.2014 | All | review | DLR |
| Version 2.9.5 14.01.2014 | All | review | DLR |
| Version 2.9.6 21.01.2014 | All | review | DLR |
| Version 2.9.7 23.01.2014 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Onboard receives a “session management” packet, NID_RBC is set to 16383 (Contact last known RBC), but there is no RBC_ID/phone number stored on-board. The order is IGNORED. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, LS, NL, LS, RV L2: FS, LS, OS, SR, SL, SB, TR, PT, LS, NL, LS, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, LS, NL, LS, RV |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 “Session management“ with an NID_RBC = 16383 (Contact last known RBC). • And no RBC_ID/phone number is stored onboard (e.g. during SoM) • The on-board shall ignore the order to contact the RBC |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.13 |  |
|  |  | Subset-026-3.5.3.13.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. |
| Constraints | Mode PT applicable for level 2/3: only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp; for level 1: rejected |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| RBC ID/Phone Number | UNKNOWN | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Radio link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 and packet 255 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train DOES NOT try to set up a safe connection. The order to Contact last known RBC is ignored. Because of the last known RBC and RBC phone number is unknown | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | NOT ESTABLISHED | DISCONNECTED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train is already in session with an RBC, but the onboard device receives and accepts a order to establish a communication session. NO RBC/RBC Handover is engaged. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, TR, NL LNTC: SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, LS, NL, LS, RV L2: FS, LS, OS, SR, SL, SB, TR, PT, LS, NL, LS, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, LS, NL, LS, RV |  |  |
| Target of Test | The target of this test is to check that: After receiving of an order to establish a communication session: • the onboard shall terminate the existing communication session • the onboard shall establish a session using the attributes of received message. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.5.2 |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 156 “Termination of a communication session” is RECORDED • Message 39 “Acknowledgement of termination of a communication session” is RECORDED • Message 159: Session established is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED to RBC A |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 (Q_RBC=1) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Steps 3-7 AND 8-17 can occur parallel if OBU is equipped with two mobiles. Otherwise in this order if only with one mobile. The Message is sent to the RBC A Communication session with RBC A terminated for onboard | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | The Message is received from RBC A. Communication session with RBC A terminated for onboard. Steps 10 to 13 can eventually take place later in the test sequence, i.e. reception of message 39 from RBC A is not necessary for sending message 159 to RBC B. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The primitive is sent to RBC A. | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Steps 3-7 AND 8-17 can occur parallel if OBU is equipped with two mobiles. Otherwise in this order if only with one mobile. The primitive is sent to RBC B. | N/A | N/A | - |
| 9 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | Communication session with RBC B established for onboard. | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 20 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 21 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 12: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 14: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 16: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 18: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 20: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Due to a RBC Handover, a OBU contacts a RBC for the first time. The onboard equipment shall check the ERTMS/ETCS system version transmitted. If the version is not compatible, the on-board equipment shall reject any information coming from the contacted RBC and continue with the previously received MA. The on-board equipment shall send a version independent message indicating No compatible version found and then terminate the communication session. An indication shall be given to the driver. |  | 3 |
| Applicable Mode/Level Combinations | L2, L3: FS, LS, OS, SR, SL, TR, NL |  |  |
| Target of Test | The target of this test is to check that : • The on-board equipment checks the ERTMS/ETCS system version after contacting the new RBC • The on-board accepts no information from Accepting RBC if versions are not compatible. • The on-board sends a message indicating No compatible version found. • The on-board terminates the communication session. • The on-board uses the previously received MA. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.3.7 |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM that: • The messages with the Accepting RBC are exchanged with the aim to establish communication session (exchange of radio message 155, 32). • The message 154 No compatible version is send. Check on JRU that: • The radio message 155 sent to the Accepting RBC is recorded. • The radio message 32 received from the Accepting RBC is recorded. • The radio message 154 sent to the Accepting RBC is recorded. |
| Constraints | A safe connection is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL / LS |
| Radio communication session | NOT ESTABLISHED | The communication with the accepting RBC is not established yet. The communication with the handing over RBC is established in case two communication sessions can be handled simultaneously, or terminated otherwise. |
| Movement Authority | STORED | - |
| RBC Transition Order | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | Packet Number 131 NID_C+NID_RBC="Accepting RBC ETCS identity" NID_RADIO="“Accepting” RBC radio subscriber number" |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | The radio link to the accepting RBC is not set-up yet. The radio link to the handing over RBC is set-up in case two communication sessions can be handled simultaneously, or not set-up otherwise. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | ERTMS/ETCS system version of RBC message is not compatible with on-board unit ERTMS/ETCS system version, i.e. M_VERSION ≠ 010 XXXX | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Trackside not compatible" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23, SYSTEM_STATUS_MESSAGE = <Bit09=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "No compatible version supported" (NID_MESSAGE=154) is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT3050500.6 “A termination of the communication session is required. The on-board equipment sends a radio message 156 “Termination of communication session” to the RBC. It receives from the RBC a radio message 24 “General message” requiring an acknowledgement. The message is ignored. On reception of the radio message 39 “Acknowledgement of termination of communication session” from the RBC, the onboard equipment considers the communication session terminated and releases the safe connection.“ | - | - | - | N/A | N/A | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | INVALID | ERTMS/ETCS system version of RBC message is not compatible with onVersion of the ERTMS/ETCS systemboard unit ERTMS/ETCS system version, i.e. M_VERSION ≠ 010 XXXX |

| Step 11: Radio Message 154 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 154 | No compatible version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 7 / 11 / 12 | FS / OS / SR / SL / TR / NL / LS |
| Radio communication session | NOT ESTABLISHED | The communication session is not established with the Accepting RBC |
| Movement Authority | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Radio link is not CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | System version is compatible and session is established. |  | 4 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 the on-board tries to initiate a communication session with the RBC via RBC ID and RBC phone number • The on-board shall establish a communication session if ordered from the trackside. • The establishment of a communication session shall be performed according to the following steps: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) b) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-3.17.3.7 |  |
|  |  | Subset-026-4.4.6.1.10 |  |
|  |  | Subset-026-4.4.6.1.10 a) |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 159: Session established is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_RBC≠16383) containing packet 42 (NID_RADIO ≠ FFFF FFFF FFFF FFFF) is received | I | BTM | Not use last known RBC and don’t use short number. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | ≠ 16383 | NOT Contact last known RBC |
| NID_RADIO | 64 | ≠ FFFF FFFF FFFF FFFF | NOT use short number |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio connection until the connection is established. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries to establish min 4 times a safe radio connection. Connection attempts are repeated until the connection is established. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.13 |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (1, 2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information to establish a communication session • The train tries to set up a safe radio connection. (min 4x times) • Save radio connection is set up |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| National Values | STORED | T_NVCONTACT = ∞ |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-Up | - | RTM | Radio Link is DISCONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 42 (NID_RBC=16383) and packet 255 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This stepshall be repeated for more than 45 seconds. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio connection until an End of Mission is performed. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until an EoM is performed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (1, 2 or 3), and has no active communication session. • The train overpasses a Balise group with information to establish a communication session • The train tries to set up a safe radio connection. (min 4x times) • The train stops • An End of Mission is performed |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| National Values | STORED | T_NVCONTACT = ∞ |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS / SR / LS | Train is moving | I | INT | - | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 2 | L1 / L2 / L3 | FS / OS / SR / LS | One Balise Group message containing packet 42 and packet 255 is received | I | BTM | - | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 3 | L1 / L2 / L3 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 4 | L1 / L2 / L3 | FS / OS / SR / LS | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step is repeated for more than 45 seconds and at least four times. | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 5 | L1 / L2 / L3 | FS / OS / SR / LS | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 6 | L1 / L2 / L3 | FS / OS / SR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 7 | L1 / L2 / L3 | FS / OS / SR / LS | Train is at standstill | I | INT | - | L1 / L2 / L3 | FS / OS / SR / LS | - |
| 8 | L1 / L2 / L3 | FS / OS / SR / LS | Cab not active | I | TIU | The driver closes the Desk. The Onboard switches to SB mode. | L1 / L2 / L3 | SB | - |
| 9 | L1 / L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | Closing of desk is RECORDED on the JRU. | L1 / L2 / L3 | SB | - |
| 10 | L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Mode change to SB is recorded | L1 / L2 / L3 | SB | - |
| 11 | L1 / L2 / L3 | SB | NOT SA-CONNECT.Request is transmitted | O | RTM | No more attempts to establish a safe connection are made | L1 / L2 / L3 | SB | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio connection until an order to terminate the communication session is received from trackside. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until an order to terminate the communication session is received from trackside |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (1, 2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information’s to establish a communication session • The train tries to set up a save radio connection. (min 4x times) • The train overpasses a Balise group with information to terminate a communication session • The train stops trying to set up a safe radio connection. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| National Values | STORED | T_NVCONTACT = ∞ |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Radio Network ID | STORED | Last RBC ID is known |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 42 (NID_RBC=16383) and packet 255 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step is repeated for more than 45 seconds and at least four times. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | One Balise Group message containing packet 42 (Q_RBC=0) and packet 255 is received | I | BTM | - | N/A | N/A | - |
| 8 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train stops trying set up a save radio connection to RBC X | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | Removal of symbol is recorded. Nothing is displayed in area E1 on the DMI. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio connection until the train passes with its front end a level transition border from a level 2/3 area to an area where level 2/3 operation is not supported. |  | 8 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until the train passes a level transition border (from level 2/3 to level 0, NTC, 1) with its front end. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information to establish a communication session • The train overpasses a Balise group with information to switch to Level y (0, NTC, 1) • The train tries to set up a save radio connection. (min 4x times) • The train overpasses the position where the level transition border is with its front end. • The train stops trying to set up a safe radio connection. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / LS | Train is moving | I | INT | - | L2 / L3 | FS / OS / SR / LS | - |
| 2 | L2 / L3 | FS / OS / SR / LS | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | NID_C = 352 ; NID_RBC = 1515 ; NID_RADIO = 003265342101FFFF | L2 / L3 | FS / OS / SR / LS | - |
| 3 | L2 / L3 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 4 | L2 / L3 | FS / OS / SR / LS | One Balise Group message containing packet 41 (M_LEVELTR=0or1or2) and packet 255 is received | I | BTM | M_LEVELTR = 0 or 1 or 2 | L2 / L3 | FS / OS / SR / LS | - |
| 5 | L2 / L3 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 6 | L2 / L3 | FS / OS / SR / LS | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step is repeated for more than 45 seconds and at least four times. | L2 / L3 | FS / OS / SR / LS | - |
| 7 | L2 / L3 | FS / OS / SR / LS | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | L2 / L3 | FS / OS / SR / LS | - |
| 8 | L2 / L3 | FS / OS / SR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 9 | L2 / L3 | FS / OS / SR / LS | Train is moving (V_TRAIN=40) | I | INT | The train switches to Level y (0, NTC, 1). If switch to L1, mode remains unchanged. If switch to L0, new mode is UN. If switch to LNTC, new mode is SN. | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 10 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2>) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 11 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | NTC SELECTED (NID_MESSAGE_JRU=26) is recorded | O | JRU | OPTIONAL: Shall be executed if changed to NTC only | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 12 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | The Level symbol "Level 0" is displayed OR The Level symbol "Level NTC" is displayed OR The Level symbol "Level 1" is displayed | O | DMI | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 13 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1¦Bit02=1¦Bit03=1>) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 14 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | NOT SA-CONNECT.Request is transmitted | O | RTM | Train stops trying to set up a safe radio connection. | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 15 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 16 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | Removal of symbol is recorded. Nothing is displayed in area E1 on the DMI. | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 1515 | - |
| NID_RADIO | 64 | 003265342101FFFF | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | FINITE VALUE | Design |
| M_LEVELTR | 3 | 0 or 1 or 2 | Level y (0, NTC, 1) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). Depends on test sequence |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio connection until an order to establish communication session with a different RBC is received from trackside and the order does not request to contact an Accepting RBC. |  | 9 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until an order to establish communication session with a different RBC is received from trackside and the order does not request to contact an Accepting RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (1, 2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information’s to establish a communication session to RBC X • The train tries to set up a save radio connection. (min 4x times) • The train overpasses a Balise group with information to establish a communication session to RBC Y • The train stops trying to set up a safe radio connection. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | NID_C=352 ; NID_RBC = 1515 ; NID_RADIO = 003265342101FFFF | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step is repeated for more than 45 seconds and at least four times. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1616; NID_RADIO=003265342102FFFF) and packet 255 is received | I | BTM | NID_C=352; NID_RBC = 1616 ; NID_RADIO = 003265342102FFFF | N/A | N/A | - |
| 8 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train stops trying set up a safe radio connection to RBC X (NID_RBC 1515) | N/A | N/A | - |
| 10 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | Removal of symbol is recorded. Nothing is displayed in area E1 on the DMI. | N/A | N/A | - |
| 12 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection with RBC Y with NID_RBC 1616. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 1515 | RBC X |
| NID_RADIO | 64 | 003265342101FFFF | RBC X radio subscriber number |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 1616 | - |
| NID_RADIO | 64 | 003265342102FFFF | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio until the train passes a RBC/RBC border with its front end. |  | 10 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until the train passes a RBC/RBC border with its front end. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 f) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information to establish a communication session to RBC X • The train overpasses a Balise group with an RBC transition order information • The train tries to set up a safe radio connection. (min 4x times) • The train passes a RBC/RBC border with its front end. • The train stops trying to set up a safe radio connection to the first RBC X |
| Constraints | There is only one mobile terminal available onboard |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | - | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection with RBC with NID_RBC 1515. This step is repeated for more than 45 seconds and at least four times. | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 131 (NID_C=352; NID_RBC=1616; NID_RADIO=003265342102FFFF) and packet 255 is received | I | BTM | - | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving | I | INT | The train passes a RBC/RBC border with its front end and stops trying to set up a safe radio connection to RBC X (NID_RBC=1515) | N/A | N/A | - |
| 10 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train stops trying set up a save radio connection to RBC X (NID_RBC=1515) | N/A | N/A | - |
| 11 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection with RBC Y with NID_RBC 1616. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 1515 | RBC X |
| NID_RADIO | 64 | 003265342101FFFF | RBC X radio subscriber number |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_RBCTR | 15 | FINITE VALUE | Depends on test sequence |
| NID_C | 10 | 352 | Acceptin. RBC identity.; Depends on test sequence |
| NID_RBC | 14 | 1616 | RBC Y |
| NID_RADIO | 64 | 003265342102FFFF | Accepting. RBC (RBC Y) radio subscriber number. |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RBC, the train tries to set up a safe radio until the train front passes the start of an announced radio hole. |  | 11 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until the train front passes the start of an announced radio hole. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 g) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level x (2 or 3), and has no active communication session. • The train tries to set up a safe radio connection. (min 4x times) • The train passes the begin of an announced radio hole. • The train stops trying to set up a safe radio connection. |
| Constraints | The beginning of the announced radio hole (information stored) is chosen such that the onboard tries at least four times to set-up a connection. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Track conditions excluding big metal masses | STORED | Starting at point x. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | NID_C=352 ; NID_RBC = 1515 ; NID_RADIO = 003265342101FFFF Received at or before x - 500m. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection with RBC with NID_RBC 1515. This step is repeated for more than 45 seconds and at least four times. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving @MaxFront | I | INT | The train enters the area of an announced radio hole and stops trying set up a save connection to RBC X | N/A | N/A | - |
| 8 | N/A | N/A | NOT SA-CONNECT.Request is transmitted @MaxFront | O | RTM | Train stops trying set up a save radio connection to RBC X | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | Removal of symbol is recorded. Nothing is displayed in area E1 on the DMI. | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 1515 | - |
| NID_RADIO | 64 | 003265342101FFFF | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The train receives an order to establish a communication session to RIU, the train tries to set up a safe radio until Level 1 is left |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | • The train tries min 4 times to establish a safe radio connection. Connection attempts are repeated until Level 1 is left. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 h) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The train is NOT at standstill. • The Onboard is in Level 1, and has no active communication session to RIU. • The train overpasses a Balise group with Radio infill area information. • The train overpasses a Balise group with information’s to switch to Level y (0, NTC, 2, 3) • The train tries to set up a safe radio connection. (min 4x times) • The train overpasses the position where the level transition border is with its front end. • The train stops trying to set up a safe radio connection. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / LS | Train is moving | I | INT | - | L1 | FS / OS / SR / LS | - |
| 2 | L1 | FS / OS / SR / LS | One Balise Group message containing packet 133 (NID_C=352; NID_RIU=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | NID_C=352 ; NID_RIU = 1515 ; NID_RADIO = 003265342101FFFF | L1 | FS / OS / SR / LS | - |
| 3 | L1 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / SR / LS | - |
| 4 | L1 | FS / OS / SR / LS | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection with RBC with NID_RBC 1515. This step is repeated for more than 45 seconds and at least four times. | L1 | FS / OS / SR / LS | - |
| 5 | L1 | FS / OS / SR / LS | One Balise Group message containing packet 41 (M_LEVELTR=0 or 1 or 3 or 4) and packet 255 is received | I | BTM | M_LEVELTR = 0 or 1 or 3 or 4 | L1 | FS / OS / SR / LS | - |
| 6 | L1 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / SR / LS | - |
| 7 | L1 | FS / OS / SR / LS | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 seconds after first connection attempt | L1 | FS / OS / SR / LS | - |
| 8 | L1 | FS / OS / SR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | L1 | FS / OS / SR / LS | - |
| 9 | L1 | FS / OS / SR / LS | Train is moving | I | INT | The train switches to Level y (0, NTC, 2, 3). If switch to L2 or L3, mode remains unchanged. If switch to L0, new mode is UN. If switch to LNTC, new mode is SN. | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 10 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=0 or 1 or 3 or 4) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 11 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | NTC SELECTED (NID_MESSAGE_JRU=26) is recorded | O | JRU | OPTIONAL: Shall be execcuted if changed to NTC only | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 12 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | The Level symbol "Level 0" is displayed OR The Level symbol "Level NTC" is displayed OR The Level symbol "Level 2" is displayed OR The Level symbol "Level 3" is displayed | O | DMI | - | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 13 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1¦Bit02=1¦Bit04=1¦Bit05=1>) is recorded | O | JRU | Level 0 MASK=0000000000000000000001; Level NTC MASK=0000000000000000000002; Level 2 MASK=0000000000000000000008; Level 3 MASK=0000000000000000000010 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 14 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | NOT SA-CONNECT.Request is transmitted | O | RTM | Train stops trying set up a save radio connection to RIU | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 15 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |
| 16 | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | Removal of symbol is recorded. Nothing is displayed in area E1 on the DMI. | L0 / LNTC / L2 / L3 | FS / OS / SR / UN / LS / SN | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| Q_RIU | 1 | 1 | Establishing Communication session |
| NID_C | 10 | 352 | RIU ETCS identity. |
| NID_RIU | 14 | 1515 | - |
| NID_RADIO | 64 | 003265342101FFFF | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | 0 or 1 or 3 or 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The onboard equipment establishes a communication session when the previous communication session is considered as terminated due to loss of safe radio connection |  | 13 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | • When the previous communication session is considered as terminated due to loss of safe radio connection, the Onboard should establish a communication session (refer to 3.5.4.2.1). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 f) |  |
|  |  | Subset-026-3.5.4.1 |  |
|  |  | Subset-026-3.5.4.2 |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.2 |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The Onboard is in Level x (2 or 3), and has an active communication session. • The connection is considered as terminated due to loss of safe radio connection • The Onboard tries to re-establish the communication session • The Session is re-established |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | At time t=t0, the OBU loses the safe radio connection (For Tester: unplug signal cabel or turn off the modul for sending RBC messages) | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. This step shall be repeated until t=t0+45. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | At time t=t0+45s, the connection status timer expires and the driver is informed that the safe connection is lost | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit041=1>) is recorded | O | JRU | The loss of safe connection is RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train keeps trying to set up a safe connection. This step is repeated until t=t0+5min. | N/A | N/A | - |
| 6 | N/A | N/A | Communication session expires (t=t0+5min) | O | INT | OBU considers the session as terminated. | N/A | N/A | - |
| 7 | N/A | N/A | Use_FT3050300.28 "A communication session initiated by the on-board unit is established successfully" | - | - | After the communication session is terminated the on-board equipment keeps trying to set up a safe connection. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | The Session management with neighbouring RIU shall be accepted by on-board equipment according to 4.8.3 table. Tested action: A by modes and level |  | 14 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS /SR |  |  |
| Target of Test | The target of this test case is to check that: - The on-board accepts the telegram from a BG containing packet 143: Session management with neighbouring RIU .(Q_RIU=1) - The on-board equipment initiates a communication session with the RIU through RIU identity and telephone number. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) a) |  |
|  |  | Subset-026-4.8.3.1.1 Session Management with neighbouring RIU |  |
|  |  | Subset-026-4.8.4.2 Session Management with neighbouring RIU |  |
|  |  | Subset-026-7.4.2.37.1 Packet Number 143: Session management with neighbouring Radio Infill Unit |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: - Reception of packet 143 is RECORDED. - The establishment of a communication session with RIU via balise group order is recorded. |
| Constraints | Communication session with RIU is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2/ 11 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | with RIU A |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | Inside the infill area |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 143 (Q_DIR=1; Q_RIU=1) is received | I | RTM | Packet is accepted. An order to connect to RIU through RIU identity and telephone number is received. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The on-board tries to set up a connection with the phone number included in the session management packet for RIU. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Safe connection is set-up | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 143 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | RIU B is a value different from the value for RIU A |
| NID_RADIO | 64 | FINITE VALUE | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2/ 11 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | Communication with RIU B |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Onboard receives a “session management” packet, NID_RBC is set to 16383 (Contact last known RBC), but a communication session is currently being established or is already established with that RBC. The order is IGNORED. |  | 15 |
| Applicable Mode/Level Combinations | L0: UN, SB, TR, NL LNTC: SB, TR, NL, SN L1: FS, LS, OS, SR, SL, SB, TR, NL, RV L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - a communication session is currently being established or is already established with a given RBC/RIU, and - an order to establish a communication session is received from the trackside - no new communication session is established |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4.1 |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED |
| Constraints | A communication session is currently being established or is already established with an RBC |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | Established or being established |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 (NID_RBC=16383) and packet 255 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train DOES NOT try to set up a new communication session. The order to Contact last known RBC is ignored because another session is currently being established or is already established with last known RBC. | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | When there is a mode change, not considered as an End of Mission, the on-board shall not establish a new communication session if a communication session is currently being established or is already established |  | 16 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that if: - a communication session is currently being established or is already established with a given RBC, and - a mode change, not considered as an End of Mission, has to be reported to the RBC - no new communication session is established |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4.1 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: - Train DOES NOT try to set up a safe connection. - The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. Check on the JRU that : - Message 136: “Train position report” is RECORDED |
| Constraints | On-board equipment has just switched into one of the applicable modes. Modes SH and SB only applicable if previous mode was PT (otherwise the switch to SH or SB would be considered as an End of Mission). A communication session is currently being established or is already established with an RBC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB /TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | Established or being established |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | Train DOES NOT try to set up a new communication session. No communication session is established because another session is currently being established or is already established with last known RBC | N/A | N/A | - |
| 2 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The on-board informs trackside about mode change | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 | FS (if preceded by FT4060300.TC21. Otherwise change accordingly) |
| M_LEVEL | 3 | 3,4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | When the driver has manually changed the level to 2 or 3, the on-board shall not establish a new communication session if a communication session is currently being established or is already established |  | 17 |
| Applicable Mode/Level Combinations | L0: UN, SB, NL LNTC: SB, NL, SN L1: FS, LS, OS, SR, SB, NL L2: FS, LS, OS, SR, SB, NL L3: FS, LS, OS, SR, SB, NL |  |  |
| Target of Test | The target of this test case is to check that if: - a communication session is currently being established or is already established with a given RBC, and - the driver has manually changed the level to 2 or 3, then - no new communication session is established |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: - Train DOES NOT try to set up a safe connection. - The onboard informs trackside about level change: Message 136: “Train Position Report” is SENT. Check on the JRU that : - Message 136: “Train position report” is RECORDED |
| Constraints | A communication session is currently being established or is already established with an RBC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / OS / SR / UN / SB / NL / LS / SN |
| Radio communication session | ESTABLISHED | Established or being established |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | Includes at least levels 2 and 3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| The Main Window is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 2 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | The driver selects "Level" | I | DMI | - | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 3 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | The ERTMS/ETCS level window is displayed | O | DMI | - | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 4 | L0 / L1 / L2 / L3 | FS / OS / SR / UN / SB / NL / LS / SN | The driver selects "Level 2" OR The driver selects "Level 3" | I | DMI | The driver selects Level 2 or 3 (the one that is not currently active) | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 5 | L0 / L1 / L2 / L3 | FS / OS / SR / SB / NL / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0010 0100 OR M_DRIVERACTIONS=0010 0101) is recorded | O | JRU | DRIVER’S ACTION is recorded | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 6 | L0 / L1 / L2 / L3 | FS / OS / SR / SB / NL / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=3 or 4) is recorded | O | JRU | Level change is recorded | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 7 | L2 / L3 | FS / OS / SR / SB / NL / LS | The Level symbol "Level 2" or "Level 3" is displayed | O | DMI | The Level symbol "Level 2" or “Level 3” is displayed | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 8 | L2 / L3 | FS / OS / SR / SB / NL / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1¦Bit05=1>) is recorded | O | JRU | If Level 2: Bit 4; If Level 3: Bit 5. | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 9 | L2 / L3 | FS / OS / SR / SB / NL / LS | NOT SA-CONNECT.Request is transmitted | O | RTM | No new communication session is established because another session is currently being established or is already established with last known RBC | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 10 | L2 / L3 | FS / OS / SR / SB / NL / LS | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 11 | L2 / L3 | FS / OS / SR / SB / NL / LS | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 12 | L2 / L3 | FS / OS / SR / SB / NL / LS | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The on-board informs trackside about level change. | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 13 | L2 / L3 | FS / OS / SR / SB / NL / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |

| Step 12: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 the newly entered level |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | Includes at least levels 2 and 3 |
| Driver ID | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| The driver is informed on of the change to L2 or L3 | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | When the train front reaches the end of an announced radio hole, the on-board shall not establish a new communication session if a communication session is currently being established or is already established |  | 18 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, TR, PT, NL L3: FS, LS, OS, SR, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test case is to check that if: - a communication session is currently being established or is already established with a given RBC, and - the train front reaches the end of an announced radio hole, then - no new communication session is established |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: - Train DOES NOT try to set up a safe connection. Check on the JRU that : - A message containing packet 68 (Track Condition) indication the start of a radio hole (M_TRACKCOND = 0100) is RECORDED |
| Constraints | A communication session is currently being established or is already established with an RBC Current level is 2 or 3 (in order for the communication session to be possibly established) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | Radio link is CONNECTED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 68 (M_TRACKCOND=0100) | I | RTM | M_TRACKCOND = 0100 - Radio hole. After receiving the message the train reaches the start of the announced radio hole. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving @MinFront | I | INT | The train front reaches the end of the announced radio hole while the communication session is still established, i.e. within 5 minutes. | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | No new communication session is established because another session is currently being established or is already established | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | 0100 | M_TRACKCOND = 0100 - Radio hole |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Onboard receives a “session management” packet, the system version is compatible and session is established.and NID_RBC is set to 16383 (Contact last known RBC). |  | 19 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV. |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 “Session management “ with an NID_RBC = 16383 (Contact last known RBC). • The on-board shall contact the last known RBC • The phone number indicated in the order shall be ignored by the onboard device • The function report train position when establishing a session with RBC is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.13 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when establishing a session with RBC |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Train Rejected |  |
|  |  | Subset-026-3.17.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. |
| Constraints | Q_Sleepsession received = 1 for the SL mode An ETCS ID of an RBC last connected to is known onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_RBC=16383) containing packet 42 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request (NID_RADIO = x) is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard does not try to set up a connection with the phone number included in the session management packet, but with the one stored before receiving the balise message of step 1. The support for the Mobile Network ID within the request is optional. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm (NID_RBC = y) is received | I | RTM | Setting up of a safe connection is successful. Check that the responding NID_RBC confirms to a specific number of a last known RBC stored before. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>; NID_ENGINE=76000) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | NOT RELEVANT | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | Contact last known RBC |
| NID_RADIO | 64 | NOT RELEVANT | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Onboard receives a “session management” packet, the system version is not compatible and session is not established. |  | 20 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV. |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 “Session Management” the onboard shall contact the last known RBC • The on-board shall establish a communication session • The establishment of a communication session shall be performed according to the following steps: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established • When the versions are not compatible, the onboard shall inform the trackside. • When the trackside receives the session established report or the information that the versions are not compatible, it shall consider the communication session as established. • The incompatibility of system version is an error condition that triggers the on-board to request the termination of the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) b) |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.13 |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.3.1.1 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.6.12 Message 154: No compatible version supported |  |
|  |  | Subset-026-3.17.3.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Trackside not compatible |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • The communication error (ETCS system version incompatibility) is DISPLAYED. Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. • Message 155: “Initiation of a communication session“ is RECORDED • Message 32: “RBC/RIU System Version “ is RECORDED • Message 154: “No compatible version supported“ |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | One Balise Group message (NID_RBC=16383) containing packet 42 is received | I | BTM | NID_RBC = 16383 (Contact last known RBC) Onboard will initiate communication to last known RBC Not in the Start of Mission | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 2 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 3 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 4 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | SA-CONNECT.Confirm is received | I | RTM | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 5 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | If desk is open | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 6 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | If desk is open | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 7 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 8 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 9 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are NOT compatible. Communication session established for on-board. | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 10 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 11 | L2, L3 | FS, LS, OS, SR, SB, TR, PT, NL, RV | The system status message "Trackside not compatible" is displayed | O | DMI | If desk is open | L2, L3 | FS, LS, OS, SR, SB, TR, PT, NL, RV | - |
| 12 | L2, L3 | FS, LS, OS, SR, SB, TR, PT, NL, RV | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit 09>) is recorded | O | JRU | If desk is open | L2, L3 | FS, LS, OS, SR, SB, TR, PT, NL, RV | - |
| 13 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | SA-DATA.Request with Euroradio Message "No compatible version supported" (NID_MESSAGE=154) is transmitted | O | RTM | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 14 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |
| 15 | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | Use_FT3050500.7 "The on-board equipment terminates a communication session due to error" | - | - | Communication error leads to termination of communication session | L2, L3 | FS, LS, OS, SR, SL, SB, TR, PT, NL, RV | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | NOT RELEVANT | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | 16383 | Contact last known RBC |
| NID_RADIO | 64 | NOT RELEVANT | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | 000 XXXX | NonVersion of the ERTMS/ETCS systemcompatible version number |

| Step 13: Radio Message 154 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 154 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Communication error" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | System version is compatible and session is established. |  | 21 |
| Applicable Mode/Level Combinations | L0: UN, SL LNTC: SL, SN L1: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 the On-board tries to establish radio communication via RBC ID and trackside given short number • The on-board shall establish a communication session if ordered from the trackside. • The establishment of a communication session shall be performed according to the following steps: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.15 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-3.17.3.7 |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_RBC≠16383) containing packet 42 (NID_RADIO = FFFF FFFF FFFF FFFF) is received | I | BTM | NID_RBC ≠ 16383 (last known RBC) NID_RADIO = FFFF FFFF FFFF FFFF (Use the short number stored onboard) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request )(NID_RADIO = FFFF FFFF FFFF FFFF) is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard does try to set up a connection with the phone number using the short number. The support for the Mobile Network ID within the request is optional. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>; NID_ENGINE=76000) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | ≠ 16383 | NOT Contact last known RBC |
| NID_RADIO | 64 | FFFF FFFF FFFF FFFF | Use short number |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | System version is compatible and session is established. |  | 22 |
| Applicable Mode/Level Combinations | L0: UN, NL LNTC: SN, NL L1: FS, LS, OS, SR, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • After receiving Packet Number 42 the on-board tries to initiate a communication session with the RBC via RBC ID and RBC phone number • The on-board shall establish a communication session if ordered from the trackside. • The establishment of a communication session shall be performed according to the following steps: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-3.17.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Packet Number 42: “Session Management” is RECORDED. • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 159: Session established is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (NID_RBC≠16383) containing packet 42 (NID_RADIO ≠ FFFF FFFF FFFF FFFF) is received | I | BTM | Not use last known RBC and don’t use short number. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | JRU step “DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded”. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT4080401.1 | - | - | Message 129 and message 8 must follow because train data are available | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | ≠ 16383 | NOT Contact last known RBC |
| NID_RADIO | 64 | ≠ FFFF FFFF FFFF FFFF | NOT use short number |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Within the Start of Mission procedure, system version is compatible and session is established. |  | 23 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: • When the driver has entered the RBC ID and RBC Telephone number or when validated driver id if level 2/3 stored (RBC ID and phone number are stored) within the Start of Mission procedure, the on-board establishes a communication session with the RBC according to the following steps:. a) The on-board shall request the set-up of a safe connection with the trackside. This attempt shall be repeated until successful or a defined number of times. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 a) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-3.17.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 159: Session established is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | JRU step “DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded”. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 6 | SB |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Within the Start of Mission procedure, system version are not compatible and session is not established. |  | 24 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: a) The on-board shall request the set-up of a safe connection with the trackside. This attempt shall be repeated until successful or a defined number of times. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established • As versions are not compatible, it shall inform the trackside. • When the trackside receives the session established report or the information that versions are not compatible, it shall consider the communication session established. • The incompatibility of system version is an error condition that triggers on-board to request the terminating. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 a) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) b) |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.12 Message 154: No compatible version supported |  |
|  |  | Subset-026-3.17.3.7 |  |
|  |  | Subset-026-5.4.3.2 A31 |  |
|  |  | Subset-026-5.4.3.2 A32 |  |
|  |  | Subset-026-5.4.3.2 A32 b) |  |
|  |  | Subset-026-4.7.2 table - Output information Trackside not compatible |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 154: No compatible version supported is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful or a defined number of times. The onboard tries to set up a connection with the phone number entered in the „preceded by“ test case. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are NOT compatible. Communication session established for on-board. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The system status message "Trackside not compatible" is displayed | O | DMI | Communication error is ETCS system version incompatibility | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit 09>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "No compatible version supported" (NID_MESSAGE=154) is transmitted | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT3050500.7 "The on-board equipment terminates a communication session due to error" | - | - | - | N/A | N/A | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 11: Radio Message 154 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 154 | No compatible version |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside not compatible" is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Within the Start of Mission procedure, setting up of a safe connection is unsuccessful. |  | 25 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: • The on-board shall request the set-up of a safe connection with the trackside. This attempt shall be repeated until successful or a defined number of times. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 a) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-5.4.3.2 A32 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that : • The safe connection cannot be established Check on the DMI that: - The connection is not successful |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | First attempt. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Second attempt. According to EURORADIO specifications at the latest when the connection establishment timer, started in Step 1, reaches its time-out value of 40 s. | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Third attempt. According to EURORADIO specifications at the latest when the connection establishment timer, started in Step 2, reaches its time-out value of 40 s. | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | The driver is informed that the connection is unsuccessful | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Safe radio connection - Connection Lost/Set-Up failed" is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | When there is a mode change, neither considered as an End of Mission nor triggered from the completion of a Start of Mission procedure in level 2 or 3 during which no communication session could be established, the on-board shall establish a communication session. |  | 26 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • When there is a mode change, neither considered as an End of Mission nor triggered from the completion of a Start of Mission procedure in level 2 or 3 during which no communication session could be established: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 c) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 b) |  |
|  |  | Subset-026-3.6.5.1.4 h) |  |
|  |  | Subset-026-3.18.3.4 |  |
|  |  | Subset-026-4.8.4.2 Trackside constituent System Version |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-4.10.1.3 line RBC ID/Phone Number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 159: Session established is RECORDED. |
| Constraints | The RBC ID and telephone number needs to be previously memorised onboard in order to establish a communication session to the last known RBC. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / NL / LS / RV An unreported mode change has occurred |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | Optional Step When the cab is NOT active. | N/A | N/A | - |
| 4 | N/A | N/A | JRU step “DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded”. | O | JRU | Optional Step When the cab is NOT active. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only if the status of the train data is NOT VALID. | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only if the status of the train data is NOT VALID. | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT4080401.1: “Radio message with new Train Data and acknowledge of the RBC” | - | - | Only if the status of the train data is VALID. Packets 129 and 8 are exchanged. | N/A | N/A | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / SN / RV |
| M_LEVEL | 3 | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 11 / 12 / 14 | FS / OS / SR / SL / SB / TR / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | A communication session initiated by the on-board unit is established successfully |  | 28 |
| Applicable Mode/Level Combinations | L0: UN, SL LNTC: SL, SN L1: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • After any of the conditions for the OBU to establish a communication session as stated in Subset-026-3.5.3.4 is met, • The establishment of a communication session shall be performed according to the following steps: a) The on-board shall request the set-up of a safe connection with the trackside. b) As soon as the safe connection is set-up, the on-board shall send the message Initiation of communication session to the trackside. c) As soon as the trackside receives the information, it shall send the system version. • When the on-board receives the system version it shall consider the communication session established and it shall send a session established report to the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.7 d) |  |
|  |  | Subset-026-3.5.3.7 d) a) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-3.18.4.4.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message 155: Initiation of a communication session is RECORDED. • Message 32: RBC/RIU System Version is RECORDED. • Message 159: Session established is RECORDED. • Message 136: Train Position Report containing packet 0 is RECORDED |
| Constraints | The TC is preceeded by any of the conditions for the OBU to establish a communication session as stated in Subset-026-3.5.3.4 having been met |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>; NID_ENGINE=76000) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Only if the status of the train data is NOT VALID. | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 is transmitted | O | RTM | Only if the status of the train data is VALID. | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 7: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |

| Step 9: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / SN / RV |
| M_LEVEL | 3 | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| Step 12: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / SN / RV |
| M_LEVEL | 3 | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SL / SB / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | When a communication session is currently being established (i.e. at any time from the first request the set-up of a safe radio connection to the reception of the system version from trackside), the on-board shall abort the process of establishing it and shall release the safe radio connection if the train passes with its front end a level transition border from a level 2/3 area to an area where level 2/3 operation is not supported. |  | 29 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | The train passes the level transition border before the reception of the system version from trackside and shall immediately release the safe connection. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 d) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The Onboard is in Level x (2 or 3), and has no active communication session, and the last RBC ID is known. • The train overpasses a Balise group with information to establish a communication session • The train overpasses a Balise group with information to switch to Level y (0, NTC, 1) • The train sets up a safe radio connection. • The train overpasses the position where the level transition border is with its front end. • The train releases the safe radio connection. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / LS | One Balise Group message containing packet 42 (NID_C=352; NID_RBC=1515; NID_RADIO=003265342101FFFF) and packet 255 is received | I | BTM | - | L2 / L3 | FS / OS / SR / LS | - |
| 2 | L2 / L3 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 3 | L2 / L3 | FS / OS / SR / LS | SA-CONNECT.Request is transmitted | O | RTM | - | L2 / L3 | FS / OS / SR / LS | - |
| 4 | L2 / L3 | FS / OS / SR / LS | SA-CONNECT.Confirm is received | I | RTM | - | L2 / L3 | FS / OS / SR / LS | - |
| 5 | L2 / L3 | FS / OS / SR / LS | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | L2 / L3 | FS / OS / SR / LS | - |
| 6 | L2 / L3 | FS / OS / SR / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 7 | L2 / L3 | FS / OS / SR / LS | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2 / L3 | FS / OS / SR / LS | - |
| 8 | L2 / L3 | FS / OS / SR / LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 9 | L2 / L3 | FS / OS / SR / LS | One Balise Group message containing packet 41 (M_LEVELTR=0 or 1 or 2) and packet 255 is received | I | BTM | M_LEVELTR = 0 or 1 or 2 | L2 / L3 | FS / OS / SR / LS | - |
| 10 | L2 / L3 | FS / OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / LS | - |
| 11 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | The Level symbol "Level 0" is displayed OR The Level symbol "Level NTC" is displayed OR The Level symbol "Level 1" is displayed | O | DMI | The train reaches the level transition border and switches to Level y (0, NTC, 1). If switch to L1, mode remains unchanged. If switch to L0, new mode is UN. If switch to LNTC, new mode is SN. | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 12 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1¦Bit02=1¦Bit03=1>) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 13 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=0 or 1 or 2) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 14 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | NTC SELECTED (NID_MESSAGE_JRU=26) is recorded | O | JRU | OPTIONAL: Shall be executed if changed to LNTC only | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 15 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 16 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | SA-DISCONNECT.Request is transmitted | O | RTM | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 17 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |
| 18 | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / LS / SN | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | 352 | RBC ETCS identity |
| NID_RBC | 14 | 1515 | - |
| NID_RADIO | 64 | 003265342101FFFF | - |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 9: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | FINITE VALUE | Design |
| M_LEVELTR | 3 | 0 or 1 or 2 | Level y (0, NTC, 1) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). Depends on test sequence |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | 0 | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Setting up a safe connection to report a mode change is unsuccessful. |  | 30 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board shall request the set-up of a safe connection with the trackside. This attempt shall be repeated until successful or a defined number of times. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 c) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • The safe connection cannot be established |
| Constraints | A mode change, which has to be reported to the RBC as of 3.5.3.4 c), has just occured." |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS, OS, SR, SH, SL, SB, TR, PT, SF, NL, LS, RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Info: No SA-CONNECT.Confirm is received First attempt. | N/A | N/A | - |
| 2 | N/A | N/A | Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | 45 s after the first connection attempt | N/A | N/A | - |
| 3 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Second attempt. According to EURORADIO specifications at the latest when the connection establishment timer, started in Step 1, reaches its time-out value of 40 s. | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Third attempt. According to EURORADIO specifications at the latest when the connection establishment timer, started in Step 4, reaches its time-out value of 40 s. | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | 40 s after Step 5. There shall be no fourth attempt to establish a safe radio connection. | N/A | N/A | - |
| 7 | N/A | N/A | Safe radio connection - Connection Lost/Set-Up failed" is removed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS, OS, SR, SH, SL, SB, TR, PT, SF, NL, LS, RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.3: Establishing a communication session |  | 3050300 |
| Test Case of Feature | Manually changing the RBC information in L2/L3 outside the Start of Mission does not initiate setting up a new radio communication session when a connection is already set up or is being established. |  | 31 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR, PT, NL, LS L3: FS, OS, SR, PT, NL, LS |  |  |
| Target of Test | The target of this test case is to check that if: - a communication session is currently being established or is already established with a given RBC/RIU, and - the RBC information is manually selected - no new communication session is established |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4.1 |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 h) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24) is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 8 / 11 / 12 | FS / OS / SR / UN / PT / NL / LS |
| Radio communication session | ESTABLISHED | Established or being established |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| V_TRAIN=0 | I | TIU | Train is at standstill |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver enters "RBC Data" | I | DMI | The RBC data manually entered by the driver is equal to the RBC data of the session which is currently established/ being established. | N/A | N/A | - |
| 3 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2; NID_C=<same as current RBC>; NID_RBC=<same as current RBC>; NID_RADIO=<same as current RBC>) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Session Established" (NID_Message = 159) is transmitted | O | RTM | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 8 / 11 / 12 | FS / OS / SR / UN / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |