| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5040500 (5.4.5: Degraded Situations) Total: 16 Test case(s) |  |
| REF: | Subset-076-5-2-5040500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv28; Correction of Requirements blank space; Feature renumbered. | INECO |
| Version 2.9.2 23.10.2013 | All | Third version/ UNISIG comments from 14.10.2013 included; Dictionary steps used. | INECO |
| Version 2.9.3 10.12.2013 | All | Fourth version/ UNISIG comments from 09.12.2013 included; Changes only in TC3 | INECO |
| Version 2.9.4 03.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, if VALID Train data and Train running number are available the ERTMS/ETCS on-board equipment shall offer to driver the possibility to select “Override“, in addition to the possibilities described in section 5.4.3 (see Subset-026-5.4.3.2 S10 and Subset-026-5.4.3.2 S20) |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: It is possible to select “Override” at S10 and S20 during SoM and if valid Train data and Train running number are available. The driver selects “Override” and the process goes to the procedure ”Override” If the position is still invalid, the on-board deletes train position data |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.8.2.1 |  |
|  |  | Subset-026-5.8.2.1 c) |  |
|  |  | Subset-026-5.8.3.1 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.3 - Following 5.4.5.3 a), f), g), on-board deletes stored position data |  |
|  |  | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects “Override” and Override window is displayed. The Override EoA button is available in addition to all possibilities offered at S10 and S20 during SoM The driver selects “Override EoA”. Check on the JRU that : The driver‘s selection action is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10 and S20. The status of the Train Data and Train running number shall be VALID. The status of train position data can be VALID, UNKNOWN or INVALID. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | VALID / UNKNOWN / INVALID | The status of train position data can be VALID, UNKNOWN or INVALID. |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Default> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | The driver presses "Override” button. | I | DMI | - | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The < EoA> button in the <Override> window is enabled | O | DMI | The on-board equipment allows the driver to select “Override“. The override window is displayed and the “EoA“ button is available. (since the status of train data and Train running number are VALID) Starting conditions (in addition to all possibilities related to S10 and S20). | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | The driver selects "Override EoA" | I | DMI | Procedure “Override“ is triggered | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1110) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | The Mode symbol "Override EOA is active" is displayed | O | DMI | DMI Object: Symbol MO03 | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | Once the “Override procedure“is triggered the mode changes to Staff Responsible. | - | - | USE_FT4060300.56. Condition Id [37]. | L2 / L3 | SR | - |
| 7 | L2 / L3 | SR | Train is at standstill | I | INT | Only if Train position is still INVALID, the on-board equipment deletes train position data.(new status: UNKNOWN) | L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SR |
| Train Position | VALID / UNKNOWN | The status of train position data can be VALID or UNKNOWN. |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Default window is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | The on-board equipment is inside the SoM-Procedure and loses the already open session further than step D11 of SoM has occurred. |  | 2 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that: Within the scope of the Start of Mission procedure and in Level 2 or 3, if the on-board equipment loses radio contact further than in step D11 of SoM-Procedure, the process shall go to S10 in the SoM procedure. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.1 |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: The on-board terminates the communication session due to loss of radio link Check on the DMI that: The driver has the possibility to continue without session open. The main window with the list of the possibilities as described in S10 is DISPLAYED to the driver. Check on the JRU that: Driver selection is RECORDED |
| Constraints | This test case takes place within the scope of the Start of Mission procedure, covering a degraded situation for any step further than D11 (Session is open). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Stop Shunting on desk opening | OPEN | - |
| Train Data | VALID | the driver has successfully entered a complete set of data and validated them |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | STORED | - |
| Train Running Number | STORED | the driver has successfully re-entered /re-validated it |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Non Leading input signal is received | I | TIU | Optional: Applies only if driver selection is Non Leading |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board terminates the communication session due to loss of radio link. | - | - | USE_FT3050400.4 | N/A | N/A | - |
| 4 | N/A | N/A | The <Main> window is displayed. | O | DMI | The process goes to S10 inside the SoM-procedure. The driver has the possibility to select Train data entry or Non Leading or Shunting. | N/A | N/A | - |
| 5 | N/A | N/A | The driver selects "TRAIN DATA ENTRY/ NON LEADING/ SHUNTING" | I | DMI | Driver selects one of the displayed possibilities from the main window. Either Train data entry or Non Leading or Shunting | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 0100/ 0000 1100/ 0000 1011) is recorded | O | JRU | Check the value of M_DRIVERACTIONS. | N/A | N/A | - |
| 7 | N/A | N/A | Driver selection action is DISPLAYED. | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated train data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If N_ITER different from 0 |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If N_ITER different from 0 |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If N_ITER different from 0 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Stop Shunting on desk opening | OPEN | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | STORED | not affected by this test case |
| Train Running Number | STORED | UNCHANGED not affected by this test case |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Driver selection action is DISPLAYED. | O | DMI | Either Train data entry, or Non Leading or Shunting are ENABLE |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the selection to re-enter the Level shall be possible at S10 and S20. Then the process shall go to S2. |  | 3 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: It is possible to re-enter the ETCS level at S10 and S20 during SoM. After requesting to modify the Level the state S2 is reached. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 e) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects to re-enter the level at S10 and S20. The driver enters a new level The on-board request the driver to enter DRIVER ID Check on the JRU that : The new level is recorded Driver action is recorded |
| Constraints | This test case takes place within the scope of the Start of Mission procedure, covering a degraded situation at S10 and S20. No radio communication session is available. Applies to all levels. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | No radio link in L0 / LNTC / L1, degraded situation in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SB | The <Level> button in the <Main> window is enabled | O | DMI | In addition to all possibilities offered at S10 and S20. | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | The driver selects from the main window list to re-enter the ETCS level. | I | DMI | The process goes to S2 inside the SoM-procedure | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | The <ERTMS/ETCS Level> window is displayed. | O | DMI | The on-board allows the driver to re-enter the ETCS level. | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SB | The driver selects "Level <X>" | I | DMI | The driver re-enters the ERTMS/ETCS level. X= different or can be the same as the previous one. | LX | SB | - |
| 5 | LX | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110) is recorded | O | JRU | Check that M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110 Is one of this value | LX | SB | - |
| 6 | LX | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=X) is recorded | O | JRU | - | LX | SB | - |
| 7 | LX | SB | The Level symbol "Level X" is displayed. | O | DMI | DMI Object: LEXX Depends on the driver level selection | LX | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 (Level changed to same level or a different one) |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level X" is displayed | O | DMI | A new or same level could be displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure, after the driver entered a Driver ID, the driver is allowed to re-enter another Driver ID at S10 and S20. |  | 4 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that : The On-Board Equipment, at S10 andS20, permits the driver: to select from the main window list the possibility of re-entered DRIVER ID in addition to the other possibilities. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 b) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Driver can select 'Driver ID entry' at S10 and S20. The driver re-enters Driver ID. The driver enters a Driver ID different from the stored value. Check on the JRU that : The driver‘s selection action is recorded. The modified Driver ID is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Driver ID is VALID |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A Valid Train Data has been entered/re-validated by the driver |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <DRIVER ID> button in the <Main> window is enabled | O | DMI | In addition to all possibilities offered at S10 and S20 | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects from the list of the main window the Driver ID. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment allows the driver to re-enter the Driver Id. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver enters the "Driver ID" | I | DMI | The driver re-enters the Driver ID. | N/A | N/A | - |
| 5 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | Triggered by: Every 5 seconds. Driver ID to be checked in the message header. | N/A | N/A | - |
| 6 | N/A | N/A | The <Main> window is displayed. | O | DMI | The on-board returns to “Main window” | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A Valid Train Data has been entered/re-validated by the driver |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | Re-entered |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window is AVAILABLE | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the selection to re-enter the Train Data shall be possible at S20. Then the process shall go to S12. |  | 5 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: The On-board equipment, within the procedure start of Mission, state S20, permits the driver to select from the main window list the possibility to re-entered Train data in addition to other possibilities. After requesting to modify the train data the state S12 is reached. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 d) |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver can choose ‘Train Data Entry’ at S20. The driver re-enters ‘Train Data Entry’. The driver enters Train Data different from the stored value. Check on the JRU that : The driver‘s selection action is recorded. The modified Train Data Entry’ are recorded |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Train Data is VALID |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <START> button in the <MAIN> window is enable | O | DMI | Waiting for driver to select “START” |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <Train data> button in the <Main> window is enabled | O | DMI | In addition to “Start” button | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects from the main window list to re-enter the Train Data. | I | DMI | The process goes to S12 inside the SoM-procedure | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0100) is recorded | O | JRU | Triggered by : Train Data Entry requested | N/A | N/A | - |
| 4 | N/A | N/A | A complete set of Train Data is displayed. | O | DMI | The driver is allowed to re-enter the train data. Those data includes the additional train data (always) and the additional NTC data (if needed). | N/A | N/A | - |
| 5 | N/A | N/A | The driver enters the "Train Data" | I | DMI | The driver re-enters the train data. The on-board equipment goes to D10 state of SoM procedure | N/A | N/A | - |
| 6 | N/A | N/A | The driver validates the “Train data” | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0101) is recorded | O | JRU | Triggered by : Validation of Train Data | N/A | N/A | - |
| 8 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : When the train data are changed | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | Changed |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Train Running Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window is AVAILABLE | O | DMI | On board at S10 or S20 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment shall offer the driver the possibility to re-enter the “Train running number” only at S10 and S20. |  | 6 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: The On-Board Equipment, at S10 and S20, permits the driver: to select from the main window list the possibility of re-entered “Train running number” in addition to the other possibilities. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 c) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Driver can select from the main window list the option to “Train running number”' at S10 and S20. The driver re-enters the “Train running number”' different from the stored value. Check on the JRU that : The driver‘s selection action is recorded. New train running number is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10 and S20 The status of the Train data (including Train Running number) is VALID |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <Train running number> button in the <Main> window is enabled | O | DMI | In addition to all possibilities offered at S10 or S20 | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects from the list of the main window the “Train Running number”. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The actual <Train running number> in the <Train running number> window is enabled. | O | DMI | The on-board equipment allows the driver to re-enter the “Train running number”. | N/A | N/A | - |
| 4 | N/A | N/A | The driver enters the "Train Running Number" | I | DMI | The driver re-enterers a new “Train running number”. | N/A | N/A | - |
| 5 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL= <new value>) is recorded | O | JRU | Check that the train running number is different. | N/A | N/A | - |
| 6 | N/A | N/A | The <Main> window is displayed. | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Train Running Number | STORED | CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment shall offer the driver the possibility to set/remove a Virtual Balise Cover, only at S10 and S20. |  | 7 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: The On-Board Equipment, at S10 and S20, permits the driver to select from the main window list the possibility to set/remove a Virtual Balise Cover in addition to the other possibilities. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 i) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The possibility for the driver to select from the main window list the option to set/remove the VBC at S10 and S20. The driver set/removes the Virtual Balise Cover Check on the JRU that : The driver‘s selection action is recorded. New VBC is RECORDED. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <SETTINGS> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <Set VBC> button or <Remove VBC> button in the <Setting> window is enabled | O | DMI | In addition to all possibilities listed in the main window. | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects one of the option “Set VBC” or “Remove VBC“ | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment allows the driver to set or to remove the VBC. | O | DMI | Depending on driver selection window “Set VBC” or “Remove VBC“ is display | N/A | N/A | - |
| 4 | N/A | N/A | The driver sets a VBC and validates "Set VBC" | I | DMI | The driver enters a new VBC code. See paragraph 11.3.12.5 for Set VBC code and 11.3.13.5 for Remove VBC code ( ERA-ERTMS-015560 v3.3.0) | N/A | N/A | - |
| 5 | N/A | N/A | VIRTUAL BALISE COVER SET BY THE DRIVER (NID_MESSAGE_JRU=28) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Virtual Balise Covers | STORED | CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the possibility to select NL mode shall be given to the driver at S20. The on-board switches immediately to NL mode. The status of the train position data changes to UNKNOWN if the position is still INVALID. |  | 8 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S20( Waiting for driver selection of Start”), the On-Board equipment offers the driver the possibility to select NL The driver selects NL The on-board immediately switch to Non Leading Mode The on-board deletes train position data (new status: Unknown) only in L0/LNTC/L1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 f) |  |
|  |  | Subset-026-5.4.3.3 - Following 5.4.5.3 a), f), g), on-board deletes stored position data |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The possibility for the driver to select either Start or NL. The driver selects NL The mode changes to NL Check on the JRU that : The driver‘s selection action is recorded. New mode NL is recorded |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. At S20, the Status of train position data is INVALID only if starting condition levels are L0 / LNTC / L1 At S20, Radio Communication Established in L2/L3 and train position is VALID / UNKNOWN... |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2/L3 |
| Non Leading input signal is received | I | TIU | The non-leading input has the value “Non-leading permitted”. |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SB | The <Non Leading mode> button in the <Main> window is enabled | O | DMI | In addition to other possibilities offered at S20. | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SB | The driver selects "NON LEADING" | I | DMI | - | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1100) is recorded. | O | JRU | Non Leading selected | L0 / LNTC / L1 / L2 / L3 | SB | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=11) is recorded | O | JRU | The on-board equipment immediately switches from SB to NL mode.(E10) | L0 / LNTC / L1 / L2 / L3 | NL | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | NL | The Mode symbol "Non-leading" is displayed. | O | DMI | DMI Object: MO12 | L0 / LNTC / L1 / L2 / L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Train Data | UNKNOWN | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Non-leading" is displayed. | O | DMI | DMI Object: MO12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the possibility to select SH mode shall be given to the driver at S20. After driver selection of SH the process shall go to the procedure “SH initiated by driver”. The status of the train position data changes to Unknown only if is still INVALID. |  | 9 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S20( Waiting for driver selection of Start”), the On-Board equipment offers to the driver the possibility to select SH The driver selects SH The process goes to the procedure “SH initiated by driver”. The on-board deletes train position data only if the selected level for testing is L0/LNTC/L1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 g) |  |
|  |  | Subset-026-5.4.3.3 - Following 5.4.5.3 a), f), g), on-board deletes stored position data |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver can select either Start or SH. The driver selects SH The mode changes to SH Check on the JRU that : The driver‘s selection action is recorded. New mode SH is recorded |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. At S20, the Status of train position data is INVALID only if starting condition levels are L0/LNTC/L1 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | in L0/ LNTC/ L1 |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SB | The <Shunting> button in the <Main> window is enabled | O | DMI | In addition to the possibilities offered at S20 | L0 / LNTC / L1 | SB | - |
| 2 | L0 / LNTC / L1 | SB | The driver selects "SHUNTING" | I | DMI | - | L0 / LNTC / L1 | SB | - |
| 3 | L0 / LNTC / L1 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1011) is recorded | O | JRU | Shunting selected | L0 / LNTC / L1 | SB | - |
| 4 | L0 / LNTC / L1 | SB | The procedure “SH initiated by driver” is triggered | - | - | USE_FT5060200.2 (for L0/ LNTC/ L1) | L0 / LNTC / L1 | SH | - |
| 5 | L0 / LNTC / L1 | SH | The <Default> window is displayed. | O | DMI | - | L0 / LNTC / L1 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Train Position | INVALID | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Shunting" is displayed. | O | DMI | DMI Object: MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the possibility to select SH mode shall be given to the driver at S20. After driver selection of SH the process shall go to the procedure “SH initiated by driver”. The RBC rejects the request of Shunting and the process goes back to S20. |  | 10 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S20( Waiting for driver selection of Start”), the On-Board equipment offers to the driver the possibility to select SH The driver selects SH The process goes to the procedure “SH initiated by driver”. The RBC refuse SH, the mode doesn’t change to SH and the process goes back to S20. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 g) |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver can select either Start or SH. The driver selects SH Check on the JRU that : The driver‘s selection action is recorded. No mode change to SH is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID / UNKNOWN | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | The <Shunting> button in the <Main> window is enabled | O | DMI | In addition to other possibilities offered at S20 | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The driver selects "SHUNTING" | I | DMI | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | The <Shunting> dialog in the <Sequence> window is disabled | O | DMI | - | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1011) is recorded | O | JRU | Shunting selected | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | The procedure “SH initiated by driver” is triggered | - | - | USE_FT5060200.4 (SH refused by RBC) | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | The <Main> window is displayed. | O | DMI | SB mode and the process goes back to S20. Symbol MO13 still displayed | L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID / UNKNOWN | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Stand By" is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select “Start. |  | 11 |
| Applicable Mode/Level Combinations | L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S10 ( Waiting for driver selection and Valid Train Data are available on-board The on-board equipment offers to the driver the possibility to select “Start” The driver selects “Start” The process goes to S24, SR mode proposed to the driver |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 h) |  |
|  |  | Subset-026-5.4.5.3 h) c) |  |
|  |  | Subset-026-5.4.5.3 h) e) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S24 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select ‘Start’ at S10. The driver selects ‘Start’, SR mode is proposed to the driver and the On-board equipment requests an acknowledgement from the driver for running in SR mode. The mode changes to SR mode Check on the JRU that : The selection by the driver of ‘Start’ is recorded. Driver acknowledgment for running in SR mode is RECORDED. Mode change to SR is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10. In level L2 or L3 no communication session shall be opened. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | A radio communication session is not established. |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. The Mode symbol "Stand By" is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SB | The <Start> button in the <Main> window is enabled. | O | DMI | In addition to all possibilities offered at S10 | L1 / L2 / L3 | SB | - |
| 2 | L1 / L2 / L3 | SB | The driver selects "Start" | I | DMI | - | L1 / L2 / L3 | SB | - |
| 3 | L1 / L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Start selected | L1 / L2 / L3 | SB | - |
| 4 | L1 / L2 / L3 | SB | The <Default> window is displayed. | O | DMI | - | L1 / L2 / L3 | SB | - |
| 5 | L1 / L2 / L3 | SB | The Mode symbol "Acknowledge Staff Responsible" is displayed | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode. DMI Object: MO10 | L1 / L2 / L3 | SB | - |
| 6 | L1 / L2 / L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SB | - |
| 7 | L1 / L2 / L3 | SB | The driver acknowledges "Staff Responsible" | I | DMI | - | L1 / L2 / L3 | SB | - |
| 8 | L1 / L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | Ack of Staff Responsible mode As soon as the driver acknowledges the on-board equipment switches to SR mode | L1 / L2 / L3 | SB | - |
| 9 | L1 / L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | Triggering Event: When the mode changes | L1 / L2 / L3 | SR | - |
| 10 | L1 / L2 / L3 | SR | The Mode symbol "Staff Responsible" is displayed | O | DMI | . DMI Object: Symbol MO09 | L1 / L2 / L3 | SR | - |
| 11 | L1 / L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | NOT ESTABLISHED | A radio communication session is not established. |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Staff Responsible" is displayed | O | DMI | DMI Object: MO09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select “Start. |  | 12 |
| Applicable Mode/Level Combinations | L0: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S10 ( Waiting for driver selection and Valid Train Data are available on-board) The On-Board equipment offers to the driver the possibility to select “Start” The driver selects “Start” The process goes to S23, UN mode proposed to the driver |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 h) |  |
|  |  | Subset-026-5.4.5.3 h) a) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S23 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select ‘Start’ at S10. The driver selects ‘Start’, UN mode is proposed to the driver and the On-board equipment requests an acknowledgement from the driver for running in UN mode. The mode changes to UN mode Check on the JRU that : The selection by the driver of ‘Start’ is recorded. Driver acknowledgment for running in UN mode is RECORDED. Mode change to UN is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 0 | L0 |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. The Mode symbol "Stand By" is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | SB | The <Start> button in the <Main> window is enabled. | O | DMI | In addition to possibilities offered at S10 | L0 | SB | - |
| 2 | L0 | SB | The driver selects "Start" | I | DMI | - | L0 | SB | - |
| 3 | L0 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Start selected | L0 | SB | - |
| 4 | L0 | SB | The <Default> window is displayed. | O | DMI | - | L0 | SB | - |
| 5 | L0 | SB | The Mode symbol "Acknowledge Unfitted" is displayed | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 | L0 | SB | - |
| 6 | L0 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded | O | JRU | - | L0 | SB | - |
| 7 | L0 | SB | The driver acknowledges "Unfitted" | I | DMI | - | L0 | SB | - |
| 8 | L0 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded | O | JRU | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode | L0 | SB | - |
| 9 | L0 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded | O | JRU | Triggering Event: When the mode changes | L0 | UN | - |
| 10 | L0 | UN | The Mode symbol "Unfitted" is displayed | O | DMI | DMI Object: Symbol MO16 | L0 | UN | - |
| 11 | L0 | UN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded | O | JRU | - | L0 | UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 0 | L0 |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "Unfitted" is displayed | O | DMI | DMI Object: Symbol MO16 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select “Start. |  | 13 |
| Applicable Mode/Level Combinations | LNTC: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S10 ( Waiting for driver selection and Valid Train Data are available on-board) The On-Board equipment offers to the driver the possibility to select “Start” The driver selects “Start” The process goes to S22, SN mode proposed to the driver |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 h) |  |
|  |  | Subset-026-5.4.5.3 h) b) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S22 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The On-Board equipment allows the driver to select ‘Start’ at S10. The driver selects ‘Start’, SN mode is proposed to the driver and the On-board equipment requests an acknowledgement from the driver for running in SN mode. The mode changes to SN mode Check on the JRU that : The selection by the driver of ‘Start’ is recorded. Driver acknowledgment for running in SN mode is RECORDED. Mode change to SN is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 1 | LNTC |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. The Mode symbol "Stand By" is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SB | The <Start> button in the <Main> window is enabled. | O | DMI | In addition to possibilities offered at S10 | LNTC | SB | - |
| 2 | LNTC | SB | The driver selects "Start" | I | DMI | - | LNTC | SB | - |
| 3 | LNTC | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Start selected | LNTC | SB | - |
| 4 | LNTC | SB | The <Default> window is displayed. | O | DMI | - | LNTC | SB | - |
| 5 | LNTC | SB | The Mode symbol "Acknowledge National System" is displayed | I | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in National system mode. DMI Object: Symbol MO20 | LNTC | SB | - |
| 6 | LNTC | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit35=1>) is recorded | O | JRU | - | LNTC | SB | - |
| 7 | LNTC | SB | The driver acknowledges "National System" | I | DMI | - | LNTC | SB | - |
| 8 | LNTC | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 1100) is recorded | O | JRU | Ack of SN mode As soon as the driver acknowledges the on-board equipment switches to SN mode | LNTC | SB | - |
| 9 | LNTC | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=13) is recorded | O | JRU | Triggering Event: When the mode changes | LNTC | SN | - |
| 10 | LNTC | SN | The Mode symbol "National System" is displayed | O | DMI | DMI Object: Symbol MO19 | LNTC | SN | - |
| 11 | LNTC | SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded | O | JRU | - | LNTC | SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 1 | LNTC |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol "National System" is displayed. | O | DMI | DMI Object: MO19 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select “Start. |  | 14 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: During SoM procedure and once at S10 ( Waiting for driver selection and Valid Train Data are available on-board The On-Board equipment offers to the driver the possibility to select “Start” The driver selects “Start” The process goes to S21, on board sends a MA request |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 h) |  |
|  |  | Subset-026-5.4.5.3 h) d) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S21 |  |
|  |  | Subset-026-4.5.2.1 table - Request MA Cyclically when “Start” is selected (only level 2/3) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-8.6.3 Message 132: MA Request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: On-board sends a MA request after “Start” button is selected Check on the DMI that : The On-Board equipment allows the driver to select ‘Start’ at S10. The driver selects ‘Start’, Check on the JRU that : The selection by the driver of ‘Start’ is recorded. MA request is RECORDED |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10. In level L2 or L3 communication session is established |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | A valid Train Running Number is available on-board |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | - |
| NOT RELEVANT | I/O | TIU | - |
| The <Main> window is displayed. The Mode symbol "Stand By" is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | The <Start> button in the <Main> window is enabled. | O | DMI | In addition to all possibilities offered at S10 | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | The driver selects "Start" | I | DMI | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Start selected | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | The <Default> window is displayed. | O | DMI | - | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 or 1 is transmitted. | O | RTM | - | L2 / L3 | SB | - |
| 6 | L2 / L3 | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 7 | L2 / L3 | SB | Transition to FS / SR / OS / SH / LS mode is performed | - | - | USE_FT4060300.48 for transition from SB to FS. Or USE_FT4060300.50 for transition from SB to SR. Or USE_FT4060300.42 for transition from SB to OS. Or USE_FT4060300.45 for transition from SB to SH. Or USE_FT4060300.60 for transition from SB to LS. | L2 / L3 | FS / SR / OS / SH / LS | - |

| Step 5: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | MA Request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 13 | FS / SR / OS / SH / LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol " FS / SR / OS / SH / LS " is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment shall offer the driver the possibility to re-enter the Radio data, only at S10 and S20. |  | 15 |
| Applicable Mode/Level Combinations | L2 / L3: SB |  |  |
| Target of Test | The target of this test case is to check that: It is possible to re-enter the Radio data at S10 and S20 during SoM. After requesting to modify the Radio data the state S3 is reached. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 j) |  |
|  |  | Subset-026-5.4.5.3 j) a) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects “Radio data” at S10 and S20 and Radio data window is displayed with the labels “Contact last RBC”, “Use short number”, “Enter RBC data” and “RBC Network ID”. The driver presses the ‘Enter RBC data’ button. The RBC data window is displayed The driver validate the RBC data Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10 and S20. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <Radio data> button in the <Main> window is enabled | O | DMI | In addition to all possibilities offered at S10 and S20. | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Radio data”. | I | DMI | The process goes to S3 inside the SoM-procedure | N/A | N/A | - |
| 3 | N/A | N/A | The <Radio data> window is displayed. | O | DMI | The on-board equipment displays the RBC data window and offers the possibility to the driver to select ‘Contact last RBC’, ‘Use short number’, ‘Enter RBC data’ and ‘Radio network ID’. | N/A | N/A | - |
| 4 | N/A | N/A | The driver presses the ‘Enter RBC data’ button | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The on-board equipment displays the RBC data window | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver validates the “RBC data” | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.4.5: Degraded Situations |  | 5040500 |
| Test Case of Feature | Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment shall offer the driver the possibility to re-enter the Radio data, only at S10 and S20. |  | 16 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1: SB |  |  |
| Target of Test | The target of this test case is to check that: It is possible to re-enter the Radio data at S10 and S20 during SoM. After requesting to modify the Radio data the driver shall only have the possibility to modify the Radio Network ID. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.4.5.3 |  |
|  |  | Subset-026-5.4.5.3 j) |  |
|  |  | Subset-026-5.4.5.3 j) b) |  |
|  |  | Subset-026-5.4.3.2 S10 |  |
|  |  | Subset-026-5.4.3.2 S20 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver selects “Radio data” at S10 and S20 and Radio data window is displayed with only the “RBC Network ID” label available. The driver presses the ‘Radio network ID’ button. The Radio Network ID window is displayed with a list of available and allowed networks. The driver validates the Radio Network ID. Check on the JRU that : The “RBC CONTACT INFORMATION ENTERED BY THE DRIVER” message is RECORDED. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure at S10 and S20. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0/ 1/ 2 | L0 / LNTC / L1 |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Main> window is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The <Radio data> button in the <Main> window is enabled | O | DMI | In addition to all possibilities offered at S10 and S20. | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Radio data”. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The <Radio data> window is displayed. | O | DMI | The on-board equipment displays the RBC data window and offers the possibility to the driver to select ‘Radio network ID’. | N/A | N/A | - |
| 4 | N/A | N/A | The driver presses the ‘Radio network ID’ button | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The Radio Network ID window is displayed with a list of available and allowed networks. | O | DMI | The Radio Network ID window is displayed with a list of available and allowed networks. | N/A | N/A | - |
| 6 | N/A | N/A | The driver validates the “Radio Network ID”. | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0/ 1/ 2 | L0 / LNTC / L1 |
| Driver ID | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |