| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3070200 (3.7.2: Responsibility for completeness of information) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3070200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 14.11.2013 | All | review | DLR |
| Version 2.9.2 06.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.2: Responsibility for completeness of information |  | 3070200 |
| Test Case of Feature | New MA is REJECTED by the on-board equipment because the SSP STORED on-board does NOT cover the whole length of the MA. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS/OS L2: FS/OS L3: FS/OS |  |  |
| Target of Test | • Precondition: • Speed profile (SSP) are receipt. • SSP matches NOT the whole length of the following MA. • The train stands in a position where a new MA will affect the DMI. • Permitted speed, target speed and distance to target are DISPLAYED on the DMI ( mode dependent) The target of this test is to check that: • New MA is not used on-board • The on-board equipment will stay in current mode Comment: Track description can be limited by using the V_STATIC=127 (finite static speed profile).Without these special values the track description is to be considered by the on-board as infinite. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Exception [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • - Balise / Radio Telegram is RECORDED Comments : This feature only aims at verifying whether the MA previously received is used or NOT. Transmission of level 1,2 or 3 MA and extension (or shortening or confirmation) of MA are part of other features. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS/OS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Movement Authority | STORED | The End Location of the MA is at position 'x' |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0;M_DUP=00;Q_LINK=0 ) containing packet 12, packet 255 is received | I | BTM | Only for Level 1 Reception of packet 12, without SSP and GP which is REJECTED because the SSP is too short according to the current MA. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for Level 1 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received | I | RTM | Only for Level 2/3 Reception of packet 15, without SSP and GP which is REJECTED because the SSP is too short according to the current MA. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for Level 2/3 | N/A | N/A | - |
| 5 | N/A | N/A | The initial MA is still displayed | O | DMI | Onboard device continues to run supervised to the initial MA. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 0 | balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Packet 12 in L1 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Iteration levels(set to zero if only one section) |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Level 2/3 MA |
| L_MESSAGE | 10 | RESERVED | - |
| T_TRAIN | 32 | RESERVED | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | RESERVED | balises |
| NID_PACKET | 8 | 15 | Level 2/3 Ma |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | RESERVED | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | NO MATTER | - |
| T_SECTIONTIMER(k) | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | NO MATTER | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | NO MATTER | - |
| T_SECTIONTIMER | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC | 15 | NO MATTER | - |
| Q_ENDTIMER | 1 | NO MATTER | - |
| T_ENDTIMER | 10 | NO MATTER | - |
| D_ENDTIMERSTARTLOC | 15 | NO MATTER | - |
| Q_DANGERPOINT | 1 | NO MATTER | - |
| D_DP | 15 | NO MATTER | - |
| V_RELEASEDP | 7 | NO MATTER | - |
| Q_OVERLAP | 1 | NO MATTER | - |
| D_STARTOL | 15 | NO MATTER | - |
| T_OL | 10 | NO MATTER | - |
| D_OL | 15 | NO MATTER | - |
| V_RELEASEOL | 7 | NO MATTER | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.2: Responsibility for completeness of information |  | 3070200 |
| Test Case of Feature | New MA is NOT used by the on-board equipment because the track description STORED on-board (Gradient profiles) does NOT cover the whole length of the MA. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS/OS L2: FS/OS L3: FS/OS |  |  |
| Target of Test | • Precondition: • Gradient profile (GP) are receipt. • GP matches NOT the whole length of the following MA. • The train stands in a position where a new MA will affect the DMI. • Permitted speed, target speed and distance to target are DISPLAYED on the DMI ( mode dependent) The target of this test is to check that: • New MA is NOT used on-board • The Gradient profile does NOT cover the whole length of the MA (new) • The on-board equipment will stay in current mode Comment: Track description can be limited by using the G_A=255 (finite gradient profile) or V_STATIC=127 (finite static speed profile).Without these special values the track description is to be considered by the on-board as infinite. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Exception [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message is RECORDED although the message with the new MA is REJECTED. |
| Constraints | Comments : This feature only aims at verifying whether the MA previously received is used or NOT. Transmission of level 1, 2 or 3 MA and extension (or shortening or confirmation) of MA are part of other features. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS/OS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Movement Authority | STORED | The End Location of the MA is at position 'x' |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0;M_DUP=00;Q_LINK=0 ) containing packet 12, packet 255 is received | I | BTM | Only for Level 1 New MA (packet 12 if Level 1), without SSP and GP is received. The new MA is REJECTED because the GP is too short according to the current MA. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for Level 1 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received | I | RTM | Only for Level 2/3 New MA (packet 15 if Level 2), without SSP and GP is received. The new MA is REJECTED because the GP is too short according to the current MA. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for Level 2/3 | N/A | N/A | - |
| 5 | N/A | N/A | The initial MA is still displayed | O | DMI | Onboard device continues to run supervised to the initial MA. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 0 | balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Packet 12 in L1 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Iteration levels(set to zero if only one section) |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Level 2/3 MA |
| L_MESSAGE | 10 | RESERVED | - |
| T_TRAIN | 32 | RESERVED | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | RESERVED | balises |
| NID_PACKET | 8 | 15 | Level 2/3 Ma |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | RESERVED | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | NO MATTER | - |
| T_SECTIONTIMER(k) | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | NO MATTER | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | NO MATTER | - |
| T_SECTIONTIMER | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC | 15 | NO MATTER | - |
| Q_ENDTIMER | 1 | NO MATTER | - |
| T_ENDTIMER | 10 | NO MATTER | - |
| D_ENDTIMERSTARTLOC | 15 | NO MATTER | - |
| Q_DANGERPOINT | 1 | NO MATTER | - |
| D_DP | 15 | NO MATTER | - |
| V_RELEASEDP | 7 | NO MATTER | - |
| Q_OVERLAP | 1 | NO MATTER | - |
| D_STARTOL | 15 | NO MATTER | - |
| T_OL | 10 | NO MATTER | - |
| D_OL | 15 | NO MATTER | - |
| V_RELEASEOL | 7 | NO MATTER | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.2: Responsibility for completeness of information |  | 3070200 |
| Test Case of Feature | New MA and the mode profile is REJECTED by the on-board equipment because the SSP STORED on-board does NOT cover the whole length of the MA. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS/OS L2: FS/OS L3: FS/OS |  |  |
| Target of Test | • Precondition: • Speed profile (SSP) are receipt. • SSP matches NOT the whole length of the following MA. • The train stands in a position where a new MA will affect the DMI. • Permitted speed, target speed and distance to target are DISPLAYED on the DMI (mode dependent) The target of this test is to check that: • New MA and the mode profile is not used on-board • The on-board equipment will stay in current mode Comment: Track description can be limited by using the G_A=255 (finite gradient profile) or V_STATIC=127 (finite static speed profile). Without these special values the track description is to be considered by the on-board as infinite. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Exception [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • - Balise / Radio Telegram is RECORDED Comments : This feature only aims at verifying whether the MA previously received is used or NOT. Transmission of level 1,2 or 3 MA and extension (or shortening or confirmation) of MA are part of other features. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/OS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Movement Authority | STORED | The End Location of the MA is at position 'x' |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0;M_DUP=00;Q_LINK=0 ) containing packet 12, packet 80, packet 255 is received | I | BTM | Only for Level 1 Reception of packet 12, without SSP and GP which is REJECTED because the SSP is too short according to the current MA. Only for Level 1 The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for Level 1 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 is received | I | RTM | Only for Level 2/3 Reception of packet 15, without SSP and GP which is REJECTED because the SSP is too short according to the current MA. Only for Level 2/3 The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for Level 2/3 | N/A | N/A | - |
| 5 | N/A | N/A | The initial MA is still displayed | O | DMI | Onboard device continues to run supervised to the initial MA. | N/A | N/A | - |
| 6 | N/A | N/A | The on-board unit does not change the mode at D_MAMODE | O | DMI | If mode is FS | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 0 | balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Packet 12 in L1 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Iteration levels(set to zero if only one section) |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | The length of the MA is shorter than the length of SSP or GP |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | D_MAMODE should within the old MA |
| M_MAMODE | 2 | FINITE VALUE | - |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Level 2/3 MA |
| L_MESSAGE | 10 | RESERVED | - |
| T_TRAIN | 32 | RESERVED | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | RESERVED | balises |
| NID_PACKET | 8 | 15 | Level 2/3 Ma |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | RESERVED | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | NO MATTER | - |
| T_SECTIONTIMER(k) | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | NO MATTER | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | NO MATTER | - |
| T_SECTIONTIMER | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC | 15 | NO MATTER | - |
| Q_ENDTIMER | 1 | NO MATTER | - |
| T_ENDTIMER | 10 | NO MATTER | - |
| D_ENDTIMERSTARTLOC | 15 | NO MATTER | - |
| Q_DANGERPOINT | 1 | NO MATTER | - |
| D_DP | 15 | NO MATTER | - |
| V_RELEASEDP | 7 | NO MATTER | - |
| Q_OVERLAP | 1 | NO MATTER | - |
| D_STARTOL | 15 | NO MATTER | - |
| T_OL | 10 | NO MATTER | - |
| D_OL | 15 | NO MATTER | - |
| V_RELEASEOL | 7 | NO MATTER | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | D_MAMODE should within the old MA |
| M_MAMODE | 2 | NO MATTER | - |
| V_MAMODE | 7 | NO MATTER | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.7.2: Responsibility for completeness of information |  | 3070200 |
| Test Case of Feature | New MA and mode profile is NOT used by the on-board equipment because the track description STORED on-board (Gradient profiles) does NOT cover the whole length of the MA. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS/OS L2: FS/OS L3: FS/OS |  |  |
| Target of Test | • Precondition: • Gradient profile (GP) are receipt. • GP matches NOT the whole length of the following MA. • The train stands in a position where a new MA will effect the DMI. • Permitted speed, target speed and distance to target are DISPLAYED on the DMI ( mode dependent) The target of this test is to check that: • New MA and mode profile is NOT used on-board • The Gradient profile does NOT cover the whole length of the MA (new) • The on-board equipment will stay in current mode Comment: Track description can be limited by using the G_A=255 (finite gradient profile) or V_STATIC=127 (finite static speed profile).Without these special values the track description is to be considered by the on-board as infinite. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.7.2.3 |  |
|  |  | Subset-026-4.8.3.1.1 Exception [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Message is RECORDED although the message with the new MA is REJECTED. |
| Constraints | Comments : This feature only aims at verifying whether the MA previously received is used or NOT. Transmission of level 1,2 or 3 MA and extension (or shortening or confirmation) of MA are part of other features. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/OS |
| Radio communication session | ESTABLISHED | Only in L2/L3 |
| Movement Authority | STORED | The End Location of the MA is at position 'x' |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0;M_DUP=00;Q_LINK=0 ) containing packet 12, packet 80, packet 255 is received | I | BTM | Only for Level 1 New MA (packet 12 if Level 1) and mode profile (packet 80): are REJECTED. New SSP and GP are not transmitted. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only for Level 1 | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 is received | I | RTM | Only for Level 2/3 New MA (packet 15 if Level 2) and mode profile (packet 80): are REJECTED. New SSP and GP are not transmitted. The End Location for MA (x+500) is at position x+500(m) | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only for Level 2/3 | N/A | N/A | - |
| 5 | N/A | N/A | The initial MA is still displayed | O | DMI | Onboard device continues to run supervised to the initial MA. | N/A | N/A | - |
| 6 | N/A | N/A | The on-board unit does not change the mode at D_MAMODE | O | DMI | If mode is FS | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | - |
| N_TOTAL | 3 | 0 | balises |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Packet 12 in L1 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | 0 cm, 1m or 10 m depends on Q_SCALE |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | Iteration levels(set to zero if only one section) |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | The length of the MA is shorter than the length of SSP or GP |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | D_MAMODE should within the old MA |
| M_MAMODE | 2 | FINITE VALUE | - |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Level 2/3 MA |
| L_MESSAGE | 10 | RESERVED | - |
| T_TRAIN | 32 | RESERVED | - |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | RESERVED | balises |
| NID_PACKET | 8 | 15 | Level 2/3 Ma |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | RESERVED | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | NO MATTER | - |
| T_SECTIONTIMER(k) | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | NO MATTER | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | NO MATTER | - |
| T_SECTIONTIMER | 10 | NO MATTER | - |
| D_SECTIONTIMERSTOPLOC | 15 | NO MATTER | - |
| Q_ENDTIMER | 1 | NO MATTER | - |
| T_ENDTIMER | 10 | NO MATTER | - |
| D_ENDTIMERSTARTLOC | 15 | NO MATTER | - |
| Q_DANGERPOINT | 1 | NO MATTER | - |
| D_DP | 15 | NO MATTER | - |
| V_RELEASEDP | 7 | NO MATTER | - |
| Q_OVERLAP | 1 | NO MATTER | - |
| D_STARTOL | 15 | NO MATTER | - |
| T_OL | 10 | NO MATTER | - |
| D_OL | 15 | NO MATTER | - |
| V_RELEASEOL | 7 | NO MATTER | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | D_MAMODE should within the old MA |
| M_MAMODE | 2 | NO MATTER | - |
| V_MAMODE | 7 | NO MATTER | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |