| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5181000 (5.18.10 Changing the traction system) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-5181000 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 21.12.2012 | All | First version/review | MULTITEL |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 | MULTITEL |
| Version 2.9.2 15.10.2013 | All | Answer to Review001_Subset076-5-2-5181000_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.10 Changing the traction system |  | 5181000 |
| Test Case of Feature | “Change of traction system” is announced and executed automatically |  | 1 |
| Applicable Mode/Level Combinations | L0 : TR, NL LNTC : TR, NL L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the “Change of traction system” is correctly announced to the driver when his execution is automated |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-5.18.10.2 |  |
|  |  | Subset-026-5.18.10.3 |  |
|  |  | Subset-026-5.18.10.4 |  |
|  |  | Subset-026-5.18.10.4 a) |  |
|  |  | Subset-026-5.18.10.5 |  |
|  |  | Subset-026-5.18.10.5 a) |  |
|  |  | Subset-026-5.18.10.5 b) |  |
|  |  | Subset-026-5.18.10.6 |  |
|  |  | Subset-026-5.20.6.2 |  |
|  |  | Subset-026-5.20.6.3 |  |
|  |  | Subset-026-5.20.6.3 a) |  |
|  |  | Subset-026-5.20.6.3 b) |  |
|  |  | Subset-026-5.20.6.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. This information shall remain displayed for a fixed time (see Appendix A3.1, 5s) after the minimum safe rear end of the train has passed the “Change of traction system” location. Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function |
| Constraints | There is a “Change of traction system” information stored on-board and the estimated front end of the train is in rear of it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | A change of traction is stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No ”Change of traction” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> announcement is displayed | O | DMI | The train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=1¦Bit68=1¦Bit70=1¦Bit72=1¦Bit74=1¦Bit76=1>) is recorded | O | JRU | The “Change of traction system announcement” display is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=7, D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=7, D_MAXSFE_TO_START>0) is recorded | O | JRU | D_MAXSFE_TO_START is decreasing | N/A | N/A | - |
| 7 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | When the min safe rear end of the train reaches the location of change of traction system | N/A | N/A | - |
| 8 | N/A | N/A | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> announcement is removed | O | DMI | When the change is automated, “Change of traction system announcement” and “New traction system” are identical. After the minimum safe rear end of the train has passed the “Change of traction system” location, the “New traction system” information shall remain displayed for a fixed time (5s, see Appendix A3.1). | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=0&Bit68=0&Bit70=0&Bit72=0&Bit74=0&Bit76=0>) is recorded | O | JRU | The “Change of traction system announcement” remove is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.10 Changing the traction system |  | 5181000 |
| Test Case of Feature | “Change of traction system” is announced and the driver is requested to execute it |  | 2 |
| Applicable Mode/Level Combinations | L0 : TR, NL LNTC : TR, NL L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the “Change of traction system” and “New traction system” are correctly displayed to the driver |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-5.18.10.2 |  |
|  |  | Subset-026-5.18.10.3 |  |
|  |  | Subset-026-5.18.10.4 |  |
|  |  | Subset-026-5.18.10.4 b) |  |
|  |  | Subset-026-5.18.10.5 |  |
|  |  | Subset-026-5.18.10.5 a) |  |
|  |  | Subset-026-5.18.10.5 b) |  |
|  |  | Subset-026-5.18.10.6 |  |
|  |  | Subset-026-5.20.6.2 |  |
|  |  | Subset-026-5.20.6.3 |  |
|  |  | Subset-026-5.20.6.3 a) |  |
|  |  | Subset-026-5.20.6.3 b) |  |
|  |  | Subset-026-5.20.6.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. When the max safe front end of the train reaches the “Change of traction system” location, “New traction system” information replace “Change of traction system announcement” information. This information shall remain displayed for a fixed time (see Appendix A3.1, 5s) after the minimum safe rear end of the train has passed the “Change of traction system” location. Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function |
| Constraints | There is a “Change of traction system” information stored on-board and the estimated front end of the train is in rear of it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | A change of traction is stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No ”Change of traction” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol <TC24¦TC26¦TC28¦TC30¦TC32¦TC34> announcement is displayed | O | DMI | The train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit67=1¦Bit69=1¦Bit71=1¦Bit73=1¦Bit75=1¦Bit77=1>) is recorded | O | JRU | The “Change of traction system announcement” display is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=7, D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=7, D_MAXSFE_TO_START>0) is recorded | O | JRU | D_MAXSFE_TO_START is decreasing | N/A | N/A | - |
| 7 | N/A | N/A | The Track Condition symbol <TC24¦TC26¦TC28¦TC30¦TC32¦TC34> announcement is removed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit67=0&Bit69=0&Bit71=0&Bit73=0&Bit75=0&Bit77=0>) is recorded | O | JRU | The “Change of traction system announcement” shown off is RECORDED | N/A | N/A | - |
| 9 | N/A | N/A | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> announcement is displayed | O | DMI | “New traction system” is displayed | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=1¦Bit68=1¦Bit70=1¦Bit72=1¦Bit74=1¦Bit76=1>) is recorded | O | JRU | The “New traction system” display is RECORDED | N/A | N/A | - |
| 11 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | When the min safe rear end of the train reaches the location of change of traction system | N/A | N/A | - |
| 12 | N/A | N/A | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33> announcement is removed | O | DMI | The “New traction system” information shall remain displayed for a fixed time (5s, see Appendix A3.1) after the minimum safe rear end of the train has passed the “Change of traction system” location. | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=0&Bit68=0&Bit70=0&Bit72=0&Bit74=0&Bit76=0>) is recorded | O | JRU | The “New traction system” shown off is RECORDED | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.10 Changing the traction system |  | 5181000 |
| Test Case of Feature | “Change of traction system” is announced and deleted by trackside |  | 3 |
| Applicable Mode/Level Combinations | L0 : TR, NL LNTC : TR, NL L1 : FS, LS, OS, TR, PT, NL L2 : FS, LS, OS, TR, PT, NL L3 : FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that the ERTMS/ETCS on-board equipment stop providing the “Change of traction system” information to the ERTMS/ETCS external function after the deletion of the change of traction system has been requested by trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-5.18.10.2 |  |
|  |  | Subset-026-5.18.10.3 |  |
|  |  | Subset-026-5.18.10.4 |  |
|  |  | Subset-026-5.20.6.2 |  |
|  |  | Subset-026-5.20.6.3 |  |
|  |  | Subset-026-5.20.6.3 a) |  |
|  |  | Subset-026-5.20.6.3 b) |  |
|  |  | Subset-026-5.20.6.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. Check on TIU and JRU that: The information related to track condition is provided to the ERTMS/ETCS external function The track condition information is no more transmitted after the trackside request the deletion of the change of traction system |
| Constraints | There is a “Change of traction system” information stored on-board and the estimated front end of the train is in rear of it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | A change of traction is stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No ”Change of traction” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The Track Condition symbol <TC23¦TC25¦TC27¦TC29¦TC31¦TC33¦TC24¦TC26¦TC28¦TC30¦TC32¦TC34> announcement is displayed | O | DMI | The train reaches a location, determined by the ERTMS/ETCS on-board equipment taking into account the time necessary for performing the required actions and the current train speed, “Change of traction system announcement” is displayed to the driver. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit66=1¦Bit68=1¦Bit70=1¦Bit72=1¦Bit74=1¦Bit76=1¦Bit67=1¦Bit69=1¦Bit71=1¦Bit73=1¦Bit75=1¦Bit77=1>) is recorded | O | JRU | The “Change of traction system announcement” display is RECORDED | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=7, D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 39 (D_TRACTION=0) is received | I | BTM | The information related to the change of traction system is deleted because M_VOLTAGE is resetted to the current value | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT Track Conditions transmitted | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 39 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | 0 | - |
| M_VOLTAGE | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |