| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080447 (4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-4080447 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 26.11.2013 | All | Corrections | CEDEX |
| Version 2.9.3 12.12.2013 | All | Editorial changes | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop |  | 4080447 |
| Test Case of Feature | Acceptance of Unconditional Emergency Stop information. A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC. The On-Board equipment switches immediately to the Trip mode. The train responds to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’, indicating that the train takes into account the Unconditional Emergency Stop. The data displayed on the DMI are updated, accordingly. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS / SR / SB |  |  |
| Target of Test | To check that the train takes into account an Unconditional Emergency Stop as soon as it receives it. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.1.4 |  |
|  |  | Subset-026-3.10.2.3 |  |
|  |  | Subset-026-3.16.3.1.2 |  |
|  |  | Subset-026-4.6.2 row 12 column 2 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.3 condition id [20] |  |
|  |  | Subset-026-4.8.3.1.1 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.8.4.2 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.10.1.3 line Unconditional Emergency Stops |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received from and sent to the RBC messages are recorded On-board switches to TR Accepted Emergency Stop message is recorder Check on the DMI that: Target speed and Target distance are displayed. On-board switches to TR TR reason “Emergency Stop” is displayed |
| Constraints | A cab needs to be active (SB Mode) A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | The distance monitoring information is displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/LS/OS/SR/SB | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The message is accepted. | L2/L3 | FS/LS/OS/SR/SB | - |
| 2 | L2/L3 | FS/LSOS/SR/SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded | O | JRU | - | L2/L3 | FS/LS/OS/SR/SB | - |
| 3 | L2/L3 | FS/LSOS/SR/SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2/L3 | TR | - |
| 4 | L2/L3 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L2/L3 | TR | - |
| 5 | L2/L3 | TR | The system status message "Emergency stop" is displayed | O | DMI | - | L2/L3 | TR | - |
| 6 | L2/L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2/L3 | TR | - |
| 7 | L2/L3 | TR | Emergency brake commanded | O | TIU | - | L2/L3 | TR | - |
| 8 | L2/L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2/L3 | TR | - |
| 9 | L2/L3 | TR | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147;Q_EMERGENCYSTOP=2) is transmitted | O | RTM | Indicating that the train has received the Unconditional Emergency Stop. Check that the messages 147 is send over the normal priority channel. | L2/L3 | TR | - |
| 10 | L2/L3 | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=147) is recorded | O | JRU | - | L2/L3 | TR | - |
| 11 | - | - | Use_FT5110200.13 | - | - | “Driver acknowledges Trip and train is at standstill (Transition [7])” | - | - | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 9: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Linking | DELETED | - |
| Movement Authority | DELETED | - |
| Gradient Profile | DELETED | - |
| International SSP | DELETED | - |
| Unconditional Emergency Stop | STORED | FINITE VALUE. Emergency message identifier is not deleted after switch to TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The system status message "Emergency stop" is displayed NOT The speed and distance monitoring information is displayed The Mode symbol "Trip" is displayed The Status symbol "Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop |  | 4080447 |
| Test Case of Feature | Acceptance of Unconditional Emergency Stop information.The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC, and there is an order to switch to L2/3 at a further location stored on-board, together with an MA for Level 2. The Message 16 is stored on-board until the transition to Level 2 is executed. Then, when the level transition takes place, the On-Board equipment switches to Trip mode. The train answers the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’, indicating that the train takes into account the Unconditional Emergency Stop. The data displayed on the DMI are updated, accordingly. |  | 2 |
| Applicable Mode/Level Combinations | L1: SB, FS, LS, SR, OS L0 : UN LSTM: SN |  |  |
| Target of Test | To check that the train takes into account an Unconditional Emergency Stop as soon as the transition to Level 2 is executed. This Test Case tests condition [2] of Subset-026-4.8.3.1.1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.1.4 |  |
|  |  | Subset-026-3.10.2.3 |  |
|  |  | Subset-026-3.16.3.1.2 |  |
|  |  | Subset-026-4.6.2 row 12 column 2 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 11 |  |
|  |  | Subset-026-4.6.2 row 14 column 17 |  |
|  |  | Subset-026-4.6.3 condition id [20] |  |
|  |  | Subset-026-4.8.3.1.1 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Unconditional Emergency Stop |  |
|  |  | Subset-026-8.6.8 Message 147: Acknowledgement of Emergency Stop |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received from and sent to the RBC messages are recorded On-board switches to TR after changing to Level 2 Check on the DMI that: On-board switches to TR after changing to Level 2 TR reason "Emergency stop" is displayed Check on the RTM: Message 147 is sent after changing to Level 2. |
| Constraints | A level 2/3 transition announcement is stored on-board. Movement Authority for Level 2/3 is stored on-board In SB mode the cab is active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | 3 / 4 | Level Transition order to level 2/3 is stored on-board |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LSTM / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The message is accepted. | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |
| 2 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded | O | JRU | - | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |
| 3 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | No changes are displayed. | O | DMI | The message is stored in the Transition Buffer before it is accepted. | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |
| 4 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | Use_FT5100100.21 | - | - | Only in Level NTC. The on-board equipment switches to Level 2. | L2 | FS/OS/SR/SB/LS | - |
| 5 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | Use_FT5100100.13 | - | - | Only in Level 0. The on-board equipment switches to Level 2. | L2 | FS/OS/SR/SB/LS | - |
| 6 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | Use_FT5100100.37 | - | - | Only in Level 1. The on-board equipment switches to Level 2. | L2 | FS/OS/SR/SB/LS | - |
| 7 | L2 | FS/OS/SR/SB/LS | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147;Q_EMERGENCYSTOP=2) is transmitted | O | RTM | Indicating that the train has accepted the Unconditional Emergency Stop. Check that the messages 147 is sent over the normal priority channel. | L2 | FS/OS/SR/SB/LS | - |
| 8 | L2 | FS/OS/SR/SB/LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=147) is recorded | O | JRU | - | L2 | FS/OS/SR/SB/LS | - |
| 9 | L2 | FS/OS/SR/SB/LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2 | TR | - |
| 10 | L2 | TR | The Mode symbol "Trip" is displayed | O | DMI | - | L2 | TR | - |
| 11 | L2 | TR | Use_FT3060500.4 | - | - | Report mode change to RBC | L2 | - | - |
| 12 | L2/L3 | TR | The system status message "Emergency stop" is displayed | O | DMI | The TR reason is displayed | L2/L3 | TR | - |
| 13 | L2/L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | L2/L3 | TR | - |
| 14 | L2 | TR | Emergency brake commanded | O | TIU | - | L2 | TR | - |
| 15 | L2 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 | TR | - |
| 16 | - | - | Use_FT5110200.12 | - | - | Only in Level 0. Driver acknowledges train trip and ETCS Level is 0 | - | - | - |
| 17 | - | - | Use_FT5110200.13 | - | - | Only in Levels 1, 2 and 3. Driver acknowledges Trip and train is at standstill | - | - | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| Step 7: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | 2 | Not Relevant (Unconditional Emergency Stop) (ref 3.10.2.3) |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Unconditional Emergency Stop | STORED | FINITE VALUE. Emergency message identifier is not deleted after switch to TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The speed and distance monitoring information is removed The Mode symbol "Trip" is displayed The Status symbol "Emergency Brake intervention" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop |  | 4080447 |
| Test Case of Feature | Rejection of Unconditional Emergency Stop information. The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC which is rejected due to the applicable level. |  | 3 |
| Applicable Mode/Level Combinations | L1: SB, FS, LS, SR, OS L0 : UN LSTM: SN |  |  |
| Target of Test | To check that the train does not take into account an Unconditional Emergency Stop. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.8.4.2 Unconditional Emergency Stop |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received from RBC message is recorded Check on the DMI that: On-board does not switches to TR |
| Constraints | In SB mode the cab is active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The message is not accepted. | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |
| 2 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded | O | JRU | - | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |
| 3 | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | The on-board equipment does not switch to TR mode. | O | DMI | - | L1 / L0 / LNTC | FS/OS/SR/SB/ UN/LS/SN | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Radio communication session | MAINTAINED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop |  | 4080447 |
| Test Case of Feature | Rejection of Unconditional Emergency Stop information. The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC which is rejected due to the applicable mode. |  | 4 |
| Applicable Mode/Level Combinations | L2: PS, SH, SL, NL, TR, PT, RV L3: PS, SH, SL, NL, TR, PT, RV |  |  |
| Target of Test | To check that the train does not take into account an Unconditional Emergency Stop. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.8.4.2 Unconditional Emergency Stop |  |
|  |  | Subset-026-8.7.7 Message 16: Unconditional Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Received from RBC message is recorded Check on the DMI that: On-board does not switch to TR |
| Constraints | In SB mode the cab is not active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 / 7 / 8 / 11 / 14 / 15 | SH / SL / SB / TR / PT / NL / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received | I | RTM | The message is not accepted. | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | - |
| 2 | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded | O | JRU | - | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | - |
| 3 | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | NOT The Mode symbol "Trip" is displayed | O | DMI | - | L2 / L3 | SH/SL/SB/TR/ PT/NL/RV/PS | - |

| Step 1: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 16 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 / 7 / 8 / 11 / 14 / 15 | SH / SL / SB / TR / PT / NL / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |