| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080423 (4.8.4: Accepted Information depending on the modes - MA Request Parameters) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-4080423 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.3.3 29.02.2012 | All | Final delivery after clean-up process according with Subset 026 v2.3.0 and Subset 108 v 1.2.0 | All |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 31.03.2013 | All | General revision | CEDEX |
| Version 2.9.2 25.10.2013 | All | Requirements correction | CEDEX |
| Version 2.9.3 10.12.2013 | TC 3 | Editorial change (RTM tables missmatch) | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - MA Request Parameters |  | 4080423 |
| Test Case of Feature | The onboard equipment receives a message including Packet 57 (MA request parameters) that is transmitted from RBC. The packet is REJECTED. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, PS, SL, SB, TR, NL LNTC: SH, PS, SL, SB, TR, NL L1: SH, PS, SL, SB, TR, PT, NL, RV L2: SH, PS, SL, SB, TR, PT, NL, RV L3: SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that The message including packet 57 is rejected and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 MA Request Parameters |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 MA Request Parameters |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 57 (MA request parameters) is recorded. The on-board changes to one of the modes where it is possible to test the MA request parameters were rejected. |
| Constraints | If the testing mode is SB mode, the desk must be closed. If the testing level/mode is L2/PT or L3/PT, no message 6 (Recognition of exit from TRIP mode) must have been received previously. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 6 / 7 / 8 / 11 / 14 / 15 | SH / SL / SB / TR / PT / NL / RV / PS |
| Radio communication session | ESTABLISHED | - |
| MA Request Parameters | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 57 is received | I | RTM | The packet is REJECTED. | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | Use_FT4060300 to make the on-board equipment change to one of the modes from which the MA request functionality is active: SB, FS, LS, OS, SR, PT, UN, TR or SN. | - | - | Only if the current mode is not SB | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | Cab active | I | TIU | Only if the current mode is SB, the driver opens the desk. | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | Reception of "track ahead free up to the level 2/3 transition location" | O | DMI | Only level 0,1,NTC. The on-board must be in one of the following scenarios (depending on the current mode), given in 4.5.2.1 table of the SRS: approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) | L0 / LNTC / L1 / L2 / L3 | SH / SL / SB / TR / PT / NL / RV / PS | - |
| 6 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | “Start” is selected | I | DMI | Only level 2/3 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | - |
| 7 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | Not SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | At this time/distance one message 132 would be sent if packet 57 had been accepted at the begginning. | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | - |
| 8 | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | Not MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | FS / OS / SR / UN / SB / TR / PT / LS / SN | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | FINITE VALUE | - |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| MA Request Parameters | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - MA Request Parameters |  | 4080423 |
| Test Case of Feature | The train receives and accepts a message including Packet 57 (MA request parameters) that is transmitted from RBC |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that The message including packet 57 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 MA Request Parameters |  |
|  |  | Subset-026-4.8.4.2 MA Request Parameters |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.14 Packet Number 57: MA request Parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 57 (MA request parameters) is recorded. The on-board equipment is in a scenario where the MA request parameters can be used. |
| Constraints | In SB mode, a cab needs to be active. In PT mode and L2/3, only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 /12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L NTC / L1 / L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET-UP | both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 57 is received | I | RTM | Complete message is accepted | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | N/A | N/A | - |
| 3 | N/A | N/A | Reception of "track ahead free up to the level 2/3 transition location" | O | DMI | Only level 0,1,NTC. The on-board must be in one of the following scenarios (depending on the current mode), given in 4.5.2.1 table of the SRS: approach of target indication point (T_MAR) or MA timer elapsing (T_TIMEOUTRQST) (only level 2/3) | N/A | N/A | - |
| 4 | N/A | N/A | “Start” is selected | I | DMI | Only level 2/3 | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | This message will be sent at the time/distance specified by the parameters of packet 57, which was received at the beginning. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 57 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| T_MAR | 8 | FINITE VALUE | - |
| T_TIMEOUTRQST | 10 | FINITE VALUE | - |
| T_CYCRQST | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 /12 / 13 | FS / OS / SR / UN / SB / PT / LS / SN |
| Radio communication session | ESTABLISHED | - |
| MA Request Parameters | STORED | MA Request Parameters have been received. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / L NTC / L1 / L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONECTION SET UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |