| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3060300 (3.6.3: Validity direction of transmitted Information) Total: 13 Test case(s) |  |
| REF: | Subset-076-5-2-3060300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 31.03.2013 | All | review | DLR |
| Version 2.9.2 12.12.2013 | All | review | DLR |
| Version 2.9.3 22.01.2014 | All | review | DLR |
| Version 2.9.4 24.01.2014 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The train passes over a single balise group for which NO co-ordinate system has been assigned to. The on-board equipment accepts all packets contained in the single balise group message that are valid for both directions and ignores all packets that are valid for the nominal direction or the reverse direction. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, if NO co-ordinate system has yet been assigned to the balise group SBGr, : • Data contained in the single balise group message and valid for the reverse direction or nominal direction is ignored by the on-board equipment, • Data contained in the single balise group message and valid for both directions is accepted by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.4 |  |
|  |  | Subset-026-3.6.3.1.3 |  |
|  |  | Subset-026-4.8.2.1 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -NO Level announcement is displayed. -NO "Balise read error" message is diplayed. Check on the JRU that: -Message display |
| Constraints | • No co-ordinate system is assigned to the single balise group SBGr. • The single balise group message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions • The single balise group must be marked as linked (Q_LINK=1), although linking information must not be stored |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | NOT STORED | Linking information is not being used |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0; Q_LINK=1) containing packet 5 (Q_DIR=0), packet 41 (Q_DIR=1) and packet 72 (Q_DIR=2) is received | I | BTM | The OBU correctly reads all the 3 packets contained in the telegram. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The single balise group message is accepted by the on-board equipment and is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The plain text message "Both directions" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NO Level symbol announcement is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NO "Balise read error" message is displayed | O | DMI | At position (Step1+D_LINK value). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | First Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | One balise in the group Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | "SBGr" | Country or region |
| NID_BG | 14 | "SBGr" | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction This packet is ignored by the onboard equipment |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | 0 | Same value as Q_DIR |
| Q_LINKREACTION | 2 | 0 | The train trip |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction. This packet is ignored by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | NOT "0" or "32767" |
| M_LEVELTR | 3 | FINITE VALUE | Set value different to the actuel level. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 2 | Valid for both directions. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important information (displayed on top) |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | 10 | 10s time display |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Both Directions | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | TIU | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The train passes over a linked balise group defined in the linking data. The onboard equipment accepts packets valid for the direction announced in the linking data or for both directions and ignores packets valid for the opposite direction to the direction announced in the linking data. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that : • All packets received from a linked balise group BGr and valid for the running direction announced in the linking data or valid for both directions are accepted by the on-board equipment. • All packets received from a linked balise group BGr and valid for the opposite direction to the running direction announced in the linking data are ignored by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -A new Level announcement is displayed. -NO "Service Break" applied (TSR rejected). Check on the JRU that: -Message display |
| Constraints | • Linking data describing the balise group BGr and the train passing direction over this BG is stored on-board. • It is assumed that there are no linking consistency errors, i.e. the train always reads balises of the group in the direction announced in the linking data (i.e. if Q_LINKORIENTATION is set to nominal, the train reads each balise in the increasing order (0 to N) and if Q_LINKORIENTATION is set to reverse, the train reads each balise in the decreasing order (N to 0)). • The balise group message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions • It is assumed that there are no inconsistency errors in the balise group message |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Linking data is available onboard Q_LINKORIENTATION= 0 or 1 (The train is expected to pass over the linked balise group BGr in either the reverse direction or the nominal direction.) NID_BG="BGr" (The balise group BGr that the train is going to pass over is defined in the linking data stored on-board) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=2, Q_LINK=1) containing packet 65 (Q_DIR=0), packet 41 (Q_DIR=1) for LevelY and packet 72 (Q_DIR=2) is received | I | BTM | The train correctly reads all packets of all balises of the group. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The balise group message is accepted by the on-board equipment and is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The plain text message "Both directions" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Level LY symbol announcement is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NO "Service Break" applied (TSR rejected) | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) |
| NID_PACKET | 8 | 65 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction This packet is ignored by the onboard equipment |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| D_TSR | 15 | 0 | Apply immediately |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | Set value very lower than actual speed. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | NOT "0" or "32767" |
| M_LEVELTR | 3 | FINITE VALUE | LY : Set value different to the actuel level. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 3/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 2 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 2 | Valid for both directions. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important information (displayed on top) |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | 10 | 10s time display |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Both Directions | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | The balise group BGr is still defined in the linking data. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | TIU | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The on-board equipment reads each balise of an unlinked balise group UBGr. The on-board equipment accepts all packets valid for the train orientation or for both directions and ignores all packets valid for the direction opposite to its orientation. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, when the on-board finds an unlinked balise group, all packets valid for the train orientation are accepted by the on-board equipment and all packets valid for the direction opposite to its orientation are ignored by the on-board equipment, • The on-board equipment always accepts all packets valid for both directions. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -A new Level announcement is displayed. -NO "Balise read error" message is diplayed. Check on the JRU that: -Message display |
| Constraints | • It is assumed that there are no linking consistency errors in the balise group message. • The balise group message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions. • It is assumed that there are no inconsistency errors in the balise group message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=2, Q_LINK=0) containing packet 5 (Q_DIR=0), packet 41 (Q_DIR=1) for LevelY and packet 72 (Q_DIR=2) is received. | I | BTM | The train correctly reads all packets of all balises of the group. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The balise group message is accepted by the on-board equipment and is recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The plain text message "Both directions" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Level LY symbol announcement is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NO "Balise read error" message is displayed | O | DMI | At position (Step1+D_LINK value). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction This packet is ignored by the onboard equipment |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | 0 | Same value as Q_DIR |
| Q_LINKREACTION | 2 | 0 | The train trip |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction.This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | NOT "0" or "32767" |
| M_LEVELTR | 3 | FINITE VALUE | LY : Set value different to the actuel level. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 3/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 2 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | 3 balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 2 | Valid for both directions. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important information (displayed on top) |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | 10 | 10s time display |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Both Directions | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | TIU | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The on-board equipment accepts all packets included in a radio message that are valid for both directions and ignores all packets that are valid for the nominal direction or the reverse direction, when the train position is unknown. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, TR, PT, NL L3: FS, LS, OS, SR, TR, PT, NL |  |  |
| Target of Test | The target of this test is to check that when the train position is unknown : • All packets included in a radio message and valid for both directions are taken into account by the on-board equipment. • All packets included in a radio message and valid either for the reverse direction or the nominal direction are ignored by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -NO Level announcement is displayed. Check on the RTM that: -NO SA-CONNECT.Request is transmitted Check on the JRU that: -Message display |
| Constraints | The radio message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions. The radio message is correctly read by the on-board equipment |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11 / 12 | FS / OS / SR / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_DIR=0; Q_RBC=0), packet 41 (Q_DIR=1) and packet 72 (Q_DIR=2) is received | I | RTM | The train correctly receives the radio message with the 3 packets. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The complete radio message is accepted by the onboard equipment and recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | The plain text message "Both directions" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NO Level symbol announcement is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction. This packet is ignored by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction. This packet is ignored by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 2 | Valid for both directions. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important information (displayed on top) |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | 10 | 10s time display |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Both Directions | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11 / 12 | FS / OS / SR / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The on-board equipment accepts a radio message valid for the same direction as the train train orientation or for both directions and ignores a radio message valid for the opposite direction to the train orientation, when the train when the train orientation in relation to the direction of the LRBG is known. |  | 6 |
| Applicable Mode/Level Combinations | L2: LS, OS L3: LS, OS |  |  |
| Target of Test | The target of this test is to check that, when the train orientation in relation to the direction of the LRBG is known (i.e. either nominal or reverse) : • The on-board equipment accepts only radio messages valid for the same direction as the train orientation (nominal OR reverse) or for both directions (nominal AND reverse). • The on-board equipment ignores radio messages valid for the opposite direction to the train orientation in relation to the direction of the LRBG. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that the complete radio message is recorded upon reception by the on-board equipment. Check on all available interfaces that : • A radio message (without packets) valid for the same direction as the train orientation in relation to the direction of the LRBG or valid for both directions is taken into account by the on-board equipment, i.e. reactions associated with the radio message are observable on at least one of the available interfaces. • A radio message (without packets) valid for the opposite direction to the train orientation in relation to the direction of the LRBG is ignored by the on-board equipment, i.e. there are no observable reactions on none of the interfaces (interfaces are unaffected by the radio message). |
| Constraints | • The radio message without packets can be for example the radio message 34 “Track ahead free request” or the radio message 15 “Conditional Emergency Stop”. In those messages, the variable Q_DIR is defined in the header of the radio message and no packets are included within the message itself. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 12 | OS / LS |
| Radio communication session | ESTABLISHED | - |
| Train Position | STORED | Train running direction in relation to the direction of the LRBG : Reverse (0) or nominal (1) Q_DIRTRAIN=0 or 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<greater than 0>) | I | INT | The train is running and the train orientation in relation to the direction of the LRBG is known. Train orientation in relation to the direction of the LRBG is described by variable Q_DIRLRBG, which can be equal to 0 (reverse) or 1 (nominal). | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | Q_DIR = opposite (Q_DIRLRBG). | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The radio message is recorded on the JRU. | N/A | N/A | - |
| 4 | N/A | N/A | NOT The system status message "Emergency stop" is displayed | O | DMI | The train does not react. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received | I | RTM | Q_DIR = Q_DIRLRBG | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The radio message is recorded on the JRU. | N/A | N/A | - |
| 7 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed. | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | Q_DIR = 2 (Both directions) | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The radio message is recorded on the JRU. | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | DMI symbol: ST01 The train reaches the stop location | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Can also be Message 34 (Track Ahead Free Request) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | 0 or 1 | Q_DIR=Opposite of (Q_DIRLRBG) |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 5: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | Can also be Message 34 (Track Ahead Free Request) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | 0 or 1 | Q_DIR=Q_DIRLRBG |
| D_TAFDISPLAY | 15 | FINITE VALUE | - |
| L_TAFDISPLAY | 15 | FINITE VALUE | - |

| Step 9: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Can also be Message 34 (Track Ahead Free Request) |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | 2 | Q_DIR=2 (Both directions) |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 12 | OS / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The on-board equipment accepts all packets included in a radio message that are valid for the same direction as the train orientation in relation to the direction of the LRBG or valid for both directions and ignores all packets that are valid for the opposite direction to the train orientation, when the train orientation in relation to the direction of the LRBG is known. |  | 7 |
| Applicable Mode/Level Combinations | L0: SH, PS, SL LNTC: SH, PS, SL L1: SH, PS, SL L2: SH, PS, SL L3: SH, PS, SL |  |  |
| Target of Test | The target of this test is to check that, when the train orientation in relation to the direction of the LRBG is known (i.e. either nominal or reverse) : • All packets included in a radio message and valid for the same direction as the train orientation or valid for both directions are taken into account by the on-board equipment. • The on-board equipment ignores all packets included in a radio message and valid for the opposite direction to the train orientation. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -A new Level announcement is displayed. -NO "Balise read error" message is diplayed. Check on the JRU that: -Message display |
| Constraints | • The radio message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions. There are no inconsistency errors with the radio message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 15 | SH / SL / PS |
| Radio communication session | ESTABLISHED | - |
| Train Data | STORED | Train running direction in relation to the direction of the LRBG : Reverse (0) or nominal (1) Q_DIRTRAIN=0 or 1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 is received | I | RTM | The message is rejected because the train has crossed the LRBG in a direction not matching the validity direction of packet 42 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The complete radio message is accepted by the on-board equipment and recorded on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | No MESSAGE 156 is transmitted because packet 42 was rejected | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3050500.5 | - | - | MESSAGE 24 with packet 42 accepted because the train has crossed the LRBG in a direction matching the validity direction of packet 42 | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction. This packet is ignored by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| Train Position | STORED | Train orientation in relation to the direction of the LRBG : Reverse (0) or nominal (1) Q_DIRLRBG=0 or 1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | TIU | - |
| Reaction/no reaction if balise message is accepted/ignored by the on-board equipment | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | The train passes over a single balise group defined in the linking data. The on-board equipment accepts packets valid for the direction announced in the linking data or for both directions and ignores packets valid for the opposite direction to the direction announced in the linking data. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that : • All packets received from a linked balise group BGr and valid for the running direction announced in the linking data or valid for both directions are accepted by the on-board equipment. • All packets received from a linked balise group BGr and valid for the opposite direction to the running direction announced in the linking data are ignored by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |
|  |  | Subset-026-3.4.2.3.2.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: -The plain text message "Both directions" is displayed. -A new Level announcement is displayed. -NO "Balise read error" message is diplayed. Check on the JRU that: -Message display |
| Constraints | • Linking information is stored on-board. • It is assumed that there are no linking consistency errors, • The balise group message is composed of at least one packet valid for the reverse direction, at least one packet valid for the nominal direction and at least one packet valid for both directions • It is assumed that there are no inconsistency errors in the balise group message |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Linking data is available onboard NID_BG(k)="BGr" (The balise group BGr that the train is going to pass over is defined in the linking data stored on-board) Q_LINKORIENTATION(k)=1 or 2 (The train is expected to pass over the linked balise group BGr in either the reverse direction or the nominal direction) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0; Q_LINK=1) containing packet 5 (Q_DIR=1-Q_LINKORIENTATION), packet 41 (Q_DIR=Q_LINKORIENTATION) and packet 72 (Q_DIR=2) is received. | I | BTM | The OBU correctly reads all the 3 packets contained in the telegram. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The plain text message "Both directions" is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Level LY symbol announcement is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | NO "Balise read error" message is displayed | O | DMI | At position (Step1+D_LINK value). | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | One balise in the group Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | 1-Q_LINKORIENTATION direction. This packet is ignored by the onboard equipment |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LINK | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 0 | The train trip |
| Q_LOCACC | 6 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_LINK(k) | 15 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | FINITE VALUE | - |
| Q_LOCACC(k) | 6 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | set to Q_LINKORIENTATION direction This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | NOT "0" or "32767" |
| M_LEVELTR | 3 | FINITE VALUE | LY : Set value different to the actuel level. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | 2 | Valid for both directions. This packet is accepted by the onboard equipment. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | 1 | Important information (displayed on top) |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | 10 | 10s time display |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Both Directions | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Linking data is available onboard NID_BG(k)="BGr" (The balise group BGr that the train is going to pass over is defined in the linking data stored on-board) Q_LINKORIENTATION(k)=1 or 2 (The train is expected to pass over the linked balise group BGr in either the reverse direction or the nominal direction) |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Reaction/no reaction if balise group message is accepted/ignored by the on-board equipment | - | TIU | - |
| Reaction/no reaction if balise group message is accepted/ignored by the on-board equipment | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | In PS mode, the ERTMS/ETCS on- board equipment shall only take into account information valid for its movement direction |  | 9 |
| Applicable Mode/Level Combinations | L0: PS LNTC: PS L1: PS L2: PS L3: PS |  |  |
| Target of Test | The target of this test is to check that when the train overpasses balises in their nominal direction: • The train receives a packet „Stop shunting“ which is valid for reverse direction only • The packet „Stop shunting“ is rejected • As a consequence, when the cab is opened, the OBE switches to SH mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-4.6.3 Condition Id [22] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Balise message containing packet 135 „Stop Shunting on desk opening“ is RECORDED Check on the DMI: • When the cab is opened, symbol MO13 „Stand by“ is displayed |
| Constraints | All balises are passed in their nominal direction. Onboard orientation can be any. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Radio communication session | NOT ESTABLISHED | - |
| Stop Shunting on desk opening | NOT STORED | This is necessary to check rejection of package 135 |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | - |
| Cab is closed | I | TIU | - |
| Since cab is closed, nothing is disaplyed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 135 (Q_DIR=0) and packet 255 is received | I | BTM | The packet 135 „Stop Shunting on desk opening“ is valid for reverse direction only | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Cab active | I | TIU | The driver opens the desk | N/A | N/A | - |
| 4 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=1) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Mode symbol "Shunting" is displayed | O | DMI | Symbol MO01 „Shunting“ is displayed | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | New mode SH is recorded | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 135 | - |
| Q_DIR | 2 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| Stop Shunting on desk opening | NOT STORED | Because of rejection of package 135 |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | O | RTM | - |
| Cab is open | - | TIU | - |
| Symbol MO01 „Shunting“ is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN=40) | I | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | In SH mode, the ERTMS/ETCS on-board equipment shall only take into account information valid for its movement direction |  | 10 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that when the train overpasses balises in their nominal direction: • The train receives a packet „Virtual Balise Cover order“ with NID_VBCMK=1 which is valid for nominal direction only • The train receives a packet „Virtual Balise Cover order“ with NID_VBCMK=2 which is valid for reverse direction only • A balise message containing Virtual Balise Cover Marker with NID_VBCMK=1 is ignored and • A balise message containing Virtual Balise Cover Marker with NID_VBCMK=2 is taken into account |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |
|  |  | Subset-026-7.4.2.0 Packet Number 0: Virtual Balise Cover marker |  |
|  |  | Subset-026-7.4.2.2.1 Packet Number 6: Virtual Balise Cover order |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Balise message containing 2 packets 6 „Virtual Balise Cover order“ is RECORDED • 2 Balise messages containing packet 0 „Virtual Balise Cover marker“ and packet 3„National Values“ are RECORDED Check on the DMI that: • The Permitted Speed is NOT UPDATED when the balise with VB Cover set is received • The Permitted Speed is UPDATED when the balise with VB Cover not set is received |
| Constraints | All balises are passed in their nominal direction. On-board orientation can be any. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab is open | I | TIU | - |
| Symbol MO01 „Shunting“ is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V<30 km/h) | I | INT | On-board orientation can be any. |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The Speed and Distance monitoring information is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM>40) is recorded | O | JRU | V_PERM is equal to V_NVSHUNT > 40 | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 6 (NID_VBCMK=1; Q_VBCO=1; Q_DIR=1) and packet 6 (NID_VBCMK=2; Q_VBCO=1; Q_DIR=0) and packet 255 is received | I | BTM | Virtual balise cover with NID_VBCMK=1 will be set because it is valid for nominal direction, but Virtual balise cover with NID_VBCMK=2 will NOT be set because it is valid for reverse direction only | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 0 (NID_VBCMK=1), packet 3 (Q_DIR=1; V_NVSHUNT=40) and packet 255 is received | I | BTM | Although packet 3 is valid for nominal direction, it should be rejected because of the packet 6 with NID_VBCMK=1 | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT The speed and distance monitoring information is displayed | O | RTM | The onboard does NOT update the indication of Permitted Speed This proofs that the Virtual Balise Cover with NID_VBCMK=1 has been set. | N/A | N/A | - |
| 9 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 0 (NID_VBCMK=2), packet 3 (Q_DIR=1; V_NVSHUNT=30) and packet 255 is received | I | BTM | Packet 3 is valid for nominal direction and should be accepted because the packet 6 has NID_VBCMK=2 but Virtual Balise Cover with NID_VBCMK=2 has not been set (see step 1) | N/A | N/A | - |
| 10 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The speed and distance monitoring information is displayed | O | RTM | The onboard updates the indication of Permitted Speed, (30 km/h) This proofs that the Virtual Balise Cover with NID_VBCMK=2 has not been set. | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 6 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 1 | - |
| NID_VBCMK | 6 | 1 | - |
| NID_C | 10 | „NID_Ca“ | - |
| T_VBC | 8 | >0 | Validity is more than 0 days |
| NID_PACKET | 8 | 6 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction. This package shall be ignored. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_VBCO | 1 | 1 | - |
| NID_VBCMK | 6 | 2 | - |
| NID_C | 10 | „NID_Ca“ | - |
| T_VBC | 8 | >0 | Validity is more than 0 days |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | „NID_Ca“ | Same as packet 6 of step 1 |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 0 | - |
| NID_VBCMK | 6 | 1 | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies |
| V_NVSHUNT | 7 | 40 | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | = 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1) If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12 |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1 Valid between V_NVKVINT and V_NVKVINT(1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23 |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1) If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12 |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1 Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23 |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | = 0km/h |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1) If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k) |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k) |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1) If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k) |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k) |
| L_NVKRINT | 5 | FINITE VALUE | = 0m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1) |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1) |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | „NID_Ca“ | Same as packet 6 of step 1 |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 0 | - |
| NID_VBCMK | 6 | 2 | - |
| NID_PACKET | 8 | 3 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_VALIDNV | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Identification of national areas to which the set applies |
| N_ITER | 5 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | Identification of additional national area(s) to which the set applies |
| V_NVSHUNT | 7 | 30 | - |
| V_NVSTFF | 7 | FINITE VALUE | - |
| V_NVONSIGHT | 7 | FINITE VALUE | - |
| V_NVLIMSUPERV | 7 | FINITE VALUE | - |
| V_NVUNFIT | 7 | FINITE VALUE | - |
| V_NVREL | 7 | FINITE VALUE | - |
| D_NVROLL | 15 | FINITE VALUE | - |
| Q_NVSBTSMPERM | 1 | FINITE VALUE | - |
| Q_NVEMRRLS | 1 | FINITE VALUE | - |
| Q_NVGUIPERM | 1 | FINITE VALUE | - |
| Q_NVSBFBPERM | 1 | FINITE VALUE | - |
| Q_NVINHSMICPERM | 1 | FINITE VALUE | - |
| V_NVALLOWOVTRP | 7 | FINITE VALUE | - |
| V_NVSUPOVTRP | 7 | FINITE VALUE | - |
| D_NVOVTRP | 15 | FINITE VALUE | - |
| T_NVOVTRP | 8 | FINITE VALUE | - |
| D_NVPOTRP | 15 | FINITE VALUE | - |
| M_NVCONTACT | 2 | FINITE VALUE | - |
| T_NVCONTACT | 8 | FINITE VALUE | - |
| M_NVDERUN | 1 | FINITE VALUE | - |
| D_NVSTFF | 15 | FINITE VALUE | - |
| Q_NVDRIVER_ADHES | 1 | FINITE VALUE | - |
| A_NVMAXREDADH1 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH2 | 6 | FINITE VALUE | - |
| A_NVMAXREDADH3 | 6 | FINITE VALUE | - |
| Q_NVLOCACC | 6 | FINITE VALUE | - |
| M_NVAVADH | 5 | FINITE VALUE | - |
| M_NVEBCL | 4 | FINITE VALUE | - |
| Q_NVKINT | 1 | FINITE VALUE | - |
| Q_NVKVINTSET | 2 | FINITE VALUE | Only if Q_NVKINT = 1, Q_NVKVINTSET and the following variables follow |
| A_NVP12 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| A_NVP23 | 6 | FINITE VALUE | Only if Q_NVKVINTSET = 1 |
| V_NVKVINT | 7 | FINITE VALUE | = 0 km/h |
| M_NVKVINT | 7 | FINITE VALUE | Valid between V_NVKVINT and V_NVKVINT(1) If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12 |
| M_NVKVINT | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1 Valid between V_NVKVINT and V_NVKVINT(1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23 |
| N_ITER | 5 | FINITE VALUE | - |
| V_NVKVINT(n) | 7 | FINITE VALUE | - |
| M_NVKVINT(n) | 7 | FINITE VALUE | Valid between V_NVKVINT(n) and V_NVKVINT(n+1) If Q_NVKVINTSET = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12 |
| M_NVKVINT(n) | 7 | FINITE VALUE | Only if Q_NVKVINTSET = 1 Valid between V_NVKVINT(n) and V_NVKVINT(n+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23 |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NVKVINTSET(k) | 2 | FINITE VALUE | - |
| A_NVP12(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| A_NVP23(k) | 6 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 |
| V_NVKVINT(k) | 7 | FINITE VALUE | = 0km/h |
| M_NVKVINT(k) | 7 | FINITE VALUE | Valid between V_NVKVINT(k) and V_NVKVINT(k,1) If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k) |
| M_NVKVINT(k) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 Valid between V_NVKVINT(k) and V_NVKVINT(k,1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k) |
| N_ITER(k) | 5 | FINITE VALUE | - |
| V_NVKVINT(k,m) | 7 | FINITE VALUE | - |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1) If Q_NVKVINTSET(k) = 1, gives the correction factor if maximum emergency brake deceleration is lower than A_NVP12(k) |
| M_NVKVINT(k,m) | 7 | FINITE VALUE | Only if Q_NVKVINTSET(k) = 1 Valid between V_NVKVINT(k,m) and V_NVKVINT(k,m+1) Gives the correction factor if maximum emergency brake deceleration is higher than A_NVP23(k) |
| L_NVKRINT | 5 | FINITE VALUE | = 0m |
| M_NVKRINT | 5 | FINITE VALUE | Valid between L_NVKRINT and L_NVKRINT(1) |
| N_ITER | 5 | FINITE VALUE | - |
| L_NVKRINT(l) | 5 | FINITE VALUE | - |
| M_NVKRINT(l) | 5 | FINITE VALUE | Valid between L_NVKRINT(l) and L_NVKRINT(l+1) |
| M_NVKTINT | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 9: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | „NID_Ca“ | Same as packet 6 of step 1 |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | O | RTM | - |
| Cab is open | - | TIU | - |
| Symbol MO01 „Shunting“ is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | In SL mode, the ERTMS/ETCS on- board equipment shall only take into account information valid for its movement direction |  | 11 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that when the train overpasses balises in their nominal direction: • A balise message containing a packet valid for reverse direction only is ignored and • A balise message containing a packet valid for nominal direction only is accepted |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.1.3 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session Management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • 2 Balise messages containing packet 42 „Session management“ is RECORDED • Message 155: Initiation of a communication session is RECORDED |
| Constraints | All balises are passed in their nominal direction. Onboard orientation can be any. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | - |
| Cab is closed | I | TIU | - |
| Since cab is closed, nothing is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | Onboard orientation can be any. |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 42 (Q_DIR=0) and packet 255 is received | I | BTM | Packet 42 is valid for reverse direction, so it shall be ignored | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT.Request is transmitted | O | RTM | The OBU does NOT request the set-up of a safe radio connection. This proofs that the packet 42 has been rejected | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 42 (Q_DIR=1) and packet 255 is received | I | BTM | This BG message is received at least some seconds after reception of last BG message in order to rest assured that the onboard does not request the set-up of a safe radio connection. Packet 42 is valid for nominal direction so it should be accepted | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The OBU requests the set-up of a safe radio connection. This proofs that the last packet 42 has been accepted | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | 0 | Valid for reverse direction. This package shall be ignored. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. First balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | 1 | Valid for nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. Second balise in the group |
| N_TOTAL | 3 | 1 | BG has two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Unlinked, so that BG shall be taken into account even of no linking information is used onboard |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | - |
| Cab is closed | I | TIU | - |
| Since cab is closed, nothing is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | Handling of Distances for Location Data |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, TR, PT, NL |  |  |
| Target of Test | Testing the handling of distances for location data (via Packet 39). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.3 |  |
|  |  | Subset-026-3.6.3.2.3 a) |  |
|  |  | Subset-026-3.6.3.2.3 b) |  |
|  |  | Subset-026-3.6.3.2.3 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that at the beginning no change of traction system is displayed (b) For distance (1) the previously received data shall be used.) Check on the DMI that after the distance (2) the change of traction system is displayed. |
| Constraints | No change of traction system is given for distance (1) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 39 (M_VOLTAGE=1) is received | I | BTM | Previous M_VOLTAGE value <> 1 ! | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz announcement" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit69=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz" is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The Track Condition symbol "Change of traction system: AC 25 kV 50 Hz announcement" is removed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit69=0&Bit68=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 39 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_TRACTION | 15 | FINITE VALUE | - |
| M_VOLTAGE | 4 | 1 | Set M_VOLTAGE <> to the actual value. |
| NID_CTRACTION | 10 | FINITE VALUE | NID_CTRACTION given only if M_VOLTAGE ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.6.3: Validity direction of transmitted Information |  | 3060300 |
| Test Case of Feature | Handling of Distances for continuous profile data |  | 13 |
| Applicable Mode/Level Combinations | L2: SB, FS, LS, OS, SR, PT L3: SB, FS, LS, OS, SR, PT |  |  |
| Target of Test | Testing the handling of distances for continuous profile data, using SSP. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-3.6.3.2.2 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 27 SSP is recorded. The received SSP corresponds to site data. One of the last eight LRBG:s is used as reference for the SSP |
| Constraints | To cover (e) If distance (n+1) = 0 then the corresponding profile value n shall still be taken into account.) it has to be ensured, that the distance (n+1) is zero, to check that the onboard takes the value of distance (n) into account. The value of V_STATIC given in packet 27 will have to be the MRSP after accepting the radio message. In PT mode the information “Recognition of Exit from TR mode” has to be received before A cab needs to be active and the Train Data must be valid (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / SB / PT / LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 27 (D_STATIC<>0;V_STATIC=<V1>;N_ITER>1;D_STATIC(k)=0;V_STATIC(k)<>0 ), packet 21 is received | I | RTM | Complete message is accepted V1 will be the MRSP after accepting this message and reaching the location referenced by D_STATIC. Ensure that distance (n+1) is zero! And v_static(n+1) <>0 to check that v_static(n) is used beyond distance(n). | L2/L3 | FS / OS / SR / SB / PT / LS | - |
| 2 | L2/L3 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | L2/L3 | FS / OS / SR / SB / PT / LS | - |
| 3 | L2/L3 | FS / OS / SR / SB / PT / LS | The Mode symbol "Full Supervision" is displayed | O | DMI | Only if not already in FS mode. | L2/L3 | FS | - |
| 4 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | Only if not already in FS mode. | L2/L3 | FS | - |
| 5 | L2/L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is updated to the value V1. Check that for distance(n+1) (has to be zero) the value n is taken into account! | L2/L3 | FS | - |
| 6 | L2/L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;V_PERM=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L2/L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |