| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3050700 (3.5.7: Safe Radio Connection Indication) Total: 17 Test case(s) |  |
| REF: | Subset-076-5-2-3050700 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/New in Baseline 3 | INECO |
| Version 2.9.1 06.08.2013 | All | Second version/ UNISIG remarks from 15.07.2013 included. | INECO |
| Version 2.9.2 25.11.2013 | All | Third version/ After bilateral meeting on 25.11.2013, two new test cases (TC16 TC17) are created. | INECO |
| Version 2.9.3 03.12.2013 | All | Fourth version/ Based on requirements field - Requirements related with A.3.1 table were added; Additional remark from review sheet 004 included. | INECO |
| Version 2.9.4 17.12.2013 | All | Fifth version/ Additional remark from review sheet 004line 36 included. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | If a Start of Mission procedure is ongoing and the final attempt to set up the safe radio connection failed, the ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status . Condition Id [1]. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3 : SB |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs the driver (via DMI) about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status if a Start of Mission procedure is ongoing and the final attempt to set up the safe radio connection failed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 |  |
|  |  | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [1] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 a) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection lost /Set-up failed“ is DISPLAYED. Check on the JRU that: DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The train is at standstill. A Start of Mission is initiated. The Driver Id, Train position, Train running number, Radio network ID and RBC ID/Phone Number are VALID. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |
| Train Running Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request. | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request. | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is DISPLAYED. | O | DMI | After three repetitions of the request to establish a safe radio connection. DMI object: Symbol ST04 | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status if the “connection status” timer expires. Condition Id [2]. |  | 2 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1: FS/ LS/ OS/ SR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status if “connection status” timer expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [2] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.2 |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection lost /Set-up failed“ is DISPLAYED. Check on the JRU that: DMI symbol status “Connection lost/Set-up failed” is RECORDED. |
| Constraints | The status of the safe radio connection is “No connection”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / PT / NL / LS / SN/ RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC/ L1 |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 1" is displayed | O | DMI | DMI object: Symbol LE03 Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | - |
| 2 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | The driver selects "Level 2 or “Level 3" | I | DMI | Level different from the applicable one and among those stored in the table of priority. | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | - |
| 3 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | - |
| 4 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | The driver confirms "Level" | I | DMI | - | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | - |
| 5 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | - |
| 6 | L0 / LNTC/ L1 | FS/ LS/ OS/ SR/ UN / PT/ NL/ SN/ RV | The Level symbol "Level 2"/ "Level 3" is displayed | O | DMI | One of the Symbols LE04 or LE05 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 7 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit04=1/ Bit05=1) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 8 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | SA-CONNECT. Request is transmitted | O | RTM | Connection status timer starts as soon as the first request is sent. | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 9 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 10 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 11 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is DISPLAYED. | O | DMI | After 45s from the first request to set up the safe connection DMI object: Symbol ST04 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |
| 12 | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ PT/ NL/ RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11/ 12/ 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 (CHANGED) |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. The new Level symbol "Level X" is displayed. | O | DMI | DMI Object: Symbol ST04 LX = LE04 / 05 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status if the “connection status” timer expires. Condition Id [2]. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ SH / SL/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status if “connection status” timer expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [2] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 b) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.2 |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up“ to “Connection lost /Set-up failed“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The status of the safe radio connection is “Connection Up”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SH / SL / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT.Indication is received | I | RTM | The connection is lost | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Connection status timer starts as soon as the first request is sent. | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | After 45s from the first request to set up the safe connection. DMI object: ST03 Not apply to SL mode | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed | O | DMI | DMI object: ST04 Not apply to SL mode | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0 ¦ Bit41=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SL/ SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI Object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status if the “connection status” timer expires. Condition Id [2]. |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RIU safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Lost / Set-up failed” status if “connection status” timer expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [2] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.2 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection lost /Set-up failed“ is DISPLAYED. Check on the JRU that: DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The status of the RIU safe radio connection is “No connection”. The train shall be into radio infill area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=1) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | Connection status timer starts as soon as the first request is sent. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | After 45s from the first request to set up the safe connection. DMI object: Symbol ST04 | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio Infill area Information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is DISPLAYED. | O | DMI | DMI Object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status if the “connection status” timer expires. Condition Id [2]. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RIU safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status if “connection status” timer expires. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [2] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 b) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.2 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection indication from “Connection Up“ to “Connection lost /Set-up failed“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The status of the RIU safe radio connection is “Connection Up”. The train shall be into Infill area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost. | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | Connection status timer starts as soon as the first request is sent. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost. | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | N/A | N/A | - |
| 5 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost. | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request after. | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | After 45s from the first request to set up the safe connection DMI object: ST04 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI Object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “No Connection” status if no Start of Mission procedure is going and the requests to set-up a safe radio connection are stopped with the relevant RBC for reason other than the successful set-up. Condition Id [3]. |  | 6 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ SH/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RBC safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “No Connection” status if the requests to set-up a safe radio connection are stopped with RBC for reason other than the successful set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [3] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 b) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.4 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: The radio connection is terminated after receiving a Balise group message that orders to terminate the radio connection. Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection lost /Set-up failed“ and to “No connection“ again is displayed. Check on the JRU that: DMI symbol status “Connection lost/Set-up failed” is RECORDED The removal of DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The status of the RBC safe radio connection is “No Connection”. No Start of Mission procedure is ongoing. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SH / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 42 (Q_RBC=1) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The on-board equipment sends 3 times, until the “connection status” timer expired, a request to establish a new safe radio connection with RBC. Connection status timer starts as soon as the first request is sent. | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol “Safe radio connection - Connection Lost/Set-Up failed” is DISPLAYED. | O | DMI | After 45s from the first request to set up the safe connection Start DMI Object: ST04 | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group message containing packet 42 (Q_RBC=0) is received. | I | BTM | The request to set-up a safe connection is stopped for reason other than the successful set-up. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol “Safe radio connection - Connection Lost/Set-Up failed” is REMOVED. | O | DMI | Stop DMI Object ST04 | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | NO status symbol “No Connection “ is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | NID_C not relevant if NID_RBC has value 16383 |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) On-board will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value 16383 |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Not relevant |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | NID_C not relevant if NID_RBC has value 16383 |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value 16383 |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Not relevant |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SH / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | For “No connection “ status, no symbol is shown. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “No Connection” status if no Start of Mission procedure is going and the requests to set-up a safe radio connection are stopped with the relevant RIU for reason other than the successful set-up. Condition Id [3]. |  | 7 |
| Applicable Mode/Level Combinations | L1 : FS |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “No Connection” status if the requests to set-up a RIU safe radio connection are stopped for reason other than the successful set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [3] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 b) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.4 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |
|  |  | Subset-026-4.7.2 table - Output information Safe radio connection indication |  |
|  |  | Subset-026-A.3.1 - The number of times to try to establish a safe radio connection |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection lost /Set-up failed“ and to “No connection“ again is DISPLAYED. Check on the JRU that: DMI symbol status “Connection lost/Set-up failed” is RECORDED The removal of DMI symbol status “Connection lost/Set-up failed” is RECORDED |
| Constraints | The status of the safe radio connection is “No Connection”. No Start of Mission procedure is ongoing. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=1) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The on-board equipment sends 3 times, until the “connection status” timer expired, a request to establish a new safe radio connection with the RIU. Connection status timer starts as soon as the first request is sent. | N/A | N/A | - |
| 4 | N/A | N/A | The Status symbol “Safe radio connection - Connection Lost/Set-Up failed” is DISPLAYED. | O | DMI | After 45s from the first request to set up the safe connection. Start DMI Object: ST04 | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=0) is received. | I | BTM | The request to set-up a safe connection is stopped for reason other than the successful set-up. | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol “Safe radio connection - Connection Lost/Set-Up failed” is REMOVED. | O | DMI | Stop DMI Object ST04 | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | NO status symbol “No Connection “ is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio Infill Area Information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio Infill Area Information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 2 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if the safe radio connection is set-up. Condition Id [4]. |  | 8 |
| Applicable Mode/Level Combinations | L0 : UN L1 : FS/ LS/ OS/ SR/ SH/ TR/ NL/ RV L2 / L3 : FS/ LS/ OS/ SR/ SH/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if safe radio connection is set-up with RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [4] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection Up” is DISPLAYED. Check on the JRU that: DMI symbol status “Connection Up” is RECORDED |
| Constraints | The status of the safe radio connection is “No connection”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 7 / 11/ 12/ 14 | FS / OS / SR / SH / UN / TR / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 2 / 3/ 4 | L0 / L1 / L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 42 (Q_RBC=1) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol “Safe radio connection - Connection Up” is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | NID_C not relevant if NID_RBC has value 16383 |
| NID_RBC | 14 | 16383 | NID_RBC = 16383 (Contact last known RBC) On-board will initiate communication to last known RBC |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value 16383 |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Not relevant |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 2/ 3/ 4 | L1 / L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the RBC safe radio connection is released. Condition Id [5]. |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ SH/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RBC safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the RBC safe radio connection is released. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [5] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up “ to “No connection“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED. |
| Constraints | The status of the RBC safe radio connection is “Connection Up”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 7 / 11/ 12/ 14 | FS / OS / SR / SH / TR / NL / LS / RV |
| Radio communication session | ESTABLISHED | With RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 42 (Q_RBC=0) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted. | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is REMOVED. | O | DMI | Stop DMI Object ST03 | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | NO status symbol "No Connection " is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | NID_C not relevant if NID_RBC has value 16383 |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value 16383 |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Not relevant |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| Radio communication session | NOT ESTABLISHED | With RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the RIU safe radio connection is released. Condition Id [5]. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RIU safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the RIU safe radio connection is released. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [5] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up “ to “No connection“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED. |
| Constraints | The status of the RIU safe radio connection is “Connection Up”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=0; D_INFILL=0) is received. | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted. | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is REMOVED. | O | DMI | Stop DMI Object ST03 | N/A | N/A | - |
| 9 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | NO status symbol "No Connection " is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio Infill Area Information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RIU |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the safe radio connection is lost and the requests to set-up a safe radio connection are stopped with the relevant RBC for reason other than the successful set-up. Condition Id [6]. |  | 11 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ SH/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RBC safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the requests to set-up a safe radio connection are stopped with RBC for reason other than the successful set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [6] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.4 |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up “ to “No connection“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED. |
| Constraints | The status of the RBC safe radio connection is “Connection Up”. Safe radio connection is lost. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SH / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | With RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The request to connect is sent | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request. | N/A | N/A | - |
| 4 | N/A | N/A | Balise Group message containing packet 42 (Q_RBC=0) is received. | I | BTM | Before the connection status timer expires. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is REMOVED. | O | DMI | Stop DMI Object ST03 | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NO status symbol "No Connection " is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | Session Management |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | NID_C not relevant if NID_RBC has value 16383 |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value 16383 |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Not relevant |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / SB / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | With RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the safe radio connection is lost and the requests to set-up a safe radio connection are stopped with the relevant RIU for reason other than the successful set-up. Condition Id [6]. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the RIU safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the requests to set-up a safe radio connection are stopped with RIU for reason other than the successful set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [6] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.4 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up “ to “No connection“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED. |
| Constraints | The status of the RIU safe radio connection is “Connection Up”. Safe radio connection is lost. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The request to connect is sent | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request. | N/A | N/A | - |
| 4 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=0) is received. | I | BTM | Before the connection status timer expires. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is REMOVED. | O | DMI | Stop DMI Object : ST03 | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | NO status symbol "No Connection " is displayed. | O | DMI | DMI object: Absence of Symbol | N/A | N/A | - |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio Infill Area Information |
| Q_DIR | 2 | 01 | Nominal direction |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_RIU | 14 | FINITE VALUE | Current RIU id |
| NID_RADIO | 64 | FINITE VALUE | Current RIU phone number |
| D_INFILL | 15 | FINITE VALUE | Location where to disconnect the RIU |
| NID_C | 10 | FINITE VALUE | Current country id |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the RBC safe radio connection is lost and the train front is inside an announced radio hole. Condition Id [7]. |  | 13 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS/ SR/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Up” status and “No Connection” status if the train front is inside an announced radio hole. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [7] |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 b) |  |
|  |  | Subset-026-3.5.7.4 |  |
|  |  | Subset-026-3.5.4.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up “ to “No connection“ is DISPLAYED. Check on the JRU that: The removal of DMI symbol status “Connection Up” is RECORDED. |
| Constraints | The status of the safe radio connection is “Connection Up”. Safe radio connection is lost and the train front is inside an announced radio hole. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | With RBC |
| Track conditions excluding big metal masses | STORED | Radio Hole |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The connection is lost | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The request to connect is sent | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | The safe connection is not confirmed. Repetition of the request. | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | The train front is inside the radio hole area. | N/A | N/A | - |
| 5 | N/A | N/A | The Track Condition symbol "Radio hole" is displayed. | O | DMI | Supervision of radio safe connection stopped DMI Object: TC12 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit55=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is REMOVED. | O | DMI | Before the connection status timer expires. Stop DMI Object ST03 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NO status symbol "No Connection " is displayed. | O | DMI | Before connection status timer expires DMI object: Absence of Symbol | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 8 / 11/ 12/ 14 | FS / OS / SR / TR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | With RBC |
| Track conditions excluding big metal masses | STORED | Radio Hole |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | During SoM, the ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if the safe radio connection is set-up. Condition Id [4]. |  | 14 |
| Applicable Mode/Level Combinations | L2 / L3 : SB |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if safe radio connection is set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [4] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection Up” is DISPLAYED. Check on the JRU that: DMI symbol status “Connection Up” is RECORDED |
| Constraints | The status of the safe radio connection is “No connection”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |
| Train Running Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol “Safe radio connection - Connection Up” is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Driver ID | VALID | Entered after the desk is opened |
| Radio Network ID | VALID | - |
| RBC ID/Phone Number | VALID | - |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “Connection Up” status if the safe radio connection is set-up. Condition Id [4]. |  | 15 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “Connection Up” status if safe radio connection is set-up. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.4.2 |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [4] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Lost / Set-up failed” to “Connection Up” is DISPLAYED. Check on the JRU that: DMI symbol status “Connection Up” is RECORDED |
| Constraints | The status of the safe radio connection is “Connection Lost / Set-up failed” status. SoM is completed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI Object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol “Safe radio connection - Connection Up” is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if the safe radio connection is set-up. Condition Id [4]. |  | 16 |
| Applicable Mode/Level Combinations | L1 : FS / LS/ OS / SR |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “No connection” status and “Connection Up” status if safe radio connection is set-up with RIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [4] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio Infill area information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “No Connection“ to “Connection Up” is DISPLAYED. Check on the JRU that: DMI symbol status “Connection Up” is RECORDED |
| Constraints | The status of the safe radio connection is “No connection”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of Symbol for “No connection “ status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=1) is received. | I | BTM | Packet “Radio infill area information” | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol “Safe radio connection - Connection Up” is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio infill area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | With RIU |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.7: Safe Radio Connection Indication |  | 3050700 |
| Test Case of Feature | The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “Connection Up” status if the safe radio connection is set-up. Condition Id [4]. |  | 17 |
| Applicable Mode/Level Combinations | L1 : SR |  |  |
| Target of Test | The target of this test case is to check that: The ERTMS on-board equipment informs (via DMI) the driver about the status of the safe radio connection. The ERTMS/ETCS shall execute the transition between “Connection Lost / Set-up failed” status and “Connection Up” status if safe radio connection is set-up with RIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.7.5 table 1 |  |
|  |  | Subset-026-3.5.7.5 Condition Id [4] |  |
|  |  | Subset-026-3.5.7.3 |  |
|  |  | Subset-026-3.5.7.3 a) |  |
|  |  | Subset-026-3.5.7.1 |  |
|  |  | Subset-026-3.5.7.7 |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio Infill area information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Lost / Set-up failed” to “Connection Up” is DISPLAYED. Check on the JRU that: DMI symbol status “Connection Up” is RECORDED |
| Constraints | The status of the safe radio connection is “Connection Lost / Set-up failed” status. Degraded situation of SoM due to loss of safe radio connection. The driver selects Override procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | DMI Object: Symbol ST04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 133 (Q_RIU=1) is received. | I | BTM | Packet “Radio infill area information” | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol “Safe radio connection - Connection Up” is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 133 | Radio infill area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Two balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BG 1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: Symbol ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |