| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3180500 (3.18.5: Date and Time) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3180500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 21.11.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.5: Date and Time |  | 3180500 |
| Test Case of Feature | Availability of UTC and Local Times on-board |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, NL, PS LNTC : SL, SB, NL, SN, PS L1: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV, PS L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV, PS L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, NL, RV, PS |  |  |
| Target of Test | The target of this test is to check that: The function Provide date and time is active On the DMI: • The on-board equipment presents the time in local time format to the driver in all situations (if the DMI is active). On the JRU: • UTC Time is recorded |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.5.1 |  |
|  |  | Subset-026-4.5.2.1 table - Provide Date and Time |  |
|  |  | Subset-026-4.7.2 table - Output information Local time |  |
|  |  | Subset-026-3.18.5.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • local time is presented to the driver in local time format (hour, minute, second). Check on the JRU that: • UTC Date and Time is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Local time is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Local time is DISPLAYED on DMI. | O | DMI | The local time (hour, minute, second) is shown to the driver in all situations and after all permitted mode- or level transitions for this feature (if the DMI is active). | N/A | N/A | - |
| 2 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Local time is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |