| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3150700 (3.15.7 Tolerance of Big Metal Mass) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3150700 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 26.09.2013 | All | review | DLR |
| Version 2.9.2 03.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.7 Tolerance of Big Metal Mass |  | 3150700 |
| Test Case of Feature | A big metal masses alarm shall be ignored for a defined distance |  | 1 |
| Applicable Mode/Level Combinations | L0 : UN, SL, SB, SH, NL LNTC: SN, SL, SB, SH, NL |  |  |
| Target of Test | 3.15.7.2 In Levels 0/NTC, the alarms which may be triggered by metal masses shall be ignored for a defined distance (see A3.1). If the alarm persists for a longer distance the ERTMS/ETCS on-board equipment shall trigger a safety reaction. The target of this test is to check that: • A big metal masses alarm shall be ignored for a defined distance D_METAL = 300 m • The on-board equipment shall trigger a safety reaction if distance > D_METAL = 300 m |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.7.2 |  |
|  |  | Subset-026-A.3.1 - Distance of metal immunity in Levels 0/NTC |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that the onboard integrity check alarms are ignored for the first 300 m of a big metal mass (no reaction is triggered), but as soon as this length is exceeded a safety reaction is triggered. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 6 / 11 / 13 | SH / UN / SL / SB / NL/ SN |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | At d0, the train reaches the starting point of a big metal object in the track, exceeding the limits for big metal masses as defined in Subset-036, section 6.5.2 “Metal Masses in the Track”. The length of the big metal mass is greater than 300 m. | N/A | N/A | - |
| 2 | N/A | N/A | ERTMS/ETCS on-board equipment does NOT trigger a safety reaction | O | INT | The alarms which may be triggered by metal masses shall be ignored The safety reaction has to be specified by the OBU supplier. | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | At d0+300 m, the train has not reached yet the end point of the big metal object in the track | N/A | N/A | - |
| 4 | N/A | N/A | ERTMS/ETCS on-board equipment does trigger a safety reaction | O | INT | The alarms which may be triggered by metal masses shall no loger be ignored The safety reaction has to be specified by the OBU supplier. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 4 / 5 / 6 / 11 / 13 | SH / UN / SL / SB / NL/ SN |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Safety reaction is applied | O | INT | - |