| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4060300 (4.6.3: Transitions Conditions Table) Total: 68 Test case(s) |  |
| REF: | Subset-076-5-2-4060300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/Creation | DLR |
| Version 2.9.0 05.10.2012 | All | Second version/review for Baseline 3; Format Template updated. TCs from all partners. | INECO/CEDEX/MULTITEL/DLR |
| Version 2.9.1 05.08.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.2 29.10.2013 | All | Fourth version/ UNISIG Review from 02/09/2013 included; TCEditor_v31 tool used; TC 69 created new to cover req. 5.7.2.4 | INECO/MULTITEL |
| Version 2.9.3 10.12.2013 | All | Fifth version/ UNISIG Review from 05/12/2013 included | INECO |
| Version 2.9.4 17.12.2013 | All | Sixth version/ Additional changes from review sheet 004 | INECO |
| Version 2.9.5 27.12.2013 | All | Seventh version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 2.9.6 22.01.2014 | TC58, TC69 | Eight version/ Additional changes from review sheet 010, line 267, implemented. | INECO |
| Version 2.9.7 24.01.2014 | TC24 | Ninth version/ Additional changes from review sheet 012, line 268, implemented | INECO |
| Version 2.9.8 04.02.2014 | TC40 | Eleventh version/ Correction of constrains: Feature 196 (old reference) replaced with FT5050300.3; Step 9 inserted new. See Review sheet 013. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  | 1 |
| Applicable Mode/Level Combinations | L0 / L1 / L2 / L3 / LNTC: FS/ LS/ OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board switches to NP mode There are no messages recorded in the JRU during a period of time. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 1 column 2 |  |
|  |  | Subset-026-4.6.2 row 1 column 3 |  |
|  |  | Subset-026-4.6.2 row 1 column 4 |  |
|  |  | Subset-026-4.6.2 row 1 column 5 |  |
|  |  | Subset-026-4.6.2 row 1 column 6 |  |
|  |  | Subset-026-4.6.2 row 1 column 7 |  |
|  |  | Subset-026-4.6.2 row 1 column 8 |  |
|  |  | Subset-026-4.6.2 row 1 column 9 |  |
|  |  | Subset-026-4.6.2 row 1 column 10 |  |
|  |  | Subset-026-4.6.2 row 1 column 11 |  |
|  |  | Subset-026-4.6.2 row 1 column 12 |  |
|  |  | Subset-026-4.6.2 row 1 column 13 |  |
|  |  | Subset-026-4.6.2 row 1 column 14 |  |
|  |  | Subset-026-4.6.2 row 1 column 16 |  |
|  |  | Subset-026-4.6.2 row 1 column 17 |  |
|  |  | Subset-026-4.6.3 Condition Id [29] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: There are no messages recorded in the JRU during a period of time, after mode change to NP. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 9/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ SF/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | Except if in SL and PS mode |
| NOT RELEVANT | - | BTM | - |
| The ERTMS/ETCS on-board unit is powered up | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS / OS/ SR/ SH/ UN/ PS / SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | The ERTMS/ETCS on-board unit is powered down | I | INT | - | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | NP | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded | O | JRU | There are no messages recorded in the JRU during a period of time | L0/ LNTC/ L1/ L2/ L3 | NP | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | - | NP |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | There is no indication of current mode NP |
| NOT RELEVANT | - | BTM | - |
| The ERTMS/ETCS on-board unit is powered down | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Sleeping signal is lost in SL mode and train is standstill. Transition Condition Id [3]. |  | 2 |
| Applicable Mode/Level Combinations | L0: SL L1: SL L2: SL L3: SL LNTC: SL |  |  |
| Target of Test | The target of this test is to check that: If the train is at standstill in SL mode and no “go sleeping” signal is received any more the on-board equipment switches immediately to SB Mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.8 |  |
|  |  | Subset-026-4.6.2 row 2 column 9 |  |
|  |  | Subset-026-4.6.3 Condition Id [3] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | The on-board is in SL mode (How the train enters in SL mode do not matter). Train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Sleeping requested Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ L1/ L2/ L3/ LNTC | SL | “Go sleeping” input signal is lost. | I | TIU | - | L0/ L1/ L2/ L3/ LNTC | SL | - |
| 2 | L0/ L1/ L2/ L3/ LNTC | SL | SLEEPING INPUT (NID_MESSAGE_JRU=30; M_SLEEPING=0) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SL | - |
| 3 | L0/ L1/ L2/ L3/ LNTC | SL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Triggering event: When the mode changes | L0/ L1/ L2/ L3/ LNTC | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Sleeping not requested Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS / SR L2 : FS / LS / OS / SR L3 : FS / LS / OS / SR LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to UN mode as soon as the ETCS level has changed to level 0 (DMI). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 11 column 5 |  |
|  |  | Subset-026-4.6.2 row 11 column 6 |  |
|  |  | Subset-026-4.6.2 row 11 column 7 |  |
|  |  | Subset-026-4.6.2 row 11 column 8 |  |
|  |  | Subset-026-4.6.2 row 11 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [21] |  |
|  |  | Subset-026-4.7.2 table - Output information ERTMS/ETCS Mode |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed on the DMI changes to UN mode when the ETCS level changed to L0. Check on the JRU that: The new current mode UN is recorded. |
| Constraints | Conditions for the level transition to level 0 shall be fulfilled. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 / 13 | FS / OS / SR / LS / SN |
| Movement Authority | STORED | Only if in FS, LS or OS |
| Gradient Profile | STORED | Only if in FS, LS or OS |
| International SSP | STORED | Only if in FS, LS or OS |
| Level Transition Order | STORED | Only if in FS, LS or OS |
| Mode Profile | STORED | Only if in LS or OS |
| ERTMS/ETCS level | 1/ 2/ 3/ 4 | LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 LNTC | FS / LS/ OS / SR / SN | The level transition to L0 is performed | - | - | USE_ FT5100100.12: “Order to switch immediately to Level 0 at level border (Level transition L1 / L0)” or USE_ FT5100100.11: “Order to switch immediately to Level 0 at level border (Level transition L2 / L0)” or USE_ FT5100100.29: “Order to switch immediately to Level 0 at level border (Level transition L3 / L0)” or USE_ FT5100100.24: “Order to switch immediately to Level 0 at level border (Level transition LNTC / L0)” | L0 | FS / LS/ OS / SR / SN | - |
| 2 | L0 | FS / LS/ OS / SR / SN | The driver acknowledge the level transition. | - | - | USE_FT5100400.6 Optional step if level was ack before the level transition | L0 | FS / LS/ OS / SR / SN | - |
| 3 | L0 | FS / LS/ OS / SR / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded | O | JRU | - | L0 | UN | - |
| 4 | L0 | UN | The Mode symbol “Unfitted” is displayed | O | DMI | DMI Object: MO16 | L0 | UN | - |
| 5 | L0 | UN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded | O | JRU | - | L0 | UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level 0” is displayed. The Mode symbol “Unfitted” is displayed. | O | DMI | DMI Object: LE01 DMI Object: MO16 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | ETCS on-board system detects safety relevant fault. Transition Condition Id [13]. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH / UN / PS / SB /TR L1: FS / LS/ OS / SR / SH / PS/ SB / TR / PT / RV L2: FS / LS/ OS / SR / SH / PS/ SB / TR / PT / RV L3: FS / LS/ OS / SR / SH / PS/ SB / TR / PT / RV LNTC: SH / PS / SB /TR / SN |  |  |
| Target of Test | The target of this test case is to check that: On-board equipment switches to SF mode in case of a fault, which affects safety. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.5.1.1 |  |
|  |  | Subset-026-4.6.2 row 14 column 2 |  |
|  |  | Subset-026-4.6.2 row 14 column 3 |  |
|  |  | Subset-026-4.6.2 row 14 column 4 |  |
|  |  | Subset-026-4.6.2 row 14 column 5 |  |
|  |  | Subset-026-4.6.2 row 14 column 6 |  |
|  |  | Subset-026-4.6.2 row 14 column 7 |  |
|  |  | Subset-026-4.6.2 row 14 column 8 |  |
|  |  | Subset-026-4.6.2 row 14 column 11 |  |
|  |  | Subset-026-4.6.2 row 14 column 12 |  |
|  |  | Subset-026-4.6.2 row 14 column 13 |  |
|  |  | Subset-026-4.6.2 row 14 column 16 |  |
|  |  | Subset-026-4.6.2 row 14 column 17 |  |
|  |  | Subset-026-4.6.3 Condition Id [13] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that The displayed mode changes from current mode to SF mode. Check on the JRU that The new current mode SF is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 6/ 7/ 8/ 12/ 13/ 14 / 15 | FS / OS /SR / SH / UN / SB / / TR / PT /LS / SN / RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS / LS / OS /SR / SH / UN / PS / SB / TR / PT / SN / RV | A safety critical fault occurs. | I | INT | On-board detects a fault, which affects safety. The on-board equipment switches to SF mode. | L0/ LNTC/ L1/ L2/ L3 | FS / LS / OS /SR / SH / UN / PS / SB / TR / PT / SN / RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS / LS / OS /SR / SH / UN / PS / SB / TR / PT / SN / RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=9) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SF | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | SF | The Mode symbol “System Failure” is displayed | O | DMI | ERA_ERTMS_015560_v3.3.0 8.2.3.1.2 Exception: If MO18 cannot be displayed on the total image display area, the SF mode shall be indicated by any other means (e.g. blank screen, frozen screen, dedicated lamp, sound,...). | L0/ LNTC/ L1/ L2/ L3 | SF | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SF | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit33=1>) is recorded | O | JRU | Optionally | L0/ LNTC/ L1/ L2/ L3 | SF | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 9 | SF |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “System Failure” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Driver isolates ERTMS / ETCS on-board equipment. Transition Condition Id [1]. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH / UN / PS / SL / SB / SF / NL L1: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / SF / NL / RV L2: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / SF / NL / RV L3: FS / LS / OS / SR / SH / PS / SL / SB / TR / PT / SF / NL / RV LNTC: PS / SL / SB / SF / NL / SN |  |  |
| Target of Test | The target of this test case is to check that: On-board switches to IS mode if the driver isolates the ERTMS/ETCS on-board equipment Comment: There is no indication in IS mode on the display; but the isolation status has to be indicated by another way [SRS 4.4.3.1.2 and 4.7.2.1.1]. (For the test has to be defined an indication device, because there are no requirements to the way, how the isolation status has to be shown to the driver.) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.3.1.2 |  |
|  |  | Subset-026-4.6.2 row 15 column 1 |  |
|  |  | Subset-026-4.6.2 row 15 column 2 |  |
|  |  | Subset-026-4.6.2 row 15 column 3 |  |
|  |  | Subset-026-4.6.2 row 15 column 4 |  |
|  |  | Subset-026-4.6.2 row 15 column 5 |  |
|  |  | Subset-026-4.6.2 row 15 column 6 |  |
|  |  | Subset-026-4.6.2 row 15 column 7 |  |
|  |  | Subset-026-4.6.2 row 15 column 8 |  |
|  |  | Subset-026-4.6.2 row 15 column 9 |  |
|  |  | Subset-026-4.6.2 row 15 column 10 |  |
|  |  | Subset-026-4.6.2 row 15 column 11 |  |
|  |  | Subset-026-4.6.2 row 15 column 12 |  |
|  |  | Subset-026-4.6.2 row 15 column 13 |  |
|  |  | Subset-026-4.6.2 row 15 column 14 |  |
|  |  | Subset-026-4.6.2 row 15 column 16 |  |
|  |  | Subset-026-4.6.2 row 15 column 17 |  |
|  |  | Subset-026-4.6.3 Condition Id [1] |  |
|  |  | Subset-026-4.7.2 table - Input information Isolation |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The Driver isolates the on-board equipment through the means offered by the ERTMS/ETCS DMI.Check on TIU that: The isolation status of isolated ETCS on-board equipment is indicated. Check on the JRU that The change to the new current mode IS is recorded. |
| Constraints | There is no indication in IS mode on the display; but the isolation status has to be indicated by another way [SRS 4.4.3.1.2 and 4.7.2.1.1]. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 6/ 7/ 8/ 9/ 11/ 12/ 13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ SF/ NL/ LS/ SN/ RV/ PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| ETCS not isolated | O | TIU | - |
| Current mode is displayed | O | DMI | In modes as NP, PS and SL current level is NOT DISPLAYED. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | The driver isolates the ERTMS/ETCS on-board equipment. | I | DMI | The ERTMS/ETCS DMI offer a means to isolate the ERTMS/ETCS on-board equipment Note: The location and form of this are implementation dependent. [ERA_ERTMS_015560_v3.6.0 req. 5.6.1.1 and 5.6.1.1.1] | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 0010) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | ETCS isolated | O | TIU | - | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ SH/ UN/ PS/ SL/ SB/ TR/ PT/ SF/ NL/ SN/ RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=10) is recorded | O | JRU | Triggering event: The Mode change to IS | L0/ LNTC/ L1/ L2/ L3 | IS | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | IS | The current mode IS mode is displayed by any means, not by a symbol. | O | DMI | There is no indication in IS mode on the display. ERA_ERTMS_015560_v3.6.0, req. 8.2.3.1.2.2 Exception: The mode IS shall be indicated by any means, for example any text message (e.g. by the isolation device) [according SRS 4.7.2]. | L0/ LNTC/ L1/ L2/ L3 | IS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 10 | IS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| ETCS isolated | O | TIU | - |
| NOT RELEVANT | - | DMI | There is no indication in IS mode on the display. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-Board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | ETCS level switches to Level NTC .Transition Condition Id [56]. |  | 6 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: When the announcement to LNTC is received, a signal is sent by the ERTMS/ETCS on-board to the STM interface specified in the announcement meaning that the STM is activated when the level transition location is passed with the estimated front end. On-board mode switches to SN when Level transitions to LNTC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 16 column 5 |  |
|  |  | Subset-026-4.6.2 row 16 column 6 |  |
|  |  | Subset-026-4.6.2 row 16 column 7 |  |
|  |  | Subset-026-4.6.2 row 16 column 8 |  |
|  |  | Subset-026-4.6.2 row 16 column 11 |  |
|  |  | Subset-026-4.6.3 Condition Id [56] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : An indication of LNTC is DISPLAYED An indication of SN mode is DISPLAYED. Check on the JRU that: Level NTC and SN Mode are RECORDED. |
| Constraints | Received NID_NTC corresponds to a SN equipment. Driver is responsible for entering the level NTC in the corresponding area at a speed not exceeding the maximum speed of the unequipped line. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 | FS / OS / SR / UN / LS |
| Level Transition Order | STORED | Transition to LNTC |
| ERTMS/ETCS level | 0/ 2/ 3/ 4 | L0/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 | FS / LS / OS / SR / UN | The train executes a level transition to LNTC. | - | - | USE_FT5100100.16: “Order to switch immediately to Level NTC from L0 at level Border“ USE_FT5100100.34:“Order to switch immediately to Level NTC form L1at level Border” USE_FT5100100.30: “Order to switch immediately to Level NTC from L2 at level Border” USE_FT5100100.25: “Order to switch immediately to Level NTC from L3 at level Border” USE_FT5100100.19 “ Order to switch immediately to Level NTC[Y] from L NTC[X] at level border | LNTC | FS / LS / OS / SR / UN | - |
| 2 | LNTC | FS / LS / OS / SR / UN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded | O | JRU | - | LNTC | SN | - |
| 3 | LNTC | SN | The Mode symbol “National System” is displayed | O | DMI | DMI Object: MO19 | LNTC | SN | - |
| 4 | LNTC | SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded | O | JRU | - | LNTC | SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level NTC” is displayed. The Mode symbol “National System” is displayed. | O | DMI | DMI Object: LE02 DMI Object: MO19 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A desk is opened .Transition Condition Id [2]. |  | 7 |
| Applicable Mode/Level Combinations | L0: SL L1: SL L2: SL L3: SL LNTC: SL |  |  |
| Target of Test | The target of this test is to check that: If the train is at standstill in SL mode and the driver opens a desk, the on-board switches immediately to SB mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 9 |  |
|  |  | Subset-026-4.6.3 Condition Id [2] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode indication is DISPLAYED. Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | The engine is remote controlled by the leading engine Desk is closed. The on-board is in SL mode (How the train entered in SL mode does not matter) Train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| ERTMS/ETCS level | 0/1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active Sleeping requested | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ L1/ L2/ L3/ LNTC | SL | CAB active | I | TIU | The ERTMS/ETCS on-board equipment switches immediately to SB mode. | L0/ L1/ L2/ L3/ LNTC | SL | - |
| 2 | L0/ L1/ L2/ L3/ LNTC | SL | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SL | - |
| 3 | L0/ L1/ L2/ L3/ LNTC | SL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 4 | L0/ L1/ L2/ L3/ LNTC | SB | The Mode symbol “Stand By” is displayed | O | DMI | DMI Object: MO13 | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 5 | L0/ L1/ L2/ L3/ LNTC | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/1/ 2/ 3/ 4 | L0/ L1/ L2/ L3/ LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Stand By” is displayed. | O | DMI | DMI Object: MO13 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The driver closes the desk. The on-board equipment immediately switches to SB mode and the end of mission procedure is triggered (there is a radio link with RBC). Transition Condition Id [28]. |  | 8 |
| Applicable Mode/Level Combinations | L2 : FS/ LS/ OS/ SR/ PT/ NL/ RV L3 : FS/ LS/ OS/ SR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that : The on-board equipment immediately switches to SB mode as soon as the driver closes the desk. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 5 |  |
|  |  | Subset-026-4.6.2 row 2 column 6 |  |
|  |  | Subset-026-4.6.2 row 2 column 7 |  |
|  |  | Subset-026-4.6.2 row 2 column 8 |  |
|  |  | Subset-026-4.6.2 row 2 column 10 |  |
|  |  | Subset-026-4.6.2 row 2 column 13 |  |
|  |  | Subset-026-4.6.2 row 2 column 17 |  |
|  |  | Subset-026-4.6.3 Condition Id [28] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to SB mode when the desk is closed. Check on the JRU that: Closing of desk is RECORDED The new current mode SB is RECORDED. |
| Constraints | The transition “Desk is closed (Transition [28]) “is only realisable at standstill. The entry of the ERTMS/ETCS on-board equipment into the SB mode, from PT mode is considered as an EoM if there was an ongoing mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 / 14 | FS / OS / SR / PT / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS/ OS / SR / PT / NL / RV | CAB not active | I | TIU | The driver closed the desk. The on-board equipment switches immediately to SB mode. | L2 / L3 | FS / LS/ OS / SR / PT / NL / RV | - |
| 2 | L2 / L3 | FS / LS/ OS / SR / PT / NL / RV | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded | O | JRU | - | L2 / L3 | FS / LS/ OS / SR / PT / NL / RV | - |
| 3 | L2 / L3 | FS / LS/ OS / SR / PT / NL / RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | End of mission procedure is triggered. | - | - | USE_FT5050300.3: Inform Trackside about end of mission Or USE_FT5050400.2: Degradate situation for EoM procedure | L2 / L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Desks are closed. Transition Condition Id [28]. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / LS / OS / SR / PT / NL / RV |  |  |
| Target of Test | The target of this test case is to check that : As soon as the driver closes the desk the on-board switches immediately to SB mode. Note: For SN mode, the conditions to end of mission depends on the National System |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 5 |  |
|  |  | Subset-026-4.6.2 row 2 column 7 |  |
|  |  | Subset-026-4.6.2 row 2 column 8 |  |
|  |  | Subset-026-4.6.2 row 2 column 10 |  |
|  |  | Subset-026-4.6.2 row 2 column 11 |  |
|  |  | Subset-026-4.6.2 row 2 column 16 |  |
|  |  | Subset-026-4.6.2 row 2 column 17 |  |
|  |  | Subset-026-4.6.2 row 2 column 13 |  |
|  |  | Subset-026-4.6.2 row 2 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [28] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed on the DMI changes to SB mode when the desk is closed. But is not displayed. Check on the JRU that: The new current mode SB is recorded. Closing of desk is RECORDED. |
| Constraints | The transition “Desks are closed (Transition [28]) “is only realisable at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 8 / 11/ 12 / 13 / 14 | FS / OS / SR / UN / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| Current mode is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / LS / OS / SR / UN / PT / NL / SN / RV | CAB not active | I | TIU | Driver closes the desk. | L0 / LNTC / L1 | FS / LS / OS / SR / UN / PT / NL / SN / RV | - |
| 2 | L0 / LNTC / L1 | FS / LS / OS / SR / UN / PT / NL / SN / RV | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / LS / OS / SR / UN / PT / NL / SN / RV | - |
| 3 | L0 / LNTC / L1 | FS / LS / OS / SR / UN / PT / NL / SN / RV | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0 / LNTC / L1 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The ETCS on-board equipment is powered .Transition Condition Id [4]. |  | 10 |
| Applicable Mode/Level Combinations | L0: NP L1: NP L2: NP L3: NP LNTC: NP |  |  |
| Target of Test | The target of this test case is to check that: The on-board changes from NP to SB mode if power is switched on. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 1 |  |
|  |  | Subset-026-4.6.3 Condition Id [4] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that The new current mode SB is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | NP |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | NP | ERTMS/ETCS on-board unit is powered up | I | INT | The on-board equipment changes to SB mode. | L0/ LNTC/ L1/ L2/ L3 | NP | - |
| 2 | L0/ L1/ L2/ L3/ LNTC | NP | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| All desks closed | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | No “Non leading” input signal is received any more and the train is at standstill. The on-board equipment immediately switches to SB mode. Transition Condition Id [47]. |  | 11 |
| Applicable Mode/Level Combinations | L0: NL L1: NL L2: NL L3: NL LNTC: NL |  |  |
| Target of Test | The target of this test is to check that: If the train is at standstill in NL mode AND no NL input signal is received any more: The on-board equipment switches immediately to SB Mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 10 |  |
|  |  | Subset-026-4.6.3 Condition Id [47] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode Indication is DISPLAYED. Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | The on-board is in NL mode. Train is at standstill. There is radio link connection. (Only in L2/L3). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Non-leading permitted | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC /L1/ L2/ L3 | NL | Non-leading not permitted | I | TIU | NL Input signal is not detected | L0 / L1/ L2/ L3 / LNTC | NL | - |
| 2 | L0 / LNTC /L1/ L2/ L3 | NL | NON LEADING INPUT (NID_MESSAGE_JRU=32; M_NON_LEADING=0) is recorded. | O | JRU | - | L0 / L1/ L2/ L3 / LNTC | NL | - |
| 3 | L0/ L1/ L2/ L3/ LNTC | NL | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 4 | L0 / LNTC /L1/ L2/ L3 | SB | The Mode symbol “Stand By” is displayed | O | DMI | Symbol MO13 | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Stand By” is displayed | O | DMI | DMI Object: Symbol MO13 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A communication session is open. The train is in SB, in L2 or L3, at standstill and receiving the sleeping signal and all desks connected to the on-board equipment are closed. The on-board equipment switches to SL mode. Transition Condition Id [14]. |  | 12 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: A communication session is opened The train is at standstill in SB mode, and receiving the sleeping input signal and all desks connected to the on-board equipment are closed The on-board equipment switches to SL mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.2 |  |
|  |  | Subset-026-4.4.6.1.5 |  |
|  |  | Subset-026-4.6.2 row 9 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [14] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that : The “Sleeping” input signal is received Check on the JRU that: The transition to SL is recorded |
| Constraints | A communication session is open. End of mission not yet executed or SOM not finished. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Sleeping not requested Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | Sleeping requested | I | TIU | “Sleeping” input signal is DETECTED. | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | SLEEPING INPUT (NID_MESSAGE_JRU=30; M_SLEEPING= 1) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | CAB not active | I | TIU | Desks are closed. | L2 / L3 | SB | - |
| 4 | L2 / L3 | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0 ) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 5 | L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=5) is recorded | O | JRU | - | L2 / L3 | SL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | I/O | RTM | - |
| Sleeping requested Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | No communication session is open. The train is in SB, in L0, L1, L2, L3 or LNTC, at standstill, all desks are closed and receives the sleeping signal. The on-board equipment switches to SL mode. Transition Condition Id [14]. |  | 13 |
| Applicable Mode/Level Combinations | L0: SB L1: SB L2: SB L3: SB LNTC:SB |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SL when the train is at standstill and all desks connected to the ERTMS/ETCS on-board equipment are closed and receiving the “sleeping” input signal. No communication session is open |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.6.1.2 |  |
|  |  | Subset-026-4.4.6.1.5 |  |
|  |  | Subset-026-4.6.2 row 9 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [14] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that : The “sleeping” input signal is received Check on the JRU that: The transition to SL is recorded |
| Constraints | The train is at standstill. No communication session is open. (only in L2 / L3) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | Disconnected in L2 / L3, not relevant in L0/LNTC/L1. |
| Sleeping not requested Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / L1 / L2 / L3 / LNTC | SB | Sleeping requested | I | TIU | “Sleeping” input signal is DETECTED. | L0 / L1 / L2 / L3 / LNTC | SB | - |
| 2 | L0 / L1 / L2 / L3 / LNTC | SB | SLEEPING INPUT (NID_MESSAGE_JRU=30; M_SLEEPING=1) is recorded | O | JRU | - | L0 / L1 / L2 / L3 / LNTC | SB | - |
| 3 | L0 / L1 / L2 / L3 / LNTC | SB | CAB not active | I | TIU | Desks are closed. | L0 / L1 / L2 / L3 / LNTC | SB | - |
| 4 | L0 / L1 / L2 / L3 / LNTC | SB | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0 ) is recorded | O | JRU | - | L0 / L1 / L2 / L3 / LNTC | SB | - |
| 5 | L0 / L1 / L2 / L3 / LNTC | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=5) is recorded | O | JRU | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=5) is recorded | L0 / L1 / L2 / L3 / LNTC | SL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET UP | I/O | RTM | Disconnected in L2 / L3, not relevant in L0/LNTC/L1. |
| Sleeping requested Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The system is in level L0 (and mode UN), switches to 1, 2 or 3 and to SH mode. Transition Condition Id [61]. |  | 14 |
| Applicable Mode/Level Combinations | L0: UN |  |  |
| Target of Test | The target of this test is to check that: A Mode profile defining a SH area is available on-board and The maximum safe front end of the train has overpassed the beginning of the SH area (the distance defined by D_MAMODE in packet 80). The system switches to shunting mode in level 1, 2, or 3 when the train passes the level transition border. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 4 column 11 |  |
|  |  | Subset-026-4.6.3 Condition Id [61] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-5.7.2.3 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Information about level (L1 / L2 / L3) and mode (SH) transition are DISPLAYED Check on the JRU that: The level and mode transitions are RECORDED |
| Constraints | The train is passing the level transition border. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1. |
| Level Transition Order | STORED | Transition to L1 / L2 / L3 |
| Mode Profile | STORED | SH (D_MAMODE) |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET UP | I/O | RTM | If level transition to L2/3 is stored |
| NOT RELEVANT | - | TIU | - |
| Indication of current mode (UN) and level (L0) is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | UN | Train is moving. | I | INT | The train reaches the point at the distance defined by the variable D_MAMODE (in packet 80) and the variable D_LEVELTR with its maximum safe front end | L0 | UN | - |
| 2 | L0 | UN | The level transition is performed. | - | - | USE_FT5100100.15: “Order to switch immediately to Level 1 at level border (Level transition L0 / L1)” OR USE_FT5100100.13: “Order to switch immediately to Level 2 at level border (Level transition L0 / L2)” OR USE_FT5100100.26: “Order to switch immediately to Level 3 at level border (Level transition L0 / L3)” | L1 / L2 /L3 | UN | - |
| 3 | L1 / L2 / L3 | UN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | L1 / L2 /L3 | SH | - |
| 4 | L1/ L2/L3 | SH | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | DMI Object: Symbol MO02 | L1/ L2/L3 | SH | - |
| 5 | L1/ L2/L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L1/ L2/L3 | SH | - |
| 6 | L1/ L2/L3 | SH | The driver acknowledges “Shunting” | I | DMI | The driver acknowledges the request of SH mode within the driver acknowledgment time (5 s). | L1/ L2/L3 | SH | - |
| 7 | L1/ L2/L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded | O | JRU | - | L1/ L2/L3 | SH | - |
| 8 | L1 / L2 / L3 | SH | The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: Symbol MO01 | L1 / L2 /L3 | SH | - |
| 9 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L1 / L2 /L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level <L1 / L2 / L3>” is displayed. The mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The on-board device is in level LNTC (and mode SN), switches to level 1, 2 or 3 and to SH mode. Transition Condition Id [61]. |  | 15 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: A Mode profile defining a SH area is available on-board The maximum safe front end of the train has overpassed the beginning of the SH area (the distance defined by D_MAMODE in packet 80). The system switches to shunting mode in level 1, 2, or 3 when the train passes the level transition border. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 4 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [61] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-5.7.2.3 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Information about level (L1 / L2 / L3) and mode (SH) transition are DISPLAYED Check on the JRU that: The level and mode transitions are RECORDED |
| Constraints | The train is passing the level transition point |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1. |
| Level Transition Order | STORED | Transition to L1 / L2 / L3 |
| Mode Profile | STORED | SH (D_MAMODE) |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | Train is moving. | I | INT | The train reaches the point at the distance defined by the variable D_MAMODE (in packet 80) and the variable D_LEVELTR with its maximum safe front end. | LNTC | SN | - |
| 2 | LNTC | SN | The train running level changes | - | - | USE_FT5100100.23: “Order to switch immediately to Level 1 at level border (Level transition LNTC / L1)” OR USE_FT5100100.21: “Order to switch immediately to Level 2 at level border (Level transition LNTC / L2)” OR USE_FT5100100.20: “Order to switch immediately to Level 3 at level border (Level transition LNTC / L3)” | L1 / L2 /L3 | SN | - |
| 3 | L1 / L2 / L3 | SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | L1 / L2 /L3 | SH | - |
| 4 | L1/ L2/L3 | SH | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | Symbol MO02 | L1/ L2/ L3 | SH | - |
| 5 | L1/ L2/L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L1/ L2/L3 | SH | - |
| 6 | L1/ L2/L3 | SH | The driver acknowledges “Shunting” | I | DMI | - | L1/ L2/ L3 | SH | - |
| 7 | L1/ L2 / L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded | O | JRU | - | L1/ L2 / L3 | SH | - |
| 8 | L1/ L2/L3 | SH | The Mode symbol “Shunting” is displayed | O | DMI | - | L1/ L2/L3 | SH | - |
| 9 | L1/ L2/L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L1/ L2/L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level <L1 / L2 / L3>” is displayed. The mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The driver selects Non Leading, the train is at standstill, and the “Non Leading” input signal is received. The system switches to NL. Transition Condition Id [46]. |  | 16 |
| Applicable Mode/Level Combinations | L0 : SH / SB LNTC : SB L1 : FS / LS / OS / SR / SH / SB |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to NL mode when the driver selects the NL mode and the “Non leading” input signal is received. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 10 column 2 |  |
|  |  | Subset-026-4.6.2 row 10 column 4 |  |
|  |  | Subset-026-4.6.2 row 10 column 5 |  |
|  |  | Subset-026-4.6.2 row 10 column 6 |  |
|  |  | Subset-026-4.6.2 row 10 column 7 |  |
|  |  | Subset-026-4.6.2 row 10 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [46] |  |
|  |  | Subset-026-4.7.2 table - Input information Non Leading request |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to NL. Check on the JRU that : The new current mode NL is recorded. Check on the TIU that : The “non leading” input signal is received. |
| Constraints | Train is at standstill. A “non leading” input signal is received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 12 | FS / OS / SR / SH / SB / LS |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | Non-leading permitted | I | TIU | - | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | - |
| 2 | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | NON LEADING INPUT (NID_MESSAGE_JRU=32; M_NON_LEADING=1) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | - |
| 3 | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | The driver selects “NON LEADING” | I | DMI | The train is at standstill | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | - |
| 4 | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1100) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | - |
| 5 | L0 / LNTC / L1 | FS / LS / OS / SR / SH / SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=11) is recorded | O | JRU | Triggering event: when the mode changes | L0 / LNTC / L1 | NL | - |
| 6 | L0 / LNTC / L1 | NL | The Mode symbol “Non-leading” is displayed | O | DMI | Symbol MO12 | L0 / LNTC / L1 | NL | - |
| 7 | L0 / LNTC / L1 | NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit27=1>) is recorded | O | JRU | - | L0 / LNTC / L1 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Non-leading” is displayed | O | DMI | DMI Object: MO12 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The driver selects NON LEADIND mode, the train is at standstill and the “non leading” input signal is received. The system switches to Non Leading Mode. Transition Condition Id [46]. |  | 17 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SH / SB L3: FS / LS / OS / SR / SH / SB |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to NL mode when the driver selects NL Mode and the “non leading” input signal is received. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 10 column 2 |  |
|  |  | Subset-026-4.6.2 row 10 column 4 |  |
|  |  | Subset-026-4.6.2 row 10 column 5 |  |
|  |  | Subset-026-4.6.2 row 10 column 6 |  |
|  |  | Subset-026-4.6.2 row 10 column 7 |  |
|  |  | Subset-026-4.6.2 row 10 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [46] |  |
|  |  | Subset-026-4.7.2 table - Input information Non Leading request |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to NL. Check on the JRU that : The new current mode NL is recorded. Check on the TIU that : The “non leading” input signal is received. |
| Constraints | Train is at standstill. A “non leading” input signal is received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 6 / 12 | FS / OS / SR / SH / SB / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS / SR / SH / SB | Non-leading permitted | I | TIU | - | L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 2 | L2 / L3 | FS / LS / OS / SR / SH / SB | NON LEADING INPUT (NID_MESSAGE_JRU=32; M_NON_LEADING=1) is recorded. | O | JRU | - | L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 3 | L2 / L3 | FS / LS / OS / SR / SH / SB | The driver selects “NON LEADING” | I | DMI | The train is at standstill | L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 4 | L2 / L3 | FS / LS / OS / SR / SH / SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1100) is recorded | O | JRU | The on-board equipment switches to NL mode. | L2 / L3 | FS / LS / OS / SR / SH / SB | - |
| 5 | L2 / L3 | FS / LS / OS / SR / SH / SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=11) is recorded | O | JRU | - | L2 / L3 | NL | - |
| 6 | L2 / L3 | NL | The Mode symbol “Non-leading” is displayed | O | DMI | Symbol MO12 | L2 / L3 | NL | - |
| 7 | L2 / L3 | NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit27=1>) is recorded | O | JRU | - | L2 / L3 | NL | - |
| 8 | L2 / L3 | NL | On-board informs RBC about the mode change | - | - | USE_FT3060500.4: “Inform trackside about mode change with existing communication session”. | L2 / L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Non-leading” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The on-board is in SH mode and the driver selects “exit shunting”. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  | 18 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: If the train is at standstill in SH mode and the driver selects “exit Shunting”, the on-board equipment switches immediately to SB Mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 4 |  |
|  |  | Subset-026-4.7.2 table - Input information “Exit of Shunting” request |  |
|  |  | Subset-026-4.6.3 Condition Id [19] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode indication is DISPLAYED. Check on the JRU that: Driver selection of “exit Shunting” is RECORDED. Mode transition to SB is RECORDED. |
| Constraints | Train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC/ L1/ L2/ L3 | SH | The Train is at standstill | I | INT | - | L0 / LNTC/ L1/ L2/ L3 | SH | - |
| 2 | L0 / LNTC/ L1/ L2/ L3 | SH | The driver selects “EXIT SHUNTING” | I | DMI | - | L0 / LNTC/ L1/ L2/ L3 | SH | - |
| 3 | L0 / LNTC/ L1/ L2/ L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded | O | JRU | - | L0 / LNTC/ L1/ L2/ L3 | SH | - |
| 4 | L0 / LNTC/ L1/ L2/ L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | The on-board equipment switches to SB mode. | L0 / LNTC/ L1/ L2/ L3 | SB | - |
| 5 | L0 / LNTC/ L1/ L2/ L3 | SB | The Mode symbol “Stand By” is displayed | O | DMI | DMI Object: symbol MO13 | L0 / LNTC/ L1/ L2/ L3 | SB | - |
| 6 | L0 / LNTC/ L1/ L2/ L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded | O | JRU | - | L0 / LNTC/ L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Stand By” is displayed | O | DMI | DMI Object: symbol MO13 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  | 19 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to FS mode as soon as the ERTMS / ETCS level switches to 1, 2 or 3 and MA + SSP + Gradient are on-board and no specific mode is required by a Mode Profile. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.4.9.1.4 |  |
|  |  | Subset-026-4.6.2 row 5 column 11 |  |
|  |  | Subset-026-4.6.2 row 5 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [25] |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that : The mode initially displayed on the DMI changes to FS when the train switches to ETCS level 1, 2 or 3. Check on the JRU that : The new current mode FS is RECORDED. |
| Constraints | The track description is available for the Front End of the train when the train passes the level border. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Movement Authority | STORED | full supervision MA from the level border track description does not cover the whole length of the train |
| Gradient Profile | STORED | track description is available starting from the level border |
| International SSP | STORED | track description is available starting from the level border |
| Level Transition Order | STORED | To level 1, 2 or 3 |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 is received | I | BTM | At the level border the order to switch to ETCS level 1,2 or 3 | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded | O | JRU | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode | L1 / L2 / L3 | FS | - |
| 4 | L1 / L2 / L3 | FS | The system status message “Entering FS” is displayed. | O | DMI | Displayed to the driver until SSP and gradients are known for the whole length of the train. | L1 / L2 / L3 | FS | - |
| 5 | L1 / L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS | - |
| 6 | L1 / L2 / L3 | FS | The Level symbol “Level <L1/ L2/ L3>” is displayed | O | DMI | - | L1 / L2 / L3 | FS | - |
| 7 | L1 / L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03/ 04/ 05=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS | - |
| 8 | L1 / L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L1 / L2 / L3 | FS | - |
| 9 | L1 / L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS | - |
| 10 | L1 / L2 / L3 | FS | The system status message “Entering FS” is removed. | O | DMI | - | L1 / L2 / L3 | FS | - |
| 11 | L1 / L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=0>) is recorded | O | JRU | - | L1 / L2 / L3 | FS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | - |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | - |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | 1 | - |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | D_LEVELTR = special value = now |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | M_LEVELTR = 2 , 3 or 4 |
| NID_NTC(k) | 8 | FINITE VALUE | - |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The new level symbol is displayed. The Mode symbol “Full Supervision” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is at standstill in level 2 or 3 in PT mode and a MA, SSP and Gradient Profile has been received onboard and no specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train. The system switches to FS. Transition Condition Id [31]. |  | 20 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: On-board switches to FS mode when the FS MA, and track description is received onboard, no specific mode is required by a Mode Profile and ERTMS/ETCS level is 2 or 3. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.6.2 row 5 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | The track description is available at least from the Front End of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | To level 1, 2 or 3 |
| Train Data | STORED | Previously entered |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Post trip” is displayed | O | DMI | DMI Object: MO06 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | Procedure Train Trip | - | - | USE_FT5110200.5 (Flowchart for PT mode in level 2/3) | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE= 0) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 3 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 2 or 3 in SR and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train The system switches to FS immediately. Transition Condition Id [31]. |  | 21 |
| Applicable Mode/Level Combinations | L2: SR L3: SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board switches to FS mode when the FS MA, and track description is received onboard, no specific mode is required by a Mode Profile and the ERTMS/ETCS level is 2 or 3. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.4.9.1.4 |  |
|  |  | Subset-026-4.6.2 row 5 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Mode Profile | NOT STORED | - |
| Train Data | VALID | A complete set of valid Train data is available on-board |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed | O | DMI | DMI Object: Symbol MO09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SR | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received | I | RTM | The track description does not cover the full length of the train. | L2 / L3 | SR | - |
| 2 | L2 / L3 | SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 3 | L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The on-board equipment switches immediately to FS mode even if the track description is not available for the whole train in this moment. | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | The system status message “Entering FS” is displayed | O | DMI | An indication “Entry in Full Supervision” mode is displayed to the driver until SSP and gradients are known for the whole length of the train. | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Packet 21: Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | Packet 27: Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision” is displayed The system status message “Entering FS” is displayed | O | DMI | DMI Object: Symbol MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The ETCS on-board equipment switches to RV Mode after the driver has acknowledged reversing. The train is at standstill. Transition Condition Id [59]. |  | 22 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The request to acknowledge Reversing is displayed to the driver when the following condition is fulfilled: the train is at standstill with the front end of the train inside the indicated area. The on-board equipment switches to RV mode after the driver acknowledges reversing. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.4.4 |  |
|  |  | Subset-026-3.15.4.7 |  |
|  |  | Subset-026-4.6.2 row 17 column 5 |  |
|  |  | Subset-026-4.6.2 row 17 column 6 |  |
|  |  | Subset-026-4.6.2 row 17 column 8 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Reversing mode |  |
|  |  | Subset-026-4.6.3 Condition Id [59] |  |
|  |  | Subset-026-5.13.1.4 |  |
|  |  | Subset-026-5.13.1.5 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that : The driver changes the direction controller position. Check on the DMI that : A confirmation for RV, symbol MO15 is DISPLAYED to the driver and the driver acknowledges. RV mode indication, symbol MO14 is DISPLAYED. Check on the JRU that: Change of direction controller position is RECORDED. Reversing acknowledgement, symbol MO15 is RECORDED. Transition to RV mode (symbol MO14) is RECORDED. |
| Constraints | Reversing information is available onboard, and the train is at standstill with the front end of the train inside the reversing area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol “Reversing permitted” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2/ L3 | FS/ LS / OS | Driver selects Reversing. | I | TIU | (e.g. from a direction controller in reverse position: the current position of the direction controller is backwards) | L1 / L2/ L3 | FS/ LS / OS | - |
| 2 | L1 / L2/ L3 | FS/ LS / OS | DIRECTION CONTROLLER POSITION (NID_MESSAGE_JRU=39; M_DIRECTION_CONTROLLER=01) is recorded | O | JRU | - | L1 / L2/ L3 | FS/ LS / OS | - |
| 3 | L1 / L2/ L3 | FS/ LS / OS | The Mode symbol “Acknowledge Reversing” is displayed. | O | DMI | DMI Object: Symbol MO15 | L1 / L2/ L3 | FS/ LS / OS | - |
| 4 | L1 / L2/ L3 | FS/ LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit30=1>) is recorded | O | JRU | - | L1 / L2/ L3 | FS/ LS / OS | - |
| 5 | L1 / L2/ L3 | FS/ LS / OS | The driver acknowledges “Reversing” | I | DMI | - | L1 / L2/ L3 | FS/ LS / OS | - |
| 6 | L1 / L2/ L3 | FS/ LS / OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0101) is recorded | O | JRU | - | L1 / L2/ L3 | FS/ LS / OS | - |
| 7 | L1 / L2/ L3 | FS/ LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=14) is recorded | O | JRU | The on-board equipment switches to RV mode. | L1 / L2/ L3 | RV | - |
| 8 | L1 / L2/ L3 | RV | The Mode symbol “Reversing” is displayed | O | DMI | DMI Object: Symbol MO14 | L1 / L2/ L3 | RV | - |
| 9 | L1 / L2/ L3 | RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit29=1>) is recorded | O | JRU | - | L1 / L2/ L3 | RV | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 14 | RV |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| The Direction Controller input has changed | I | TIU | - |
| The Mode symbol “Reversing” is displayed | O | DMI | DMI Object: Symbol MO14 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 2 or 3 in OS mode and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train .The system switches to FS immediately. Transition Condition Id [31]. |  | 23 |
| Applicable Mode/Level Combinations | L2: OS L3: OS |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to FS mode when the FS MA, and track description is received onboard, no specific mode is required by a Mode Profile and the ERTMS/ETCS level is 2 or 3. On the reception of a new MA without Mode Profile the on-board equipment shall delete the current Mode Profile. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.4.3 |  |
|  |  | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.4.9.1.4 |  |
|  |  | Subset-026-4.6.2 row 5 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | The train is in OS because a MP for OS was previously Received. On the reception of a new MA without Mode Profile the on-board equipment shall delete the current Mode Profile. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “On Sight” is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | OS | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 15, packet 21 and packet 27 is received | I | RTM | The track description does not cover the full length of the train. Packet 80 not included. | L2 / L3 | OS | - |
| 2 | L2 / L3 | OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | OS | - |
| 3 | L2 / L3 | OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The on-board equipment switches immediately to FS mode even if the track description is not available for the whole train in this moment. | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | The system status message “Entering FS” is displayed. | O | DMI | Displayed to the driver until SSP and gradients are known for the whole length of the train. | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in FS / LS / OS and the train/engine overpasses the EOA/LOA of its MA. The system switches to TR Mode. Transition Condition Id [16]. |  | 24 |
| Applicable Mode/Level Combinations | L2 : FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that: On-board switches to TR mode when the train/engine overpasses the EOA/LOA with its minimum safe front end in OS/LS/FS mode and the ERTMS/ETCS level is 2 or 3, and In TR mode the on-board commands emergency brakes and indicates to the driver the reason of the train trip. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 c) |  |
|  |  | Subset-026-3.13.10.2.6 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [16] |  |
|  |  | Subset-026-4.4.13.1.2 |  |
|  |  | Subset-026-4.4.13.1.3 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed on the DMI changes to TR. The driver is informed about the Trip reason The Emergency brakes symbol is displayed Check on the TIU that: The Emergency Brakes are applied Check on the JRU that : The new current mode TR and trip reason are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision/ Limited Supervision/ On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS | The train overpasses EoA / LoA with its minimum safe front end. | I | INT | - | L2 / L3 | FS / LS / OS | - |
| 2 | L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 3 | L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17 “Indication of Trip mode and reason to the driver” The trip reason is :“ Unauthorized passing of EOA / LOA “. | L2 / L3 | TR | - |
| 4 | L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit12=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 5 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 6 | L2 / L3 | TR | Emergency brake commanded | O | TIU | (Only if EB not commanded before) | L2 / L3 | TR | - |
| 7 | L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | (Only if EB not commanded before) | L2 / L3 | TR | - |
| 8 | L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object Symbol: ST01 (Trip due to passing EOA/LOA) (Only if EB not commanded before) | L2 / L3 | TR | - |
| 9 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | (Only if EB not commanded before) | L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in FS / LS / OS mode and the train/engine performs transition to TR due to a linking reaction with the expected balise group. Transition Condition Id [17]. |  | 25 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS L2 : FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to TR mode due to linking reaction set to “trip” in FS / LS / OS mode. In trip mode, the on-board commands the emergency brakes, and indicates to the driver the reason of the trip. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.2.6.1 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [17] |  |
|  |  | Subset-026-4.4.13.1.3 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The intervention due to linking reaction problem with the expected balise group is displayed on the DMI. The mode initially displayed on the DMI changes to TR. An indication of the trip reason is displayed to the driver Check on the TIU that: The Emergency Brakes are applied Check on the JRU that : The new current mode TR, the trip reason and the application of the Service brake are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Q_LINKREACTION=00 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision/ Limited Supervision/ On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS | NOT TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Linking consistency problem: The location reference of a BG marked as linked and included in the linking information is not found in the expectation window. | L1 / L2 / L3 | FS / LS / OS | - |
| 2 | L1 / L2 / L3 | FS / LS / OS | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | FS / LS / OS | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17 “Indication of Trip mode and TRIP reason to the driver.” Trip Reason: System Message “Balise read error”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit01=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Service Brake” is displayed | O | DMI | DMI Objects: MO04; ST01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train receives and uses trip information given by balise and override is not active. Transition Condition Id [18]. |  | 26 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: When the train is in one of the applicable mode/level combinations and receives a trip information given by a balise (Train trip is not inhibited) and Override is not active The train is tripped. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [18] |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode displayed is TR The Driver is informed about Trip reason Check on the JRU that: The transition to TR and the application of the Emergency brake are recorded Check in the TIU that: Emergency Brake is applied |
| Constraints | Train trip is not inhibited. Override is not active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Linking | STORED | If BGa is marked as linked, it must be included in the linking information, so that the train accepts its message. If BGa is marked as unlinked, there is no need for linking information. |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision/ Limited Supervision/ On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS / SR | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 12 (V_MAIN=0) is received | I | BTM | It contains the packet 12 “Level 1 Movement Authority” with V_MAIN=0, which means train trip | L1 | FS / LS / OS / SR | - |
| 2 | L1 | FS / LS / OS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / LS / OS / SR | - |
| 3 | L1 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 4 | L1 | FS / LS / OS / SR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17 Indication of Trip mode and TRIP reason to the driver. Trip Reason: System Message “Unauthorized passing of EOA / LOA” | L1 | TR | - |
| 5 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 | TR | - |
| 6 | L1 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit12=1>) is recorded | O | JRU | - | L1 | TR | - |
| 7 | L1 | TR | Emergency brake commanded | O | TIU | - | L1 | TR | - |
| 8 | L1 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 | TR | - |
| 9 | L1 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Symbol: ST01 | L1 | TR | - |
| 10 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | means Train Trip |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in SR and the train/engine perform transition to TR due to the identity of the over-passed balise group is not in the list of expected balises related to SR mode AND override is not active. Transition Condition Id [36]. |  | 27 |
| Applicable Mode/Level Combinations | L1 : SR L2 : SR L3 : SR |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to TR mode due to the identity of the over-passed balise group is not in the list of expected balises related to SR mode and Override function is not active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [36] |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR. The Driver is informed about Trip reason Check on the TIU that: The Emergency Brake is applied. Check on the JRU that : The new current mode TR, Trip reason and application of the Emergency brake are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SR | The train overpasses by a balise group that is not in the list of expected balises in SR AND override is not active. | - | - | USE_ FT4041100.4 “Supervision of list of balises during staff responsible movement” | L1 / L2 / L3 | SR | - |
| 2 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17 “Indication of Trip mode and TRIP reason to the driver” Trip Reason: System Message “SR stop order” | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit16=1>) is recorded | O | JRU | SR stop order | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Service Brake” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train switches from Level 0, NTC to Levels 1, 2, or 3 and no MA related to the new level has been accepted. The system switches to Trip mode. Transition Condition Id [39]. |  | 28 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to TR mode when the train enters into the new Level and no MA and Track description has been accepted before. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 11 |  |
|  |  | Subset-026-4.6.2 row 12 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [39] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR The driver is informed about the trip reason. Check on the TIU that: The EB is applied. Check on the JRU that : The new current mode TR, Trip reason and application of the Emergency brake are recorded. |
| Constraints | No MA has been accepted before the execution of the level transition. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Unfitted / National System” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | The train just executed a level transition. | - | - | Use _FT5100100.15: “Order to switch immediately to Level 1 at level border (Level transition L0 / L1)” USE_FT5100100.17: “Execution of level transition L0/L1 at announced position” or USE_FT5100100.13: “Order to switch immediately to Level 2 at level border (Level transition L0 / L2)” USE_FT5100100.17: “Execution of level transition L0/L2 at announced position” or USE_FT5100100.26: “Order to switch immediately to Level 3 at level border (Level transition L0 / L3)” USE_FT5100100.17: “Execution of level transition L0/L3 at announced position” or USE_FT5100100.23: “Order to switch immediately to Level 1 at level border (Level transition LNTC / L1)” USE_FT5100100.17: “Execution of level transition LNTC/L1 at announced position” or USE_FT5100100.21: “Order to switch immediately to Level 2 at level border (Level transition LNTC / L2)” USE_FT5100100.17: “Execution of level transition LNTC/L2 at announced position” or USE_FT5100100.20: “Order to switch immediately to Level 3 at level border (Level transition LNTC / L3)” USE_FT5100100.17: “Execution of level transition LNTC/L3 at announced position” | L1 / L2 / L3 | UN / SN | - |
| 2 | L1 / L2 / L3 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | No MA and Track description are available for the new area | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “No MA received at level transition”. | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit13=1>) is recorded | O | JRU | No MA received at level transition | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. The new level symbol is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train switches from Level 0, NTC to Levels 1, 2, or 3 MA related to the new level has been received but track description is missing or incomplete. The system switches to Trip mode. Transition Condition Id [39]. |  | 29 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that : The on-board switches to TR mode when the train enters into the new Level and Track description are missing or incomplete. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 11 |  |
|  |  | Subset-026-4.6.2 row 12 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [39] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR. Trip reason is displayed. Check on the TIU that: The EB is applied. Check on the JRU that : The new current mode and application of the Emergency brake are recorded. |
| Constraints | The stored track description is incomplete. If transition to Level 2/ 3 a communication session with RBC shall be established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | If transition to L2/L3 |
| Movement Authority | STORED | - |
| Gradient Profile | UNKNOWN | INCOMPLETE |
| International SSP | UNKNOWN | INCOMPLETE |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | The train just executed a level transition. | - | - | Use _FT5100100.15: “Order to switch immediately to Level 1 at level border (Level transition L0 / L1)” USE_FT5100100.17: “Execution of level transition L0/L1 at announced position” or USE_FT5100100.13: “Order to switch immediately to Level 2 at level border (Level transition L0 / L2)” USE_FT5100100.17: “Execution of level transition L0/L2 at announced position” or USE_FT5100100.26: “Order to switch immediately to Level 3 at level border (Level transition L0 / L3)” USE_FT5100100.17: “Execution of level transition L0/L3 at announced position” or USE_FT5100100.23: “Order to switch immediately to Level 1 at level border (Level transition LNTC / L1)” USE_FT5100100.17: “Execution of level transition LNTC/L1 at announced position” or USE_FT5100100.21: “Order to switch immediately to Level 2 at level border (Level transition LNTC / L2)” USE_FT5100100.17: “Execution of level transition LNTC/L2 at announced position” or USE_FT5100100.20: “Order to switch immediately to Level 3 at level border (Level transition LNTC / L3)” USE_FT5100100.17: “Execution of level transition LNTC/L3 at announced position” | L1 / L2 / L3 | UN / SN | - |
| 2 | L1 / L2 / L3 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | No Track description is available for the new area. | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “No MA received at level transition”. | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit13=1>) is recorded | O | JRU | No MA received at level transition | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | If transition to L2/L3 |
| Movement Authority | STORED | - |
| Gradient Profile | UNKNOWN | INCOMPLETE |
| International SSP | UNKNOWN | INCOMPLETE |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | If transition to L2/L3 |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train/ engine perform transition to trip mode due to a loss of radio contact. Transition Condition Id [41]. |  | 30 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to TR mode due to a loss of radio contact (T_NVCONTACT is passed) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [41] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR The Trip reason is displayed to the driver. EB indication is Displayed. Check on the TIU that: The EB is applied Check on the JRU that : The new current mode TR and reason are recorded. EB application is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | M_NVCONTACT =00 |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / OS | The train losses radio contact.(T_NVCONTACT is passed) | - | - | USE_FT9990200.17: “Detection of loss of radio link an initiation of the installed reaction.” | L2 / L3 | FS / LS / OS | - |
| 2 | L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 3 | L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “Communication error”. | L2 / L3 | TR | - |
| 4 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 5 | L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit03=1>) is recorded | O | JRU | Communication error | L2 / L3 | TR | - |
| 6 | L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L2 / L3 | TR | - |
| 7 | L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 8 | L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L2 / L3 | TR | - |
| 9 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in SR and the distance allowed to run in SR is passed. The system switches to TR Mode. Transition Condition Id [42]. |  | 31 |
| Applicable Mode/Level Combinations | L1 : SR L2 : SR L3 : SR |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to TR mode when the train/engine overpasses the SR distance with its estimated front end and override is not active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 a) |  |
|  |  | Subset-026-4.4.11.1.3 |  |
|  |  | Subset-026-4.4.11.1.3 b) |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [42] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR. The driver is informed about the Trip reason. Check on the TIU that: The EB is applied. Check on the JRU that : the new current mode TR is recorded. the TRIP reason is recorded. EB application is recorded. |
| Constraints | Override not active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SR | The train is running in SR and the distance allowed to run in SR is passed. Override is not active. | - | - | USE_ FT4041100.1: “ Supervision of passage of end of staff responsible authorisation.” | L1 / L2 / L3 | SR | - |
| 2 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “SR distance exceeded”. | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit14=1>) is recorded | O | JRU | SR distance exceeded | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | Still applied | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 Still displayed | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  | 32 |
| Applicable Mode/Level Combinations | L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: When the train is in shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 AND override is not active , the train is tripped. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 4 |  |
|  |  | Subset-026-4.6.3 Condition Id [49] |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR. The driver is informed about the trip reason Check in the TIU that: Emergency Brake is applied Check on the JRU that: The transition to TR is recorded TRIP reason is recorded. EB application is recorded. |
| Constraints | Train trip is not inhibited. Override is not active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting “ is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SH | The train is in shunting mode in L1, L2 or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 (which means that the train will be tripped if it’s in shunting mode). | - | - | USE_FT4080408.1 | L1 / L2 / L3 | SH | - |
| 2 | L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “SH stop order”. | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded | O | JRU | SH stop order | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in shunting mode in L0 or LNTC and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 (which means that the train will be tripped if it’s in shunting mode).This information is ignored in L0 and LNTC. Transition Condition Id [49]. |  | 33 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH |  |  |
| Target of Test | The target of this test is to check that: When the train is in SH mode in L0/ LNTC and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0, the train is NOT tripped. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 4 |  |
|  |  | Subset-026-4.6.3 Condition Id [49] |  |
|  |  | Subset-026-4.8.3.1.1 Danger for SH information |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The train does not transit to Trip |
| Constraints | Train trip is not inhibited |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | SH | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 132 is received | I | BTM | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. | L0 / LNTC | SH | - |
| 2 | L0 / LNTC | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Train is not tripped | L0 / LNTC | SH | - |
| 3 | L0 / LNTC | SH | NOT The Mode symbol “Mode <TR>” is displayed | O | DMI | - | L0 / LNTC | SH | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Bga not included in packet 49 stored |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 132 | Danger for shunting information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | 0 | Stop if in SH mode (train trip) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Before entering the shunting area, the train receives a list of balises for shunting area. When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied. Transition Condition Id [52]. |  | 34 |
| Applicable Mode/Level Combinations | L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to TR mode after passing a balise not included in the list of expected balises for shunting areas and override is not active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 4 |  |
|  |  | Subset-026-4.6.3 Condition Id [52] |  |
|  |  | Subset-026-4.4.8.1.1 |  |
|  |  | Subset-026-4.4.8.1.1 b) |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The indication of TR mode is DISPLAYED. The indication of Trip reason is DISPLAYED The indication of emergency brakes application is DISPLAYED. Check on the TIU that: EB are applied. Check on the JRU that: Balise Group (BGa) message is RECORDED. Transition to TR mode is RECORDED. Trip reason is RECORDED. Application of emergency brakes is RECORDED. |
| Constraints | The train trip is not inhibited. (override is not active) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| List of Balises for SH area | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting “ is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SH | One Balise Group message (N_TOTAL=1; NID_C=<comment>) is received | I | BTM | NID_BG variable in BGa message is different from NID_BG values in the list of balises for SH stored onboard OR the list of balises for SH is empty. | L1 / L2 / L3 | SH | - |
| 2 | L1 / L2 / L3 | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 3 | L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | SH | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “SH stop order”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded | O | JRU | SH stop order | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 000 | 1 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Bga not included in packet 49 stored |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | FINITE VALUE | Others packet can be included |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | MO04 ST01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is moving in SR mode. It passes over a balise group sending a message which contains packet 137 “Stop if in Staff Responsible” AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. Therefore, the train is tripped. Transition Condition Id [54]. |  | 35 |
| Applicable Mode/Level Combinations | L1: SR L2: SR L3: SR |  |  |
| Target of Test | The target of this test is to check that: The mode changes to TR when it passes a balise group sending a telegram with packet 137 “Stop if in Staff Responsible” with the variable Q_SRSTOP set to 0 AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. The emergency brakes are applied. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.11.1.3 |  |
|  |  | Subset-026-4.4.11.1.3 d) |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [54] |  |
|  |  | Subset-026-4.8.3.1.1 Stop if in SR mode |  |
|  |  | Subset-026-4.8.4.2 Stop if in SR mode |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-7.4.2.33 Packet Number 137: Stop if in SR |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Packet 137: “Stop if in Staff Responsible” is RECORDED. The variable Q_SRSTOP is set to 0. The transition to TRIP mode is RECORDED Trip reason is RECORDED. EB application is RECORDED. Check on the TIU that: EB are APPLIED. Check on the DMI that: The change to TR mode is DISPLAYED. Trip reason is DISPLAYED The EB application is DISPLAYED. |
| Constraints | The variable Q_DIR in packet 137 ensures that the train direction coincides with the packet validity direction. The train trip is not inhibited (Procedure Override EOA) Override is not active |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SR | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 137 (Q_SRSTOP=0) is received | I | BTM | BGa telegram contains Packet 137: “Stop if in Staff Responsible” with variable Q_SRSTOP = 0. | L1 / L2 / L3 | SR | - |
| 2 | L1 / L2 / L3 | SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |
| 3 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver.” Trip Reason: System Message “SR stop order”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit16=1>) is recorded | O | JRU | SR stop order | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | First balise in the group |
| N_TOTAL | 3 | FINITE VALUE | N balises (1<= N <= 8) |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 137 | Stop if in Staff Responsible |
| Q_DIR | 2 | FINITE VALUE | = train direction |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 1, 2 or 3 and passes over a balise group (BGa) sending a message with an incompatible ETCS version. The train is tripped and an indication is given to the driver. Transition Condition Id [65]. |  | 36 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / SH L2: FS / LS / OS / SR / SH L3: FS / LS / OS / SR / SH |  |  |
| Target of Test | The target of this test is to check that: In levels 1, 2 or 3, if this system version number X is greater than the highest version number X supported by the on-board equipment, the information from this balise is ignored, The on-board commands the emergency brakes and indicates to the driver the reason of the train trip. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.17.3.5 |  |
|  |  | Subset-026-3.17.3.5 d) |  |
|  |  | Subset-026-4.6.2 row 12 column 4 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [65] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: -The balise group BGa message is RECORDED. -The transition to TRIP mode is RECORDED. -The Trip reason is RECORDED. -EB application is RECORDED. Check on the DMI that: -The change to TR mode is DISPLAYED. -The Trip reason is DISPLAYED. -The EB application is DISPLAYED. Check on the TIU that: -EB command is applied. |
| Constraints | The train trip is not inhibited. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 12 | FS / OS / SR / SH / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS / SR / SH | One Balise Group message (N_TOTAL=1; NID_C=<comment>) is received | I | BTM | The On-board equipment notices that M_VERSION in BGa message is greater than the highest M_VERSION supported by the on-board equipment, so the information from this balise shall be ignored. (M _VERSION is different to M_VERSION 000xxxxx ) | L1 / L2 / L3 | FS / LS / OS / SR / SH | - |
| 2 | L1 / L2 / L3 | FS / LS / OS / SR / SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS / SR / SH | - |
| 3 | L1 / L2 / L3 | FS / LS / OS / SR / SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | FS / LS / OS / SR / SH | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “Trackside not compatible”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit09=1>) is recorded | O | JRU | Trackside not compatible | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_SRSTOP | 1 | 0 | Stop if in SR |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Incompatible version (M _VERSION is different to M_VERSION 000xxxxx ) |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | First balise in the group |
| N_TOTAL | 3 | FINITE VALUE | N balises (1<= N <= 8) |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train performs a transition to TR mode when a balise group (BGa) is passed in unexpected direction. The train is tripped. Transition condition Id [66]. |  | 37 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS L2 : FS / LS / OS L3 : FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The on-board switches to TR mode when a balise group contained in the linking information is passed in the unexpected direction The emergency brakes command is applied and the train is tripped. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.2.3.2 |  |
|  |  | Subset-026-3.16.2.6.1 |  |
|  |  | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [66] |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change to TR mode is DISPLAYED. The trip reason is DISPLAYED. The EB application is DISPLAYED. NO level transition is DISPLAYED. Check on the TIU that: The Emergency Brake is applied. Check on the JRU that: Balise Group (BGa) message is recorded. The transition to TR mode is RECORDED The trip reason is RECORDED. EB intervention is RECORDED The train reaches standstill (V_TRAIN = 0 is RECORDED) |
| Constraints | Train trip is not inhibited |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Linking | STORED | Q_LINKREACTION=00 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 is received | I | BTM | The train passes the balise group (BGa) in unexpected direction a message with Packet 41 (Level Transition Order ). For single balise group, see Q_DIR in first packet of message (it will be different from Q_LINKORIENTATION stored on-board) For balise group composed of several balises the following can occur: N_PIG=1 for first balise N_PIG=0 for second balise | L1 / L2 / L3 | FS / LS / OS | - |
| 2 | L1 / L2 / L3 | FS / LS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= <changed>) is recorded | O | JRU | Packet 41 is rejected. Level transition not effective. | L1 / L2 / L3 | FS / LS / OS | - |
| 4 | L1 / L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | FS / LS / OS | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver.” Trip Reason: System Message “Balise read error”. | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit01=1>) is recorded | O | JRU | Balise read error | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 11 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | - |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | <> 0 and <>32767 |
| M_LEVELTR | 3 | FINITE VALUE | - |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | - |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train performs a transition to TR mode after switching to level 1 and receiving a trip order and when override is not active. Transition Condition Id [67]. |  | 38 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: When the train is in UN or SN, override is not active, and overpasses a balise which orders the train to switch to level 1, with V_MAIN=0, the train is tripped. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 11 |  |
|  |  | Subset-026-4.6.2 row 12 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [67] |  |
|  |  | Subset-026-5.11.2.2 A025 |  |
|  |  | Subset-026-5.11.2.2 S010 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The initial displayed level changes to level 1 The mode initially displayed changes to TR mode. The trip reason is DISPLAYED. Check on the JRU that: The transition to TR is RECORDED. The trip reason is RECORDED. The EB application is RECORDED. Check in the TIU that: Emergency Brake is applied |
| Constraints | Train Trip is not inhibited. Override is not active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Linking | STORED | If BGa is marked as linked, it must be included in the linking information, so that the train accepts its message. If BGa is marked as unlinked, there is no need for linking information. |
| ERTMS/ETCS level | 0/ 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 12 and packet 41 is received | I | BTM | Packet number 12 : Level 1 Movement Authority ) with the variable V_MAIN set to zero The order to switch to level 1 (packet 41). | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2) is recorded | O | JRU | Level transition effective | L1 | UN / SN | - |
| 4 | L0 / LNTC | UN / SN | The Level symbol “Level 1” is displayed | O | DMI | DMI Object: Symbol LE03 | L1 | UN / SN | - |
| 5 | L1 | UN / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | UN / SN | - |
| 6 | L1 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 7 | L1 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “No MA received at level transition”. | L1 | TR | - |
| 8 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 | TR | - |
| 9 | L1 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit13=1>) is recorded | O | JRU | No MA received at level transition | L1 | TR | - |
| 10 | L1 | TR | Emergency brake commanded | O | TIU | - | L1 | TR | - |
| 11 | L1 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 | TR | - |
| 12 | L1 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 | TR | - |
| 13 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | V_MAIN=0 means train trip |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to ”no time out” |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 |
| NID_PACKET | 8 | 41 | Level Transition Order (This second packet could be in another balise inside the group. For this example, we’ve used the same balise with both packets). |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now (The level transition is performed upon receipt of the order) |
| M_LEVELTR | 3 | 2 | Switch to level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_SNTC(k) | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Acknowledgement for shunting is requested, driver acknowledges, the system switches to SH mode and an end of mission takes place (only in levels 2 and 3). Transition Condition Id [50]. |  | 39 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the acknowledgement of the driver takes place. There is an end of mission (in levels 2 and 3). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [50] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.7.3.5 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication SH mode is DISPLAYED Check on the JRU that: The transition to SH mode is RECORDED Termination of communication session is RECORDED (in levels 2 and 3) |
| Constraints | The speed is lower than the Shunting mode permitted speed For levels 2 and 3, the entry of the ERTMS/ETCS on-board equipment in the Shunting mode shall be considered as an end of mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Mode Profile | STORED | SH at a further location |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS | Train is moving. | I | INT | Train Location: The train reaches the acknowledgement window defined by the variable L_ACKMODE (in packet 80) with its maximum safe front end. V_TRAIN: The speed is lower than the maximum shunting speed: D_MAMODE>D>D_MAMODE–L_ACKMAMODE V<V_NVSHUNT | L1 / L2 / L3 | FS / LS / OS | - |
| 2 | L1 / L2 / L3 | FS / LS / OS | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | DMI Object: Symbol MO02 | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 4 | L1 / L2 / L3 | FS / LS / OS | The driver acknowledges “Shunting” | I | DMI | - | L1 / L2 / L3 | FS / LS / OS | - |
| 5 | L1 / L2 / L3 | FS / LS / OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS | - |
| 6 | L1 / L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | Triggering event: when the mode changes | L1 / L2 / L3 | SH | - |
| 7 | L1 / L2 / L3 | SH | The Mode symbol “Mode <SH>” is displayed | O | DMI | DMI Object: MO01 or USE_FT4070201.20 | L1 / L2 / L3 | SH | - |
| 8 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 9 | L2 / L3 | SH | End of Mission procedure takes places | - | - | USE_FT5050300.3 ( only in levels 2 and 3) | L2 / L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 40

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | There is a former EOA/LOA stored on-board. The driver selects Override EOA. The procedure of Override ends because T_NVOVTRP expires. The train overpasses the former EOA/LOA (when Override was activated) with its min safe antenna position and the train switches to TR Mode. Transition Condition Id [43]. |  | 40 |
| Applicable Mode/Level Combinations | L1 : SR L2 : SR L3 : SR |  |  |
| Target of Test | The target of this test case is to check that : The min safe antenna position overpasses the former EOA/LOA.(when Override was activated) when the train trip suppression is not active The on-board switches to TR mode due to the overpassed EOA/LOA |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [43] |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.3.1.1 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Override status is not activated The mode initially displayed changes to TR. Trip reason is displayed. Check on the TIU that: The EB is applied. Check on the JRU that : the new current mode TR is recorded. |
| Constraints | The current mode is SR and there is a former EOA/LOA stored on-board because the driver previously selected Override when the EVC was in FS, LS, OS, SB or PT mode. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed NOT The Mode symbol “Override EOA is active” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | SR | The Mode symbol “Mode <SR>” is displayed | O | DMI | The on-board is in SR mode, the override status is not active and the former EOA/LOA is still stored on-board. | L1 / L2 / L3 | SR | - |
| 2 | L1 / L2 / L3 | SR | Train is moving | I | INT | The train moves and the minimum safe antenna position pass the former EOA/LOA location. | L1 / L2 / L3 | TR | - |
| 3 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | The train is tripped | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | TR | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and reason to the driver” The trip reason is :“ Unauthorized passing of EOA / LOA “. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit12=1>) is recorded | O | JRU | Unauthorized passing of EOA / LOA | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 41

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC when “override” function is active and store it. The On-Board equipment switches immediately to the Trip mode when the train switched to level 2/3 at the announced transition point. The train responds to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’, indicating that the train takes into account the Unconditional Emergency Stop. The data displayed on the DMI are updated, accordingly. Transition Condition Id [20]. |  | 41 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | To check that: the train takes into account an Unconditional Emergency Stop, received when “override” function was active, when the train switch to level 2/3 at the announced transition point. The ERTMS/ETCS on-board equipment switches to TRIP mode and the Override procedure shall end. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.1.4 |  |
|  |  | Subset-026-3.16.3.1.2 |  |
|  |  | Subset-026-4.6.2 row 12 column 11 |  |
|  |  | Subset-026-4.6.2 row 12 column 16 |  |
|  |  | Subset-026-4.6.3 condition id [20] |  |
|  |  | Subset-026-4.8.3.1.1 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Unconditional Emergency Stop |  |
|  |  | Subset-026-4.7.2 table - Output information Trip reason |  |
|  |  | Subset-026-5.8.4.1 |  |
|  |  | Subset-026-5.8.4.1 i) |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on that DMI that: Override symbol is removed. Trip mode symbol is displayed Trip reason, “Emergency Stop” is displayed. Check on the JRU that: Received from and sent to the RBC messages are recorded. On-board switches to level 2/3. Override symbol is recorded. On-board switches to TR mode. Trip reason indicating “Emergency stop” is recorded. Check on the TIU that: EB Intervention commanded |
| Constraints | A level 2/3 transition order is stored on-board. The On-board equipment offers the possibility for driver input “override request”. The conditions for re-activation of the transition to TR mode: “max. time for train trip suppression when override EoA function is triggered” and “distance for train trip suppression when override EOA function is triggered” are not fulfilled. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| Movement Authority | STORED | For L2/L3 area |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | To Level 2/3 at a further location |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT The Mode symbol “Override EOA is active” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN<V_NVALLOWOVTRP |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | The driver selects “Override EoA” | I | DMI | - | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1110) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | The Mode symbol “Override EOA is active” is displayed | O | DMI | DMI Object: MO03 | L0 / LNTC | UN / SN | - |
| 4 | L0 / LNTC | UN / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 5 | L0 / LNTC | UN / SN | SA-DATA. Indication with Euroradio Message “Unconditional Emergency Stop” (NID_MESSAGE=16) is received | I | RTM | The message is accepted. | L0 / LNTC | UN / SN | - |
| 6 | L0 / LNTC | UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 7 | L0 / LNTC | UN / SN | A level transition order to L2/L3 is received or a level transition execution to L2/L3 is to performed | - | - | USE_FT5100100.13: Order to switch immediately to Level 2 at level border (Level transition L0/L2) or USE_F5100100.17: Execution of level transition L0/L2 at announced position or USE_FT5100100.26: Order to switch immediately to Level 3 at level border (Level transition L0/L3) or USE_FT5100100.17: Execution of level transition L0/L3 at announced position or USE_FT5100100.21: Order to switch immediately to Level 2 at level border (Level transition LNTC/L2) or USE_FT5100100.17: Execution of level transition LNTC/L2 at announced position or USE_FT5100100.20: Order to switch immediately to Level 3 at level border (Level transition LNTC/L3) or USE_FT5100100.17: Execution of level transition LNTC/L3 at announced position | L2 / L3 | UN / SN | - |
| 8 | L2 / L3 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 9 | L2 / L3 | TR | The override status is REMOVED | O | DMI | - | L2 / L3 | TR | - |
| 10 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=0>) is recorded | O | JRU | The ERTMS/ETCS on-board equipment switched to TR mode so the cancellation of override status is RECORDED. | L2 / L3 | TR | - |
| 11 | L2 / L3 | TR | The Mode symbol “Trip” is displayed | O | DMI | DMI Object: Symbol MO04 | L2 / L3 | TR | - |
| 12 | L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 13 | L2 / L3 | TR | Report mode change to RBC | - | - | USE_FT3060500.4 | L2 / L3 | TR | - |
| 14 | L2 / L3 | TR | The system status message “Emergency stop” is displayed | O | DMI | Trip Reason: System Message “Emergency stop”. | L2 / L3 | TR | - |
| 15 | L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit17=1>) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 16 | L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L2 / L3 | TR | - |
| 17 | L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L2 / L3 | TR | - |
| 18 | L2 / L3 | TR | SA-DATA. Request with Euroradio Message “Acknowledgement of Emergency Stop” (NID_MESSAGE=147) is transmitted | O | RTM | Indicating that the train has received the Unconditional Emergency Stop. Check that the messages 147 is send over the normal priority channel. | L2 / L3 | TR | - |
| 19 | L2 / L3 | TR | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2 / L3 | TR | - |

| Step 5: Radio Message 16 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | - | 16 | Unconditional Emergency Stop |
| L_MESSAGE | - | FINITE VALUE | - |
| T_TRAIN | - | FINITE VALUE | - |
| M_ACK | 1 | 0/1 | 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | - | FINITE VALUE | - |
| NID_EM | - | FINITE VALUE | Emergency Stop Identification Number |

| Step 18: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. The system message “Emergency Stop” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 42

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | An acknowledgement request for OS mode is displayed to the driver. The driver acknowledges OS mode. Mode is changed to On Sight. Transition Condition Id [15]. |  | 42 |
| Applicable Mode/Level Combinations | L2: SB / PT L3: SB / PT |  |  |
| Target of Test | The target of this test is to check that: The acknowledgement request for running on OS mode is displayed to the driver (DMI). As soon as the driver acknowledges (DMI), the mission starts in On Sight mode The acknowledgement request for OS mode is removed as soon as the driver acknowledges OS mode (DMI). The mode transition is recorded |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.12.1.2 |  |
|  |  | Subset-026-4.6.2 row 8 column 2 |  |
|  |  | Subset-026-4.6.2 row 8 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [15] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of On Sight mode |  |
|  |  | Subset-026-5.4.3.2 S25 |  |
|  |  | Subset-026-5.11.2.2 S150 |  |
|  |  | Subset-026-5.11.2.2 S170 |  |
|  |  | Subset-026-5.9.1.1 |  |
|  |  | Subset-026-5.9.3.2 |  |
|  |  | Subset-026-5.9.3.2 a) |  |
|  |  | Subset-026-5.9.3.2 b) |  |
|  |  | Subset-026-5.9.3.2 c) |  |
|  |  | Subset-026-5.9.3.6 |  |
|  |  | Subset-026-5.9.5.1 |  |
|  |  | Subset-026-5.9.5.2 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The mode initially DISPLAYED on the DMI changes to OS - The driver acknowledges the OS mode Check on the JRU that: The Driver Acknowledgement is RECORDED The transition to OS mode is RECORDED |
| Constraints | A Mode Profile allowing OS has been received from RBC To be in On Sight mode, SSP and gradient shall be at least available from the FRONT END of the train. The request for acknowledgement shall be displayed to the driver, when the distance between the estimated front end of the train and the beginning of On Sight area is shorter than a value, contained in the mode profile. The speed is lower than the OS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L 3 | SB / PT | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 80 (M_MAMODE=00) is received | I | RTM | The train receives a message including a packet 80 Mode profile with : NID_PACKET = 80: Mode Profile M_MAMODE =00: On Sight Reception from RBC of the MA allowing OS | L2 / L 3 | SB / PT | - |
| 2 | L2 / L 3 | SB / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L 3 | SB / PT | - |
| 3 | L2 / L 3 | SB / PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded | O | JRU | - | L2 / L 3 | OS | - |
| 4 | L2 / L 3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | DMI Object: Symbol MO07 | L2 / L 3 | OS | - |
| 5 | L2 / L 3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded | O | JRU | - | L2 / L 3 | OS | - |
| 6 | L2 / L 3 | OS | The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | DMI Object: Symbol MO08 | L2 / L 3 | OS | - |
| 7 | L2 / L 3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded | O | JRU | - | L2 / L 3 | OS | - |
| 8 | L2 / L 3 | OS | The driver acknowledges “On Sight” | I | DMI | - | L2 / L 3 | OS | - |
| 9 | L2 / L 3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded | O | JRU | - | L2 / L 3 | OS | - |
| 10 | L2 / L 3 | OS | The Mode symbol “Acknowledge On Sight” is removed. | O | DMI | - | L2 / L 3 | OS | - |
| 11 | L2 / L 3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=0>) is recorded | O | JRU | - | L2 / L 3 | OS | - |
| 12 | L2 / L 3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | DMI Object: Symbol MO07 | L2 / L 3 | OS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 00 | OS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “On Sight” is displayed. NOT The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 43

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 1 and overpasses the EOA/LOA with its min safe antenna position. The train is tripped. Transition Condition Id [12]. |  | 43 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that : The train switches to TR mode when the train/engine overpasses an EOA/LoA with its min safe antenna position and the Emergency Brake is applied. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [12] |  |
|  |  | Subset-026-4.7.2 table - Output information Trip reason |  |
|  |  | Subset-026-3.13.10.2.7 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to TR. The driver is informed about the trip reason The emergency brake is applied. Check on the TIU that: The Emergency Brakes is applied Check on the JRU that : the new current mode TR and the application of the Emergency brake is recorded. The indication of Trip reason is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. EoA/LoA is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS | Train is moving. | I | INT | The train overpasses EoA/LoA with its minimum antenna position | L1 | FS / LS / OS | - |
| 2 | L1 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded | O | JRU | - | L1 | TR | - |
| 3 | L1 | FS / LS / OS | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17: “Indication of Trip mode and TRIP reason to the driver”. Trip Reason: System Message “Unauthorized passing of EOA / LOA”. | L1 | TR | - |
| 4 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 | TR | - |
| 5 | L1 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit12=1>) is recorded | O | JRU | Unauthorized passing of EOA / LOA | L1 | TR | - |
| 6 | L1 | TR | Emergency brake commanded | O | TIU | - | L1 | TR | - |
| 7 | L1 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 | TR | - |
| 8 | L1 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Object: Symbol ST01 | L1 | TR | - |
| 9 | L1 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Mode symbol “Trip” is displayed. The Status symbol “Emergency Brake intervention” is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 44

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  | 44 |
| Applicable Mode/Level Combinations | L1: LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: On-board switches to FS mode when the FS MA, and track description is received onboard . |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.6.2 row 5 column 6 |  |
|  |  | Subset-026-4.6.2 row 5 column 7 |  |
|  |  | Subset-026-4.6.2 row 5 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [32] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : the new current mode FS is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 / 2 / 12 | OS / SR / LS |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Mode Profile | STORED | If in OS / LS |
| Train Data | VALID | - |
| ERTMS/ETCS level | 2 | L1 |
| Driver ID | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level 1” is displayed. The Mode symbol “Mode <LS / OS / SR>” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | OS / SR / LS | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received | I | BTM | Packet 80 not included for current location | L1 | OS / SR / LS | - |
| 2 | L1 | OS / SR / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | OS / SR / LS | - |
| 3 | L1 | OS / SR / LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The on-board equipment switches immediately to FS mode | L1 | FS | - |
| 4 | L1 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L1 | FS | - |
| 5 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L1 | FS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | means Train Trip |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 45

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A SoM or a Train Trip procedure was performed. The on-board equipment receives a mode profile defining a shunting area. The train has already entered the area. A Transition is performed. Transition Condition Id [50]. |  | 45 |
| Applicable Mode/Level Combinations | L2: SB / PT L3: SB / PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SH mode after the acknowledgement of the driver takes place. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 4 column 2 |  |
|  |  | Subset-026-4.6.2 row 4 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [50] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Shunting mode |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-5.7.3.2 |  |
|  |  | Subset-026-5.7.3.2 a) |  |
|  |  | Subset-026-5.7.3.2 b) |  |
|  |  | Subset-026-5.7.3.5 |  |
|  |  | Subset-026-5.7.4.1 |  |
|  |  | Subset-026-5.7.4.2 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Mode symbol “Acknowledge Shunting” is displayed Indication SH mode is DISPLAYED Check on the JRU that: Driver action is recorded. The transition to SH mode is RECORDED |
| Constraints | The train is at standstill inside SH area. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | - |
| Mode Profile | STORED | SH |
| ERTMS/ETCS level | 3 / 4 | L2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L 3 | SB / PT | Train is at standstill. | I | INT | Train Location: The train is inside the shunting area | L2 / L 3 | SB / PT | - |
| 2 | L2 / L 3 | SB / PT | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | D_MAMODE > D > D_MAMODE –L_ACKMAMODE V = 0 < V_NVSHUNT DMI Object: Symbol MO02 | L2 / L 3 | SB / PT | - |
| 3 | L2 / L 3 | SB / PT | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L2 / L 3 | SB / PT | - |
| 4 | L2 / L 3 | SB / PT | The driver acknowledges “Shunting” | I | DMI | The driver acknowledges the request of SH mode within the driver acknowledgment time (5 s). | L2 / L 3 | SB / PT | - |
| 5 | L2 / L 3 | SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded | O | JRU | - | L2 / L 3 | SB / PT | - |
| 6 | L2 / L 3 | SB / PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | L2 / L 3 | SH | - |
| 7 | L2 / L 3 | SH | The Mode symbol “Shunting” is displayed | O | DMI | symbol MO01 | L2 / L 3 | SH | - |
| 8 | L2 / L 3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L2 / L 3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: Symbol MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 46

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode. Transition Condition Id [40]. |  | 46 |
| Applicable Mode/Level Combinations | L1: FS / SR L2: FS / SR L3: FS / SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to OS the ERTMS/ETCS on-board equipment shall switch immediately to OS mode and a request for acknowledgement shall be displayed to the driver The Driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.12.1.2 |  |
|  |  | Subset-026-4.6.2 row 8 column 5 |  |
|  |  | Subset-026-4.6.2 row 8 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [40] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of On Sight mode |  |
|  |  | Subset-026-5.9.1.1 |  |
|  |  | Subset-026-5.9.2.1 |  |
|  |  | Subset-026-5.9.2.2 |  |
|  |  | Subset-026-5.9.2.3 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially DISPLAYED changes to OS A Request for Acknowledgement is DISPLAYED Check on the JRU that : Transition to OS mode is RECORDED Driver Acknowledgement is RECORDED. |
| Constraints | The train is running under the permitted speed in OS Mode before reaching the OS area. If the Service Brake were not available for any reason, Emergency Brake shall be used. For OS mode, the mode profile shall define the entry and the length of the On Sight area. To be in On Sight mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 | FS / SR |
| Radio communication session | ESTABLISHED | Only if in L2/L3 |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L 3 | FS / SR | One Balise Group message (N_TOTAL=1) containing packet 12 and packet 80 (M_MAMODE = 00) is received. | I | BTM | Only if in L1 Reception of the mode transition announcement (mode transition order at current location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On sight | L1 / L2 / L 3 | FS / SR | - |
| 2 | L1 / L2 / L 3 | FS / SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET= 80) is recorded | O | JRU | Only if in L1 | L1 / L2 / L 3 | FS / SR | - |
| 3 | L1 / L2 / L 3 | FS / SR | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE = 00) is received | I | RTM | Only if in L2/L3 Reception of the mode transition announcement (mode transition order at current location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On sight | L1 / L2 / L 3 | FS / SR | - |
| 4 | L1 / L2 / L 3 | FS / SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET= 80) is recorded | O | JRU | Only if in L2/L3 | L1 / L2 / L 3 | FS / SR | - |
| 5 | L1 / L2 / L 3 | FS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded | O | JRU | - | L1 / L2 / L 3 | OS | - |
| 6 | L1 / L2 / L 3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | DMI Object: MO07 Or USE_FT4070201.23 | L1 / L2 / L 3 | OS | - |
| 7 | L1 / L2 / L 3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded | O | JRU | - | L1 / L2 / L 3 | OS | - |
| 8 | L1 / L2 / L 3 | OS | The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | DMI Object: MO08 | L1 / L2 / L 3 | OS | - |
| 9 | L1 / L2 / L 3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded | O | JRU | - | L1 / L2 / L 3 | OS | - |
| 10 | L1 / L2 / L 3 | OS | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Message 136: “Train Position Report” Only in L2 / L3 | L1 / L2 / L 3 | OS | - |
| 11 | L1 / L2 / L 3 | OS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L1 / L2 / L 3 | OS | - |
| 12 | L1 / L2 / L 3 | OS | The driver acknowledges “On Sight” | I | DMI | Driver acknowledges within 5 sec after the change to OS The driver acknowledgement time (refer to Appendix A3.1) | L1 / L2 / L 3 | OS | - |
| 13 | L1 / L2 / L 3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded | O | JRU | - | L1 / L2 / L 3 | OS | - |
| 14 | L1 / L2 / L 3 | OS | Service brake not commanded | O | TIU | Negative Testing | L1 / L2 / L 3 | OS | - |
| 15 | L1 / L2 / L 3 | OS | NOT The Status symbol “Service Brake intervention” is displayed | O | DMI | Negative Testing | L1 / L2 / L 3 | OS | - |
| 16 | L1 / L2 / L 3 | OS | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | Negative Testing | L1 / L2 / L 3 | OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode change starting in the current location |
| M_MAMODE | 2 | 0 | OS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | Mode transition for the current location |
| M_MAMODE | 2 | 00 | OS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | - |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE (k) | 1 | FINITE VALUE | - |

| Step 10: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 1 | OS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | Only if in L2/L3 |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “On sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 47

| DENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in OS mode and it reaches the end of the OS area (The end of the OS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately. Transition Condition Id [31]. |  | 47 |
| Applicable Mode/Level Combinations | L2: OS L3: OS |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to FS mode when the train reaches the end of the OS area . |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.6.2 row 5 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-5.9.6.1.1 |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | The end of the On Sight area has the same location as the EOA/LOA of the MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | OS | Train is moving | I | INT | The train reaches the end of the OS area with its min. safe front end. | L2 / L3 | OS | - |
| 2 | L2 / L3 | OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 3 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 48

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A MA, SSP and Gradient Profile have been received on-board. The track description is available at least from the front end of the train. The system switches to FS. Transition Condition Id [10]. |  | 48 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to FS mode when the FS MA, and track description is received onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.9.1.1 |  |
|  |  | Subset-026-4.6.2 row 5 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [10] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | - |
| International SSP | NOT STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 21 and packet 27 is received | I | RTM | - | L2 / L3 | SB | - |
| 2 | L2 / L3 | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | SB | - |
| 3 | L2 / L3 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | Triggered event: When the mode changes | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed. | O | DMI | The driver is informed on the DMI of the change to FS mode. DMI Object: MO11 | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full Supervision” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 49

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. |  | 49 |
| Applicable Mode/Level Combinations | L1: SB / PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SR mode after the acknowledgement of the driver takes place, in level 1 and mode PT or SB. (If the position is invalid, the train position data is deleted (new status: “unknown”)) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 7 column 2 |  |
|  |  | Subset-026-4.6.2 row 7 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [8] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Staff Resp. Mode |  |
|  |  | Subset-026-5.4.3.2 S24 |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |
|  |  | Subset-026-5.11.2.2 S160 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication SR mode is DISPLAYED Check on the JRU that: Driver acknowledgement is recorded The transition to SR mode is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Train Position | INVALID | if coming from NP |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Acknowledge Staff Responsible” is displayed | O | DMI | DMI Object: Symbol MO10 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | SB / PT | The driver acknowledges “Staff Responsible” | I | DMI | Symbol MO10 | L1 | SB / PT | - |
| 2 | L1 | SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - | L1 | SB / PT | - |
| 3 | L1 | SB / PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L1 | SR | - |
| 4 | L1 | SR | The Mode symbol “Staff Responsible” is displayed | O | DMI | Symbol MO09 | L1 | SR | - |
| 5 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L1 | SR | - |
| 6 | L1 | SR | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. | O | JRU | Optional step: The status of the Train position information will change to “Unknown” only if it was “Invalid”. | L1 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | SR |
| Train Position | UNKNOWN | The EVC sets the state of train position data to unknown, if the status of train position is invalid (e.g. performing the SoM after switching on the OBU) |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed | O | DMI | Symbol MO09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 50

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Acknowledgement for SR mode is requested in L2/ L3 and driver acknowledges. Transition Condition Id [8]. |  | 50 |
| Applicable Mode/Level Combinations | L2: SB / PT L3: SB / PT |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SR mode after the acknowledgement of the driver takes place, in the level and modes above mentioned. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 7 column 2 |  |
|  |  | Subset-026-4.6.2 row 7 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [8] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Staff Resp. Mode |  |
|  |  | Subset-026-5.4.3.2 S24 |  |
|  |  | Subset-026-5.11.2.2 S160 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Indication SR mode is DISPLAYED Check on the JRU that: Driver acknowledgement is recorded The transition to SR mode is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | - |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | - |
| Train Position | UNKNOWN / VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Acknowledge Staff Responsible” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | SB / PT | The driver acknowledges “Staff Responsible” | I | DMI | Symbol MO10 | L2 / L3 | SB / PT | - |
| 2 | L2 / L3 | SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - | L2 / L3 | SB / PT | - |
| 3 | L2 / L3 | SB / PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 4 | L2 / L3 | SR | The Mode symbol “Staff Responsible” is displayed | O | DMI | Symbol MO09 | L2 / L3 | SR | - |
| 5 | L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN / VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed | O | DMI | Symbol MO09 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 51

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The ETCS on-board equipment switches to SN Mode within the Start of Mission procedure. Transition Condition Id [58]. |  | 51 |
| Applicable Mode/Level Combinations | LNTC: SB |  |  |
| Target of Test | The target of this test is to check that: After driver acknowledgement for running in SN mode, the mission starts in NTC National mode. (If the position is still invalid, the on-board shall delete the train position data („unknown“)) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 16 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [58] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of SN mode |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |
|  |  | Subset-026-5.4.3.2 S22 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : SN mode indication is DISPLAYED. Check on the JRU that: Transition to LNTC – SN is RECORDED. Driver acknowledgement is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | if coming from NP |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Stand By” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SB | The Mode symbol “Acknowledge National System” is displayed | O | DMI | DMI Object: MO20 | LNTC | SB | - |
| 2 | LNTC | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit35=1>) is recorded | O | JRU | - | LNTC | SB | - |
| 3 | LNTC | SB | The driver acknowledges “National System” | I | DMI | - | LNTC | SB | - |
| 4 | LNTC | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11: M_DRIVERACTIONS= ) is recorded | O | JRU | - | LNTC | SB | - |
| 5 | LNTC | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=13) is recorded | O | JRU | - | LNTC | SN | - |
| 6 | LNTC | SN | The Mode symbol “National System” is displayed | O | DMI | DMI Object: MO19 Or USE_FT4070201.12: Indication of NTC National mode to the driver. | LNTC | SN | - |
| 7 | LNTC | SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded | O | JRU | - | LNTC | SN | - |
| 8 | LNTC | SN | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. | O | JRU | Optional step: The status of the Train position information will change to “Unknown” only if it was “Invalid”. | LNTC | SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Train Position | UNKNOWN | The EVC sets the state of train position data to unknown, if the status of train position is invalid (e.g. performing the SoM after switching on the OBU) |
| ERTMS/ETCS level | 1 | LNTC |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “National System” is displayed | O | DMI | DMI Object: MO19 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 52

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The Override status is active and the current mode is UN (level 0) or SN (level NTC). When the system reaches the border of the level transition the level changes to level 1 and the system automatically switches to mode SR. Transition Condition Id [44]. |  | 52 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: When override function is active, on-board switches to SR mode as soon as the ETCS level switches to Level1 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 7 column 11 |  |
|  |  | Subset-026-4.6.2 row 7 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [44] |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-3.11.10.1 |  |
|  |  | Subset-026-5.8.3.1 |  |
|  |  | Subset-026-5.8.3.1 c) |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level changes to Level1 The mode SR is DISPLAYED on the DMI The driver requests the speed and distance monitoring information to be displayed Check on the JRU that: Level transition is RECORDED The transition to SR mode is RECORDED |
| Constraints | The override status is ACTIVE The transition to SR mode occurs exactly at the level transition border, i. e. as soon as the level transition occurs. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| National Values | STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. The Mode symbol “Override EOA is active” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | A level transition order to L1 is received or a level transition execution to L1 is to performed | - | - | USE_FT5100100.15: Order to switch immediately to Level 1 at level border (Level transition L0/L1) or USE_FT5100100.17: Execution of level transition L0/L1 at announced position or USE_FT5100100.23: Order to switch immediately to Level 1 at level border (Level transition LNTC/L1) or USE_FT5100100.17: Execution of level transition LNTC/L1 at announced position | L1 | UN / SN | - |
| 2 | L1 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The on-board equipment switches to SR mode at requested level transition location | L1 | SR | - |
| 3 | L1 | SR | The Mode symbol “Staff Responsible” is displayed | O | DMI | DMI Object: MO09 | L1 | SR | - |
| 4 | L1 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L1 | SR | - |
| 5 | L1 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1 | SR | - |
| 6 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 | SR | - |
| 7 | L1 | SR | The speed and distance monitoring information is displayed | O | DMI | The V_NVSUPOVTRP is displayed as permitted speed. | L1 | SR | - |
| 8 | L1 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed The new level is DISPLAYED V_NVSUPOVTRP is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 53

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The Override status is active and the current mode is UN (level 0) or SN (level NTC). If no unconditional emergency stop message has been received when the system reaches the border of the level transition the level changes to level 2 or 3 and the system automatically switches to mode SR. Transition Condition Id [45]. |  | 53 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: When override function is active and no unconditional emergency stop message has been received, on-board switches to SR mode as soon as the ETCS level switches to 2 or 3 (Transition [45]) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 7 column 11 |  |
|  |  | Subset-026-4.6.2 row 7 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [45] |  |
|  |  | Subset-026-4.7.2 table - Output information Override status |  |
|  |  | Subset-026-5.8.3.6 |  |
|  |  | Subset-026-5.8.3.1 |  |
|  |  | Subset-026-5.8.3.1 c) |  |
|  |  | Subset-026-3.11.10.1 |  |
|  |  | Subset-026-4.5.2.1 table - Override related speed restriction |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Level changes to 2 or 3 The mode SR is DISPLAYED on the DMI Check on the JRU that: No unconditional emergency stop message has been received Level transition is RECORDED The transition to SR mode is RECORDED |
| Constraints | The override status is ACTIVE and no unconditional emergency stop message has been received. The transition to SR mode occurs exactly at the level transition border, i. e. as soon as the level transition occurs. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| National Values | STORED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. The Mode symbol “Override EOA is active” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN<V_NVSUPOVTRP |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | A level transition order to L2/L3 is received or a level transition execution to L2/L3 is to performed | - | - | USE_FT5100100.13: Order to switch immediately to Level 2 at level border (Level transition L0/L2) or USE_FT5100100.17: Execution of level transition L0/L2 at announced position or USE_FT5100100.26: Order to switch immediately to Level 3 at level border (Level transition L0/L3) or USE_FT5100100.17: Execution of level transition L0/L3 at announced position or USE_FT5100100.21: Order to switch immediately to Level 2 at level border (Level transition LNTC/L2) or USE_FT5100100.17: Execution of level transition LNTC/L2 at announced position or USE_FT5100100.20: Order to switch immediately to Level 3 at level border (Level transition LNTC/L3) or USE_FT5100100.17: Execution of level transition LNTC/L3 at announced position | L2 / L3 | UN / SN | - |
| 2 | L2 / L3 | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | The on-board equipment switches to SR mode at requested level transition location | L2 / L3 | SR | - |
| 3 | L2 / L3 | SR | The Mode symbol “Staff Responsible” is displayed | O | DMI | DMI Object: MO09 | L2 / L3 | SR | - |
| 4 | L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 5 | L2 / L3 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L2 / L3 | SR | - |
| 6 | L2 / L3 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2 / L3 | SR | - |
| 7 | L2 / L3 | SR | The speed and distance monitoring information is displayed | O | DMI | The V_NVSUPOVTRP is displayed as permitted speed. | L2 / L3 | SR | - |
| 8 | L2 / L3 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed The new level is DISPLAYED The speed and distance monitoring information is displayed | O | DMI | V_NVSUPOVTRP |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 54

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | On Sight is requested for a further location when running in FS or LS mode. An acknowledgement request for OS mode is displayed to the driver. The driver acknowledges OS mode. Mode is changed to On Sight. Transition Condition Id [15]. |  | 54 |
| Applicable Mode/Level Combinations | L1: FS / LS L2: FS / LS L3: FS / LS |  |  |
| Target of Test | The target of this test is to check that: After receiving a On Sight request for a further location a request for acknowledgement is DISPLAYED to the driver and that The beginning of the on sight area shall be considered as the EOA (instead of the EOA/LOA derived from the MA) and as the SvL (with no release speed), until the driver acknowledges When the driver acknowledges, the ERTMS/ETCS on-board equipment shall immediately switch to OS mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.12.1.2 |  |
|  |  | Subset-026-4.6.2 row 8 column 5 |  |
|  |  | Subset-026-4.6.2 row 8 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [15] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of On Sight mode |  |
|  |  | Subset-026-5.9.1.1 |  |
|  |  | Subset-026-5.9.3.2 |  |
|  |  | Subset-026-5.9.3.2 a) |  |
|  |  | Subset-026-5.9.3.2 b) |  |
|  |  | Subset-026-5.9.3.2 c) |  |
|  |  | Subset-026-5.9.3.4 |  |
|  |  | Subset-026-5.9.3.5 |  |
|  |  | Subset-026-5.9.3.5 b) |  |
|  |  | Subset-026-5.9.3.6 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Request for acknowledgement is DISPLAYED to the driver The mode initially DISPLAYED on the DMI changes to OS when the driver acknowledges the OS mode Check on the JRU that: The Driver Acknowledgement is RECORDED The transition to OS mode is RECORDED |
| Constraints | To be in On Sight mode, SSP and gradient shall be at least available from the FRONT END of the train. The request for acknowledgement shall be displayed to the driver, when the distance between the estimated front end of the train and the beginning of On Sight area is shorter than a value, contained in the mode profile. The speed is lower than the OS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 12 and packet 80 (M_MAMODE = 00) is received | I | BTM | Only if in L1 Reception of the mode transition announcement (mode transition order at further location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On Sight D_MAMODE = FINITE VALUE (<>0) L_MAMODE = FINITE VALUE (<>0) L_ACKMODE = FINITE VALUE (<>0) Q_MAMODE= 1: beginning of mode profile to be considered as the SvL | L1 / L2 / L3 | FS / LS | - |
| 2 | L1 / L2 / L3 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET= 80) is recorded | O | JRU | Only if in L1 | L1 / L2 / L3 | FS / LS | - |
| 3 | L2 / L3 | FS / LS | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE = 00) is received | I | RTM | Only if in L2/L3 Reception of the mode transition announcement (mode transition order at further location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On Sight D_MAMODE = FINITE VALUE (<>0) L_MAMODE = FINITE VALUE (<>0) L_ACKMODE = FINITE VALUE (<>0) Q_MAMODE = 1: beginning of mode profile to be considered as the SvL | L2 / L3 | FS / LS | - |
| 4 | L2 / L3 | FS / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET= 80) is recorded | O | JRU | Only if in L2/L3 | L2 / L3 | FS / LS | - |
| 5 | L1 / L2 / L3 | FS / LS | Service brake / Emergency brake commanded | O | TIU | Trains starts braking to OS area border as if it was the SvL (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS / LS | - |
| 6 | L1 / L2 / L3 | FS / LS | The Status symbol “Service Brake or Emergency Brake intervention” is displayed | O | DMI | (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS / LS | - |
| 7 | L1 / L2 / L3 | FS / LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) / EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS / LS | - |
| 8 | L1 / L2 / L3 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Normal Status when braking to a target. Only if in FS mode the target distance is updated. | L1 / L2 / L3 | FS / LS | - |
| 9 | L1 / L2 / L3 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE = 2; M_SDMSUPSTAT = 0) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) | L1 / L2 / L3 | FS / LS | - |
| 10 | L1 / L2 / L3 | FS / LS | The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | Distance between estimated front end and beginning of On Sight Area < L_ACKMAMODE (stored Packet 80) AND The speed is lower than the On Sight mode permitted speed. V < V_NVONSIGHT (Both conditions are fulfilled) | L1 / L2 / L3 | FS / LS | - |
| 11 | L1 / L2 / L3 | FS / LS | NOT The Mode symbol “Acknowledge On Sight” is removed | O | DMI | The train increases the speed in such a way that V>V_NVONSIGHT | L1 / L2 / L3 | FS / LS | - |
| 12 | L1 / L2 / L3 | FS / LS | The driver acknowledges “On Sight” | I | DMI | Driver acknowledges OS mode before reaching the OS area | L1 / L2 / L3 | OS | - |
| 13 | L1 / L2 / L3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 14 | L1 / L2 / L3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | - | L1 / L2 / L3 | OS | - |
| 15 | L1 / L2 / L3 | OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 16 | L1 / L2 / L3 | OS | The Mode symbol “Mode <SH / TR / OS>” is displayed | O | DMI | USE_FT4070201.23 | L1 / L2 / L3 | OS | - |
| 17 | L1 / L2 / L3 | OS | Service brake / Emergency brake not commanded | O | TIU | Only if it was applied for EOA supervision | L1 / L2 / L3 | OS | - |
| 18 | L1 / L2 / L3 | OS | The Status symbol “Service Brake or Emergency Brake intervention” is removed | O | DMI | Only if it was applied for EOA supervision | L1 / L2 / L3 | OS | - |
| 19 | L1 / L2 / L3 | OS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) / EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Only if it was applied for EOA supervision | L1 / L2 / L3 | OS | - |
| 20 | L1 / L2 / L3 | OS | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Message 136: “Train Position Report” Only in L2 / L3. | L1 / L2 / L3 | OS | - |
| 21 | L1 / L2 / L3 | OS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | means Train Trip |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 0 | OS (if on-board is in LS on reception of this packet, a LS information must precede the OS information to avoid an immediate switch to FS) |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | beginning of mode profile to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority. Should be sent also or previously stored. |
| Q_UPDOWN | 1 | 1 | Uplink |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | FINITE VALUE | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 00 | OS (if on-board is in LS on reception of this packet, a LS information must precede the OS information to avoid an immediate switch to FS) |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | beginning of mode profile to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |

| Step 20: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 1 | OS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Mode symbol “On Sight” is displayed NOT The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 55

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A Mode profile defining an OS area is available on-board. The train passes the beginning of the On Sight area with its front end and the ERTMS/ETCS level switches to 1, 2 or 3. Then the on-board switches to OS mode. Transition Condition Id [34]. |  | 55 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: On-board switches to OS mode as soon as the ETCS level switches to 1, 2 or 3 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.4.2 |  |
|  |  | Subset-026-4.4.12.1.2 |  |
|  |  | Subset-026-4.4.12.1.7 |  |
|  |  | Subset-026-4.6.2 row 8 column 11 |  |
|  |  | Subset-026-4.6.2 row 8 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [34] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of On Sight mode |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in OS |  |
|  |  | Subset-026-5.9.1.1 |  |
|  |  | Subset-026-5.9.2.3 |  |
|  |  | Subset-026-5.9.3.7 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: mode profile |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver acknowledges the OS mode The mode initially DISPLAYED on the DMI changes to OS Check on the JRU that: The acknowledgement of OS mode is recorded. The transition to OS mode is RECORDED |
| Constraints | There is a Mode Profile stored on-board For OS mode, the mode profile shall define the entry and the length of the On Sight area. To be in On Sight mode, SSP and gradient shall be at least available from the FRONT END of the train. The train passes the beginning of the On Sight area with its front end |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to levels L1, L2 or L3 |
| Mode Profile | STORED | OS |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | Mode transition was not valuated before the level change | I | INT | “To perform the mode change the mode profile shall define the entry and the length of the On Sight area. The beginning of the Mode Profile relates to the max safe front end of the train and the end of the mode profile relates to the min safe front end of the train” | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | The train level changes | - | - | - USE_FT5100100.15 (Order to switch immediately to Level 1 at level border (Level transition L0 / L1)) or USE_FT5100100.17 (Execution of level transition L0/L1 at announced position) if level transition L0/L1 - USE_FT5100100.13 (Order to switch immediately to Level 2 at level border (Level transition L0 / L2)) or USE_FT5100100.17 (Execution of level transition L0/L2 at announced position) if level transition L0/L2 - USE_FT5100100.26 (Order to switch immediately to Level 3 at level border (Level transition L0 / L3)) or FT5100100.17 (Execution of level transition L0/L3 at announced position) if level transition L0/L3 - USE_FT5100100.23 (Order to switch immediately to Level 1 at level border (Level transition LNTC / L1)) or USE_FT5100100.17 (Execution of level transition L NTC/L1 at announced position) if level transition LNTC/L1 - USE_FT5100100.21 (Order to switch immediately to Level 2 at level border (Level transition LNTC / L2)) or USE_FT5100100.17 (Execution of level transition L NTC/L2 at announced position) if level transition LNTC/L2 - USE_FT5100100.20 (Order to switch immediately to Level 3 at level border (Level transition LNTC / L3)) or USE_FT5100100.17 (Execution of level transition L NTC/L3 at announced position) if level transition LNTC/L3 | L1 / L2 / L3 | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=2/3/4) is recorded | O | JRU | Train has just changed Level to 1, 2 or 3 and directly after that the On-board switches to OS mode. | L1 / L2 / L3 | UN / SN | - |
| 4 | L1 / L2 / L3 | OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 5 | L1 / L2 / L3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | DMI Object: MO07 | L1 / L2 / L3 | OS | - |
| 6 | L1 / L2 / L3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 7 | L1 / L2 / L3 | OS | The system status message “Entering OS” is displayed. | O | DMI | displayed to the driver until SSP and gradients are known for the whole length of the train. | L1 / L2 / L3 | OS | - |
| 8 | L1 / L2 / L3 | OS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit05=1>) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 9 | L1 / L2 / L3 | OS | The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | DMI Object: MO08 | L1 / L2 / L3 | OS | - |
| 10 | L1 / L2 / L3 | OS | The driver acknowledges “On Sight” | I | DMI | - | L1 / L2 / L3 | OS | - |
| 11 | L1 / L2 / L3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 12 | L1 / L2 / L3 | OS | The system status message “Entering OS” is removed. | O | DMI | SSp and gradients are known for the whole length of the train | L1 / L2 / L3 | OS | - |
| 13 | L1 / L2 / L3 | OS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit05=0>) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “On Sight” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 56

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Driver selects “override”, train speed is under or equal to the speed limit for triggering the “override” function and the mode is FS, LS, SB, PT and OS. Transition Condition Id [37]. |  | 56 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / PT L2: FS / LS / OS/ SB / PT L3: FS / LS / OS/ SB / PT |  |  |
| Target of Test | The target of this test is to check that: The On-board system switches to SR mode after the driver selection of “override” and when the train speed is under or equal to the speed limit for triggering the override function. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 7 column 2 |  |
|  |  | Subset-026-4.6.2 row 7 column 5 |  |
|  |  | Subset-026-4.6.2 row 7 column 6 |  |
|  |  | Subset-026-4.6.2 row 7 column 8 |  |
|  |  | Subset-026-4.6.2 row 7 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [37] |  |
|  |  | Subset-026-4.7.2 table - Input information Override request |  |
|  |  | Subset-026-5.8.2.3 |  |
|  |  | Subset-026-5.8.3.1 |  |
|  |  | Subset-026-5.8.3.1 a) |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Driver selects “override” Indication SR mode is DISPLAYED Check on the JRU that: Selection of override is RECORDED The transition to SR mode is RECORDED |
| Constraints | In SB mode and only in L2/L3 the on-board equipment allows the driver to select “Override” ( the “Override” button becomes available; see req. 5.8.2.1 b); Driver ID is valid) AND (Train data are valid) AND (Train running number is valid)) In PT mode the on-board equipment allows the driver to select “Override ( the “Override” button becomes available; see req. 5.8.2.1; (Train data are valid) AND (Train running number is valid)) If mode is different from SB and PT, the train speed is under or equal to the speed limit for triggering the “override” function." |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 6 / 8 / 12 | FS / OS / SB / PT/ LS |
| National Values | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The <Override> button is enabled | O | DMI | If in SB only in L2/L3 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS / SB / PT | The driver selects “Override EoA” | I | DMI | - | L1 / L2 / L3 | FS / LS / OS / SB / PT | - |
| 2 | L1 / L2 / L3 | FS / LS / OS / SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded | O | JRU | - | L1 / L2 / L3 | FS / LS / OS / SB / PT | - |
| 3 | L1 / L2 / L3 | FS / LS / OS / SB / PT | The Mode symbol “Staff Responsible” is displayed | O | DMI | - | L1 / L2 / L3 | SR | - |
| 4 | L1 / L2 / L3 | SR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |
| 5 | L1 / L2 / L3 | SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |
| 6 | L1 / L2 / L3 | SR | The Mode symbol “Override EOA is active” is displayed | O | DMI | - | L1 / L2 / L3 | SR | - |
| 7 | L1 / L2 / L3 | SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | - | L1 / L2 / L3 | SR | - |
| 8 | L1 / L2 / L3 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |
| 9 | L1 / L2 / L3 | SR | The speed and distance monitoring information is displayed | O | DMI | The permitted speed V_NVSUPOVTRP is indicated to the driver. | L1 / L2 / L3 | SR | - |
| 10 | L1 / L2 / L3 | SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L1 / L2 / L3 | SR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Staff Responsible” is displayed The Mode symbol “Override EOA is active” is displayed V_NVSUPOVTRP is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 57

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Mode profile requires shunting at current location in level 1. Transition Condition Id [51]. |  | 57 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The system changes to shunting mode when a mode profile definig the entry of a Shunting area is used on-board and the maximum safe front end of the train is inside the SH area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.4.2 |  |
|  |  | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.2 row 4 column 7 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [51] |  |
|  |  | Subset-026-5.7.2.3 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode profile |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Acknowledgement request for SH is DISPLAYED Check on the JRU that: The transition to SH mode is RECORDED |
| Constraints | The value of the variable D_MAMODE in packet 80 is 0 (SH is requested for the current location) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Mode Profile | STORED | SH for the current location |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | The train is at the location defined by the variable D_MAMODE (in packet 80) D_MAMODE = 0 | L1 / L2 / L3 | FS / LS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | The system switches to SH mode immediately | L1 / L2 / L3 | SH | - |
| 3 | L1 / L2 / L3 | SH | The Mode symbol “Mode <SH>” is displayed | O | DMI | DMI Object: MO01 Or USE_FT4070201.20 | L1 / L2 / L3 | SH | - |
| 4 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 5 | L1 / L2 / L3 | SH | Inform track-side about End of Mission procedure | - | - | Optional step: USE_ FT5050300.3 Only if a communication session with RBC exists | L1 / L2 / L3 | SH | - |
| 6 | L1 / L2 / L3 | SH | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | DMI Object: MO02 | L1 / L2 / L3 | SH | - |
| 7 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 8 | L1 / L2 / L3 | SH | The driver acknowledges “Shunting” | I | DMI | - | L1 / L2 / L3 | SH | - |
| 9 | L1 / L2 / L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 58

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | After having selected the start, an acknowledgement request for UN mode is displayed to the driver. The driver acknowledges UN mode. Mode is changed to Unfitted. Transition Condition Id [60]. |  | 58 |
| Applicable Mode/Level Combinations | L0 : SB |  |  |
| Target of Test | The target of this test case is to check that : The acknowledgement request for UN mode is displayed to the driver (DMI) after the driver selected start during the start of mission procedure. On-board switches to UN mode as soon as the driver acknowledges UN mode (DMI). The acknowledgement request for UN mode is removed (DMI) as soon as the driver acknowledges UN mode (DMI). (if the position is invalid, the on-board shall delete the train position data (“unknown)) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 11 column 2 |  |
|  |  | Subset-026-4.6.3 Condition Id [60] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Unfitted mode |  |
|  |  | Subset-026-5.4.3.2 S20 |  |
|  |  | Subset-026-5.4.3.2 S20 b) |  |
|  |  | Subset-026-5.4.3.2 S23 |  |
|  |  | Subset-026-5.4.3.3 - Following E10, E12, E30, E31, E32, on-board deletes stored position data |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The acknowledgement request for UN mode is displayed to the driver (DMI). The mode-indication on the DMI changes to UN mode when the driver acknowledges. The acknowledgement request for UN mode is removed (DMI). Check on the JRU that: The acknowledgement of UN mode is RECORDED. The new mode UN is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Train Position | INVALID | - |
| ERTMS/ETCS level | 0 | L0 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Stand By” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 | SB | The Mode symbol “Acknowledge Unfitted” is displayed | O | DMI | DMI Object: Symbol MO17 | L0 | SB | - |
| 2 | L0 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded | O | JRU | - | L0 | SB | - |
| 3 | L0 | SB | The driver acknowledges “Unfitted” | I | DMI | - | L0 | SB | - |
| 4 | L0 | SB | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0100 ) is recorded | O | JRU | - | L0 | SB | - |
| 5 | L0 | SB | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded | O | JRU | - | L0 | UN | - |
| 6 | L0 | UN | The Mode symbol “Unfitted” is displayed | O | DMI | DMI Object : Symbol MO16 | L0 | UN | - |
| 7 | L0 | UN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded | O | JRU | - | L0 | UN | - |
| 8 | L0 | UN | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. | O | JRU | - | L0 | UN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 | UN |
| Train Position | UNKNOWN | if the status of train location was invalid The EVC sets the state of train position data to unknown, if the status of train location is invalid (e.g. performing the SoM after switching on the OBU) |
| ERTMS/ETCS level | 0 | L0 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Unfitted” is displayed | O | DMI | DMI Object: Symbol MO16 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 59

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 2 or 3 in LS mode and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train .The system switches to FS immediately. Transition Condition Id [31]. |  | 59 |
| Applicable Mode/Level Combinations | L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that : On-board switches to FS mode when the FS MA, and track description is received onboard, no specific mode is required by a Mode Profile and the ERTMS/ETCS level is 2 or 3. On the reception of a new MA without Mode Profile the on-board equipment shall delete the current Mode Profile. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.6.2 row 5 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-4.7.2 table - Output information Entry in FS |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode initially displayed changes to FS. Check on the JRU that : The new current mode FS is recorded. |
| Constraints | The train is in LS because a MP for LS was previously Received. MA and track description Not required for the whole length of the train, at least available from the Front End of the train. On the reception of a new MA without Mode Profile the on-board equipment shall delete the current Mode Profile. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | NOT STORED | - |
| Gradient Profile | NOT STORED | Not required for the whole length of the train at least available from the Front End of the train. |
| International SSP | NOT STORED | Not required for the whole length of the train at least available from the Front End of the train. |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED. The Mode symbol “Limited Supervision” is displayed | - | DMI | DMI Object: Symbol MO21 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | SA-DATA. Indication with Euroradio Message “Movement Authority” (NID_MESSAGE=3) containing packet 15, packet 21 and packet 27 is received | I | RTM | The track description does not cover the full length of the train. On the reception of a new MA without Mode Profile the on-board equipment shall delete the current Mode Profile. | L2 / L3 | LS | - |
| 2 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 3 | L2 / L3 | LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The on-board equipment switches immediately to FS mode even if the track description is not available for the whole train in this moment. | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | The system status message “Entering FS” is displayed. | O | DMI | An indication “Entry in Full Supervision” mode is displayed to the driver until SSP and gradients are known for the whole length of the train. | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | The Mode symbol “Full Supervision” is displayed | O | DMI | DMI Object: Symbol MO11 | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out” |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Full supervision” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 60

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | In ERTMS Level 2 or 3, an acknowledgment request for Limited Supervision is proposed to the driver AND the driver acknowledges. All conditions are fulfilled (during SoM or Train Trip procedure ) and the on-board equipment switches to LS mode. Transition Condition Id [70]. |  | 60 |
| Applicable Mode/Level Combinations | L2: SB / PT L3: SB / PT |  |  |
| Target of Test | The target of this test case is to check that : Within the scope of the Start of Mission procedure in SB or Train Trip Procedure in PT mode and in Level 2 or 3 An acknowledgement request for LS mode is displayed to the driver The driver acknowledges LS mode The on-board immediately switches to LS (all conditions for mode change are fulfilled) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 6 column 2 |  |
|  |  | Subset-026-4.6.2 row 6 column 13 |  |
|  |  | Subset-026-4.6.3 Condition Id [70] |  |
|  |  | Subset-026-5.4.3.2 S25 |  |
|  |  | Subset-026-5.11.2.2 S170 |  |
|  |  | Subset-026-5.19.5.1 |  |
|  |  | Subset-026-5.19.5.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : An acknowledgment request for LS is displayed The driver acknowledges LS mode New mode LS is displayed. Check on the JRU that : Driver acknowledgment is recorded. New mode , LS is recorded. |
| Constraints | This test case takes place in L2/L3 within the scope of the Start of Mission procedure in SB mode and Train trip Procedure in PT mode. The MA allowing LS already stored. SSP and gradients are not required for the whole length of the train, but shall be at least available from the Front End of the train. The train has already entered with its max safe front end inside the LS area |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |
| Train Running Number | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | SB / PT | The Mode symbol “Acknowledge Limited Supervision” is displayed | O | DMI | DMI Object: Symbol MO22 | L2/L3 | SB / PT | - |
| 2 | L2/L2 | SB / PT | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded. | O | JRU | - | L2/L3 | SB / PT | - |
| 3 | L2/L3 | SB / PT | The driver acknowledges “Limited Supervision” | I | DMI | - | L2/L3 | SB / PT | - |
| 4 | L2/L3 | SB / PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1101) is recorded | O | JRU | - | L2/L3 | SB / PT | - |
| 5 | L2/L2 | SB / PT | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded | O | JRU | - | L2/L2 | LS | - |
| 6 | L2/L2 | LS | The Mode symbol “Limited Supervision” is displayed | O | DMI | (all conditions are fulfilled) DMI Object: Symbol MO21 | L2/L3 | LS | - |
| 7 | L2/L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit36=1>) is recorded. | O | JRU | - | L2/L3 | LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Driver ID | VALID | - |
| Train Running Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “Limited supervision” is displayed | O | DMI | DMI Object: Symbol MO21 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 61

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | A desk is opened and “Stop shunting on desk opening” information is stored on-board. Transition condition Id [22]. |  | 61 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3: PS |  |  |
| Target of Test | The target of this test case is to check that : A desk is opened The on-board equipment switches to SB mode because the “Stop shunting on desk opening” information is stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.20.1.9 |  |
|  |  | Subset-026-4.6.2 row 2 column 3 |  |
|  |  | Subset-026-4.6.3 Condition Id [22] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode indication is DISPLAYED. Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | The desk of a Passive Shunting engine (slave engine) must be closed. The train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Stop Shunting on desk opening | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC /L1/ L2/ L3 | PS | Cab active | I | TIU | The driver opens the desk of the Passive Shunting engine. | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 2 | L0/ LNTC /L1/ L2/ L3 | PS | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 3 | L0/ LNTC /L1/ L2/ L3 | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 4 | L0/ LNTC /L1/ L2/ L3 | SB | The Mode symbol “Stand By” is displayed | O | DMI | DMI Object: symbol MO13 | L0/ L1/ L2/ L3/ LNTC | SB | - |
| 5 | L0/ LNTC /L1/ L2/ L3 | SB | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded. | O | JRU | - | L0/ LNTC /L1/ L2/ L3 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| The Mode symbol “Stand By” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 62

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The desk of a Passive Shunting engine is open and NO “Stop Shunting on desk opening” information received from balise group is stored on-board. The ERTMS/ETCS on-board equipment shall switch to SH mode. Transition Condition Id [23]. |  | 62 |
| Applicable Mode/Level Combinations | L0: PS LNTC: PS L1: PS L2: PS L3: PS |  |  |
| Target of Test | The target of this test is to check that: If a desk of a Passive Shunting engine is opened and NO “Stop Shunting on desk opening” information is stored on-board The on-board equipment switches immediately to SH Mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.20.1.8 |  |
|  |  | Subset-026-4.6.2 row 4 column 3 |  |
|  |  | Subset-026-4.6.3 Condition Id [23] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SH mode indication is DISPLAYED. Check on the JRU that: Mode transition to SH is RECORDED. |
| Constraints | The desk of a Passive Shunting engine (slave engine) must be closed. The train is at standstill. In level 2/3 a communication session is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Stop Shunting on desk opening | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC /L1/ L2/ L3 | PS | Cab active | I | TIU | The driver opens the desk of the Passive Shunting engine. | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 2 | L0/ LNTC /L1/ L2/ L3 | PS | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=1) is recorded | O | JRU | - | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 3 | L0/ LNTC /L1/ L2/ L3 | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | L0/ L1/ L2/ L3/ LNTC | SH | - |
| 4 | L0/ LNTC /L1/ L2/ L3 | SH | The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: symbol MO01 | L0/ LNTC /L1/ L2/ L3 | SH | - |
| 5 | L0/ LNTC /L1/ L2/ L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded. | O | JRU | - | L0/ LNTC /L1/ L2/ L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: symbol MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 63

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | No communication session is open. The train is in PS, in L0, L1, L2, L3 or LNTC, at standstill, all desks are closed and receives the sleeping input signal. The on-board equipment switches to SL mode. Transition Condition Id [14]. |  | 63 |
| Applicable Mode/Level Combinations | L0: PS L1: PS L2: PS L3: PS LNTC:PS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment switches to SL when the train is at standstill and all desk connected to the ERTMS/ETCS on-board equipment are closed and receiving the “sleeping” input signal No communication session is open |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.20.1.2 |  |
|  |  | Subset-026-4.4.6.1.5 |  |
|  |  | Subset-026-4.6.2 row 9 column 3 |  |
|  |  | Subset-026-4.6.3 Condition Id [14] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that : The sleeping input signal is received Check on the JRU that: The transition to SL is recorded |
| Constraints | The train is at standstill and all desks connected to the ERTMS/ETCS on-board equipment are closed. No communication session is open. (only in L2 / L3) The function “Continue Shunting on desk closure” is inactive and “passive shunting input signal is received from the train interface. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Cab not active. Passive shunting permitted | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC /L1/ L2/ L3 | PS | Sleeping requested | I | TIU | - | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 2 | L0/ LNTC /L1/ L2/ L3 | PS | SLEEPING INPUT (NID_MESSAGE_JRU=30; M_SLEEPING=1) is recorded | O | JRU | - | L0/ LNTC /L1/ L2/ L3 | PS | - |
| 3 | L0/ LNTC /L1/ L2/ L3 | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=5) is recorded | O | JRU | No indication on DMI | L0/ LNTC /L1/ L2/ L3 | SL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | Not established in L2 / L3, not relevant in L0/LNTC/L1 |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Sleeping requested | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 64

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | In ERTMS Level 1, 2 or 3 in LS mode, a Mode Profile defining an On Sight area is on-board. The maximum safe front end of the train is inside the On Sight area and the estimated front end of the train is not inside an LS acknowledgement area. Transition Condition Id [73]. |  | 64 |
| Applicable Mode/Level Combinations | L1 /L2 / L3: LS |  |  |
| Target of Test | The target of this test case is to check that : The train is running inside the LS area A Mode Profile is defining an On Sight area is stored on-board and The on-board equipment switches to OS Mode when the max safe front end of the train is inside the OS area and the estimate front end of the train is not inside an LS acknowledgement area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 8 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [73] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of On Sight mode |  |
|  |  | Subset-026-5.9.3.7 |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The mode changes to OS mode. Check on the JRU that : New mode , OS is recorded. |
| Constraints | The MA allowing OS already stored, SSP and gradients are not required for the whole length of the train, but shall be at least available from the FRONT END of the train. The speed is lower than the OS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | For LS and OS mode |
| ERTMS/ETCS level | 2 / 3 / 4 | L1/ L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED. The Mode symbol “Limited Supervision” is displayed | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | The train is running inside LS area |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | LS | The train is moving. | I | INT | running inside LS area. When the maximum safe front end of the train is inside the On Sight area and the estimated front end of the train is not inside an LS acknowledgement area than the on-board equipment switches to OS mode. | L1 / L2 / L3 | LS | - |
| 2 | L1 / L2 / L3 | LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded | O | JRU | - | L1 / L2 / L3 | OS | - |
| 3 | L1 / L2 / L3 | OS | The Mode symbol “Acknowledge On Sight” is displayed | O | DMI | Symbol MO08 | L1 / L2 / L3 | OS | - |
| 4 | L1 / L2 / L3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded. | O | JRU | - | L1 / L2 / L3 | OS | - |
| 5 | L1 / L2 / L3 | OS | The driver acknowledges “On Sight” | I | DMI | - | L1 / L2 / L3 | OS | - |
| 6 | L1 / L2 / L3 | OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVER_ACTIONS=0000 0000) is recorded | O | JRU | Within the 5 sec ack. time | L1 / L2 / L3 | OS | - |
| 7 | L1 / L2 / L3 | OS | The Mode symbol “On Sight” is displayed | O | DMI | Symbol MO07 | L1 / L2 / L3 | OS | - |
| 8 | L1 / L2 / L3 | OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded. | O | JRU | - | L1 / L2 / L3 | OS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | For LS and OS mode |
| ERTMS/ETCS level | 2 / 3 / 4 | L1/ L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Mode symbol “On Sight” is displayed | O | DMI | Symbol MO07 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 65

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is at standstill in SH mode , the desks are closed and the “Continue Shunting on desk closure” function is not active. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [27]. |  | 65 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: The train is at standstill in SH mode, The desks are closed and the “Continue Shunting on desk closure” function is not active The on-board equipment switches immediately to SB Mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 4 |  |
|  |  | Subset-026-4.4.20.1.6 |  |
|  |  | Subset-026-4.6.3 Condition Id [27] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode indication is DISPLAYED. Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | Train is at standstill. The driver shall not enables the function “Continue Shunting on desk closure”.(Disabling transition to PS mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SH | The <MAINTAIN SHUNTING> button in the <MAIN> window is disabled/enabled | I | DMI | Disabling transition to PS mode | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SH | Cab not active | I | TIU | The desks are closed and the “Continue Shunting on desk closure” function is not active. | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L4 | SH | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L2 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 66

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is at standstill in SH mode , the desks are closed and the “passive shunting” input signal is not received . The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [30]. |  | 66 |
| Applicable Mode/Level Combinations | L0: SH LNTC: SH L1: SH L2: SH L3: SH |  |  |
| Target of Test | The target of this test is to check that: The train is at standstill in SH mode, The desks are closed and the “passive shunting” signal is not received The on-board equipment switches immediately to SB Mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 2 column 4 |  |
|  |  | Subset-026-4.4.20.1.6 |  |
|  |  | Subset-026-4.6.3 Condition Id [30] |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: SB mode indication is DISPLAYED. Check on the JRU that: Mode transition to SB is RECORDED. |
| Constraints | Train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2 / L3 | SH | Passive shunting not permitted | I | TIU | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 2 | L0 / LNTC / L1 / L2 / L3 | SH | PASSIVE SHUNTING INPUT (NID_MESSAGE_JRU=31; M_PASSIVE_SHUNTING=0) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 3 | L0 / LNTC / L1 / L2 / L3 | SH | Cab not active | I | TIU | The desks are closed and the “passive shunting” input signal is not received . | L0 / LNTC / L1 / L2 / L3 | SH | - |
| 4 | L0 / LNTC / L1 / L2 / L3 | SH | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L4 | SH | - |
| 5 | L0 / LNTC / L1 / L2 / L3 | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - | L0 / LNTC / L1 / L2 / L2 | SB | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 67

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | The train is in level 1, 2 or 3 at standstill. The train moves backwards until its estimated front end is in rear of the start location of SSP. SSP already stored on-board. The train is tripped and an indication is given to the driver. Transition Condition Id [69]. |  | 67 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that: In levels 1, 2 or 3, if the train performs a roll away backwards movement (using D_NVROLL) until the estimate train front end is in rear of the start location of SSP The on-board equipment switches to TRIP mode And the driver is informed about the mode change and TRIP reason. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 12 column 5 |  |
|  |  | Subset-026-4.6.2 row 12 column 6 |  |
|  |  | Subset-026-4.6.2 row 12 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [69] |  |
|  |  | Subset-026-4.7.2 table - Output information Trip reason |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The change to TR mode is DISPLAYED. The Trip reason is DISPLAYED. The EB application is DISPLAYED. Check on the JRU that: The balise group BGa message is RECORDED. The transition to TRIP mode is RECORDED. The Trip reason is RECORDED. EB application is RECORDED. Check on the TIU that: EB command is applied. |
| Constraints | The train trip is not inhibited The MA is the first received by the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | for OS and LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS | The train is at standstill . | I | INT | - | L1 / L2 / L3 | FS / LS / OS | - |
| 2 | L1 / L2 / L3 | FS / LS / OS | Train performs a rollaway backwards movement until the estimate train front end is in rear of the start location of SSP. | I | INT | The On-board equipment notices that a backwards movement within the limits of D_NVROLL is performed. ( for L2/L3: D_NVROLL has to be greater than the distance from the starting train position until the start location of SSP and for L1: D_NVROLL > D_LRBG) | L1 / L2 / L3 | FS / LS / OS | - |
| 3 | L1 / L2 / L3 | FS / LS / OS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded. | O | JRU | - | L1 / L2 / L3 | TR | - |
| 4 | L1 / L2 / L3 | FS / LS / OS | There is a permanent indication of TR mode to the driver. The reason of the train trip (Trip Reason) displayed. | - | - | USE_ FT4070201.17 Indication of Trip mode and reason to the driver The trip reason is :“ No track description”. | L1 / L2 / L3 | TR | - |
| 5 | L1 / L2 / L3 | TR | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 6 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 7 | L1 / L2 / L3 | TR | Emergency brake commanded | O | TIU | - | L1 / L2 / L3 | TR | - |
| 8 | L1 / L2 / L3 | TR | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |
| 9 | L1 / L2 / L3 | TR | The Status symbol “ Emergency Brake “ is displayed | O | DMI | DMI Symbol: ST01 | L1 / L2 / L3 | TR | - |
| 10 | L1 / L2 / L3 | TR | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | L1 / L2 / L3 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| The Status symbol “ Emergency Brake “ is displayed The Mode symbol “Trip” is displayed. | O | DMI | ST01 MO04 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 68

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.6.3: Transitions Conditions Table |  | 4060300 |
| Test Case of Feature | Mode profile requires Shunting at current location in level 1 and the driver doesn’t acknowledge within the driver acknowledgement time. Transition Condition Id [51]. |  | 68 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR L2 / L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The system changes to Shunting mode when the maximum safe front end of the train reaches the distance defined by the variable D_MAMODE =0 in packet 80, that is, at current location. As the driver does not acknowledge within the driver acknowledgement time (refer to Appendix A3.1) after the change to SH mode, the service brake command is triggered. The command is released as soon as the driver acknowledges (unless the command was triggered also for other reasons). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.4.2 |  |
|  |  | Subset-026-4.6.2 row 4 column 5 |  |
|  |  | Subset-026-4.6.2 row 4 column 6 |  |
|  |  | Subset-026-4.6.2 row 4 column 7 |  |
|  |  | Subset-026-4.6.2 row 4 column 8 |  |
|  |  | Subset-026-4.6.3 Condition Id [51] |  |
|  |  | Subset-026-5.7.2.3 |  |
|  |  | Subset-026-5.7.2.4 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: mode profile |  |
|  |  | Subset-026-A.3.1 - Level/Mode transitions: Driver acknowledgement time |  |
|  |  | Subset-026-4.5.2.1 table - Determine ERTMS/ETCS Mode |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Acknowledgement request for SH is displayed After the acknowledgment time, the driver hasn’t acknowledged the request of SH mode and the service brake command is triggered The driver acknowledge the request of SH mode and the service brake is released. Check on the JRU that: The transition to SH mode is RECORDED The application and release of the Service brake are recorded |
| Constraints | The value of the variable D_MAMODE in packet 80 is 0 (SH is requested for the current location) The Service Brake command is not triggered for any other reasons. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Mode Profile | STORED | SH to current location |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / LS / OS / SR | Train is moving. | I | INT | The train is at the location defined by the variable D_MAMODE (in packet 80) with its maximum safe front end D_MAMODE = 0 | L1 / L2 / L3 | FS / LS / OS / SR | - |
| 2 | L1 / L2 / L3 | FS / LS / OS / SR | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 3 | L1 / L2 / L3 | SH | Inform track-side about End of Mission procedure | - | - | Optional step: USE_ FT5050300.3 Only if a communication session with RBC exists | L1 / L2 / L3 | SH | - |
| 4 | L1 / L2 / L3 | SH | The Mode symbol “Acknowledge Shunting” is displayed | O | DMI | DMI Object: MO02 | L1 / L2 / L3 | SH | - |
| 5 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 6 | L1 / L2 / L3 | SH | The driver doesn’t acknowledge the request of SH mode within the driver acknowledgment time | I | DMI | A.3.1 - Level/Mode transitions: Driver acknowledgement time (5s) | L1 / L2 / L3 | SH | - |
| 7 | L1 / L2 / L3 | SH | Service brake commanded | O | TIU | - | L1 / L2 / L3 | SH | - |
| 8 | L1 / L2 / L3 | SH | The Status symbol “Service Brake or Emergency Brake intervention” is displayed | O | DMI | - | L1 / L2 / L3 | SH | - |
| 9 | L1 / L2 / L3 | SH | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 10 | L1 / L2 / L3 | SH | The driver acknowledges “Shunting” | I | DMI | - | L1 / L2 / L3 | SH | - |
| 11 | L1 / L2 / L3 | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0001) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 12 | L1 / L2 / L3 | SH | Service brake not commanded | O | TIU | - | L1 / L2 / L3 | SH | - |
| 13 | L1 / L2 / L3 | SH | The Status symbol “Service Brake or Emergency Brake intervention” is removed | O | DMI | - | L1 / L2 / L3 | SH | - |
| 14 | L1 / L2 / L3 | SH | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |
| 15 | L1 / L2 / L3 | SH | The Mode symbol “Mode <SH>” is displayed | O | DMI | DMI Object: MO01 Or USE_FT4070201.20 | L1 / L2 / L3 | SH | - |
| 16 | L1 / L2 / L3 | SH | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded | O | JRU | - | L1 / L2 / L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 2 /3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | O | TIU | - |
| The Mode symbol “Shunting” is displayed | O | DMI | DMI Object: MO01 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |