| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 9990502 (A3.5: Handling of Actions in Specific Situations) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-9990502 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 20.01.2014 | All | First version/review | MULTITEL |
| Version 2.9.1 07.02.2014 | All | Corrections following informations received from the ERA | MULTITEL |
| Version 2.9.2 10.02.2014 | All | Corrections following issues 2 to 5 of Review005_Subset-076-5-2-9990502_v291.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | A3.5: Handling of Actions in Specific Situations |  | 9990502 |
| Test Case of Feature | After the ERTMS/ETCS on-board equipment receives a balise group message, the action resulting from its content takes into account the train position measured at the time of reception of the last balise telegram of the balise group and takes precedence on any other action related to a further location that is reached before the message has been fully processed. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS |  |  |
| Target of Test | After the ERTMS/ETCS on-board equipment receives a balise group message, the action resulting from its content : - takes into account the train position measured at the time of reception of the last balise telegram of the balise group - takes precedence on any other action related to a further location that is reached before the message has been fully processed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-A.3.5.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The train receives from BG A a new MA with a LOA. BG A message also includes linking information to correct the confidence interval when the train passes over the next balises. The EVC corrects a first time its confidence interval when it passes over BG B. Then, the train receives from BG C a MA with a new LOA extending the previous LOA (given by BG A). The last balise of BG C is located at 1,3m + the distance run during 100ms at the train speed, in rear of the LOA (given by BG A). This is to ensure that the BTM has the time to transmit the data to the ERTMS/ETCS Kernel before the min safe antenna position overpasses the LOA (see Subset-040v320-4.1.1.4 and Subset-036v300-4.2.9). The train should not switch to TR mode as it should consider the MA extension as happening before the min safe antenna position overpasses the LOA. The message sent by the balises of BG C contains in each balise a large telegram with a number of bits close to the maximum of 830 user bits that should request more EVC calculation effort than a short telegram. The MA is not given by the first balise of BG C, but by the second (last) balise to ensure the EVC cannot have access to the MA after having read the first balise. Moreover, the linking used reduces the confidence interval and thus the time when the min safe antenna position overpasses the LOA should happen earlier than when the confidence interval is big. However, even with these conditions, there is no way to ensure the EVC has not finished to process BG C telegram before the min safe antenna position overpasses the LOA. Thus, to maximize the chances to prove that the EVC fulfils correctly requirement A.3.5.2, the test of the extension of the LOA done with BG C is done again with BG D to BG L. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving (V_TRAIN=180) | I | INT | 180km/h |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=713), packet 21, packet 27, packet 5 is received | I | BTM | BG A at position: D0 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=713; NID_PACKET=21; NID_PACKET=27; NID_PACKET=5) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=0; Q_LINK=1) is received | I | BTM | BG B at position: D0 + 50m Aim of BG B: correction of the confidence interval | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; N_TOTAL=0; Q_LINK=1) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG C first balise at position: D0 + 60m BG C second balise at position: D0 + 65m | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG A: D0 + 71.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 8 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 10 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG D first balise at position: D0 + 80m BG D second balise at position: D0 + 85m | N/A | N/A | - |
| 11 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG C: D0 + 60m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 13 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 14 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG D first balise at position: D0 + 100m BG D second balise at position: D0 + 105m | N/A | N/A | - |
| 16 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG D: D0 + 80m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 18 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 19 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 20 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG E first balise at position: D0 + 120m BG E second balise at position: D0 + 125m | N/A | N/A | - |
| 21 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 22 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG E: D0 + 100m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 23 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 24 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 25 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG G first balise at position: D0 + 140m BG G second balise at position: D0 + 145m | N/A | N/A | - |
| 26 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 27 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG F: D0 + 120m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 28 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 29 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 30 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG H first balise at position: D0 + 160m BG H second balise at position: D0 + 165m | N/A | N/A | - |
| 31 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 32 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG G: D0 + 140m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 33 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 34 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 35 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG I first balise at position: D0 + 180m BG I second balise at position: D0 + 185m | N/A | N/A | - |
| 36 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 37 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG H: D0 + 160m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 38 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 39 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 40 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG J first balise at position: D0 + 200m BG J second balise at position: D0 + 205m | N/A | N/A | - |
| 41 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 42 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG I: D0 + 180m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 43 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 44 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 45 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12 (Q_SCALE=0; V_EMA=36; L_ENDSECTION=313), packet 21, packet 27, packet 72 is received | I | BTM | BG K first balise at position: D0 + 220m BG K second balise at position: D0 + 225m | N/A | N/A | - |
| 46 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; Q_SCALE=0; V_EMA=36; L_ENDSECTION=313; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 47 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG J: D0 + 200m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 48 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 49 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |
| 50 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 12, packet 21, packet 27, packet 72 is received | I | BTM | BG L first balise at position: D0 + 240m BG L second balise at position: D0 + 245m | N/A | N/A | - |
| 51 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; Q_LINK=1; NID_PACKET=12; NID_PACKET=21; NID_PACKET=27; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 52 | N/A | N/A | NOT The Mode symbol "Trip" is displayed | O | DMI | Position of the LOA given in BG K: D0 + 220m +31.3m The train reaches the LOA with its min safe antenna position without being tripped | N/A | N/A | - |
| 53 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |
| 54 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; <D_TARGET is updated according to the new received MA>) is recorded | O | JRU | The process of the balise group message finishes after the train has overpassed the LOA with its min safe antenna position | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG A | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 713 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 5 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LINK | 15 | 50 | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | if Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | BG B | - |
| Q_LINKORIENTATION | 1 | FINITE VALUE | - |
| Q_LINKREACTION | 2 | 2 | - |
| Q_LOCACC | 6 | 0 | 0 is used to have the smaller confidence interval |
| N_ITER | 5 | 10 | - |
| D_LINK(k) | 15 | 10 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG C | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG D | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG E | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG F | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG G | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG H | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG I | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG J | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG K | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| D_LINK(k) | 15 | 20 | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | BG L | - |
| Q_LINKORIENTATION(k) | 1 | FINITE VALUE | - |
| Q_LINKREACTION(k) | 2 | 2 | - |
| Q_LOCACC(k) | 6 | 0 | 0 is used to have the smaller confidence interval |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG A | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG B | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG C | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG C by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG C | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG C by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG D | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG D by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 10: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG D | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG D by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 15: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG E | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG E by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 15: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG E | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG E by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 20: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG F | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG F by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 20: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG F | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG F by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 25: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG G | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG G by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 25: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG G | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG G by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 30: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG H | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG H by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 30: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG H | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG H by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 35: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG I | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG I by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 35: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG I | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG I by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 40: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG J | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG J by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 40: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG J | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG J by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 45: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG K | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG K by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 45: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG K | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 0 | - |
| V_MAIN | 7 | FINITE VALUE | Greater than V_EMA |
| V_EMA | 7 | 36 | 180 km/h |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | 0 | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_ENDSECTION | 15 | 313 | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. Greater than V_EMA |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG K by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 50: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG L | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of the first balise of BG L by the EVC!!!! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 50: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | BG L | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | Text stored to increase the process time of BG L by the EVC! | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | New received MA |
| Gradient Profile | STORED | New received Gradient Profile |
| International SSP | STORED | New received ISSP |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |