| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4070200 (4.7.2: DMI versus Mode Table - Input) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-4070200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | All: Template standardised with a dedicated tool, Answer to Review001 All but TC 8,9,20: "4.7.2 table - output" reqs removed TC5 (old TC256.4) and TC10 (old TC290.1) : Transferred in FT4070201 TC3: TC of FT, target of test and method simplified TC6: Req. 3.18.4.1.3 removed, method of test simplified, required starting condition on DMI removed TC7: Req. 4.4.7.1.3 removed TC8: TC of FT, target of test, method and constraints reworded TC8: SF/IS removed from the available modes TC9: Constraint and start condition added concerning the language TC20: Applicable Mode/Level Combinations made complete TC21: Clause "within a default list configured on-board" pointed out in the TC TCs renumbered: TC20 -> TC5 , TC21 -> TC2 , TC26 -> TC1 | MULTITEL |
| Version 2.9.2 14.10.2013 | all | Some corrections related to restricted values and misprints | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | At standstill, for operational fallback situations the ETCS on-board allows the driver to change the current level amongst those stored in the table of priority if available. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN / SB / NL LNTC: SB / NL / SN L1: FS / LS / OS / SR / SB / NL L2: FS / LS / OS / SR / SB / NL L3: FS / LS / OS / SR / SB / NL |  |  |
| Target of Test | The target of this test case is to check that the on-board equipment allows the driver to modify the level amongst those stored in the table of priority, at standstill, to cover degraded situations. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-3.18.4.2.5 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: At standstill, the ETCS on-board allows the driver to change the current level amongst those stored in the table of priority. Check on the JRU that: The new current level is RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / SR / OS / UN / SB / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 <LEVEL_X> |
| Table of priority of trackside supported levels | STORED | There is at least <LEVEL_X> contained in the table and a <LEVEL_Y> contained in the table and different from <LEVEL_X> . There is at least a <LEVEL_Z> that is not contained in the table. |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED | O | DMI | <LEVEL_X> |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LEVEL_X | N/A | The driver selects "Main" | I | DMI | Current level is <LEVEL_X> | LEVEL_X | N/A | - |
| 2 | LEVEL_X | N/A | The driver selects "Level" | I | DMI | - | LEVEL_X | N/A | - |
| 3 | LEVEL_X | N/A | The <LEVEL_Z> button is not enabled | O | DMI | <LEVEL_Z> is not contained in the table of priority of trackside supported levels | LEVEL_X | N/A | - |
| 4 | LEVEL_X | N/A | The driver selects "Level 0/1/2/3/NTC" | I | DMI | Selected level <LEVEL_Y> is different from <LEVEL_X> and is contained in the table of priority of trackside supported levels | LEVEL_Y | N/A | - |
| 5 | LEVEL_Y | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | M_DRIVERACTIONS: 34: Level 0 selected 35: Level 1 selected 36: Level 2 selected 37: Level 3 selected 38: Level NTC selected | LEVEL_Y | N/A | - |
| 6 | LEVEL_Y | N/A | The Level symbol "Level 0/1/2/3/NTC" is displayed | O | DMI | Selected level <LEVEL_Y> is different from <LEVEL_X> and is contained in the table of priority of trackside supported levels | LEVEL_Y | N/A | - |
| 7 | LEVEL_Y | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<LEVEL_Y>) is recorded | O | JRU | - | LEVEL_Y | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / SR / OS / UN / SB / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 CHANGED | L0 / LNTC / L1 / L2 / L3 <LEVEL_Y> |
| Table of priority of trackside supported levels | STORED UNCHANGED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Selected level is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | At standstill, in order to cover degraded situations, when the table of priority is not available, the driver can select any level within a default list configured on-board. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN / SB / NL LNTC: SB / NL / SN L1: FS / LS / OS / SR / SB / NL L2: FS / LS / OS / SR / SB / NL L3: FS / LS / OS / SR / SB / NL |  |  |
| Target of Test | The target of this test case is to check that the on-board equipment allows the driver to select any level within a default list configured on-board when the table of priority is not available, at standstill, to cover degraded situations |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.5 |  |
|  |  | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: At standstill, the ETCS on-board allows the driver to select any level within a default list configured on-board when the table of priority is not available. Check on the JRU that: The new current level is RECORDED. |
| Constraints | The table of priority is not available The driver can select any level within a default list configured on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / SR / OS / UN / SB / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 <LEVEL_X> |
| Table of priority of trackside supported levels | NOT STORED | - |
| Driver ID | VALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current level is DISPLAYED | O | DMI | <LEVEL_X> |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LEVEL_X | N/A | The driver selects "Main" | I | DMI | Current level is <LEVEL_X> | LEVEL_X | N/A | - |
| 2 | LEVEL_X | N/A | The driver selects "Level 0/1/2/3/NTC" | I | DMI | Selected level <LEVEL_Y> is different from <LEVEL_X> | LEVEL_Y | N/A | - |
| 3 | LEVEL_Y | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | M_DRIVERACTIONS: 34: Level 0 selected 35: Level 1 selected 36: Level 2 selected 37: Level 3 selected 38: Level NTC selected | LEVEL_Y | N/A | - |
| 4 | LEVEL_Y | N/A | The Level symbol "Level 0/1/2/3/NTC" is displayed | O | DMI | Selected level <LEVEL_Y> is different from <LEVEL_X> | LEVEL_Y | N/A | - |
| 5 | LEVEL_Y | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<LEVEL_Y>) is recorded | O | JRU | - | LEVEL_Y | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / SR / OS / UN / SB / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 CHANGED | L0 / LNTC / L1 / L2 / L3 <LEVEL_Y> |
| Table of priority of trackside supported levels | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Selected level is DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | The on-board equipment, when the train is NOT at standstill, allows the driver to modify the train running number and the Driver ID (if allowed by national value). The driver modifies the train running number. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN / NL LNTC: NL / SN L1: FS / LS / OS / SR / NL L2: FS / LS / OS / SR / NL L3: FS / LS / OS / SR / NL |  |  |
| Target of Test | The target of this test case is to check that : When the train is running, the driver is permitted to modify the train running number and the Driver ID (if allowed by national value). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.1.2 |  |
|  |  | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |
|  |  | Subset-026-4.7.2 table - Input information Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board equipment allows the driver to modify the Driver Id and/or the Train Running Number at any time. Check on the JRU that : The new data entered by the driver is recorded. |
| Constraints | In this test case, the driver is expected to attempt to modify the Train Running Number. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | Train speed >< 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=<different from zero>) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The "Train running number" button is enabled | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses “Train Running Number” | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) | N/A | N/A | - |
| 7 | N/A | N/A | The Main Window is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=<NID_OPERATIONAL_1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 UNCHANGED | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED UNCHANGED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 /L2 / L3 |
| Train Running Number | STORED CHANGED | NID_OPERATIONAL=<NID_OPERATIONAL_1> |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | Train speed >< 0 |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | The on-board equipment allows the driver to modify the current set of Train Data when the train is NOT in the process of SoM if the train is at standstill. The driver modifies the train category. |  | 4 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that the On-board Equipment: Allows the driver to select the “Train data” button Displays to the driver the valid train data Allows the driver to modify the train category Records the modifications of the train data in the JRU |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.3.3.2 |  |
|  |  | Subset-026-3.18.3.3 |  |
|  |  | Subset-026-4.7.2 table - Input information Train Data (refer to 3.18.3.2) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : • After the driver has requested the train data, the train data are displayed. • The driver is allowed by the on-board equipment to modify the current set of Train Data. • The driver is not allowed by the on-board equipment to validate the set of Train Data if no train category is selected. • The driver is allowed by the on-board equipment to validate the modified set of Train Data. Check on the JRU that : • Driver’s actions on the DMI are recorded and driver’s validation action is recorded. • The modified set of Train Data is recorded upon validation by the driver. |
| Constraints | The test case can be used in SB mode but outside the scope of the Start of mission procedure. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/4/6/12/13 | FS/OS/SR/UN/SB/LS/SN |
| Radio communication session | ESTABLISHED | Necessary only if level is L2 or L3 |
| Train Data | VALID | There is a valid set of Train Data stored on-board. NC_CDTRAIN=<NC_CDTRAIN_1> NC_TRAIN=<NC_TRAIN_1> |
| ERTMS/ETCS level | 0/1/2/3/4 | L0/LNTC/L1/L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| ‘TRAIN DATA’ BUTTON IS ENABLED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects ‘Train Data Entry’ | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=20) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The “Train data window” is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver modifies the "Train Category" and validates the "Flexible Train Data" | I | DMI | The on-board equipment does not authorize the driver to let the “Train category” value in blank before the validation of the Train Data. The driver sets the "Train Category" such that: <NC_CDTRAIN_2> different from <NC_CDTRAIN_1> <NC_TRAIN_2> different from <NC_TRAIN_1> | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=21) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2; NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 (NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is transmitted | O | RTM | Only if a communication session is established | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129; NID_PACKET=11; NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is recorded | O | JRU | Only if a communication session is established | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | Only if a communication session is established | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | Only if a communication session is established | N/A | N/A | - |
| 11 | N/A | N/A | The Main window is displayed. | O | DMI | - | N/A | N/A | - |

| Step 7: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | <NC_CDTRAIN_2> | Different from <NC_CDTRAIN_1> |
| NC_TRAIN | 15 | <NC_TRAIN_2> | Different from <NC_TRAIN_1> |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 9: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/1/2/4/6/12/13 UNCHANGED | FS/OS/SR/UN/SB/LS/SN |
| Radio communication session | ESTABLISHED | Necessary only if level is L2 or L3 |
| Train Data | STORED CHANGED | NC_CDTRAIN=<NC_CDTRAIN_2> NC_TRAIN=<NC_TRAIN_2> |
| ERTMS/ETCS level | 0/1/2/3/4 UNCHANGED | L0/LNTC/L1/L2 /L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Necessary only if level is L2 or L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | Texts displayed on DMI, fixed text messages and the data entry dialogs are displayed in language selected by the driver |  | 5 |
| Applicable Mode/Level Combinations | L0: SH / UN / SB / TR / NL L1: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV L2: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV L3: FS / LS / OS / SR / SH / SB / TR / PT / NL / RV LNTC: SH / SB / TR / NL / SN |  |  |
| Target of Test | The target of this test is to check that : The texts are displayed in language previously selected by the driver. Fixed text messages are displayed in language previously selected by the driver. The data entry dialogs are displayed in language previously selected by the driver. Note: Plain text messages are NOT displayed in language selected by the driver but in the language chosen by the trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Selection of language |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that all information visible on the DMI are permanently displayed in the language chosen by the driver (i.e. DMI data is influenced by the chosen language). |
| Constraints | All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_A> |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Language used to display information to the driver | STORED | <LANGUAGE_A> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Settings" [MAIN] | I | DMI | Language=<LANGUAGE_A> | N/A | N/A | - |
| 2 | N/A | N/A | The driver modifies "Language" | I | DMI | Selection of <LANGUAGE_B> | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The Settings window is displayed | O | DMI | All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_B> chosen by the driver SRS 3.12.3.1.4.1 Note : In case of plain text messages the trackside selects the language in which the message is displayed. | N/A | N/A | - |
| 5 | N/A | N/A | The driver modifies "Language" | I | DMI | Selection of <LANGUAGE_A> | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The Settings window is displayed | O | DMI | All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_A> chosen by the driver SRS 3.12.3.1.4.1 Note : In case of plain text messages the trackside selects the language in which the message is displayed. | N/A | N/A | - |
| 8 | N/A | N/A | The driver closes the "Settings" window | I | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 UNCHANGED | FS / OS / SR / SH / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | When the train is at standstill, the on-board equipment allows the driver to modify the current Driver ID at any time. |  | 6 |
| Applicable Mode/Level Combinations | L0: SH, UN, NL LNTC: NL, SN L1: FS, LS, OS, SR, SH, SB, NL L2: FS, LS, OS, SR, SH, SB, NL L3: FS, LS, OS, SR, SH, SB, NL |  |  |
| Target of Test | The target of this test case is to check that, when the train is at standstill, the on-board equipment allows the driver to modify the current Driver ID. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information Driver id |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board equipment allows the driver to modify the Driver Id. Check on the JRU that : The modified driver ID is recorded and is identical to the data entered by the driver. |
| Constraints | The train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 11 / 12 / 13 | FS / SR / OS / SH / SB / UN / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters the "Driver ID" | I | DMI | Entered Driver ID: DRIVER_ID_1 (different from DRIVER_ID_0) | N/A | N/A | - |
| 4 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=<DRIVER_ID_1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 11 / 12 / 13 UNCHANGED | FS / SR / OS / SH / SB / UN / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _1> CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill (V_TRAIN=0) | I | INT | Train speed = 0 |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | In normal operation during a mission the driver has no possibility to modify the current ERTMS/ETCS Level while the train is running. |  | 7 |
| Applicable Mode/Level Combinations | L0: SH, UN, NL LNTC: NL, SN L1: FS, LS, OS , SR, SH, TR, PT, NL, RV L2: FS, LS, OS , SR, SH, TR, PT, NL, RV L3: FS, LS, OS , SR, SH, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test case is to check that the on-board equipment does NOT allow the driver to modify the current ERTMS/ETCS Level data in a normal situation during a mission (i.e. case where the situation is NOT degraded) while the train is running. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Input information ERTMS/ETCS level |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that it is not possible for the driver to modify the ERTMS:ETCS level and/or to validate it. |
| Constraints | The on-board equipment is in normal operation during a mission. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 8 / 11 / 12 / 13 / 14 | FS / SR / OS / SH / UN / SL / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 VALID | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | Train speed >< 0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The "Level" button is not enabled | O | DMI | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 7 / 8 / 11 / 12 / 13 / 14 UNCHANGED | FS / SR / OS / SH / UN / SL / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | Train speed >< 0 |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.7.2: DMI versus Mode Table - Input |  | 4070200 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |