| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180200 (5.18.2: Passing a powerless section with pantograph to be lowered) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5180200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 21.08.2013 | All | Answer to Review003_Subset-076-5-2-5180200_v291 | MULTITEL |
| Version 2.9.3 27.11.2013 | All | Answer to Review001_Subset-076-5-2-5180200_v292 | MULTITEL |
| Version 2.9.4 13.12.2013 | TC1 | Add the requirement “4.5.2.1 table - Determine train position referenced to LRBG” | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.2: Passing a powerless section with pantograph to be lowered |  | 5180200 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of a powerless section where lowering of the pantograph is required. The on-board equipment displays to the driver a profile indicating the location where lowering of the pantograph is required and where raising of the pantograph is permitted. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that : -The on-board equipment displays to the driver a state of pantograph profile according to the received track condition information -The on-board equipment updates the state of pantograph profile according to train location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.5.2.1 table - Determine train position referenced to LRBG |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Pantograph control |  |
|  |  | Subset-026-5.18.2.2 |  |
|  |  | Subset-026-5.18.2.2.1 |  |
|  |  | Subset-026-5.18.2.2.2 |  |
|  |  | Subset-026-5.18.2.2.2 a) |  |
|  |  | Subset-026-5.18.2.2.2 b) |  |
|  |  | Subset-026-5.18.2.3 |  |
|  |  | Subset-026-5.18.2.3 a) |  |
|  |  | Subset-026-5.18.2.3 b) |  |
|  |  | Subset-026-5.18.2.5 |  |
|  |  | Subset-026-5.18.2.5 a) |  |
|  |  | Subset-026-5.18.2.5 b) |  |
|  |  | Subset-026-5.18.2.5.1 |  |
|  |  | Subset-026-5.18.2.5.1 a) |  |
|  |  | Subset-026-5.18.2.5.1 b) |  |
|  |  | Subset-026-5.18.2.6 |  |
|  |  | Subset-026-5.20.1.2 |  |
|  |  | Subset-026-5.20.2.2 |  |
|  |  | Subset-026-5.20.2.3 |  |
|  |  | Subset-026-5.20.2.3 a) |  |
|  |  | Subset-026-5.20.2.3 b) |  |
|  |  | Subset-026-5.20.2.4 |  |
|  |  | Subset-026-5.20.2.5 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI/JRU that: The state of pantograph profile is displayed according to the received track condition information. The state of pantograph profile is updated according to train location. Check on TIU/JRU that: The track condition information is transmitted according to the received profile |
| Constraints | The pantograph status is raised before the TC begins. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the powerless section L_TRACKCOND=FINITE VALUE, Length of the powerless section M_TRACKCOND=3, Powerless section – lower pantograph |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Lower pantograph announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -automatically (symbol TC02) -manually (the driver is requested to act) (symbol TC03) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the powerless section, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=1¦Bit46=1>) is recorded | O | JRU | TC02 (automatic execution) or TC03 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=0, D_MAXSFE_TO_START>0, D_MINSFE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=0, D_MAXSFE_TO_START>0, D_MINSFE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START and D_MINSFE_TO_END are decreasing | N/A | N/A | - |
| 7 | N/A | N/A | “Lower pantograph announcement” (symbol TC02 or symbol TC03) is REMOVED. | O | DMI | As soon as the train reaches the start location of the powerless section with its max safe front end ( = D_TRACKCOND ) | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=0&Bit46=0>) is recorded | O | JRU | TC02 (automatic execution) TC03 (driver requested to act) | N/A | N/A | - |
| 9 | N/A | N/A | “Lowered pantograph” (symbol TC01) is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit44=1>) is recorded | O | JRU | TC01 | N/A | N/A | - |
| 11 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 12 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=0, D_MAXSFE_TO_START=-32768, D_MINSFE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START not relevant and D_MINSFE_TO_END decreased | N/A | N/A | - |
| 13 | N/A | N/A | ”Lowered Pantograph” (symbol TC01) is REMOVED. | O | DMI | As soon as the train reaches the end location of the powerless section with its min safe front end. ( = D_TRACKCOND + L_ TRACKCOND ) | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit44=0>) is recorded | O | JRU | TC01 | N/A | N/A | - |
| 15 | N/A | N/A | “Raise Pantograph” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -Automatically (TC04) -Manually (the driver is requested to act) (TC05) | O | DMI | - | N/A | N/A | - |
| 16 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit47=1¦Bit48=1>) is recorded | O | JRU | TC04 (automatic execution) or TC05 (driver requested to act) | N/A | N/A | - |
| 17 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | “Raise pantograph” (symbol TC04 or symbol TC05) is REMOVED | O | DMI | The “Raise pantograph” information shall remain displayed for a fixed time (see Appendix A3.1, 5s) after the train reached the min safe rear end of the end location of powerless section. | N/A | N/A | - |
| 19 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit47=0&Bit48=0>) is recorded | O | JRU | TC04 (automatic execution) TC05 (driver requested to act) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.2: Passing a powerless section with pantograph to be lowered |  | 5180200 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of a powerless section where lowering of the pantograph is required. The on-board equipment displays to the driver a profile indicating the location where lowering of the pantograph is required and where raising of the pantograph is permitted. Since the deletion of the powerless section is requested by trackside, the ERTMS/ETCS on-board equipment stop indicating those locations and stop providing information related to this powerless section to the ERTMS/ETCS external function. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that : -The on-board equipment displays to the driver a state of pantograph profile according to the received track condition information -The on-board equipment updates the state of pantograph profile according to train location. -After the trackside request to the deletion of the powerless section, the ERTMS/ETCS on-board equipment stop providing information related to this powerless section to the ERTMS/ETCS external function |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 c) |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.5.2.1 table - Determine train position referenced to LRBG |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Pantograph control |  |
|  |  | Subset-026-5.18.2.2 |  |
|  |  | Subset-026-5.18.2.2.1 |  |
|  |  | Subset-026-5.18.2.2.2 |  |
|  |  | Subset-026-5.18.2.2.2 a) |  |
|  |  | Subset-026-5.18.2.2.2 b) |  |
|  |  | Subset-026-5.20.1.2 |  |
|  |  | Subset-026-5.20.2.2 |  |
|  |  | Subset-026-5.20.2.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI/JRU that: The state of pantograph profile is displayed according to the received track condition information. The state of pantograph profile is updated according to train location. Check on TIU/JRU that: The track condition information is transmitted according to the received profile The track condition information is no more transmitted after the trackside request the deletion of the powerless section |
| Constraints | The pantograph status is raised before the TC begins. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the powerless section L_TRACKCOND=FINITE VALUE, Length of the powerless section M_TRACKCOND=3, Powerless section – lower pantograph |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Lower pantograph announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -automatically (symbol TC02) -manually (the driver is requested to act) (symbol TC03) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the powerless section, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=1¦Bit46=1>) is recorded | O | JRU | TC02 (automatic execution) or TC03 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the powerless section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=0, D_MAXSFE_TO_START>0, D_MINSFE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 68 (Q_TRACKINIT = 1; D_TRACKINIT=0) is received | I | BTM | The information related to the powerless section is deleted | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT Track Conditions transmitted | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1 |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |