| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5120400 (5.12.4: The driver uses the same engine (a Shunting movement is ongoing)) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5120400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 12.09.2013 | All | Modifications requested in Review004_Subset-076-5-2-5120400_v290 | MULTITEL |
| Version 2.9.3 29.11.2013 | All | Modifications requested in Review001_Subset-076-5-2-5120400_v292 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.12.4: The driver uses the same engine (a Shunting movement is ongoing) |  | 5120400 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment is in Shunting mode, the driver closes the desk A and leaves the cab A of the Shunting engine, to go to cab B and open desk B of this same engine. Desk A and desk B are connected to the same ERTMS/ETCS on-board equipment. |  | 1 |
| Applicable Mode/Level Combinations | L0 / LNTC / L1 / L2 / L3 : SH |  |  |
| Target of Test | The target of this test is to check that : The driver enables the function “Continue Shunting on desk closure”. When the driver closes the desk A, the ERTMS/ETCS on-board equipment shall immediately go to Passive Shunting mode. When the driver opens the desk B, the ERTMS/ETCS on-board equipment shall immediately switch back to Shunting mode. The new train position data are calculated by use of the previous data, when the driver opens the desk B. The function Continue Shunting on desk closure (Enabling transition to Passive Shunting mode) is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.12.4.2.1 |  |
|  |  | Subset-026-5.12.4.2.2 |  |
|  |  | Subset-026-5.12.4.3 |  |
|  |  | Subset-026-4.4.20.1.2 |  |
|  |  | Subset-026-4.4.20.1.5 |  |
|  |  | Subset-026-4.4.20.1.6 |  |
|  |  | Subset-026-4.5.2.1 table - Continue Shunting on desk closure (Enabling transition to Passive Shunting mode) |  |
|  |  | Subset-026-4.6.2 row 3 column 4 |  |
|  |  | Subset-026-4.6.3 Condition Id [26] |  |
|  |  | Subset-026-4.7.2 table - Input information "Continue Shunting on desk closure" request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the TIU that : -The driver closes the desk A. -The driver opens the desk B. Check on the DMI that : -The driver enables the function “Continue Shunting on desk closure”. -The ERTMS/ETCS on-board equipment switches to Passive Shunting mode. -The ERTMS/ETCS on-board equipment switches back to Shunting mode. Check on the JRU that : -““Continue Shunting on desk closure” selected” is recorded -“CAB STATUS” closed and open are RECORDED -The change of modes are RECORDED -The updated “TRAIN_POSITION” field is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk A is open The “Passive shunting” input has the value “Passive shunting permitted” | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | SH | The driver enables the function “Continue Shunting on desk closure”. | I | DMI | The “Passive shunting” input must have the value “Passive shunting permitted” ‘Maintain Shunting’ is selected | N/A | SH | - |
| 2 | N/A | SH | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=15) is recorded | O | JRU | “Continue Shunting on desk closure” selected | N/A | SH | - |
| 3 | N/A | SH | The driver closes the desk A. | I | TIU | - | N/A | SH | - |
| 4 | N/A | SH | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | “CAB STATUS” closed is RECORDED | N/A | SH | - |
| 5 | N/A | SH | The ERTMS/ETCS on-board equipment switches immediately to Passive Shunting mode. | O | DMI | - | N/A | PS | - |
| 6 | N/A | PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=15) is recorded | O | JRU | - | N/A | PS | - |
| 7 | N/A | PS | The driver opens the desk B. | I | TIU | - | N/A | PS | - |
| 8 | N/A | PS | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | “CAB STATUS” open is RECORDED | N/A | PS | - |
| 9 | N/A | PS | The ERTMS/ETCS on-board equipment switches immediately back to Shunting mode. | O | DMI | - | N/A | SH | - |
| 10 | N/A | SH | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded | O | JRU | - | N/A | SH | - |
| 11 | N/A | SH | Any message (NID_MESSAGE_JRU=ALL) is recorded | O | JRU | The new train position data are calculated by use of the previous data, when the driver open the desk B. The variables of the TRAIN_POSITION field of any JRU message are updated: D_LRBG Q_DIRLRBG Q_DLRBG | N/A | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 UNCHANGED | SH |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Desk B is open | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |