| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080436 (4.8.4: Accepted Information depending on the modes - Route Suitability Data) Total: 9 Test case(s) |  |
| REF: | Subset-076-5-2-4080436 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 02.09.2013 | ALL | EDITROIAL CHANGES | CEDEX |
| Version 2.9.2 25.11.2013 | ALL | EDITORIAL CHANGES | CEDEX |
| Version 2.9.3 11.12.2013 | ALL | EDITORIAL CHANGES | CEDEX |
| Version 2.9.4 11.12.2013 | ALL | EDITORIAL CHANGES | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Acceptance of message 24 (General message) including packet 70 (Route Suitability) that is transmitted from RBC (in L2, L3). |  | 1 |
| Applicable Mode/Level Combinations | L2: SB / FS / OS / SR / LS / PT L3: SB / FS / OS / SR / LS / PT |  |  |
| Target of Test | The target of this test is to check that The message 24 including packet 70 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message 24 including packet 70 (Route Suitability Data) is recorded. |
| Constraints | A cab needs to be active and valid Train Data are stored on-board (SB Mode) In PT mode, only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR MODE |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 is received | I | RTM | The message is accepted. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3120200.1 | - | - | After changing to FS, OS or LS, if it is necessary, check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of loading gauge profile | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3120200.2 | - | - | After changing to FS, OS or LS, if it is necessary, check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of power supply profile | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3120200.3 | - | - | After changing to FS, OS or LS, if it is necessary, check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of axle load profile | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR MODE |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Rejection of Route Suitability Data in L0, LNTC, or L1 if no level transition announcement to L2/3 has been received. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / LS / OS / SR / SB |  |  |
| Target of Test | The target of this test is to check that The message 24 including packet 70 is rejected in case of no announcement of level transition to level 2 or 3 has been received by the onboard equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message 24 including packet 70 (Route Suitability Data) is recorded. Check on the DMI that: the information about unsuitability is not displayed, when the on-board equipment is in FS, OS or LS mode |
| Constraints | Packet 41 (Level Transition announcement to switch to L2 or L3 at a further location, i.e. D_LEVELTR ( 0 and ( 32767) was not received and is not stored onboard. The data of packet 70 do not allow the train to enter a defined route. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / / 2 / 4 / 6 / 12 / 13 | FS/ OS / SR / UN / SB / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | To L2 / 3 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 is received | I | RTM | Route Suitability Data (Packet 70) is not accepted because no Level transition to L2 or L3 is announced (see starting conditions) | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 2 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | - | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 3 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27, packet 41 (D_LEVELTR<>0; M_LEVELTR=<3¦4>) is received | I | RTM | Only if in L0, LNTC, L1/SR or L1/SB. | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 4 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=3) is recorded | O | JRU | Only if in L0, LNTC, L1/SR or L1/SB. | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 5 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=<3¦4>) is received | I | BTM | - | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 6 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | - |
| 7 | L0 / LNTC / L1 | SB / FS / OS / LS / SR / UN / SN | The Level symbol "Level <L2 / L3>" is displayed | O | DMI | - | L2 / L3 | FS / OS / LS | - |
| 8 | L2 / L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1¦Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | Not the system status message "Route unsuitable - <ALC / LG / TS>" is displayed | O | DMI | The on-board equipment will have to be in FS, LS or OS mode to test this step, hence a mode transition may be needed before testing this step. | L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | Not DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1¦Bit21=1¦Bit22=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | <>0 | Different from 0 |
| M_LEVELTR | 3 | 3/4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Rejection of Route Suitability Data, if train data has been sent to the RBC and the acknowledgement of train data has not been received yet. |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR, SB, PT, LS L3: FS, OS, SR, SB, PT, LS |  |  |
| Target of Test | The target of this test is to check that Packet 70 is rejected by the on-board equipment |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : the not accepted Packet 70 (Route Suitability Data) is recorded. Check on the DMI that: the information about unsuitability is not displayed, when the on-board equipment is in FS, OS or LS mode |
| Constraints | The data of packet 70 do not allow the train to enter a defined route. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 is received | I | RTM | Route Suitability Data is rejected because one message 24 was sent by the on-board equipment but message 8 has not been received yet. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=24) is recorded | O | JRU | Route Suitability Data is recorded | N/A | N/A | - |
| 5 | N/A | N/A | Not the system status message "Route unsuitable - <ALC / LG / TS>" is displayed | O | DMI | Train at standstill: The train position is right in front of location given in D_SUITABILITY. The on-board equipment will have to be in FS, LS or OS mode to test this step, hence a mode transition may be needed before testing this step. | N/A | N/A | - |
| 6 | N/A | N/A | Not DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1¦Bit21=1¦Bit22=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| Step 7: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Acceptance of Eurobalise telegram including packet 70 (Route Suitability) that is transmitted via balise group (L1). |  | 4 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / SB |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 70 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: a balise Telegram including packet 70 (Route Suitability Data) is recorded. |
| Constraints | In SB mode the BG shall consist of only one balise. A cab needs to be active and valid Train Data are stored on-board (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Movement Authority | STORED | Only for FS / OS / LS |
| Gradient Profile | STORED | Only for FS / OS / LS |
| International SSP | STORED | Only for FS / OS / LS |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR MODE |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 70 is received | I | BTM | In SB mode the BG shall consist of only one balise | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3120200.1 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of loading gauge profile | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3120200.2 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of power supply profile | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3120200.3 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of axle load profile | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| List of Balises in SR Authority + SR mode speed limit and distance | STORED | Only in SR MODE |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Rejection of Route Suitability Data, if no level transition announcement to L1 has been received. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2; FS / LS / OS / SR / SB L3: FS / LS / OS / SR / SB |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 70 is rejected in case of no announcement of level transition to level 1 has been received by the onboard equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Telegram including packet 70 (Route Suitability Data) is recorded. Check on the DMI that: the information about unsuitability is not displayed, when the on-board equipment is in FS, OS or LS mode |
| Constraints | Packet 41 (Level Transition announcement to switch to L1 at a further location, i.e. D_LEVELTR ( 0 and ( 32767) was not received and is not stored onboard. The data of packet 70 do not allow the train to enter a defined route. A cab needs to be active and valid Train Data are stored on-board (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Level Transition Order | NOT STORED | To L1 |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |
| Radio Network ID | STORED | only for L2 / L3 |
| RBC ID/Phone Number | STORED | only for L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | One Balise Group message containing packet 70 is received | I | BTM | Route Suitability Data (Packet 70) is not accepted because no Level transition to L1 is announced (see starting conditions) | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | - |
| 2 | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | - |
| 3 | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=2) is received | I | BTM | Packet 12, 21 and 27 will also be sent if in L0 or LNTC. | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | - |
| 4 | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | - |
| 5 | L0/LNTC/L2/L3 | UN / SN / FS / LS / OS / SR / SB | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | Not the system status message "Route unsuitable - <ALC / LG / TS>" is displayed | O | DMI | - | L1 | FS / OS / LS | - |
| 8 | L1 | FS / OS / LS | Not DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1¦Bit21=1¦Bit22=1>) is recorded | O | JRU | - | L1 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 2 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Acceptance of Eurobalise telegram including packet 70 (Route Suitability) that is transmitted via infill balise group (L1). |  | 6 |
| Applicable Mode/Level Combinations | L1: FS / LS |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 70 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: a balise Telegram including packet 70 (Route Suitability Data) is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 136, packet 70 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3120200.1 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of loading gauge profile | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3120200.2 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of power supply profile | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3120200.3 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the form: Supervision of axle load profile | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Rejection of Route Suitability Data, if the current mode is not the appropriate one. |  | 7 |
| Applicable Mode/Level Combinations | L2: TR L3: TR |  |  |
| Target of Test | The target of this test is to check that The Telegram including packet 70 is rejected in case the onboard equipment is in TR mode, which is one of the modes where the Route Suitability Data is rejected. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Radio message or BG message including packet 70 (Route Suitability Data) is recorded. Check on the DMI that: the information about unsuitability is not displayed, when the on-board equipment has changed to FS, OS or LS mode |
| Constraints | The data of packet 70 do not allow the train to enter a defined route. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | - |
| Train Running Number | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | TR | One Balise Group message containing packet 70 is received | I | BTM | Only if the current level is L1. Route Suitability Data (Packet 70) is rejected because the current mode does not support its acceptance | L2 / L3 | TR | - |
| 2 | L2 / L3 | TR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if the current level is L1. | L2 / L3 | TR | - |
| 3 | L2 / L3 | TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 is received | I | RTM | Only if the current level is L2. Route Suitability Data (Packet 70) is rejected because the current mode does not support its acceptance | L2 / L3 | TR | - |
| 4 | L2 / L3 | TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if the current level is L2. | L2 / L3 | TR | - |
| 5 | L2 / L3 | TR | Use_FT5110200.TC13 | - | - | the driver ack TRIP the on-board equipment has to change to PT mode and then to one of the following modes: FS, LS or OS, in order to properly test the last step of this TC. Use the correct | L2 / L3 | PT | - |
| 6 | L2 / L3 | PT | Use_FT4060300.TC20 | - | - | For Transition [31] to FS | L2 / L3 | PT | - |
| 7 | L2 / L3 | PT | Use_FT4060300.TC42 | - | - | For Transition [15] to OS mode | L2 / L3 | PT | - |
| 8 | L2 / L3 | FS / OS | Not the system status message "Route unsuitable - <ALC / LG / TS>" is displayed | O | DMI | The train is allowed to pass the location given in D_SUITABILITY (Packet 70 was rejected). The on-board equipment will have to be in FS, LS or OS mode to test this step, hence a mode transition may be needed before testing this step. | L2 / L3 | FS / OS | - |
| 9 | L2 / L3 | FS / OS | Not DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1¦Bit21=1¦Bit22=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Acceptance of message 24 (General message) including packet 70 (Route Suitability) that is transmitted from RBC in L0, LNTC or L1 when a Level Transition announcement to switch to L2 or L3 at a further location is stored on-board. |  | 8 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that The message 24 including packet 70 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Message 24 including packet 70 (Route Suitability Data) is recorded. |
| Constraints | Packet 41 (Level Transition announcement to switch to L2 or L3 at a further location, i.e. D_LEVELTR ( 0 and ( 32767) was received and is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| Level Transition Order | STORED | M_LEVELTR=3/4 |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 0 / 1 / 2 | L 0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 70 is received | I | RTM | The message is stored on-board and then it will be accepted. | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 2 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 3 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 is received | I | RTM | Only if in L0 or LNTC. | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 4 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=3) is recorded | O | JRU | Only if in L0 or LNTC. | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 5 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=<3¦4>) is received | I | BTM | - | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 6 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / LS / UN / SN | - |
| 7 | L0 / LNTC / L1 | FS / OS / LS / UN / SN | The Level symbol "Level <L2 / L3>" is displayed | O | DMI | - | L2 / L3 | FS / OS / LS | - |
| 8 | L2 / L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1¦Bit05=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |
| 9 | L2 / L3 | FS / OS / LS | The system status message "Route unsuitable - <ALC / LG / TS>" is displayed | O | DMI | - | L2 / L3 | FS / OS / LS | - |
| 10 | L2 / L3 | FS / OS / LS | Not DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit20=1¦Bit21=1¦Bit22=1>) is recorded | O | JRU | - | L2 / L3 | FS / OS / LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 3 / 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Route Suitability Data |  | 4080436 |
| Test Case of Feature | Acceptance of Eurobalise telegram including packet 70 (Route Suitability) that is transmitted via balise group in L0, LNTC or L2/L3, when a Level Transition announcement to switch to L1 at a further location is stored on-board. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that The BG message including packet 70 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Route Suitability Data |  |
|  |  | Subset-026-4.8.4.2 Route Suitability Data |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.21 Packet Number 70: route suitability data |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The BG message including packet 70 (Route Suitability Data) is recorded. |
| Constraints | Packet 41 (Level Transition announcement to switch to L1 at a further location, i.e. D_LEVELTR ( 0 and ( 32767) was received and is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Only in FS/ OS / LS mode |
| Gradient Profile | STORED | Only in FS/ OS / LS mode |
| International SSP | STORED | Only in FS/ OS / LS mode |
| Level Transition Order | STORED | D_LEVELTR=0 and 32767. M_LEVELTR=2 |
| Mode Profile | STORED | For OS / LS MODE |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L 0 / LNTC / L2 / L3 |
| Radio Network ID | STORED | only for L2 / L3 |
| RBC ID/Phone Number | STORED | only for L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 70 is received | I | BTM | The message is stored on-board and then it will be accepted. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The on-board equipment changes to L1. | - | - | - | N/A | N/A | - |
| 4 | N/A | N/A | The on-board equipment changes to FS, LS or OS mode. | - | - | Only if the current mode is not FS, LS or OS already. (The information “Route Suitability” will not be removed when changing to any of these modes) | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3120200 | - | - | After changing to FS, OS or LS, if it is necessary, use one of the TCs to check the reaction of accepted packet 70 (e.g. the Fixed text information (ROUTE UNSUITABILITY) is DISPLAYED) in the for: Supervision of loading gauge profile Supervision of power supply profile Supervision of axle load profile | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 70 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_SUITABILITY | 15 | FINITE VALUE | Only If Q_TRACKINIT = 0, D_SUITABILITY and the following variables follows. |
| Q_SUITABILITY | 2 | FINITE VALUE | - |
| M_LINEGAUGE | 8 | FINITE VALUE | If Q_SUITABILITY= loading gauge. |
| M_AXLELOADCAT | 7 | FINITE VALUE | If Q_SUITABILITY= Max axle load. |
| M_VOLTAGE | 4 | FINITE VALUE | If Q_SUITABILITY = traction system. |
| NID_CTRACTION | 10 | FINITE VALUE | If Q_SUITABILITY = traction system and M_VOLTAGE ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| D_SUITABILITY(k) | 15 | FINITE VALUE | - |
| Q_SUITABILITY(k) | 2 | FINITE VALUE | - |
| M_LINEGAUGE(k) | 8 | FINITE VALUE | If Q_SUITABILITY(k) = loading gauge. |
| M_AXLELOADCAT(k) | 7 | FINITE VALUE | If Q_SUITABILITY(k) = Max axle load. |
| M_VOLTAGE(k) | 4 | FINITE VALUE | If Q_SUITABILITY(k) = traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | If Q_SUITABILITY(k) = traction system and M_VOLTAGE(k) ≠ 0. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |