| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5200700 (Changing the allowed current consumption) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5200700 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 15.09.2015 | All | First version/review | CEDEX |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Changing the allowed current consumption |  | 5200700 |
| Test Case of Feature | This TC specifies the information generated by the ERTMS/ETCS on-board equipment to an ERTMS/ETCS external function in relation to a change of allowed current consumption. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check the generation of the information related to a change of allowed current consumption by the ERTMS/ETCS on-board equipment to the ERTMS/ETCS external function. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-5.20.7.2 |  |
|  |  | Subset-026-5.20.7.3 |  |
|  |  | Subset-026-5.20.7.4 |  |
|  |  | Subset-026-5.20.7.4 a) |  |
|  |  | Subset-026-5.20.7.4 b) |  |
|  |  | Subset-026-5.20.7.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that: The command of changing current consumption is changed according to the received profile and the EVC Action Check on JRU that: JRU Message 45 'Track Conditions' is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 40 (Q_TRACKINIT=0; D_TRACKCOND > 0), is received | I | BTM | Only in L1. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 40 (Q_TRACKINIT=0; D_TRACKCOND>0) is received | I | RTM | Only in L2/L3. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 5 | N/A | N/A | CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The max safe front end of the train reaches a location (point C) in rear of the location of change of allowed current consumption (point F), the ERTMS/ETCS starts the generation of the information to the ERTMS/ETCS external function. | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The min safe rear end of the train is in rear of the location of change of allowed current consumption . The ERTMS/ETCS on-board equipment shall provide the following information to the ERTMS/ETCS external function: -the remaining distance from the max safe front end of the train tothe location of change of allowed current consumption (point F) -the new allowed current consumption | N/A | N/A | - |
| 8 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START= -32768,D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The min safe rear end of the train reaches the location of change of allowed current consumption (Point F). The remaining distance from the max safe front end of the train to point F and the new allowed current consumption shall no longer be generated. | N/A | N/A | - |
| 10 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MINSRE_TO_END>0) is recorded | O | JRU | Message 45: Trak Conditions. The distance variables have to be changed D_MINSRE_TO_END | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 40 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_CURRENT | 15 | FINITE VALUE | - |
| M_CURRENT | 10 | FINITE VALUE | Allowed current consumption. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 40 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_CURRENT | 15 | FINITE VALUE | - |
| M_CURRENT | 10 | FINITE VALUE | Allowed current consumption. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | Only in L2/L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Changing the allowed current consumption |  | 5200700 |
| Test Case of Feature | This TC specifies when the information generated by the ERTMS/ETCS on-board equipment to an ERTMS/ETCS external function in relation to a change of allowed current consumption shall STOP. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check when the ERTMS/ETCS shall stop providing information related to the change of allowed current consumption to the ERTMS/ETCS external function. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-5.20.7.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that: The command of changing current consumption is stop due to linking reaction (service brake is applied and the train reached at standstill) Check on JRU that: JRU Message 45 'Track Conditions' is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 40 (Q_TRACKINIT=0; D_TRACKCOND > 0), is received | I | BTM | Only in L1. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 40 (Q_TRACKINIT=0; D_TRACKCOND>0) is received | I | RTM | Only in L2/L3. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 5 | N/A | N/A | CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The max safe front end of the train reaches a location (point C) in rear of the location of change of allowed current consumption (point F), the ERTMS/ETCS starts the generation of the information to the ERTMS/ETCS external function. | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The min safe rear end of the train is in rear of the location of change of allowed current consumption . The ERTMS/ETCS on-board equipment shall provide the following information to the ERTMS/ETCS external function: -the remaining distance from the max safe front end of the train tothe location of change of allowed current consumption (point F) -the new allowed current consumption | N/A | N/A | - |
| 8 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START= -32768,D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | Use_FT3160200.70 | - | - | - | N/A | N/A | - |
| 10 | N/A | N/A | NOT CHANGE OF ALLOWED CURRENT CONSUMPTION | O | TIU | The allowed current consumption shall no longer be generated by the ERTMS/ETCS. | N/A | N/A | - |
| 11 | N/A | N/A | NOT TRACK CONDITIONS (D_MAXSFE_TO_START= -32768,D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 40 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_CURRENT | 15 | FINITE VALUE | - |
| M_CURRENT | 10 | FINITE VALUE | Allowed current consumption. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 40 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_CURRENT | 15 | FINITE VALUE | - |
| M_CURRENT | 10 | FINITE VALUE | Allowed current consumption. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |