| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5200800 (Station platform) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5200800 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 15.09.2015 | All | First version/review | CEDEX |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Station platform |  | 5200800 |
| Test Case of Feature | This TC specifies the information generated by the ERTMS/ETCS on-board equipment to an ERTMS/ETCS external function in relation to Station Platform |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check the generation of the information related to Station Platform by the ERTMS/ETCS on-board equipment to the ERTMS/ETCS external function. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-5.20.8.2 |  |
|  |  | Subset-026-5.20.8.3 |  |
|  |  | Subset-026-5.20.8.4 |  |
|  |  | Subset-026-5.20.8.4 a) |  |
|  |  | Subset-026-5.20.8.4 b) |  |
|  |  | Subset-026-5.20.8.4 c) |  |
|  |  | Subset-026-5.20.8.4 d) |  |
|  |  | Subset-026-5.20.8.5 |  |
|  |  | Subset-026-5.20.8.6 |  |
|  |  | Subset-026-7.4.2.20.1 Packet Number 69: Track Condition Station Platforms |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that: The command of Station Platform is changed according to the received profile and the EVC Action Check on JRU that: JRU Message 45 'Track Conditions' is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 69 (Q_TRACKINIT=0; D_TRACKCOND > 0), is received | I | BTM | Only in L1. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 69 (Q_TRACKINIT=0; D_TRACKCOND>0) is received | I | RTM | Only in L2/L3. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 5 | N/A | N/A | STATION PLATFORMS | O | TIU | The max safe front end of the train reaches a location (point C) in rear of the start location of the station platform (point D), the ERTMS/ETCS starts the generation of the information to the ERTMS/ETCS external function. Point C shall be determined by the ERTMS taking into accont the time necessary for performing thee required actions and the current train speed | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The <Main> window is displayed. | O | DMI | DMI does not change. Station Platforms does not display | N/A | N/A | - |
| 8 | N/A | N/A | STATION PLATFORMS | O | TIU | The min safe rear end of the train is in rear of the start location (point D) of the station platform. The ERTMS/ETCS on-board equipment shall provide the following information to the ERTMS/ETCS external function: -the remaining distance from the max safe front end of the train to the start location (point D) of the station platform -the remaining distance from the min safe front end of the train to the end location (point E) of the station platform -the nominal height of platform above rail level (refer to TSI infrastructure) -the position of the station platform (left side, right side, both sides) in reference to the train orientation | N/A | N/A | - |
| 9 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START= -32768,D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | STATION PLATFORMS | O | TIU | The min safe rear end of the train reaches the start location (point D) of the station platform. The remaining distance from the max safe front end of the train to point D shall no longer be generated. The remaining distance from the min safe front end of the train to the end location (point E) of the station platform shall continue to be generated. | N/A | N/A | - |
| 11 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START= -32768, D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | STATION PLATFORMS | O | TIU | The min safe rear end of the train reaches the end location (point E) of the Station Platform. The remaining distance from the min safe front end of the train to point E shall no longer be generated | N/A | N/A | - |
| 13 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 69 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 3/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 69 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_PLATFORM | 4 | FINITE VALUE | - |
| Q_PLATFORM | 2 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_PLATFORM(k) | 4 | FINITE VALUE | - |
| Q_PLATFORM(k) | 2 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Station platform |  | 5200800 |
| Test Case of Feature | This TC specifies when the information generated by the ERTMS/ETCS on-board equipment to an ERTMS/ETCS external function in relation to Station Platform shall STOP |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check when the ERTMS/ETCS shall stop the generation of the information related to Station Platform to the ERTMS/ETCS external function. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-5.20.8.9 |  |
|  |  | Subset-026-7.4.2.20.1 Packet Number 69: Track Condition Station Platforms |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that: The command of Station Platform is stop due to linking reaction (service brake is applied and the train reached at standstill) Check on JRU that: JRU Message 45 'Track Conditions' is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 69 (Q_TRACKINIT=0; D_TRACKCOND>0), is received | I | BTM | Only in L1. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only in L1. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 69 (Q_TRACKINIT=0; D_TRACKCOND>0) is received | I | RTM | Only in L2/L3. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only in L2/L3. | N/A | N/A | - |
| 5 | N/A | N/A | STATION PLATFORMS | O | TIU | The max safe front end of the train reaches a location (point C) in rear of the start location of the station platform (point D), the ERTMS/ETCS starts the generation of the information to the ERTMS/ETCS external function. Point C shall be determined by the ERTMS taking into accont the time necessary for performing thee required actions and the current train speed | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START>0) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | The <Main> window is displayed. | O | DMI | DMI does not change. Station Platforms does not display | N/A | N/A | - |
| 8 | N/A | N/A | STATION PLATFORMS | O | TIU | The min safe rear end of the train is in rear of the start location (point D) of the station platform. The ERTMS/ETCS on-board equipment shall provide the following information to the ERTMS/ETCS external function: -the remaining distance from the max safe front end of the train to the start location (point D) of the station platform -the remaining distance from the min safe front end of the train to the end location (point E) of the station platform -the nominal height of platform above rail level (refer to TSI infrastructure) -the position of the station platform (left side, right side, both sides) in reference to the train orientation | N/A | N/A | - |
| 9 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MAXSFE_TO_START= -32768,D_MINSFE_TO_END>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | Use_FT3160200.70 | - | - | Linking consistency problem: The location reference of the expected BG marked as linked and included in the linking information is found in rear of the expectation window. The system reacts according to the linking reaction: application of the SB. | N/A | N/A | - |
| 11 | N/A | N/A | NOT STATION PLATFORMS | O | TIU | The station platforms shall no longer be generated by the ERTMS/ETCS | N/A | N/A | - |
| 12 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45; D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 69 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY(k) | 1 | FINITE VALUE | - |
| NID_C(k) | 10 | FINITE VALUE | if Q_NEWCOUNTRY(k) = 1. |
| NID_BG(k) | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 3/3) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 2 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 69 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow. |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_PLATFORM | 4 | FINITE VALUE | - |
| Q_PLATFORM | 2 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_PLATFORM(k) | 4 | FINITE VALUE | - |
| Q_PLATFORM(k) | 2 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | FS / OS / TR / PT / NL / LS | 0 / 1 / 7 / 8 / 11 / 12 |
| ERTMS/ETCS level | L1 / L2 / L3 | 2 / 3 / 4 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |