| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3110300_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3110300_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 19:08 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | INECO |

# Sequence description

Supervision of static speed profile depending on train categories Tested here TC1, TC2, TC4 and TC3

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | NP | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1. [59.-1.1] |  |  | L1 |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | FS | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 43 |  | 5 |  |  | 100.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3110300: 3.11.3: Static Speed Profile (SSP) Test Case 1: Supervision of static speed profile depending on train categories. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 300.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 45 |  | 2 |  |  | 400.00 |  | L1 | FS |  | Balise Group message containing packet 27 is received. |  | I |  | BTM |  | 0<V_STATIC<121 N_ITER must be > 0 to define different speed depending on train category. Q_DIFF (n)=0 NC_CDDIFF(n) is the specific SSP category for which a different value of the static line speed exists. V_DIFF(n) value must be the most restrictive in the SSP to ensure that the restriction is not rejected by the train. |  |  | L1 |  | FS | - |  |
|  | 46 |  | 3 |  |  | 400.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 47 |  | 4 |  |  | 700.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train runs the distance D_STATIC. |  |  | L1 |  | FS | - |  |
|  | 50 |  | 7 |  |  | 700.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | If in FS or OS mode, permitted speed is the one defined by the train category in the SSP.(V_DIFF) V_TRAIN < V_DIFF The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section If in LS mode the DMI aspect is coherent with Overspeed status in a ceiling speed monitoring section. V_TRAIN > V_DIFF |  |  | L1 |  | FS | - |  |
|  | 51 |  | 8 |  |  | 700.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_DIFF Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) |  |  | L1 |  | FS | - |  |
|  | 52 |  | 9 |  |  | 1200.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED+10>) |  |  | L1 |  | FS | - |  |
|  | 53 |  | 10 |  |  | 1200.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | Status requested: Intervention status |  |  | L1 |  | FS | - |  |
|  | 54 |  | 11 |  |  | 1200.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V_DIFF> ; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L1 |  | FS | - |  |
|  | 55 |  | 12 |  |  | 1200.00 |  | L1 | FS |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 56 |  | 13 |  |  | 1200.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L1 |  | FS | - |  |
|  | 57 |  | 14 |  |  | 1200.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 58 |  | 15 |  |  | 1230.00 |  | L1 | FS |  | Train is moving |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 59 |  | 16 |  |  | 1230.00 |  | L1 | FS |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 60 |  | 17 |  |  | 1230.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | The speed is lower or equal to the minimum of speed permitted by the SSP. |  |  | L1 |  | FS | - |  |
|  | 61 |  | 18 |  |  | 1230.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 62 |  | 19 |  |  | 1230.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | If in FS or OS mode Indication of permitted speed: V_DIFF Status requested: Normal status If in LS mode the V_TRAIN < V_DIFF. The DMI aspect is coherent with Normal Status. |  |  | L1 |  | FS | - |  |
|  | 63 |  | 20 |  |  | 1230.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L1 |  | FS | - |  |
|  | Feature 3110300: 3.11.3: Static Speed Profile (SSP) Test Case 2: Supervision of specific line speed depending on train categories. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 1500.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 65 |  | 2 |  |  | 1600.00 |  | L1 | FS |  | Balise Group message containing packet 27 is received. |  | I |  | BTM |  | 0<V_STATIC<121 N_ITER must be > 2 to define different speed depending on train category. Q_DIFF (n)=0 NC_CDDIFF(n=a, b, d) = finite values; Specific SSP category, different from NC_CDTRAIN (c) a<b<c<d |  |  | L1 |  | FS | - |  |
|  | 66 |  | 3 |  |  | 1600.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 67 |  | 4 |  |  | 1900.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train runs the distance D_STATIC. The on-board equipment selects the 'Cant Deficiency' SSP with the highest Cant Deficiency value below the value of its 'Cant Deficiency' train category |  |  | L1 |  | FS | - |  |
|  | 70 |  | 7 |  |  | 1900.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: V_DIFF(2) If in LS mode the train speed will be a bit over the V_DIFF(2). The DMI aspect is coherent with Overspeed Status. V_TRAIN>V_DIFF(2) |  |  | L1 |  | FS | - |  |
|  | 71 |  | 8 |  |  | 1900.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS: Check that V_PERM is updated with V_DIFF(2) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) |  |  | L1 |  | FS | - |  |
|  | 72 |  | 9 |  |  | 2100.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED+10>) |  |  | L1 |  | FS | - |  |
|  | 73 |  | 10 |  |  | 2100.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | Status requested: Intervention status |  |  | L1 |  | FS | - |  |
|  | 74 |  | 11 |  |  | 2100.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L1 |  | FS | - |  |
|  | 75 |  | 12 |  |  | 2100.00 |  | L1 | FS |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 76 |  | 13 |  |  | 2100.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L1 |  | FS | - |  |
|  | 77 |  | 14 |  |  | 2100.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 78 |  | 15 |  |  | 2130.00 |  | L1 | FS |  | Train is moving |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 79 |  | 16 |  |  | 2130.00 |  | L1 | FS |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 80 |  | 17 |  |  | 2130.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | The speed is lower or equal to the minimum of speed permitted by the SSP. |  |  | L1 |  | FS | - |  |
|  | 81 |  | 18 |  |  | 2130.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 82 |  | 19 |  |  | 2130.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Normal status If in FS or OS mode indication of permitted speed: V_DIFF(2) If in LS mode the V_TRAIN < V_DIFF(2). The DMI aspect is coherent with Normal Status. |  |  | L1 |  | FS | - |  |
|  | 83 |  | 20 |  |  | 2130.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L1 |  | FS | - |  |
|  | Feature 3110300: 3.11.3: Static Speed Profile (SSP) Test Case 4: Supervision of static speed profile depending on train categories. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 1 |  |  | 2400.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 85 |  | 2 |  |  | 2500.00 |  | L1 | FS |  | Balise Group message containing packet 27 is received. |  | I |  | BTM |  | 0 < V_STATIC < 121 N_ITER>0 to define different speed depending on train category. Q_DIFF(n)=1 NC_DIFF(n)= It is the specific SSP category for which a different value of the static line speed exists.. V_DIFF(n) must be the most restrictive in the SSP to ensure that the restriction is not rejected by the train. |  |  | L1 |  | FS | - |  |
|  | 86 |  | 3 |  |  | 2500.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 87 |  | 4 |  |  | 2800.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train runs the distance D_STATIC. |  |  | L1 |  | FS | - |  |
|  | 90 |  | 7 |  |  | 2800.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | If in FS or OS mode check that the permitted speed is the one defined by the train category in the SSP. (V_DIFF(n)) If in LS mode the train speed will be a bit over the V_DIFF(n). The DMI aspect is coherent with Overspeed Status. V_TRAIN > V_DIFF(n) |  |  | L1 |  | FS | - |  |
|  | 91 |  | 8 |  |  | 2800.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(n)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS: Check that V_PERM is updated with V_DIFF(n) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) |  |  | L1 |  | FS | - |  |
|  | 92 |  | 9 |  |  | 3300.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED+10>) |  |  | L1 |  | FS | - |  |
|  | 93 |  | 10 |  |  | 3300.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | Indication of permitted speed: V_DIFF(n) Status requested: Intervention status |  |  | L1 |  | FS | - |  |
|  | 94 |  | 11 |  |  | 3300.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V_DIFF(n)> ; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L1 |  | FS | - |  |
|  | 95 |  | 12 |  |  | 3300.00 |  | L1 | FS |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 96 |  | 13 |  |  | 3300.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L1 |  | FS | - |  |
|  | 97 |  | 14 |  |  | 3300.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 98 |  | 15 |  |  | 3330.00 |  | L1 | FS |  | Train is moving |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 99 |  | 16 |  |  | 3330.00 |  | L1 | FS |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  | FS | - |  |
|  | 100 |  | 17 |  |  | 3330.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | The speed is lower or equal to the minimum of speed permitted by the SSP. |  |  | L1 |  | FS | - |  |
|  | 101 |  | 18 |  |  | 3330.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 102 |  | 19 |  |  | 3330.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Normal status If in FS or OS mode indication of permitted speed: V_DIFF(n) If in LS mode the V_TRAIN < V_DIFF(n). The DMI aspect is coherent with Normal Status. |  |  | L1 |  | FS | - |  |
|  | 103 |  | 20 |  |  | 3330.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(n)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L1 |  | FS | - |  |
|  | Feature 3110300: 3.11.3: Static Speed Profile (SSP) Test Case 3: Supervision of specific line speed depending on train categories. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 104 |  | 1 |  |  | 3500.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L1 |  | FS | - |  |
|  | 105 |  | 2 |  |  | 3600.00 |  | L1 | FS |  | Balise Group message containing packet 27 is received. |  | I |  | BTM |  | 0<V_STATIC<121and More restrictive than V_DIFF Q_DIFF (n)=2 |  |  | L1 |  | FS | - |  |
|  | 106 |  | 3 |  |  | 3600.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 107 |  | 4 |  |  | 3900.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train runs the distance D_STATIC. |  |  | L1 |  | FS | - |  |
|  | 110 |  | 7 |  |  | 3900.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: (V_STATIC) If in LS mode the train speed will be a bit over the V_STATIC. The DMI aspect is coherent with Overspeed Status. V_TRAIN>V_STATIC. |  |  | L1 |  | FS | - |  |
|  | 111 |  | 8 |  |  | 3900.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_STATIC>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in FS/OS: Check that V_PERM is updated with V_STATIC Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) |  |  | L1 |  | FS | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 113 |  | 1 |  |  | 4100.00 |  | L1 | FS |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  | FS | - |  |
|  | 114 |  | 2 |  |  | 4100.00 |  | L1 | FS |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 115 |  | 3 |  |  | 4100.00 |  | L1 | FS |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L1 |  | FS | - |  |
|  | 116 |  | 4 |  |  | 4100.00 |  | L1 | FS |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  | FS | - |  |
|  | Feature 3110300: 3.11.3: Static Speed Profile (SSP) Test Case 3: Supervision of specific line speed depending on train categories. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 117 |  | 10 |  |  | 4200.00 |  | L2 | FS |  | Train is moving. |  | I |  | INT |  | (V_TRAIN=<V_PERMITTED>) |  |  | L2 |  | FS | - |  |
|  | 118 |  | 11 |  |  | 4200.00 |  | L2 | FS |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: V_STATIC No modification on permitted speed value occurred If in LS mode the train speed will be a bit over the V_STATIC. The DMI aspect is coherent with NORMAL Status. |  |  | L2 |  | FS | - |  |
|  | 119 |  | 12 |  |  | 4200.00 |  | L2 | FS |  | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TRAIN> <V_STATIC>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded. |  | O |  | JRU |  | No change on speed and distance information after the level transition.- |  |  | L2 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  | 1 |  |  | 4800.00 |  | L2 | FS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | N/A |  | NP | - |  |
|  | 121 |  | 2 |  |  | 4800.00 |  | N/A | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 6000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 2 d 1/2 - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 300 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_DIFF | 2 | 0 d |  |  |
| NC_CDDIFF | 4 | 0111 b |  |  |
| V_DIFF | 7 | 16 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 2 d 2/2 - Distance 405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 65 : Balise Group BG 3 d 1/2 - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 97 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 300 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 3 d |  |  |
| Q_DIFF | 2 | 0 d |  |  |
| NC_CDDIFF | 4 | 0101 b |  |  |
| V_DIFF | 7 | 8 d |  |  |
| Q_DIFF | 2 | 0 d |  |  |
| NC_CDDIFF | 4 | 0110 b |  |  |
| V_DIFF | 7 | 12 d |  |  |
| Q_DIFF | 2 | 0 d |  |  |
| NC_CDDIFF | 4 | 1000 b |  |  |
| V_DIFF | 7 | 24 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 65 : Balise Group BG 3 d 2/2 - Distance 1605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 85 : Balise Group BG 4 d 1/2 - Distance 2500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 300 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_DIFF | 2 | 1 |  |  |
| NC_DIFF | 4 | 0010 b |  |  |
| V_DIFF | 7 | 16 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 85 : Balise Group BG 4 d 2/2 - Distance 2505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 105 : Balise Group BG 5 d 1/2 - Distance 3600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 300 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_DIFF | 2 | 2 |  |  |
| NC_DIFF | 4 | 0010 b |  |  |
| V_DIFF | 7 | 40 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 105 : Balise Group BG 5 d 2/2 - Distance 3605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 113 : Balise Group BG 6 d 1/2 - Distance 4100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 113 : Balise Group BG 6 d 2/2 - Distance 4105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |