| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3120300_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3120300_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 19:37 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | MLT |

# Sequence description

3.12.3 Text Transmission

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | NP | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | N/A |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | FS | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 50.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 45 |  | 2 |  |  | 50.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 2: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a location condition. The end condition is limited to a time condition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 100.00 |  | L1 | FS |  | The "plain text message" is displayed |  | O |  | DMI |  | The end conditions will be evaluated. D = D_TEXTDISPLAY T = T0 The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED _____ X_TEXT=Test of FT3120300.2 |  |  | L1 |  | FS | - |  |
|  | 49 |  | 2 |  |  | 100.00 |  | L1 | FS |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 50 |  | 3 |  |  | 200.00 |  | L1 | FS |  | The "plain text message" is removed |  | O |  | DMI |  | T = T0 + T_TEXTDISPLAY The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED _____ T_TEXTDISPLAY=10s |  |  | L1 |  | FS | - |  |
|  | 51 |  | 4 |  |  | 200.00 |  | L1 | FS |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  | O |  | JRU |  | The deletion of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080414: 4.8.4: Accepted Information depending on the modes - Fixed Text Information Test Case 3: A fixed text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 250.00 |  | L1 | FS |  | One Balise Group message containing packet 76 is received |  | I |  | BTM |  | If in standby mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 53 |  | 2 |  |  | 250.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 6: Texts displayed on DMI, fixed text messages and the data entry dialogs are displayed in language selected by the driver |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 250.00 |  | L1 | FS |  | The driver selects "Settings" [MAIN] |  | I |  | DMI |  | Language=<LANGUAGE_A> |  |  | L1 |  | FS | - |  |
|  | 57 |  | 2 |  |  | 250.00 |  | L1 | FS |  | The driver modifies "Language" |  | I |  | DMI |  | Selection of <LANGUAGE_B> |  |  | L1 |  | FS | - |  |
|  | 58 |  | 3 |  |  | 250.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 59 |  | 4 |  |  | 350.00 |  | L1 | FS |  | The "fixed text message" is displayed |  | O |  | DMI |  | The FIXED text messages is displayed in <LANGUAGE_B> |  |  | L1 |  | FS | - |  |
|  | 60 |  | 5 |  |  | 400.00 |  | L1 | FS |  | The driver modifies "Language" |  | I |  | DMI |  | Selection of <LANGUAGE_A> |  |  | L1 |  | FS | - |  |
|  | 61 |  | 6 |  |  | 400.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 62 |  | 7 |  |  | 400.00 |  | L1 | FS |  | The "fixed text message" is displayed |  | O |  | DMI |  | The FIXED text messages is displayed in <LANGUAGE_A> |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 1 |  |  | 450.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 64 |  | 2 |  |  | 450.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 7: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. Only one of the Start / End conditions shall be fulfilled to display the message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  | 1 |  |  | 500.00 |  | L1 | FS |  | The "plain text message" is displayed |  | O |  | DMI |  | It is not necessary to switch to OS to display the message When distance condition is fulfilled The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 68 |  | 2 |  |  | 500.00 |  | L1 | FS |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 69 |  | 3 |  |  | 600.00 |  | L1 | FS |  | The "plain text message" is removed |  | O |  | DMI |  | It is not necessary to leave FS to remove the message When the L_TEXTDISPLAY is covered The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED |  |  | L1 |  | FS | - |  |
|  | 70 |  | 4 |  |  | 600.00 |  | L1 | FS |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  | O |  | JRU |  | The deletion of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 1 |  |  | 650.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 72 |  | 2 |  |  | 650.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 8: The on-board equipment has received information about plain text messages. The plain text message is not displayed if the end condition is immediately fulfilled. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  | 1 |  |  | 650.00 |  | L1 | FS |  | No "plain text message" is displayed |  | O |  | DMI |  | As soon as D = D_TEXTDISPLAY (third start condition is fulfilled), the end conditions (L_TEXTDISPLAY=0) will be evaluated and the plain text message ‘X_TEXT(L_TEXT)’ is NOT DISPLAYED |  |  | L1 |  | FS | - |  |
|  | Feature 4080414: 4.8.4: Accepted Information depending on the modes - Fixed Text Information Test Case 3: A fixed text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  | 1 |  |  | 700.00 |  | L1 | FS |  | One Balise Group message containing packet 76 is received |  | I |  | BTM |  | If in standby mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 77 |  | 2 |  |  | 700.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 13: The on-board equipment has received information about fixed text messages. The text message requires EB application when end conditions are reached and the message has not been confirmed by the driver. The text message is displayed according to the display conditions. As the message is not acknowledged when the end conditions are reached, the EB is applied. As soon as the driver confirms the message, the EB is released and the message is removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  | 1 |  |  | 750.00 |  | L1 | FS |  | The "fixed text message" is displayed |  | O |  | DMI |  | If train front end at location of LRBG+ D_TEXTDISPLAY D = location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 81 |  | 2 |  |  | 750.00 |  | L1 | FS |  | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 82 |  | 3 |  |  | 850.00 |  | L1 | FS |  | Emergency brake commanded |  | O |  | TIU |  | As soon as end conditions are reached. D = location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The EB is APPLIED |  |  | L1 |  | FS | - |  |
|  | 83 |  | 4 |  |  | 850.00 |  | L1 | FS |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | As soon as end conditions are reached. The EB application is RECORDED |  |  | L1 |  | FS | - |  |
|  | 84 |  | 5 |  |  | 850.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | As soon as end conditions are reached. The EB application is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 85 |  | 6 |  |  | 900.00 |  | L1 | FS |  | The driver acknowledges |  | I |  | DMI |  | The driver confirms the message Before standstill |  |  | L1 |  | FS | - |  |
|  | 86 |  | 7 |  |  | 900.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 87 |  | 8 |  |  | 900.00 |  | L1 | FS |  | Emergency brake not commanded |  | O |  | TIU |  | As soon as the driver confirms the message The EB is RELEASED |  |  | L1 |  | FS | - |  |
|  | 88 |  | 9 |  |  | 900.00 |  | L1 | FS |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | As soon as the driver confirms the message The EB revocation is RECORDED |  |  | L1 |  | FS | - |  |
|  | 89 |  | 10 |  |  | 900.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | As soon as the driver confirms the message The message is REMOVED The EB application is NOT DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 90 |  | 11 |  |  | 900.00 |  | L1 | FS |  | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded |  | O |  | JRU |  | The removal of the text message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080414: 4.8.4: Accepted Information depending on the modes - Fixed Text Information Test Case 3: A fixed text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 91 |  | 1 |  |  | 1000.00 |  | L1 | FS |  | One Balise Group message containing packet 76 is received |  | I |  | BTM |  | If in standby mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 92 |  | 2 |  |  | 1000.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 16: The on-board equipment has received information about fixed text messages. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. The text message is displayed according to the display conditions. As the message is not acknowledged when the end conditions are reached, the SB is applied. As soon as the driver confirms the message, the SB is released and the message is removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  | 1 |  |  | 1050.00 |  | L1 | FS |  | The "fixed text message" is displayed |  | O |  | DMI |  | If train front end at location of LRBG+ D_TEXTDISPLAY D = location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 96 |  | 2 |  |  | 1050.00 |  | L1 | FS |  | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 97 |  | 3 |  |  | 1150.00 |  | L1 | FS |  | Service brake commanded |  | O |  | TIU |  | As soon as end conditions are reached. D = location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The SB is APPLIED |  |  | L1 |  | FS | - |  |
|  | 98 |  | 4 |  |  | 1150.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | As soon as end conditions are reached. The SB application is RECORDED |  |  | L1 |  | FS | - |  |
|  | 99 |  | 5 |  |  | 1150.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | As soon as end conditions are reached. The SB application is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 100 |  | 6 |  |  | 1200.00 |  | L1 | FS |  | The driver acknowledges |  | I |  | DMI |  | The driver confirms the message Before standstill |  |  | L1 |  | FS | - |  |
|  | 101 |  | 7 |  |  | 1200.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 102 |  | 8 |  |  | 1200.00 |  | L1 | FS |  | Service brake not commanded |  | O |  | TIU |  | As soon as the driver confirms the message The SB is RELEASED |  |  | L1 |  | FS | - |  |
|  | 103 |  | 9 |  |  | 1200.00 |  | L1 | FS |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | As soon as the driver confirms the message The SB revocation is RECORDED |  |  | L1 |  | FS | - |  |
|  | 104 |  | 10 |  |  | 1200.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | As soon as the driver confirms the message The message is REMOVED The SB application is NOT DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 105 |  | 11 |  |  | 1200.00 |  | L1 | FS |  | The "fixed text message" is removed |  | O |  | DMI |  | As soon as the message is confirmed by the driver The text message is REMOVED |  |  | L1 |  | FS | - |  |
|  | 106 |  | 12 |  |  | 1200.00 |  | L1 | FS |  | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded |  | O |  | JRU |  | The removal of the text message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 107 |  | 1 |  |  | 1350.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 108 |  | 2 |  |  | 1350.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 17: The on-board equipment has received information about a fixed or plain text message. The text message requires to be confirmed by the driver. The text message is displayed according to the display conditions. A newer message of the same class which requires no driver confirmation is received and displayed but it is hidden behind the previous one. Another message to be confirmed is received and displayed. The third message is hidden until the first one is confirmed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  |  | 1350.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | When start conditions are fulfilled: The first text message is DISPLAYED This message should not be confirmed yet. |  |  | L1 |  | FS | - |  |
|  | 112 |  | 2 |  |  | 1350.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 113 |  | 3 |  |  | 1400.00 |  | L1 | FS |  | One Balise Group message containing packet 72 (Q_TEXTCONFIRM=0) is received |  | I |  | BTM |  | If fixed text message is used, packet 76 is sent instead of packet 72. A newer message of the same class as the previous one which requires no driver confirmation is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 00 or 01 (Same as in start conditions) Q_TEXTCONFIRM = 00 (No confirmation required) End condition shall be not restrictive to produce the displaying of this text message after the other two have been confirmed. |  |  | L1 |  | FS | - |  |
|  | 114 |  | 4 |  |  | 1400.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCONFIRM=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 116 |  | 6 |  |  | 1400.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | The second message is hidden behind the first one. |  |  | L1 |  | FS | - |  |
|  | 117 |  | 7 |  |  | 1450.00 |  | L1 | FS |  | One Balise Group message containing packet 72 (Q_TEXTCONFIRM=1) is received |  | I |  | BTM |  | If fixed text message is used, packet 76 is sent instead of packet 72. Another message to be confirmed is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 00 or 01 (Same as in start conditions) Q_TEXTCONFIRM = 01 (Confirmation Required) All the other end conditions shall be not restrictive. |  |  | L1 |  | FS | - |  |
|  | 118 |  | 8 |  |  | 1450.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCONFIRM=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 119 |  | 9 |  |  | 1450.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | The third message is hidden behind the first one |  |  | L1 |  | FS | - |  |
|  | 120 |  | 10 |  |  | 1500.00 |  | L1 | FS |  | The driver acknowledges "plain or fixed text message" |  | I |  | DMI |  | The driver confirms the first message |  |  | L1 |  | FS | - |  |
|  | 121 |  | 11 |  |  | 1500.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 122 |  | 12 |  |  | 1500.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | The first message is REMOVED |  |  | L1 |  | FS | - |  |
|  | 123 |  | 13 |  |  | 1500.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 124 |  | 14 |  |  | 1500.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | As soon as the driver confirms the first message The third message appears. |  |  | L1 |  | FS | - |  |
|  | 125 |  | 15 |  |  | 1500.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the third message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 126 |  | 16 |  |  | 1550.00 |  | L1 | FS |  | The driver acknowledges "plain or fixed text message" |  | I |  | DMI |  | The driver confirms the third message |  |  | L1 |  | FS | - |  |
|  | 127 |  | 17 |  |  | 1550.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the third message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 128 |  | 18 |  |  | 1550.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | The third message is REMOVED |  |  | L1 |  | FS | - |  |
|  | 129 |  | 19 |  |  | 1550.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the third message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 130 |  | 20 |  |  | 1550.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | The second message appears |  |  | L1 |  | FS | - |  |
|  | 131 |  | 21 |  |  | 1550.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the second message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 132 |  | 22 |  |  | 1650.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | When the end conditions of the second message are fulfilled The second message is removed If the other messages have not been confirmed before the end conditions of the second message this one may not be presented. |  |  | L1 |  | FS | - |  |
|  | 133 |  | 23 |  |  | 1650.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the second message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 |  | 1 |  |  | 1700.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 135 |  | 2 |  |  | 1700.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 18: The on-board equipment has received information about fixed or plain text messages. The text message regards auxiliary information and requires to be confirmed by the driver. The text message is displayed according to the display conditions. A newer message regarding important information which requires no driver confirmation is received and is not displayed although it regards important information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 138 |  | 1 |  |  | 1700.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | When start conditions are fulfilled: The first text message is DISPLAYED This message should not be confirmed yet. |  |  | L1 |  | FS | - |  |
|  | 139 |  | 2 |  |  | 1700.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 140 |  | 3 |  |  | 1750.00 |  | L1 | FS |  | One Balise Group message containing packet 72 (Q_TEXTCLASS=1; Q_TEXTCONFIRM=0) is received |  | I |  | BTM |  | If fixed text message is used, packet 76 is sent instead of packet 72. A newer message of a higher class than the previous one which requires no driver confirmation is received and its start conditions to be displayed are fulfilled Q_TEXTCLASS = 01 (Important information) Q_TEXTCONFIRM = 00 (No confirmation required) End condition should not exceed the conditions of message one |  |  | L1 |  | FS | - |  |
|  | 141 |  | 4 |  |  | 1750.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=72; Q_TEXTCLASS=1; Q_TEXTCONFIRM=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 143 |  | 6 |  |  | 1750.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | The second message is hidden behind the first one. |  |  | L1 |  | FS | - |  |
|  | 144 |  | 7 |  |  | 1850.00 |  | L1 | FS |  | The driver acknowledges "plain or fixed text message" |  | I |  | DMI |  | The driver confirms the first message |  |  | L1 |  | FS | - |  |
|  | 145 |  | 8 |  |  | 1850.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 146 |  | 9 |  |  | 1850.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | The first message is REMOVED |  |  | L1 |  | FS | - |  |
|  | 147 |  | 10 |  |  | 1850.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the first message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 1950.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 149 |  | 2 |  |  | 1950.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 19: The on-board equipment has received information about fixed or plain text messages. The text message requires SB application when end conditions are reached and the message has not been confirmed by the driver. The text message requires driver acknowledgement and one or more events to be removed. The text message is displayed according to the display conditions. As the message is acknowledged and the end conditions are reached, the message is removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 152 |  | 1 |  |  | 2000.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | If train front end at location of LRBG+ D_TEXTDISPLAY D = location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 153 |  | 2 |  |  | 2000.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 154 |  | 3 |  |  | 2050.00 |  | L1 | FS |  | The driver acknowledges "plain or fixed text message" |  | I |  | DMI |  | Before reaching end conditions, the driver confirms the message |  |  | L1 |  | FS | - |  |
|  | 155 |  | 4 |  |  | 2050.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 156 |  | 5 |  |  | 2100.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | As soon as the end conditions are reached D = location of LRBG + D_TEXTDISPLAY + L_TEXTDISPLAY The message is REMOVED |  |  | L1 |  | FS | - |  |
|  | 157 |  | 6 |  |  | 2100.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is NOT displayed |  | O |  | DMI |  | The SB application is NOT DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 158 |  | 7 |  |  | 2100.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the text message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 159 |  | 1 |  |  | 2150.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 160 |  | 2 |  |  | 2150.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 163 |  | 1 |  |  | 2200.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 164 |  | 2 |  |  | 2200.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 167 |  | 1 |  |  | 2250.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 168 |  | 2 |  |  | 2250.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 20: The on-board equipment has received information about fixed or plain text messages: The first message to be displayed is an auxiliary information. The second message be displayed is an important information. The third message to be displayed is an auxiliary information. The first message is displayed. When the second message has to be displayed, it hides the first one. When the third message has to be displayed, it is hidden by the second one. When the second message is removed according to its end condition for display, the third message reappears. The appearance of the auxiliary information message is not the same than the appearance of the important information message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 171 |  | 1 |  |  | 2300.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | The N text message is DISPLAYED with the appearance of auxiliary information The number (N) of messages with auxiliary information will depend on the DMI configuration |  |  | L1 |  | FS | - |  |
|  | 172 |  | 2 |  |  | 2300.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the N message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 173 |  | 3 |  |  | 2350.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | A text message is DISPLAYED with the appearance of important information. The message is an important information and its start conditions to be displayed are fulfilled The first message is hidden. |  |  | L1 |  | FS | - |  |
|  | 174 |  | 4 |  |  | 2350.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the text message with the appearance of important information is RECORDED. |  |  | L1 |  | FS | - |  |
|  | 175 |  | 5 |  |  | 2400.00 |  | L1 | FS |  | The "plain or fixed text message" is NOT displayed |  | O |  | DMI |  | A new message is hidden behind the second one. The new message is an auxiliary information and its start conditions to be displayed are fulfilled |  |  | L1 |  | FS | - |  |
|  | 176 |  | 6 |  |  | 2450.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | When the end conditions of the text message with the appearance of important information are reached, the message is REMOVED n |  |  | L1 |  | FS | - |  |
|  | 177 |  | 7 |  |  | 2450.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | …and the last message is DISPLAYED with the appearance of auxiliary information |  |  | L1 |  | FS | - |  |
|  | 178 |  | 8 |  |  | 2450.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the last message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 179 |  | 1 |  |  | 2550.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 180 |  | 2 |  |  | 2550.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 14: The on-board equipment has received information about fixed or plain text messages. The text message requires driver acknowledgement to be removed. The text message is displayed according to the display conditions. As the message is acknowledged, the message is removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 183 |  | 1 |  |  | 2600.00 |  | L1 | FS |  | The "plain or fixed text message" is displayed |  | O |  | DMI |  | If train front end at location of LRBG+ D_TEXTDISPLAY D = location of LRBG + D_TEXTDISPLAY The text message is DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 184 |  | 2 |  |  | 2600.00 |  | L1 | FS |  | START DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=16¦18) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 185 |  | 3 |  |  | 2650.00 |  | L1 | FS |  | The driver acknowledges "plain or fixed text message" |  | I |  | DMI |  | - |  |  | L1 |  | FS | - |  |
|  | 186 |  | 4 |  |  | 2650.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | The confirmation of the message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 187 |  | 5 |  |  | 2650.00 |  | L1 | FS |  | The "plain or fixed text message" is removed |  | O |  | DMI |  | - |  |  | L1 |  | FS | - |  |
|  | 188 |  | 6 |  |  | 2650.00 |  | L1 | FS |  | The Status symbol "Service Brake or Emergency Brake intervention" is NOT displayed |  | O |  | DMI |  | The SB application is NOT DISPLAYED |  |  | L1 |  | FS | - |  |
|  | 189 |  | 7 |  |  | 2650.00 |  | L1 | FS |  | STOP DISPLAYING FIXED OR PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=17¦19) is recorded |  | O |  | JRU |  | The removal of the text message is RECORDED |  |  | L1 |  | FS | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 190 |  | 1 |  |  | 2750.00 |  | L1 | FS |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. |  |  | L1 |  | FS | - |  |
|  | 191 |  | 2 |  |  | 2750.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 46: On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode. Transition Condition Id [40]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 195 |  | 1 |  |  | 2800.00 |  | L1 | FS |  | One Balise Group message (N_TOTAL=1) containing packet 12 and packet 80 (M_MAMODE = 00) is received. |  | I |  | BTM |  | Only if in L1 Reception of the mode transition announcement (mode transition order at current location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On sight |  |  | L1 |  | FS | - |  |
|  | 196 |  | 2 |  |  | 2800.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET= 80) is recorded |  | O |  | JRU |  | Only if in L1 |  |  | L1 |  | FS | - |  |
|  | 199 |  | 5 |  |  | 2800.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 23: On Sight mode is permanently displayed to the driver when the on-board equipment is in On Sight mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 201 |  | 1 |  |  | 2800.00 |  | L1 | OS |  | Permanent Indication of OS mode to the driver |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  | OS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 46: On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode. Transition Condition Id [40]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 202 |  | 7 |  |  | 2800.00 |  | L1 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | 203 |  | 8 |  |  | 2800.00 |  | L1 | OS |  | The Mode symbol 'Acknowledge On Sight' is displayed |  | O |  | DMI |  | DMI Object: MO08 |  |  | L1 |  | OS | - |  |
|  | 204 |  | 9 |  |  | 2800.00 |  | L1 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | 207 |  | 12 |  |  | 2800.00 |  | L1 | OS |  | The driver acknowledges 'On Sight' |  | I |  | DMI |  | Driver acknowledges within 5 sec after the change to OS The driver acknowledgement time (refer to Appendix A3.1) |  |  | L1 |  | OS | - |  |
|  | 208 |  | 13 |  |  | 2800.00 |  | L1 | OS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | 209 |  | 14 |  |  | 2850.00 |  | L1 | OS |  | Service brake not commanded |  | O |  | TIU |  | Negative Testing |  |  | L1 |  | OS | - |  |
|  | 210 |  | 15 |  |  | 2850.00 |  | L1 | OS |  | NOT The Status symbol 'Service Brake intervention' is displayed |  | O |  | DMI |  | Negative Testing |  |  | L1 |  | OS | - |  |
|  | 211 |  | 16 |  |  | 2850.00 |  | L1 | OS |  | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | Negative Testing |  |  | L1 |  | OS | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 5: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. All of the Start / End conditions shall be fulfilled to display the message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 212 |  | 2 |  |  | 2950.00 |  | L1 | OS |  | The "plain text message" is displayed |  | O |  | DMI |  | All start conditions are fulfilled The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED D1=D0+D_TEXTDISPLAY (D0 is the location reference of the BG transmitting the text information or of the LRBG given in the message transmitting the text information) |  |  | L1 |  | OS | - |  |
|  | 213 |  | 3 |  |  | 2950.00 |  | L1 | OS |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | The display of the message is RECORDED |  |  | L1 |  | OS | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 12: The train receives an order to switch immediately from L1 to L0 at the current position. on-board equipment immediately switches to L0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 216 |  | 1 |  |  | 3000.00 |  | L1 | OS |  | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 |  | I |  | BTM |  | - |  |  | L1 |  | OS | - |  |
|  | 217 |  | 2 |  |  | 3000.00 |  | L1 | OS |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | 218 |  | 3 |  |  | 3000.00 |  | L1 | OS |  | Level 0 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 0. |  |  | L0 |  | OS | - |  |
|  | 219 |  | 4 |  |  | 3000.00 |  | L0 | OS |  | New Level 0 is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  | OS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 3: ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 221 |  | 3 |  |  | 3000.00 |  | L0 | OS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | 222 |  | 4 |  |  | 3000.00 |  | L0 | UN |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object: MO16 |  |  | L0 |  | UN | - |  |
|  | 223 |  | 5 |  |  | 3000.00 |  | L0 | UN |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 6: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 224 |  | 1 |  |  | 3000.00 |  | L0 | UN |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  | UN | - |  |
|  | 225 |  | 2 |  |  | 3000.00 |  | L0 | UN |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | 226 |  | 3 |  |  | 3000.00 |  | L0 | UN |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | L0 |  | UN | - |  |
|  | 227 |  | 4 |  |  | 3000.00 |  | L0 | UN |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | 228 |  | 5 |  |  | 3000.00 |  | L0 | UN |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  | UN | - |  |
|  | 229 |  | 6 |  |  | 3000.00 |  | L0 | UN |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 5: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. All of the Start / End conditions shall be fulfilled to display the message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 230 |  | 5 |  |  | 3150.00 |  | L0 | UN |  | The "plain text message" is removed |  | O |  | DMI |  | When the L_TEXTDISPLAY is covered The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED D2=D0+D_TEXTDISPLAY+L_TEXTDISPLAY (D0 is the location reference of the BG transmitting the text information or of the LRBG given in the message transmitting the text information) |  |  | L0 |  | UN | - |  |
|  | 231 |  | 6 |  |  | 3150.00 |  | L0 | UN |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  | O |  | JRU |  | The deletion of the message is RECORDED |  |  | L0 |  | UN | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 232 |  | 1 |  |  | 3200.00 |  | L0 | UN |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. |  |  | L0 |  | UN | - |  |
|  | 233 |  | 2 |  |  | 3200.00 |  | L0 | UN |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | L0 |  | UN | - |  |
|  | 234 |  | 3 |  |  | 3200.00 |  | L0 | UN |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L0 |  | UN | - |  |
|  | 235 |  | 4 |  |  | 3200.00 |  | L0 | UN |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 236 |  | 1 |  |  | 3200.00 |  | L0 | UN |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L0 |  | UN | - |  |
|  | 237 |  | 2 |  |  | 3200.00 |  | L0 | UN |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 15: The train receives an order to switch immediately from L0 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 240 |  | 1 |  |  | 3300.00 |  | L0 | UN |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa : Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR=0 M_LEVELTR=2 |  | I |  | BTM |  | - |  |  | L0 |  | UN | - |  |
|  | 241 |  | 2 |  |  | 3300.00 |  | L0 | UN |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED |  | O |  | JRU |  | - |  |  | L0 |  | UN | - |  |
|  | 242 |  | 3 |  |  | 3300.00 |  | L0 | UN |  | Level 1 is DISPLAYED |  | O |  | DMI |  | (Except in SL mode – desk is closed) The on-board system switches to Level 1 |  |  | L1 |  | UN | - |  |
|  | 243 |  | 4 |  |  | 3300.00 |  | L1 | UN |  | New Level L1 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 244 |  | 1 |  |  | 3350.00 |  | L1 | FS |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  | FS | - |  |
|  | 245 |  | 2 |  |  | 3350.00 |  | L1 | FS |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 246 |  | 3 |  |  | 3350.00 |  | L1 | FS |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  | FS | - |  |
|  | 247 |  | 4 |  |  | 3350.00 |  | L2 | FS |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 8: The driver closes the desk. The on-board equipment immediately switches to SB mode and the end of mission procedure is triggered (there is a radio link with RBC). Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 248 |  | 1 |  |  | 3400.00 |  | L2 | FS |  | CAB not active |  | I |  | TIU |  | The driver closed the desk. The on-board equipment switches immediately to SB mode. _____ The train reaches standstill. |  |  | L2 |  | FS | - |  |
|  | 249 |  | 2 |  |  | 3400.00 |  | L2 | FS |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | FS | - |  |
|  | 250 |  | 3 |  |  | 3400.00 |  | L2 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 252 |  | 1 |  |  | 3400.00 |  | L2 | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | L2 |  | SB | - |  |
|  | 253 |  | 2 |  |  | 3400.00 |  | L2 | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 254 |  | 3 |  |  | 3400.00 |  | L2 | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 255 |  | 1 |  |  | 3400.00 |  | L2 | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L2 |  | SB | - |  |
|  | 256 |  | 2 |  |  | 3400.00 |  | L2 | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L2 |  | SB | - |  |
|  | 257 |  | 3 |  |  | 3400.00 |  | L2 | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 34: During the SoM procedure, if the stored train position and level are valid (for example, when closing the desk and making the transition SR-SB) and if level is 2 or 3, and if no mobile terminal is registered to a radio network (D7), the ERTMS/ETCS on-board equipment shall wait a sufficient time is elapsed since it has sent the latest Radio Network registration order to a Mobile Terminal (S4); in such case the driver shall be informed (A29) and the process shall go to S10. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 259 |  | 2 |  |  | 3400.00 |  | L2 | SB |  | The Main window is displayed. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 260 |  | 3 |  |  | 3400.00 |  | L2 | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit42=1>) is recorded |  | O |  | DMI |  | The Main window is presented to the driver with all buttons ‘disabled’. The symbol ST05 is displayed to show that the onboard still awaits the registration to the Radio Network. |  |  | L2 |  | SB | - |  |
|  | 261 |  | 4 |  |  | 3400.00 |  | L2 | SB |  | NOT SA-REGISTRATION.Indication containing a non-empty MNID list is received |  | I |  | RTM |  | After the "Time from the latest Radio Network registration order to a Mobile Terminal after which the registration is considered as failed.", "SA-REGISTRATION.Indication" is not received or is received with an empty MNID list. |  |  | L2 |  | SB | - |  |
|  | 262 |  | 5 |  |  | 3400.00 |  | L2 | SB |  | The system status message ‘Radio network registration failed’ is DISPLAYED. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 263 |  | 6 |  |  | 3400.00 |  | L2 | SB |  | The 'DMI SYSTEM STATUS MESSAGE' message is RECORDED with the Bit 23 (Radio network registration failed) = 1 (displayed) for variable SYSTEM_STATUS_MESSAGE. |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 264 |  | 1 |  |  | 3400.00 |  | L2 | SB |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train remains standstill. |  |  | L2 |  | NP | - |  |
|  | 265 |  | 2 |  |  | 3400.00 |  | L2 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 0 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 244 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 50 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 10 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 19 d |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 2 |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 0 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 00000000000052 1/2 - Distance 250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 76 d | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001011100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 000000001100100 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000001100100 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| Q_TEXT | 8 | 00000000 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 00000000000052 2/2 - Distance 255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 00000000000063 1/2 - Distance 450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011110100 b |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 000000111110100 b |  |  |
| M_MODETEXTDISPLAY | 4 | 0001 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 011 b |  |  |
| L_TEXTDISPLAY | 15 | 000001111101000 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 0000 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 010 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 19 d |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01101111 b |  |  |
| X_TEXT | 8 | 01100110 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110111 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 63 : Balise Group BG 00000000000063 2/2 - Distance 455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 71 : Balise Group BG 00000000000071 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011110100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 0000 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 010 b |  |  |
| L_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 19 d |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01101111 b |  |  |
| X_TEXT | 8 | 01100110 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00111000 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 71 : Balise Group BG 00000000000071 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 76 : Balise Group BG 00000000000176 1/2 - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001100 b | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001011110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000110010 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000001100100 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 11 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| Q_TEXT | 8 | 00000000 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 76 : Balise Group BG 00000000000176 2/2 - Distance 705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 91 : Balise Group BG 00000000000191 1/2 - Distance 1000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000101 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001100 b | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001011110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000110010 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000001100100 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 10 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| Q_TEXT | 8 | 00000000 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 91 : Balise Group BG 00000000000191 2/2 - Distance 1005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000101 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 107 : Balise Group BG 000000000001107 1/2 - Distance 1350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000110 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000100011110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 111111111111111 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 01 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| L_TEXT | 8 | 24 d |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110111 b |  |  |
| X_TEXT | 8 | 00111010 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01101001 b |  |  |
| X_TEXT | 8 | 01110010 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01111000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 107 : Balise Group BG 000000000001107 2/2 - Distance 1355.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000110 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 113 : Balise Group BG 000000000001113 1/2 - Distance 1400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000111 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000100001100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000011111010 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 22 d |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110111 b |  |  |
| X_TEXT | 8 | 00111010 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 01101110 b |  |  |
| X_TEXT | 8 | 01100100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01111000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 113 : Balise Group BG 000000000001113 2/2 - Distance 1405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000111 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 117 : Balise Group BG 000000000010117 1/2 - Distance 1450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001000 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000100001110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 111111111111111 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 01 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| L_TEXT | 8 | 22 d |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110111 b |  |  |
| X_TEXT | 8 | 00111010 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 01110010 b |  |  |
| X_TEXT | 8 | 01100100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01111000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 117 : Balise Group BG 000000000010117 2/2 - Distance 1455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001000 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 134 : Balise Group BG 000000000010134 1/2 - Distance 1700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001001 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000100011110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 111111111111111 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 01 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| L_TEXT | 8 | 24 d |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00111000 b |  |  |
| X_TEXT | 8 | 00111010 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01101001 b |  |  |
| X_TEXT | 8 | 01110010 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01111000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 134 : Balise Group BG 000000000010134 2/2 - Distance 1705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001001 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 140 : Balise Group BG 000000000010140 1/2 - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000100001100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 01 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000000000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000000110010 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 22 d |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00111000 b |  |  |
| X_TEXT | 8 | 00111010 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 01101110 b |  |  |
| X_TEXT | 8 | 01100100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01111000 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 140 : Balise Group BG 000000000010140 2/2 - Distance 1755.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 148 : Balise Group BG 11 d 1/2 - Distance 1950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 254 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 50 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 100 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 10 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 1 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| L_TEXT | 8 | 20 d |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 9 |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 148 : Balise Group BG 11 d 2/2 - Distance 1955.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 159 : Balise Group BG 12 d 1/2 - Distance 2150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 572 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 150 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 200 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 60 d |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | : |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | - |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | y |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | m |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | . |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 159 : Balise Group BG 12 d 2/2 - Distance 2155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 163 : Balise Group BG 13 d 1/2 - Distance 2200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 788 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 01 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 150 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 100 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 87 d |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | : |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | - |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | m |  |  |
| X_TEXT | 8 | p |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | m |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | y |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | . |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 163 : Balise Group BG 13 d 2/2 - Distance 2205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 167 : Balise Group BG 14 d 1/2 - Distance 2250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 14 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 572 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 150 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 100 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 60 d |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | : |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | - |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | y |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | m |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | . |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 167 : Balise Group BG 14 d 2/2 - Distance 2255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 14 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 179 : Balise Group BG 000000000011179 1/2 - Distance 2550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001111 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011111110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000000110010 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 000000001100100 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 10 b |  |  |
| Q_CONFTEXTDISPLAY | 1 | 0 b |  |  |
| Q_TEXTREPORT | 1 | 0 b |  |  |
| L_TEXT | 8 | 20 d |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01101111 b |  |  |
| X_TEXT | 8 | 01100110 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 179 : Balise Group BG 000000000011179 2/2 - Distance 2555.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001111 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 190 : Balise Group BG 000000000100190 1/2 - Distance 2750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000010000 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011110100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 000000011001000 b |  |  |
| M_MODETEXTDISPLAY | 4 | 0001 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 010 b |  |  |
| L_TEXTDISPLAY | 15 | 000000011001000 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 0001 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 010 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 19 d |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01110011 b |  |  |
| X_TEXT | 8 | 01110100 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01101111 b |  |  |
| X_TEXT | 8 | 01100110 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01000110 b |  |  |
| X_TEXT | 8 | 01010100 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00110010 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110011 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00110000 b |  |  |
| X_TEXT | 8 | 00101110 b |  |  |
| X_TEXT | 8 | 00110101 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 190 : Balise Group BG 000000000100190 2/2 - Distance 2755.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000010000 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 195 : Balise Group BG 17 d 1/2 - Distance 2800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 17 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1000 d |  |  |
| L_ACKMAMODE | 15 | 50 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 195 : Balise Group BG 17 d 2/2 - Distance 2805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 17 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 216 : Balise Group BG 000000000100216 1/2 - Distance 3000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000010010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00101001 b | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 000000000000000 b |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 216 : Balise Group BG 000000000100216 2/2 - Distance 3005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000010010 b |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 232 : Balise Group BG 000000011101232 1/2 - Distance 3200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001110110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 100 d |  |  |
| M_LEVELTR | 3 | 010 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 232 : Balise Group BG 000000011101232 2/2 - Distance 3205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001110110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 40 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 240 : Balise Group BG 000000011101240 1/2 - Distance 3300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001110111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 111111111111111 b |  |  |
| M_LEVELTR | 3 | 010 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 240 : Balise Group BG 000000011101240 2/2 - Distance 3305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001110111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 244 : Balise Group BG 000000011110244 1/2 - Distance 3350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001111000 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00101001 b | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 111111111111111 b |  |  |
| M_LEVELTR | 3 | 011 b |  |  |
| L_ACKLEVELTR | 15 | 000000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 244 : Balise Group BG 000000011110244 2/2 - Distance 3355.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001111000 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |