| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3130233_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3130233_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 20:15 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | CEDEX |

# Sequence description

The main objective is to test the TCs 1 and 3 of FT3130233. Some special values for the NVs are sent at 50m. A "Lambda 1" train is used for testing.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | SB | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] Level 1 is selected. |  |  | L1 |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 18 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 19 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 20 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 21 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 24 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 25 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 | SR |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ V_NVSTFF=80km/h V_NVALLOWOVTRP=80km/h V_NVSUPOVTRP=80km/h D_NVOVTRP=20m |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 5080300: 5.8.3: Once the 'Override' procedure has been triggered Test Case 4: The train is in SR mode and the driver selects Override EOA. The SR distance and speed defined by the National values (D_NVOVTRP and V_NVSUPOVTRP) substitute the previous values given by the trackside (V_NVSTFF, D_NVSTFF). When the Override status finishes, the driver checks that the Permitted Speed (PS) and Target Distance (TD) change to be in line with the default values for V_NVSTFF and D_NVSTFF. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 2 |  |  | 70.00 |  | L1 | SR |  | The driver requests to see the PS and TD. |  | I |  | DMI |  | If not yet available |  |  | L1 |  | SR | - |  |
|  | 49 |  | 8 |  |  | 100.00 |  | L1 | SR |  | The driver selects 'Override Request' before reaching the end of the SR area. |  | I |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 50 |  | 9 |  |  | 100.00 |  | L1 | SR |  | The driver’s action is recorded. |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 51 |  | 10 |  |  | 100.00 |  | L1 | SR |  | The Override status is displayed. |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 53 |  | 12 |  |  | 120.00 |  | L1 | SR |  | The Override status finishes. |  | O |  | DMI |  | The override status finishes because the D_NVOVTRP is passed or T_NVOVTRP expires. |  |  | L1 |  | SR | - |  |
|  | Feature 3130233: 3.13.2.3.3: Gradients Test Case 3: Use gradient = 0 when no default gradient for TSR profile in the determination of permitted speed supervision limits for target speed monitoring |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 55 |  | 1 |  |  | 120.00 |  | L1 | SR |  | Train is moving (V_TRAIN=<V_PERMITTED-20>) |  | I |  | INT |  | - |  |  | L1 |  | SR | - |  |
|  | 57 |  | 3 |  |  | 300.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=0) containing , packet 65 (Q_DIR=1; V_TSR=<Vmax>; L_TSR=<L>; D_TSR=0) is received |  | I |  | BTM |  | Only in L1 |  |  | L1 |  | SR | - |  |
|  | 58 |  | 4 |  |  | 300.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only in L1 |  |  | L1 |  | SR | - |  |
|  | 61 |  | 7 |  |  | 350.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The DMI aspect is coherent with Normal Status in a CSM section. |  |  | L1 |  | SR | - |  |
|  | 62 |  | 8 |  |  | 482.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | When the train has passed with its estimated front end the location of the Indication Speed supervision limit, valid for the estimated speed and calculated from the SBD, then the supervision status changes to "Indication Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Indication Status in a Target Speed Monitoring section. _____ The comment above is wrong, since the on-board is following the supervision limits derived from the EBD, not from the SBD. In order to properly test this TC, the change from the Normal Status to the Indication Status, for example, has to be checked. The Indication supervision limit is 1318m from the start location of the TSR (50km/h) running at 60km/h, according to the ERA tool for the braking curves calculation (see Subset-076-6-3_3130233_02a.xls). |  |  | L1 |  | SR | - |  |
|  | 63 |  | 9 |  |  | 482.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=1; V_PERM =<V1>) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 1 (Indication Status) This location and V_PERM are compared to ERA tool calculation. |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 900.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | Feature 4080420: 4.8.4: Accepted Information depending on the modes - Linking Test Case 3: A telegram including packet 5 Linking Information is transmitted via balise group and is received by the onboard equipment. No linking data were received previously. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 900.00 |  | L1 | SR |  | One Balise Group message containing packet 5 is received |  | I |  | BTM |  | Complete message is accepted |  |  | L1 |  | SR | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 5: The Temporary Speed Restrictions shall only be revoked on request from the trackside. Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 1 |  |  | 900.00 |  | L1 | SR |  | Balise Group message (N_TOTAL=1) containing packet 66 (Temporary Speed Restriction Revocation) is received. |  | I |  | BTM |  | NID_TSR = NID_TSR (A) |  |  | L1 |  | SR | - |  |
|  | 67 |  | 2 |  |  | 900.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Reception of TSR Revocation order is attested |  |  | L1 |  | SR | - |  |
|  | 68 |  | 3 |  |  | 900.00 |  | L1 | SR |  | Train is moving |  | I |  | INT |  | The train is located in the area of TSR, given in packet 65. |  |  | L1 |  | SR | - |  |
|  | 71 |  | 6 |  |  | 900.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Permitted Speed is less restrictive immediately after the revocation of the TSR, i.e., the PS is higher than the previously received value of V_TSR. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. |  |  | L1 |  | SR | - |  |
|  | 72 |  | 7 |  |  | 900.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= < other than V_TSR>; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 3 |  |  | 900.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | FS | - |  |
|  | 75 |  | 4 |  |  | 900.00 |  | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 79 |  | 5 |  |  | 900.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3130233: 3.13.2.3.3: Gradients Test Case 1: Use of gradient profile in the determination of permitted speed supervision limit when it is derived from the SBD in a target speed monitoring section and M_rotating_nom is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  | 1 |  |  | 900.00 |  | L1 | FS |  | Train is moving (V_TRAIN=<V_PERMITTED-20>) |  | I |  | INT |  | - |  |  | L1 |  | FS | - |  |
|  | 82 |  | 3 |  |  | 1100.00 |  | L1 | FS |  | One Balise Group message (N_TOTAL=0) containing packet 5, packet 12 (Q_DIR=1; V_MAIN=<Vmax>; L_ENDSECTION=<L>; V_EMA=0), packet 27 (V_STATIC=<Vmax>), packet 21 (D_GRADIENT=0; Q_GDIR=1; G_A=<G>; N_ITER=0) is received |  | I |  | BTM |  | Only in L1 gradient <G> =HIGH UPHILL GRADIENT |  |  | L1 |  | FS | - |  |
|  | 83 |  | 4 |  |  | 1100.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only in L1 |  |  | L1 |  | FS | - |  |
|  | 86 |  | 7 |  |  | 1400.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The DMI aspect is coherent with Indication Status when braking to a target. |  |  | L1 |  | FS | - |  |
|  | 87 |  | 8 |  |  | 1626.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | When the train has passed with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the SBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. _____ The Overspeed supervision limit is 574m from the EoA running at 60km/h, according to the ERA tool for the braking curves calculation (see Subset-076-6-3_3130233_02b.xls). |  |  | L1 |  | FS | - |  |
|  | 88 |  | 9 |  |  | 1626.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM =<V1>) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only if not in LS mode: This location and V_PERM are compared to ERA tool calculation. Only if in LS mode: V_TRAIN will be used instead of V_PERM. So, this location and the current train speed are compared to ERA tool calculation |  |  | L1 |  | FS | - |  |
|  | 89 |  | 10 |  |  | 1626.00 |  | L1 | FS |  | Train is moving (V_TRAIN=<V_PERMITTED-20>) |  | I |  | INT |  | - |  |  | L1 |  | FS | - |  |
|  | 90 |  | 11 |  |  | 1700.00 |  | L1 | FS |  | One Balise Group message (N_TOTAL=0) containing packet 5, packet 12 (Q_DIR=1; V_MAIN=<Vmax>; L_ENDSECTION=<L>; V_EMA=0), packet 27 (V_STATIC=<Vmax>), packet 21 (D_GRADIENT=0; Q_GDIR=0; G_A=<G>; N_ITER=0) is received |  | I |  | BTM |  | Only in L1 gradient <G> =HIGH DOWNHILL GRADIENT |  |  | L1 |  | FS | - |  |
|  | 91 |  | 12 |  |  | 1700.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only in L1 |  |  | L1 |  | FS | - |  |
|  | 94 |  | 15 |  |  | 2100.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The DMI aspect is coherent with Indication Status when braking to a target. |  |  | L1 |  | FS | - |  |
|  | 95 |  | 16 |  |  | 2335.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | When the train has passed with its estimated front end the location of the Permitted Speed supervision limit, valid for the estimated speed and calculated from the SBD, then the supervision status changes to "Overspeed Status" (t4 in 3.13.10.4.10 table 9 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Target Speed Monitoring section. _____ The Overspeed supervision limit is 865m from the EoA running at 60km/h, according to the ERA tool for the braking curves calculation (see Subset-076-6-3_3130233_02c.xls). |  |  | L1 |  | FS | - |  |
|  | 96 |  | 17 |  |  | 2335.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=2; M_SDMSUPSTAT=2; V_PERM =<V2>) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 2 (Target speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) Only if not in LS mode: This location and V_PERM are compared to ERA tool calculation. Only if in LS mode: V_TRAIN will be used instead of V_PERM. So, this location and the current train speed are compared to ERA tool calculation |  |  | L1 |  | FS | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 2550.00 |  | L1 | FS |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  | FS | - |  |
|  | 98 |  | 2 |  |  | 2550.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 1 |  |  | 2850.00 |  | L1 | FS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  | NP | - |  |
|  | 101 |  | 2 |  |  | 2850.00 |  | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 286 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 16 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 16 d |  |  |
| V_NVSUPOVTRP | 7 | 16 d |  |  |
| D_NVOVTRP | 15 | 20 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 1 |  |  |
| Q_NVKVINTSET | 2 | 0 b |  |  |
| V_NVKVINT | 7 | 0 d |  |  |
| M_NVKVINT | 7 | 48 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_NVKRINT | 5 | 0 d |  |  |
| M_NVKRINT | 5 | 20 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| L_NVKRINT | 5 | 10 d |  |  |
| M_NVKRINT | 5 | 18 d |  |  |
| M_NVKTINT | 5 | 20 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 57 : Balise Group BG 2 d 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 0 d |  |  |
| D_TSR | 15 | 1500 d |  |  |
| L_TSR | 15 | 200 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 10 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 57 : Balise Group BG 2 d 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 64 : Balise Group BG 3 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 16 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 147 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 200 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| D_LINK | 15 | 600 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| D_LINK | 15 | 850 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 64 : Balise Group BG 3 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 66 d | Temporary speed restriction revocation |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 31 d |  |  |
| NID_TSR | 8 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 82 : Balise Group BG 4 d 1/2 - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 95 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 16 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1100 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 1 |  |  |
| D_DP | 15 | 500 d |  |  |
| V_RELEASEDP | 7 | 6 d |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 1 b |  |  |
| G_A | 8 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 108 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 600 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_LINK | 15 | 850 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 82 : Balise Group BG 4 d 2/2 - Distance 1105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 90 : Balise Group BG 5 d 1/2 - Distance 1700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 95 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 16 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 1 |  |  |
| D_DP | 15 | 800 d |  |  |
| V_RELEASEDP | 7 | 6 d |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 69 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 850 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 90 : Balise Group BG 5 d 2/2 - Distance 1705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 97 : Balise Group BG 6 d 1/2 - Distance 2550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 97 : Balise Group BG 6 d 2/2 - Distance 2555.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |