| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3060300_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3060300_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 17:32 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | RINA |

# Sequence description

FT3060300: Validity direction of transmitted Information

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | NP | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | NP | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | NP |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | NP | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | NP |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 50: Following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level 2/3" and enters "RBC Data" |  | I |  | DMI |  | The status of the ERTMS/ETCS Level is set to valid. The status of the RBC ID/Phone Number is set to valid. _____ The Driver selects L2 [19.-1.1] The Driver enters: - RBC ID = 1050091 (NID_C=64 and NID_RBC=1515) - RBC phone number = 003265342101 (NID_RADIO) |  |  | N/A |  | SB | - |  |
|  | 14 |  | 5 |  |  | 0.00 |  | N/A | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<36¦37>) is recorded |  | O |  | JRU |  | The driver selects Level |  |  | L2 |  | SB | - |  |
|  | 15 |  | 6 |  |  | 0.00 |  | L2 | SB |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The driver enters "RBC Data" |  |  | L2 |  | SB | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L2 | SB |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  |  | L2 |  | SB | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L2 | SB |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  |  | L2 |  | SB | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L2 | SB |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L2 | SB |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 24 |  | 7 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  | SB | - |  |
|  | 25 |  | 8 |  |  | 0.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 26 |  | 9 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 27 |  | 10 |  |  | 0.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 28 |  | 11 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 29 |  | 12 |  |  | 0.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 50: Following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 9 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Train Accepted" (NID_MESSAGE=41) is received |  | I |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 31 |  | 10 |  |  | 0.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=41) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 32 |  | 11 |  |  | 0.00 |  | L2 | SB |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | If the status of the Train Position is invalid, it is set to unknown |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 0.00 |  | L2 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L2 |  | SB | - |  |
|  | 44 |  | 2 |  |  | 0.00 |  | L2 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 45 |  | 3 |  |  | 0.00 |  | L2 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 46 |  | 4 |  |  | 0.00 |  | L2 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 0.00 |  | L2 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L2 |  | SB | - |  |
|  | 49 |  | 2 |  |  | 0.00 |  | L2 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 50 |  | 3 |  |  | 0.00 |  | L2 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L2 |  | SB | - |  |
|  | 51 |  | 4 |  |  | 0.00 |  | L2 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L2 |  | SB | - |  |
|  | 52 |  | 5 |  |  | 0.00 |  | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L2 |  | SB | - |  |
|  | 53 |  | 6 |  |  | 0.00 |  | L2 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 55 |  | 1 |  |  | 0.00 |  | L2 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 56 |  | 2 |  |  | 0.00 |  | L2 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L2 |  | SB | - |  |
|  | 57 |  | 3 |  |  | 0.00 |  | L2 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 |  | 3 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 59 |  | 4 |  |  | 0.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 60 |  | 5 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 61 |  | 6 |  |  | 0.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 62 |  | 7 |  |  | 0.00 |  | L2 | SB |  | The DMI allows the driver to select ‘START’. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 17: Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3: Driver presses start, an MA request is sent and the On-board receives an SR authorisation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 1 |  |  | 0.00 |  | L2 | SB |  | T = t’ The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 64 |  | 2 |  |  | 0.00 |  | L2 | SB |  | Driver’s selection of 'Start' is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 65 |  | 3 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | 66 |  | 4 |  |  | 0.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded |  | O |  | JRU |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | 67 |  | 5 |  |  | 0.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) is received |  | I |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 68 |  | 6 |  |  | 0.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=2) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 69 |  | 7 |  |  | 0.00 |  | L2 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode It shall also be checked that there is no more request to the RBC for a MA. |  |  | L2 |  | SB | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 50: Acknowledgement for SR mode is requested in L2/ L3 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  | 1 |  |  | 0.00 |  | L2 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L2 |  | SB | - |  |
|  | 71 |  | 2 |  |  | 0.00 |  | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 72 |  | 3 |  |  | 0.00 |  | L2 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SR | - |  |
|  | 73 |  | 4 |  |  | 0.00 |  | L2 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | Symbol MO09 |  |  | L2 |  | SR | - |  |
|  | 74 |  | 5 |  |  | 0.00 |  | L2 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SR | - |  |
|  | Feature 3060300: 3.6.3: Validity direction of transmitted Information Test Case 5: The on-board equipment accepts all packets included in a radio message that are valid for both directions and ignores all packets that are valid for the nominal direction or the reverse direction, when the train position is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  | 1 |  |  | 0.00 |  | L2 | SR |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_DIR=0; Q_RBC=0), packet 41 (Q_DIR=1) and packet 72 (Q_DIR=2) is received |  | I |  | RTM |  | The train correctly receives the radio message with the 3 packets. |  |  | L2 |  | SR | - |  |
|  | 76 |  | 2 |  |  | 0.00 |  | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | The complete radio message is accepted by the onboard equipment and recorded on the JRU. |  |  | L2 |  | SR | - |  |
|  | 77 |  | 3 |  |  | 0.00 |  | L2 | SR |  | The plain text message "Both directions" is displayed. |  | O |  | DMI |  | - |  |  | L2 |  | SR | - |  |
|  | 78 |  | 4 |  |  | 0.00 |  | L2 | SR |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SR | - |  |
|  | 79 |  | 5 |  |  | 0.00 |  | L2 | SR |  | NO Level symbol announcement is displayed |  | O |  | DMI |  | - |  |  | L2 |  | SR | - |  |
|  | 80 |  | 6 |  |  | 0.00 |  | L2 | SR |  | NOT SA-CONNECT.Request is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SR | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 81 |  | 1 |  |  | 50.00 |  | L2 | SR |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. _____ Plain text message not displayed nor used in this sequence. The mean of this BG is to set a LRBG. |  |  | L2 |  | SR | - |  |
|  | 82 |  | 2 |  |  | 50.00 |  | L2 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SR | - |  |
|  | Feature 3060300: 3.6.3: Validity direction of transmitted Information Test Case 13: Handling of Distances for continuous profile data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 85 |  | 1 |  |  | 70.00 |  | L2 | SR |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 27 (D_STATIC<>0;V_STATIC=<V1>;N_ITER>1;D_STATIC(k)=0;V_STATIC(k)<>0 ), packet 21 is received |  | I |  | RTM |  | Complete message is accepted V1 will be the MRSP after accepting this message and reaching the location referenced by D_STATIC. Ensure that distance (n+1) is zero! And v_static(n+1) <>0 to check that v_static(n) is used beyond distance(n). |  |  | L2 |  | SR | - |  |
|  | 86 |  | 2 |  |  | 70.00 |  | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Complete message is recorded |  |  | L2 |  | SR | - |  |
|  | 87 |  | 3 |  |  | 70.00 |  | L2 | SR |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | Only if not already in FS mode. |  |  | L2 |  | FS | - |  |
|  | 88 |  | 4 |  |  | 70.00 |  | L2 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | Only if not already in FS mode. |  |  | L2 |  | FS | - |  |
|  | 89 |  | 5 |  |  | 150.00 |  | L2 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Permitted Speed is updated to the value V1. Check that for distance(n+1) (has to be zero) the value n is taken into account! _____ V1=45km/h V2=40km/h (distance(3)=0) |  |  | L2 |  | FS | - |  |
|  | 90 |  | 6 |  |  | 150.00 |  | L2 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;V_PERM=<V1>) is recorded |  | O |  | JRU |  | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. _____ V1=45km/h |  |  | L2 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 46: On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode. Transition Condition Id [40]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 93 |  | 3 |  |  | 200.00 |  | L2 | FS |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE = 00) is received |  | I |  | RTM |  | Only if in L2/L3 Reception of the mode transition announcement (mode transition order at current location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On sight |  |  | L2 |  | FS | - |  |
|  | 94 |  | 4 |  |  | 200.00 |  | L2 | FS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET= 80) is recorded |  | O |  | JRU |  | Only if in L2/L3 |  |  | L2 |  | FS | - |  |
|  | 95 |  | 5 |  |  | 200.00 |  | L2 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 96 |  | 6 |  |  | 200.00 |  | L2 | OS |  | The Mode symbol 'On Sight' is displayed |  | O |  | DMI |  | DMI Object: MO07 Or USE_FT4070201.23 |  |  | L2 |  | OS | - |  |
|  | 97 |  | 7 |  |  | 200.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS |  |  |
|  | 98 |  | 8 |  |  | 200.00 |  | L2 | OS |  | The Mode symbol 'Acknowledge On Sight' is displayed |  | O |  | DMI |  | DMI Object: MO08 |  |  | L2 |  | OS | - |  |
|  | 99 |  | 9 |  |  | 200.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 102 |  | 12 |  |  | 200.00 |  | L2 | OS |  | The driver acknowledges 'On Sight' |  | I |  | DMI |  | Driver acknowledges within 5 sec after the change to OS The driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  | OS | - |  |
|  | 103 |  | 13 |  |  | 200.00 |  | L2 | OS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | Feature 3060300: 3.6.3: Validity direction of transmitted Information Test Case 6: The on-board equipment accepts a radio message valid for the same direction as the train train orientation or for both directions and ignores a radio message valid for the opposite direction to the train orientation, when the train when the train orientation in relation to the direction of the LRBG is known. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 107 |  | 1 |  |  | 200.00 |  | L2 | OS |  | Train is moving (V_TRAIN=<greater than 0>) |  | I |  | INT |  | The train is running and the train orientation in relation to the direction of the LRBG is known. Train orientation in relation to the direction of the LRBG is described by variable Q_DIRLRBG, which can be equal to 0 (reverse) or 1 (nominal). |  |  | L2 |  | OS | - |  |
|  | 108 |  | 2 |  |  | 250.00 |  | L2 | OS |  | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received |  | I |  | RTM |  | Q_DIR = opposite (Q_DIRLRBG). _____ Q_DIR=0 |  |  | L2 |  | OS | - |  |
|  | 109 |  | 3 |  |  | 250.00 |  | L2 | OS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | The radio message is recorded on the JRU. |  |  | L2 |  | OS | - |  |
|  | 110 |  | 4 |  |  | 250.00 |  | L2 | OS |  | NOT The system status message "Emergency stop" is displayed |  | O |  | DMI |  | The train does not react. |  |  | L2 |  | OS | - |  |
|  | 111 |  | 5 |  |  | 300.00 |  | L2 | OS |  | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received |  | I |  | RTM |  | Q_DIR = Q_DIRLRBG _____ Q_DIR=1 |  |  | L2 |  | OS | - |  |
|  | 112 |  | 6 |  |  | 300.00 |  | L2 | OS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | The radio message is recorded on the JRU. |  |  | L2 |  | OS | - |  |
|  | 113 |  | 7 |  |  | 300.00 |  | L2 | OS |  | The Driver Request symbol "Track Ahead Free" is displayed. |  | O |  | DMI |  | - |  |  | L2 |  | OS | - |  |
|  | 114 |  | 8 |  |  | 300.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 115 |  | 9 |  |  | 350.00 |  | L2 | OS |  | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received |  | I |  | RTM |  | Q_DIR = 2 (Both directions) |  |  | L2 |  | OS | - |  |
|  | 116 |  | 10 |  |  | 350.00 |  | L2 | OS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | The radio message is recorded on the JRU. |  |  | L2 |  | OS | - |  |
|  | 117 |  | 11 |  |  | 350.00 |  | L2 | OS |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | DMI symbol: ST01 The train reaches the stop location |  |  | L2 |  | OS | - |  |
|  | 118 |  | 12 |  |  | 350.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 1 |  |  | 400.00 |  | L2 | OS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reaches standstill. |  |  | L2 |  | NP | - |  |
|  | 120 |  | 2 |  |  | 400.00 |  | L2 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  | NP | - |  |
| Step 13: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=2 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Enter Radio Data |  |  |  |  |  | RBC_EVENT=2 RBC_ID=1050091 RBC_PHONE_NUMBER=003265342101 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 22 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 24 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 26 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 28 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 00 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 30 : Message 41 - Train Accepted (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 41 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |

| - Step 58 : Message 129 - Validated Train Data (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 60 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 65 : Message 132 - MA Request (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 0 |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 67 : Message 2 - SR Authorisation (RBC n? 1) - Distance 0.00 m Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 2 d |  |  |
| L_MESSAGE | 10 | 12 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_SR | 15 | 10000 d |  |  |

| - Step 75 : Message 24 - General Message (RBC n? 1) - Distance 0.00 m Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 58 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 111111111111111 b |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 10 b |  |  |
| L_PACKET | 13 | 212 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 01 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 10 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 15 d |  |  |
| X_TEXT | 8 | B |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | D |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | c |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | s |  |  |

| - Step 81 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 236 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 10000 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 18 d |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | x |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | p |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | y |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 81 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 85 : Message 3 - Movement Authority (RBC n? 1) - Distance 70.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 39 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 12 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| D_STATIC | 15 | 100 d |  |  |
| V_STATIC | 7 | 9 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 8 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 93 : Message 3 - Movement Authority (RBC n? 1) - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 29 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 5000 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 108 : Message 15 - Conditional Emergency Stop (RBC n? 1) - Distance 250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 15 d |  |  |
| L_MESSAGE | 10 | 15 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_EM | 4 | 0 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 0 d |  |  |
| Q_DIR | 2 | 00 b |  |  |
| D_EMERGENCYSTOP | 15 | 0 d |  |  |

| - Step 111 : Message 34 - Track Ahead Free Request (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 34 d |  |  |
| L_MESSAGE | 10 | 16 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 0 d |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_TAFDISPLAY | 15 | 250 d |  |  |
| L_TAFDISPLAY | 15 | 200 d |  |  |

| - Step 115 : Message 15 - Conditional Emergency Stop (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 15 d |  |  |
| L_MESSAGE | 10 | 15 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_EM | 4 | 1 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 0 d |  |  |
| Q_DIR | 2 | 10 b |  |  |
| D_EMERGENCYSTOP | 15 | 400 d |  |  |