| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3070300_06_RIU_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3070300_06_RIU_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 18:03 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | RINA |

# Sequence description

TC16: New Speed Restriction to ensure Permitted Braking Distance information shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  | Dist. (m) | Previous |  |  | Description of Events | I/O | Interface | Comments |  | Next |  |  | Test Result |
|  |  |  |  |  | Levels | Mode |  |  |  |  |  |  | Levels |  | Mode |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 | 1 |  | 0.00 | N/A | NP |  | ERTMS/ETCS on-board unit is powered up | I | INT | The on-board equipment changes to SB mode. |  | N/A |  | NP | - |
|  | 2 | 2 |  | 0.00 | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - |  | N/A |  | NP | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 | 1 |  | 0.00 | N/A | NP |  | Driver opens desk | I | TIU | - |  | N/A |  | NP | - |
|  | 4 | 2 |  | 0.00 | N/A | NP |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). | O | JRU | - |  | N/A |  | SB | - |
|  | 5 | 3 |  | 0.00 | N/A | SB |  | The actual mode SB is DISPLAYED | O | DMI | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 | 1 |  | 0.00 | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed | O | DMI | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  | SB | - |
|  | 7 | 2 |  | 0.00 | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. | O | DMI | - |  | N/A |  | SB | - |
|  | 8 | 3 |  | 0.00 | N/A | SB |  | The driver enters and validates a correct Driver ID | I | DMI | - |  | N/A |  | SB | - |
|  | 9 | 4 |  | 0.00 | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. | O | JRU | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 1 |  | 0.00 | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" | I | DMI | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver select Level 1 [59.-1.1] |  | L1 |  | SB | - |
|  | 11 | 2 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 12 | 3 |  | 0.00 | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 | 1 |  | 0.00 | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ | O | DMI | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  | SB | - |
|  | 14 | 2 |  | 0.00 | L1 | SB |  | The driver selects ‘Train Data Entry’ | I | DMI | - |  | L1 |  | SB | - |
|  | 15 | 3 |  | 0.00 | L1 | SB |  | A Train data window is is displayed on the DMI. | O | DMI | - |  | L1 |  | SB | - |
|  | 16 | 4 |  | 0.00 | L1 | SB |  | The driver’s selection is recorded on the JRU. | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | 1 |  | 0.00 | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. | O | DMI | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  | SB | - |
|  | 19 | 2 |  | 0.00 | L1 | SB |  | The driver enters each Train data unknown on-board | I | DMI | - |  | L1 |  | SB | - |
|  | 20 | 3 |  | 0.00 | L1 | SB |  | The driver is allowed to validate the train data | O | DMI | A complete set of valid Train Data is available |  | L1 |  | SB | - |
|  | 21 | 4 |  | 0.00 | L1 | SB |  | The driver validates the train data. | I | DMI | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L1 |  | SB | - |
|  | 22 | 5 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. |  | L1 |  | SB | - |
|  | 23 | 6 |  | 0.00 | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 | 1 |  | 0.00 | L1 | SB |  | The driver is requested to enter the Train running number. | O | DMI | - |  | L1 |  | SB | - |
|  | 26 | 2 |  | 0.00 | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 |  | L1 |  | SB | - |
|  | 27 | 3 |  | 0.00 | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 | 3 |  | 0.00 | L1 | SB |  | The DMI allows the driver to select the option ‘START’. | O | DMI | - |  | L1 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 | 1 |  | 0.00 | L1 | SB |  | The driver selects 'Start'. | I | DMI | - |  | L1 |  | SB | - |
|  | 30 | 2 |  | 0.00 | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU | O | JRU | Triggered by: The driver acts on the on-board DMI. |  | L1 |  | SB | - |
|  | 31 | 3 |  | 0.00 | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  | SB | - |
|  | 32 | 4 |  | 0.00 | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 33 | 5 |  | 0.00 | L1 | SB |  | The driver acknowledges 'Staff Responsible' | I | DMI | Symbol MO10 |  | L1 |  | SB | - |
|  | 34 | 6 |  | 0.00 | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - |  | L1 |  | SB | - |
|  | 35 | 7 |  | 0.00 | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | 36 | 8 |  | 0.00 | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed | O | DMI | - |  | L1 |  | SR | - |
|  | 37 | 9 |  | 0.00 | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | 38 | 10 |  | 0.00 | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 | 1 |  | 50.00 | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received | I | BTM | Packet 80 not included for current location |  | L1 |  | SR | - |
|  | 40 | 2 |  | 50.00 | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L1 |  | SR | - |
|  | 41 | 3 |  | 50.00 | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The on-board equipment switches immediately to FS mode |  | L1 |  | FS | - |
|  | 42 | 4 |  | 50.00 | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed | O | DMI | DMI Object: Symbol MO11 |  | L1 |  | FS | - |
|  | 43 | 5 |  | 50.00 | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | Feature 4080429: 4.8.4: Accepted Information depending on the modes - Radio Infill Area information Test Case 3: Transmission of radio infill area information for establishing a communication session with RIU via BG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 | 1 |  | 100.00 | L1 | FS |  | One Balise Group message (N_TOTAL=1) containing packet 133 (Q_RIU=1) is received | I | BTM | Q_RIU=1 means "Establish communication session" |  | L1 |  | FS | - |
|  | 45 | 2 |  | 100.00 | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | Feature 3090300: 3.9.3: Infill by radio Test Case 1: Starting event is received from the balise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 | 2 |  | 100.00 | L1 | FS |  | SA-CONNECT.Request is transmitted | O | RTM | - |  | L1 |  | FS | - |
|  | 49 | 3 |  | 100.00 | L1 | FS |  | SA-CONNECT.Confirm is received | I | RTM | Confirmation of the vital connection from RTM |  | L1 |  | FS | - |
|  | 50 | 4 |  | 100.00 | L1 | FS |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - |  | L1 |  | FS | - |
|  | 51 | 5 |  | 100.00 | L1 | FS |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 52 | 6 |  | 100.00 | L1 | FS |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | The trackside ETCS system version is accepted since it is compatible with at least one of the ETCS versions stored on-board. At this point, the on- board equipment considers the communication session with the RBC as established. |  | L1 |  | FS | - |
|  | 53 | 7 |  | 100.00 | L1 | FS |  | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 54 | 8 |  | 100.00 | L1 | FS |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | - |  | L1 |  | FS | - |
|  | 55 | 9 |  | 100.00 | L1 | FS |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | Communication session established for trackside. Only for information ' not relevant for on-board equipment. |  | L1 |  | FS | - |
|  | 56 | 10 |  | 100.00 | L1 | FS |  | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | The on-board unit shall inform the radio infill unit using message 153 as soon as the location received during preceding TC FT4080429.3 is passed |  | L1 |  | FS | - |
|  | 57 | 11 |  | 100.00 | L1 | FS |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | Feature 3070300: 3.7.3: Extension, replacement of track description and linking information Test Case 16: New Speed Restriction to ensure Permitted Braking Distance information shall replace all stored Speed Restriction to ensure Permitted Braking Distance information from the start location of the first element of the new information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 | 1 |  | 100.00 | L1 | FS |  | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | (The onbard has established a radio communication session to radio infill unit.) |  | L1 |  | FS | - |
|  | 59 | 2 |  | 350.00 | L1 | FS |  | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 12 (MA), packet 136 (infill location reference), packet 52 (Permitted Braking Distance Information), is received. | I | RTM | - |  | L1 |  | FS | - |
|  | 60 | 3 |  | 350.00 | L1 | FS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 61 | 4 |  | 350.00 | L1 | FS |  | The speed and distance monitoring information is displayed | O | DMI | New MA is displayed. Complete message is accepted and suitable information is presented to driver. The information is valid from the main balise group. The corresponding speed restriction is DISPLAYED |  | L1 |  | FS | - |
|  | 62 | 5 |  | 350.00 | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 63 | 6 |  | 1000.00 | L1 | FS |  | One Balise Group message (N_TOTAL=1; NID_BG=<BGa>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a balise group (BGa). The BGa contains a packet 52 BGa is the Main BG for infill message in step 2 |  | L1 |  | FS | - |
|  | 64 | 7 |  | 1000.00 | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 65 | 8 |  | 1000.00 | L1 | FS |  | The speed and distance monitoring information is displayed | O | DMI | The corresponding speed restriction is DISPLAYED |  | L1 |  | FS | - |
|  | 66 | 9 |  | 1000.00 | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 67 | 10 |  | 1150.00 | L1 | FS |  | One Balise Group message (N_TOTAL=1; NID_BG=<BGb>) containing packet 52 and packet 255 is received | I | BTM | L1: The train overpasses a second balise group (BGb). The BGb contains a packet 52 As BGb is overpassed, the Permitted Braking Distance of BGa is still effective |  | L1 |  | FS | - |
|  | 68 | 11 |  | 1150.00 | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 69 | 12 |  | 1150.00 | L1 | FS |  | The speed and distance monitoring information is displayed | O | DMI | The speed and distance monitoring information is displayed. Check that the Permitted Braking Distance Information is updated and replaces the first one from the start location of the first element of the new information. |  | L1 |  | FS | - |
|  | 70 | 13 |  | 1150.00 | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 71 | 14 |  | 1300.00 | L1 | FS |  | One Balise Group message (N_TOTAL=1; NID_BG=<BGv>) containing packet 52 (Q_TRACKINIT=1) and packet 255 is received | I | BTM | L1: The train overpasses a third BG (BGc) The BGc contains a packet 52 with Q_TRACKINIT= 1 (Empty profile, initial states to be resumed) As BGc is overpassed, the Permitted Braking Distance of BGb is still effective |  | L1 |  | FS | - |
|  | 72 | 15 |  | 1300.00 | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | 73 | 16 |  | 1300.00 | L1 | FS |  | The speed and distance monitoring information is displayed | O | DMI | All speed restrictions derived from packets 52 in BGa and BGb are RELEASED |  | L1 |  | FS | - |
|  | 74 | 17 |  | 1300.00 | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - |  | L1 |  | FS | - |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 | 1 |  | 1450.00 | L1 | FS |  | The ERTMS/ETCS on-board unit is powered down | I | INT | - |  | L1 |  | NP | - |
|  | 76 | 2 |  | 1450.00 | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded | O | JRU | There are no messages recorded in the JRU during a period of time |  | L1 |  | NP | - |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  | Event Name |  |  |  | Data |  |  |  |  | Delay |  |  |  |  |
| 29 |  |  | Request Change Level |  |  |  | LEVEL=1 |  |  |  |  | 0 s |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 16 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 3000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 50 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 3000 d |  |  |
| V_STATIC | 7 | 1111111 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 186 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 50 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 12 d |  |  |
| N_ITER | 5 | 3 d |  |  |
| D_LINK | 15 | 900 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 12 d |  |  |
| D_LINK | 15 | 150 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 12 d |  |  |
| D_LINK | 15 | 150 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 12 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 50 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 52 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 54 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 56 : Message 153 - Radio infill Request (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 153 d |  |  |
| L_MESSAGE | 10 | 27 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_INFILL | 1 | 0 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 010 b |  |  |

| - Step 59 : Message 37 - Infill MA (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 37 d |  |  |
| L_MESSAGE | 10 | 34 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 136 d | In-fill location reference |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 38 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 16 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 52 d | Permitted Braking Distance Information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_PBD | 15 | 100 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_PBDSR | 8 | 50 d |  |  |
| Q_PBDSR | 1 | 1 b |  |  |
| D_PBDSR | 15 | 0 d |  |  |
| L_PBDSR | 15 | 1000 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 63 : Balise Group BG 3 d 1/2 - Distance 1000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 52 d | Permitted Braking Distance Information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_PBD | 15 | 100 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_PBDSR | 8 | 50 d |  |  |
| Q_PBDSR | 1 | 1 b |  |  |
| D_PBDSR | 15 | 0 d |  |  |
| L_PBDSR | 15 | 1000 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 3 d 2/2 - Distance 1005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 67 : Balise Group BG 4 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 52 d | Permitted Braking Distance Information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_PBD | 15 | 100 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_PBDSR | 8 | 0 d |  |  |
| Q_PBDSR | 1 | 0 b |  |  |
| D_PBDSR | 15 | 0 d |  |  |
| L_PBDSR | 15 | 1000 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 67 : Balise Group BG 4 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 5 d 1/2 - Distance 1300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 52 d | Permitted Braking Distance Information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 41 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 1 |  |  |
| D_TRACKINIT | 15 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 5 d 2/2 - Distance 1305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |