| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3120400_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3120400_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 19:48 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | MLT |

# Sequence description

Tested Feature 3.12.4: Mode Profile

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | NP | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver select Level 1 [59.-1.1] |  |  | N/A |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance _____ Added linking Packet 5 |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  | FS | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 100.00 |  | L1 | FS |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted |  |  | L1 |  | FS | - |  |
|  | 44 |  | 2 |  |  | 100.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  | FS | - |  |
|  | Feature 3120400: 3.12.4: Mode Profile Test Case 3: When receiving a new MA by infill, any currently supervised Mode Profile shall be deleted only beyond the reference location of the infill information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 |  | 1 |  |  | 200.00 |  | L1 | FS |  | One Balise Group message containing containing packet 136, packet 12, packet 21, packet 27 is received |  | I |  | BTM |  | The supervised Mode Profile is deleted only beyond the reference location of the infill information (beyond the location of balise group NID_BGref). |  |  | L1 |  | FS | - |  |
|  | 47 |  | 2 |  |  | 200.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Infill message is RECORDED |  |  | L1 |  | FS | - |  |
|  | 48 |  | 3 |  |  | 200.00 |  | L1 | FS |  | The Mode symbol "Acknowledge Shunting" is displayed |  | O |  | DMI |  | Only if the previously stored Mode Profile had M_MAMODE=01 (SH). DMI Object: Symbol MO02. The driver will not acknowledge this message (see below). |  |  | L1 |  | FS | - |  |
|  | 49 |  | 4 |  |  | 200.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded |  | O |  | JRU |  | Only if the previously stored Mode Profile had M_MAMODE=01 (SH) |  |  | L1 |  | FS | - |  |
|  | 54 |  | 9 |  |  | 300.00 |  | L1 | FS |  | Train is moving (V_TRAIN=<above zero speed>) |  | I |  | INT |  | The train passes over the location of the main BG, without reading any BG (no reaction is applied, according to the linking information stored on-board) |  |  | L1 |  | FS | - |  |
|  | 55 |  | 10 |  |  | 300.00 |  | L1 | FS |  | The Mode symbol "Acknowledge Shunting" is removed |  | O |  | DMI |  | Only if the previously stored Mode Profile had M_MAMODE=01 (SH). DMI Object: Symbol MO02. This meesage will be removed without the driver acknowledging it. |  |  | L1 |  | FS | - |  |
|  | 56 |  | 11 |  |  | 300.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=0>) is recorded |  | O |  | JRU |  | Only if the previously stored Mode Profile had M_MAMODE=01 (SH) |  |  | L1 |  | FS | - |  |
|  | 63 |  | 18 |  |  | 300.00 |  | L1 | FS |  | The Mode symbol "Mode < FS >" is displayed |  | O |  | DMI |  | Only if the Starting mode was FS mode. |  |  | L1 |  | FS | - |  |
|  | 64 |  | 19 |  |  | 300.00 |  | L1 | FS |  | GENERAL MESSAGE (M_MODE=0) is recorded |  | O |  | JRU |  | Only if the Starting mode was FS mode. |  |  | L1 |  | FS | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 400.00 |  | L1 | FS |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted _____ Packet 49: List of Balises for SH included |  |  | L1 |  | FS | - |  |
|  | 66 |  | 2 |  |  | 400.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  | FS | - |  |
|  | Feature 3120400: 3.12.4: Mode Profile Test Case 1: Acceptance of a mode profile with a new list of balises for Shunting and rejection of the previously stored list before the on-board equipment switches to SH mode (independent of an empty or not empty list) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  | 3 |  |  | 550.00 |  | L1 | FS |  | One Balise Group message containing packet 12, packet 80 (Q_DIR=1; M_MAMODE=01), packet 21, packet 27, packet 49 (Q_DIR=1) is received |  | I |  | BTM |  | Only for L1 |  |  | L1 |  | FS | - |  |
|  | 71 |  | 4 |  |  | 550.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only for L1 |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 39: Acknowledgement for shunting is requested, driver acknowledges, the system switches to SH mode and an end of mission takes place (only in levels 2 and 3). Transition Condition Id [50]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 1 |  |  | 600.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train Location: The train reaches the acknowledgement window defined by the variable L_ACKMODE (in packet 80) with its maximum safe front end. V_TRAIN: The speed is lower than the maximum shunting speed: D_MAMODE>D>D_MAMODE–L_ACKMAMODE V<V_NVSHUNT |  |  | L1 |  | FS | - |  |
|  | 74 |  | 2 |  |  | 650.00 |  | L1 | FS |  | The Mode symbol 'Acknowledge Shunting' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO02 |  |  | L1 |  | FS | - |  |
|  | 75 |  | 3 |  |  | 650.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 76 |  | 4 |  |  | 750.00 |  | L1 | FS |  | The driver acknowledges 'Shunting' |  | I |  | DMI |  | - |  |  | L1 |  | FS | - |  |
|  | 77 |  | 5 |  |  | 750.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 78 |  | 6 |  |  | 750.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | Triggering event: when the mode changes |  |  | L1 |  | SH | - |  |
|  | 79 |  | 7 |  |  | 750.00 |  | L1 | SH |  | The Mode symbol 'Mode <SH>' is displayed |  | O |  | DMI |  | DMI Object: MO01 or USE_FT4070201.20 |  |  | L1 |  | SH | - |  |
|  | 80 |  | 8 |  |  | 750.00 |  | L1 | SH |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | Feature 4040800: 4.4.8: SHUNTING Test Case 2: The on-board system supervises the list of expected balises in SH mode, if such list was sent by trackside |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 85 |  | 1 |  |  | 850.00 |  | L1 | SH |  | One Balise Group message (N_TOTAL=0) received |  | I |  | BTM |  | Train passes balises that are included in the list of expected balises for shunting |  |  | L1 |  | SH | - |  |
|  | 86 |  | 2 |  |  | 850.00 |  | L1 | SH |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | no reaction was applied |  |  | L1 |  | SH | - |  |
|  | 87 |  | 3 |  |  | 850.00 |  | L1 | SH |  | Emergency brake not commanded |  | O |  | TIU |  | passage is allowed |  |  | L1 |  | SH | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 34: Before entering the shunting area, the train receives a list of balises for shunting area. When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied. Transition Condition Id [52]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 89 |  | 1 |  |  | 950.00 |  | L1 | SH |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) is received |  | I |  | BTM |  | NID_BG variable in BGa message is different from NID_BG values in the list of balises for SH stored onboard OR the list of balises for SH is empty. |  |  | L1 |  | SH | - |  |
|  | 90 |  | 2 |  |  | 950.00 |  | L1 | SH |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 91 |  | 3 |  |  | 950.00 |  | L1 | SH |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | TR | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 93 |  | 1 |  |  | 950.00 |  | L1 | TR |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  | TR | - |  |
|  | 94 |  | 2 |  |  | 950.00 |  | L1 | TR |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  | TR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 34: Before entering the shunting area, the train receives a list of balises for shunting area. When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied. Transition Condition Id [52]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  | 5 |  |  | 950.00 |  | L1 | TR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | TR | - |  |
|  | 96 |  | 6 |  |  | 950.00 |  | L1 | TR |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded |  | O |  | JRU |  | SH stop order |  |  | L1 |  | TR | - |  |
|  | 97 |  | 7 |  |  | 950.00 |  | L1 | TR |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  | TR | - |  |
|  | 98 |  | 8 |  |  | 950.00 |  | L1 | TR |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | TR | - |  |
|  | 99 |  | 9 |  |  | 950.00 |  | L1 | TR |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  | TR | - |  |
|  | 100 |  | 10 |  |  | 950.00 |  | L1 | TR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | TR | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 101 |  | 1 |  |  | 960.00 |  | L1 | TR |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  | TR | - |  |
|  | 102 |  | 2 |  |  | 990.00 |  | L1 | TR |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  | TR | - |  |
|  | 103 |  | 3 |  |  | 990.00 |  | L1 | TR |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  | TR | - |  |
|  | 104 |  | 4 |  |  | 990.00 |  | L1 | TR |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  | TR | - |  |
|  | 105 |  | 5 |  |  | 990.00 |  | L1 | TR |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | PT | - |  |
|  | 106 |  | 6 |  |  | 990.00 |  | L1 | PT |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  | PT | - |  |
|  | 107 |  | 7 |  |  | 990.00 |  | L1 | PT |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  | PT | - |  |
|  | 110 |  | 10 |  |  | 990.00 |  | L1 | PT |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  | PT | - |  |
|  | 111 |  | 11 |  |  | 990.00 |  | L1 | PT |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  | PT | - |  |
|  | 112 |  | 12 |  |  | 990.00 |  | L1 | PT |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | PT | - |  |
|  | 113 |  | 13 |  |  | 990.00 |  | L1 | PT |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  | PT | - |  |
|  | 114 |  | 14 |  |  | 990.00 |  | L1 | PT |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | PT | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 8: In PT mode, the ERTMS/ETCS on-board equipment shall NOT offer the possibility to the driver to select „start', because the train data is not available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 1 |  |  | 990.00 |  | L1 | PT |  | The <START> button in the <MAIN> window is disabled. |  | O |  | DMI |  | The On-Board equipment does not allows the driver to select 'Start' because no valid Train data available on-board. |  |  | L1 |  | PT | - |  |
|  | 116 |  | 2 |  |  | 990.00 |  | L1 | PT |  | The <SHUNTING> button in the <MAIN> window is enable. |  | O |  | DMI |  | - |  |  | L1 |  | PT | - |  |
|  | 117 |  | 3 |  |  | 990.00 |  | L1 | PT |  | Driver presses Shunting. |  | I |  | DMI |  | - |  |  | L1 |  | PT | - |  |
|  | 118 |  | 4 |  |  | 990.00 |  | L1 | PT |  | Driver action is recorded. |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 1 |  |  | 990.00 |  | L1 | SH |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  | SH | - |  |
|  | 120 |  | 2 |  |  | 990.00 |  | L1 | SH |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | 121 |  | 3 |  |  | 990.00 |  | L1 | SH |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 122 |  | 4 |  |  | 990.00 |  | L1 | SH |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  | SB | - |  |
|  | 123 |  | 5 |  |  | 990.00 |  | L1 | SB |  | The Mode symbol 'Stand By' is displayed |  | O |  | DMI |  | DMI Object: symbol MO13 |  |  | L1 |  | SB | - |  |
|  | 124 |  | 6 |  |  | 990.00 |  | L1 | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 |  | 1 |  |  | 990.00 |  | L1 | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  | SB | - |  |
|  | 126 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  | SB | - |  |
|  | 127 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 129 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 130 |  | 3 |  |  | 990.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 131 |  | 4 |  |  | 990.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 134 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 135 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 136 |  | 4 |  |  | 990.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 137 |  | 5 |  |  | 990.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 138 |  | 6 |  |  | 990.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 |  | 1 |  |  | 990.00 |  | L1 | SB |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 140 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L1 |  | SB | - |  |
|  | 141 |  | 3 |  |  | 990.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 142 |  | 4 |  |  | 990.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 144 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 145 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 146 |  | 3 |  |  | 990.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 147 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 51: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver presses 'Shunting' button. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 149 |  | 2 |  |  | 990.00 |  | L1 | SB |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 150 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The SH symbol, MO01 is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | 151 |  | 4 |  |  | 990.00 |  | L1 | SH |  | The displayed symbol for SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 152 |  | 5 |  |  | 990.00 |  | L1 | SH |  | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 153 |  | 1 |  |  | 990.00 |  | L1 | SH |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  | SH | - |  |
|  | 154 |  | 2 |  |  | 990.00 |  | L1 | SH |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | 155 |  | 3 |  |  | 990.00 |  | L1 | SH |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 156 |  | 4 |  |  | 990.00 |  | L1 | SH |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  | SB | - |  |
|  | 157 |  | 5 |  |  | 990.00 |  | L1 | SB |  | The Mode symbol 'Stand By' is displayed |  | O |  | DMI |  | DMI Object: symbol MO13 |  |  | L1 |  | SB | - |  |
|  | 158 |  | 6 |  |  | 990.00 |  | L1 | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 159 |  | 1 |  |  | 990.00 |  | L1 | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  | SB | - |  |
|  | 160 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  | SB | - |  |
|  | 161 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 162 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 163 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 164 |  | 3 |  |  | 990.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 165 |  | 4 |  |  | 990.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 167 |  | 1 |  |  | 990.00 |  | L1 | SB |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 168 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L1 |  | SB | - |  |
|  | 169 |  | 3 |  |  | 990.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 170 |  | 4 |  |  | 990.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 172 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 173 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 174 |  | 3 |  |  | 990.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 175 |  | 3 |  |  | 990.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 7: The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start', because the train data are available on-board (new TC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 176 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The On-Board equipment waits for the driver to select 'Start'. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 177 |  | 2 |  |  | 990.00 |  | L1 | SB |  | The Driver selects 'Start' |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 178 |  | 3 |  |  | 990.00 |  | L1 | SB |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 179 |  | 4 |  |  | 990.00 |  | L1 | SB |  | The Mode symbol "Acknowledge Staff Responsible" is displayed. |  | O |  | DMI |  | DMI Object: Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 180 |  | 5 |  |  | 990.00 |  | L1 | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 49: Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 181 |  | 1 |  |  | 990.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 182 |  | 2 |  |  | 990.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 183 |  | 3 |  |  | 990.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 184 |  | 4 |  |  | 990.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | Symbol MO09 |  |  | L1 |  | SR | - |  |
|  | 185 |  | 5 |  |  | 990.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 186 |  | 6 |  |  | 990.00 |  | L1 | SR |  | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. |  | O |  | JRU |  | Optional step: The status of the Train position information will change to 'Unknown' only if it was 'Invalid'. |  |  | L1 |  | SR | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 187 |  | 1 |  |  | 1065.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  | SR | - |  |
|  | 188 |  | 2 |  |  | 1065.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 189 |  | 3 |  |  | 1065.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  | FS | - |  |
|  | 190 |  | 4 |  |  | 1065.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 191 |  | 1 |  |  | 1065.00 |  | L1 | FS |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted |  |  | L1 |  | FS | - |  |
|  | 192 |  | 2 |  |  | 1065.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  | FS | - |  |
|  | Feature 3120400: 3.12.4: Mode Profile Test Case 2: Acceptance of mode profile for shunting that overwrites a previously stored mode profile for shunting and deletes a list of balises for SH stored on-board, before the on-board equipment switches to SH mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 196 |  | 3 |  |  | 1150.00 |  | L1 | FS |  | One Balise Group message containing packet 12, packet 80 (Q_DIR=1; M_MAMODE=01), packet 21, packet 27 is received |  | I |  | BTM |  | Only for L1. No list of balises for SH area is received in this message |  |  | L1 |  | FS | - |  |
|  | 197 |  | 4 |  |  | 1150.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only for L1 |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 39: Acknowledgement for shunting is requested, driver acknowledges, the system switches to SH mode and an end of mission takes place (only in levels 2 and 3). Transition Condition Id [50]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 201 |  | 1 |  |  | 1250.00 |  | L1 | FS |  | Train is moving. |  | I |  | INT |  | Train Location: The train reaches the acknowledgement window defined by the variable L_ACKMODE (in packet 80) with its maximum safe front end. V_TRAIN: The speed is lower than the maximum shunting speed: D_MAMODE>D>D_MAMODE–L_ACKMAMODE V<V_NVSHUNT |  |  | L1 |  | FS | - |  |
|  | 202 |  | 2 |  |  | 1250.00 |  | L1 | FS |  | The Mode symbol 'Acknowledge Shunting' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO02 |  |  | L1 |  | FS | - |  |
|  | 203 |  | 3 |  |  | 1250.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 204 |  | 4 |  |  | 1350.00 |  | L1 | FS |  | The driver acknowledges 'Shunting' |  | I |  | DMI |  | - |  |  | L1 |  | FS | - |  |
|  | 205 |  | 5 |  |  | 1350.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 206 |  | 6 |  |  | 1350.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | Triggering event: when the mode changes |  |  | L1 |  | SH | - |  |
|  | 207 |  | 7 |  |  | 1350.00 |  | L1 | SH |  | The Mode symbol 'Mode <SH>' is displayed |  | O |  | DMI |  | DMI Object: MO01 or USE_FT4070201.20 |  |  | L1 |  | SH | - |  |
|  | 208 |  | 8 |  |  | 1350.00 |  | L1 | SH |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | Feature 3120400: 3.12.4: Mode Profile Test Case 2: Acceptance of mode profile for shunting that overwrites a previously stored mode profile for shunting and deletes a list of balises for SH stored on-board, before the on-board equipment switches to SH mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 210 |  | 8 |  |  | 1450.00 |  | L1 | SH |  | One Balise Group message is received |  | I |  | BTM |  | The identity of this BG has to be different from any of the BG identities in the previously stored list of balises for SH area. The train does not supervise this previously stored list of balises during shunting movement, because it deleted it. |  |  | L1 |  | SH | - |  |
|  | 211 |  | 9 |  |  | 1450.00 |  | L1 | SH |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 212 |  | 10 |  |  | 1450.00 |  | L1 | SH |  | Not emergency brake commanded |  | O |  | TIU |  | This steps is enough to to test the on-board equipment does not change to TR mode after receiving the BG. |  |  | L1 |  | SH | - |  |
|  | Feature 3120400: 3.12.4: Mode Profile Test Case 4: For SH mode, any length given in the mode profile shall be ignored by the on-board. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 213 |  | 1 |  |  | 1550.00 |  | L1 | SH |  | Train is moving (V_TRAIN=<above zero speed>) |  | I |  | INT |  | - |  |  | L1 |  | SH | - |  |
|  | 214 |  | 2 |  |  | 1550.00 |  | L1 | SH |  | The Mode symbol "Shunting" is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | 215 |  | 3 |  |  | 1600.00 |  | L1 | SH |  | Train is moving (V_TRAIN=<above zero speed>) |  | I |  | INT |  | The estimated front end reaches the location defined by L_MAMODE, which was previously received. |  |  | L1 |  | SH | - |  |
|  | 216 |  | 4 |  |  | 1600.00 |  | L1 | SH |  | Not service brake commanded |  | O |  | TIU |  | The train keeps on running without problems. |  |  | L1 |  | SH | - |  |
|  | 217 |  | 5 |  |  | 1600.00 |  | L1 | SH |  | Not emergency brake commanded |  | O |  | TIU |  | The train keeps on running without problems. |  |  | L1 |  | SH | - |  |
|  | 218 |  | 6 |  |  | 1600.00 |  | L1 | SH |  | The Mode symbol "Shunting" is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 219 |  | 1 |  |  | 1750.00 |  | L1 | SH |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  | SH | - |  |
|  | 220 |  | 2 |  |  | 1750.00 |  | L1 | SH |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  | SH | - |  |
|  | 221 |  | 3 |  |  | 1750.00 |  | L1 | SH |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SH | - |  |
|  | 222 |  | 4 |  |  | 1750.00 |  | L1 | SH |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  | SB | - |  |
|  | 223 |  | 5 |  |  | 1750.00 |  | L1 | SB |  | The Mode symbol 'Stand By' is displayed |  | O |  | DMI |  | DMI Object: symbol MO13 |  |  | L1 |  | SB | - |  |
|  | 224 |  | 6 |  |  | 1750.00 |  | L1 | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 225 |  | 1 |  |  | 1750.00 |  | L1 | SB |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  | SB | - |  |
|  | 226 |  | 2 |  |  | 1750.00 |  | L1 | SB |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  | SB | - |  |
|  | 227 |  | 3 |  |  | 1750.00 |  | L1 | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 228 |  | 1 |  |  | 1750.00 |  | L1 | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Select L2 |  |  | L1 |  | SB | - |  |
|  | 229 |  | 2 |  |  | 1750.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 230 |  | 3 |  |  | 1750.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 36: On-board equipment offers the driver the possibility to re-enter the Radio Network ID. The list of available and allowed networks is empty. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 231 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | The on-board equipment displays the RBC contact window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 232 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | The driver presses the ‘Radio network ID’ button |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 233 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | The system status message ‘Radio network registration failed’ is DISPLAYED. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 234 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit23=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 38: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to enter the RBC-ID/phone number if its status is 'unknown'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 235 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - _____ Driver presses level and selects L2 to reach Radio data window |  |  | L2 |  | SB | - |  |
|  | 236 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | The driver presses the ‘Enter RBC data’ button |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 237 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | The on-board equipment displays the RBC data window with no value for the RBC ID/Phone number (since the status of this information is UNKNOWN). |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 238 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | The driver enters valid RBC ID/Phone number. |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 239 |  | 5 |  |  | 1750.00 |  | L2 | SB |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 240 |  | 6 |  |  | 1750.00 |  | L2 | SB |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 241 |  | 7 |  |  | 1750.00 |  | L2 | SB |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 22: On-board Equipment reports its valid position to RBC at start up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 242 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 243 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 244 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 245 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 246 |  | 5 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions are compatible, so that the communication session established for on-board |  |  | L2 |  | SB | - |  |
|  | 247 |  | 6 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 248 |  | 7 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 249 |  | 8 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159; NID_PACKET=2) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 250 |  | 9 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157, Q_STATUS=1) containing packet 0 is transmitted |  | O |  | RTM |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | 251 |  | 10 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=157; Q_STATUS=1; NID_PACKET=0) is recorded |  | O |  | JRU |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 3: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) The on-board equipment allows the driver to modify train data before the validation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 253 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L2 |  | SB | - |  |
|  | 254 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | The driver modifies at least one of the train data. |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 255 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | The driver validates the train data. |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 256 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L2 |  | SB | - |  |
|  | 257 |  | 5 |  |  | 1750.00 |  | L2 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 259 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L2 |  | SB | - |  |
|  | 260 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 261 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 262 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 263 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 264 |  | 5 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  | SB | - |  |
|  | 265 |  | 6 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 266 |  | 7 |  |  | 1750.00 |  | L2 | SB |  | The DMI allows the driver to select ‘START’. |  | O |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 18: Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3, the driver selects 'Start'. The on-board equipment then requests the RBC for a Movement Authority and a Movement Authority for OS/LS/SH is received from the RBC while the front end of the train is already inside the OS/LS/SH area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 267 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | T = t’ The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L2 |  | SB | - |  |
|  | 268 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | Driver’s selection of 'Start'is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 269 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | 270 |  | 4 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded |  | O |  | JRU |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  | SB | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 42: An acknowledgement request for OS mode is displayed to the driver. The driver acknowledges OS mode. Mode is changed to On Sight. Transition Condition Id [15]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 274 |  | 1 |  |  | 1750.00 |  | L2 | SB |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 80 (M_MAMODE=00) is received |  | I |  | RTM |  | The train receives a message including a packet 80 Mode profile with : NID_PACKET = 80: Mode Profile M_MAMODE =00: On Sight Reception from RBC of the MA allowing OS |  |  | L2 |  | SB | - |  |
|  | 275 |  | 2 |  |  | 1750.00 |  | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | SB | - |  |
|  | 276 |  | 3 |  |  | 1750.00 |  | L2 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 277 |  | 4 |  |  | 1750.00 |  | L2 | OS |  | The Mode symbol 'On Sight' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO07 |  |  | L2 |  | OS | - |  |
|  | 278 |  | 5 |  |  | 1750.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 279 |  | 6 |  |  | 1750.00 |  | L2 | OS |  | The Mode symbol 'Acknowledge On Sight' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO08 |  |  | L2 |  | OS | - |  |
|  | 280 |  | 7 |  |  | 1750.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 281 |  | 8 |  |  | 1750.00 |  | L2 | OS |  | The driver acknowledges 'On Sight' |  | I |  | DMI |  | - |  |  | L2 |  | OS | - |  |
|  | 282 |  | 9 |  |  | 1750.00 |  | L2 | OS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 283 |  | 10 |  |  | 1750.00 |  | L2 | OS |  | The Mode symbol 'Acknowledge On Sight' is removed. |  | O |  | DMI |  |  |  |  | L2 |  | OS | - |  |
|  | 284 |  | 11 |  |  | 1750.00 |  | L2 | OS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 285 |  | 12 |  |  | 1750.00 |  | L2 | OS |  | The Mode symbol 'On Sight' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO07 |  |  | L2 |  | OS | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 286 |  | 1 |  |  | 1900.00 |  | L2 | OS |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  | OS | - |  |
|  | 287 |  | 2 |  |  | 1900.00 |  | L2 | OS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 288 |  | 3 |  |  | 1900.00 |  | L2 | OS |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  | OS | - |  |
|  | 289 |  | 4 |  |  | 1900.00 |  | L2 | OS |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  | OS | - |  |
|  | 290 |  | 5 |  |  | 1900.00 |  | L2 | OS |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  | OS | - |  |
|  | 291 |  | 6 |  |  | 1900.00 |  | L2 | OS |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  | OS | - |  |
|  | 292 |  | 7 |  |  | 1900.00 |  | L2 | OS |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  | OS | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 2: At standstill, in order to cover degraded situations, when the table of priority is not available, the driver can select any level within a default list configured on-board. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 293 |  | 1 |  |  | 2000.00 |  | L2 | OS |  | The driver selects "Main" |  | I |  | DMI |  | Current level is <LEVEL_X> _____ The train reached standstill |  |  | L2 |  | OS | - |  |
|  | 294 |  | 2 |  |  | 2000.00 |  | L2 | OS |  | The driver selects "Level 0/1/2/3/NTC" |  | I |  | DMI |  | Selected level <LEVEL_Y> is different from <LEVEL_X> _____ All levels are available. Select L1 |  |  | L1 |  | OS | - |  |
|  | 295 |  | 3 |  |  | 2000.00 |  | L1 | OS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | M_DRIVERACTIONS: 34: Level 0 selected 35: Level 1 selected 36: Level 2 selected 37: Level 3 selected 38: Level NTC selected |  |  | L1 |  | OS | - |  |
|  | 296 |  | 4 |  |  | 2000.00 |  | L1 | OS |  | The Level symbol "Level 0/1/2/3/NTC" is displayed |  | O |  | DMI |  | Selected level <LEVEL_Y> is different from <LEVEL_X> _____ L1 displayed |  |  | L1 |  | OS | - |  |
|  | 297 |  | 5 |  |  | 2000.00 |  | L1 | OS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<LEVEL_Y>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | OS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 298 |  | 1 |  |  | 2000.00 |  | L1 | OS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train is at standstill. |  |  | L1 |  | NP | - |  |
|  | 299 |  | 2 |  |  | 2000.00 |  | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 00000000000039 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 39 : Balise Group BG 00000000000039 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000100101100 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0001000 b |  |  |
| L_MAMODE | 15 | 000000110010000 b |  |  |
| L_ACKMAMODE | 15 | 000000011001000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00110001 b | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| N_ITER | 5 | 2 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00001111110010 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00001111110011 b |  |  |
| NID_PACKET | 8 | 00000101 b | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000010111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 000000001100100 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 001100 b |  |  |
| N_ITER | 5 | 3 d |  |  |
| D_LINK | 15 | 000000001100100 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000000001001 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 001100 b |  |  |
| D_LINK | 15 | 000000001100100 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000000001010 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 001100 b |  |  |
| D_LINK | 15 | 000000010010110 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000000001011 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 00 b |  |  |
| Q_LOCACC | 6 | 001100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 46 : Balise Group BG 00000000000046 1/1 - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 000 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 10001000 b | In-fill location reference |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000100110 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000000001001 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 65 : Balise Group BG 00000000001065 1/2 - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0000000000000 b |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| D_MAMODE | 15 | 000000000000000 b |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 0000000 b |  |  |
| L_MAMODE | 15 | 000000000000000 b |  |  |
| L_ACKMAMODE | 15 | 000000000000000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 65 : Balise Group BG 00000000001065 2/2 - Distance 405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000110010000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0001000 b |  |  |
| L_MAMODE | 15 | 000001001011000 b |  |  |
| L_ACKMAMODE | 15 | 000000001100100 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00110001 b | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| N_ITER | 5 | 2 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00001111101000 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00001111101001 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 70 : Balise Group BG 00000000001070 1/2 - Distance 550.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0000000000000 b |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| D_MAMODE | 15 | 000000000000000 b |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 0000000 b |  |  |
| L_MAMODE | 15 | 000000000000000 b |  |  |
| L_ACKMAMODE | 15 | 000000000000000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00110001 b | List of balises for SH area |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0000000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 70 : Balise Group BG 00000000001070 2/2 - Distance 555.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000011001000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0001000 b |  |  |
| L_MAMODE | 15 | 000000011111010 b |  |  |
| L_ACKMAMODE | 15 | 000000001100100 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00110001 b | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| N_ITER | 5 | 2 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00000001110000 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 85 : Balise Group BG 00000001101185 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 85 : Balise Group BG 00000001101185 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 89 : Balise Group BG 00001111101089 1/2 - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00001111101000 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00000000 b | Virtual Balise Cover Marker |  |
| NID_VBCMK | 6 | 000000 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 89 : Balise Group BG 00001111101089 2/2 - Distance 955.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00001111101000 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 187 : Balise Group BG 000000000011187 1/2 - Distance 1065.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 187 : Balise Group BG 000000000011187 2/2 - Distance 1070.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000110010000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0000110 b |  |  |
| L_MAMODE | 15 | 000001001011000 b |  |  |
| L_ACKMAMODE | 15 | 000000011001000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00110001 b | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| N_ITER | 5 | 2 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00011111010000 b |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 00011111010001 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 196 : Balise Group BG 000000000011196 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0010100 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 001001110001000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0000000000000 b |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| D_MAMODE | 15 | 000000000000000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0000000 b |  |  |
| L_MAMODE | 15 | 000000000000000 b |  |  |
| L_ACKMAMODE | 15 | 000000000000000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0010100 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 196 : Balise Group BG 000000000011196 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000011001000 b |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 0000110 b |  |  |
| L_MAMODE | 15 | 000000111000010 b |  |  |
| L_ACKMAMODE | 15 | 000000001100100 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 210 : Balise Group BG 000001010011210 1/2 - Distance 1450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000101001101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 210 : Balise Group BG 000001010011210 2/2 - Distance 1455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000101001101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| Step 228: DMI Events Distance 1750.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=2 | 10 s |

| - Step 244 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10011011 b |  |  |
| L_MESSAGE | 10 | 0000001010 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 246 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 1750.00 m Delay : 50.00 s Conditional Message Id : 155 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 00100000 b |  |  |
| L_MESSAGE | 10 | 0000001011 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 248 : Message 159 - Session Established (RBC n? 1) - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 250 : Message 157 - SoM Position Report (RBC n? 1) - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10011101 b |  |  |
| L_MESSAGE | 10 | 0000011000 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 01 b |  |  |
| NID_PACKET | 8 | 00000000 b | Position report |  |
| L_PACKET | 13 | 0000001110010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 000100000000000101001101 b |  |  |
| D_LRBG | 15 | 000000100101100 b |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 000000000000000 b |  |  |
| L_DOUBTUNDER | 15 | 000000000000000 b |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0000000 b |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 262 : Message 129 - Validated Train Data (RBC n? 1) - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10000001 b |  |  |
| L_MESSAGE | 10 | 0000100100 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 00000000 b | Position report |  |
| L_PACKET | 13 | 0000001110010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 000100000000000101001101 b |  |  |
| D_LRBG | 15 | 000000100101100 b |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 000000000000000 b |  |  |
| L_DOUBTUNDER | 15 | 000000000000000 b |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0000000 b |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 00001011 b | Validated train data |  |
| L_PACKET | 13 | 0000001100000 b |  |  |
| NC_CDTRAIN | 4 | 0000 b |  |  |
| NC_TRAIN | 15 | 000000000000000 b |  |  |
| L_TRAIN | 12 | 000000101000 b |  |  |
| V_MAXTRAIN | 7 | 0110010 b |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 0000000 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 264 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 1750.00 m Delay : 30.00 s Conditional Message Id : 129 Back Delay : 30.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 00001000 b |  |  |
| L_MESSAGE | 10 | 0000001110 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 000100000000000101001101 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |

| - Step 269 : Message 132 - MA Request (RBC n? 1) - Distance 1750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10000100 b |  |  |
| L_MESSAGE | 10 | 0000011001 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 00000 b |  |  |
| NID_PACKET | 8 | 00000000 b | Position report |  |
| L_PACKET | 13 | 0000001110010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 000100000000000101001101 b |  |  |
| D_LRBG | 15 | 000000110010000 b |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 000000000000000 b |  |  |
| L_DOUBTUNDER | 15 | 000000000000000 b |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0000000 b |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 274 : Message 3 - Movement Authority (RBC n? 1) - Distance 1750.00 m Delay : 15.00 s Conditional Message Id : 132 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 00000011 b |  |  |
| L_MESSAGE | 10 | 0000101011 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 000100000000000101001101 b |  |  |
| NID_PACKET | 8 | 00001111 b | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001000010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 000010111011100 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0101000 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 01010000 b | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001010101 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 000000000000000 b |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 0001100 b |  |  |
| L_MAMODE | 15 | 000010101111000 b |  |  |
| L_ACKMAMODE | 15 | 000000000000000 b |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 286 : Message 24 - General Message (RBC n? 1) - Distance 1900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048909 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 288 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 1900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 290 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 1900.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048909 d |  |  |

| Step 294: DMI Events Distance 2000.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=1 | 10 s |